3GPP TSG-CT WG1 Meeting #154	C1-252543

# Wuhan, China, 07-11 April 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 24.812 | CR | 0012 | rev | 2 | Current version: | 19.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | x | Radio Access Network |  | Core Network | x |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Update of conclusion for KI#3 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | LG Electronics, Apple, Huawei, HiSilicon |  |  |  |  |  |  |  |  |  |
| Source to TSG: | C1 |  |  |  |  |  |  |  |  |  |
| Work item code: | FS_MINT_Ph2 |  |  |  |  |  | Date: |  |  | 2025-03-31 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | TR 24.812 does not include details of the Tracking Area Update (TAU) procedure in the conclusion for Key Issue #3 (Attach to the 4G VPLMN providing disaster roaming services). This CR updates the conclusion based on the agreed Solution #3 by specifying the TAU initiation behavior in disaster roaming scenarios, aligned with Rel-17 MINT. |  |  |  |  |  |  |  |  |
| Summary of change: |  | This CR updates the conclusion for Key Issue #3 to reflect that when a disaster condition is detected on the 5G PLMN according to the conclusion of Key Issue #1 and the UE is registered in the 5G PLMN, the UE may initiate a tracking area update procedure to perform an inter-system change from 5GS to EPS of the PLMN for disaster roaming services. This TAU procedure may include a new EPS update type (e.g., "disaster roaming update") to indicate that the procedure is triggered by a disaster condition. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | If not approved, the conclusion for Key Issue #3 in TR 24.812 will not reflect the updated TAU initiation behavior in disaster roaming scenarios as described in Solution #3. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 8.3 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

* * * First Change * * * *

## 8.3	Conclusions on Key Issue #3

Solution #3 for Key Issue #3 which is able to distinguish attach request for normal service from disaster roaming service shall be progressed for normative work.

When the UE having 5G-only national roaming access to a VPLMN to obtain 4G connectivity service under disaster conditions, performs an attach procedure in a 4G VPLMN which supports disaster roaming, the UE indicates a new EPS attach type in the ATTACH REQUEST message to differentiate between normal attach and attach due to disaster roaming. The 4G VPLMN that provides disaster roaming service should be able to distinguish the attach request for normal service from disaster roaming service. The 4G VPLMN that provides disaster roaming service determines a registration area for the UE such that the EPS tracking area list contains only those tracking area identities (TAIs) that cover the area of the disaster condition.

If the UE does not have a valid mapped 4G-GUTI assigned by the 5G PLMN with disaster condition and the 5G PLMN with disaster condition is not the HPLMN of the UE, in addition to the new EPS attach type, the UE also indicates the 5G PLMN with disaster condition in the ATTACH REQUEST message.

The higher priority PLMN search can be modified under disaster condition.

In addition, when a disaster condition is detected on the 5G PLMN according to the conclusion of Key Issue #1, and the UE is registered in the 5G PLMN, the UE may initiate a tracking area update procedure to perform an inter-system change from 5GS to EPS of the PLMN for disaster roaming services. This tracking area update procedure may include a new EPS update type (e.g., "disaster roaming update") to indicate that the procedure is triggered by a disaster condition.

* * * End of Changes * * * *