3GPP TS 29.521 V16.1.0 (2019-09)

Technical Specification

3rd Generation Partnership Project;

Technical Specification Group Core Network and Terminals;

5G System; Binding Support Management Service;
Stage 3

(Release 16)

The present document has been developed within the 3rd Generation Partnership Project (3GPP TM) and may be further elaborated for the purposes of 3GPP.
The present document has not been subject to any approval process by the 3GPP Organizational Partners and shall not be implemented.
This Specification is provided for future development work within 3GPP only. The Organizational Partners accept no liability for any use of this Specification.
Specifications and Reports for implementation of the 3GPP TM system should be obtained via the 3GPP Organizational Partners' Publications Offices.

Keywords

3GPP

Postal address

3GPP support office address

650 Route des Lucioles - Sophia Antipolis

Valbonne - FRANCE

Tel.: +33 4 92 94 42 00 Fax: +33 4 93 65 47 16

Internet

http://www.3gpp.org

Copyright Notification

No part may be reproduced except as authorized by written permission.
The copyright and the foregoing restriction extend to reproduction in all media.

© 2019, 3GPP Organizational Partners (ARIB, ATIS, CCSA, ETSI, TSDSI, TTA, TTC).

All rights reserved.

UMTS™ is a Trade Mark of ETSI registered for the benefit of its members

3GPP™ is a Trade Mark of ETSI registered for the benefit of its Members and of the 3GPP Organizational Partners
LTE™ is a Trade Mark of ETSI registered for the benefit of its Members and of the 3GPP Organizational Partners

GSM® and the GSM logo are registered and owned by the GSM Association

Contents

Foreword	5

1	Scope	6

2	References	6

3	Definitions and abbreviations	7

3.1	Definitions	7

3.2	Abbreviations	7

4	Binding Support Management Service	7

4.1	Service Description	7

4.1.1	Overview	7

4.1.2	Service Architecture	8

4.1.3	Network Functions	8

4.1.3.1	Binding Support Function (BSF)	8

4.1.3.2	NF Service Consumers	8

4.2	Service Operations	9

4.2.1	Introduction	9

4.2.2	Nbsf_Management_Register Service Operation	9

4.2.2.1	General	9

4.2.2.2	Register a new PCF Session binding information	9

4.2.3	Nbsf_Management_Deregister Service Operation	11

4.2.3.1	General	11

4.2.3.2	Deregister an individual PCF Session binding information	11

4.2.4	Nbsf_Management_Discovery Service Operation	11

4.2.4.1	General	11

4.2.4.2	Retrieve the PCF Session binding information for a given tuple	12

5	Nbsf_Management Service API	13

5.1	Introduction	13

5.2	Usage of HTTP	13

5.2.1	General	13

5.2.2	HTTP standard headers	14

5.2.2.1	General	14

5.2.2.2	Content type	14

5.2.3	HTTP custom headers	14

5.2.3.1	General	14

5.3	Resources	14

5.3.1	Resource Structure	14

5.3.2	Resource: PCF Session Bindings	15

5.3.2.1	Description	15

5.3.2.2	Resource definition	15

5.3.2.3	Resource Standard Methods	15

5.3.2.3.1	POST	15

5.3.2.3.2	GET	16

5.3.3	Resource: Individual PCF Session Binding	17

5.4	Custom Operations without associated resources	18

5.5	Notifications	18

5.6	Data Model	18

5.6.1	General	18

5.6.2	Structured data types	19

5.6.2.1	Introduction	19

5.6.2.2	Type PcfBinding	20

5.6.3	Simple data types and enumerations	21

5.6.3.1	Introduction	21

5.6.3.2	Simple data types	21

5.7	Error handling	21

5.8	Feature negotiation	22

5.9	Security	22

Annex A (normative):	OpenAPI specification	23

A.1	General	23

A.2	Nbsf_Management API	23

Annex C (informative):	Change history	29

# Foreword

This Technical Specification has been produced by the 3rd Generation Partnership Project (3GPP).

The contents of the present document are subject to continuing work within the TSG and may change following formal TSG approval. Should the TSG modify the contents of the present document, it will be re-released by the TSG with an identifying change of release date and an increase in version number as follows:

Version x.y.z

where:

x	the first digit:

1	presented to TSG for information;

2	presented to TSG for approval;

3	or greater indicates TSG approved document under change control.

y	the second digit is incremented for all changes of substance, i.e. technical enhancements, corrections, updates, etc.

z	the third digit is incremented when editorial only changes have been incorporated in the document.

# 1	Scope

The present specification provides the stage 3 definition of the Binding Support Management Service of the 5G System.

The 5G System Architecture is defined in 3GPP TS 23.501 [2]. The stage 2 definition and related procedures for Binding Support Management Service is specified in 3GPP TS 23.502 [3] and 3GPP TS 23.503 [4].

The 5G System stage 3 call flows are provided in 3GPP TS 29.513 [5].

The Technical Realization of the Service Based Architecture and the Principles and Guidelines for Services Definition are specified in 3GPP TS 29.500 [6] and 3GPP TS 29.501 [7].

The Binding Support Management Service is provided by the Binding Support Function (BSF).

# 2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[2]	3GPP TS 23.501: "System Architecture for the 5G System; Stage 2".

[3]	3GPP TS 23.502: "Procedures for the 5G System; Stage 2".

[4]	3GPP TS 23.503: "Policy and Charging Control Framework for the 5G System; Stage 2".

[5]	3GPP TS 29.513: "5G System; Policy and Charging Control signalling flows and QoS parameter mapping; Stage 3".

[6]	3GPP TS 29.500: "5G System; Technical Realization of Service Based Architecture; Stage 3".

[7]	3GPP TS 29.501: "5G System; Principles and Guidelines for Services Definition; Stage 3".

[8]	IETF RFC 7540: "Hypertext Transfer Protocol Version 2 (HTTP/2)".

[9]	IETF RFC 8259: "The JavaScript Object Notation (JSON) Data Interchange Format".

[10]	3GPP TS 29.571: "5G System; Common Data Types for Service Based Interfaces Stage 3".

[11]	OpenAPI: "OpenAPI 3.0.0 Specification", https://github.com/OAI/OpenAPI-Specification/blob/master/versions/3.0.0.md.

[12]	3GPP TS 29.510: "5G System; Network Function Repository Services; Stage 3".

[13]	IETF RFC 7807: "Problem Details for HTTP APIs".

[14]	3GPP TS 29.213: " Policy and Charging Control signalling flows and Quality of Service (QoS) parameter mapping".

[15]	3GPP TS 33.501: "Security architecture and procedures for 5G system".

[16]	IETF RFC 6749: "The OAuth 2.0 Authorization Framework".

[17]	3GPP TS 23.527: "5G System; Restoration Procedures".

[18]	3GPP TR 21.900: "Technical Specification Group working methods".

[19]	3GPP TS 23.316: "Wireless and wireline convergence access support for the 5G System (5GS)".

# 3	Definitions and abbreviations

## 3.1	Definitions

For the purposes of the present document, the terms and definitions given in 3GPP TR 21.905 [1] and the following apply. A term defined in the present document takes precedence over the definition of the same term, if any, in 3GPP TR 21.905 [1].

## 3.2	Abbreviations

For the purposes of the present document, the abbreviations given in 3GPP TR 21.905 [1] and the following apply. An abbreviation defined in the present document takes precedence over the definition of the same abbreviation, if any, in 3GPP TR 21.905 [1].

5G-RG	5G Residential Gateway

AF	Application Function

BSF	Binding Support Function

DNN	Data Network Name

DRA	Diameter Routing Agent

HTTP	Hypertext Transfer Protocol

FN-RG	Fixed Network Residential Gateway

FQDN	Fully Qualified Domain Name

GPSI	Generic Public Subscription Identifier

JSON	JavaScript Object Notation HTTP	Hypertext Transfer Protocol

MAC	Media Access Control

NEF	Network Exposure Function

NRF	Network Repository Function

PCF	Policy Control Function

SMF	Session Management Function

S-NSSAI	Single Network Slice Selection Assistance Information

SUPI	Subscription Permanent Identifier

UDR	Unified Data Repository

# 4	Binding Support Management Service

## 4.1	Service Description

### 4.1.1	Overview

The Binding Support Management Service as defined in 3GPP TS 23.502 [3] and 3GPP TS 23.503 [4], is provided by the Binding Support Function (BSF).

The Nbsf_Management service is used for the BSF to provide a PDU session binding functionality, which ensures that an AF request for a certain PDU Session reaches the relevant PCF holding the PDU Session information.

This service:

-	allows NF service consumers to register, update and remove the binding information; and

-	allows NF service consumers to retrieve the binding information.

### 4.1.2	Service Architecture

The 5G System Architecture is defined in 3GPP TS 23.501 [2]. The Policy and Charging related 5G architecture is also described in 3GPP TS 23.503 [4] and 3GPP TS 29.513 [5].

The Binding Support Management Service (Nbsf_Management) is exhibited by the Binding Support Function (BSF).

Known consumers of the Nbsf_management service are:

-	Policy Control Function (PCF)

-	Network Exposure Function (NEF)

-	Application Function (AF)

As described in 3GPP TS 23.503 [4], the BSF is a function that can be deployed standalone or can be the functionality provided by other network functions, such as PCF, UDR, NRF, SMF.

NOTE:	The PCF accesses the Nbsf_management service at the BSF via an internal interface when it is collocated with BSF.

Figure 4.1.2-1: Reference Architecture for the Nbsf_management service; SBI representation

### 4.1.3	Network Functions

#### 4.1.3.1	Binding Support Function (BSF)

The BSF:

-	stores the binding information for a certain PDU Session; and

-	discovers the binding information (e.g. the address information of the selected PCF).

The BSF allows NF service consumers (e.g.PCF) to register, update and remove the binding information, and allows NF service consumers (e.g. AF, NEF) to discover the binding information (e.g the address information of the selected PCF).

The BSF can be deployed standalone or can be collocated with other network functions, such as PCF, UDR, NRF andSMF.

#### 4.1.3.2	NF Service Consumers

The Policy Control Function (PCF):

-	registers the binding information in the BSF for a UE when an IPv4 address and/or IPv6 prefix is allocated, or a MAC address is used for the PDU session; and,

-	removes the binding information in the BSF when an IPv4 address and/or IPv6 prefix is released, or a MAC address is not used for the PDU Session.

The Network Exposure Function (NEF):

-	provides a means for the Application Functions to securely interact with the Policy framework for policy control to 3GPP network. During the procedure, it needs to discover the selected PCF by using the Nbsf_Management_Discovery service operation.

The Application Function (AF):

-	discover the selected PCF by using the Nbsf_Management_Discovery service operation when it is allowed to interact directly with the policy framework for policy control.

## 4.2	Service Operations

### 4.2.1	Introduction

Table 4.2.1-1: Operations of the Nbsf_Management Service

| Service operation name | Description | Initiated by |
| --- | --- | --- |
| Nbsf_Management_Register | This service operation is used to register the binding information for a UE when an IPv4 address and/or an IPv6 prefix is allocated for an IP PDU Session or a MAC address is used for an Ethernet PDU session. | NF service consumer (PCF) |
| Nbsf_Management_Deregister | This service operation is used to deregister the binding information for a UE when the PDU Session is released. | NF service consumer (PCF) |
| Nbsf_Management_Discovery | This service operation is used by an NEF or AF to discover a selected PCF . | NF service consumer (NEF, AF) |

### 4.2.2	Nbsf_Management_Register Service Operation

#### 4.2.2.1	General

This service operation allows the NF service consumer (e.g. PCF) to register the session binding information for a UE in the BSF by providing the user identity, the DNN, the UE address(es) and the selected PCF address for a certain PDU Session to the BSF, and BSF stores the information.

If the BindingUpdate feature is not supported and if the NF service consumer (e.g. PCF) receives a new UE address (e.g. IPv6 prefix) and already registered session binding information for this PDU session, the NF service consumer (e.g. PCF) shall register a new session binding information in the BSF.

#### 4.2.2.2	Register a new PCF Session binding information

Figure 4.2.2.2-1: NF service consumer register a new PCF Session binding information

The NF service consumer shall invoke the Nbsf_Management_Register service operation to register the session binding information for a UE in the BSF. The NF service consumer shall send an HTTP POST request with "{apiRoot}/nbsf-management/v1/pcfBindings" as Resource URI representing the "PCF Session Bindings", as shown in figure 4.2.2.2-1, step 1, to create a binding information for an "Individual PCF Session Binding" according to the information (e.g. UE address(es), SUPI, GPSI, DNN, S-NSSAI) in message body. The PcfBinding data structure provided in the request body shall include:

-	if the MultiUeAddr feature is not supported or if the feature not yet known, address information of the served UE consisting of:

(i)	either IP address information consisting of:

+	the IPv4 address encoded as "ipv4Addr" attribute; and/or

+	the /128 IPv6 address, the IPv6 address prefix or an IPv6 prefix shorter than/64 encoded as "ipv6Prefix" attribute; or

(ii)	the MAC address encoded as "macAddr48" attribute;

Otherwise, address information of the served UE consisting of:

(i)	any IP address information consisting of:

+	the IPv4 address encoded as "ipv4Addr" attribute;

+		the IPv6 address prefix encoded as "ipv6Prefix" attribute; and/or

+		the additional IPv6 address prefixes encoded as "addIpv6Prefixes" attribute; or

(ii)	the MAC address encoded as "macAddr48" attribute and/or the additional MAC addresses encoded as "addMacAddrs" attribute;

-	PCF address information consisting of:

(i)	if the PCF supports the Npcf_PolicyAuthorization service:

+	the FQDN of the PCF encoded as "pcfFqdn" attribute; and/or

+	a description of IP endpoints at the PCF hosting the Npcf_PolicyAuthorization service encoded as "pcfIpEndPoints" attribute; and

(ii)	if the PCF supports the Rx interface:

+	the Diameter host id of the PCF encoded as "pcfDiamHost"; and

+	the Diameter realm of the PCF and "pcfDiamRealm" attributes;

-	DNN encoded as "dnn" attribute; and

-	S-NSSAI encoded as "snssai" attribute;

and may include:

-	SUPI encoded as "supi" attribute;

-	GPSI encoded as "gpsi" attribute; and

-	IPv4 address domain encoded as "ipDomain" attribute.

Upon the reception of an HTTP POST request with: "{apiRoot}/nbsf-management/v1/pcfBindings" as Resource URI and PcfBinding data structure as request body, the BSF shall:

-	create new binding information;

-	assign a bindingId; and

-	store the binding information.

The PCF as NF service consumer may provide PCF Id in "pcfId" attribute and recovery timestamp in "recoveryTime" attribute. The BSF may use the "pcfId" attribute to supervise the status of the PCF as described in subclause 5.2 of 3GPP TS 29.510 [12] and perform necessary cleanup upon status change of the PCF later, and/or both the "pcfId" attribute and the "recoveryTime" attribute in cleanup procedure as described in subclause 6.4 of 3GPP TS 23.527 [17].

If the BSF created an "Individual PCF Session Binding" resource, the BSF shall respond with "201 Created" with the message body containing a representation of the created binding information, as shown in figure 4.2.2.2.2-1, step 2. The BSF shall include a Location HTTP header field. The Location header field shall contain the URI of the created binding information i.e. "{apiRoot}/nbsf-management/v1/pcfBindings/{bindingId}".

### 4.2.3	Nbsf_Management_Deregister Service Operation

#### 4.2.3.1	General

This service operation allows the service consumer to remove the session binding information for a UE in the BSF. It is executed by deleting a given resource identified by an "Individual PCF Session Binding" resource identifier. The operation is invoked by issuing an HTTP DELETE request on the URI representing the specific session binding information.

#### 4.2.3.2	Deregister an individual PCF Session binding information

Figure 4.2.3.2-1: Session Binding Information Deregistration

The NF service consumer shall invoke the Nbsf_Management_DeRegister service operation to deregister the session binding information for a UE in the BSF. The NF service consumer shall send an HTTP DELETE request with "{apiRoot}/nbsf-management/v1/pcfBindings/{bindingId}" as Resource URI, where "{bindingId}" is the "Individual PCF Session Binding" resource identifier that is to be deleted.

Upon the the reception of an HTTP DELETE request with: "{apiRoot}/nbsf-management/v1/pcfBindings/{bindingId}" as Resource URI, the BSF shall:

-	remove the corresponding binding information.

If the HTTP DELETE request message from the NF service consumer is accepted, the BSF shall respond with "204 No Content". If the Individual PCF Session Binding resource does not exist, the BSF shall respond with "404 Not Found".

### 4.2.4	Nbsf_Management_Discovery Service Operation

#### 4.2.4.1	General

This service operation allows the service consumer to use the HTTP GET method to obtain the address information of the selected PCF.

#### 4.2.4.2	Retrieve the PCF Session binding information for a given tuple

Figure 4.2.4.2-1: NF service consumer retrieve the PCF Session binding information for a given tuple

The NF service consumer shall invoke the Nbsf_Management_Discovery service operation to obtain address information of the the selected PCF for a PDU session in the BSF. The NF service consumer shall send an HTTP GET request with "{apiRoot}/nbsf-management/v1/pcfBindings" as Resource URI, where "query parameters" shall include:

-	UE address;

and may include:

-	SUPI or GPSI;

-	DNN and optionally S-NSSAI; and

-	IPv4 address domain.

NOTE:	The query parameters S-NSSAI and/or IPv4 addres domain is helpful in the scenario of IPv4 address overlapping where the same IPv4 address may be allocated to UE PDU sessions.

Upon the reception of an HTTP GET request with: "{apiRoot}/nbsf-management/v1/pcfBindings" as Resource URI, the BSF shallsearch the corresponding binding information. If "ipv6Prefix" is used as an UE IPv6 address in the query parameter, the BSF shall use the longest prefix match to find a matching IPv6 prefix so that the IPv6 address in the query parameter is within the address range covered by that matching IPv6 prefix. The IPv6 address in the query parameter shall be formatted as an IPv6 prefix value including the trailing prefix length "/128".

If the HTTP request message from the NF service consumer is accepted and a session binding resource matching the query parameters exists, the BSF shall reply with a "200 OK" HTTP response containing the corresponding PcfBinding data structure, as provided by the PCF during the Nbsf_Management_Register Service Operation in the response body. If the PCF Session Binding resource does not exist, the BSF shall respond with "404 Not Found". If an invalid combination of query parameters (i.e. a combination without UE address(es)) is contained in the request URI, the BSF shall respond with an "400 Bad Request" HTTP error code containing "INVALID_QUERY_PARAM" as application error within the ProblemDetails IE. If more then one PCF Session Binding resources are found, the BSF shall respond with "400 Bad Request" HTTP error code containing "MULTIPLE_BINDING_INFO_FOUND" as application error within the ProblemDetails IE.

- 4.2.5	Nbsf_Management_Update Service Operation

- 4.2.5.1	General

This service operation allows the NF service consumer to update an existing session binding information for a UE in the BSF by providing information to be updated (e.g. the UE address(es)) for a PDU Session, and BSF updates the session binding information.

- 4.2.5.2	Update an existing PCF Session binding information

Figure 4.2.5.2-1: NF service consumer update an existing PCF Session binding information

If the feature BindingUpdate is supported, the NF service consumer shall invoke the Nbsf_Management_Update service operation to update the session binding information for a UE in the BSF. The NF service consumer shall send an HTTP PATCH request with "{apiRoot}/nbsf-management/v1/pcfBindings/{bindingId}" as Resource URI, where "{bindingId}" is the "Individual PCF Session Binding" resource identifier that is to be updated. The PcfBindingPatch data structure provided in the request body shall include the information to be updated (e.g. the UE address(es)).

Upon the reception of an HTTP PATCH request with: "{apiRoot}/nbsf-management/v1/pcfBindings/{bindingId}" as Resource URI and PcfBindingPatch data structure as request body, the BSF shall update the binding information.

If the BSF successfully updated an "Individual PCF Session Binding" resource, the BSF shall respond with "200 OK" with the message body containing a representation of the updated session binding information, as shown in figure 4.2.5.2-1, step 2.

Editor's note:	200 OK with the latest resource representation helps the consumer know the updated status without issuing a GET. Whether 204 needs to be supported in addition in the PATCH response for saving the transmission resource is for FFS.

# 5	Nbsf_Management Service API

## 5.1	Introduction

The Nbsf_management Service shall use the Nbsf_management API.

The request URI used in each HTTP request from the NF service consumer towards the BSF shall have the structure defined in subclause 4.4.1 of 3GPP TS 29.501 [7], i.e.:

{apiRoot}/{apiName}/{apiVersion}/{apiSpecificResourceUriPart}

with the following components:

-	The {apiRoot} shall be set as described in 3GPP TS 29.501 [7].

-	The {apiName} shall be "nbsf-management".

-	The {apiVersion} shall be "v1".

-	The {apiSpecificResourceUriPart} shall be set as described in subclause 5.3.

## 5.2	Usage of HTTP

### 5.2.1	General

HTTP/2, IETF RFC 7540 [8], shall be used as specified in clause 5 of 3GPP TS 29.500 [6].

HTTP/2 shall be transported as specified in subclause 5.3 of 3GPP TS 29.500 [6].

The OpenAPI [11] specification of HTTP messages and content bodies for the Nbsf_management is contained in Annex A.

### 5.2.2	HTTP standard headers

#### 5.2.2.1	General

See subclause 5.2.2 of 3GPP TS 29.500 [6] for the usage of HTTP standard headers.

#### 5.2.2.2	Content type

JSON, IETF RFC 8259 [9], shall be used as content type of the HTTP bodies specified in the present specification as specified in subclause 5.4 of 3GPP TS 29.500 [6]. The use of the JSON format shall be signalled by the content type "application/json".

"Problem Details" JSON object shall be used to indicate additional details of the error in a HTTP response body and shall be signalled by the content type "application/problem+json", as defined in IETF RFC 7807 [13].

### 5.2.3	HTTP custom headers

#### 5.2.3.1	General

The Nbsf_Management Service API shall support HTTP custom header fields specified in subclause 5.2.3.2 of 3GPP TS 29.500 [6].

In this release of the specification, no specific custom headers are defined for the Nbsf_Management Service API.

## 5.3	Resources

### 5.3.1	Resource Structure

The structure of the Resource URI of the Nbsf_Management service is shown in figure 5.3.1-1.

Figure 5.3.1-1: Resource URI structure of the Nbsf_Management API

Table 5.3.1-1 provides an overview of the resources and applicable HTTP methods.

Table 5.3.1-1: Resources and methods overview

| Resource name | Resource URI | HTTP method or custom operation | Description |
| --- | --- | --- | --- |
| PCF Session Bindings | {apiRoot}/ nbsf-management/v1 /pcfBindings | POST | Register new PCF Session binding information of a given UE address in BSF. |
|  |  | GET | Retrieve the Session binding information i.e. PCF address information of a given tuple (UE address(es), SUPI; GPSI, DNN, snssai). |
| Individual PCF Session Binding | {apiRoot}/ nbsf-management/v1 /pcfBindings /{bindingId} | DELETE | Deregister existing PCF Session binding information from BSF. |
|  |  | PATCH | Update an existing PCF Session binding information from BSF. |

### 5.3.2	Resource: PCF Session Bindings

#### 5.3.2.1	Description

This resource represents a collection of the different PCF Session binding information of given UE address(es) registered in the BSF.

#### 5.3.2.2	Resource definition

Resource URI: {apiRoot}/nbsf-management/v1/pcfBindings

This resource shall support the resource URI variables defined in table 5.3.2.2-1.

Table 5.3.2.2-1: Resource URI variables for this resource

| Name | Definition |
| --- | --- |
| apiRoot | See subclause 5.1 |

#### 5.3.2.3	Resource Standard Methods

##### 5.3.2.3.1	POST

This method shall support the URI query parameters specified in table 5.3.2.3.1-1.

Table 5.3.2.3.1-1: URI query parameters supported by the POST method on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| n/a |  |  |  |  |

This method shall support the request data structures specified in table 5.3.2.3.1-2 and the response data structures and response codes specified in table 5.3.2.3.1-3.

Table 5.3.2.3.1-2: Data structures supported by the POST Request Body on this resource

| Data type | P | Cardinality | Description |
| --- | --- | --- | --- |
| PcfBinding | M | 1 | Register a new Individual PCF binding information. |

Table 5.3.2.3.1-3: Data structures supported by the POST Response Body on this resource

| Data type | P | Cardinality | Response codes | Description |
| --- | --- | --- | --- | --- |
| PcfBinding | M | 1 | 201 Created | The creation of an individual PCF session biding. |
| NOTE: The mandatory HTTP error status codes for the POST method listed in table 5.2.7.1-1 of 3GPP TS 29.500 [6] shall also apply. |  |  |  |  |

##### 5.3.2.3.2	GET

This method shall support the URI query parameters specified in table 5.3.3.3.1-1.

Table 5.3.2.3.1-1: URI query parameters supported by the GET method on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| ipv4Addr | Ipv4Addr | C | 0..1 | The IPv4 Address of the served UE. (NOTE 1) (NOTE 3) |
| ipv6Prefix | Ipv6Prefix | C | 0..1 | The IPv6 Address of the served UE. (NOTE 1) (NOTE 3) The NF service consumer shall append '/128' to the IPv6 address in the attribute value. E.g. '2001:db8:85a3::8a2e:370:7334/128'. |
| macAddr48 | MacAddr48 | C | 0..1 | The MAC Address of the served UE. (NOTE 1) |
| dnn | Dnn | O | 0..1 | DNN |
| supi | Supi | O | 0..1 | Subscription Permanent Identifier |
| gpsi | Gpsi | O | 0..1 | Generic Public Subscription Identifier |
| snssai | Snssai | O | 0..1 | The identification of slice. (NOTE 2) |
| ipDomain | String | O | 0..1 | The IPv4 address domain identifier. (NOTE 2) |
| supp-feat | SupportedFeatures | O | 0..1 | To filter irrelevant responses related to unsupported features. |
| NOTE 1: One and only one of query parameter ipv4Addr, ipv6Prefix or macAddr48 shall be present. NOTE 2: The query parameters snssai and/or ipDomain, if applicable (IPv4 address overlapping), shall be present with query parameter ipv4Addr. NOTE 3: 5G-RG and FN-RG replaces UE for wireline access support. See 3GPP TS 23.316 [19]. |  |  |  |  |

This method shall support the request data structures specified in table 5.3.3.3.1-2 and the response data structures and response codes specified in table 5.3.3.3.1-3.

Table 5.3.2.3.1-2: Data structures supported by the GET Request Body on this resource

| Data type | P | Cardinality | Description |
| --- | --- | --- | --- |
| n/a |  |  |  |

Table 5.3.2.3.1-3: Data structures supported by the GET Response Body on this resource

| Data type | P | Cardinality | Response codes | Description |
| --- | --- | --- | --- | --- |
| PcfBinding | M | 1 | 200 OK | The individual PCF session binding information resource matching the query parameter(s) is returned. |
| n/a |  |  | 204 No Content | There is no PCF session binding information matching the query parameter(s). |
| ProblemDetails | M | 1 | 400 Bad Request | More than one binding information is found. (NOTE 2) |
| NOTE 1: The mandatory HTTP error status codes for the GET method listed in table 5.2.7.1-1 of 3GPP TS 29.500 [6] shall also apply. NOTE 2: Failure cases are described in subclause 5.7. |  |  |  |  |

### 5.3.3	Resource: Individual PCF Session Binding

- 5.3.3.1	Description

This resource represents a collection of the different PCF Session binding information of given UE address(es) registered in the BSF.

- 5.3.3.2	Resource definition

Resource URI: {apiRoot}/nbsf-management/v1/pcfBindings/{bindingId}

This resource shall support the resource URI variables defined in table 5.3.3.2-1.

Table 5.3.3.2-1: Resource URI variables for this resource

| Name | Definition |
| --- | --- |
| apiRoot | See subclause 5.1 |
| bindingId | Represents the individual PCF Session Binding. To enable that the value is used as part of a URI, the string shall only contain characters allowed according to the "lower-with-hyphen" naming convention defined in 3GPP TS 29.501 [7]. |

- 5.3.3.3	Resource Standard Methods

- 5.3.3.3.1	DELETE

This method shall support the URI query parameters specified in table 5.3.3.3.2-1.

Table 5.3.3.3.1-1: URI query parameters supported by the DELETE method on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| n/a |  |  |  |  |

This method shall support the request data structures specified in table 5.3.3.3.1-2 and the response data structures and response codes specified in table 5.3.3.3.1-3.

Table 5.3.3.3.1-2: Data structures supported by the DELETE Request Body on this resource

| Data type | P | Cardinality | Description |
| --- | --- | --- | --- |
| n/a |  |  |  |

Table 5.3.3.3.1-3: Data structures supported by the DELETE Response Body on this resource

| Data type | P | Cardinality | Response codes | Description |
| --- | --- | --- | --- | --- |
| n/a |  |  | 204 No Content | Successful case: The Individual PCF session binding information resource is deleted. |
| NOTE: The mandatory HTTP error status codes for the POST method listed in table 5.2.7.1-1 of 3GPP TS 29.500 [6] shall also apply. |  |  |  |  |

- 5.3.3.3.2	PATCH

This method shall support the URI query parameters specified in table 5.3.3.3.2-1.

Table 5.3.3.3.2-1: URI query parameters supported by the PATCH method on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| n/a |  |  |  |  |

This method shall support the request data structures specified in table 5.3.3.3.2-2 and the response data structures and response codes specified in table 5.3.3.3.2-3.

Table 5.3.3.3.2-2: Data structures supported by the PATCH Request Body on this resource

| Data type | P | Cardinality | Description |
| --- | --- | --- | --- |
| PcfBindingPatch | M | 1 | Update an individual PCF binding information. |

Table 5.3.3.3.2-3: Data structures supported by the PATCH Response Body on this resource

| Data type | P | Cardinality | Response codes | Description |
| --- | --- | --- | --- | --- |
| PcfBinding | M | 1 | 200 OK | Successful case: The Individual PCF session binding information resource is updated. |
| NOTE: The mandatory HTTP error status codes for the PATCH method listed in table 5.2.7.1-1 of 3GPP TS 29.500 [6] shall also apply. |  |  |  |  |

## 5.4	Custom Operations without associated resources

None in this release of this specification.

## 5.5	Notifications

None in this release of this specification.

## 5.6	Data Model

### 5.6.1	General

This subclause specifies the application data model supported by the API.

Table 5.6.1-1 specifies the data types defined for the Nbsf service based interface protocol.

Table 5.6.1-1: Nbsf specific Data Types

| Data type | Section defined | Description | Applicability |
| --- | --- | --- | --- |
| PcfBinding | 5.6.2.2 | Identifies an Individual PCF binding. |  |
| PcfBindingPatch | 5.6.2.3 | Identifies an Individual PCF binding used for Patch method. | BindingUpdate |

Table 5.6.1-2 specifies data types re-used by the Nbsf service based interface protocol from other specifications, including a reference to their respective specifications and when needed, a short description of their use within the Nbsf service based interface.

Table 5.6.1-2: Nbsf re-used Data Types

| Data type | Reference | Comments | Applicability |
| --- | --- | --- | --- |
| DateTime | 3GPP TS 29.571 [10] |  |  |
| DiameterIdentity | 3GPP TS 29.571 [10] |  |  |
| Dnn | 3GPP TS 29.571 [10] |  |  |
| Fqdn | 3GPP TS 29.510 [12] |  |  |
| Gpsi | 3GPP TS 29.571 [10] |  |  |
| IpEndPoint | 3GPP TS 29.510 [12] |  |  |
| Ipv4Addr | 3GPP TS 29.571 [10] |  |  |
| Ipv4AddrRm | 3GPP TS 29.571 [10] |  |  |
| Ipv6Prefix | 3GPP TS 29.571 [10] |  |  |
| Ipv6PrefixRm | 3GPP TS 29.571 [10] |  |  |
| MacAddr48 | 3GPP TS 29.571 [10] |  |  |
| NfInstanceId | 3GPP TS 29.571 [10] |  |  |
| ProblemDetails | 3GPP TS 29.571 [10] | Used in error responses to provide more detailed information about an error. |  |
| Snssai | 3GPP TS 29.571 [10] |  |  |
| Supi | 3GPP TS 29.571 [10] |  |  |
| SupportedFeatures | 3GPP TS 29.571 [10] | Used to negotiate the applicability of the optional features defined in table 5.8-1. |  |

### 5.6.2	Structured data types

#### 5.6.2.1	Introduction

This subclause defines the structures to be used in resource representations.

#### 5.6.2.2	Type PcfBinding

Table 5.6.2.2-1: Definition of type PcfBinding

| Attribute name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| Supi | Supi | O | 0..1 | Subscription Permanent Identifier |  |
| Gpsi | Gpsi | O | 0..1 | Generic Public Subscription Identifier |  |
| ipv4Addr | Ipv4Addr | C | 0..1 | The IPv4 Address of the served UE. (NOTE 4) |  |
| ipv6Prefix | Ipv6Prefix | C | 0..1 | The IPv6 Address Prefix of the served UE. (NOTE 4) (NOTE 5) |  |
| addIpv6Prefixes | array(Ipv6Prefix) | O | 1..N | The additional IPv6 Address Prefixes of the served UE. (NOTE 4) (NOTE 5) | MultiUeAddr |
| ipDomain | string | O | 0..1 | IPv4 address domain identifier. (NOTE 1) |  |
| macAddr48 | MacAddr48 | C | 0..1 | The MAC Address of the served UE. |  |
| addMacAddrs | array(MacAddr48) | O | 1..N | The additional MAC Addresses of the served UE. | MultiUeAddr |
| Dnn | Dnn | M | 1 | DNN |  |
| pcfFqdn | Fqdn | C | 0..1 | FQDN of the PCF hosting the Npcf_PolicyAuthorization service. (NOTE 2) |  |
| pcfIpEndPoints | array(IpEndPoint) | C | 1..N | IP end points of the PCF hosting the Npcf_PolicyAuthorization service. (NOTE 2) |  |
| pcfDiamHost | DiameterIdentity | C | 0..1 | The diameter host for an individual PCF. (NOTE 3) |  |
| pcfDiamRealm | DiameterIdentity | C | 0..1 | The diameter realm for an individual PCF. (NOTE 3) |  |
| Snssai | Snssai | M | 1 | The identification of slice. |  |
| suppFeat | SupportedFeatures | O | 1 | Used to negotiate the supported optional features as described in subclause 5.8. |  |
| pcfId | NfInstanceId | O | 0..1 | PCF instance identifier |  |
| recoveryTime | DateTime | O | 0..1 | Recovery time of the PCF |  |
| NOTE 1: The ipDomain attribute may only be provided if the ipv4Addr attribute is present. NOTE 2: At least one of pcfFqdn or pcfIpEndPoints shall be included if the PCF supports the Npcf_PolicyAuthorization service. NOTE 3: Both pcfDiamHost and pcfDiamRealm are provided if the PCF supports Rx interface. NOTE 4: 5G-RG and FN-RG replaces UE for wireline access support. See 3GPP TS 23.316 [19]. NOTE 5: An IPv6 prefix shorter than /64 or a full IPv6 address with a /128 prefix may be encoded as the "ipv6Prefix" attribute, according to 3GPP TS 23.316 [19], subclause 8.3.1. |  |  |  |  |  |

- 5.6.2.3	Type PcfBindingPatch

Table 5.6.2.3-1: Definition of type PcfBindingPatch

| Attribute name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| ipv4Addr | Ipv4AddrRm | O | 0..1 | The IPv4 Address of the served UE. (NOTE 2) |  |
| ipDomain | string | O | 0..1 | IPv4 address domain identifier. (NOTE 1) |  |
| ipv6Prefix | Ipv6PrefixRm | O | 0..1 | The IPv6 Address Prefix of the served UE. |  |
| addIpv6Prefixes | array(Ipv6Prefix) | O | 1..N | The additional IPv6 Address Prefixes of the served UE. (NOTE 2) (NOTE 3) | MultiUeAddr |
| macAddr48 | MacAddr48Rm | O | 0..1 | The MAC Address of the served UE. |  |
| addMacAddrs | array(MacAddr48) | O | 1..N | The additional MAC Addresses of the served UE. | MultiUeAddr |
| NOTE 1: If applicable, the consumer (e.g. PCF) shall also request to remove the ipDomain attribute if the ipv4Addr attribute is requested to be removed. NOTE 2: 5G-RG and FN-RG replaces UE for wireline access support. See 3GPP TS 23.316 [19]. NOTE 3: An IPv6 prefix shorter than /64 or a full IPv6 address with a /128 prefix can be encoded as the "ipv6Prefix" attribute, according to 3GPP TS 23.316 [19], subclause 8.3.1. |  |  |  |  |  |

### 5.6.3	Simple data types and enumerations

#### 5.6.3.1	Introduction

This subclause defines simple data types and enumerations that can be referenced from data structures defined in the previous subclauses.

#### 5.6.3.2	Simple data types

The simple data types defined in table 5.6.3.2-1 shall be supported.

Table 5.6.3.2-1: Simple data types

| Type Name | Type Definition | Description | Applicability |
| --- | --- | --- | --- |
| n/a |  |  |  |

## 5.7	Error handling

- 5.7.1	General

HTTP error handling shall be supported as specified in subclause 5.2.4 of 3GPP TS 29.500 [6].

For the Nbsf_Management Service API, HTTP error responses shall be supported as specified in subclause 4.8 of 3GPP TS 29.501 [7]. Protocol errors and application errors specified in table 5.2.7.2-1 of 3GPP TS 29.500 [6] shall be supported for an HTTP method if the corresponding HTTP status codes are specified as mandatory for that HTTP method in table 5.2.7.1-1 of 3GPP TS 29.500 [6]. In addition, the requirements in the following subclauses shall apply.

- 5.7.2	Protocol Errors

In this Release of the specification, there are no additional protocol errors applicable for the Nbsf_Management Service API.

- 5.7.3	Application Errors

The application errors defined for the Nbsf_Management Service API are listed in table 5.7.3-1. The PCF shall include in the HTTP status code a "ProblemDetails" data structure with the "cause" attribute indicating the application error as listed in table 5.7.3-1.

Table 5.7.3-1: Application errors

| Application Error | HTTP status code | Description |
| --- | --- | --- |
| MULTIPLE_BINDING_INFO_FOUND | 400 Bad Request | Indicates that the BSF found more than one binding resource so it cannot provide the selected PCF to the consumer. (NOTE) |
| NOTE: This application error is included in the responses to the GET request. |  |  |

## 5.8	Feature negotiation

The optional features in table 5.8-1 are defined for the Nbsf_Management Service API. They shall be negotiated using the extensibility mechanism defined in subclause 6.6 of 3GPP TS 29.500 [6].

Table 5.8-1: Supported Features

| Feature number | Feature Name | Description |
| --- | --- | --- |
| 1 | MultiUeAddr | This feature indicates the support of multiple UE addresses (IPv6 prefixes or MAC addresses) in the same binding information. |
| 2 | BindingUpdate | The consumer can use this feature for updating the session binding information. |

## 5.9	Security

As indicated in 3GPP TS 33.501 [15] and 3GPP TS 29.500 [6], the access to the Nbsf_Management API may be authorized by means of the OAuth2 protocol (see IETF RFC 6749 [16]), based on local configuration, using the "Client Credentials" authorization grant, where the NRF (see 3GPP TS 29.510 [12]) plays the role of the authorization server.

If OAuth2 is used, a n NF Service Consumer, prior to consuming services offered by the Nbsf_Management API, shall obtain a "token" from the authorization server, by invoking the Access Token Request service, as described in 3GPP TS 29.510 [12], subclause 5.4.2.2.

NOTE:	When multiple NRFs are deployed in a network, the NRF used as authorization server is the same NRF that the NF Service Consumer used for discovering the Nbsf_Management service.

The Nbsf_Management API defines a single scope "nbsf-management" for the entire service, and it does not define any additional scopes at resource or operation level.

Annex A (normative):
OpenAPI specification

# A.1	General

The present Annex contains an OpenAPI [11] specification of HTTP messages and content bodies used by the Nbsf_Management API.

This Annex shall take precedence when being discrepant to other parts of the specification with respect to the encoding of information elements and methods within the API.

NOTE 1:	The semantics and procedures, as well as conditions, e.g. for the applicability and allowed combinations of attributes or values, not expressed in the OpenAPI definitions but defined in other parts of the specification also apply.

Informative copies of the OpenAPI specification file contained in this 3GPP Technical Specification are available on the public 3GPP file server in the following locations (see clause 5B of the 3GPP TR 21.900 [18] for further information):

-	https://www.3gpp.org/ftp/Specs/archive/OpenAPI/<Release>/, and

-	https://www.3gpp.org/ftp/Specs/<Plenary>/<Release>/OpenAPI/.

NOTE 2:	To fetch the OpenAPI specification file after CT#83 plenary meeting for Release 15 in the above links <Plenary> must be replaced with the date the CT Plenary occurs, in the form of year-month (yyyy-mm), e.g. for CT#83 meeting <Plenary> must be replaced with value "2019-03" and <Release> must be replaced with value "Rel-15".

# A.2	Nbsf_Management API

openapi: 3.0.0

info:

version: 1.1.0.alpha-2

title: Nbsf_Management

description: |

Binding Support Management Service API.

© 2019, 3GPP Organizational Partners (ARIB, ATIS, CCSA, ETSI, TSDSI, TTA, TTC).

All rights reserved.

externalDocs:

description: 3GPP TS 29.521 V16.1.0; 5G System; Binding Support Management Service.

url: 'http://www.3gpp.org/ftp/Specs/archive/29_series/29.521/'

servers:

- url: '{apiRoot}/nbsf-management/v1'

variables:

apiRoot:

default: https://example.com

description: apiRoot as defined in subclause 4.4 of 3GPP TS 29.501.

security:

- {}

- oAuth2ClientCredentials:

- nbsf-management

paths:

/pcfBindings:

post:

requestBody:

required: true

content:

application/json:

schema:

$ref: '#/components/schemas/PcfBinding'

responses:

'201':

description: The creation of an individual PCF session binding.

content:

application/json:

schema:

$ref: '#/components/schemas/PcfBinding'

headers:

Location:

description: 'Contains the URI of the newly created resource, according to the structure: {apiRoot}/nbsf-management/v1/pcfBindings/{bindingId}'

required: true

schema:

type: string

'400':

$ref: 'TS29571_CommonData.yaml#/components/responses/400'

'401':

$ref: 'TS29571_CommonData.yaml#/components/responses/401'

'403':

$ref: 'TS29571_CommonData.yaml#/components/responses/403'

'404':

$ref: 'TS29571_CommonData.yaml#/components/responses/404'

'411':

$ref: 'TS29571_CommonData.yaml#/components/responses/411'

'413':

$ref: 'TS29571_CommonData.yaml#/components/responses/413'

'415':

$ref: 'TS29571_CommonData.yaml#/components/responses/415'

'429':

$ref: 'TS29571_CommonData.yaml#/components/responses/429'

'500':

$ref: 'TS29571_CommonData.yaml#/components/responses/500'

'503':

$ref: 'TS29571_CommonData.yaml#/components/responses/503'

default:

$ref: 'TS29571_CommonData.yaml#/components/responses/default'

get:

parameters:

- name: ipv4Addr

in: query

description: The IPv4 Address of the served UE.

required: false

schema:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Ipv4Addr'

- name: ipv6Prefix

in: query

description: The IPv6 Address of the served UE. The NF service consumer shall append '/128' to the IPv6 address in the attribute value. E.g. '2001:db8:85a3::8a2e:370:7334/128'.

required: false

schema:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Ipv6Prefix'

- name: macAddr48

in: query

description: The MAC Address of the served UE.

required: false

schema:

$ref: 'TS29571_CommonData.yaml#/components/schemas/MacAddr48'

- name: dnn

in: query

description: DNN.

required: false

schema:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Dnn'

- name: supi

in: query

description: Subscription Permanent Identifier.

required: false

schema:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Supi'

- name: gpsi

in: query

description: Generic Public Subscription Identifier

required: false

schema:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Gpsi'

- name: snssai

in: query

description: The identification of slice.

required: false

content:

application/json:

schema:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Snssai'

- name: ipDomain

in: query

description: The IPv4 address domain identifier.

required: false

schema:

type: string

- name: supp-feat

in: query

description: To filter irrelevant responses related to unsupported features

schema:

$ref: 'TS29571_CommonData.yaml#/components/schemas/SupportedFeatures'

responses:

'200':

description: The individual PCF session binding session binding information resource matching the query parameter(s) is returned.

content:

application/json:

schema:

$ref: '#/components/schemas/PcfBinding'

'204':

description: There is no PCF session binding information matching the query parameter(s).

'400':

$ref: 'TS29571_CommonData.yaml#/components/responses/400'

'401':

$ref: 'TS29571_CommonData.yaml#/components/responses/401'

'403':

$ref: 'TS29571_CommonData.yaml#/components/responses/403'

'404':

$ref: 'TS29571_CommonData.yaml#/components/responses/404'

'406':

$ref: 'TS29571_CommonData.yaml#/components/responses/406'

'414':

$ref: 'TS29571_CommonData.yaml#/components/responses/414'

'429':

$ref: 'TS29571_CommonData.yaml#/components/responses/429'

'500':

$ref: 'TS29571_CommonData.yaml#/components/responses/500'

'503':

$ref: 'TS29571_CommonData.yaml#/components/responses/503'

default:

$ref: 'TS29571_CommonData.yaml#/components/responses/default'

/pcfBindings/{bindingId}:

delete:

parameters:

- name: bindingId

in: path

description: Represents the individual PCF Session Binding.

required: true

schema:

type: string

responses:

'204':

description: No Content. The Individual PCF session binding information resource is deleted.

'400':

$ref: 'TS29571_CommonData.yaml#/components/responses/400'

'401':

$ref: 'TS29571_CommonData.yaml#/components/responses/401'

'403':

$ref: 'TS29571_CommonData.yaml#/components/responses/403'

'404':

$ref: 'TS29571_CommonData.yaml#/components/responses/404'

'429':

$ref: 'TS29571_CommonData.yaml#/components/responses/429'

'500':

$ref: 'TS29571_CommonData.yaml#/components/responses/500'

'503':

$ref: 'TS29571_CommonData.yaml#/components/responses/503'

default:

$ref: 'TS29571_CommonData.yaml#/components/responses/default'

patch:

parameters:

- name: bindingId

in: path

description: Represents the individual PCF Session Binding.

required: true

schema:

type: string

requestBody:

description: Parameters to update the existing session binding

required: true

content:

application/merge-patch+json:

schema:

$ref: '#/components/schemas/PcfBindingPatch'

responses:

'200':

description: OK (Successful update of the session binding)

content:

application/json:

schema:

$ref: '#/components/schemas/PcfBinding'

'400':

$ref: 'TS29571_CommonData.yaml#/components/responses/400'

'401':

$ref: 'TS29571_CommonData.yaml#/components/responses/401'

'403':

$ref: 'TS29571_CommonData.yaml#/components/responses/403'

'404':

$ref: 'TS29571_CommonData.yaml#/components/responses/404'

'411':

$ref: 'TS29571_CommonData.yaml#/components/responses/411'

'413':

$ref: 'TS29571_CommonData.yaml#/components/responses/413'

'415':

$ref: 'TS29571_CommonData.yaml#/components/responses/415'

'429':

$ref: 'TS29571_CommonData.yaml#/components/responses/429'

'500':

$ref: 'TS29571_CommonData.yaml#/components/responses/500'

'503':

$ref: 'TS29571_CommonData.yaml#/components/responses/503'

default:

$ref: 'TS29571_CommonData.yaml#/components/responses/default'

components:

securitySchemes:

oAuth2ClientCredentials:

type: oauth2

flows:

clientCredentials:

tokenUrl: '{nrfApiRoot}/oauth2/token'

scopes:

nbsf-management: Access to the Nbsf_Management API

schemas:

PcfBinding:

type: object

properties:

supi:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Supi'

gpsi:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Gpsi'

ipv4Addr:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Ipv4Addr'

ipv6Prefix:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Ipv6Prefix'

addIpv6Prefixes:

type: array

items:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Ipv6Prefix'

minItems: 1

description: The additional IPv6 Address Prefixes of the served UE.

ipDomain:

type: string

macAddr48:

$ref: 'TS29571_CommonData.yaml#/components/schemas/MacAddr48'

addMacAddrs:

type: array

items:

$ref: 'TS29571_CommonData.yaml#/components/schemas/MacAddr48'

minItems: 1

description: The additional MAC Addresses of the served UE.

dnn:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Dnn'

pcfFqdn:

$ref: 'TS29510_Nnrf_NFManagement.yaml#/components/schemas/Fqdn'

pcfIpEndPoints:

type: array

items:

$ref: 'TS29510_Nnrf_NFManagement.yaml#/components/schemas/IpEndPoint'

minItems: 1

description: IP end points of the PCF hosting the Npcf_PolicyAuthorization service.

pcfDiamHost:

$ref: 'TS29571_CommonData.yaml#/components/schemas/DiameterIdentity'

pcfDiamRealm:

$ref: 'TS29571_CommonData.yaml#/components/schemas/DiameterIdentity'

snssai:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Snssai'

suppFeat:

$ref: 'TS29571_CommonData.yaml#/components/schemas/SupportedFeatures'

pcfId:

$ref: 'TS29571_CommonData.yaml#/components/schemas/NfInstanceId'

recoveryTime:

$ref: 'TS29571_CommonData.yaml#/components/schemas/DateTime'

required:

- dnn

- snssai

oneOf:

- anyOf:

- required: [ipv4Addr]

- required: [ipv6Prefix]

- required: [addIpv6Prefixes]

- anyOf:

- required: [macAddr48]

- required: [addMacAddrs]

anyOf:

- anyOf:

- required: [pcfFqdn]

- required: [pcfIpEndPoints]

- required: [pcfDiamHost, pcfDiamRealm]

PcfBindingPatch:

type: object

properties:

ipv4Addr:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Ipv4AddrRm'

ipDomain:

type: string

nullable: true

ipv6Prefix:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Ipv6PrefixRm'

addIpv6Prefixes:

type: array

items:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Ipv6Prefix'

minItems: 1

description: The additional IPv6 Address Prefixes of the served UE.

nullable: true

macAddr48:

$ref: 'TS29571_CommonData.yaml#/components/schemas/MacAddr48Rm'

addMacAddrs:

type: array

items:

$ref: 'TS29571_CommonData.yaml#/components/schemas/MacAddr48'

minItems: 1

description: The additional MAC Addresses of the served UE.

nullable: true

- Annex B (informative):
Deployment option to support BSF and DRA coexistence due to network migration

As described in Annex B of 3GPP TS 23.503 [4], the Diameter Routing Agent (DRA) and the BSF can coexist in an operator's network during the network migration to 5GC. The DRA is described in 3GPP TS 29.213 [14] and can be a service consumer of the Nbsf_Management_Discovery service.

During the Rx session establishment, the DRA can discover the selected PCF for the related subscriber by using the Nbsf_Management_Discovery service operation to obtain the related PCF address if it has no stored binding information derived from an ongoing Gx session for that subscriber.

NOTE 1:	For a UE in the EPC there is a Gx session and the DRA stores the binding information. For a UE in the 5GC the Npcf_SmPolicyControl service is used and the BSF stores the binding information.

NOTE 2:	If the DRA has no stored binding information derived from an ongoing Gx session for a subscriber, the DRA needs to request new binding information for each Rx session establishment because the information in the BSF could have changed compared to any previous binding information the DRA requested.

Annex C (informative):
Change history

| Change history |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Date | TSG # | TSG Doc. | CR | Rev | Cat | Subject/Comment | New |
| 2018-01 |  |  |  |  |  | TS skeleton of Binding Support Management Service specification | 0.0.0 |
| 2018-01 |  |  |  |  |  | Inclusion of documents agreed in CT3#94 C3-180301, C3-180191, C3-180192 and C3-180193. | 0.1.0 |
| 2018-03 |  |  |  |  |  | Inclusion of documents agreed in CT3#95 C3-181350 and C3-181352. | 0.2.0 |
| 2018-04 |  |  |  |  |  | Inclusion of documents agreed in CT3#96 C3-182424 and C3-182510. | 0.3.0 |
| 2018-05 |  |  |  |  |  | Inclusion of documents agreed in CT3#97 C3-183287, C3-183500, C3-183881, C3-183502 and C3-183733. | 0.4.0 |
| 2018-06 | CT#80 | CP-181031 |  |  |  | TS sent to plenary for approval. | 1.0.0 |
| 2018-06 | CT#80 | CP-181031 |  |  |  | TS approved by plenary | 15.0.0 |
| 2018-09 | CT#81 | CP-182015 | 0001 | 2 | F | PCF id correction for BSF | 15.1.0 |
| 2018-09 | CT#81 | CP-182015 | 0002 |  | F | Reference update: OpenAPI specification | 15.1.0 |
| 2018-09 | CT#81 | CP-182015 | 0004 | 2 | F | Clarification on mandatory HTTP error status codes | 15.1.0 |
| 2018-09 | CT#81 | CP-182015 | 0005 | 6 | B | OpenAPI for TS 29.521 | 15.1.0 |
| 2018-09 | CT#81 | CP-182015 | 0006 | 1 | F | Description of Structured data types | 15.1.0 |
| 2018-09 | CT#81 | CP-182015 | 0007 | 1 | B | Support of IPv4 overlapping | 15.1.0 |
| 2018-09 | CT#81 | CP-182015 | 0008 |  | F | Correction of the service name | 15.1.0 |
| 2018-09 | CT#81 | CP-182015 | 0009 | 1 | F | Resource structure presentation | 15.1.0 |
| 2018-12 | CT#82 | CP-183205 | 0011 |  | F | Default value for apiRoot Default value for apiRoot | 15.2.0 |
| 2018-12 | CT#82 | CP-183205 | 0012 |  | F | Correction to DELETE Method for Nbsf_Management Service API | 15.2.0 |
| 2018-12 | CT#82 | CP-183205 | 0013 |  | F | Correction to Typos in URI Paths | 15.2.0 |
| 2018-12 | CT#82 | CP-183205 | 0015 |  | F | API version | 15.2.0 |
| 2018-12 | CT#82 | CP-183205 | 0016 |  | F | ExternalDocs OpenAPI field | 15.2.0 |
| 2018-12 | CT#82 | CP-183205 | 0017 |  | F | Location header field in OpenAPI | 15.2.0 |
| 2018-12 | CT#82 | CP-183205 | 0018 | 1 | F | Security | 15.2.0 |
| 2018-12 | CT#82 | CP-183205 | 0019 | 1 | F | supported content types | 15.2.0 |
| 2018-12 | CT#82 | CP-183205 | 0020 | 2 | F | HTTP Error responses | 15.2.0 |
| 2018-12 | CT#82 | CP-183205 | 0021 | 2 | F | DRA as service consumer | 15.2.0 |
| 2018-12 | CT#82 | CP-183205 | 0023 |  | F | Change presence in BSF binding | 15.2.0 |
| 2018-12 | CT#82 | CP-183205 | 0024 | 1 | F | Presence conditions in yaml file | 15.2.0 |
| 2018-12 | CT#82 | CP-183205 | 0025 | 1 | F | Missing 201 response body for POST to /pcfBindings | 15.2.0 |
| 2019-03 | CT#83 | CP-190113 | 0028 | 2 | F | Handling of unsupported query parameter combinations | 15.3.0 |
| 2019-03 | CT#83 | CP-190113 | 0029 | 1 | F | Correction of description of the Nbsf_Management_Register Service and Nbsf_Management_Discovery service operations | 15.3.0 |
| 2019-03 | CT#83 | CP-190113 | 0030 |  | F | BSF resource cleanup | 15.3.0 |
| 2019-03 | CT#83 | CP-190113 | 0031 | 1 | F | Formatting of structured data types in query parameters | 15.3.0 |
| 2019-03 | CT#83 | CP-190113 | 0032 | 1 | F | Correction on the handling of UE addresses | 15.3.0 |
| 2019-03 | CT#83 | CP-190110 | 0033 | 2 | F | Miscellaneous BSF correction | 15.3.0 |
| 2019-03 | CT#83 | CP-190140 | 0034 | 1 | F | OpenAPI Version number update | 15.3.0 |
| 2019-06 | CT#84 | CP-191079 | 0036 |  | F | Remove NSI ID | 15.4.0 |
| 2019-06 | CT#84 | CP-191106 | 0037 | 5 | B | Support multiple UE addresses in one binding | 16.0.0 |
| 2019-06 | CT#84 | CP-191106 | 0038 | 5 | B | Binding update support | 16.0.0 |
| 2019-06 | CT#84 | CP-191079 | 0039 | 1 | F | Precedence of OpenAPI file | 15.4.0 |
| 2019-06 | CT#84 | CP-191079 | 0040 | 1 | F | Copyright Note in YAML files | 15.4.0 |
| 2019-06 | CT#84 | CP-191089 | 0041 | 1 | F | Correction of Location header in Nbsf_Management OpenAPI | 16.0.0 |
| 2019-06 | CT#84 | CP-191101 | 0043 | 2 | F | OpenAPI version number update | 16.0.0 |
| 2019-09 | CT#85 | CP-192199 | 0045 | 2 | F | Session binding for IPv6 addresses | 16.1.0 |
| 2019-09 | CT#85 | CP-192156 | 0046 |  | F | Support multiple UE addresses in BSF | 16.1.0 |
| 2019-09 | CT#85 | CP-192152 | 0047 | 1 | B | IP address handling in wireline access | 16.1.0 |
| 2019-09 | CT#85 | CP-192234 | 0050 | 2 | F | OpenAPI version update TS 29.521 Rel-16 | 16.1.0 |