3GPP TS 29.519 V15.7.0 (2020-06)

Technical Specification

3rd Generation Partnership Project;

Technical Specification Group Core Network and Terminals;

5G System; Usage of the Unified Data Repository service for Policy Data, Application Data and Structured Data for Exposure;
Stage 3

(Release 15)

The present document has been developed within the 3rd Generation Partnership Project (3GPP TM) and may be further elaborated for the purposes of 3GPP.
The present document has not been subject to any approval process by the 3GPP Organizational Partners and shall not be implemented.
This Specification is provided for future development work within 3GPP only. The Organizational Partners accept no liability for any use of this Specification.
Specifications and Reports for implementation of the 3GPP TM system should be obtained via the 3GPP Organizational Partners' Publications Offices.

Keywords

3GPP

Postal address

3GPP support office address

650 Route des Lucioles - Sophia Antipolis

Valbonne - FRANCE

Tel.: +33 4 92 94 42 00 Fax: +33 4 93 65 47 16

Internet

http://www.3gpp.org

Copyright Notification

No part may be reproduced except as authorized by written permission.
The copyright and the foregoing restriction extend to reproduction in all media.

© 2020, 3GPP Organizational Partners (ARIB, ATIS, CCSA, ETSI, TSDSI, TTA, TTC).

All rights reserved.

UMTS™ is a Trade Mark of ETSI registered for the benefit of its members

3GPP™ is a Trade Mark of ETSI registered for the benefit of its Members and of the 3GPP Organizational Partners
LTE™ is a Trade Mark of ETSI registered for the benefit of its Members and of the 3GPP Organizational Partners

GSM® and the GSM logo are registered and owned by the GSM Association

# Contents

Contents	3

Foreword	7

1	Scope	8

2	References	8

3	Definitions and abbreviations	9

3.1	Definitions	9

3.2	Abbreviations	9

4	Overview	9

5	Usage of Nudr_DataRepository Service API for Policy Data	9

5.1	Introduction	9

5.2	Resources	9

5.2.1	Overview	9

5.2.2	Resource Structure	10

5.2.3	Resource: AccessAndMobilityPolicyData	11

5.2.3.1	Description	11

5.2.3.2	Resource definition	12

5.2.3.3	Resource Standard Methods	12

5.2.3.3.1	GET	12

5.2.4	Resource: UEPolicySet	12

5.2.4.1	Description	12

5.2.4.2	Resource definition	12

5.2.4.3	Resource Standard Methods	13

5.2.4.3.1	PUT	13

5.2.4.3.2	PATCH	13

5.2.4.3.3	GET	14

5.2.5	Resource: SessionManagementPolicyData	14

5.2.5.1	Description	14

5.2.5.2	Resource definition	14

5.2.5.3	Resource Standard Methods	15

5.2.5.3.1	GET	15

5.2.5.3.2	PATCH	15

5.2.6	Resource: UsageMonitoringInformation	16

5.2.6.1	Description	16

5.2.6.2	Resource definition	16

5.2.6.3	Resource Standard Methods	16

5.2.6.3.1	PUT	16

5.2.6.3.2	DELETE	16

5.2.6.3.3	GET	17

5.2.7	Resource: SponsorConnectivityData	17

5.2.7.1	Description	17

5.2.7.2	Resource definition	18

5.2.7.3	Resource Standard Methods	18

5.2.7.3.1	GET	18

5.2.8	Resource: BdtData	18

5.2.8.1	Description	18

5.2.8.2	Resource definition	18

5.2.8.3	Resource Standard Methods	19

5.2.8.3.1	GET	19

5.2.9	Resource: IndividualBdtData	19

5.2.9.1	Description	19

5.2.9.2	Resource definition	19

5.2.9.3	Resource Standard Methods	20

5.2.9.3.1	GET	20

5.2.9.3.2	PUT	20

5.2.9.3.3	DELETE	21

5.2.10	Resource: PolicyDataSubscriptions	21

5.2.10.1	Description	21

5.2.10.2	Resource definition	21

5.2.10.3	Resource Standard Methods	21

5.2.10.3.1	POST	21

5.2.11	Resource: IndividualPolicyDataSubscription	22

5.2.11.1	Description	22

5.2.11.2	Resource definition	22

5.2.11.3	Resource Standard Methods	22

5.2.11.3.1	PUT	22

5.2.11.3.2	DELETE	23

5.2.12	Resource: OperatorSpecificData	23

5.2.12.1	Description	23

5.2.12.2	Resource Definition	23

5.2.12.3	Resource Standard Methods	24

5.2.12.3.1	GET	24

5.2.12.3.2	PATCH	24

5.2.12.3.3	PUT	25

5.2.13	Resource: PlmnUePolicySet	25

5.2.13.1	Description	25

5.2.13.2	Resource definition	26

5.2.13.3	Resource Standard Methods	26

5.2.13.3.1	GET	26

5.3	Notifications	26

5.3.1	General	26

5.3.2	Policy Data Change Notification	26

5.4	Data Model	27

5.4.1	General	27

5.4.2	Structured data types	29

5.4.2.1	Introduction	29

5.4.2.2	Type AmPolicyData	29

5.4.2.3	Type UePolicySection	30

5.4.2.4	Type UePolicySet	30

5.4.2.5	Type SmPolicyData	30

5.4.2.6	Type UsageMonDataLimit	31

5.4.2.7	Type UsageMonData	31

5.4.2.8	Type SponsorConnectivityData	32

5.4.2.9	Type BdtData	32

5.4.2.10	Type PolicyDataSubscription	32

5.4.2.11	Type PolicyDataChangeNotification	33

5.4.2.12	Type TimePeriod	34

5.4.2.13	Type UsageMonDataScope	34

5.4.2.14	Type SmPolicySnssaiData	34

5.4.2.15	Type SmPolicyDnnData	35

5.4.2.16	Type LimitIdToMonitoringKey	36

5.4.2.17	Type UePolicySetPatch	36

5.4.2.18	Type PlmnRouteSelectionDescriptor	36

5.4.2.19	Type SnssaiRouteSelectionDescriptor	36

5.4.2.20	Type DnnRouteSelectionDescriptor	36

5.4.3	Simple data types and enumerations	37

5.4.3.1	Introduction	37

5.4.3.2	Simple data types	37

5.4.3.3	Enumeration: UsageMonLevel	37

5.4.3.4	Enumeration: Periodicity	37

5.5	Error handling	37

5.5.1	General	37

5.5.2	Protocol Errors	38

5.5.3	Application Errors	38

5.6	Feature negotiation	38

6	Usage of Nudr_DataRepository Service API for Application Data	38

6.1	Introduction	38

6.2	Resources	38

6.2.1	Overview	38

6.2.2	Resource Structure	39

6.2.3	Resource: PFD Data	40

6.2.3.1	Description	40

6.2.3.2	Resource definition	40

6.2.3.3	Resource Standard Methods	41

6.2.3.3.1	GET	41

6.2.4	Resource: Individual PFD Data	41

6.2.4.1	Description	41

6.2.4.2	Resource definition	41

6.2.4.3	Resource Standard Methods	41

6.2.4.3.1	GET	41

6.2.4.3.2	DELETE	42

6.2.4.3.3	PUT	42

6.2.5	Resource: Influence Data	43

6.2.5.1	Description	43

6.2.5.2	Resource definition	43

6.2.5.3	Resource Standard Methods	43

6.2.5.3.1	GET	43

6.2.6	Resource: Individual Influence Data	44

6.2.6.1	Description	44

6.2.6.2	Resource definition	44

6.2.6.3	Resource Standard Methods	44

6.2.6.3.1	PUT	44

6.2.6.3.2	PATCH	45

6.2.6.3.3	DELETE	45

6.2.7	Resource: Influence Data Subscription	46

6.2.7.1	Description	46

6.2.7.2	Resource definition	46

6.2.7.3	Resource Standard Methods	46

6.2.7.3.1	POST	46

6.2.7.3.2	GET	46

6.2.8	Resource: Individual Influence Data Subscription	47

6.2.8.1	Description	47

6.2.8.2	Resource definition	47

6.2.8.3	Resource Standard Methods	47

6.2.8.3.1	GET	47

6.2.8.3.2	PUT	48

6.2.8.3.3	DELETE	48

6.3	Notifications	49

6.3.1	General	49

6.3.2	Void	49

6.3.3	Influence Data Update Notification	49

6.3.3.1	Description	49

6.3.3.2	Operation Definition	49

6.4	Data Model	50

6.4.1	General	50

6.4.2	Structured data types	51

6.4.2.1	Introduction	51

6.4.2.2	Type TrafficInfluData	51

6.4.2.5	Void	53

6.5	Error handling	53

6.5.1	General	53

6.5.2	Protocol Errors	53

6.5.3	Application Errors	54

6.6	Feature negotiation	54

7	Usage of Nudr_DataRepository Service API for Structured Data for Exposure	54

7.1	Introduction	54

7.2	Resources	54

7.2.1	Overview	54

7.2.2	Resource Structure	55

7.2.3	Resource: AccessAndMobilityData	56

7.2.3.1	Description	56

7.2.3.2	Resource definition	56

7.2.3.3	Resource Standard Methods	56

7.2.3.3.1	PUT	56

7.2.3.3.2	GET	56

7.2.3.3.3	DELETE	57

7.2.4	Resource: PduSessionManagementData	57

7.2.4.1	Description	57

7.2.4.2	Resource definition	57

7.2.4.3	Resource Standard Methods	58

7.2.4.3.1	PUT	58

7.2.4.3.2	GET	58

7.2.4.3.3	DELETE	59

7.2.5	Resource: ExposureDataSubscriptions	59

7.2.5.1	Description	59

7.2.5.2	Resource Definition	59

7.2.5.3	Resource Standard Methods	60

7.2.5.3.1	POST	60

7.2.6	Resource: IndividualExposureDataSubscription	60

7.2.6.1	Description	60

7.2.6.2	Resource Definition	60

7.2.6.3	Resource Standard Methods	60

7.2.6.3.1	PUT	60

7.2.6.3.2	DELETE	61

7.3	Data Model	61

7.3.1	General	61

7.3.2	Structured data types	62

7.3.2.1	Introduction	62

7.3.2.2	Type AccessAndMobilityData	63

7.3.2.3	Type PduSessionManagementData	64

7.3.2.4	Type ExposureDataSubscription	64

7.3.2.5	Type ExposureDataChangeNotification	65

7.3.3	Simple data types and enumerations	65

7.3.3.1	Introduction	65

7.3.3.2	Simple data types	65

7.3.3.3	Enumeration: PduSessionStatus	65

7.4	Error handling	65

7.4.1	General	65

7.4.2	Protocol Errors	66

7.4.3	Application Errors	66

7.5	Feature negotiation	66

7.6	Notifications	66

7.6.1	General	66

7.6.2	Exposure Data Change Notification	66

Annex A (normative): OpenAPI specification	68

A.1	General	68

A.2	Nudr_DataRepository API for Policy Data	68

A.3	Nudr_DataRepository API for Application Data	85

A.4	Nudr_DataRepository API for Exposure Data	96

Annex B (informative): Change history	105

# Foreword

This Technical Specification has been produced by the 3rd Generation Partnership Project (3GPP).

The contents of the present document are subject to continuing work within the TSG and may change following formal TSG approval. Should the TSG modify the contents of the present document, it will be re-released by the TSG with an identifying change of release date and an increase in version number as follows:

Version x.y.z

where:

x	the first digit:

1	presented to TSG for information;

2	presented to TSG for approval;

3	or greater indicates TSG approved document under change control.

y	the second digit is incremented for all changes of substance, i.e. technical enhancements, corrections, updates, etc.

z	the third digit is incremented when editorial only changes have been incorporated in the document.

# 1	Scope

The present specification provides the stage 3 definition for the usage of the Unified Data Repository service (Nudr_DataRepository) of 5G System for Policy Data, Application Data and Structured Data for Exposure.

The Unified Data Repository (UDR) provides the Unified Data Repository service. This service provides a repository for policy data, application data and exposure data.

The stage 3 protocol definition for Nudr Service Based Interface which is common for subscription data, policy data, application data and exposure data is specified in 3GPP TS 29.504 [6].

# 2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[2]	Void.

[3]	OpenAPI: "OpenAPI 3.0.0 Specification", https://github.com/OAI/OpenAPI-Specification/blob/master/versions/3.0.0.md.

[4]	3GPP TS 29.500: "5G System; Technical Realization of Service Based Architecture; Stage 3".

[5]	3GPP TS 29.501: "5G System; Principles and Guidelines for Services Definition; Stage 3".

[6]	3GPP TS 29.504:"5G System; Unified Data Repository Services; Stage 3".

[7]	3GPP TS 29.571: "5G System; Common Data Types for Service Based Interfaces Stage 3".

[8]	3GPP TS 29.551: "5G System; Packet Flow Description Management Service; Stage 3".

[9]	3GPP TS 29.122: "T8 reference point for Northbound APIs".

[10]	3GPP TS 29.518: "5G System; Access and Mobility Management Services; Stage 3".

[11]	3GPP TS 24.501: "Non-Access-Stratum (NAS) protocol for 5G System (5GS); Stage 3".

[12]	3GPP TS 29.512: "5G System; Session Management Policy Control Service; Stage 3".

[13]	3GPP TS 29.554: "5G System; Background Data Transfer Policy Control Service; Stage 3".

[14]	Void.

[15]	3GPP TS 29.505: "5G System; Usage of the Unified Data Repository services for Subscription Data; Stage 3".

[16]	3GPP TS 29.514: "5G System; Policy Authorization Service; Stage 3".

[17]	IETF RFC 4122: "A Universally Unique IDentifier (UUID) URN Namespace".

[18]	3GPP TR 21.900: "Technical Specification Group working methods".

[19]	3GPP TS 29.522: "5G System; Network Exposure Function Northbound APIs; Stage 3".

# 3	Definitions and abbreviations

## 3.1	Definitions

For the purposes of the present document, the terms and definitions given in 3GPP TR 21.905 [1] and the following apply. A term defined in the present document takes precedence over the definition of the same term, if any, in 3GPP TR 21.905 [1].

## 3.2	Abbreviations

For the purposes of the present document, the abbreviations given in 3GPP TR 21.905 [1] and the following apply. An abbreviation defined in the present document takes precedence over the definition of the same abbreviation, if any, in 3GPP TR 21.905 [1].

AM	Access and Mobility Management

BDT	Background Data Transfer

DNAI	DN Access Identifier

DNN	Data Network Name

GPSI	Generic Public Subscription Identifier

JSON	JavaScript Object Notation

NEF	Network Exposure Function

NR	New Radio

NRF	Network Repository Function

PCF	Policy Control Function

PFD	Packet Flow Description

SM	Session Management

S-NSSAI	Single Network Slice Selection Assistance Information

SUPI	Subscription Permanent Identifier

UDR	Unified Data Repository

UPSI	UE Policy Section Identifier

# 4	Overview

The resources, data model and usage defined in the present document are used by the NFs (e.g. PCF, NEF) to access the policy data, application data or exposure data via the Unified Data Repository services as specified in 3GPP TS 29.504 [6].

# 5	Usage of Nudr_DataRepository Service API for Policy Data

## 5.1	Introduction

The following subclauses specify the usage of Nudr_DataRepository service for policy data. The principles specified in 3GPP TS 29.504 [6] are followed unless explicitly specified otherwise in the following subclauses.

## 5.2	Resources

### 5.2.1	Overview

This clause specifies the resources for the usage of the Unified Data Repository service for Policy Data.

### 5.2.2	Resource Structure

Figure 5.2.2-1: Resource URI structure of the Nudr_DataRepository API for policy data

Table 5.2.2-1 provides an overview of the resources and applicable HTTP methods.

Table 5.2.2-1: Resources and methods overview

| Resource name | Resource URI | HTTP method or custom operation | Description |
| --- | --- | --- | --- |
| AccessAndMobilityPolicyData | {apiRoot}/nudr-dr/{apiVersion}/ policy-data/ues/{ueId}/ am-data | GET | Retrieves the access and mobility policy data for a subscriber. |
| UEPolicySet | {apiRoot}/nudr-dr/{apiVersion}/ policy-data/ues/{ueId}/ ue-policy-set | GET | Retrieves the UE policy set data for a subscriber. |
|  |  | PUT | Create or modify the UE policy set data for a subscriber. |
|  |  | PATCH | Modify the UE policy set data for a subscriber. |
| SessionManagementPolicyData | {apiRoot}/nudr-dr/{apiVersion}/ policy-data/ues/{ueId}/ sm-data | GET | Retrieves the session management policy data for a subscriber. |
| UsageMonitoringInformation | {apiRoot}/nudr-dr/{apiVersion}/ policy-data/ues/{ueId}/ sm-data/{usageMonId} | GET | Retrieve a usage monitoring resource. |
|  |  | PUT | Create a usage monitoring resource. |
|  |  | DELETE | Delete a usage monitoring resource. |
| OperatorSpecificData | {apiRoot}/nudr-dr/{apiVersion}/ policy-data/ues/{ueId}/ operator-specific-data | GET | Retrieve the operator specific policy data of an UE |
|  |  | PATCH | Modify the operator specific policy data of an UE. |
|  |  | PUT | Modify the operator specific policy data of an UE. |
| SponsorConnectivityData | {apiRoot}/nudr-dr/{apiVersion}/ policy-data/sponsor-connectivity-data/{sponsorId} | GET | Retrieves the sponsored connectivity information for a given sponsorId. |
| BdtData | {apiRoot}/nudr-dr/{apiVersion}/ policy-data/bdt-data | GET | Retrieves the BDT data collection. |
| IndividualBdtData | {apiRoot}/nudr-dr/{apiVersion}/ policy-data/bdt-data/ {bdtReferenceId} | GET | Retrieves the BDT data information associated with a BDT reference Id. |
|  |  | PUT | Creates an BDT data resource associated with an BDT reference Id. |
|  |  | DELETE | Deletes an BDT data resource associated with an BDT reference Id. |
| PolicyDataSubscriptions | {apiRoot}/nudr-dr/{apiVersion}/ policy-data/subs-to-notify | POST | Create a subscription to receive notification of policy data changes. |
| IndividualPolicyDataSubscription | {apiRoot}/nudr-dr/{apiVersion}/ policy-data/subs-to-notify/ {subsId} | PUT | Modify a subscription to receive notification of policy data changes. |
|  |  | DELETE | Delete the subscription identified by {subsId}. |
| PlmnUePolicySet | {apiRoot}/nudr-dr/{apiVersion}/ policy-data/plmns/{plmnId}/ ue-policy-set | GET | Retrieve the UE policy set data for a H-PLMN. |

### 5.2.3	Resource: AccessAndMobilityPolicyData

#### 5.2.3.1	Description

The resource represents all UE related mobility and access policies attributes in the UDR for a given "ueId".

#### 5.2.3.2	Resource definition

Resource URI: {apiRoot}/nudr-dr/{apiVersion}/policy-data/ues/{ueId}/am-data

This resource shall support the resource URI variables defined in table 5.2.3.2-1.

Table 5.2.3.2-1: Resource URI variables for this resource

| Name | Definition |
| --- | --- |
| apiRoot | See 3GPP TS 29.504 [6] subclause 6.1.1 |
| apiVersion | See 3GPP TS 29.504 [6] subclause 6.1.1. |
| ueId | Represents the Subscription Identifier SUPI or GPSI, see the data type VarUeId as defined in 3GPP TS 29.571 [7]. |

#### 5.2.3.3	Resource Standard Methods

##### 5.2.3.3.1	GET

This method shall support the URI query parameters specified in table 5.2.3.3.1-1.

Table 5.2.3.3.1-1: URI query parameters supported by the GET method on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| n/a |  |  |  |  |

This method shall support the request data structures specified in table 5.2.3.3.1-2 and the response data structures and response codes specified in table 5.2.3.3.1-3.

Table 5.2.3.3.1-2: Data structures supported by the GET Request Body on this resource

| Data type | P | Cardinality | Description |
| --- | --- | --- | --- |
| n/a |  |  |  |

Table 5.2.3.3.1-3: Data structures supported by the GET Response Body on this resource

| Data type | P | Cardinality | Response codes | Description |
| --- | --- | --- | --- | --- |
| AmPolicyData | M | 1 | 200 OK | Upon success, a response body containing access and mobility policies shall be returned. |
| NOTE: The mandatory HTTP error status codes for the GET method listed in table 5.2.7.1-1 of 3GPP TS 29.500 [4] also apply. |  |  |  |  |

### 5.2.4	Resource: UEPolicySet

#### 5.2.4.1	Description

The resource represents UE policy set for a given "ueId" stored in the UDR of the H-PLMN.

#### 5.2.4.2	Resource definition

Resource URI: {apiRoot}/nudr-dr/{apiVersion}/policy-data/ues/{ueId}/ue-policy-set

This resource shall support the resource URI variables defined in table 5.2.4.2-1.

Table 5.2.4.2-1: Resource URI variables for this resource

| Name | Definition |
| --- | --- |
| apiRoot | See 3GPP TS 29.504 [6] subclause 6.1.1 |
| apiVersion | See 3GPP TS 29.504 [6] subclause 6.1.1. |
| ueId | Represents the Subscription Identifier SUPI or GPSI, see the data type VarUeId as defined in 3GPP TS 29.571 [7]. |

#### 5.2.4.3	Resource Standard Methods

##### 5.2.4.3.1	PUT

This method shall support the URI query parameters specified in table 5.2.4.3.1-1.

Table 5.2.4.3.1-1: URI query parameters supported by the PUT method on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| n/a |  |  |  |  |

This method shall support the request data structures specified in table 5.2.4.3.1-2 and the response data structures and response codes specified in table 5.2.4.3.1-3.

Table 5.2.4.3.1-2: Data structures supported by the PUT Request Body on this resource

| Data type | P | Cardinality | Description |
| --- | --- | --- | --- |
| UePolicySet | M | 1 | Create or modify the UE policy set resource for a given subscriber. |

Table 5.2.4.3.1-3: Data structures supported by the PUT Response Body on this resource

| Data type | P | Cardinality | Response codes | Description |
| --- | --- | --- | --- | --- |
| UePolicySet | M | 1 | 201 Created | Successful case. The resource has been successfully created and a response body containing a representation of the created UEPolicySet resource shall be returned. |
| UePolicySet | M | 1 | 200 OK | Successful case. The resource has been successfully updated and a response body containing UE policies shall be returned. |
| n/a |  |  | 204 No Content | Successful case. The resource has been successfully updated and no additional content is to be sent in the response message. |
| NOTE: The mandatory HTTP error status codes for the PUT method listed in table 5.2.7.1-1 of 3GPP TS 29.500 [4] also apply. |  |  |  |  |

##### 5.2.4.3.2	PATCH

This method shall support the URI query parameters specified in table 5.2.4.3.2-1.

Table 5.2.4.3.2-1: URI query parameters supported by the PATCH method on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| n/a |  |  |  |  |

This method shall support the request data structures specified in table 5.2.4.3.2-2 and the response data structures and response codes specified in table 5.2.4.3.2-3.

Table 5.2.4.3.2-2: Data structures supported by the PATCH Request Body on this resource

| Data type | P | Cardinality | Description |
| --- | --- | --- | --- |
| UePolicySetPatch | M | 1 | Modify the UE policy set resource for a given subscriber. |

Table 5.2.4.3.2-3: Data structures supported by the PATCH Response Body on this resource

| Data type | P | Cardinality | Response codes | Description |
| --- | --- | --- | --- | --- |
| n/a |  |  | 204 No Content | Successful case. The resource has been successfully updated and no additional content is to be sent in the response message. |
| NOTE: The mandatory HTTP error status codes for the PATCH method listed in table 5.2.7.1-1 of 3GPP TS 29.500 [4] also apply. |  |  |  |  |

##### 5.2.4.3.3	GET

This method shall support the URI query parameters specified in table 5.2.4.3.3-1.

Table 5.2.4.3.3-1: URI query parameters supported by the GET method on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| n/a |  |  |  |  |

This method shall support the request data structures specified in table 5.2.4.3.3-2 and the response data structures and response codes specified in table 5.2.4.3.3-3.

Table 5.2.4.3.3-2: Data structures supported by the GET Request Body on this resource

| Data type | P | Cardinality | Description |
| --- | --- | --- | --- |
| n/a |  |  |  |

Table 5.2.4.3.3-3: Data structures supported by the GET Response Body on this resource

| Data type | P | Cardinality | Response codes | Description |
| --- | --- | --- | --- | --- |
| UePolicySet | M | 1 | 200 OK | Upon success, a response body containing the UE policies shall be returned. |
| NOTE: The mandatory HTTP error status codes for the GET method listed in table 5.2.7.1-1 of 3GPP TS 29.500 [4] also apply. |  |  |  |  |

### 5.2.5	Resource: SessionManagementPolicyData

#### 5.2.5.1	Description

The resource represents all PDU session related subscription attributes in the UDR for a given ueId.

#### 5.2.5.2	Resource definition

Resource URI: {apiRoot}/nudr-dr/{apiVersion}/policy-data/ues/{ueId}/sm-data

This resource shall support the resource URI variables defined in table 5.2.5.2-1.

Table 5.2.5.2-1: Resource URI variables for this resource

| Name | Definition |
| --- | --- |
| apiRoot | See 3GPP TS 29.504 [6] subclause 6.1.1. |
| apiVersion | See 3GPP TS 29.504 [6] subclause 6.1.1. |
| ueId | Represents the Subscription Identifier SUPI or GPSI, see the data type VarUeId as defined in 3GPP TS 29.571 [7]. |

#### 5.2.5.3	Resource Standard Methods

##### 5.2.5.3.1	GET

This method shall support the URI query parameters specified in table 5.2.5.3.1-1.

Table 5.2.5.3.1-1: URI query parameters supported by the GET method on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| snssai | Snssai | O | 0..1 | Identifies a Single Network Slice Selection Assistance Information. |
| dnn | Dnn | O | 0..1 | Identifies an Data Network Name. |
| fields | array(string) | C | 1..N | When the NF consumer only retrieves a subset of the resource, the "fields" query parameter shall be included. The "fields" query parameter contains the pointers of the attribute(s) to be retrieved. |

When an optional query parameter is not included, the UDR shall return session management policy data per DNN matching the value of the included query parameter(s) for all the possible values of the omitted query parameter. E.g. if "snssai" is not included, and "dnn" is included, UDR shall return the session management policy data per DNN for the DNN identified by "dnn" for all network slices where such DNN is available.

This method shall support the request data structures specified in table 5.2.5.3.1-2 and the response data structures and response codes specified in table 5.2.5.3.1-3.

Table 5.2.5.3.1-2: Data structures supported by the GET Request Body on this resource

| Data type | P | Cardinality | Description |
| --- | --- | --- | --- |
| n/a |  |  |  |

Table 5.2.5.3.1-3: Data structures supported by the GET Response Body on this resource

| Data type | P | Cardinality | Response codes | Description |
| --- | --- | --- | --- | --- |
| SmPolicyData | M | 1 | 200 OK | Upon success, a response body containing SmPolicyData shall be returned. |
| NOTE: The mandatory HTTP error status codes for the GET method listed in table 5.2.7.1-1 of 3GPP TS 29.500 [4] also apply. |  |  |  |  |

NOTE:	This release supports that only one PCF can access to the UDR to retrieve the SM Policy Data under one S-NSSAI and DNN association.

##### 5.2.5.3.2	PATCH

Void.

### 5.2.6	Resource: UsageMonitoringInformation

#### 5.2.6.1	Description

The resource represents an individual usage monitoring resource created in the UDR and associated with a ueId and a usageMonId.

#### 5.2.6.2	Resource definition

Resource URI: {apiRoot}/nudr-dr/{apiVersion}/policy-data/ues/{ueId}/sm-data/{usageMonId}

This resource shall support the resource URI variables defined in table 5.2.6.2-1.

Table 5.2.6.2-1: Resource URI variables for this resource

| Name | Definition |
| --- | --- |
| apiRoot | See 3GPP TS 29.504 [6] subclause 6.1.1. |
| apiVersion | See 3GPP TS 29.504 [6] subclause 6.1.1. |
| ueId | Represents the Subscription Identifier SUPI or GPSI, see the data type VarUeId as defined in 3GPP TS 29.571 [7]. |
| usageMonId | Unique identifier of the individual SM Policy usage monitoring resource. It contains the identity of the corresponding limit as defined in the "limitId" attribute of the UsageMonDataLimit data type (see subclause 5.4.2.6). |

#### 5.2.6.3	Resource Standard Methods

##### 5.2.6.3.1	PUT

This method shall support the URI query parameters specified in table 5.2.6.3.1-1.

Table 5.2.6.3.1-1: URI query parameters supported by the PUT method on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| n/a |  |  |  |  |

This method shall support the request data structures specified in table 5.2.6.3.1-2 and the response data structures and response codes specified in table 5.2.6.3.1-3.

Table 5.2.6.3.1-2: Data structures supported by the PUT Request Body on this resource

| Data type | P | Cardinality | Description |
| --- | --- | --- | --- |
| UsageMonData | M | 1 | Create the usage monitoring resource. |

Table 5.2.6.3.1-3: Data structures supported by the PUT Response Body on this resource

| Data type | P | Cardinality | Response codes | Description |
| --- | --- | --- | --- | --- |
| UsageMonData | M | 1 | 201 Created | Successful case. The resource has been successfully created and a response body is returned containing a representation of the resource. |
| NOTE: The mandatory HTTP error status codes for the PUT method listed in table 5.2.7.1-1 of 3GPP TS 29.500 [4] also apply. |  |  |  |  |

##### 5.2.6.3.2	DELETE

This method shall support the URI query parameters specified in table 5.2.6.3.2-1.

Table 5.2.6.3.2-1: URI query parameters supported by the DELETE method on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| n/a |  |  |  |  |

This method shall support the request data structures specified in table 5.2.6.3.2-2 and the response data structures and response codes specified in table 5.2.6.3.2-3.

Table 5.2.6.3.2-2: Data structures supported by the DELETE Request Body on this resource

| Data type | P | Cardinality | Description |
| --- | --- | --- | --- |
| n/a |  |  |  |

Table 5.2.6.3.2-3: Data structures supported by the DELETE Response Body on this resource

| Data type | P | Cardinality | Response codes | Description |
| --- | --- | --- | --- | --- |
| n/a |  |  | 204 No Content | Successful case. The resource has been successfully deleted. |
| NOTE: The mandatory HTTP error status codes for the DELETE method listed in table 5.2.7.1-1 of 3GPP TS 29.500 [4] also apply. |  |  |  |  |

##### 5.2.6.3.3	GET

This method shall support the URI query parameters specified in table 5.2.6.3.3-1.

Table 5.2.6.3.3-1: URI query parameters supported by the GET method on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| n/a |  |  |  |  |

This method shall support the request data structures specified in table 5.2.6.3.3-2 and the response data structures and response codes specified in table 5.2.6.3.3-3.

Table 5.2.6.3.3-2: Data structures supported by the GET Request Body on this resource

| Data type | P | Cardinality | Description |
| --- | --- | --- | --- |
| n/a |  |  |  |

Table 5.2.6.3.3-3: Data structures supported by the GET Response Body on this resource

| Data type | P | Cardinality | Response codes | Description |
| --- | --- | --- | --- | --- |
| UsageMonData | M | 1 | 200 Ok | Successful case. The usage monitoring data is returned. |
| n/a |  |  | 204 No Content | The resource was found but no usage monitoring data is available. |
| NOTE: The mandatory HTTP error status codes for the GET method listed in table 5.2.7.1-1 of 3GPP TS 29.500 [4] also apply. |  |  |  |  |

### 5.2.7	Resource: SponsorConnectivityData

#### 5.2.7.1	Description

The resource represents sponsored data connectivity related information stored in the UDR for a given sponsorId.

#### 5.2.7.2	Resource definition

Resource URI: {apiRoot}/nudr-dr/{apiVersion}/policy-data/sponsor-connectivity-data/{sponsorId}

This resource shall support the resource URI variables defined in table 5.2.7.2-1.

Table 5.2.7.2-1: Resource URI variables for this resource

| Name | Definition |
| --- | --- |
| apiRoot | See 3GPP TS 29.504 [6] subclause 6.1.1. |
| apiVersion | See 3GPP TS 29.504 [6] subclause 6.1.1. |
| sponsorId | String that represents the sponsor identity. |

#### 5.2.7.3	Resource Standard Methods

##### 5.2.7.3.1	GET

This method shall support the URI query parameters specified in table 5.2.7.3.1-1.

Table 5.2.7.3.1-1: URI query parameters supported by the GET method on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| n/a |  |  |  |  |

This method shall support the request data structures specified in table 5.2.7.3.1-2 and the response data structures and response codes specified in table 5.2.7.3.1-3.

Table 5.2.7.3.1-2: Data structures supported by the GET Request Body on this resource

| Data type | P | Cardinality | Description |
| --- | --- | --- | --- |
| n/a |  |  |  |

Table 5.2.7.3.1-3: Data structures supported by the GET Response Body on this resource

| Data type | P | Cardinality | Response codes | Description |
| --- | --- | --- | --- | --- |
| SponsorConnectivityData | M | 1 | 200 OK | Upon success, a response body containing Sponsor Connectivity Data shall be returned. |
| n/a |  |  | 204 No Content | The resource was found but no Sponsor Connectivity Data is available. |
| NOTE: The mandatory HTTP error status codes for the GET method listed in table 5.2.7.1-1 of 3GPP TS 29.500 [4] also apply. |  |  |  |  |

### 5.2.8	Resource: BdtData

#### 5.2.8.1	Description

The resource represents background data transfer data stored in the UDR.

#### 5.2.8.2	Resource definition

Resource URI: {apiRoot}/nudr-dr/{apiVersion}/policy-data/bdt-data

This resource shall support the resource URI variables defined in table 5.2.8.2-1.

Table 5.2.8.2-1: Resource URI variables for this resource

| Name | Definition |
| --- | --- |
| apiRoot | See 3GPP TS 29.504 [6] subclause 6.1.1. |
| apiVersion | See 3GPP TS 29.504 [6] subclause 6.1.1. |

#### 5.2.8.3	Resource Standard Methods

##### 5.2.8.3.1	GET

This method shall support the URI query parameters specified in table 5.2.8.3.1-1.

Table 5.2.8.3.1-1: URI query parameters supported by the GET method on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| n/a |  |  |  |  |

This method shall support the request data structures specified in table 5.2.8.3.1-2 and the response data structures and response codes specified in table 5.2.8.3.1-3.

Table 5.2.8.3.1-2: Data structures supported by the GET Request Body on this resource

| Data type | P | Cardinality | Description |
| --- | --- | --- | --- |
| n/a |  |  |  |

Table 5.2.8.3.1-3: Data structures supported by the GET Response Body on this resource

| Data type | P | Cardinality | Response codes | Description |
| --- | --- | --- | --- | --- |
| array(BdtData) | M | 0..N | 200 OK | Upon success, a response body containing the BDT data shall be returned. Each member of the array is an individual BDT data. |
| NOTE: The mandatory HTTP error status codes for the GET method listed in table 5.2.7.1-1 of 3GPP TS 29.500 [4] also apply. |  |  |  |  |

### 5.2.9	Resource: IndividualBdtData

#### 5.2.9.1	Description

The resource represents individual background data transfer data stored in the UDR for a given "bdtReferenceId".

#### 5.2.9.2	Resource definition

Resource URI: {apiRoot}/nudr-dr/{apiVersion}/policy-data/bdt-data/{bdtReferenceId}

This resource shall support the resource URI variables defined in table 5.2.9.2-1.

Table 5.2.9.2-1: Resource URI variables for this resource

| Name | Definition |
| --- | --- |
| apiRoot | See 3GPP TS 29.504 [6] subclause 6.1.1. |
| apiVersion | See 3GPP TS 29.504 [6] subclause 6.1.1. |
| bdtReferenceId | String that represents the transfer policy of background data transfer for provided ASP as defined in 3GPP TS 29.122 [9]. |

#### 5.2.9.3	Resource Standard Methods

##### 5.2.9.3.1	GET

This method shall support the URI query parameters specified in table 5.2.9.3.1-1.

Table 5.2.9.3.1-1: URI query parameters supported by the GET method on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| n/a |  |  |  |  |

This method shall support the request data structures specified in table 5.2.9.3.1-2 and the response data structures and response codes specified in table 5.2.9.3.1-3.

Table 5.2.9.3.1-2: Data structures supported by the GET Request Body on this resource

| Data type | P | Cardinality | Description |
| --- | --- | --- | --- |
| n/a |  |  |  |

Table 5.2.9.3.1-3: Data structures supported by the GET Response Body on this resource

| Data type | P | Cardinality | Response codes | Description |
| --- | --- | --- | --- | --- |
| BdtData | M | 1 | 200 OK | Upon success, a response body containing the BDT data shall be returned. |
| NOTE: The mandatory HTTP error status codes for the GET method listed in table 5.2.7.1-1 of 3GPP TS 29.500 [4] also apply. |  |  |  |  |

##### 5.2.9.3.2	PUT

This method shall support the URI query parameters specified in table 5.2.9.3.2-1.

Table 5.2.9.3.2-1: URI query parameters supported by the PUT method on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| n/a |  |  |  |  |

This method shall support the request data structures specified in table 5.2.9.3.2-2 and the response data structures and response codes specified in table 5.2.9.3.2-3.

Table 5.2.9.3.2-2: Data structures supported by the PUT Request Body on this resource

| Data type | P | Cardinality | Description |
| --- | --- | --- | --- |
| BdtData | M | 1 | Create the BDT data for a given bdtReferenceId. |

Table 5.2.9.3.2-3: Data structures supported by the PUT Response Body on this resource

| Data type | P | Cardinality | Response codes | Description |
| --- | --- | --- | --- | --- |
| BdtData | M | 1 | 201 Created | Successful case. The resource has been successfully created. |
| ProblemDetails | M | 1 | 403 Forbidden | The resource is not allowed to be updated. (NOTE 2) |
| NOTE 1: The mandatory HTTP error status codes for the PUT method listed in table 5.2.7.1-1 of 3GPP TS 29.500 [4] also apply. NOTE 2: The "cause" attribute within the "ProblemDetails" data structure may be set to "UPDATE_NOT_ALLOWED" as defined in subclause 5.5.3. |  |  |  |  |

##### 5.2.9.3.3	DELETE

This method shall support the URI query parameters specified in table 5.2.9.3.3-1.

Table 5.2.9.3.3-1: URI query parameters supported by the DELETE method on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| n/a |  |  |  |  |

This method shall support the request data structures specified in table 5.2.9.3.3-2 and the response data structures and response codes specified in table 5.2.9.3.3-3.

Table 5.2.9.3.3-2: Data structures supported by the DELETE Request Body on this resource

| Data type | P | Cardinality | Description |
| --- | --- | --- | --- |
| n/a |  |  |  |

Table 5.2.9.3.3-3: Data structures supported by the DELETE Response Body on this resource

| Data type | P | Cardinality | Response codes | Description |
| --- | --- | --- | --- | --- |
| n/a |  |  | 204 No Content | Successful case. The resource has been successfully deleted. |
| NOTE: The mandatory HTTP error status codes for the DELETE method listed in table 5.2.7.1-1 of 3GPP TS 29.500 [4] also apply. |  |  |  |  |

### 5.2.10	Resource: PolicyDataSubscriptions

#### 5.2.10.1	Description

This resource is used to represent subscriptions to notification of policy data modification.

#### 5.2.10.2	Resource definition

Resource URI: {apiRoot}/nudr-dr/{apiVersion}/policy-data/subs-to-notify

This resource shall support the resource URI variables defined in table 5.2.10.2-1.

Table 5.2.10.2-1: Resource URI variables for this resource

| Name | Definition |
| --- | --- |
| apiRoot | See 3GPP TS 29.504 [6] subclause 6.1.1. |
| apiVersion | See 3GPP TS 29.504 [6] subclause 6.1.1. |

#### 5.2.10.3	Resource Standard Methods

##### 5.2.10.3.1	POST

This method shall support the URI query parameters specified in table 5.2.10.3.1-1.

Table 5.2.10.3.1-1: URI query parameters supported by the POST method on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| n/a |  |  |  |  |

This method shall support the request data structures specified in table 5.2.10.3.1-2 and the response data structures and response codes specified in table 5.2.10.3.1-3.

Table 5.2.10.3.1-2: Data structures supported by the POST Request Body on this resource

| Data type | P | Cardinality | Description |
| --- | --- | --- | --- |
| PolicyDataSubscription | M | 1 | Identifies the subscription to notification of policy data modification. |

Table 5.2.10.3.1-3: Data structures supported by the POST Response Body on this resource

| Data type | P | Cardinality | Response codes | Description |
| --- | --- | --- | --- | --- |
| PolicyDataSubscription | M | 1 | 201 Created | Upon success, a response body containing a representation of each Individual subscription resource shall be returned. The HTTP response shall include a "Location" HTTP header that contains the resource URI of the created resource. |
| NOTE: The mandatory HTTP error status codes for the POST method listed in table 5.2.7.1-1 of 3GPP TS 29.500 [4] also apply. |  |  |  |  |

### 5.2.11	Resource: IndividualPolicyDataSubscription

#### 5.2.11.1	Description

This resource is used to represent an individual subscription to notification of policy data modification.

#### 5.2.11.2	Resource definition

Resource URI: {apiRoot}/nudr-dr/{apiVersion}/policy-data/subs-to-notify/{subsId}

This resource shall support the resource URI variables defined in table 5.2.11.2-1.

Table 5.2.11.2-1: Resource URI variables for this resource

| Name | Definition |
| --- | --- |
| apiRoot | See 3GPP TS 29.504 [6] subclause 6.1.1. |
| apiVersion | See 3GPP TS 29.504 [6] subclause 6.1.1. |
| subsId | The subsId identifies an individual policy data subscription to notifications. The value is allocated by the UDR during creation of the PolicyDataSubscriptions resource. |

#### 5.2.11.3	Resource Standard Methods

##### 5.2.11.3.1	PUT

This method shall support the URI query parameters specified in table 5.2.11.3.1-1.

Table 5.2.11.3.1-1: URI query parameters supported by the PUT method on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| n/a |  |  |  |  |

This method shall support the request data structures specified in table 5.2.11.3.1-2 and the response data structures and response codes specified in table 5.2.11.3.1-3.

Table 5.2.11.3.1-2: Data structures supported by the PUT Request Body on this resource

| Data type | P | Cardinality | Description |
| --- | --- | --- | --- |
| PolicyDataSubscription | M | 1 | Updates the subscription to notification of policy data modification. |

Table 5.2.11.3.1-3: Data structures supported by the PUT Response Body on this resource

| Data type | P | Cardinality | Response codes | Description |
| --- | --- | --- | --- | --- |
| PolicyDataSubscription | M | 1 | 200 OK | The individual subscription resource was updated successfully. |
| n/a |  |  | 204 No Content | The individual subscription resource was updated successfully and no additional content is to be sent in the response message. |
| NOTE: The mandatory HTTP error status codes for the PUT method listed in table 5.2.7.1-1 of 3GPP TS 29.500 [4] also apply. |  |  |  |  |

##### 5.2.11.3.2	DELETE

This method shall support the URI query parameters specified in table 5.2.11.3.2-1.

Table 5.2.11.3.2-1: URI query parameters supported by the DELETE method on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| n/a |  |  |  |  |

This method shall support the request data structures specified in table 5.2.11.3.2-2 and the response data structures and response codes specified in table 5.2.11.3.2-3.

Table 5.2.11.3.2-2: Data structures supported by the DELETE Request Body on this resource

| Data type | P | Cardinality | Description |
| --- | --- | --- | --- |
| n/a |  |  | The request body shall be empty. |

Table 5.2.11.3.2-3: Data structures supported by the DELETE Response Body on this resource

| Data type | P | Cardinality | Response codes | Description |
| --- | --- | --- | --- | --- |
| n/a |  |  | 204 No Content | Upon success, an empty response body shall be returned. |
| NOTE: The mandatory HTTP error status codes for the DELETE method listed in table 5.2.7.1-1 of 3GPP TS 29.500 [4] also apply. |  |  |  |  |

### 5.2.12	Resource: OperatorSpecificData

#### 5.2.12.1	Description

This resource represents the UE’s operator specific data.

This resource is modelled with the Document resource archetype (see subclause C.1 of 3GPP TS 29.501 [5]).

#### 5.2.12.2	Resource Definition

Resource URI: {apiRoot}/nudr-dr/{apiVersion}/policy-data/ues/{ueId}/operator-specific-data

This resource shall support the resource URI variables defined in table 5.2.12.2-1.

Table 5.2.12.2-1: Resource URI variables for this resource

| Name | Definition |
| --- | --- |
| apiRoot | See 3GPP TS 29.504 [6] subclause 6.1.1. |
| apiVersion | See 3GPP TS 29.504 [6] subclause 6.1.1. |
| ueId | Represents the Subscription Identifier SUPI or GPSI, see the data type VarUeId as defined in 3GPP TS 29.571 [7]. |

#### 5.2.12.3	Resource Standard Methods

##### 5.2.12.3.1	GET

This method is used to retrieve operator specific data from the UDR.

This method shall support the URI query parameters specified in table 5.2.12.3.1-1.

Table 5.2.12.3.1-1: URI query parameters supported by the GET method on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| fields | array(string) | C | 1..N | When the NF consumer only retrieves a subset of the resource, the "fields" query parameter shall be included. The "fields" query parameter contains the pointers of the attribute(s) to be retrieved. |

This method shall support the request data structures specified in table 5.2.12.3.1-2 and the response data structures and response codes specified in table 5.2.12.3.1-3.

Table 5.2.12.3.1-2: Data structures supported by the GET Request Body on this resource

| Data type | P | Cardinality | Description |
| --- | --- | --- | --- |
| n/a |  |  |  |

Table 5.2.12.3.1-3: Data structures supported by the GET Response Body on this resource

| Data type | P | Cardinality | Response codes | Description |
| --- | --- | --- | --- | --- |
| map(OperatorSpecificDataContainer) | M | 1..N | 200 OK | Upon success, a response body containing a map shall be returned. The key of the map is operator specific data element name and the value is the operator specific data of the UE. |
| NOTE: The mandatory HTTP error status codes for the GET method listed in table 5.2.7.1-1 of 3GPP TS 29.500 [4] also apply. |  |  |  |  |

##### 5.2.12.3.2	PATCH

This method is used to modify operator specific data in the UDR.

This method shall support the URI query parameters specified in table 5.2.12.3.2-1.

Table 5.2.12.3.2-1: URI query parameters supported by the PATCH method on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| n/a |  |  |  |  |

This method shall support the request data structures specified in table 5.2.12.3.2-2 and the response data structures and response codes specified in table 5.2.12.3.2-3.

Table 5.2.12.3.2-2: Data structures supported by the PATCH Request Body on this resource

| Data type | P | Cardinality | Description |
| --- | --- | --- | --- |
| array(PatchItem) | M | 1..N | Contains the delta data to the operator specific data |

Table 5.2.12.3.2-3: Data structures supported by the PATCH Response Body on this resource

| Data type | P | Cardinality | Response codes | Description |
| --- | --- | --- | --- | --- |
| n/a |  |  | 204 No Content | Upon successful modification there is no body in the response message. |
| ProblemDetails | M | 1 | 403 Forbidden | One or more attributes are not allowed to be modified according to e.g. policy or local configuration. The invalidParams attribute shall contain the JSON pointers of attributes which are not allowed to be modified. The cause attribute shall be set to "UPDATE_NOT_ALLOWED" as defined in subclause 5.5.3. |
| NOTE: The mandatory HTTP error status codes for the PATCH method listed in table 5.2.7.1-1 of 3GPP TS 29.500 [4] also apply. |  |  |  |  |

##### 5.2.12.3.3	PUT

This method is used to modify operator specific data in the UDR.

This method shall support the URI query parameters specified in table 5.2.12.3.3-1.

Table 5.2.12.3.3-1: URI query parameters supported by the PUT method on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| n/a |  |  |  |  |

This method shall support the request data structures specified in table 5.2.12.3.3-2 and the response data structures and response codes specified in table 5.2.12.3.3-3.

Table 5.2.12.3.3-2: Data structures supported by the PUT Request Body on this resource

| Data type | P | Cardinality | Description |
| --- | --- | --- | --- |
| map(OperatorSpecificDataContainer) | M | 1..N | A map containing the delta data to the operator specific data. The key of the map is operator specific data element name and the value is the operator specific data of the UE. |

Table 5.2.12.3.3-3: Data structures supported by the PUT Response Body on this resource

| Data type | P | Cardinality | Response codes | Description |
| --- | --- | --- | --- | --- |
| map(OperatorSpecificDataContainer) | M | 1 | 200 OK | Upon successful modification a response body containing a map shall be returned. The key of the map is operator specific data element name and the value is the operator specific data of the UE. |
| n/a |  |  | 204 No Content | The resource has been successfully updated. |
| ProblemDetails | M | 1 | 403 Forbidden | One or more attributes are not allowed to be modified according to e.g. policy or local configuration. The invalidParams attribute shall contain the JSON pointers of attributes which are not allowed to be modified. The cause attribute shall be set to "UPDATE_NOT_ALLOWED" as defined in subclause 5.5.3. |
| NOTE: The mandatory HTTP error status codes for the PUT method listed in table 5.2.7.1-1 of 3GPP TS 29.500 [4] also apply. |  |  |  |  |

### 5.2.13	Resource: PlmnUePolicySet

#### 5.2.13.1	Description

The resource represents the UE policy set for a given "plmnId" representing a H-PLMN stored in the UDR of a V-PLMN.

#### 5.2.13.2	Resource definition

Resource URI: {apiRoot}/nudr-dr/{apiVersion}/policy-data/plmns/{plmnId}/ue-policy-set

This resource shall support the resource URI variables defined in table 5.2.13.2-1.

Table 5.2.13.2-1: Resource URI variables for this resource

| Name | Definition |
| --- | --- |
| apiRoot | See 3GPP TS 29.504 [6] subclause 6.1.1. |
| apiVersion | See 3GPP TS 29.504 [6] subclause 6.1.1. |
| plmnId | String concatenating a Mcc and Mnc as defined in 3GPP TS 29.505 [15]. |

#### 5.2.13.3	Resource Standard Methods

##### 5.2.13.3.1	GET

This method shall support the URI query parameters specified in table 5.2.13.3.1-1.

Table 5.2.13.3.1-1: URI query parameters supported by the GET method on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| n/a |  |  |  |  |

This method shall support the request data structures specified in table 5.2.13.3.1-2 and the response data structures and response codes specified in table 5.2.13.3.1-3.

Table 5.2.13.3.1-2: Data structures supported by the GET Request Body on this resource

| Data type | P | Cardinality | Description |
| --- | --- | --- | --- |
| n/a |  |  |  |

Table 5.2.13.3.1-3: Data structures supported by the GET Response Body on this resource

| Data type | P | Cardinality | Response codes | Description |
| --- | --- | --- | --- | --- |
| UePolicySet | M | 1 | 200 OK | Upon success, a response body containing the UE policies shall be returned. |
| NOTE: The mandatory HTTP error status codes for the GET method listed in table 5.2.7.1-1 of 3GPP TS 29.500 [4] also apply. |  |  |  |  |

## 5.3	Notifications

### 5.3.1	General

Notifications shall comply with subclause 6.2 of 3GPP TS 29.500 [4] and subclause 4.6.2.3 of 3GPP TS 29.501 [5].

This subclause describes the resources to provide notification to NF service consumers which have subscribed to be notified when policy data is changed.

### 5.3.2	Policy Data Change Notification

The POST method shall be used for policy data change notification and the URI shall be provided during the subscription procedure.

URI: {notificationUri}

This method shall support the URI query parameters specified in table 5.3.2-1.

Table 5.3.2-1: URI query parameters supported by the POST method

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| n/a |  |  |  |  |

This method shall support the request data structures specified in table 5.3.2-2 and the response data structures and response codes specified in table 5.3.2-3.

Table 5.3.2-2: Data structures supported by the POST Request Body

| Data type | P | Cardinality | Description |
| --- | --- | --- | --- |
| array(PolicyDataChangeNotification) | M | 1..N | Notification of policy data changes. |

Table 5.3.2-3: Data structures supported by the POST Response Body

| Data type | P | Cardinality | Response codes | Description |
| --- | --- | --- | --- | --- |
| n/a |  |  | 204 No Content | The reception of the notification is acknowledged. |
| NOTE: The mandatory HTTP error status codes for the POST method listed in table 5.2.7.1-1 of 3GPP TS 29.500 [4] also apply. |  |  |  |  |

## 5.4	Data Model

### 5.4.1	General

This subclause specifies the application data model supported by the API.

Table 5.4.1-1 specifies the data types defined for the Nudr_DataRepository for Policy Data service based interface protocol.

Table 5.4.1-1: Nudr_DataRepository specific Data Types for Policy Data

| Data type | Section defined | Description | Applicability |
| --- | --- | --- | --- |
| AmPolicyData | 5.4.2.2 | Contains the AM policy data for a given subscriber. |  |
| BdtData | 5.4.2.9 | Contains the background data transfer data. |  |
| DnnRouteSelectionDescriptor | 5.4.2.20 | Contains the route selector parameters per DNN. |  |
| LimitIdToMonitoringKey | 5.4.2.16 | Contains the limit identifier and the corresponding monitoring key for a given S-NSSAI and DNN. |  |
| OsId | 5.4.3.2 | Operating System supported by the UE. |  |
| Periodicity | 5.4.3.4 | Indicates a type of time period. |  |
| PlmnRouteSelectionDescriptor | 5.4.2.18 | Contains the route selectors for a serving PLMN. |  |
| PolicyDataChangeNotification | 5.4.2.11 | Contains changed policy data for which notification was requested. |  |
| PolicyDataSubscription | 5.4.2.10 | Identifies a subscription to policy data change notification. |  |
| SmPolicyData | 5.4.2.5 | Contains SM policy data for a subscriber and S-NSSAI. |  |
| SmPolicyDnnData | 5.4.2.15 | Contains SM policy data for a DNN and S-NSSAI. |  |
| SmPolicySnssaiData | 5.4.2.14 | Contains SM policy data for a S-NSSAI. |  |
| SnssaiRouteSelectionDescriptor | 5.4.2.19 | Contains the route selector parameters per SNSSAI. |  |
| SponsorConnectivityData | 5.4.2.8 | Contains the sponsored data connectivity related information for a sponsor identifier. |  |
| TimePeriod | 5.4.2.12 | Contains the periodicity for the defined usage monitoring data limits. |  |
| UePolicySetPatch | 5.4.2.17 | Contains the UE policy data that can be modified by the PCF. |  |
| UePolicySection | 5.4.2.3 | Contains the UE policy section. |  |
| UePolicySet | 5.4.2.4 | Contains the UE policy set for a given subscriber. |  |
| UsageMonLevel | 5.4.3.3 | Indicates the level of the usage monitoring instance (PDU Session level or per Service) |  |
| UsageMonData | 5.4.2.7 | Contains remain allowed usage data for a subscriber. |  |
| UsageMonDataLimit | 5.4.2.6 | Contains usage monitoring control data for a subscriber. |  |
| UsageMonDataScope | 5.4.2.13 | Contains SNSSAI and DNN combinations to which the UsageMonData instance belongs to. |  |

Table 5.4.1-2 specifies data types re-used by the Nudr_DataRepository for Policy Data service based interface protocol from other specifications, including a reference to their respective specifications and when needed, a short description of their use within the Nudr_DataRepository for Policy Data service based interface.

Table 5.4.1-2: Nudr_DataRepository re-used Data Types for Policy Data

| Data type | Reference | Comments | Applicability |
| --- | --- | --- | --- |
| BitRate | 3GPP TS 29.571 [7] | String representing a bit rate that shall be formatted as follows: pattern: "^\d+(\.\d+)? (bps¦Kbps¦Mbps¦Gbps¦Tbps)$" Examples: "125 Mbps", "0.125 Gbps", "125000 Kbps". |  |
| Bytes | 3GPP TS 29.571 [7] | String with format "byte" as defined in OpenAPI Specification [3], i.e., base64-encoded characters. |  |
| ChargingInformation | 3GPP TS 29.512 [12] | The address of the Charging Function. |  |
| DateTime | 3GPP TS 29.571 [7] | String with format "date-time" as defined in OpenAPI Specification [3]. |  |
| Dnn | 3GPP TS 29.571 [7] | Data Network Name. |  |
| NetworkAreaInfo | 3GPP TS 29.554 [13] | Represents a network area information. |  |
| OperatorSpecificDataContainer | 3GPP TS 29.505 [15] | Container for operator specific data |  |
| PatchItem | 3GPP TS 29.571 [7] | Data structure used for JSON patch. |  |
| PduSessionType | 3GPP TS 29.571 [7] | PDU Session Type. |  |
| Pei | 3GPP TS 29.571 [7] | Personal Equipment Identifier. |  |
| PlmnId | 3GPP TS 29.571 [7] | PLMN Identity. |  |
| PresenceInfo | 3GPP TS 29.571 [7] | Presence Reporting Area Information. |  |
| ProblemDetails | 3GPP TS 29.571 [7] | Used in error responses to provide more detailed information about an error. |  |
| Snssai | 3GPP TS 29.571 [7] | Identifies the S-NSSAI. |  |
| SscMode | 3GPP TS 29.571 [7] | SSC mode. |  |
| SupportedFeatures | 3GPP TS 29.571 [7] | Used to negotiate the applicability of the optional features |  |
| TransferPolicy | 3GPP TS 29.554 [13] | Represents a transfer policy. |  |
| Uinteger | 3GPP TS 29.571 [7] | Unsigned Integer, i.e. only value 0 and integers greater than 0 are allowed. |  |
| Uri | 3GPP TS 29.571 [7] | String providing an URI. |  |
| UsageThreshold | 3GPP TS 29.122 [9] | Usage Thresholds (a data volume expected to be transferred per UE and/or time duration in seconds). |  |
| VarUeId | 3GPP TS 29.571 [7] | String represents the SUPI or GPSI. |  |

### 5.4.2	Structured data types

#### 5.4.2.1	Introduction

This subclause defines the structures to be used in resource representations.

#### 5.4.2.2	Type AmPolicyData

Table 5.4.2.2-1: Definition of type AmPolicyData

| Attribute name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| praInfos | map(PresenceInfo) | O | 1..N | Presence reporting area information. Each PresenceInfo element shall include the Presence Reporting Area Identifier within the "praId" attribute and, for a UE-dedicated presence reporting area, may also include the list of elements composing the presence reporting area. A "praId" may indicate a Presence Reporting Area Set. The "praId" attribute within the PresenceInfo data type shall also be the key of the map. The attribute "presenceState" shall not be present. |
| subscCats | array(string) | O | 1..N | List of categories associated with the subscriber |

#### 5.4.2.3	Type UePolicySection

Table 5.4.2.3-1: Definition of type UePolicySection

| Attribute name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| uePolicySectionInfo | Bytes | M | 1 | Contains the UE Policy Section Information for the given UPSI |
| upsi | string | M | 1 | Identifier for UE Policy Section. The format is represented in 3GPP TS 24.501 [11] subclause D.6.2. |

#### 5.4.2.4	Type UePolicySet

Table 5.4.2.4-1: Definition of type UePolicySet

| Attribute name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| praInfos | map(PresenceInfo) | O | 1..N | Presence reporting area information. Each PresenceInfo element shall include the Presence Reporting Area Identifier within the "praId" attribute and, for a UE-dedicated presence reporting area, may also include the list of elements composing the presence reporting area. A "praId" may indicate a Presence Reporting Area Set. The "praId" attribute within the PresenceInfo data type shall also be the key of the map. The attribute "presenceState" shall not be present. |
| subscCats | array(string) | O | 1..N | List of categories associated with the subscriber |
| uePolicySections | map(UePolicySection) | O | 1..N | Contains the UE Policy Sections. The UPSI (UE Policy Section Identifier) is used as the key in the map. |
| upsis | array(string) | O | 1..N | List of identifiers for the "uePolicySections". The format of the UPSI is represented in 3GPP TS 24.501 [11] subclause D.6.2 |
| allowedRouteSelDescs | map(PlmnRouteSelectionDescriptor) | O | 1..N | Contains allowed route selection descriptors per serving PLMN for a UE. The serving PLMN identifier is the key of the map. |
| andspInd | boolean | O | 0..1 | Indication of UE supporting ANDSP. Default value is false. |
| pei | Pei | O | 0..1 | Personal Equipment Identifier. |
| osIds | array(OsId) | O | 1..N | Identification of the UE Operating System(s). |

#### 5.4.2.5	Type SmPolicyData

Table 5.4.2.5-1: Definition of type SmPolicyData

| Attribute name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| smPolicySnssaiData | map(SmPolicySnssaiData) | M | 1..N | Session Management Policy data per S-NSSAI for all the SNSSAIs of the subscriber. The key of the map is the S-NSSAI. |  |
| umDataLimits | map(UsageMonDataLimit) | O | 1..N | Contains a list of usage monitoring profiles associated with the subscriber. The limit identifier is used as the key of the map. |  |
| umData | map(UsageMonData) | O | 1..N | Contains the remaining allowed usage data associated with the subscriber. The limit identifier is used as the key in the map. |  |
| NOTE: In this Release of the specification the "limitId" corresponds to the Monitoring Key. |  |  |  |  |  |

#### 5.4.2.6	Type UsageMonDataLimit

Table 5.4.2.6-1: Definition of type UsageMonDataLimit

| Attribute name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| limitId | string | M | 1 | Identifies the limit, i.e. the usage monitoring control instance |
| scopes | map(UsageMonDataScope) | O | 1..N | Identifies the SNSSAI and DNN combinations to which the usage monitoring data limit applies. The S-NSSAI is the key of the map. If omitted, the limit applies to every S-NSSAI and DNN. |
| umLevel | UsageMonLevel | O | 0..1 | Indicates the level of the usage monitoring instance (PDU Session level or per Service). |
| startDate | DateTime | C | 0..1 | Start date and time when the usage monitoring instance applies. It shall be provided when the resetPeriod attribute is provided. |
| endDate | DateTime | O | 0..1 | End date and time when the usage monitoring instance applies. |
| usageLimit | UsageThreshold | O | 0..1 | Maximum allowed traffic volume/time usage for a usage monitoring instance. If a resetPeriod is specified, this limit applies to every period separately and not for the entire time between the start and end dates. So for example, if the resetPeriod is daily, the usageLimit is applicable for each day until the end date is specified. |
| resetPeriod | TimePeriod | O | 0..1 | Time period to reset the remaining allowed usage for a periodic usage monitoring instance (postpaid subscriptions). This attribute along with the startDate determine the actual absolute time at which usage needs to be reset. |
| NOTE: In this Release of the specification the "limitId" attribute is the Monitoring Key, and the "scopes" attribute shall always be present including one distinct SNSSAI and DNN pair. |  |  |  |  |

#### 5.4.2.7	Type UsageMonData

Table 5.4.2.7-1: Definition of type UsageMonData

| Attribute name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| limitId | string | M | 1 | Identifies the limit as defined in the "limitId" attribute of the UsageMonDataLimit data type (see subclause 5.4.2.6). |  |
| umLevel | UsageMonLevel | O | 0..1 | Indicates the level of the usage monitoring instance (PDU Session level or per Service). |  |
| allowedUsage | UsageThreshold | O | 0..1 | Remaining allowed traffic volume usage and/or resource time usage. |  |
| resetTime | DateTime | O | 0..1 | Time to reset the allowed usage back to the usageLimit of the corresponding UsageMonDataLimit. This is computed based on the corresponding startDate, resetPeriod and endDate of the corresponding UsageMonDataLimit. The allowedUsage is only applicable if the resetTime is in the future relative to the current time. |  |
| NOTE: In this Release of the specification the "limitId" attribute is the Monitoring Key, and the "scopes" attribute shall always be present including one distinct SNSSAI and DNN pair. |  |  |  |  |  |

#### 5.4.2.8	Type SponsorConnectivityData

Table 5.4.2.8-1: Definition of type SponsorConnectivityData

| Attribute name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| aspIds | array(string) | M | 1..N | Identifies a list of Application Service Provider Identifiers associated with the sponsor identity. |

#### 5.4.2.9	Type BdtData

Table 5.4.2.9-1: Definition of type BdtData

| Attribute name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| aspId | string | M | 1 | This IE contains an identity of an application service provider. |
| transPolicy | TransferPolicy | M | 1 | This IE contains the transfer policy. |
| bdtRefId | string | O | 0..1 | This IE indicates the background data transfer reference ID for the transfer policy. |
| nwAreaInfo | NetworkAreaInfo | O | 0..1 | This IE represents a network area information. |
| numOfUes | Uinteger | O | 0..1 | This IE contains the number of UEs using the indicated transfer policy. It shall be present when available. |
| volPerUe | UsageThreshold | O | 0..1 | This IE contains a data volume expected to be transferred per UE for the indicated transfer policy. It shall be present when available. |

#### 5.4.2.10	Type PolicyDataSubscription

Table 5.4.2.10-1: Definition of type PolicyDataSubscription

| Attribute name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| notificationUri | Uri | M | 1 | URI provided by the NF service consumer indicating where to receive the subscribed notifications from the UDR. |
| monitoredResourceUris | array(Uri) | M | 1..N | A set of URIs that identify the resources as defined in table 5.2.2-1 for which a modification triggers a notification. (NOTE) |
| expiry | DateTime | C | 0..1 | This IE shall be included in a subscription response if, based on operator policy and taking into account the expiry time included in the request, the UDR needs to include an expiry time. This IE may be included in a subscription request. When present, this IE shall represent the time after which the subscription becomes invalid. The absence of this attribute in the subscription response means the subscription to be valid without an expiry time. |
| supportedFeatures | SupportedFeatures | C | 0..1 | Used to negotiate the applicability of the optional features. This attribute shall be provided in the POST request and in the response of successful resource creation. |
| NOTE: Neither the resource URI of the IndividualPolicyDataSubscription resource nor the resource URI of the PolicyDataSubscriptions resource shall be included in the "monitoredResourceUris" attribute. |  |  |  |  |

#### 5.4.2.11	Type PolicyDataChangeNotification

Table 5.4.2.11-1: Definition of type PolicyDataChangeNotification

| Attribute name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| amPolicyData | AmPolicyData | O | 0..1 | Access and Mobility Policy Data, if changed and notification was requested. |  |
| uePolicySet | UePolicySet | O | 0..1 | UE Policy Set, if changed and notification was requested. |  |
| plmnUePolicySet | UePolicySet | O | 0..1 | PLMN UE Policy Set, if changed and notification was requested. |  |
| smPolicyData | SmPolicyData | O | 0..1 | Session Management Policy Data, if changed and notification was requested. |  |
| usageMonData | UsageMonData | O | 0..1 | Usage Monitoring Data, if changed and notification was requested. |  |
| sponsorConnectivityData | SponsorConnectivityData | O | 0..1 | Sponsor data connectivity profile information, if changed and notification was requested. |  |
| bdtData | BdtData | O | 0..1 | Background Data Transfer Data, if changed and notification was requested. |  |
| opSpecData | OperatorSpecificDataContainer | O | 0..1 | Operator Specific Data, if changed and notification was requested. It may only be used when the receiver of the notification is able to univocally identify the changed operator specific data. (NOTE 3) |  |
| opSpecDataMap | map(OperatorSpecificDataContainer) | O | 1..N | Operator Specific Data resource data, if changed and notification was requested. The key of the map is operator specific data element name and the value is the operator specific data of the UE. | OpSpecDataMapNotification |
| ueId | VarUeId | C | 0..1 | Represents the UE subscription identifier SUPI or GPSI. It shall only be present when the "amPolicyData", "uePolicySet", "smPolicyData", "opSpecData" and/or "usageMonData" attribute is present. |  |
| sponsorId | string | C | 0..1 | Represents the sponsor identity. It shall only be present when the "sponsorConnectivityData" attribute is present. |  |
| bdtRefId | string | C | 0..1 | Represents the BDT reference identifier. It shall only be present when the "bdtData" attribute is present. |  |
| usageMonId | string | C | 0..1 | Represents the unique identifier of the individual SM Policy usage monitoring resource. It shall only be present when the "usageMonData" attribute is present. |  |
| plmnId | PlmnId | C | 0..1 | Represents the PLMN identifier. It shall only be present when the "plmnUePolicySet" attribute is present. |  |
| delResources | array(Uri) | O | 1..N | The resources, as defined in table 5.2.2-1, if removed from UDR and notification on resource data change was requested. (NOTE 2) | ResourceRemovalNotificationPolicyData |
| NOTE 1: When the "ResourceRemovalNotificationPolicyData" feature is not supported, at least one of the "amPolicyData", "uePolicySet", "smPolicyData", "usageMonData", "sponsorConnectivityData", "bdtData", "opSpecData" or "plmnUePolicySet" shall be present. NOTE 2: When the "ResourceRemovalNotificationPolicyData" feature is supported, and the "delResources" attribute is present, the attributes that correspond with the values included in the "delResources" attribute shall be omitted. E.g., when the "delResources" attribute includes the value "../policy-data/ues/{ueId}/am-data" the attribute "amPolicyData" shall be omitted. NOTE 3: This attribute should not be used if the receiver of the notification is not able to univocally identify the changed operator specific data, and "opSpecDataMap" should be used instead. |  |  |  |  |  |

#### 5.4.2.12	Type TimePeriod

Table 5.4.2.12-1: Definition of type TimePeriod

| Attribute name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| period | Periodicity | M | 1 | Indicates whether the periodicity is "YEARLY", "MONTHLY", "WEEKLY", "DAILY" or "HOURLY". |  |
| maxNumPeriod | Uinteger | O | 0..1 | Indicates the maximum number of periods after which the usage monitoring instance does not apply. If omitted, there is no limit in the number of periods. |  |

#### 5.4.2.13	Type UsageMonDataScope

Table 5.4.2.13-1: Definition of type UsageMonDataScope

| Attribute name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| snssai | Snssai | M | 1 | Represents the SNSSAI. |  |
| dnn | array(Dnn) | O | 1..N | Represents the Dnn. If omitted, the usage monitoring scope applies to all the DNNs. |  |
| NOTE: In this Release of the specification the "dnn" attribute shall always be present with one and only one entry. |  |  |  |  |  |

#### 5.4.2.14	Type SmPolicySnssaiData

Table 5.4.2.14-1: Definition of type SmPolicySnssaiData

| Attribute name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| snssai | Snssai | M | 1 | S-NSSAI associated with the data. |  |
| smPolicyDnnData | map(SmPolicyDnnData) | O | 1..N | Session Management Policy data per DNN for all the DNNs of the indicated S-NSSAI. The key of the map is the DNN. |  |

#### 5.4.2.15	Type SmPolicyDnnData

Table 5.4.2.15-1: Definition of type SmPolicyDnnData

| Attribute name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| dnn | Dnn | M | 1 | DNN associated with the data |  |
| allowedServices | array(string) | O | 1..N | List of subscriber's allowed service identifiers |  |
| subscCats | array(string) | O | 1..N | List of categories associated with the subscriber |  |
| gbrUI | BitRate | O | 0..1 | Maximum aggregate UL bitrate that can be provided across all GBR QoS Flows in the DNN |  |
| gbrDl | BitRate | O | 0..1 | Maximum aggregate DL bitrate that can be provided across all GBR QoS Flows in the DNN |  |
| adcSupport | boolean | O | 0..1 | Indicates whether application detection and control can be enabled for a subscriber. The default value is "FALSE". |  |
| subscSpendingLimits | boolean | O | 0..1 | Indicates whether the PCF must enforce policies based on subscriber spending limits. The default value is "FALSE". |  |
| ipv4Index | IpIndex | O | 0..1 | Information that identifies which IP pool or external server is used to allocate the IPv4 address. |  |
| ipv6Index | IpIndex | O | 0..1 | Information that identifies which IP pool or external server is used to allocate the IPv6 address. |  |
| offline | boolean | O | 0..1 | Indicates the offline charging is applicable to the PDU session. The default value is "FALSE". |  |
| online | boolean | O | 0..1 | Indicates the online charging is applicable to the PDU session. The default value is "FALSE". |  |
| chfInfo | ChargingInformation | O | 0..1 | The address of the Charging Function |  |
| refUmdLimitIds | map(LimitIdToMonitoringKey) | O | 1..N | A reference to the "UsageMonitoringDataLimit" or "UsageMonitoringData" instances for this DNN and SNSSAI that may also include the related monitoring key(s). The key of the map is the limit identifier. |  |
| mpsPriority | boolean | O | 0..1 | Indicates subscription to the MPS priority service; priority applies to all traffic on the PDU Session. The default value is "FALSE". |  |
| imsSignallingPrio | boolean | O | 0..1 | Indicates subscription to the IMS signalling priority service; priority only applies to IMS signalling traffic. The default value is "FALSE". |  |
| mpsPriorityLevel | integer | O | 0..1 | Relative priority level for the multimedia priority services |  |
| praInfos | map(PresenceInfo) | O | 1..N | Presence reporting area information. Each PresenceInfo element shall include the Presence Reporting Area Identifier within the "praId" attribute and, for a UE-dedicated presence reporting area, may also include the list of elements composing the presence reporting area. A "praId" may indicate a Presence Reporting Area Set. The "praId" attribute within the PresenceInfo data type shall also be the key of the map. The attribute "presenceState" shall not be present. |  |

#### 5.4.2.16	Type LimitIdToMonitoringKey

Table 5.4.2.16-1: Definition of type LimitIdToMonitoringKey

| Attribute name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| limitId | string | M | 1 | Limit Identifier. |  |
| monKeys | array(string) | O | 1..N | Monitoring Keys. |  |
| NOTE: In this Release of the specification there is only one Monitoring Key per Limit Identifier |  |  |  |  |  |

#### 5.4.2.17	Type UePolicySetPatch

Table 5.4.2.17-1: Definiton of type UePolicySetPatch

| Attribute name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| uePolicySections | map(UePolicySection) | O | 1..N | Contains the UE Policy Sections. The UPSI (UE Policy Section Identifier) is used as the key in the map. |
| upsis | array(string) | O | 1..N | List of identifiers for the "uePolicySections". The format of the UPSI is represented in 3GPP TS 24.501 [11] subclause D.6.2. |
| andspInd | boolean | O | 0..1 | Indication of UE supporting ANDSP. |
| pei | Pei | O | 0..1 | Personal Equipment Identifier. |
| osIds | array(OsId) | O | 1..N | Identification of the UE Operating System(s). |

#### 5.4.2.18	Type PlmnRouteSelectionDescriptor

Table 5.4.2.18-1: Definition of type PlmnRouteSelectionDescriptor

| Attribute name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| servingPlmn | PlmnId | M | 1 | Identity of the serving PLMN. |
| snssaiRouteSelDescs | array(SnssaiRouteSelectionDescriptor) | O | 1..N | Contains the route selection descriptor parameters (DNNs, PDU session types and SSC modes) per SNSSAI. |

#### 5.4.2.19	Type SnssaiRouteSelectionDescriptor

Table 5.4.2.19-1: Definition of type SnssaiRouteSelectionDescriptor

| Attribute name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| snssai | Snssai | M | 1 | SNSSAI. |
| dnnRouteSelDescs | array(DnnRouteSelectionDescriptor) | O | 1..N | Contains the route selection descriptor parameters (PDU session types and SSC modes) per DNN. |

#### 5.4.2.20	Type DnnRouteSelectionDescriptor

Table 5.4.2.20-1: Definition of type DnnRouteSelectionDescriptor

| Attribute name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| dnn | Dnn | M | 1 | DNN. |
| sscModes | array(SscMode) | O | 1..N | Contains the allowed SSC modes for the traffic to this DNN. |
| pduSessTypes | array(PduSessionType) | O | 1..N | Contains the allowed PDU Session types for the traffic to this DNN. |

### 5.4.3	Simple data types and enumerations

#### 5.4.3.1	Introduction

This subclause defines simple data types and enumerations that can be referenced from data structures defined in the previous subclauses.

#### 5.4.3.2	Simple data types

The simple data types defined in table 5.4.3.2-1 shall be supported.

Table 5.4.3.2-1: Simple data types

| Type Name | Type Definition | Description | Applicability |
| --- | --- | --- | --- |
| IpIndex | integer | Information that identifies which IP pool or external server is used to allocate the IP address. |  |
| OsId | string | Operating System of the served UE. It contains a Universally Unique IDentifier (UUID) as specified in IETF RFC 4122 [17]. The format of the OS Id is represented in 3GPP TS 24.501 [11]. |  |

#### 5.4.3.3	Enumeration: UsageMonLevel

The enumeration represents the usage monitoring level. It shall comply with the provisions defined in table 5.4.3.3-1.

Table 5.4.3.3-1: Enumeration UsageMonLevel

| Enumeration value | Description |
| --- | --- |
| SESSION_LEVEL | The usage monitoring is applicable for PDU session level. |
| SERVICE_LEVEL | The usage monitoring is applicable for service level. |

#### 5.4.3.4	Enumeration: Periodicity

The enumeration represents the time period. It shall comply with the provisions defined in table 5.4.3.4-1.

Table 5.4.3.4-1: Enumeration Periodicity

| Enumeration value | Description |
| --- | --- |
| YEARLY | Time period is yearly |
| MONTHLY | Time period is monthly |
| WEEKLY | Time period is weekly |
| DAILY | Time period is daily |
| HOURLY | Time period is hourly |

## 5.5	Error handling

### 5.5.1	General

For the Nudr_DataRepository service API for Policy Data, HTTP error responses shall be supported as specified in subclause 4.8 of 3GPP TS 29.501 [5]. Protocol errors and application errors specified in table 5.2.7.2-1 of 3GPP TS 29.500 [4] shall be supported for an HTTP method if the corresponding HTTP status codes are specified as mandatory for that HTTP method in table 5.2.7.1-1 of 3GPP TS 29.500 [4].

In addition, the requirements in the following subclauses are applicable for the Nudr_DataRepository service API for Policy Data.

### 5.5.2	Protocol Errors

No specific procedures for the Nudr_DataRepository service API for Policy Data are specified.

### 5.5.3	Application Errors

The application errors defined for the Nudr_DataRepository service API for Policy Data are listed in table 5.5.3-1. The PCF shall include in the HTTP status code a "ProblemDetails" data structure with the "cause" attribute indicating the application error as listed in table 5.5.3-1.

Table 5.5.3-1: Application errors

| Application Error | HTTP status code | Description |
| --- | --- | --- |
| UPDATE_NOT_ALLOWED | 403 Forbidden | Indicates that the resource is not allowed to be updated. |

## 5.6	Feature negotiation

The optional features in table 6.1.8-1 of 3GPP TS 29.504 [6] are defined for the Nudr_DataRepository API. They shall be negotiated using the extensibility mechanism defined in subclause 6.6 of 3GPP TS 29.500 [4].

# 6	Usage of Nudr_DataRepository Service API for Application Data

## 6.1	Introduction

The following subclauses specify the usage of Nudr_DataRepository service for application data. The principles specified in 3GPP TS 29.504 [6] are followed unless explicitly specified otherwise in the following subclauses.

## 6.2	Resources

### 6.2.1	Overview

This clause specifies the resources for the usage of the Unified Data Repository service for Application Data.

### 6.2.2	Resource Structure

Figure 6.2.2-1: Resource URI structure of the Nudr_DataRepository API for application data

Table 6.2.2-1 provides an overview of the resources and applicable HTTP methods.

Table 6.2.2-1: Resources and methods overview

| Resource name | Resource URI | HTTP method or custom operation | Description |
| --- | --- | --- | --- |
| PFD Data | {apiRoot}/nudr-dr/{apiVersion}/ application-data/pfds | GET | Retrieve PFDs for application identifier(s) identified by query parameter(s). Retrieve PFDs for all application identifier(s) if no query parameter is included in the Request URI. |
| Individual PFD Data | {apiRoot}/nudr-dr/{apiVersion}/ application-data/pfds/{appId} | GET | Retrieve the corresponding PFDs of the specified application identifier. |
|  |  | DELETE | Delete the corresponding PFDs of the specified application identifier. |
|  |  | PUT | Create or update the corresponding PFDs for the specified application identifier. |
| Influence Data | {apiRoot}/nudr-dr/{apiVersion}/ application-data/influenceData | GET | Retrieve the Session Influence Data of given services, S-NSSAIs and DNNs or Internal Group Identifiers or SUPIs. |
| Individual Influence Data | {apiRoot}/nudr-dr/{apiVersion}/ application-data/influenceData/ {influenceId} | PUT | Create an individual Influence Data resource identified by {influenceId}, or modify all of the properties of an individual Influence Data resource identified by {influenceId}. |
|  |  | PATCH | Modify part of the properties of an individual Influence Data resource identified by {influenceId}. |
|  |  | DELETE | Delete an individual Influence Data resource identified by {influenceId}. |
| Influence Data Subscription | {apiRoot}/nudr-dr/{apiVersion}/ application-data/influenceData/ subs-to-notify | POST | Create a new Individual Influence Data Subscription resource. |
|  |  | GET | Read subscriptions for a given S-NSSAI and DNN or Internal Group Identifier or SUPI. |
| Individual Influence Data Subscription | {apiRoot}/nudr-dr/{apiVersion}/ application-data/influenceData/ subs-to-notify/{subscriptionId} | GET | Get an existing individual Influence Data Subscription resource identified by {subscriptionId}. |
|  |  | PUT | Modify an existing individual Influence Data Subscription resource identified by {subscriptionId}. |
|  |  | DELETE | Delete an individual Influence Data Subscription resource identified by {subscriptionId}. |

### 6.2.3	Resource: PFD Data

#### 6.2.3.1	Description

The PFD Data resource represents all the PFD Datas to the Nudr_DataRepository Service at a given UDR.

#### 6.2.3.2	Resource definition

Resource URI: {apiRoot}/nudr-dr/{apiVersion}/application-data/pfds

Table 6.2.3.2-1: Resource URI variables for this resource

| Name | Definition |
| --- | --- |
| apiRoot | See 3GPP TS 29.504 [6] subclause 6.1.1. |
| apiVersion | See 3GPP TS 29.504 [6] subclause 6.1.1. |

#### 6.2.3.3	Resource Standard Methods

##### 6.2.3.3.1	GET

This method shall support the URI query parameters specified in table 6.2.3.3.1-1.

Table 6.2.3.3.1-1: URI query parameters supported by the GET method on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| appId | array(ApplicationId) | O | 1..N | Contains the information of the application identifier(s) for the querying PFD Data resource. If none appId is included in the URI, it applies to all application identifier(s) for the querying PFD Data resource. |

This method shall support the request data structures specified in table 6.2.3.3.1-2 and the response data structures and response codes specified in table 6.2.3.3.1-3.

Table 6.2.3.3.1-2: Data structures supported by the GET Request Body on this resource

| Data type | P | Cardinality | Description |
| --- | --- | --- | --- |
| n/a |  |  |  |

Table 6.2.3.3.1-3: Data structures supported by the GET Response Body on this resource

| Data type | P | Cardinality | Response codes | Description |
| --- | --- | --- | --- | --- |
| array(PfdDataForApp) | M | 0..N | 200 OK | A representation of PFDs for request applications is returned. |
| NOTE: The mandatory HTTP error status codes for the GET method listed in table 5.2.7.1-1 of 3GPP TS 29.500 [4] also apply. |  |  |  |  |

### 6.2.4	Resource: Individual PFD Data

#### 6.2.4.1	Description

The Individual PFD Data resource represents Individual PFD Datas to the Nudr_DataRepository Service at a given UDR.

#### 6.2.4.2	Resource definition

Resource URI: {apiRoot}/nudr-dr/{apiVersion}/application-data/pfds/{appId}

This resource shall support the resource URI variables defined in table 6.2.4.2-1.

Table 6.2.4.2-1: Resource URI variables for this resource

| Name | Definition |
| --- | --- |
| apiRoot | See 3GPP TS 29.504 [6] subclause 6.1.1. |
| apiVersion | See 3GPP TS 29.504 [6] subclause 6.1.1. |
| appId | Indicates the application identifier for the request pfd(s). It shall be formatted as data type ApplicationId, see 3GPP TS 29.571 [7]. |

#### 6.2.4.3	Resource Standard Methods

##### 6.2.4.3.1	GET

This method shall support the URI query parameters specified in table 6.2.4.3.1-1.

Table 6.2.4.3.1-1: URI query parameters supported by the GET method on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| n/a |  |  |  |  |

This method shall support the request data structures specified in table 6.2.4.3.1-2 and the response data structures and response codes specified in table 6.2.4.3.1-3.

Table 6.2.4.3.1-2: Data structures supported by the GET Request Body on this resource

| Data type | P | Cardinality | Description |
| --- | --- | --- | --- |
| n/a |  |  |  |

Table 6.2.4.3.1-3: Data structures supported by the GET Response Body on this resource

| Data type | P | Cardinality | Response codes | Description |
| --- | --- | --- | --- | --- |
| PfdDataForApp | M | 1 | 200 OK | A representation of PFDs for the request application identified by the application identifier is returned. |
| NOTE: The mandatory HTTP error status codes for the GET method listed in table 5.2.7.1-1 of 3GPP TS 29.500 [4] also apply. |  |  |  |  |

##### 6.2.4.3.2	DELETE

This method shall support the URI query parameters specified in table 6.2.4.3.2-1.

Table 6.2.4.3.2-1: URI query parameters supported by the DELETE method on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| n/a |  |  |  |  |

This method shall support the request data structures specified in table 6.2.4.3.2-2 and the response data structures and response codes specified in table 6.2.4.3.2-3.

Table 6.2.4.3.2-2: Data structures supported by the DELETE Request Body on this resource

| Data type | P | Cardinality | Description |
| --- | --- | --- | --- |
| n/a |  |  |  |

Table 6.2.4.3.2-3: Data structures supported by the DELETE Response Body on this resource

| Data type | P | Cardinality | Response codes | Description |
| --- | --- | --- | --- | --- |
| n/a |  |  | 204 No content | Successful case. The Individual PFD Data resource related to the application identifier was deleted. |
| NOTE: The mandatory HTTP error status codes for the DELETE method listed in table 5.2.7.1-1 of 3GPP TS 29.500 [4] also apply. |  |  |  |  |

##### 6.2.4.3.3	PUT

This method shall support the URI query parameters specified in table 6.2.4.3.3-1.

Table 6.2.4.3.3-1: URI query parameters supported by the PUT method on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| n/a |  |  |  |  |

This method shall support the request data structures specified in table 6.2.4.3.3-2 and the response data structures and response codes specified in table 6.2.4.3.3-3.

Table 6.2.4.3.3-2: Data structures supported by the PUT Request Body on this resource

| Data type | P | Cardinality | Description |
| --- | --- | --- | --- |
| PfdDataForApp | M | 1 | Contains the information for the creation or modification of an individual PFD Data resource. |

Table 6.2.4.3.3-3: Data structures supported by the PUT Response Body on this resource

| Data type | P | Cardinality | Response codes | Description |
| --- | --- | --- | --- | --- |
| PfdDataForApp | M | 1 | 201 Created | The resource has been successfully created and a response body is returned containing a representation of the resource. |
| PfdDataForApp | M | 1 | 200 OK | The resource has been successfully updated and a response body is returned containing a representation of the resource. |
| n/a |  |  | 204 No Content | The resource has been successfully updated. |
| NOTE: The mandatory HTTP error status codes for the PUT method listed in table 5.2.7.1-1 of 3GPP TS 29.500 [4] also apply. |  |  |  |  |

### 6.2.5	Resource: Influence Data

#### 6.2.5.1	Description

The Influence Data resource represents a Traffic Influence Data to the Nudr_DataRepository Service at a given UDR.

#### 6.2.5.2	Resource definition

Resource URI: {apiRoot}/nudr-dr/{apiVersion}/application-data/influenceData

This resource shall support the resource URI variables defined in table 6.2.5.2-1.

Table 6.2.5.2-1: Resource URI variables for this resource

| Name | Definition |
| --- | --- |
| apiRoot | See 3GPP TS 29.504 [6] subclause 6.1.1. |
| apiVersion | See 3GPP TS 29.504 [6] subclause 6.1.1. |

#### 6.2.5.3	Resource Standard Methods

##### 6.2.5.3.1	GET

This method shall support the URI query parameters specified in table 6.2.5.3.1-1.

Table 6.2.5.3.1-1: URI query parameters supported by the GET method on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| influence-Ids | array(string) | O | 1..N | Each element identifies a service. |
| dnns | array(Dnn) | O | 1..N | Each element identifies a DNN. |
| snssais | array(Snssai) | O | 1..N | Each element identifies a slice. |
| internal-Group-Ids | array(GroupId) | O | 1..N | Each element identifies a group of users. |
| supis | array(Supi) | O | 1..N | Each element identifies a user. |
| NOTE: At least one of the above attributes shall be provided. |  |  |  |  |

This method shall support the request data structures specified in table 6.2.5.3.1-2 and the response data structures and response codes specified in table 6.2.5.3.1-3.

Table 6.2.5.3.1-2: Data structures supported by the GET Request Body on this resource

| Data type | P | Cardinality | Description |
| --- | --- | --- | --- |
| n/a |  |  |  |

Table 6.2.5.3.1-3: Data structures supported by the GET Response Body on this resource

| Data type | P | Cardinality | Response codes | Description |
| --- | --- | --- | --- | --- |
| array(TrafficInfluData) | M | 0..N | 200 OK | The Traffic Influence Data stored in the UDR are returned. |
| NOTE: The mandatory HTTP error status codes for the GET method listed in table 5.2.7.1-1 of 3GPP TS 29.500 [4] also apply. |  |  |  |  |

### 6.2.6	Resource: Individual Influence Data

#### 6.2.6.1	Description

The Individual Influence Data resource represents an Individual Traffic Influence Data to the Nudr_DataRepository Service at a given UDR.

#### 6.2.6.2	Resource definition

Resource URI: {apiRoot}/nudr-dr/{apiVersion}/application-data/influenceData/{influenceId}

This resource shall support the resource URI variables defined in table 6.2.6.2-1.

Table 6.2.6.2-1: Resource URI variables for this resource

| Name | Definition |
| --- | --- |
| apiRoot | See 3GPP TS 29.504 [6] subclause 6.1.1. |
| apiVersion | See 3GPP TS 29.504 [6] subclause 6.1.1. |
| influenceId | The Identifier of an Individual Influence Data to be updated. It shall apply the format of Data type string. To enable the value to be used as part of a URI, the string shall only contain allowed characters according to the "lower-with-hyphen" naming convention defined in subclause 5.1.3 of 3GPP TS 29.501 [5]. |

#### 6.2.6.3	Resource Standard Methods

##### 6.2.6.3.1	PUT

This method shall support the URI query parameters specified in table 6.2.6.3.1-1.

Table 6.2.6.3.1-1: URI query parameters supported by the PUT method on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| n/a |  |  |  | n/a |

This method shall support the request data structures specified in table 6.2.6.3.1-2 and the response data structures and response codes specified in table 6.2.6.3.1-3.

Table 6.2.6.3.1-2: Data structures supported by the PUT Request Body on this resource

| Data type | P | Cardinality | Description |
| --- | --- | --- | --- |
| TrafficInfluData | M | 1 | The Traffic Influence Data to be stored in the UDR. |

Table 6.2.6.3.1-3: Data structures supported by the PUT Response Body on this resource

| Data type | P | Cardinality | Response codes | Description |
| --- | --- | --- | --- | --- |
| TrafficInfluData | M | 1 | 201 Created | The resource has been successfully created and a response body is returned containing a representation of the resource. |
| TrafficInfluData | M | 1 | 200 OK | The resource has been successfully updated and a response body is returned containing a representation of the resource. |
| n/a |  |  | 204 No Content | The resource has been successfully updated. |
| NOTE: The mandatory HTTP error status codes for the PUT method listed in table 5.2.7.1-1 of 3GPP TS 29.500 [4] also apply. |  |  |  |  |

##### 6.2.6.3.2	PATCH

This method shall support the URI query parameters specified in table 6.2.6.3.2-1.

Table 6.2.6.3.2-1: URI query parameters supported by the PATCH method on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| n/a |  |  |  | n/a |

This method shall support the request data structures specified in table 6.2.6.3.2-2 and the response data structures and response codes specified in table 6.2.6.3.2-3.

Table 6.2.6.3.2-2: Data structures supported by the PATCH Request Body on this resource

| Data type | P | Cardinality | Description |
| --- | --- | --- | --- |
| TrafficInfluDataPatch | M | 1 | The Traffic Influence Data to be updated in the UDR. |

Table 6.2.6.3.2-3: Data structures supported by the PATCH Response Body on this resource

| Data type | P | Cardinality | Response codes | Description |
| --- | --- | --- | --- | --- |
| TrafficInfluData | M | 1 | 200 OK | The update of an Individual Traffic Influence Data resource is confirmed and a response body containing Traffic Influence Data shall be returned. |
| n/a |  |  | 204 No Content | The Individual Traffic Influence Data resource has been successfully updated. |
| NOTE: The mandatory HTTP error status codes for the PATCH method listed in table 5.2.7.1-1 of 3GPP TS 29.500 [4] also apply. |  |  |  |  |

##### 6.2.6.3.3	DELETE

This method shall support the URI query parameters specified in table 6.2.6.3.3-1.

Table 6.2.6.3.3-1: URI query parameters supported by the DELETE method on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| n/a |  |  |  | n/a |

This method shall support the request data structures specified in table 6.2.6.3.3-2 and the response data structures and response codes specified in table 6.2.6.3.3-3.

Table 6.2.6.3.3-2: Data structures supported by the DELETE Request Body on this resource

| Data type | P | Cardinality | Description |
| --- | --- | --- | --- |
| n/a |  |  |  |

Table 6.2.6.3.3-3: Data structures supported by the DELETE Response Body on this resource

| Data type | P | Cardinality | Response codes | Description |
| --- | --- | --- | --- | --- |
| n/a |  |  | 204 No Content | The Individual Influence Data was deleted successfully. |
| NOTE: The mandatory HTTP error status codes for the DELETE method listed in table 5.2.7.1-1 of 3GPP TS 29.500 [4] also apply. |  |  |  |  |

### 6.2.7	Resource: Influence Data Subscription

#### 6.2.7.1	Description

The Influence Data Subscription resource represents all Influence Data subscriptions to the Nudr_DataRepository Service at a given UDR.

#### 6.2.7.2	Resource definition

Resource URI: {apiRoot}/nudr-dr/{apiVersion}/application-data/influenceData/subs-to-notify

This resource shall support the resource URI variables defined in table 6.2.7.2-1.

Table 6.2.7.2-1: Resource URI variables for this resource

| Name | Definition |
| --- | --- |
| apiRoot | See 3GPP TS 29.504 [6] subclause 6.1.1. |
| apiVersion | See 3GPP TS 29.504 [6] subclause 6.1.1. |

#### 6.2.7.3	Resource Standard Methods

##### 6.2.7.3.1	POST

This method shall support the URI query parameters specified in table 6.2.7.3.1-1.

Table 6.2.7.3.1-1: URI query parameters supported by the POST method on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| n/a |  |  |  |  |

This method shall support the request data structures specified in table 6.2.7.3.1-2 and the response data structures and response codes specified in table 6.2.7.3.1-3.

Table 6.2.7.3.1-2: Data structures supported by the POST Request Body on this resource

| Data type | P | Cardinality | Description |
| --- | --- | --- | --- |
| TrafficInfluSub | M | 1 | Parameters to register a subscription to Traffic Influence Data. |

Table 6.2.7.3.1-3: Data structures supported by the POST Response Body on this resource

| Data type | P | Cardinality | Response codes | Description |
| --- | --- | --- | --- | --- |
| TrafficInfluSub | M | 1 | 201 Created | The subscription was created successfully. |
| NOTE: The mandatory HTTP error status codes for the POST method listed in table 5.2.7.1-1 of 3GPP TS 29.500 [4] also apply. |  |  |  |  |

##### 6.2.7.3.2	GET

This method shall support the URI query parameters specified in table 6.2.7.3.2-1.

Table 6.2.7.3.2-1: URI query parameters supported by the GET method on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| dnn | Dnn | O | 0..1 | Identifies a DNN. |
| snssai | Snssai | O | 0..1 | Identifies a slice. |
| internal-Group-Id | GroupId | O | 0..1 | Identifies a group of users. (NOTE 2) |
| supi | Supi | O | 0..1 | Identifies a user. |
| NOTE 1: At least one of the above attribute shall be provided. NOTE 2: For the Traffic Influence Data applies to any UE, the "internal-Group-Id" sets to "AnyUE". |  |  |  |  |

This method shall support the request data structures specified in table 6.2.7.3.2-2 and the response data structures and response codes specified in table 6.2.7.3.2-3.

Table 6.2.7.3.2-2: Data structures supported by the GET Request Body on this resource

| Data type | P | Cardinality | Description |
| --- | --- | --- | --- |
| n/a |  |  |  |

Table 6.2.7.3.2-3: Data structures supported by the GET Response Body on this resource

| Data type | P | Cardinality | Response codes | Description |
| --- | --- | --- | --- | --- |
| array(TrafficInfluSub) | M | 0..N | 200 OK | The subscription information as request in the request URI query parameter(s) are returned. |
| NOTE: The mandatory HTTP error status codes for the GET method listed in table 5.2.7.1-1 of 3GPP TS 29.500 [4] also apply. |  |  |  |  |

### 6.2.8	Resource: Individual Influence Data Subscription

#### 6.2.8.1	Description

The Individual Influence Data Subscription resource represents a single Influence Data subscription to the Nudr_DataRepository Service at a given UDR.

#### 6.2.8.2	Resource definition

Resource URI: {apiRoot}/nudr-dr/{apiVersion}/application-data/influenceData/subs-to-notify/{subscriptionId}

This resource shall support the resource URI variables defined in table 6.2.8.2-1.

Table 6.2.8.2-1: Resource URI variables for this resource

| Name | Definition |
| --- | --- |
| apiRoot | See 3GPP TS 29.504 [6] subclause 6.1.1. |
| apiVersion | See 3GPP TS 29.504 [6] subclause 6.1.1. |
| subscriptionId | String identifying an Individual Influence Data Subscription resource. To enable the value to be used as part of a URI, the string shall only contain allowed characters according to the "lower-with-hyphen" naming convention defined in subclause 5.1.3 of 3GPP TS 29.501 [5]. |

#### 6.2.8.3	Resource Standard Methods

##### 6.2.8.3.1	GET

This method shall support the URI query parameters specified in table 6.2.8.3.1-1.

Table 6.2.8.3.1-1: URI query parameters supported by the GET method on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| n/a |  |  |  |  |

This method shall support the request data structures specified in table 6.2.8.3.1-2 and the response data structures and response codes specified in table 6.2.8.3.1-3.

Table 6.2.8.3.1-2: Data structures supported by the GET Request Body on this resource

| Data type | P | Cardinality | Description |
| --- | --- | --- | --- |
| n/a |  |  |  |

Table 6.2.8.3.1-3: Data structures supported by the GET Response Body on this resource

| Data type | P | Cardinality | Response codes | Description |
| --- | --- | --- | --- | --- |
| TrafficInfluSub | M | 1 | 200 OK | The subscription information is returned. |

##### 6.2.8.3.2	PUT

This method shall support the URI query parameters specified in table 6.2.8.3.2-1.

Table 6.2.8.3.2-1: URI query parameters supported by the PUT method on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| n/a |  |  |  |  |

This method shall support the request data structures specified in table 6.2.8.3.2-2 and the response data structures and response codes specified in table 6.2.8.3.2-3.

Table 6.2.8.3.2-2: Data structures supported by the PUT Request Body on this resource

| Data type | P | Cardinality | Description |
| --- | --- | --- | --- |
| TrafficInfluSub | M | 1 | Modify an existing subscription to Traffic Influence Data. |

Table 6.2.8.3.2-3: Data structures supported by the PUT Response Body on this resource

| Data type | P | Cardinality | Response codes | Description |
| --- | --- | --- | --- | --- |
| TrafficInfluSub | M | 1 | 200 OK | The subscription was updated successfully. |
| n/a |  |  | 204 No Content | The subscription has been successfully updated. |
| NOTE: The mandatory HTTP error status codes for the PUT method listed in table 5.2.7.1-1 of 3GPP TS 29.500 [4] also apply. |  |  |  |  |

##### 6.2.8.3.3	DELETE

This method shall support the URI query parameters specified in table 6.2.8.3.3-1.

Table 6.2.8.3.3-1: URI query parameters supported by the DELETE method on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| n/a |  |  |  |  |

This method shall support the request data structures specified in table 6.2.8.3.3-2 and the response data structures and response codes specified in table 6.2.8.3.3-3.

Table 6.2.8.3.3-2: Data structures supported by the DELETE Request Body on this resource

| Data type | P | Cardinality | Description |
| --- | --- | --- | --- |
| n/a |  |  |  |

Table 6.2.8.3.3-3: Data structures supported by the DELETE Response Body on this resource

| Data type | P | Cardinality | Response codes | Description |
| --- | --- | --- | --- | --- |
| n/a |  |  | 204 No Content | The subscription was terminated successfully. |
| NOTE: The mandatory HTTP error status codes for the DELETE method listed in table 5.2.7.1-1 of 3GPP TS 29.500 [4] also apply. |  |  |  |  |

## 6.3	Notifications

### 6.3.1	General

Notifications shall comply with subclause 6.2 of 3GPP TS 29.500 [4] and subclause 4.6.2.3 of 3GPP TS 29.501 [5].

Table 6.3.1-1: Notifications

| Custom operation URI | Mapped HTTP method | Description |
| --- | --- | --- |
| {notificationUri} | POST | One of notification reference provided by the service consumer during the subscription of the AF Influence Data. |

### 6.3.2	Void

### 6.3.3	Influence Data Update Notification

#### 6.3.3.1	Description

The Influence Update Notification is used by the UDR to report one or several observed Influence Data(s) to a NF service consumer that has subscribed to such Notifications.

#### 6.3.3.2	Operation Definition

The POST method shall be used for traffic influence data change notification and the URI shall be provided during the subscription procedure.

URI: {notificationUri}

This method shall support the URI query parameters specified in table 6.3.3.2-1.

Table 6.3.3.2-1: URI query parameters supported by the POST method

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| n/a |  |  |  |  |

This method shall support the request data structures specified in table 6.3.3.2-2 and the response data structures and response codes specified in table 6.3.3.2-3.

Table 6.3.3.2-2: Data structures supported by the POST Request Body on this resource

| Data type | P | Cardinality | Description |
| --- | --- | --- | --- |
| array(TrafficInfluData) | M | 1..N | Provides Information about observed Influence Data (s). |

Table 6.3.3.2-3: Data structures supported by the POST Response Body on this resource

| Data type | P | Cardinality | Response codes | Description |
| --- | --- | --- | --- | --- |
| n/a |  |  | 204 No Content | The receipt of the Notification is acknowledged. |
| NOTE: The mandatory HTTP error status codes for the POST method listed in table 5.2.7.1-1 of 3GPP TS 29.500 [4] also apply. |  |  |  |  |

## 6.4	Data Model

### 6.4.1	General

This subclause specifies the application data model supported by the API.

Table 6.4.1-1 specifies the data types defined for the Nudr_DataRepository Service API for Application Data service based interface protocol.

Table 6.4.1-1: Nudr_DataRepository specific Data Types for Application Data

| Data type | Section defined | Description | Applicability |
| --- | --- | --- | --- |
| TrafficInfluData | 6.4.2.2 | Contains traffic influence data. |  |
| TrafficInfluDataPatch | 6.4.2.3 | Contains modification instructions to be performed on the traffic influence data. |  |
| TrafficInfluSub | 6.4.2.4 | Contains traffic influence subscription data. |  |

Table 6.4.1-2 specifies data types re-used by the Nudr_DataRepository Service API for Application Data service based interface protocol from other specifications, including a reference to their respective specifications and when needed, a short description of their use within the Nudr_DataRepository Service API for Application Data service based interface.

Table 6.4.1-2: Nudr_DataRepository re-used Data Types for Application Data

| Data type | Reference | Comments | Applicability |
| --- | --- | --- | --- |
| ApplicationId | 3GPP TS 29.571 [7] | Indicates an application identifier. |  |
| DateTime | 3GPP TS 29.571 [7] | Indicates a date and time. |  |
| DnaiChangeType | 3GPP TS 29.571 [7] | Describes the types of DNAI change. |  |
| Dnn | 3GPP TS 29.571 [7] | Identifies a Data Network Name. |  |
| EthFlowDescription | 3GPP TS 29.514 [16] | Contains the Ethernet data flow information. |  |
| FlowInfo | 3GPP TS 29.122 [9] | Contains the flow information. |  |
| NetworkAreaInfo | 3GPP TS 29.554 [13] | Describes a network area information. |  |
| PfdDataForApp | 3GPP TS 29.551 [8] | Identifies the PFDs for one application identifier. |  |
| RouteToLocation | 3GPP TS 29.571 [7] | Identifies the N6 traffic routing requirement. |  |
| Snssai | 3GPP TS 29.571 [7] | Identifies a Single Network Slice Selection Assistance Information. |  |
| SubscribedEvent | 3GPP TS 29.522 [19] | Identified the type of UP path management events of which the AF requests to be notified. |  |
| Supi | 3GPP TS 29.571 [7] | Identifies a SUPI that shall contain either an IMSI or an NAI. |  |
| SupportedFeatures | 3GPP TS 29.571 [7] | Used to negotiate the applicability of the optional features. |  |
| Uri | 3GPP TS 29.571 [7] | Identifies a URI. |  |

### 6.4.2	Structured data types

#### 6.4.2.1	Introduction

This subclause defines the structures to be used in resource representations.

#### 6.4.2.2	Type TrafficInfluData

Table 6.4.2.2-1: Definition of type TrafficInfluData

| Attribute name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| upPathChgNotifCorreId | string | C | 0..1 | Contains the Notification Correlation Id allocated by the NEF for the UP path change notification. It shall be included when the NEF requests the UP path change notification. |  |
| appReloInd | boolean | O | 0..1 | Identifies whether an application can be relocated once a location of the application has been selected. |  |
| afAppId | string | O | 0..1 | Identifies an application. (NOTE 1) (NOTE 3) |  |
| dnn | Dnn | O | 0..1 | Identifies a DNN |  |
| ethTrafficFilters | array(EthFlowDescription) | O | 1..N | Identifies Ethernet packet filters. (NOTE 1) (NOTE 3) |  |
| snssai | Snssai | O | 0..1 | The identification of slice. |  |
| interGroupId | GroupId | O | 0..1 | Identifies a group of users. (NOTE 2) (NOTE 3) (NOTE 4) |  |
| supi | Supi | O | 0..1 | Identifies a user. (NOTE 2) (NOTE 3) |  |
| trafficFilters | array(FlowInfo) | O | 1..N | Identifies IP packet filters. (NOTE 1) (NOTE 3) |  |
| trafficRoutes | array(RouteToLocation) | O | 1..N | Identifies the N6 traffic routing requirement. |  |
| validStartTime | DateTime | O | 0..1 | Identifies when the traffic routings are start to be applicable. |  |
| validEndTime | DateTime | O | 0..1 | Identifies when the traffic routings are not applicable. |  |
| nwAreaInfo | NetworkAreaInfo | O | 0..1 | Identifies a network area information that the request applies only to the traffic of UE(s) located in this specific zone. |  |
| upPathChgNotifUri | Uri | C | 0..1 | Contains the URI where the NEF receives the UP path change notification. It shall be included when the NEF requests the UP path change notification. |  |
| subscribedEvents | array(SubscribedEvent) | O | 1..N | Identifies the requirement to be notified of the event(s). |  |
| dnaiChgType | DnaiChangeType | O | 0..1 | Identifies a type of notification regarding UP path management event. |  |
| NOTE 1: One of "afAppId", "trafficFilters" or "ethTrafficFilters" shall be included. NOTE 2: Either "supi" or "interGroupId" shall be included. NOTE 3: To indicate the deletion of a Traffic Individual Influence Data resource, only the appropriate combination of "supi" or "interGroupId", and "afAppId", "trafficFilters" or "ethTrafficFilters" that identify the resource shall be included. The rest of attributes shall be omitted. NOTE 4: If the Traffic Influence Data applies to any UE, then the "interGroupId" sets to "AnyUE". |  |  |  |  |  |

- 6.4.2.3	Type TrafficInfluDataPatch

Table 6.4.2.3-1: Definition of type TrafficInfluDataPatch

| Attribute name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| upPathChgNotifCorreId | string | O | 0..1 | Contains the Notification Correlation Id allocated by the NEF for the UP path change notification. |  |
| appReloInd | boolean | O | 0..1 | Identifies whether an application can be relocated once a location of the application has been selected. |  |
| dnn | Dnn | O | 0..1 | Identifies a DNN. |  |
| snssai | Snssai | O | 0..1 | The identification of slice |  |
| internalGroupId | GroupId | O | 0..1 | Identifies a group of users. |  |
| ethTrafficFilters | array(EthFlowDescription) | O | 1..N | Identifies Ethernet packet filters. |  |
| supi | Supi | O | 0..1 | Identifies a user. |  |
| trafficFilters | array(FlowInfo) | O | 1..N | Identifies IP packet filters |  |
| trafficRoutes | array(RouteToLocation) | O | 1..N | Identifies the N6 traffic routing requirement. |  |
| validStartTime | DateTime | O | 0..1 | Identifies when the traffic routings start to be applicable. |  |
| validEndTime | DateTime | O | 0..1 | Identifies when the traffic routings are not applicable. |  |
| nwAreaInfo | NetworkAreaInfo | O | 0..1 | Identifies a network area information that the request applies only to the traffic of UE(s) located in this specific zone. |  |
| upPathChgNotifUri | Uri | O | 0..1 | Contains the URI where the NEF receives the UP path change notification. |  |

- 6.4.2.4	Type TrafficInfluSub

Table 6.4.2.4-1: Definition of type TrafficInfluSub

| Attribute name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| dnns | array(Dnn) | O | 1..N | Each element identifies a DNN. |  |
| snssais | array(Snssai) | O | 1..N | Each element identifies a slice. |  |
| internalGroupIds | array(GroupId) | O | 1..N | Each element identifies a group of users.(NOTE 2) |  |
| supis | array(Supi) | O | 1..N | Each element identifies the user. |  |
| notificationUri | Uri | M | 1 | URI provided by the NF service consumer indicating where to receive the subscribed notifications from the UDR. |  |
| expiry | DateTime | C | 0..1 | This IE shall be included in a subscription response if, based on operator policy and taking into account the expiry time included in the request, the UDR needs to include an expiry time. This IE may be included in a subscription request. When present, this IE shall represent the time after which the subscription becomes invalid. The absence of this attribute in the subscription response means the subscription to be valid without an expiry time. |  |
| supportedFeatures | SupportedFeatures | O | 0..1 | Used to negotiate the applicability of the optional features. This attribute shall be provided in the POST request and in the response of successful resource creation. |  |
| NOTE 1: At least one of "dnns", "snssais", "internalGroupIds" or "supis" shall be provided. NOTE 2: For the Traffic Influence Data applies to any UE, the "internalGroupIds" needs to have an entry set to "AnyUE". |  |  |  |  |  |

#### 6.4.2.5	Void

## 6.5	Error handling

### 6.5.1	General

HTTP error handling shall be supported as specified in subclause 5.2.4 of 3GPP TS 29.500 [4].

For the Nudr_DataRepository Service API for Application Data, HTTP error responses shall be supported as specified in subclause 4.8 of 3GPP TS 29.501 [5]. Protocol errors and application errors specified in table 5.2.7.2-1 of 3GPP TS 29.500 [4] shall be supported for an HTTP method if the corresponding HTTP status codes are specified as mandatory for that HTTP method in table 5.2.7.1-1 of 3GPP TS 29.500 [4]. In addition, the requirements in the following subclauses shall apply.

### 6.5.2	Protocol Errors

In this Release of the specification, there are no additional protocol errors applicable for the Nudr_DataRepository Service API for Application Data.

### 6.5.3	Application Errors

The application errors defined for the Nudr_DataRepository Service API for Application Data are listed in table 6.5.3-1. The PCF shall include in the HTTP status code a "ProblemDetails" data structure with the "cause" attribute indicating the application error as listed in table 6.5.3-1.

Table 6.5.3-1: Application errors

| Application Error | HTTP status code | Description |
| --- | --- | --- |

## 6.6	Feature negotiation

The optional features in table 6.1.8-1 of 3GPP TS 29.504 [6] are defined for the Nudr_DataRepository API. They shall be negotiated using the extensibility mechanism defined in subclause 6.6 of 3GPP TS 29.500 [4].

# 7	Usage of Nudr_DataRepository Service API for Structured Data for Exposure

## 7.1	Introduction

The following subclauses specify the usage of Nudr_DataRepository service for exposure data. The principles specified in 3GPP TS 29.504 [6] are followed unless explicitly specified otherwise in the following subclauses.

## 7.2	Resources

### 7.2.1	Overview

This clause specifies the resources for the usage of the Unified Data Repository service for Structured Data for Exposure.

### 7.2.2	Resource Structure

Figure 7.2.2-1: Resource URI structure of the Nudr_DataRepository API for exposure data

Table 7.2.2-1 provides an overview of the resources and applicable HTTP methods.

Table 7.2.2-1: Resources and methods overview

| Resource name | Resource URI | HTTP method | Description |
| --- | --- | --- | --- |
| AccessAndMobilityData | {apiRoot}/nudr-dr/{apiVersion}/ exposure-data/{ueId}/ access-and-mobility-data | PUT | Creates and updates the access and mobility exposure data for a UE |
|  |  | GET | Retrieves the access and mobility exposure data for a UE |
|  |  | DELETE | Deletes the access and mobility exposure data for a UE |
| PduSessionManagementData | {apiRoot}/nudr-dr/{apiVersion}/ exposure-data/{ueId}/ session-management-data/ {pduSessionId} | PUT | Creates and updates the session management data for a UE and for an individual PDU session |
|  |  | GET | Retrieves the session management data for a UE and for an individual PDU session |
|  |  | DELETE | Deletes the session management data for a UE and for an individual PDU session |
| ExposureDataSubscriptions | {apiRoot}/nudr-dr/{apiVersion}/ exposure-data/subs-to-notify | POST | Create a subscription to receive notification of exposure data changes. |
| IndividuaExposureDataSubscription | {apiRoot}/nudr-dr/{apiVersion}/ exposure-data/subs-to-notify/ {subId} | PUT | Modify a subscription to receive notification of exposure data changes. |
|  |  | DELETE | Delete the subscription identified by {subsId}. |

### 7.2.3	Resource: AccessAndMobilityData

#### 7.2.3.1	Description

This resource represents the exposure data related to access and mobility.

#### 7.2.3.2	Resource definition

Resource URI: {apiRoot}/nudr-dr/{apiVersion}/exposure-data/{ueId}/access-and-mobility-data

This resource shall support the resource URI variables defined in table 7.2.3.2-1.

Table 7.2.3.2-1: Resource URI variables for this resource

| Name | Definition |
| --- | --- |
| apiRoot | See 3GPP TS 29.504 [6] subclause 6.1.1. |
| apiVersion | See 3GPP TS 29.504 [6] subclause 6.1.1. |
| ueId | Represents the SUPI or GPSI, see the data type VarUeId as defined in 3GPP TS 29.571 [7]. |

#### 7.2.3.3	Resource Standard Methods

##### 7.2.3.3.1	PUT

This method shall support the URI query parameters specified in table 7.2.3.3.1-1.

Table 7.2.3.3.1-1: URI query parameters supported by the PUT method on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| n/a |  |  |  |  |

This method shall support the request data structures specified in table 7.2.3.3.1-2 and the response data structures and response codes specified in table 7.2.3.3.1-3.

Table 7.2.3.3.1-2: Data structures supported by the PUT Request Body on this resource

| Data type | P | Cardinality | Description |
| --- | --- | --- | --- |
| AccessAndMobilityData | M | 1 | Access and mobility data for the UE is created. |

Table 7.2.3.3.1-3: Data structures supported by the PUT Response Body on this resource

| Data type | P | Cardinality | Response codes | Description |
| --- | --- | --- | --- | --- |
| AccessAndMobilityData | M | 1 | 201 Created | Upon success, a response body is returned containing a representation of the access and mobility data. |
| NOTE: The HTTP status code for the PUT method listed in table 5.2.7.1-1 of 3GPP TS 29.500 [4] also apply. |  |  |  |  |

##### 7.2.3.3.2	GET

This method shall support the URI query parameters specified in table 7.2.3.3.2-1.

Table 7.2.3.3.2-1: URI query parameters supported by the GET method on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| supp-feat | SupportedFeatures | O | 0..1 | see 3GPP TS 29.500 [4] subclause 6.6.2 and 3GPP TS 29.571 [7]. |

This method shall support the request data structures specified in table 7.2.3.3.2-2 and the response data structures and response codes specified in table 7.2.3.3.2-3.

Table 7.2.3.3.2-2: Data structures supported by the GET Request Body on this resource

| Data type | P | Cardinality | Description |
| --- | --- | --- | --- |
| n/a |  |  |  |

Table 7.2.3.3.2-3: Data structures supported by the GET Response Body on this resource

| Data type | P | Cardinality | Response codes | Description |
| --- | --- | --- | --- | --- |
| AccessAndMobilityData | M | 1 | 200 OK | The response body contains the access and mobility data. |
| NOTE: The HTTP status code for the GET method listed in table 5.2.7.1-1 of 3GPP TS 29.500 [4] also apply. |  |  |  |  |

##### 7.2.3.3.3	DELETE

This method shall support the URI query parameters specified in table 7.2.3.3.3-1.

Table 7.2.3.3.3-1: URI query parameters supported by the DELETE method on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| n/a |  |  |  |  |

This method shall support the request data structures specified in table 7.2.3.3.3-2 and the response data structures and response codes specified in table 7.2.3.3.3-3.

Table 7.2.3.3.3-2: Data structures supported by the DELETE Request Body on this resource

| Data type | P | Cardinality | Description |
| --- | --- | --- | --- |
| n/a |  |  |  |

Table 7.2.3.3.3-3: Data structures supported by the DELETE Response Body on this resource

| Data type | P | Cardinality | Response codes | Description |
| --- | --- | --- | --- | --- |
| n/a |  |  | 204 No Content | Upon success, an empty response body shall be returned. |
| NOTE: The HTTP status code for the DELETE method listed in table 5.2.7.1-1 of 3GPP TS 29.500 [4] also apply. |  |  |  |  |

### 7.2.4	Resource: PduSessionManagementData

#### 7.2.4.1	Description

This resource represents the exposure data related to session and management data for a given subscriber and a given PDU session.

#### 7.2.4.2	Resource definition

Resource URI: {apiRoot}/nudr-dr/{apiVersion}/exposure-data/{ueId}/session-management-data/{pduSessionId}

This resource shall support the resource URI variables defined in table 7.2.4.2-1.

Table 7.2.4.2-1: Resource URI variables for this resource

| Name | Definition |
| --- | --- |
| apiRoot | See 3GPP TS 29.504 [6] subclause 6.1.1. |
| apiVersion | See 3GPP TS 29.504 [6] subclause 6.1.1. |
| ueId | Represents the SUPI or GPSI, see the data type VarUeId as defined in 3GPP TS 29.571 [7]. |
| pduSessionId | Identifies an individual PDU session, see the data type PduSessionId as defined in 3GPP TS 29.571 [7]. |

#### 7.2.4.3	Resource Standard Methods

##### 7.2.4.3.1	PUT

This method shall support the URI query parameters specified in table 7.2.4.3.1-1.

Table 7.2.4.3.1-1: URI query parameters supported by the PUT method on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| n/a |  |  |  |  |

This method shall support the request data structures specified in table 7.2.4.3.1-2 and the response data structures and response codes specified in table 7.2.4.3.1-3.

Table 7.2.4.3.1-2: Data structures supported by the PUT Request Body on this resource

| Data type | P | Cardinality | Description |
| --- | --- | --- | --- |
| PduSessionManagementData | M | 1 | Session management data for a UE and a PDU session is created. |

Table 7.2.4.3.1-3: Data structures supported by the PUT Response Body on this resource

| Data type | P | Cardinality | Response Codes | Description |
| --- | --- | --- | --- | --- |
| PduSessionManagementData | M | 1 | 201 Created | Upon success, a response body is returned containing a representation of the session management data. |
| NOTE: The HTTP status code for the PUT method listed in Table 5.2.7.1-1 of 3GPP TS 29.500 [4] also apply. |  |  |  |  |

##### 7.2.4.3.2	GET

This method shall support the URI query parameters specified in table 7.2.4.3.2-1.

Table 7.2.4.3.2-1: URI query parameters supported by the GET method on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| ipv4-addr | Ipv4Addr | O | 0..1 | UE IPv4 address. |
| ipv6-prefix | Ipv6Prefix | O | 0..1 | UE IPv6 prefix. |
| dnn | Dnn | O | 0..1 | Identifies a DNN. |
| fields | array(string) | C | 1..N | When the NF consumer only retrieves a subset of the resource, the "fields" query parameter shall be included. The "fields" query parameter contains the pointers of the attribute(s) to be retrieved. |
| supp-feat | SupportedFeatures | O | 0..1 | see 3GPP TS 29.500 [4] subclause 6.6.2 and 3GPP TS 29.571 [7]. |

This method shall support the request data structures specified in table 7.2.4.3.2-2 and the response data structures and response codes specified in table 7.2.4.3.2-3.

Table 7.2.4.3.2-2: Data structures supported by the GET Request Body on this resource

| Data type | P | Cardinality | Description |
| --- | --- | --- | --- |
| n/a |  |  |  |

Table 7.2.4.3.2-3: Data structures supported by the GET Response Body on this resource

| Data type | P | Cardinality | Response Codes | Description |
| --- | --- | --- | --- | --- |
| PduSessionManagementData | M | 1 | 200 OK | The response body contains the session management data. |
| NOTE: The HTTP status code for the GET method listed in Table 5.2.7.1-1 of 3GPP TS 29.500 [4] also apply. |  |  |  |  |

##### 7.2.4.3.3	DELETE

This method shall support the URI query parameters specified in table 7.2.4.3.3-1.

Table 7.2.4.3.3-1: URI query parameters supported by the DELETE method on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| n/a |  |  |  |  |

This method shall support the request data structures specified in table 7.2.4.3.3-2 and the response data structures and response codes specified in table 7.2.4.3.3-3.

Table 7.2.4.3.3-2: Data structures supported by the DELETE Request Body on this resource

| Data type | P | Cardinality | Description |
| --- | --- | --- | --- |
| n/a |  |  |  |

Table 7.2.4.3.3-3: Data structures supported by the DELETE Response Body on this resource

| Data type | P | Cardinality | Response codes | Description |
| --- | --- | --- | --- | --- |
| n/a |  |  | 204 No Content | Upon success, an empty response body shall be returned. |
| NOTE: The HTTP status code for the DELETE method listed in table 5.2.7.1-1 of 3GPP TS 29.500 [4] also apply. |  |  |  |  |

### 7.2.5	Resource: ExposureDataSubscriptions

#### 7.2.5.1	Description

This resource is used to represent subscriptions to notifications of exposure data modifications.

#### 7.2.5.2	Resource Definition

Resource URI: {apiRoot}/nudr-dr/{apiVersion}/exposure-data/subs-to-notify

This resource shall support the resource URI variables defined in table 7.2.5.2-1.

Table 7.2.5.2-1: Resource URI variables for this resource

| Name | Definition |
| --- | --- |
| apiRoot | See 3GPP TS 29.504 [6] subclause 6.1.1. |
| apiVersion | See 3GPP TS 29.504 [6] subclause 6.1.1. |

#### 7.2.5.3	Resource Standard Methods

##### 7.2.5.3.1	POST

This method shall support the URI query parameters specified in table 7.2.5.3.1-1.

Table 7.2.5.3.1-1: URI query parameters supported by the POST method on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| n/a |  |  |  |  |

This method shall support the request data structures specified in table 7.2.5.3.1-2 and the response data structures and response codes specified in table 7.2.5.3.1-3.

Table 7.2.5.3.1-2: Data structures supported by the POST Request Body on this resource

| Data type | P | Cardinality | Description |
| --- | --- | --- | --- |
| ExposureDataSubscription | M | 1 | Identifies an individual subscription to notifications of exposure data modifications. |

Table 7.2.5.3.1-3: Data structures supported by the POST Response Body on this resource

| Data type | P | Cardinality | Response codes | Description |
| --- | --- | --- | --- | --- |
| ExposureDataSubscription | M | 1 | 201 Created | Upon success, a response body containing a representation of the Individual subscription resource shall be returned. The HTTP response shall include a "Location" HTTP header that contains the resource URI of the created resource. |
| NOTE: The mandatory HTTP error status codes for the POST method listed in table 5.2.7.1-1 of 3GPP TS 29.500 [4] also apply. |  |  |  |  |

### 7.2.6	Resource: IndividualExposureDataSubscription

#### 7.2.6.1	Description

This resource represents an individual subscription to notifications of exposure data modifications.

#### 7.2.6.2	Resource Definition

Resource URI: {apiRoot}/nudr-dr/{apiVersion}/exposure-data/subs-to-notify/{subId}

This resource shall support the resource URI variables defined in table 7.2.6.2-1.

Table 7.2.6.2-1: Resource URI variables for this resource

| Name | Definition |
| --- | --- |
| apiRoot | See 3GPP TS 29.504 [6] subclause 6.1.1. |
| apiVersion | See 3GPP TS 29.504 [6] subclause 6.1.1. |
| subId | The subId is a string that identifies an individual subscription to notifications. The value is allocated by the UDR during creation of the IndividualExposureDataSubscription resource. |

#### 7.2.6.3	Resource Standard Methods

##### 7.2.6.3.1	PUT

This method shall support the URI query parameters specified in table 7.2.6.3.1-1.

Table 7.2.6.3.1-1: URI query parameters supported by the PUT method on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| n/a |  |  |  |  |

This method shall support the request data structures specified in table 7.2.6.3.1-2 and the response data structures and response codes specified in table 7.2.6.3.1-3.

Table 7.2.6.3.1-2: Data structures supported by the PUT Request Body on this resource

| Data type | P | Cardinality | Description |
| --- | --- | --- | --- |
| ExposureDataSubscription | M | 1 | Identifies an individual subscription to notifications of exposure data modifications. |

Table 7.2.6.3.1-3: Data structures supported by the PUT Response Body on this resource

| Data type | P | Cardinality | Response codes | Description |
| --- | --- | --- | --- | --- |
| ExposureDataSubscription | M | 1 | 200 OK | The individual subscription resource was updated successfully and a body with the modified subscription to notifications about exposure data is returned. |
| n/a |  |  | 204 No Content | The individual subscription resource was successfully modified. |
| NOTE: The mandatory HTTP error status codes for the PUT method listed in Table 5.2.7.1-1 of 3GPP TS 29.500 [4] also apply. |  |  |  |  |

##### 7.2.6.3.2	DELETE

This method shall support the URI query parameters specified in table 7.2.6.3.2-1.

Table 7.2.6.3.1-1: URI query parameters supported by the DELETE method on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| n/a |  |  |  |  |

This method shall support the request data structures specified in table 7.2.6.3.2-2 and the response data structures and response codes specified in table 7.2.6.3.2-3.

Table 7.2.6.3.2-2: Data structures supported by the DELETE Request Body on this resource

| Data type | P | Cardinality | Description |
| --- | --- | --- | --- |
| n/a |  |  | The request body shall be empty. |

Table 7.2.6.3.2-3: Data structures supported by the DELETE Response Body on this resource

| Data type | P | Cardinality | Response codes | Description |
| --- | --- | --- | --- | --- |
| n/a |  |  | 204 No Content | Upon success, an empty response body shall be returned. |
| NOTE: The mandatory HTTP error status codes for the DELETE method listed in Table 5.2.7.1-1 of 3GPP TS 29.500 [4] also apply. |  |  |  |  |

## 7.3	Data Model

### 7.3.1	General

This subclause specifies the application data model supported by the API.

Table 7.3.1-1 specifies the data types defined for the Nudr_DataRepository for Exposure Data service based interface protocol.

Table 7.3.1-1: Nudr_DataRepository specific Data Types for Exposure Data

| Data type | Section defined | Description | Applicability |
| --- | --- | --- | --- |
| AccessAndMobilityData | 7.3.2.2 |  |  |
| ExposureDataChangeNotification | 7.3.2.5 | Contains changed exposure data for one UE for which Notification was requested. |  |
| ExposureDataSubscription | 7.3.2.4 | Subscription to Notifications about Exposure Data. |  |
| PduSessionManagementData | 7.3.2.3 |  |  |
| PduSessionStatus | 7.3.3.3 | Represents the state of the PDU session, i.e. active or released. |  |

Table 7.3.1-2 specifies data types re-used by the Nudr_DataRepository for Exposure Data service based interface protocol from other specifications, including a reference to their respective specifications and when needed, a short description of their use within the Nudr_DataRepository for Exposure Data service based interface.

Table 7.3.1-2: Nudr_DataRepository re-used Data Types for Exposure Data

| Data type | Reference | Comments | Applicability |
| --- | --- | --- | --- |
| AccessType | 3GPP TS 29.571 [7] | Identifies the access type, i.e. 3GPP access or Non-3GPP access. |  |
| CmInfo | 3GPP TS 29.518 [10] | Describes the connectivity management state of a UE. |  |
| DateTime | 3GPP TS 29.571 [7] |  |  |
| Dnai | 3GPP TS 29.571 [7] | Identifies a Data network access identifier. |  |
| Dnn | 3GPP TS 29.571 [7] | Identifies a Data Network Name. |  |
| Ipv4Addr | 3GPP TS 29.571 [7] | The IPv4 Address. |  |
| Ipv6Prefix | 3GPP TS 29.571 [7] | The Ipv6 Address Prefix. |  |
| PlmnId | 3GPP TS 29.571 [7] | Identifies a PLMN. |  |
| PduSessionId | 3GPP TS 29.571 [7] | PDU Session Identifier |  |
| RatType | 3GPP TS 29.571 [7] | Identifies a RAT type. |  |
| RmInfo | 3GPP TS 29.518 [10] | Describes the registration management state of a UE. |  |
| RouteToLocation | 3GPP TS 29.571 [7] | A traffic route to/from an DNAI. |  |
| TimeZone | 3GPP TS 29.571 [7] | Identifies a time zone. |  |
| SmsSupport | 3GPP TS 29.518 [10] | Indicates the supported SMS delivery of a UE. |  |
| SupportedFeatures | 3GPP TS 29.571 [7] | Used to negotiate the applicability of the optional features. |  |
| UeReachability | 3GPP TS 29.518 [10] | Describes the reachability of the UE. |  |
| Uri | 3GPP TS 29.571 [7] |  |  |
| UserLocation | 3GPP TS 29.571 [7] | Identifies the user location. |  |
| VarUeId | 3GPP TS 29.571 [7] | String represents the SUPI or GPSI. |  |

### 7.3.2	Structured data types

#### 7.3.2.1	Introduction

This subclause defines the structures to be used in resource representations.

#### 7.3.2.2	Type AccessAndMobilityData

Table 7.3.2.2-1: Definition of type AccessAndMobilityData

| Attribute name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| location | UserLocation | O | 0..1 | Current location for the UE, which can be a combination of E-UTRA, NR and non-3GPP access location. |  |
| locationTs | DateTime | O | 0..1 | Indicates when the location was last updated. |  |
| timeZone | TimeZone | O | 0..1 | Current time zone for the UE. |  |
| timeZoneTs | DateTime | O | 0..1 | Indicates when the timeZone was last updated. |  |
| accessType | AccessType | O | 0..1 | Current access type for the UE (3GPP access or non-3GPP access). |  |
| accessTypeTs | DateTime | O | 0..1 | Indicates when the accessType was last updated. |  |
| regStates | array(RmInfo) | O | 1..N | Describes the registration management state of the UE. |  |
| regStatesTs | DateTime | O | 0..1 | Indicates when the regStates were last updated. |  |
| connStates | array(CmInfo) | O | 1..N | Describes the connectivity management state of the UE. |  |
| connStatesTs | DateTime | O | 0..1 | Indicates when the connStates were last updated. |  |
| reachabilityStatus | UeReachability | O | 0..1 | Describes the reachability of the UE. |  |
| reachabilityStatusTs | DateTime | O | 0..1 | Indicates when the reachabilityStatus was last updated. |  |
| smsOverNasStatus | SmsSupport | O | 0..1 | Indicates the supported SMS delivery of the UE. |  |
| smsOverNasStatusTs | DateTime | O | 0..1 | Indicates when thesmsOverNasStatus was last updated. |  |
| roamingStatus | boolean | O | 0..1 | True: The serving PLMN of the UE is different from the HPLMN of the UE; False: The serving PLMN of the UE is the HPLMN of the UE. |  |
| roamingStatusTs | DateTime | O | 0..1 | Indicates when the roamingStatus was last updated. |  |
| currentPlmn | PlmnId | O | 0..1 | The identity of the PLMN serving the UE. |  |
| currentPlmnTs | DateTime | O | 0..1 | Indicates when the currentPlmn was last updated. |  |
| ratTypes | array(RatType) | O | 1..N | RAT type(s) |  |
| ratTypesTs | DateTime | O | 0..1 | Indicates when the ratTypes was last updated. |  |

#### 7.3.2.3	Type PduSessionManagementData

Table 7.3.2.3-1: Definition of type PduSessionManagementData

| Attribute name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| pduSessionStatus | PduSessionStatus | O | 0..1 | Status of the PDU session. |  |
| pduSessionStatusTs | DateTime | O | 0..1 | Indicates when the pduSessionStatus was last updated. |  |
| dnai | Dnai | O | 0..1 | DNAI |  |
| dnaiTs | DateTime | O | 0..1 | Indicates when the dnai was last updated. |  |
| n6TrafficRoutingInfo | array(RouteToLocation) | O | 1..N | N6 traffic routing information. |  |
| n6TrafficRoutingInfoTs | DateTime | O | 0..1 | Indicates when the n6TrafficRoutingInfo was last updated. |  |
| ipv4Addr | Ipv4Addr | O | 0..1 | UE IPv4 address. |  |
| ipv6Prefix | array(Ipv6Prefix) | O | 1..N | UE IPv6 prefix. |  |
| ipAddrTs | DateTime | O | 0..1 | Indicates when the ipv4Addr and Ipv6Prefix were last updated. |  |
| dnn | Dnn | O | 0..1 | DNN |  |
| pduSessionId | PduSessionId | O | 0..1 | The identifier of a PduSessionManagement resource. (NOTE). | ResourceNotificationExposureDataFix |
| NOTE: The "pduSessionId" attribute shall be present when the ResourceNotificationExposureDataFix is supported. |  |  |  |  |  |

#### 7.3.2.4	Type ExposureDataSubscription

Table 7.3.2.4-1: Definition of type ExposureDataSubscription

| Attribute name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| notificationUri | Uri | M | 1 | URI provided by the NF service consumer indicating where to receive to subscribed notifications from the UDR. |
| monitoredResourceUris | array(Uri) | M | 1..N | A set of URIs that identify the resources for which a modification triggers a notification. |
| expiry | DateTime | C | 0..1 | This IE shall be included in a subscription response if, based on operator policy and taking into account the expiry time included in the request, the UDR needs to include an expiry time. This IE may be included in a subscription request. When present, this IE shall represent the time after which the subscription becomes invalid. The absence of this attribute in the subscription response means the subscription to be valid without an expiry time. |
| supportedFeatures | SupportedFeatures | C | 0..1 | Used to negotiate the applicability of the optional features. This attribute shall be provided in the POST request and in the response of successful resource creation. |

#### 7.3.2.5	Type ExposureDataChangeNotification

Table 7.3.2.5-1: Definition of type ExposureDataChangeNotification

| Attribute name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| ueId | VarUeId | O | 0..1 | User identity of modified data. |  |
| accessAndMobilityData | AccessAndMobilityData | O | 0..1 | Access and Mobility Data for the UE. |  |
| pduSessionManagementData | array(PduSessionManagementData) | O | 1..N | PDU Session Management Data for one or several PDU sessions of the UE. |  |
| delResources | array(Uri) | O | 1..N | The resources, as defined in table 7.2.2-1, if removed from UDR and notification on resource data change was requested. It may be present when the ResourceNotificationExposureDataFix is supported (NOTE). |  |
| NOTE: When the "delResources" attribute is present, the attributes that correspond with the values included in the "delResources" attribute shall be omitted. I.e., when the "delResources" attribute includes the value "../{ueId}/access-and-mobility-data" the attribute "accessAndMobilityData" shall be omitted, and when the "delResources" attribute includes all the existing "../{ueId}/session-management-data/{pduSessionId}" resource URIs, the attribute "pduSessionManagementData" shall be omitted. |  |  |  |  |  |

### 7.3.3	Simple data types and enumerations

#### 7.3.3.1	Introduction

This subclause defines simple data types and enumerations that can be referenced from data structures defined in the previous subclauses.

#### 7.3.3.2	Simple data types

The simple data types defined in table 7.3.3.2-1 shall be supported.

Table 7.3.3.2-1: Simple data types

| Type Name | Type Definition | Description | Applicability |
| --- | --- | --- | --- |

#### 7.3.3.3	Enumeration: PduSessionStatus

Table 7.3.3.3-1: Enumeration PduSessionStatus

| Enumeration value | Description | Applicability |
| --- | --- | --- |
| "ACTIVE" | The PDU session is active. |  |
| "RELEASED" | The PDU session is released. |  |

## 7.4	Error handling

### 7.4.1	General

For the Nudr_DataRepository service API for Exposure Data, HTTP error responses shall be supported as specified in subclause 4.8 of 3GPP TS 29.501 [5]. Protocol errors and application errors specified in table 5.2.7.2-1 of 3GPP TS 29.500 [4] shall be supported for an HTTP method if the corresponding HTTP status codes are specified as mandatory for that HTTP method in table 5.2.7.1-1 of 3GPP TS 29.500 [4].

In addition, the requirements in the following subclauses are applicable for the Nudr_DataRepository service API for Exposure Data.

### 7.4.2	Protocol Errors

No specific procedures for the Nudr_DataRepository service API for Exposure Data are specified.

### 7.4.3	Application Errors

The application errors defined for the Nudr_DataRepository service API for Exposure Data are listed in table 7.4.3-1.

Table 7.4.3-1: Application errors

| Application Error | HTTP status code | Description |
| --- | --- | --- |

## 7.5	Feature negotiation

The optional features in table 6.1.8-1 of 3GPP TS 29.504 [6] are defined for the Nudr_DataRepository API. They shall be negotiated using the extensibility mechanism defined in subclause 6.6 of 3GPP TS 29.500 [4].

## 7.6	Notifications

### 7.6.1	General

Notifications shall comply with subclause 6.2 of 3GPP TS 29.500 [4] and subclause 4.6.2.3 of 3GPP TS 29.501 [5].

This subclause describes the resources to provide Notification to NF service consumers which have subscribed to be notified when exposure data is changed.

### 7.6.2	Exposure Data Change Notification

The POST method shall be used for Data Change Notifications and the URI shall be as provided during the subscription procedure.

Resource URI: {notificationUri}

Support of URI query parameters is specified in table 7.6.2-1.

Table 7.6.2-1: URI query parameters supported by the POST method

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| n/a |  |  |  |  |

Support of request data structures is specified in table 7.6.2-2 and of response data structures and response codes is specified in table 7.6.2-3.

Table 7.6.2-2: Data structures supported by the POST Request Body

| Data type | P | Cardinality | Description |
| --- | --- | --- | --- |
| array(ExposureDataChangeNotification) | M | 1..N | Each element in the array provides the changed exposure data for one user. |

Table7.6.2-3: Data structures supported by the POST Response Body

| Data type | P | Cardinality | Response codes | Description |
| --- | --- | --- | --- | --- |
| n/a |  |  | 204 No Content | Upon success, an empty response body shall be returned. |
| NOTE: The mandatory HTTP error status codes for the POST method listed in Table 5.2.7.1-1 of 3GPP TS 29.500 [4] also apply. |  |  |  |  |

Annex A (normative):
OpenAPI specification

# A.1	General

The present Annex contains an OpenAPI [3] specification of HTTP messages and content bodies used by the Nudr_DataRepository service for Policy Data, Application Data and Exposure Data.

This Annex shall take precedence when being discrepant to other parts of the specification with respect to the encoding of information elements and methods.

NOTE 1:	The semantics and procedures, as well as conditions, e.g. for the applicability and allowed combinations of attributes or values, not expressed in the OpenAPI definitions but defined in other parts of the specification also apply.

Informative copies of the OpenAPI specification files contained in this 3GPP Technical Specification are available on the public 3GPP file server in the following locations (see clause 5B of the 3GPP TR 21.900 [18] for further information):

-	https://www.3gpp.org/ftp/Specs/archive/OpenAPI/<Release>/, and

-	https://www.3gpp.org/ftp/Specs/<Plenary>/<Release>/OpenAPI/.

NOTE 2:	To fetch the OpenAPI specification file after CT#83 plenary meeting for Release 15 in the above links <Plenary> must be replaced with the date the CT Plenary occurs, in the form of year-month (yyyy-mm), e.g. for CT#83 meeting <Plenary> must be replaced with value "2019-03" and <Release> must be replaced with value "Rel-15".

# A.2	Nudr_DataRepository API for Policy Data

For the purpose of referencing entities in the Open API file defined in this Annex, it shall be assumed that this Open API file is contained in a physical file named "TS29519_Policy_Data.yaml".

openapi: 3.0.0

info:

version: '-'

title: Unified Data Repository Service API file for policy data

description: |

The API version is defined in 3GPP TS 29.504

© 2020, 3GPP Organizational Partners (ARIB, ATIS, CCSA, ETSI, TSDSI, TTA, TTC).

All rights reserved.

externalDocs:

description: 3GPP TS 29.519 V15.7.0; 5G System; Usage of the Unified Data Repository Service for Policy Data, Application Data and Structured Data for Exposure.

url: 'http://www.3gpp.org/ftp/Specs/archive/29_series/29.519/'

paths:

/policy-data/ues/{ueId}/am-data:

parameters:

- name: ueId

in: path

required: true

schema:

$ref: 'TS29571_CommonData.yaml#/components/schemas/VarUeId'

get:

summary: Retrieves the access and mobility policy data for a subscriber

operationId: ReadAccessAndMobilityPolicyData

tags:

- AccessAndMobilityPolicyData (Document)

responses:

'200':

description: Upon success, a response body containing access and mobility policies shall be returned.

content:

application/json:

schema:

$ref: '#/components/schemas/AmPolicyData'

'400':

$ref: 'TS29571_CommonData.yaml#/components/responses/400'

'401':

$ref: 'TS29571_CommonData.yaml#/components/responses/401'

'403':

$ref: 'TS29571_CommonData.yaml#/components/responses/403'

'404':

$ref: 'TS29571_CommonData.yaml#/components/responses/404'

'406':

$ref: 'TS29571_CommonData.yaml#/components/responses/406'

'429':

$ref: 'TS29571_CommonData.yaml#/components/responses/429'

'500':

$ref: 'TS29571_CommonData.yaml#/components/responses/500'

'503':

$ref: 'TS29571_CommonData.yaml#/components/responses/503'

default:

$ref: 'TS29571_CommonData.yaml#/components/responses/default'

/policy-data/ues/{ueId}/ue-policy-set:

parameters:

- name: ueId

in: path

required: true

schema:

$ref: 'TS29571_CommonData.yaml#/components/schemas/VarUeId'

get:

summary: Retrieves the UE policy set data for a subscriber

operationId: ReadUEPolicySet

tags:

- UEPolicySet (Document)

responses:

'200':

description: Upon success, a response body containing UE policies shall be returned.

content:

application/json:

schema:

$ref: '#/components/schemas/UePolicySet'

'400':

$ref: 'TS29571_CommonData.yaml#/components/responses/400'

'401':

$ref: 'TS29571_CommonData.yaml#/components/responses/401'

'403':

$ref: 'TS29571_CommonData.yaml#/components/responses/403'

'404':

$ref: 'TS29571_CommonData.yaml#/components/responses/404'

'406':

$ref: 'TS29571_CommonData.yaml#/components/responses/406'

'429':

$ref: 'TS29571_CommonData.yaml#/components/responses/429'

'500':

$ref: 'TS29571_CommonData.yaml#/components/responses/500'

'503':

$ref: 'TS29571_CommonData.yaml#/components/responses/503'

default:

$ref: 'TS29571_CommonData.yaml#/components/responses/default'

put:

summary: Create or modify the UE policy set data for a subscriber

operationId: CreateOrReplaceUEPolicySet

tags:

- UEPolicySet (Document)

requestBody:

content:

application/json:

schema:

$ref: '#/components/schemas/UePolicySet'

responses:

'201':

description: Successful case. The resource has been successfully created and a response body containing a representation of the created UEPolicySet resource shall be returned.

content:

application/json:

schema:

$ref: '#/components/schemas/UePolicySet'

headers:

Location:

description: 'Contains the URI of the newly created resource'

required: true

schema:

type: string

'200':

description: Successful case. The resource has been successfully created and a response body containing UE policies shall be returned.

content:

application/json:

schema:

$ref: '#/components/schemas/UePolicySet'

'204':

description: Successful case. The resource has been successfully updated and no additional content is to be sent in the response message.

'400':

$ref: 'TS29571_CommonData.yaml#/components/responses/400'

'401':

$ref: 'TS29571_CommonData.yaml#/components/responses/401'

'403':

$ref: 'TS29571_CommonData.yaml#/components/responses/403'

'404':

$ref: 'TS29571_CommonData.yaml#/components/responses/404'

'411':

$ref: 'TS29571_CommonData.yaml#/components/responses/411'

'413':

$ref: 'TS29571_CommonData.yaml#/components/responses/413'

'415':

$ref: 'TS29571_CommonData.yaml#/components/responses/415'

'429':

$ref: 'TS29571_CommonData.yaml#/components/responses/429'

'500':

$ref: 'TS29571_CommonData.yaml#/components/responses/500'

'503':

$ref: 'TS29571_CommonData.yaml#/components/responses/503'

default:

$ref: 'TS29571_CommonData.yaml#/components/responses/default'

patch:

summary: Modify the UE policy set data for a subscriber

operationId: UpdateUEPolicySet

tags:

- UEPolicySet (Document)

requestBody:

content:

application/merge-patch+json:

schema:

$ref: '#/components/schemas/UePolicySetPatch'

responses:

'204':

description: Successful case. The resource has been successfully updated and no additional content is to be sent in the response message.

'400':

$ref: 'TS29571_CommonData.yaml#/components/responses/400'

'401':

$ref: 'TS29571_CommonData.yaml#/components/responses/401'

'403':

$ref: 'TS29571_CommonData.yaml#/components/responses/403'

'404':

$ref: 'TS29571_CommonData.yaml#/components/responses/404'

'411':

$ref: 'TS29571_CommonData.yaml#/components/responses/411'

'413':

$ref: 'TS29571_CommonData.yaml#/components/responses/413'

'415':

$ref: 'TS29571_CommonData.yaml#/components/responses/415'

'429':

$ref: 'TS29571_CommonData.yaml#/components/responses/429'

'500':

$ref: 'TS29571_CommonData.yaml#/components/responses/500'

'503':

$ref: 'TS29571_CommonData.yaml#/components/responses/503'

default:

$ref: 'TS29571_CommonData.yaml#/components/responses/default'

/policy-data/ues/{ueId}/sm-data:

get:

summary: Retrieves the session management policy data for a subscriber

operationId: ReadSessionManagementPolicyData

tags:

- SessionManagementPolicyData (Document)

parameters:

- name: ueId

in: path

required: true

schema:

$ref: 'TS29571_CommonData.yaml#/components/schemas/VarUeId'

- name: snssai

in: query

required: false

content:

application/json:

schema:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Snssai'

- name: dnn

in: query

required: false

schema:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Dnn'

- name: fields

in: query

description: attributes to be retrieved

required: false

schema:

type: array

items:

type: string

minItems: 1

responses:

'200':

description: Upon success, a response body containing SmPolicyData shall be returned.

content:

application/json:

schema:

$ref: '#/components/schemas/SmPolicyData'

'400':

$ref: 'TS29571_CommonData.yaml#/components/responses/400'

'401':

$ref: 'TS29571_CommonData.yaml#/components/responses/401'

'403':

$ref: 'TS29571_CommonData.yaml#/components/responses/403'

'404':

$ref: 'TS29571_CommonData.yaml#/components/responses/404'

'406':

$ref: 'TS29571_CommonData.yaml#/components/responses/406'

'414':

$ref: 'TS29571_CommonData.yaml#/components/responses/414'

'429':

$ref: 'TS29571_CommonData.yaml#/components/responses/429'

'500':

$ref: 'TS29571_CommonData.yaml#/components/responses/500'

'503':

$ref: 'TS29571_CommonData.yaml#/components/responses/503'

default:

$ref: 'TS29571_CommonData.yaml#/components/responses/default'

/policy-data/ues/{ueId}/sm-data/{usageMonId}:

get:

summary: Retrieve a usage monitoring resource

operationId: ReadUsageMonitoringInformation

tags:

- UsageMonitoringInformation (Document)

parameters:

- name: ueId

in: path

required: true

schema:

$ref: 'TS29571_CommonData.yaml#/components/schemas/VarUeId'

- name: usageMonId

in: path

required: true

schema:

type: string

responses:

'200':

description: Successful case. The usage monitoring data is returned.

content:

application/json:

schema:

$ref: '#/components/schemas/UsageMonData'

'204':

description: The resource was found but no usage monitoring data is available.

'400':

$ref: 'TS29571_CommonData.yaml#/components/responses/400'

'401':

$ref: 'TS29571_CommonData.yaml#/components/responses/401'

'403':

$ref: 'TS29571_CommonData.yaml#/components/responses/403'

'404':

$ref: 'TS29571_CommonData.yaml#/components/responses/404'

'406':

$ref: 'TS29571_CommonData.yaml#/components/responses/406'

'414':

$ref: 'TS29571_CommonData.yaml#/components/responses/414'

'429':

$ref: 'TS29571_CommonData.yaml#/components/responses/429'

'500':

$ref: 'TS29571_CommonData.yaml#/components/responses/500'

'503':

$ref: 'TS29571_CommonData.yaml#/components/responses/503'

default:

$ref: 'TS29571_CommonData.yaml#/components/responses/default'

put:

summary: Create a usage monitoring resource

operationId: CreateUsageMonitoringResource

tags:

- UsageMonitoringInformation (Document)

parameters:

- name: ueId

in: path

required: true

schema:

$ref: 'TS29571_CommonData.yaml#/components/schemas/VarUeId'

- name: usageMonId

in: path

required: true

schema:

type: string

requestBody:

content:

application/json:

schema:

$ref: '#/components/schemas/UsageMonData'

responses:

'201':

description: Successful case. The resource has been successfully created and a response body is returned containing a representation of the resource.

content:

application/json:

schema:

$ref: '#/components/schemas/UsageMonData'

headers:

Location:

description: 'Contains the URI of the newly created resource'

required: true

schema:

type: string

'400':

$ref: 'TS29571_CommonData.yaml#/components/responses/400'

'401':

$ref: 'TS29571_CommonData.yaml#/components/responses/401'

'403':

$ref: 'TS29571_CommonData.yaml#/components/responses/403'

'404':

$ref: 'TS29571_CommonData.yaml#/components/responses/404'

'411':

$ref: 'TS29571_CommonData.yaml#/components/responses/411'

'413':

$ref: 'TS29571_CommonData.yaml#/components/responses/413'

'414':

$ref: 'TS29571_CommonData.yaml#/components/responses/414'

'415':

$ref: 'TS29571_CommonData.yaml#/components/responses/415'

'429':

$ref: 'TS29571_CommonData.yaml#/components/responses/429'

'500':

$ref: 'TS29571_CommonData.yaml#/components/responses/500'

'503':

$ref: 'TS29571_CommonData.yaml#/components/responses/503'

default:

$ref: 'TS29571_CommonData.yaml#/components/responses/default'

delete:

summary: Delete a usage monitoring resource

operationId: DeleteUsageMonitoringInformation

tags:

- UsageMonitoringInformation (Document)

parameters:

- name: ueId

in: path

required: true

schema:

$ref: 'TS29571_CommonData.yaml#/components/schemas/VarUeId'

- name: usageMonId

in: path

required: true

schema:

type: string

responses:

'204':

description: Successful case. The resource has been successfully deleted.

'400':

$ref: 'TS29571_CommonData.yaml#/components/responses/400'

'401':

$ref: 'TS29571_CommonData.yaml#/components/responses/401'

'403':

$ref: 'TS29571_CommonData.yaml#/components/responses/403'

'404':

$ref: 'TS29571_CommonData.yaml#/components/responses/404'

'429':

$ref: 'TS29571_CommonData.yaml#/components/responses/429'

'500':

$ref: 'TS29571_CommonData.yaml#/components/responses/500'

'503':

$ref: 'TS29571_CommonData.yaml#/components/responses/503'

default:

$ref: 'TS29571_CommonData.yaml#/components/responses/default'

/policy-data/sponsor-connectivity-data/{sponsorId}:

parameters:

- name: sponsorId

in: path

required: true

schema:

type: string

get:

summary: Retrieves the sponsored connectivity information for a given sponsorId

operationId: ReadSponsorConnectivityData

tags:

- SponsorConnectivityData (Document)

responses:

'200':

description: Upon success, a response body containing Sponsor Connectivity Data shall be returned.

content:

application/json:

schema:

$ref: '#/components/schemas/SponsorConnectivityData'

'204':

description: The resource was found but no Sponsor Connectivity Data is available.

'400':

$ref: 'TS29571_CommonData.yaml#/components/responses/400'

'401':

$ref: 'TS29571_CommonData.yaml#/components/responses/401'

'403':

$ref: 'TS29571_CommonData.yaml#/components/responses/403'

'404':

$ref: 'TS29571_CommonData.yaml#/components/responses/404'

'406':

$ref: 'TS29571_CommonData.yaml#/components/responses/406'

'429':

$ref: 'TS29571_CommonData.yaml#/components/responses/429'

'500':

$ref: 'TS29571_CommonData.yaml#/components/responses/500'

'503':

$ref: 'TS29571_CommonData.yaml#/components/responses/503'

default:

$ref: 'TS29571_CommonData.yaml#/components/responses/default'

/policy-data/bdt-data:

get:

summary: Retrieves the BDT data collection

operationId: ReadBdtData

tags:

- BdtData (Store)

responses:

'200':

description: Upon success, a response body containing the BDT data shall be returned.

content:

application/json:

schema:

type: array

items:

$ref: '#/components/schemas/BdtData'

'400':

$ref: 'TS29571_CommonData.yaml#/components/responses/400'

'401':

$ref: 'TS29571_CommonData.yaml#/components/responses/401'

'403':

$ref: 'TS29571_CommonData.yaml#/components/responses/403'

'404':

$ref: 'TS29571_CommonData.yaml#/components/responses/404'

'406':

$ref: 'TS29571_CommonData.yaml#/components/responses/406'

'429':

$ref: 'TS29571_CommonData.yaml#/components/responses/429'

'500':

$ref: 'TS29571_CommonData.yaml#/components/responses/500'

'503':

$ref: 'TS29571_CommonData.yaml#/components/responses/503'

default:

$ref: 'TS29571_CommonData.yaml#/components/responses/default'

/policy-data/bdt-data/{bdtReferenceId}:

parameters:

- name: bdtReferenceId

in: path

required: true

schema:

type: string

get:

summary: Retrieves the BDT data information associated with a BDT reference Id

operationId: ReadIndividualBdtData

tags:

- IndividualBdtData (Document)

responses:

'200':

description: Upon success, a response body containing the BDT data shall be returned.

content:

application/json:

schema:

$ref: '#/components/schemas/BdtData'

'400':

$ref: 'TS29571_CommonData.yaml#/components/responses/400'

'401':

$ref: 'TS29571_CommonData.yaml#/components/responses/401'

'403':

$ref: 'TS29571_CommonData.yaml#/components/responses/403'

'404':

$ref: 'TS29571_CommonData.yaml#/components/responses/404'

'406':

$ref: 'TS29571_CommonData.yaml#/components/responses/406'

'429':

$ref: 'TS29571_CommonData.yaml#/components/responses/429'

'500':

$ref: 'TS29571_CommonData.yaml#/components/responses/500'

'503':

$ref: 'TS29571_CommonData.yaml#/components/responses/503'

default:

$ref: 'TS29571_CommonData.yaml#/components/responses/default'

put:

summary: Creates an BDT data resource associated with an BDT reference Id

operationId: CreateIndividualBdtData

tags:

- IndividualBdtData (Document)

requestBody:

content:

application/json:

schema:

$ref: '#/components/schemas/BdtData'

responses:

'201':

description: Successful case. The resource has been successfully created.

content:

application/json:

schema:

$ref: '#/components/schemas/BdtData'

headers:

Location:

description: 'Contains the URI of the newly created resource'

required: true

schema:

type: string

'400':

$ref: 'TS29571_CommonData.yaml#/components/responses/400'

'401':

$ref: 'TS29571_CommonData.yaml#/components/responses/401'

'403':

$ref: 'TS29571_CommonData.yaml#/components/responses/403'

'404':

$ref: 'TS29571_CommonData.yaml#/components/responses/404'

'411':

$ref: 'TS29571_CommonData.yaml#/components/responses/411'

'413':

$ref: 'TS29571_CommonData.yaml#/components/responses/413'

'414':

$ref: 'TS29571_CommonData.yaml#/components/responses/414'

'415':

$ref: 'TS29571_CommonData.yaml#/components/responses/415'

'429':

$ref: 'TS29571_CommonData.yaml#/components/responses/429'

'500':

$ref: 'TS29571_CommonData.yaml#/components/responses/500'

'503':

$ref: 'TS29571_CommonData.yaml#/components/responses/503'

default:

$ref: 'TS29571_CommonData.yaml#/components/responses/default'

delete:

summary: Deletes an BDT data resource associated with an BDT reference Id

operationId: DeleteIndividualBdtData

tags:

- IndividualBdtData (Document)

responses:

'204':

description: Successful case. The resource has been successfully deleted.

'400':

$ref: 'TS29571_CommonData.yaml#/components/responses/400'

'401':

$ref: 'TS29571_CommonData.yaml#/components/responses/401'

'403':

$ref: 'TS29571_CommonData.yaml#/components/responses/403'

'404':

$ref: 'TS29571_CommonData.yaml#/components/responses/404'

'429':

$ref: 'TS29571_CommonData.yaml#/components/responses/429'

'500':

$ref: 'TS29571_CommonData.yaml#/components/responses/500'

'503':

$ref: 'TS29571_CommonData.yaml#/components/responses/503'

default:

$ref: 'TS29571_CommonData.yaml#/components/responses/default'

/policy-data/subs-to-notify:

post:

summary: Create a subscription to receive notification of policy data changes

operationId: CreateIndividualPolicyDataSubscription

tags:

- PolicyDataSubscriptions (Collection)

requestBody:

content:

application/json:

schema:

$ref: '#/components/schemas/PolicyDataSubscription'

responses:

'201':

description: Upon success, a response body containing a representation of each Individual subscription resource shall be returned.

content:

application/json:

schema:

$ref: '#/components/schemas/PolicyDataSubscription'

headers:

Location:

description: 'Contains the URI of the newly created resource'

required: true

schema:

type: string

'400':

$ref: 'TS29571_CommonData.yaml#/components/responses/400'

'401':

$ref: 'TS29571_CommonData.yaml#/components/responses/401'

'403':

$ref: 'TS29571_CommonData.yaml#/components/responses/403'

'404':

$ref: 'TS29571_CommonData.yaml#/components/responses/404'

'411':

$ref: 'TS29571_CommonData.yaml#/components/responses/411'

'413':

$ref: 'TS29571_CommonData.yaml#/components/responses/413'

'415':

$ref: 'TS29571_CommonData.yaml#/components/responses/415'

'429':

$ref: 'TS29571_CommonData.yaml#/components/responses/429'

'500':

$ref: 'TS29571_CommonData.yaml#/components/responses/500'

'503':

$ref: 'TS29571_CommonData.yaml#/components/responses/503'

default:

$ref: 'TS29571_CommonData.yaml#/components/responses/default'

callbacks:

policyDataChangeNotification:

'{$request.body#/notificationUri}':

post:

requestBody:

required: true

content:

application/json:

schema:

type: array

items:

$ref: '#/components/schemas/PolicyDataChangeNotification'

minItems: 1

responses:

'204':

description: No Content, Notification was successful

'400':

$ref: 'TS29571_CommonData.yaml#/components/responses/400'

'401':

$ref: 'TS29571_CommonData.yaml#/components/responses/401'

'403':

$ref: 'TS29571_CommonData.yaml#/components/responses/403'

'404':

$ref: 'TS29571_CommonData.yaml#/components/responses/404'

'411':

$ref: 'TS29571_CommonData.yaml#/components/responses/411'

'413':

$ref: 'TS29571_CommonData.yaml#/components/responses/413'

'415':

$ref: 'TS29571_CommonData.yaml#/components/responses/415'

'429':

$ref: 'TS29571_CommonData.yaml#/components/responses/429'

'500':

$ref: 'TS29571_CommonData.yaml#/components/responses/500'

'503':

$ref: 'TS29571_CommonData.yaml#/components/responses/503'

default:

$ref: 'TS29571_CommonData.yaml#/components/responses/default'

/policy-data/subs-to-notify/{subsId}:

parameters:

- name: subsId

in: path

required: true

schema:

type: string

put:

summary: Modify a subscription to receive notification of policy data changes

operationId: ReplaceIndividualPolicyDataSubscription

tags:

- IndividualPolicyDataSubscription (Document)

requestBody:

content:

application/json:

schema:

$ref: '#/components/schemas/PolicyDataSubscription'

responses:

'200':

description: The individual subscription resource was updated successfully.

content:

application/json:

schema:

$ref: '#/components/schemas/PolicyDataSubscription'

'204':

description: The individual subscription resource was updated successfully and no additional content is to be sent in the response message.

'400':

$ref: 'TS29571_CommonData.yaml#/components/responses/400'

'401':

$ref: 'TS29571_CommonData.yaml#/components/responses/401'

'403':

$ref: 'TS29571_CommonData.yaml#/components/responses/403'

'404':

$ref: 'TS29571_CommonData.yaml#/components/responses/404'

'411':

$ref: 'TS29571_CommonData.yaml#/components/responses/411'

'413':

$ref: 'TS29571_CommonData.yaml#/components/responses/413'

'415':

$ref: 'TS29571_CommonData.yaml#/components/responses/415'

'429':

$ref: 'TS29571_CommonData.yaml#/components/responses/429'

'500':

$ref: 'TS29571_CommonData.yaml#/components/responses/500'

'503':

$ref: 'TS29571_CommonData.yaml#/components/responses/503'

default:

$ref: 'TS29571_CommonData.yaml#/components/responses/default'

delete:

summary: Delete the individual Policy Data subscription

operationId: DeleteIndividualPolicyDataSubscription

tags:

- IndividualPolicyDataSubscription (Document)

responses:

'204':

description: Upon success, an empty response body shall be returned.

'400':

$ref: 'TS29571_CommonData.yaml#/components/responses/400'

'401':

$ref: 'TS29571_CommonData.yaml#/components/responses/401'

'403':

$ref: 'TS29571_CommonData.yaml#/components/responses/403'

'404':

$ref: 'TS29571_CommonData.yaml#/components/responses/404'

'429':

$ref: 'TS29571_CommonData.yaml#/components/responses/429'

'500':

$ref: 'TS29571_CommonData.yaml#/components/responses/500'

'503':

$ref: 'TS29571_CommonData.yaml#/components/responses/503'

default:

$ref: 'TS29571_CommonData.yaml#/components/responses/default'

/policy-data/ues/{ueId}/operator-specific-data:

get:

summary: Retrieve the operator specific policy data of an UE

operationId: ReadOperatorSpecificData

tags:

- OperatorSpecificData (Document)

parameters:

- name: ueId

in: path

description: UE Id

required: true

schema:

$ref: 'TS29571_CommonData.yaml#/components/schemas/VarUeId'

- name: fields

in: query

description: attributes to be retrieved

required: false

schema:

type: array

items:

type: string

minItems: 1

responses:

'200':

description: Expected response to a valid request

content:

application/json:

schema:

type: object

additionalProperties:

$ref: 'TS29505_Subscription_Data.yaml#/components/schemas/OperatorSpecificDataContainer'

'400':

$ref: 'TS29571_CommonData.yaml#/components/responses/400'

'401':

$ref: 'TS29571_CommonData.yaml#/components/responses/401'

'403':

$ref: 'TS29571_CommonData.yaml#/components/responses/403'

'404':

$ref: 'TS29571_CommonData.yaml#/components/responses/404'

'406':

$ref: 'TS29571_CommonData.yaml#/components/responses/406'

'414':

$ref: 'TS29571_CommonData.yaml#/components/responses/414'

'429':

$ref: 'TS29571_CommonData.yaml#/components/responses/429'

'500':

$ref: 'TS29571_CommonData.yaml#/components/responses/500'

'503':

$ref: 'TS29571_CommonData.yaml#/components/responses/503'

default:

$ref: 'TS29571_CommonData.yaml#/components/responses/default'

patch:

summary: Modify the operator specific policy data of an UE

operationId: UpdateOperatorSpecificData

tags:

- OperatorSpecificData (Document)

parameters:

- name: ueId

in: path

description: UE Id

required: true

schema:

$ref: 'TS29571_CommonData.yaml#/components/schemas/VarUeId'

requestBody:

content:

application/json-patch+json:

schema:

type: array

items:

$ref: 'TS29571_CommonData.yaml#/components/schemas/PatchItem'

required: true

responses:

'204':

description: No content. Response to successful modification.

'400':

$ref: 'TS29571_CommonData.yaml#/components/responses/400'

'401':

$ref: 'TS29571_CommonData.yaml#/components/responses/401'

'403':

$ref: 'TS29571_CommonData.yaml#/components/responses/403'

'404':

$ref: 'TS29571_CommonData.yaml#/components/responses/404'

'411':

$ref: 'TS29571_CommonData.yaml#/components/responses/411'

'413':

$ref: 'TS29571_CommonData.yaml#/components/responses/413'

'415':

$ref: 'TS29571_CommonData.yaml#/components/responses/415'

'429':

$ref: 'TS29571_CommonData.yaml#/components/responses/429'

'500':

$ref: 'TS29571_CommonData.yaml#/components/responses/500'

'503':

$ref: 'TS29571_CommonData.yaml#/components/responses/503'

default:

$ref: 'TS29571_CommonData.yaml#/components/responses/default'

put:

summary: Modify the operator specific policy data of an UE

operationId: ReplaceOperatorSpecificData

tags:

- OperatorSpecificData (Document)

parameters:

- name: ueId

in: path

description: UE Id

required: true

schema:

$ref: 'TS29571_CommonData.yaml#/components/schemas/VarUeId'

requestBody:

required: true

content:

application/json:

schema:

type: object

additionalProperties:

$ref: 'TS29505_Subscription_Data.yaml#/components/schemas/OperatorSpecificDataContainer'

responses:

'200':

description: OK

content:

application/json:

schema:

type: object

additionalProperties:

$ref: 'TS29505_Subscription_Data.yaml#/components/schemas/OperatorSpecificDataContainer'

'204':

description: The resource has been successfully updated.

'400':

$ref: 'TS29571_CommonData.yaml#/components/responses/400'

'401':

$ref: 'TS29571_CommonData.yaml#/components/responses/401'

'403':

$ref: 'TS29571_CommonData.yaml#/components/responses/403'

'404':

$ref: 'TS29571_CommonData.yaml#/components/responses/404'

'411':

$ref: 'TS29571_CommonData.yaml#/components/responses/411'

'413':

$ref: 'TS29571_CommonData.yaml#/components/responses/413'

'415':

$ref: 'TS29571_CommonData.yaml#/components/responses/415'

'429':

$ref: 'TS29571_CommonData.yaml#/components/responses/429'

'500':

$ref: 'TS29571_CommonData.yaml#/components/responses/500'

'503':

$ref: 'TS29571_CommonData.yaml#/components/responses/503'

default:

$ref: 'TS29571_CommonData.yaml#/components/responses/default'

/policy-data/plmns/{plmnId}/ue-policy-set:

parameters:

- name: plmnId

in: path

required: true

schema:

$ref: 'TS29505_Subscription_Data.yaml#/components/schemas/VarPlmnId'

get:

summary: Retrieve the UE policy set data for an H-PLMN

operationId: ReadPlmnUePolicySet

tags:

- PlmnUePolicySet (Document)

responses:

'200':

description: Upon success, a response body containing UE policies shall be returned.

content:

application/json:

schema:

$ref: '#/components/schemas/UePolicySet'

'400':

$ref: 'TS29571_CommonData.yaml#/components/responses/400'

'401':

$ref: 'TS29571_CommonData.yaml#/components/responses/401'

'403':

$ref: 'TS29571_CommonData.yaml#/components/responses/403'

'404':

$ref: 'TS29571_CommonData.yaml#/components/responses/404'

'406':

$ref: 'TS29571_CommonData.yaml#/components/responses/406'

'412':

$ref: 'TS29571_CommonData.yaml#/components/responses/412'

'429':

$ref: 'TS29571_CommonData.yaml#/components/responses/429'

'500':

$ref: 'TS29571_CommonData.yaml#/components/responses/500'

'503':

$ref: 'TS29571_CommonData.yaml#/components/responses/503'

default:

$ref: 'TS29571_CommonData.yaml#/components/responses/default'

components:

schemas:

AmPolicyData:

description: Contains the AM policy data for a given subscriber.

type: object

properties:

praInfos:

type: object

additionalProperties:

$ref: 'TS29571_CommonData.yaml#/components/schemas/PresenceInfo'

minProperties: 1

subscCats:

type: array

items:

type: string

minItems: 1

UePolicySet:

description: Contains the UE policy data for a given subscriber.

type: object

properties:

praInfos:

type: object

additionalProperties:

$ref: 'TS29571_CommonData.yaml#/components/schemas/PresenceInfo'

minProperties: 1

subscCats:

type: array

items:

type: string

minItems: 1

uePolicySections:

type: object

additionalProperties:

$ref: '#/components/schemas/UePolicySection'

minProperties: 1

upsis:

type: array

items:

type: string

minItems: 1

allowedRouteSelDescs:

type: object

additionalProperties:

$ref: '#/components/schemas/PlmnRouteSelectionDescriptor'

minProperties: 1

andspInd:

type: boolean

pei:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Pei'

osIds:

type: array

items:

$ref: '#/components/schemas/OsId'

minItems: 1

UePolicySetPatch:

description: Contains the UE policy set for a given subscriber.

type: object

properties:

uePolicySections:

type: object

additionalProperties:

$ref: '#/components/schemas/UePolicySection'

minProperties: 1

upsis:

type: array

items:

type: string

minItems: 1

andspInd:

type: boolean

pei:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Pei'

osIds:

type: array

items:

$ref: '#/components/schemas/OsId'

minItems: 1

UePolicySection:

description: Contains the UE policy section.

type: object

properties:

uePolicySectionInfo:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Bytes'

upsi:

type: string

required:

- uePolicySectionInfo

- upsi

SmPolicyData:

description: Contains the SM policy data for a given subscriber.

type: object

properties:

smPolicySnssaiData:

type: object

additionalProperties:

$ref: '#/components/schemas/SmPolicySnssaiData'

minProperties: 1

umDataLimits:

type: object

additionalProperties:

$ref: '#/components/schemas/UsageMonDataLimit'

minProperties: 1

umData:

type: object

additionalProperties:

$ref: '#/components/schemas/UsageMonData'

minProperties: 1

required:

- smPolicySnssaiData

SmPolicySnssaiData:

description: Contains the SM policy data for a given subscriber and S-NSSAI.

type: object

properties:

snssai:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Snssai'

smPolicyDnnData:

type: object

additionalProperties:

$ref: '#/components/schemas/SmPolicyDnnData'

minProperties: 1

required:

- snssai

SmPolicyDnnData:

description: Contains the SM policy data for a given DNN (and S-NSSAI).

type: object

properties:

dnn:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Dnn'

allowedServices:

type: array

items:

type: string

minItems: 1

subscCats:

type: array

items:

type: string

minItems: 1

gbrUl:

$ref: 'TS29571_CommonData.yaml#/components/schemas/BitRate'

gbrDl:

$ref: 'TS29571_CommonData.yaml#/components/schemas/BitRate'

adcSupport:

type: boolean

subscSpendingLimits:

type: boolean

ipv4Index:

$ref: '#/components/schemas/IpIndex'

ipv6Index:

$ref: '#/components/schemas/IpIndex'

offline:

type: boolean

online:

type: boolean

chfInfo:

$ref: 'TS29512_Npcf_SMPolicyControl.yaml#/components/schemas/ChargingInformation'

refUmDataLimitIds:

type: object

additionalProperties:

$ref: '#/components/schemas/LimitIdToMonitoringKey'

minProperties: 1

mpsPriority:

type: boolean

imsSignallingPrio:

type: boolean

mpsPriorityLevel:

type: integer

praInfos:

type: object

additionalProperties:

$ref: 'TS29571_CommonData.yaml#/components/schemas/PresenceInfo'

minProperties: 1

required:

- dnn

UsageMonDataLimit:

description: Contains usage monitoring control data for a subscriber.

type: object

properties:

limitId:

type: string

scopes:

type: object

additionalProperties:

$ref: '#/components/schemas/UsageMonDataScope'

minProperties: 1

umLevel:

$ref: '#/components/schemas/UsageMonLevel'

startDate:

$ref: 'TS29571_CommonData.yaml#/components/schemas/DateTime'

endDate:

$ref: 'TS29571_CommonData.yaml#/components/schemas/DateTime'

usageLimit:

$ref: 'TS29122_CommonData.yaml#/components/schemas/UsageThreshold'

resetPeriod:

$ref: 'TS29571_CommonData.yaml#/components/schemas/DateTime'

required:

- limitId

UsageMonData:

description: Contains remain allowed usage data for a subscriber.

type: object

properties:

limitId:

type: string

scopes:

type: object

additionalProperties:

$ref: '#/components/schemas/UsageMonDataScope'

minProperties: 1

umLevel:

$ref: '#/components/schemas/UsageMonLevel'

allowedUsage:

$ref: 'TS29122_CommonData.yaml#/components/schemas/UsageThreshold'

resetTime:

$ref: '#/components/schemas/TimePeriod'

required:

- limitId

LimitIdToMonitoringKey:

description: Contains the limit identifier and the corresponding monitoring key for a given S-NSSAI and DNN.

type: object

properties:

limitId:

type: string

monkey:

type: array

items:

type: string

minItems: 1

required:

- limitId

UsageMonDataScope:

description: Contains a SNSSAI and DNN combinations to which the UsageMonData instance belongs to.

type: object

properties:

snssai:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Snssai'

dnn:

type: array

items:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Dnn'

minItems: 1

required:

- snssai

TimePeriod:

description: Contains the periodicity for the defined usage monitoring data limits.

type: object

properties:

period:

$ref: '#/components/schemas/Periodicity'

maxNumPeriod:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Uinteger'

required:

- period

SponsorConnectivityData:

description: Contains the sponsored data connectivity related information for a sponsor identifier.

type: object

properties:

aspIds:

type: array

items:

type: string

required:

- aspIds

BdtData:

description: Contains the background data transfer data.

type: object

properties:

aspId:

type: string

transPolicy:

$ref: 'TS29554_Npcf_BDTPolicyControl.yaml#/components/schemas/TransferPolicy'

bdtRefId:

type: string

nwAreaInfo:

$ref: 'TS29554_Npcf_BDTPolicyControl.yaml#/components/schemas/NetworkAreaInfo'

numOfUes:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Uinteger'

volPerUe:

$ref: 'TS29122_CommonData.yaml#/components/schemas/UsageThreshold'

required:

- aspId

- transPolicy

PolicyDataSubscription:

description: Identifies a subscription to policy data change notification.

type: object

properties:

notificationUri:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Uri'

monitoredResourceUris:

type: array

items:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Uri'

expiry:

$ref: 'TS29571_CommonData.yaml#/components/schemas/DateTime'

supportedFeatures:

$ref: 'TS29571_CommonData.yaml#/components/schemas/SupportedFeatures'

required:

- notificationUri

- monitoredResourceUris

PolicyDataChangeNotification:

description: Contains changed policy data for which notification was requested.

type: object

properties:

amPolicyData:

$ref: '#/components/schemas/AmPolicyData'

uePolicySet:

$ref: '#/components/schemas/UePolicySet'

plmnUePolicySet:

$ref: '#/components/schemas/UePolicySet'

smPolicyData:

$ref: '#/components/schemas/SmPolicyData'

usageMonData:

$ref: '#/components/schemas/UsageMonData'

SponsorConnectivityData:

$ref: '#/components/schemas/SponsorConnectivityData'

bdtData:

$ref: '#/components/schemas/BdtData'

opSpecData:

$ref: 'TS29505_Subscription_Data.yaml#/components/schemas/OperatorSpecificDataContainer'

opSpecDataMap:

type: object

additionalProperties:

$ref: 'TS29505_Subscription_Data.yaml#/components/schemas/OperatorSpecificDataContainer'

minProperties: 1

ueId:

$ref: 'TS29571_CommonData.yaml#/components/schemas/VarUeId'

sponsorId:

type: string

bdtRefId:

type: string

usageMonId:

type: string

plmnId:

$ref: 'TS29571_CommonData.yaml#/components/schemas/PlmnId'

delResources:

type: array

items:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Uri'

minItems: 1

PlmnRouteSelectionDescriptor:

description: Contains the route selection descriptors (combinations of SNSSAI, DNNs, PDU session types, and SSC modes) allowed by subscription to the UE for a serving PLMN

type: object

properties:

servingPlmn:

$ref: 'TS29571_CommonData.yaml#/components/schemas/PlmnId'

snssaiRouteSelDescs:

type: array

items:

$ref: '#/components/schemas/SnssaiRouteSelectionDescriptor'

minItems: 1

required:

- servingPlmn

SnssaiRouteSelectionDescriptor:

description: Contains the route selector parameters (DNNs, PDU session types and SSC modes) per SNSSAI

type: object

properties:

snssai:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Snssai'

dnnRouteSelDescs:

type: array

items:

$ref: '#/components/schemas/DnnRouteSelectionDescriptor'

minItems: 1

required:

- snssai

DnnRouteSelectionDescriptor:

description: Contains the route selector parameters (PDU session types and SSC modes) per DNN

type: object

properties:

dnn:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Dnn'

sscModes:

type: array

items:

$ref: 'TS29571_CommonData.yaml#/components/schemas/SscMode'

minItems: 1

pduSessTypes:

type: array

items:

$ref: 'TS29571_CommonData.yaml#/components/schemas/PduSessionType'

minItems: 1

required:

- dnn

IpIndex:

type: integer

OsId:

type: string

format: uuid

UsageMonLevel:

anyOf:

- type: string

enum:

- SESSION_LEVEL

- SERVICE_LEVEL

- type: string

Periodicity:

anyOf:

- type: string

enum:

- YEARLY

- MONTHLY

- WEEKLY

- DAILY

- HOURLY

- type: string

# A.3	Nudr_DataRepository API for Application Data

For the purpose of referencing entities in the Open API file defined in this Annex, it shall be assumed that this Open API file is contained in a physical file named "TS29519_Application_Data.yaml".

openapi: 3.0.0

info:

version: '-'

title: Unified Data Repository Service API file for Application Data

description: |

The API version is defined in 3GPP TS 29.504

© 2020, 3GPP Organizational Partners (ARIB, ATIS, CCSA, ETSI, TSDSI, TTA, TTC).

All rights reserved.

externalDocs:

description: 3GPP TS 29.519 V15.7.0; 5G System; Usage of the Unified Data Repository Service for Policy Data, Application Data and Structured Data for Exposure.

url: 'http://www.3gpp.org/ftp/Specs/archive/29_series/29.519/'

paths:

/application-data/pfds:

get:

summary: Retrieve PFDs for application identifier(s)

operationId: ReadPFDData

tags:

- PFD Data (Store)

parameters:

- name: appId

in: query

description: Contains the information of the application identifier(s) for the querying PFD Data resource. If none appId is included in the URI, it applies to all application identifier(s) for the querying PFD Data resource.

required: false

schema:

type: array

items:

$ref: 'TS29571_CommonData.yaml#/components/schemas/ApplicationId'

minItems: 1

responses:

'200':

description: A representation of PFDs for request applications is returned.

content:

application/json:

schema:

type: array

items:

$ref: 'TS29551_Nnef_PFDmanagement.yaml#/components/schemas/PfdDataForApp'

'400':

$ref: 'TS29571_CommonData.yaml#/components/responses/400'

'401':

$ref: 'TS29571_CommonData.yaml#/components/responses/401'

'403':

$ref: 'TS29571_CommonData.yaml#/components/responses/403'

'404':

$ref: 'TS29571_CommonData.yaml#/components/responses/404'

'406':

$ref: 'TS29571_CommonData.yaml#/components/responses/406'

'414':

$ref: 'TS29571_CommonData.yaml#/components/responses/414'

'429':

$ref: 'TS29571_CommonData.yaml#/components/responses/429'

'500':

$ref: 'TS29571_CommonData.yaml#/components/responses/500'

'503':

$ref: 'TS29571_CommonData.yaml#/components/responses/503'

default:

$ref: 'TS29571_CommonData.yaml#/components/responses/default'

/application-data/pfds/{appId}:

get:

summary: Retrieve the corresponding PFDs of the specified application identifier

operationId: ReadIndividualPFDData

tags:

- Individual PFD Data (Document)

parameters:

- name: appId

in: path

description: Indicate the application identifier for the request pfd(s). It shall apply the format of Data type ApplicationId.

required: true

schema:

type: string

responses:

'200':

description: A representation of PFDs for the request application identified by the application identifier is returned.

content:

application/json:

schema:

$ref: 'TS29551_Nnef_PFDmanagement.yaml#/components/schemas/PfdDataForApp'

'400':

$ref: 'TS29571_CommonData.yaml#/components/responses/400'

'401':

$ref: 'TS29571_CommonData.yaml#/components/responses/401'

'403':

$ref: 'TS29571_CommonData.yaml#/components/responses/403'

'404':

$ref: 'TS29571_CommonData.yaml#/components/responses/404'

'406':

$ref: 'TS29571_CommonData.yaml#/components/responses/406'

'429':

$ref: 'TS29571_CommonData.yaml#/components/responses/429'

'500':

$ref: 'TS29571_CommonData.yaml#/components/responses/500'

'503':

$ref: 'TS29571_CommonData.yaml#/components/responses/503'

default:

$ref: 'TS29571_CommonData.yaml#/components/responses/default'

delete:

summary: Delete the corresponding PFDs of the specified application identifier

operationId: DeleteIndividualPFDData

tags:

- Individual PFD Data (Document)

parameters:

- name: appId

in: path

description: Indicate the application identifier for the request pfd(s). It shall apply the format of Data type ApplicationId.

required: true

schema:

type: string

responses:

'204':

description: Successful case. The Individual PFD Data resource related to the application identifier was deleted.

'400':

$ref: 'TS29571_CommonData.yaml#/components/responses/400'

'401':

$ref: 'TS29571_CommonData.yaml#/components/responses/401'

'403':

$ref: 'TS29571_CommonData.yaml#/components/responses/403'

'404':

$ref: 'TS29571_CommonData.yaml#/components/responses/404'

'429':

$ref: 'TS29571_CommonData.yaml#/components/responses/429'

'500':

$ref: 'TS29571_CommonData.yaml#/components/responses/500'

'503':

$ref: 'TS29571_CommonData.yaml#/components/responses/503'

default:

$ref: 'TS29571_CommonData.yaml#/components/responses/default'

put:

summary: Create or update the corresponding PFDs for the specified application identifier

operationId: CreateOrReplaceIndividualPFDData

tags:

- Individual PFD Data (Document)

requestBody:

required: true

content:

application/json:

schema:

$ref: 'TS29551_Nnef_PFDmanagement.yaml#/components/schemas/PfdDataForApp'

parameters:

- name: appId

in: path

description: Indicate the application identifier for the request pfd(s). It shall apply the format of Data type ApplicationId.

required: true

schema:

type: string

responses:

'201':

description: The creation of an Individual PFD Data resource related to the application-identifier is confirmed and a representation of that resource is returned.

content:

application/json:

schema:

$ref: 'TS29551_Nnef_PFDmanagement.yaml#/components/schemas/PfdDataForApp'

headers:

Location:

description: 'Contains the URI of the newly created resource, according to the structure: {apiRoot}/nudr-dr/{apiVersion}/application-data/pfds/{appId}'

required: true

schema:

type: string

'200':

description: Successful case. The upgrade of an Individual PFD Data resource related to the application identifier is confirmed and a representation of that resource is returned.

content:

application/json:

schema:

$ref: 'TS29551_Nnef_PFDmanagement.yaml#/components/schemas/PfdDataForApp'

'204':

description: No content

'400':

$ref: 'TS29571_CommonData.yaml#/components/responses/400'

'401':

$ref: 'TS29571_CommonData.yaml#/components/responses/401'

'403':

$ref: 'TS29571_CommonData.yaml#/components/responses/403'

'404':

$ref: 'TS29571_CommonData.yaml#/components/responses/404'

'411':

$ref: 'TS29571_CommonData.yaml#/components/responses/411'

'413':

$ref: 'TS29571_CommonData.yaml#/components/responses/413'

'414':

$ref: 'TS29571_CommonData.yaml#/components/responses/414'

'415':

$ref: 'TS29571_CommonData.yaml#/components/responses/415'

'429':

$ref: 'TS29571_CommonData.yaml#/components/responses/429'

'500':

$ref: 'TS29571_CommonData.yaml#/components/responses/500'

'503':

$ref: 'TS29571_CommonData.yaml#/components/responses/503'

default:

$ref: 'TS29571_CommonData.yaml#/components/responses/default'

/application-data/influenceData:

get:

summary: Retrieve Traffic Influence Data

operationId: ReadInfluenceData

tags:

- Influence Data (Store)

parameters:

- name: influence-Ids

in: query

description: Each element identifies a service.

required: false

schema:

type: array

items:

type: string

minItems: 1

- name: dnns

in: query

description: Each element identifies a DNN.

required: false

schema:

type: array

items:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Dnn'

minItems: 1

- name: snssais

in: query

description: Each element identifies a slice.

required: false

content:

application/json:

schema:

type: array

items:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Snssai'

minItems: 1

- name: internal-Group-Ids

in: query

description: Each element identifies a group of users.

required: false

schema:

type: array

items:

$ref: 'TS29571_CommonData.yaml#/components/schemas/GroupId'

minItems: 1

- name: supis

in: query

description: Each element identifies the user.

required: false

schema:

type: array

items:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Supi'

minItems: 1

responses:

'200':

description: The Traffic Influence Data stored in the UDR are returned.

content:

application/json:

schema:

type: array

items:

$ref: '#/components/schemas/TrafficInfluData'

'400':

$ref: 'TS29571_CommonData.yaml#/components/responses/400'

'401':

$ref: 'TS29571_CommonData.yaml#/components/responses/401'

'403':

$ref: 'TS29571_CommonData.yaml#/components/responses/403'

'404':

$ref: 'TS29571_CommonData.yaml#/components/responses/404'

'406':

$ref: 'TS29571_CommonData.yaml#/components/responses/406'

'414':

$ref: 'TS29571_CommonData.yaml#/components/responses/414'

'429':

$ref: 'TS29571_CommonData.yaml#/components/responses/429'

'500':

$ref: 'TS29571_CommonData.yaml#/components/responses/500'

'503':

$ref: 'TS29571_CommonData.yaml#/components/responses/503'

default:

$ref: 'TS29571_CommonData.yaml#/components/responses/default'

/application-data/influenceData/{influenceId}:

put:

summary: Create or update an individual Influence Data resource

operationId: CreateOrReplaceIndividualInfluenceData

tags:

- Individual Influence Data (Document)

requestBody:

required: true

content:

application/json:

schema:

$ref: '#/components/schemas/TrafficInfluData'

parameters:

- name: influenceId

in: path

description: The Identifier of an Individual Influence Data to be created or updated. It shall apply the format of Data type string.

required: true

schema:

type: string

responses:

'201':

description: The creation of an Individual Traffic Influence Data resource is confirmed and a representation of that resource is returned.

content:

application/json:

schema:

$ref: '#/components/schemas/TrafficInfluData'

headers:

Location:

description: 'Contains the URI of the newly created resource, according to the structure: {apiRoot}/nudr-dr/{apiVersion}/application-data/influenceData/{influenceId}'

required: true

schema:

type: string

'200':

description: The update of an Individual Traffic Influence Data resource is confirmed and a response body containing Traffic Influence Data shall be returned.

content:

application/json:

schema:

$ref: '#/components/schemas/TrafficInfluData'

'204':

description: No content

'400':

$ref: 'TS29571_CommonData.yaml#/components/responses/400'

'401':

$ref: 'TS29571_CommonData.yaml#/components/responses/401'

'403':

$ref: 'TS29571_CommonData.yaml#/components/responses/403'

'404':

$ref: 'TS29571_CommonData.yaml#/components/responses/404'

'411':

$ref: 'TS29571_CommonData.yaml#/components/responses/411'

'413':

$ref: 'TS29571_CommonData.yaml#/components/responses/413'

'414':

$ref: 'TS29571_CommonData.yaml#/components/responses/414'

'415':

$ref: 'TS29571_CommonData.yaml#/components/responses/415'

'429':

$ref: 'TS29571_CommonData.yaml#/components/responses/429'

'500':

$ref: 'TS29571_CommonData.yaml#/components/responses/500'

'503':

$ref: 'TS29571_CommonData.yaml#/components/responses/503'

default:

$ref: 'TS29571_CommonData.yaml#/components/responses/default'

patch:

summary: Modify part of the properties of an individual Influence Data resource

operationId: UpdateIndividualInfluenceData

tags:

- Individual Influence Data (Document)

requestBody:

required: true

content:

application/merge-patch+json:

schema:

$ref: '#/components/schemas/TrafficInfluDataPatch'

parameters:

- name: influenceId

in: path

description: The Identifier of an Individual Influence Data to be updated. It shall apply the format of Data type string.

required: true

schema:

type: string

responses:

'200':

description: The update of an Individual Traffic Influence Data resource is confirmed and a response body containing Traffic Influence Data shall be returned.

content:

application/json:

schema:

$ref: '#/components/schemas/TrafficInfluData'

'204':

description: No content

'400':

$ref: 'TS29571_CommonData.yaml#/components/responses/400'

'401':

$ref: 'TS29571_CommonData.yaml#/components/responses/401'

'403':

$ref: 'TS29571_CommonData.yaml#/components/responses/403'

'404':

$ref: 'TS29571_CommonData.yaml#/components/responses/404'

'411':

$ref: 'TS29571_CommonData.yaml#/components/responses/411'

'413':

$ref: 'TS29571_CommonData.yaml#/components/responses/413'

'415':

$ref: 'TS29571_CommonData.yaml#/components/responses/415'

'429':

$ref: 'TS29571_CommonData.yaml#/components/responses/429'

'500':

$ref: 'TS29571_CommonData.yaml#/components/responses/500'

'503':

$ref: 'TS29571_CommonData.yaml#/components/responses/503'

default:

$ref: 'TS29571_CommonData.yaml#/components/responses/default'

delete:

summary: Delete an individual Influence Data resource

operationId: DeleteIndividualInfluenceData

tags:

- Individual Influence Data (Document)

parameters:

- name: influenceId

in: path

description: The Identifier of an Individual Influence Data to be updated. It shall apply the format of Data type string.

required: true

schema:

type: string

responses:

'204':

description: The Individual Influence Data was deleted successfully.

'400':

$ref: 'TS29571_CommonData.yaml#/components/responses/400'

'401':

$ref: 'TS29571_CommonData.yaml#/components/responses/401'

'403':

$ref: 'TS29571_CommonData.yaml#/components/responses/403'

'404':

$ref: 'TS29571_CommonData.yaml#/components/responses/404'

'429':

$ref: 'TS29571_CommonData.yaml#/components/responses/429'

'500':

$ref: 'TS29571_CommonData.yaml#/components/responses/500'

'503':

$ref: 'TS29571_CommonData.yaml#/components/responses/503'

default:

$ref: 'TS29571_CommonData.yaml#/components/responses/default'

/application-data/influenceData/subs-to-notify:

post:

summary: Create a new Individual Influence Data Subscription resource

operationId: CreateIndividualInfluenceDataSubscription

tags:

- Influence Data Subscriptions (Collection)

requestBody:

required: true

content:

application/json:

schema:

$ref: '#/components/schemas/TrafficInfluSub'

responses:

'201':

description: The subscription was created successfully.

content:

application/json:

schema:

$ref: '#/components/schemas/TrafficInfluSub'

headers:

Location:

description: 'Contains the URI of the newly created resource'

required: true

schema:

type: string

'400':

$ref: 'TS29571_CommonData.yaml#/components/responses/400'

'401':

$ref: 'TS29571_CommonData.yaml#/components/responses/401'

'403':

$ref: 'TS29571_CommonData.yaml#/components/responses/403'

'404':

$ref: 'TS29571_CommonData.yaml#/components/responses/404'

'411':

$ref: 'TS29571_CommonData.yaml#/components/responses/411'

'413':

$ref: 'TS29571_CommonData.yaml#/components/responses/413'

'415':

$ref: 'TS29571_CommonData.yaml#/components/responses/415'

'429':

$ref: 'TS29571_CommonData.yaml#/components/responses/429'

'500':

$ref: 'TS29571_CommonData.yaml#/components/responses/500'

'503':

$ref: 'TS29571_CommonData.yaml#/components/responses/503'

default:

$ref: 'TS29571_CommonData.yaml#/components/responses/default'

callbacks:

trafficInfluenceDataChangeNotification:

'{$request.body#/notificationUri}':

post:

requestBody:

required: true

content:

application/json:

schema:

type: array

items:

$ref: '#/components/schemas/TrafficInfluData'

minItems: 1

responses:

'204':

description: No Content, Notification was successful

'400':

$ref: 'TS29571_CommonData.yaml#/components/responses/400'

'403':

$ref: 'TS29122_CommonData.yaml#/components/responses/403'

'404':

$ref: 'TS29122_CommonData.yaml#/components/responses/404'

'411':

$ref: 'TS29571_CommonData.yaml#/components/responses/411'

'413':

$ref: 'TS29571_CommonData.yaml#/components/responses/413'

'415':

$ref: 'TS29571_CommonData.yaml#/components/responses/415'

'429':

$ref: 'TS29571_CommonData.yaml#/components/responses/429'

'500':

$ref: 'TS29571_CommonData.yaml#/components/responses/500'

'503':

$ref: 'TS29571_CommonData.yaml#/components/responses/503'

default:

$ref: 'TS29571_CommonData.yaml#/components/responses/default'

get:

summary: Read Influence Data Subscriptions

operationId: ReadInfluenceDataSubscriptions

tags:

- Influence Data Subscriptions (Collection)

parameters:

- name: dnn

in: query

description: Identifies a DNN.

required: false

schema:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Dnn'

- name: snssai

in: query

description: Identifies a slice.

required: false

content:

application/json:

schema:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Snssai'

- name: internal-Group-Id

in: query

description: Identifies a group of users.

required: false

schema:

$ref: 'TS29571_CommonData.yaml#/components/schemas/GroupId'

- name: supi

in: query

description: Identifies a user.

required: false

schema:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Supi'

responses:

'200':

description: The subscription information as request in the request URI query parameter(s) are returned.

content:

application/json:

schema:

type: array

items:

$ref: '#/components/schemas/TrafficInfluSub'

minItems: 0

'400':

$ref: 'TS29571_CommonData.yaml#/components/responses/400'

'401':

$ref: 'TS29571_CommonData.yaml#/components/responses/401'

'403':

$ref: 'TS29571_CommonData.yaml#/components/responses/403'

'404':

$ref: 'TS29571_CommonData.yaml#/components/responses/404'

'406':

$ref: 'TS29571_CommonData.yaml#/components/responses/406'

'414':

$ref: 'TS29571_CommonData.yaml#/components/responses/414'

'429':

$ref: 'TS29571_CommonData.yaml#/components/responses/429'

'500':

$ref: 'TS29571_CommonData.yaml#/components/responses/500'

'503':

$ref: 'TS29571_CommonData.yaml#/components/responses/503'

default:

$ref: 'TS29571_CommonData.yaml#/components/responses/default'

/application-data/influenceData/subs-to-notify/{subscriptionId}:

get:

summary: Get an existing individual Influence Data Subscription resource

operationId: ReadIndividualInfluenceDataSubscription

tags:

- Individual Influence Data Subscription (Document)

parameters:

- name: subscriptionId

in: path

description: String identifying a subscription to the Individual Influence Data Subscription

required: true

schema:

type: string

responses:

'200':

description: The subscription information is returned.

content:

application/json:

schema:

$ref: '#/components/schemas/TrafficInfluSub'

'400':

$ref: 'TS29571_CommonData.yaml#/components/responses/400'

'401':

$ref: 'TS29571_CommonData.yaml#/components/responses/401'

'403':

$ref: 'TS29571_CommonData.yaml#/components/responses/403'

'404':

$ref: 'TS29571_CommonData.yaml#/components/responses/404'

'406':

$ref: 'TS29571_CommonData.yaml#/components/responses/406'

'414':

$ref: 'TS29571_CommonData.yaml#/components/responses/414'

'429':

$ref: 'TS29571_CommonData.yaml#/components/responses/429'

'500':

$ref: 'TS29571_CommonData.yaml#/components/responses/500'

'503':

$ref: 'TS29571_CommonData.yaml#/components/responses/503'

default:

$ref: 'TS29571_CommonData.yaml#/components/responses/default'

put:

summary: Modify an existing individual Influence Data Subscription resource

operationId: ReplaceIndividualInfluenceDataSubscription

tags:

- Individual Influence Data Subscription (Document)

requestBody:

required: true

content:

application/json:

schema:

$ref: '#/components/schemas/TrafficInfluSub'

parameters:

- name: subscriptionId

in: path

description: String identifying a subscription to the Individual Influence Data Subscription

required: true

schema:

type: string

responses:

'200':

description: The subscription was updated successfully.

content:

application/json:

schema:

$ref: '#/components/schemas/TrafficInfluSub'

'204':

description: No content

'400':

$ref: 'TS29571_CommonData.yaml#/components/responses/400'

'401':

$ref: 'TS29571_CommonData.yaml#/components/responses/401'

'403':

$ref: 'TS29571_CommonData.yaml#/components/responses/403'

'404':

$ref: 'TS29571_CommonData.yaml#/components/responses/404'

'411':

$ref: 'TS29571_CommonData.yaml#/components/responses/411'

'413':

$ref: 'TS29571_CommonData.yaml#/components/responses/413'

'415':

$ref: 'TS29571_CommonData.yaml#/components/responses/415'

'429':

$ref: 'TS29571_CommonData.yaml#/components/responses/429'

'500':

$ref: 'TS29571_CommonData.yaml#/components/responses/500'

'503':

$ref: 'TS29571_CommonData.yaml#/components/responses/503'

default:

$ref: 'TS29571_CommonData.yaml#/components/responses/default'

delete:

summary: Delete an individual Influence Data Subscription resource

operationId: DeleteIndividualInfluenceDataSubscription

tags:

- Individual Influence Data Subscription (Document)

parameters:

- name: subscriptionId

in: path

description: String identifying a subscription to the Individual Influence Data Subscription

required: true

schema:

type: string

responses:

'204':

description: The subscription was terminated successfully.

'400':

$ref: 'TS29571_CommonData.yaml#/components/responses/400'

'401':

$ref: 'TS29571_CommonData.yaml#/components/responses/401'

'403':

$ref: 'TS29571_CommonData.yaml#/components/responses/403'

'404':

$ref: 'TS29571_CommonData.yaml#/components/responses/404'

'429':

$ref: 'TS29571_CommonData.yaml#/components/responses/429'

'500':

$ref: 'TS29571_CommonData.yaml#/components/responses/500'

'503':

$ref: 'TS29571_CommonData.yaml#/components/responses/503'

default:

$ref: 'TS29571_CommonData.yaml#/components/responses/default'

components:

schemas:

TrafficInfluData:

type: object

properties:

upPathChgNotifCorreId:

type: string

description: Contains the Notification Correlation Id allocated by the NEF for the UP path change notification.

appReloInd:

type: boolean

description: Identifies whether an application can be relocated once a location of the application has been selected.

afAppId:

type: string

description: Identifies an application.

dnn:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Dnn'

ethTrafficFilters:

type: array

items:

$ref: 'TS29514_Npcf_PolicyAuthorization.yaml#/components/schemas/EthFlowDescription'

minItems: 1

description: Identifies Ethernet packet filters. Either "trafficFilters" or "ethTrafficFilters" shall be included if applicable.

snssai:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Snssai'

interGroupId:

$ref: 'TS29571_CommonData.yaml#/components/schemas/GroupId'

description: Identifies a group of users.

supi:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Supi'

trafficFilters:

type: array

items:

$ref: 'TS29122_CommonData.yaml#/components/schemas/FlowInfo'

minItems: 1

description: Identifies IP packet filters. Either "trafficFilters" or "ethTrafficFilters" shall be included if applicable.

trafficRoutes:

type: array

items:

$ref: 'TS29571_CommonData.yaml#/components/schemas/RouteToLocation'

minItems: 1

description: Identifies the N6 traffic routing requirement.

validStartTime:

$ref: 'TS29571_CommonData.yaml#/components/schemas/DateTime'

validEndTime:

$ref: 'TS29571_CommonData.yaml#/components/schemas/DateTime'

nwAreaInfo:

$ref: 'TS29554_Npcf_BDTPolicyControl.yaml#/components/schemas/NetworkAreaInfo'

upPathChgNotifUri:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Uri'

subscribedEvents:

type: array

items:

$ref: 'TS29522_TrafficInfluence.yaml#/components/schemas/SubscribedEvent'

minItems: 1

dnaiChgType:

$ref: 'TS29571_CommonData.yaml#/components/schemas/DnaiChangeType'

allOf:

- oneOf:

- required: [afAppId]

- required: [trafficFilters]

- required: [ethTrafficFilters]

- oneOf:

- required: [supi]

- required: [interGroupId]

TrafficInfluDataPatch:

type: object

properties:

upPathChgNotifCorreId:

type: string

description: Contains the Notification Correlation Id allocated by the NEF for the UP path change notification.

appReloInd:

type: boolean

description: Identifies whether an application can be relocated once a location of the application has been selected.

dnn:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Dnn'

ethTrafficFilters:

type: array

items:

$ref: 'TS29514_Npcf_PolicyAuthorization.yaml#/components/schemas/EthFlowDescription'

minItems: 1

description: Identifies Ethernet packet filters. Either "trafficFilters" or "ethTrafficFilters" shall be included if applicable.

snssai:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Snssai'

internalGroupId:

$ref: 'TS29571_CommonData.yaml#/components/schemas/GroupId'

description: Identifies a group of users.

supi:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Supi'

trafficFilters:

type: array

items:

$ref: 'TS29122_CommonData.yaml#/components/schemas/FlowInfo'

minItems: 1

description: Identifies IP packet filters. Either "trafficFilters" or "ethTrafficFilters" shall be included if applicable.

trafficRoutes:

type: array

items:

$ref: 'TS29571_CommonData.yaml#/components/schemas/RouteToLocation'

minItems: 1

description: Identifies the N6 traffic routing requirement.

validStartTime:

$ref: 'TS29571_CommonData.yaml#/components/schemas/DateTime'

validEndTime:

$ref: 'TS29571_CommonData.yaml#/components/schemas/DateTime'

nwAreaInfo:

$ref: 'TS29554_Npcf_BDTPolicyControl.yaml#/components/schemas/NetworkAreaInfo'

upPathChgNotifUri:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Uri'

TrafficInfluSub:

type: object

properties:

dnns:

type: array

items:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Dnn'

minItems: 1

description: Each element identifies a DNN.

snssais:

type: array

items:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Snssai'

minItems: 1

description: Each element identifies a slice.

internalGroupIds:

type: array

items:

$ref: 'TS29571_CommonData.yaml#/components/schemas/GroupId'

minItems: 1

description: Each element identifies a group of users.

supis:

type: array

items:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Supi'

minItems: 1

description: Each element identifies the user.

notificationUri:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Uri'

expiry:

$ref: 'TS29571_CommonData.yaml#/components/schemas/DateTime'

supportedFeatures:

$ref: 'TS29571_CommonData.yaml#/components/schemas/SupportedFeatures'

required:

- notificationUri

oneOf:

- required: [dnns]

- required: [snssais]

- required: [internalGroupIds]

- required: [supis]

# A.4	Nudr_DataRepository API for Exposure Data

For the purpose of referencing entities defined in the Open API file defined in this Annex, it shall be assumed that this Open API file is contained in a physical file termed "TS29519_Exposure_Data.yaml".

openapi: 3.0.0

info:

version: '-'

title: Unified Data Repository Service API file for structured data for exposure

description: |

The API version is defined in 3GPP TS 29.504

© 2019, 3GPP Organizational Partners (ARIB, ATIS, CCSA, ETSI, TSDSI, TTA, TTC).

All rights reserved.

externalDocs:

description: 3GPP TS 29.519 V15.4.0; 5G System; Usage of the Unified Data Repository Service for Policy Data, Application Data and Structured Data for Exposure.

url: 'http://www.3gpp.org/ftp/Specs/archive/29_series/29.519/'

paths:

/exposure-data/{ueId}/access-and-mobility-data:

put:

summary: Creates and updates the access and mobility exposure data for a UE

operationId: CreateOrReplaceAccessAndMobilityData

tags:

- AccessAndMobilityData

parameters:

- name: ueId

in: path

description: UE id

required: true

schema:

$ref: 'TS29571_CommonData.yaml#/components/schemas/VarUeId'

requestBody:

content:

application/json:

schema:

$ref: '#/components/schemas/AccessAndMobilityData'

responses:

'201':

description: Upon success, a response body is returned containing a representation of the access and mobility data

content:

application/json:

schema:

$ref: '#/components/schemas/AccessAndMobilityData'

headers:

Location:

description: 'Contains the URI of the newly created resource'

required: true

schema:

type: string

'400':

$ref: 'TS29571_CommonData.yaml#/components/responses/400'

'401':

$ref: 'TS29571_CommonData.yaml#/components/responses/401'

'403':

$ref: 'TS29571_CommonData.yaml#/components/responses/403'

'404':

$ref: 'TS29571_CommonData.yaml#/components/responses/404'

'411':

$ref: 'TS29571_CommonData.yaml#/components/responses/411'

'413':

$ref: 'TS29571_CommonData.yaml#/components/responses/413'

'415':

$ref: 'TS29571_CommonData.yaml#/components/responses/415'

'429':

$ref: 'TS29571_CommonData.yaml#/components/responses/429'

'500':

$ref: 'TS29571_CommonData.yaml#/components/responses/500'

'503':

$ref: 'TS29571_CommonData.yaml#/components/responses/503'

default:

$ref: 'TS29571_CommonData.yaml#/components/responses/default'

get:

summary: Retrieves the access and mobility exposure data for a UE

operationId: QueryAccessAndMobilityData

tags:

- AccessAndMobilityData

parameters:

- name: ueId

in: path

description: UE id

required: true

schema:

$ref: 'TS29571_CommonData.yaml#/components/schemas/VarUeId'

- name: supp-feat

in: query

description: Supported Features

schema:

$ref: 'TS29571_CommonData.yaml#/components/schemas/SupportedFeatures'

responses:

'200':

description: The response body contains the access and mobility data

content:

application/json:

schema:

$ref: '#/components/schemas/AccessAndMobilityData'

'400':

$ref: 'TS29571_CommonData.yaml#/components/responses/400'

'401':

$ref: 'TS29571_CommonData.yaml#/components/responses/401'

'403':

$ref: 'TS29571_CommonData.yaml#/components/responses/403'

'404':

$ref: 'TS29571_CommonData.yaml#/components/responses/404'

'406':

$ref: 'TS29571_CommonData.yaml#/components/responses/406'

'414':

$ref: 'TS29571_CommonData.yaml#/components/responses/414'

'429':

$ref: 'TS29571_CommonData.yaml#/components/responses/429'

'500':

$ref: 'TS29571_CommonData.yaml#/components/responses/500'

'503':

$ref: 'TS29571_CommonData.yaml#/components/responses/503'

default:

$ref: 'TS29571_CommonData.yaml#/components/responses/default'

delete:

summary: Deletes the access and mobility exposure data for a UE

operationId: DeleteAccessAndMobilityData

tags:

- AccessAndMobilityData

parameters:

- name: ueId

in: path

description: UE id

required: true

schema:

$ref: 'TS29571_CommonData.yaml#/components/schemas/VarUeId'

responses:

'204':

description: Upon success, an empty response body shall be returned

'400':

$ref: 'TS29571_CommonData.yaml#/components/responses/400'

'401':

$ref: 'TS29571_CommonData.yaml#/components/responses/401'

'403':

$ref: 'TS29571_CommonData.yaml#/components/responses/403'

'404':

$ref: 'TS29571_CommonData.yaml#/components/responses/404'

'429':

$ref: 'TS29571_CommonData.yaml#/components/responses/429'

'500':

$ref: 'TS29571_CommonData.yaml#/components/responses/500'

'503':

$ref: 'TS29571_CommonData.yaml#/components/responses/503'

default:

$ref: 'TS29571_CommonData.yaml#/components/responses/default'

/exposure-data/{ueId}/session-management-data/{pduSessionId}:

put:

summary: Creates and updates the session management data for a UE and for an individual PDU session

operationId: CreateOrReplaceSessionManagementData

tags:

- PduSessionManagementData

parameters:

- name: ueId

in: path

description: UE id

required: true

schema:

$ref: 'TS29571_CommonData.yaml#/components/schemas/VarUeId'

- name: pduSessionId

in: path

description: PDU session id

required: true

schema:

$ref: 'TS29571_CommonData.yaml#/components/schemas/PduSessionId'

requestBody:

content:

application/json:

schema:

$ref: '#/components/schemas/PduSessionManagementData'

responses:

'201':

description: Upon success, a response body is returned containing a representation of the session management data

content:

application/json:

schema:

$ref: '#/components/schemas/PduSessionManagementData'

headers:

Location:

description: 'Contains the URI of the newly created resource'

required: true

schema:

type: string

'400':

$ref: 'TS29571_CommonData.yaml#/components/responses/400'

'401':

$ref: 'TS29571_CommonData.yaml#/components/responses/401'

'403':

$ref: 'TS29571_CommonData.yaml#/components/responses/403'

'404':

$ref: 'TS29571_CommonData.yaml#/components/responses/404'

'411':

$ref: 'TS29571_CommonData.yaml#/components/responses/411'

'413':

$ref: 'TS29571_CommonData.yaml#/components/responses/413'

'415':

$ref: 'TS29571_CommonData.yaml#/components/responses/415'

'429':

$ref: 'TS29571_CommonData.yaml#/components/responses/429'

'500':

$ref: 'TS29571_CommonData.yaml#/components/responses/500'

'503':

$ref: 'TS29571_CommonData.yaml#/components/responses/503'

default:

$ref: 'TS29571_CommonData.yaml#/components/responses/default'

get:

summary: Retrieves the session management data for a UE and for an individual PDU session

operationId: QuerySessionManagementData

tags:

- PduSessionManagementData

parameters:

- name: ueId

in: path

description: UE id

required: true

schema:

$ref: 'TS29571_CommonData.yaml#/components/schemas/VarUeId'

- name: pduSessionId

in: path

description: PDU session id

required: true

schema:

$ref: 'TS29571_CommonData.yaml#/components/schemas/PduSessionId'

- name: ipv4-addr

in: query

description: IPv4 Address of the UE

schema:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Ipv4Addr'

- name: ipv6-prefix

in: query

description: IPv6 Address Prefix of the UE

schema:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Ipv6Prefix'

- name: dnn

in: query

description: DNN of the UE

schema:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Dnn'

- name: fields

in: query

description: attributes to be retrieved

required: false

schema:

type: array

items:

type: string

minItems: 1

- name: supp-feat

in: query

description: Supported Features

schema:

$ref: 'TS29571_CommonData.yaml#/components/schemas/SupportedFeatures'

responses:

'200':

description: The response body contains the session management data

content:

application/json:

schema:

$ref: '#/components/schemas/PduSessionManagementData'

'400':

$ref: 'TS29571_CommonData.yaml#/components/responses/400'

'401':

$ref: 'TS29571_CommonData.yaml#/components/responses/401'

'403':

$ref: 'TS29571_CommonData.yaml#/components/responses/403'

'404':

$ref: 'TS29571_CommonData.yaml#/components/responses/404'

'406':

$ref: 'TS29571_CommonData.yaml#/components/responses/406'

'414':

$ref: 'TS29571_CommonData.yaml#/components/responses/414'

'429':

$ref: 'TS29571_CommonData.yaml#/components/responses/429'

'500':

$ref: 'TS29571_CommonData.yaml#/components/responses/500'

'503':

$ref: 'TS29571_CommonData.yaml#/components/responses/503'

default:

$ref: 'TS29571_CommonData.yaml#/components/responses/default'

delete:

summary: Deletes the session management data for a UE and for an individual PDU session

operationId: DeleteSessionManagementData

tags:

- PduSessionManagementData

parameters:

- name: ueId

in: path

description: UE id

required: true

schema:

$ref: 'TS29571_CommonData.yaml#/components/schemas/VarUeId'

- name: pduSessionId

in: path

description: PDU session id

required: true

schema:

$ref: 'TS29571_CommonData.yaml#/components/schemas/PduSessionId'

responses:

'204':

description: Upon success, an empty response body shall be returned

'400':

$ref: 'TS29571_CommonData.yaml#/components/responses/400'

'401':

$ref: 'TS29571_CommonData.yaml#/components/responses/401'

'403':

$ref: 'TS29571_CommonData.yaml#/components/responses/403'

'404':

$ref: 'TS29571_CommonData.yaml#/components/responses/404'

'429':

$ref: 'TS29571_CommonData.yaml#/components/responses/429'

'500':

$ref: 'TS29571_CommonData.yaml#/components/responses/500'

'503':

$ref: 'TS29571_CommonData.yaml#/components/responses/503'

default:

$ref: 'TS29571_CommonData.yaml#/components/responses/default'

/exposure-data/subs-to-notify:

post:

summary: Create a subscription to receive notification of exposure data changes

operationId: CreateIndividualExposureDataSubscription

tags:

- ExposureDataSubscriptions (Collection)

requestBody:

required: true

content:

application/json:

schema:

$ref: '#/components/schemas/ExposureDataSubscription'

responses:

'201':

description: Created

content:

application/json:

schema:

$ref: '#/components/schemas/ExposureDataSubscription'

headers:

Location:

description: 'Contains the URI of the newly created resource'

required: true

schema:

type: string

'400':

$ref: 'TS29571_CommonData.yaml#/components/responses/400'

'401':

$ref: 'TS29571_CommonData.yaml#/components/responses/401'

'403':

$ref: 'TS29571_CommonData.yaml#/components/responses/403'

'404':

$ref: 'TS29571_CommonData.yaml#/components/responses/404'

'411':

$ref: 'TS29571_CommonData.yaml#/components/responses/411'

'413':

$ref: 'TS29571_CommonData.yaml#/components/responses/413'

'415':

$ref: 'TS29571_CommonData.yaml#/components/responses/415'

'429':

$ref: 'TS29571_CommonData.yaml#/components/responses/429'

'500':

$ref: 'TS29571_CommonData.yaml#/components/responses/500'

'503':

$ref: 'TS29571_CommonData.yaml#/components/responses/503'

default:

$ref: 'TS29571_CommonData.yaml#/components/responses/default'

callbacks:

exposureDataChangeNotification:

'{$request.body#/notificationUri}':

post:

requestBody:

required: true

content:

application/json:

schema:

type: array

items:

$ref: '#/components/schemas/ExposureDataChangeNotification'

minItems: 1

responses:

'204':

description: No Content, Notification was successful

'400':

$ref: 'TS29571_CommonData.yaml#/components/responses/400'

'401':

$ref: 'TS29571_CommonData.yaml#/components/responses/401'

'403':

$ref: 'TS29571_CommonData.yaml#/components/responses/403'

'404':

$ref: 'TS29571_CommonData.yaml#/components/responses/404'

'411':

$ref: 'TS29571_CommonData.yaml#/components/responses/411'

'413':

$ref: 'TS29571_CommonData.yaml#/components/responses/413'

'415':

$ref: 'TS29571_CommonData.yaml#/components/responses/415'

'429':

$ref: 'TS29571_CommonData.yaml#/components/responses/429'

'500':

$ref: 'TS29571_CommonData.yaml#/components/responses/500'

'503':

$ref: 'TS29571_CommonData.yaml#/components/responses/503'

default:

$ref: 'TS29571_CommonData.yaml#/components/responses/default'

/exposure-data/subs-to-notify/{subId}:

put:

summary: updates a subscription to receive notifications of exposure data changes

operationId: ReplaceIndividualExposureDataSubscription

tags:

- IndividualExposureDataSubscription (Document)

parameters:

- name: subId

in: path

description: Subscription id

required: true

schema:

type: string

requestBody:

required: true

content:

application/json:

schema:

$ref: '#/components/schemas/ExposureDataSubscription'

responses:

'200':

description: Resource was successfully modified and a body with the modified subscription to notifications about exposure data is returned..

content:

application/json:

schema:

$ref: '#/components/schemas/ExposureDataSubscription'

'204':

description: The individual subscription resource was successfully modified.

'400':

$ref: 'TS29571_CommonData.yaml#/components/responses/400'

'401':

$ref: 'TS29571_CommonData.yaml#/components/responses/401'

'403':

$ref: 'TS29571_CommonData.yaml#/components/responses/403'

'404':

$ref: 'TS29571_CommonData.yaml#/components/responses/404'

'411':

$ref: 'TS29571_CommonData.yaml#/components/responses/411'

'413':

$ref: 'TS29571_CommonData.yaml#/components/responses/413'

'415':

$ref: 'TS29571_CommonData.yaml#/components/responses/415'

'429':

$ref: 'TS29571_CommonData.yaml#/components/responses/429'

'500':

$ref: 'TS29571_CommonData.yaml#/components/responses/500'

'503':

$ref: 'TS29571_CommonData.yaml#/components/responses/503'

default:

$ref: 'TS29571_CommonData.yaml#/components/responses/default'

delete:

summary: Deletes the individual Exposure Data subscription

operationId: DeleteIndividualExposureDataSubscription

tags:

- IndividualExposureDataSubscription (Document)

parameters:

- name: subId

in: path

description: Subscription id

required: true

schema:

type: string

responses:

'204':

description: Resource was successfully deleted.

'400':

$ref: 'TS29571_CommonData.yaml#/components/responses/400'

'401':

$ref: 'TS29571_CommonData.yaml#/components/responses/401'

'403':

$ref: 'TS29571_CommonData.yaml#/components/responses/403'

'404':

$ref: 'TS29571_CommonData.yaml#/components/responses/404'

'429':

$ref: 'TS29571_CommonData.yaml#/components/responses/429'

'500':

$ref: 'TS29571_CommonData.yaml#/components/responses/500'

'503':

$ref: 'TS29571_CommonData.yaml#/components/responses/503'

default:

$ref: 'TS29571_CommonData.yaml#/components/responses/default'

components:

schemas:

AccessAndMobilityData:

type: object

properties:

location:

$ref: 'TS29571_CommonData.yaml#/components/schemas/UserLocation'

locationTs:

$ref: 'TS29571_CommonData.yaml#/components/schemas/DateTime'

timeZone:

$ref: 'TS29571_CommonData.yaml#/components/schemas/TimeZone'

timeZoneTs:

$ref: 'TS29571_CommonData.yaml#/components/schemas/DateTime'

accessType:

$ref: 'TS29571_CommonData.yaml#/components/schemas/AccessType'

regStates:

type: array

items:

$ref: 'TS29518_Namf_EventExposure.yaml#/components/schemas/RmInfo'

regStatesTs:

$ref: 'TS29571_CommonData.yaml#/components/schemas/DateTime'

connStates:

type: array

items:

$ref: 'TS29518_Namf_EventExposure.yaml#/components/schemas/CmInfo'

connStatesTs:

$ref: 'TS29571_CommonData.yaml#/components/schemas/DateTime'

reachabilityStatus:

$ref: 'TS29518_Namf_EventExposure.yaml#/components/schemas/UeReachability'

reachabilityStatusTs:

$ref: 'TS29571_CommonData.yaml#/components/schemas/DateTime'

smsOverNasStatus:

$ref: 'TS29518_Namf_Communication.yaml#/components/schemas/SmsSupport'

smsOverNasStatusTs:

$ref: 'TS29571_CommonData.yaml#/components/schemas/DateTime'

roamingStatus:

type: boolean

description: True  The serving PLMN of the UE is different from the HPLMN of the UE; False  The serving PLMN of the UE is the HPLMN of the UE.

roamingStatusTs:

$ref: 'TS29571_CommonData.yaml#/components/schemas/DateTime'

currentPlmn:

$ref: 'TS29571_CommonData.yaml#/components/schemas/PlmnId'

currentPlmnTs:

$ref: 'TS29571_CommonData.yaml#/components/schemas/DateTime'

ratType:

type: array

items:

$ref: 'TS29571_CommonData.yaml#/components/schemas/RatType'

ratTypesTs:

$ref: 'TS29571_CommonData.yaml#/components/schemas/DateTime'

PduSessionManagementData:

type: object

properties:

pduSessionStatus:

$ref: '#/components/schemas/PduSessionStatus'

pduSessionStatusTs:

$ref: 'TS29571_CommonData.yaml#/components/schemas/DateTime'

dnai:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Dnai'

dnaiTs:

$ref: 'TS29571_CommonData.yaml#/components/schemas/DateTime'

n6TrafficRoutingInfo:

type: array

items:

$ref: 'TS29571_CommonData.yaml#/components/schemas/RouteToLocation'

n6TrafficRoutingInfoTs:

$ref: 'TS29571_CommonData.yaml#/components/schemas/DateTime'

ipv4Addr:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Ipv4Addr'

ipv6Prefix:

type: array

items:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Ipv6Prefix'

minItems: 1

description: UE IPv6 prefix.

ipAddrTs:

$ref: 'TS29571_CommonData.yaml#/components/schemas/DateTime'

dnn:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Dnn'

pduSessionId:

$ref: 'TS29571_CommonData.yaml#/components/schemas/PduSessionId'

ExposureDataSubscription:

type: object

properties:

notificationUri:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Uri'

monitoredResourceUris:

type: array

items:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Uri'

minItems: 1

expiry:

$ref: 'TS29571_CommonData.yaml#/components/schemas/DateTime'

supportedFeatures:

$ref: 'TS29571_CommonData.yaml#/components/schemas/SupportedFeatures'

required:

- notificationUri

- monitoredResourceUris

ExposureDataChangeNotification:

type: object

properties:

ueId:

$ref: 'TS29571_CommonData.yaml#/components/schemas/VarUeId'

accessAndMobilityData:

$ref: '#/components/schemas/AccessAndMobilityData'

pduSessionManagementData:

type: array

items:

$ref: '#/components/schemas/PduSessionManagementData'

minItems: 1

delResources:

type: array

items:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Uri'

minItems: 1

PduSessionStatus:

anyOf:

- type: string

enum:

- "ACTIVE"

- "RELEASED"

- type: string

description: >

This string provides forward-compatibility with future

extensions to the enumeration but is not used to encode

content defined in the present version of this API.

description: >

Possible values are

- "ACTIVE"

- "RELEASED"

Annex B (informative):
Change history

| Change history |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Date | Meeting # | TSG Doc. | CR | Rev | Cat | Subject/Comment | New |
| 2018-01 | CT3#94 |  |  |  |  | TS skeleton of Usage of the Unified Data Repository service for Policy Data and Structured Data for exposure | 0.0.0 |
| 2018-01 | CT3#94 |  |  |  |  | Inclusion of C3-180352, C3-180326, C3-180327 | 0.1.0 |
| 2018-03 | CT3#95 |  |  |  |  | Inclusion of C3-181145, C3-181288, C3-181289, C3-181313 | 0.2.0 |
| 2018-04 | CT3#96 |  |  |  |  | Inclusion of C3-182402, C3-182476, C3-182456, C3-182404, C3-182457, C3-182458 | 0.3.0 |
| 2018-05 | CT3#97 |  |  |  |  | Inclusion of C3-183872, C3-183705, C3-183704, C3-183873, C3-183732, C3-183706 C3-183874, C3-183709, C3-183876, C3-183263, C3-183877, C3-183712, C3-183715 and editorial changes by Rapporteur | 0.4.0 |
| 2018-06 | CT#80 | CP-181033 |  |  |  | TS sent to plenary for approval | 1.0.0 |
| 2018-06 | CT#80 | CP-181033 |  |  |  | TS approved by plenary | 15.0.0 |
| 2018-09 | CT#81 | CP-182015 | 0006 |  | F | Scope and Overview | 15.1.0 |
| 2018-09 | CT#81 | CP-182015 | 0007 |  | F | Introduction subclauses 5.1, 6.1 and 7.1 | 15.1.0 |
| 2018-09 | CT#81 | CP-182015 | 0008 |  | F | Resource overview subclauses 5.2.1, 6.2.1 and 7.2.1 | 15.1.0 |
| 2018-09 | CT#81 | CP-182015 | 0009 | 3 | F | Nudr reused data types | 15.1.0 |
| 2018-09 | CT#81 | CP-182015 | 0010 | 6 | B | AM policy data resource definition | 15.1.0 |
| 2018-09 | CT#81 | CP-182015 | 0011 | 8 | B | SM policy data resource definition | 15.1.0 |
| 2018-09 | CT#81 | CP-182015 | 0012 | 3 | B | Sponsored data connectivity profiles | 15.1.0 |
| 2018-09 | CT#81 | CP-182015 | 0014 | 5 | B | Background Data Transfer data | 15.1.0 |
| 2018-09 | CT#81 | CP-182015 | 0015 | 4 | B | Policy data change subcription | 15.1.0 |
| 2018-09 | CT#81 | CP-182015 | 0016 | 5 | B | Data change notification | 15.1.0 |
| 2018-09 | CT#81 | CP-182015 | 0017 | 2 | F | Error handling subcaluses | 15.1.0 |
| 2018-09 | CT#81 | CP-182015 | 0018 | 1 | F | Feature negotiation | 15.1.0 |
| 2018-09 | CT#81 | CP-182015 | 0019 | 1 | F | Resource PFD Datas and Data types definition | 15.1.0 |
| 2018-09 | CT#81 | CP-182015 | 0020 | 1 | F | OpenAPI schema for Policy Data | 15.1.0 |
| 2018-09 | CT#81 | CP-182015 | 0021 | 3 | F | Stage 2 Allignments for Structured Data for Exposure | 15.1.0 |
| 2018-09 | CT#81 | CP-182015 | 0022 | 4 | B | Subscription for Structured Data for Exposure | 15.1.0 |
| 2018-09 | CT#81 | CP-182015 | 0023 | 3 | F | Get query parameters for structured data for exposure | 15.1.0 |
| 2018-09 | CT#81 | CP-182015 | 0024 | 5 | F | HTTP error responses for structured data for exposure | 15.1.0 |
| 2018-09 | CT#81 | CP-182015 | 0025 | 1 | F | Description of Structured data types | 15.1.0 |
| 2018-09 | CT#81 | CP-182032 | 0026 | 3 | B | Resource Structure for policy data | 15.1.0 |
| 2018-09 | CT#81 | CP-182015 | 0027 |  | F | n6 Traffic Routing Information encoding | 15.1.0 |
| 2018-09 | CT#81 | CP-182015 | 0028 | 1 | F | Removal of ExternalDocs section in OpenApi file for Exposure Data | 15.1.0 |
| 2018-09 | CT#81 | CP-182015 | 0029 | 1 | F | VarUeId used in TS 29.519 | 15.1.0 |
| 2018-09 | CT#81 | CP-182015 | 0034 | 1 | F | OpenAPI schema for PolicyData | 15.1.0 |
| 2018-09 | CT#81 | CP-182015 | 0035 | 3 | F | External information stored in UDR for Application Data | 15.1.0 |
| 2018-09 | CT#81 | CP-182015 | 0036 |  | F | Clean up the Editor Note in applicatoin data | 15.1.0 |
| 2018-09 | CT#81 | CP-182015 | 0037 | 4 | F | Correction the data type of application data | 15.1.0 |
| 2018-09 | CT#81 | CP-182015 | 0038 | 3 | B | Introduce the operator policy data | 15.1.0 |
| 2018-09 | CT#81 | CP-182015 | 0039 | 2 | B | OpenAPI of Application data | 15.1.0 |
| 2018-09 | CT#81 | CP-182015 | 0040 | 1 | F | Update of resource structure of application data 29.519 | 15.1.0 |
| 2018-09 | CT#81 | CP-182015 | 0041 | 1 | F | Ethernet packet filter for traffic influence data of applicatoin data 29.519 | 15.1.0 |
| 2018-12 | CP#82 | CP-183205 | 0042 | 1 | F | DNAI reused data | 15.2.0 |
| 2018-12 | CP#82 | CP-183205 | 0043 | 1 | F | ExternalDocs field | 15.2.0 |
| 2018-12 | CP#82 | CP-183205 | 0044 | - | F | PduSessionStatus | 15.2.0 |
| 2018-12 | CP#82 | CP-183205 | 0045 | 1 | F | Query parameters correction on application data | 15.2.0 |
| 2018-12 | CP#82 | CP-183205 | 0046 | 1 | F | PUT operation for Individual Influence Data Subscription | 15.2.0 |
| 2018-12 | CP#82 | CP-183205 | 0047 | 3 | F | Background Data Transfer Data | 15.2.0 |
| 2018-12 | CP#82 | CP-183205 | 0048 | 1 | F | Cardinality for array and map | 15.2.0 |
| 2018-12 | CP#82 | CP-183205 | 0049 | 1 | F | Resource URI for ExposureData | 15.2.0 |
| 2018-12 | CP#82 | CP-183205 | 0050 | - | F | UeId definition | 15.2.0 |
| 2018-12 | CP#82 | CP-183205 | 0052 | 1 | F | Error status codes for HTTP responses | 15.2.0 |
| 2018-12 | CP#82 | CP-183205 | 0053 | - | F | Service name | 15.2.0 |
| 2018-12 | CP#82 | CP-183205 | 0054 | 1 | F | Reference to the yaml file of Npcf_BDTPolicyControl API | 15.2.0 |
| 2018-12 | CP#82 | CP-183205 | 0055 | - | F | Missed GET method on UEPolicySetResource | 15.2.0 |
| 2018-12 | CP#82 | CP-183205 | 0055 | 2 | F | Missed GET method on UEPolicySetResource | 15.2.0 |
| 2018-12 | CP#82 | CP-183205 | 0056 | 4 | F | Corrections on AM Policy and UE Policy data | 15.2.0 |
| 2018-12 | CP#82 | CP-183205 | 0057 | 6 | F | Corrections on SM Policy Data | 15.2.0 |
| 2018-12 | CP#82 | CP-183205 | 0058 | 1 | F | Correct TrafficRoute data type | 15.2.0 |
| 2018-12 | CP#82 | CP-183205 | 0060 | 4 | F | Correct subscription for application data | 15.2.0 |
| 2018-12 | CP#82 | CP-183205 | 0062 | 3 | F | Correction to the AF Influence on traffic steering control | 15.2.0 |
| 2018-12 | CP#82 | CP-183205 | 0063 | 3 | F | PLMN ID as key for UE data sets | 15.2.0 |
| 2018-12 | CP#82 | CP-183205 | 0066 | 1 | F | Supported features | 15.2.0 |
| 2018-12 | CP#82 | CP-183205 | 0067 | 1 | F | Add AF application ID for traffic influence | 15.2.0 |
| 2018-12 | CP#82 | CP-183205 | 0068 | - | F | Correct referenced file in openAPI for application data | 15.2.0 |
| 2018-12 | CP#82 | CP-183205 | 0069 | - | F | Correct referenced file in openAPI for exposure data | 15.2.0 |
| 2018-12 | CP#82 | CP-183205 | 0070 | 1 | F | Use correct naming convention | 15.2.0 |
| 2018-12 | CP#82 | CP-183205 | 0071 | 1 | F | Correct subscription for Application Data | 15.2.0 |
| 2018-12 | CP#82 | CP-183205 | 0072 | 1 | F | Status code update for Application Data | 15.2.0 |
| 2018-12 | CP#82 | CP-183205 | 0073 | 1 | F | Correction on UsageThreshold | 15.2.0 |
| 2018-12 | CP#82 | CP-183205 | 0075 | 2 | F | Status code update for Exposure Data | 15.2.0 |
| 2018-12 | CP#82 | CP-183205 | 0076 | 2 | F | Status code update for Policy Data | 15.2.0 |
| 2018-12 | CP#82 | CP-183205 | 0077 | 2 | F | Incorrect response code of PUT method for individual PFD Data creation | 15.2.0 |
| 2018-12 | CP#82 | CP-183205 | 0078 | 1 | F | corrections on individual Influence Data | 15.2.0 |
| 2018-12 | CP#82 | CP-183205 | 0079 | 2 | F | Adding timestamp for allowed usage storage | 15.2.0 |
| 2018-12 | CP#82 | CP-183205 | 0080 | - | F | Missing 201 response body for /policy-data/subs-to-notify | 15.2.0 |
| 2018-12 | CP#82 | CP-183205 | 0081 | 1 | F | Location header | 15.2.0 |
| 2018-12 | CP#82 | CP-183205 | 0083 | 4 | F | Corrections on Usage Monitoring information | 15.2.0 |
| 2018-12 | CP#82 | CP-183205 | 0087 | - | F | Correction on PduSessionManagementData | 15.2.0 |
| 2019-03 | CP#83 | CP-190115 | 0090 | 1 | F | Correction on ChargingInformation data type reference in the OpenAPI file | 15.3.0 |
| 2019-03 | CP#83 | CP-190115 | 0091 | 1 | F | Formatting of structured data types in query parameters | 15.3.0 |
| 2019-03 | CP#83 | CP-190115 | 0092 | 1 | F | Correcting resource definitions | 15.3.0 |
| 2019-03 | CP#83 | CP-190115 | 0093 | - | F | References to data types | 15.3.0 |
| 2019-03 | CP#83 | CP-190115 | 0095 | 2 | F | Correction on Policy Data change notification | 15.3.0 |
| 2019-03 | CP#83 | CP-190115 | 0096 | 1 | F | Correction on BDT data | 15.3.0 |
| 2019-03 | CP#83 | CP-190115 | 0098 | 1 | F | Support of allowed Route Selection data | 15.3.0 |
| 2019-03 | CP#83 | CP-190115 | 0099 | 1 | F | Support of PEI, OSid and ANDSP support indication in UE Policy Set | 15.3.0 |
| 2019-03 | CP#83 | CP-190115 | 0100 | - | F | Support of PRA information | 15.3.0 |
| 2019-06 | CP#84 | CP-191077 | 0101 | - | F | Correction of OpenAPI file and main body misalignments for Notification messages | 15.4.0 |
| 2019-06 | CP#84 | CP-191077 | 0102 | 2 | F | Corrections in PolicyDataSubscription and PolicyDataChangeNotification types | 15.4.0 |
| 2019-06 | CP#84 | CP-191077 | 0103 | 4 | F | "201 Created" response without body and other HTTP response errors | 15.4.0 |
| 2019-06 | CP#84 | CP-191077 | 0106 | 1 | F | Precedence of OpenAPI file | 15.4.0 |
| 2019-06 | CP#84 | CP-191077 | 0113 | - | F | Correcting the name of URI to receive notifications | 15.4.0 |
| 2019-06 | CP#84 | CP-191077 | 0116 | 1 | F | Expiry time in subscription data types | 15.4.0 |
| 2019-06 | CP#84 | CP-191184 | 0117 | 3 | F | Notification of resource deletion | 15.4.0 |
| 2019-06 | CP#84 | CP-191208 | 0118 | 3 | F | Copyright Note in YAML file | 15.4.0 |
| 2019-06 | CP#84 | CP-191077 | 0119 | 1 | F | Missed Traffic Influence information | 15.4.0 |
| 2019-06 | CP#84 | CP-191183 | 0120 | 4 | F | Notification of resource deletion for Application and Exposure data sets | 15.4.0 |
| 2019-09 | CP#85 | CP-192145 | 0126 | 1 | F | Missed any UE indication in traffic influence data | 15.5.0 |
| 2019-09 | CP#85 | CP-192145 | 0129 | 1 | F | Remove PFD change notification | 15.5.0 |
| 2019-12 | CT#86 | CP-193187 | 0146 | 2 | F | Correction to SessionManagementPolicyData resource | 15.6.0 |
| 2019-12 | CT#86 | CP-193187 | 0154 | 2 | F | Update of TS version_Rel-15 | 15.6.0 |
| 2019-12 | CT#86 | CP-193187 | 0157 | - | F | Definition of BdtData in OpenAPI | 15.6.0 |
| 2020-06 | CT#88e | CP-201220 | 0178 | 2 | F | internalGroupId in Influence Data | 15.7.0 |
| 2020-06 | CT#88e | CP-201220 | 0186 | 1 | F | Correction to notifications of Operator Specific Data changes | 15.7.0 |
| 2020-06 | CT#88e | CP-201220 | 0187 | - | F | Correct content type in PATCHing traffic influence application data | 15.7.0 |
| 2020-06 | CT#88e | CP-201254 | 0205 | - | F | Update of TS version in externalDocs field | 15.7.0 |