3GPP TS 29.551 V17.2.0 (2021-03)

Technical Specification

3rd Generation Partnership Project;

Technical Specification Group Core Network and Terminals;

5G System; Packet Flow Description Management Service;

Stage 3

(Release 17)

The present document has been developed within the 3rd Generation Partnership Project (3GPP TM) and may be further elaborated for the purposes of 3GPP..
The present document has not been subject to any approval process by the 3GPP Organizational Partners and shall not be implemented.
This Specification is provided for future development work within 3GPP only. The Organizational Partners accept no liability for any use of this Specification.
Specifications and Reports for implementation of the 3GPP TM system should be obtained via the 3GPP Organizational Partners' Publications Offices.

Keywords

3GPP

Postal address

3GPP support office address

650 Route des Lucioles - Sophia Antipolis

Valbonne - FRANCE

Tel.: +33 4 92 94 42 00 Fax: +33 4 93 65 47 16

Internet

http://www.3gpp.org

Copyright Notification

No part may be reproduced except as authorized by written permission.
The copyright and the foregoing restriction extend to reproduction in all media.

© 2021, 3GPP Organizational Partners (ARIB, ATIS, CCSA, ETSI, TSDSI, TTA, TTC).

All rights reserved.

UMTS™ is a Trade Mark of ETSI registered for the benefit of its members

3GPP™ is a Trade Mark of ETSI registered for the benefit of its Members and of the 3GPP Organizational Partners
LTE™ is a Trade Mark of ETSI registered for the benefit of its Members and of the 3GPP Organizational Partners

GSM® and the GSM logo are registered and owned by the GSM Association

Contents

Foreword	5

1	Scope	5

2	References	5

3	Definitions, symbols and abbreviations	6

3.1	Definitions	6

3.2	Abbreviations	6

4	Packet Flow Description Management Service	6

4.1	Service Description	6

4.1.1	Overview	6

4.1.2	Service Architecture	7

4.1.3	Network Functions	7

4.1.3.1	Packet Flow Description Function (PFDF)	7

4.1.3.2	NF Service Consumers	8

4.2	Service Operations	8

4.2.1	Introduction	8

4.2.2	Nnef_PFDmanagement_Fetch Service Operation	8

4.2.2.1	General	8

4.2.2.2	Retrieval of PFDs by the full pull	9

4.2.2.3	Retrieval of PFDs by the partial pull	9

4.2.3	Nnef_PFDmanagement_Subscribe Service Operation	11

4.2.3.1	General	11

4.2.3.2	Subscription for event notifications on PFDs change	11

4.2.3.3	Subscription update for event notifications on PFDs change	11

4.2.4	Nnef_PFDmanagement_Notify Service Operation	12

4.2.4.1	General	12

4.2.4.2	Notification of PFD change	13

4.2.4.3	Notification PUSH	13

4.2.5	Nnef_PFDmanagement_Unsubscribe Service Operation	14

4.2.5.1	General	14

4.2.5.2	Unsubscribe from event notifications on PFDs change	14

5	Nnef_PFDmanagement API	15

5.1	Introduction	15

5.2	Usage of HTTP	15

5.2.1	General	15

5.2.2	HTTP standard headers	15

5.2.2.1	General	15

5.2.2.2	Content type	15

5.2.3	HTTP custom headers	15

5.2.3.1	General	15

5.3	Resources	16

5.3.1	Resource Structure	16

5.3.2	Resource: PFD of applications	17

5.3.2.1	Description	17

5.3.2.2	Resource definition	17

5.3.2.3	Resource Standard Methods	17

5.3.2.3.1	GET	17

5.3.2.4	Resource Custom Operations	18

5.3.2.4.1	Overview	18

5.3.2.4.2	Operation: Partialpull	18

5.3.2.4.2.1	Description	18

5.3.2.4.2.2	Operation Definition	18

5.3.3	Resource: Individual application PFD	18

5.3.3.1	Description	18

5.3.3.2	Resource definition	18

5.3.3.3	Resource Standard Methods	19

5.3.3.3.1	GET	19

5.3.3.4	Resource Custom Operations	19

5.3.4	Resource: PFD subscriptions	20

5.3.4.1	Description	20

5.3.4.2	Resource definition	20

5.3.4.3	Resource Standard Methods	20

5.3.4.3.1	POST	20

5.3.4.4	Resource Custom Operations	20

5.3.5	Resource: Individual PFD subscription	21

5.3.5.1	Description	21

5.3.5.2	Resource definition	21

5.3.5.3	Resource Standard Methods	21

5.3.5.3.1	DELETE	21

5.3.5.3.2	PUT	22

5.3.5.4	Resource Custom Operations	23

5.4	Custom Operations without associated resources	23

5.5	Notifications	23

5.5.1	General	23

5.5.2	PFD Change Notification	23

5.5.2.1	Description	23

5.5.2.2	Target URI	23

5.5.2.3	Standard Methods	23

5.5.2.3.1	POST	23

5.5.3	Notification Push	25

5.5.3.1	Description	25

5.5.3.2	Target URI	25

5.5.3.3	Standard Methods	25

5.5.3.3.1	POST	25

5.6	Data Model	26

5.6.1	General	26

5.6.2	Structured data types	27

5.6.2.1	Introduction	27

5.6.2.2	Type: PfdDataForApp	27

5.6.2.3	Type: PfdSubscription	27

5.6.2.4	Type: PfdChangeNotification	28

5.6.2.5	Type: PfdContent	28

5.6.2.6	Type: PfdChangeReport	29

5.6.2.7	Type: NotificationPush	29

5.6.2.8	Type: ApplicationForPfdRequest	29

5.6.3	Simple data types and enumerations	29

5.6.3.1	Introduction	29

5.6.3.2	Simple data types	29

5.6.3.3	Enumeration: PfdOperation	30

5.7	Error handling	30

5.7.1	General	30

5.7.2	Protocol Errors	30

5.7.3	Application Errors	30

5.8	Feature negotiation	31

5.9	Security	31

Annex A (normative):	OpenAPI specification	32

A.1	General	32

A.2	Nnef_PFDmanagement API	32

Annex B (informative):	Change history	42

# Foreword

This Technical Specification has been produced by the 3rd Generation Partnership Project (3GPP).

The contents of the present document are subject to continuing work within the TSG and may change following formal TSG approval. Should the TSG modify the contents of the present document, it will be re-released by the TSG with an identifying change of release date and an increase in version number as follows:

Version x.y.z

where:

x	the first digit:

1	presented to TSG for information;

2	presented to TSG for approval;

3	or greater indicates TSG approved document under change control.

y	the second digit is incremented for all changes of substance, i.e. technical enhancements, corrections, updates, etc.

z	the third digit is incremented when editorial only changes have been incorporated in the document.

# 1	Scope

The present document provides the stage 3 specification of the PFD Management Service of the 5G system.

The stage 2 definition and related procedures of the PFD Management Service are contained in 3GPP TS 23.502 [3] and 3GPP TS 23.503 [4]. The 5G System Architecture is defined in 3GPP TS 23.501 [2].

The Technical Realization of the Service Based Architecture and the Principles and Guidelines for Services Definition of the 5G System are specified in 3GPP TS 29.500 [5] and 3GPP TS 29.501 [6].

The Packet Flow Description Function (PFDF) provides the PFD Management Service to NF service consumers (e.g. Session Management Function). The PFDF is functionality within the NEF.

# 2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[2]	3GPP TS 23.501: "System Architecture for the 5G System; Stage 2".

[3]	3GPP TS 23.502: "Procedures for the 5G System; Stage 2".

[4]	3GPP TS 23.503: "Policy and Charging Control Framework for the 5G System; Stage 2".

[5]	3GPP TS 29.500: "5G System; Technical Realization of Service Based Architecture; Stage 3".

[6]	3GPP TS 29.501: "5G System; Principles and Guidelines for Services Definition; Stage 3".

[7]	IETF RFC 7540: "Hypertext Transfer Protocol Version 2 (HTTP/2)".

[8]	IETF RFC 8259: "The JavaScript Object Notation (JSON) Data Interchange Format".

[9]	OpenAPI: "OpenAPI Specification Version 3.0.0", https://spec.openapis.org/oas/v3.0.0.

[10]	3GPP TS 29.571: "5G System; Common Data Types for Service Based Interfaces Stage 3".

[11]	3GPP TS 29.122: "T8 reference point for Northbound APIs".

[12]	3GPP TS 29.251: "Gw and Gwn reference points for sponsored data connectivity".

[13]	3GPP TS 29.510: "5G System; Network Function Repository Services; Stage 3".

[14]	3GPP TS 33.501: "Security architecture and procedures for 5G system".

[15]	IETF RFC 6749: "The OAuth 2.0 Authorization Framework".

[16]	IETF RFC 7807: "Problem Details for HTTP APIs".

[17]	3GPP TR 21.900: "Technical Specification Group working methods".

# 3	Definitions, symbols and abbreviations

## 3.1	Definitions

For the purposes of the present document, the terms and definitions given in 3GPP TR 21.905 [1] and the following apply. A term defined in the present document takes precedence over the definition of the same term, if any, in 3GPP TR 21.905 [1].

## 3.2	Abbreviations

For the purposes of the present document, the abbreviations given in 3GPP TR 21.905 [1] and the following apply. An abbreviation defined in the present document takes precedence over the definition of the same abbreviation, if any, in 3GPP TR 21.905 [1].

JSON	JavaScript Object Notation

NEF	Network Exposure Function

NRF	Network Repository Function

PFD	Packet Flow Description

PFDF	Packet Flow Description Function

SBI	Service Based Interface

SMF	Session Management Function

# 4	Packet Flow Description Management Service

## 4.1	Service Description

### 4.1.1	Overview

The PFD Management Service, as defined as defined in 3GPP TS 23.501 [2], 3GPP TS 23.502 [3] and 3GPP TS 23.503 [4], is provided by the Packet Flow Description Function (PFDF).

The only known NF service consumer is the SMF.

This service:

-	allows a NF service consumer (e.g. SMF) to subscribe to and unsubscribe from PFD changes;

-	notifies a NF service consumer (e.g.SMF) about changes of PFDs;

-	notifies a NF service consumer to retrieve the PFDs; and

-	allows a NF service consumer (e.g. SMF) to retrieve PFDs.

### 4.1.2	Service Architecture

The 5G System Architecture is defined in 3GPP TS 23.501 [2]. The Policy and Charging related 5G architecture is also described in 3GPP TS 23.503 [4].

The PFD Management Service is provided by the PFDF to NF service consumers (e.g. SMF) and shown in the SBI representation model in Figure 4.1.2-1. The PFDF is a functionality within the NEF.

NEF

PFDF

Nnef

SMF

Nnef_PFDmanagement

Figure 4.1.2-1: Reference Architecture for the Nnef_PFDmanagement Service; SBI representation

NEF

PFDF

SMF

N29

Figure 4.1.2-2: Reference Architecture for the Nnef_PFDmanagement Service; reference point representation

### 4.1.3	Network Functions

#### 4.1.3.1	Packet Flow Description Function (PFDF)

The Packet Flow Description Function (PFDF):

-	provides PFDs associated with one or more Application Identifiers;

-	notifies a NF service consumer to retrieve the PFDs; and

-	allows NF service consumers to subscribe to and unsubscribe from notifications on changes of PFDs for Application Identifier.

#### 4.1.3.2	NF Service Consumers

The SMF shall support:

-	requesting and receiving the PFD(s) for one or more Application Identifiers.

## 4.2	Service Operations

### 4.2.1	Introduction

Service operations defined for the Nnef_PFDmanagement Service are shown in table 4.2.1-1.

Table 4.2.1-1: Nnef_PFDmanagement Service Operations

| Service Operation Name | Description | Initiated by |
| --- | --- | --- |
| Nnef_PFDmanagement_Fetch | Provides the PFDs for application identifier(s) to the NF service consumer by the full pull or partial pull. | NF service consumer (e.g. SMF) |
| Nnef_PFDmanagement_Subscribe | Allows NF service consumers to subscribe to notifications on events when the PFDs for application identifier(s) change. | NF service consumer (e.g. SMF) |
| Nnef_PFDmanagement_Notify | Notifies NF service consumers to update and/or delete the PFDs for application identifier(s) or notifies NF service consumer to retrieve the PFDs for application identifier(s). | PFDF |
| Nnef_PFDmanagement_Unsubscribe | Allows NF service consumers to unsubscribe from notifications on PFDs change events. | NF service consumer (e.g. SMF) |

### 4.2.2	Nnef_PFDmanagement_Fetch Service Operation

#### 4.2.2.1	General

The Nnef_PFDmanagement_Fetch service operation provides means for the NF service consumer to retrieve the PFDs for one or more application identifier(s). This service operation enables the NF service consumer to retrieve PFDs for an Application Identifier(s) from the PFDF:

-	when a PCC rule with this application identifier is provided/activated by the PCF and the PFDs provided by the PFDF are not available at the NF service consumer;

-	when the caching timer for an application identifier elapses and a PCC rule for this application identifier is still active.

When the NF service consumer removes the last PCC rule that refers to the corresponding application identifier, or when the caching timer expires and no PCC rule refers to the application identifier, the NF service consumer may remove the PFD(s) related with the application identifier.

The PFDs retrieved from PFDF take precedence over any PFDs pre-configured in the NF service consumer. If all PFDs retrieved from the PFDF are removed for an application identifier, the pre-configured PFDs shall be applied again for the application identifier.

The following procedures using the Nnef_PFDmanagement_Fetch service operation are supported:

-	Retrieval of PFDs by the full pull.

-	Retrieval of PFDs by the partial pull.

#### 4.2.2.2	Retrieval of PFDs by the full pull

This procedure, as shown in Figure 4.2.2.2-1, is used to retrieve PFDs for an application identifier from the PFDF by the full pull for requested application identifier(s).

Figure 4.2.2.2-1: Retrieval of PFDs by the full pull

1.	The NF service consumer (e.g. SMF) shall send a GET request to the resource representing the PFDs for the requested application identifier(s):

-	for PFDs of an individual application identifier, the request URI shall be set to "{apiRoot}/nnef-pfdmanagement/v1/applications/{appId}"; and

-	for PFD of a collection of application identifiers, the request URI shall be set to "{apiRoot}/nnef-pfdmanagement/v1/applications/" with query parameters indicating the requested application identifier(s).

2.	On success, an HTTP "200 OK" response shall be returned, with the payload body containing a representation of an "Individual application PFD" resource or a "PFD of applications" resource for the requested application identifier(s). The NF service consumer shall replace the stored PFD(s) retrieved from the PFDF with the new received PFD(s) for the requested application identifier(s). If the PFD(s) of one or more requested application identifier(s) are not provided in the response, the NF service consumer shall remove the PFD(s) of these requested application identifier(s) and re-apply the pre-configured PFDs.

If errors occur when processing the HTTP GET request, the PFDF shall send an HTTP error response or, if the feature "ES3XX" is supported, an HTTP redirect response as specified in subclause 5.7. For "404 Not Found", the NF service consumer shall remove the PFD(s) of the requested application identifier(s) in the NF service consumer and re-apply the pre-configured PFDs.

#### 4.2.2.3	Retrieval of PFDs by the partial pull

This procedure, as shown in Figure 4.2.2.3-1, is used to retrieve PFDs for an application identifier from the PFDF by the partial pull for requested application identifier(s) if the "PartialPull" feature defined in subclause 5.8 is supported both by NF service consumer and PFDF.

Figure 4.2.2.3-1: Retrieval of PFDs by the partial pull

1.	The NF service consumer (e.g. SMF) shall send an HTTP POST message to the resource "{apiRoot}/nnef-pfdmanagement/v1/applications/partial-pull". The NF service consumer shall include one or more ApplicationForPfdRequest data structure in the payload body of the HTTP POST. With the ApplicationForPfdRequest data structure, the NF service consumer shall include the application identifier within the "applicationId" attribute and the timestamp of the PFDs received in the last provisioning for the application identifier within the "pfdTimestamp" attribute. The NF service consumer may also request full set of PFD(s) of an application identifier without including the timestamp if it is not available.

2.	If the PFDF accepted the HTTP POST request, the PFDF shall send to the NF service consumer:

-	the HTTP "200 OK" response (as shown in figure 4.2.2.3-1, step 2a) including one or more PfdDataForApp data structure if the NF service consumer requests the PFD(s) for an application identifier(s) without the timestamp or if the NF service consumer requests PFD(s) for an application identifier(s) with timestamp and the PFDF determines that corresponding PFD(s) is changed since the last request based on the timestamp received in the request for the application identifier. Within the PfdDataForApp data type, the PFDF shall include the application identifier within the "applicationId" attribute, the new timestamp within the "pfdTimestamp" attirbute, the "partialFlag" attribute if applicable and create/update/remove the PFDs as follows:

-	include the full list of the PFD(s) within the "pfds" attribute for the application identifier which is requested without the timestamp;

-	include the full list of the PFD(s) for the application identifier if all the PFD(s) are changed for the application identifier(s) since the last request based on the timestamp;

-	for the application identifier whose the PFD(s) are partially changed:

-	include the new PFD(s) with new PFD identifier(s) within the "pfds" attribute if the new PFD(s) is added for the application identifier and the "partialFlag" attribute set to true;

-	include the new PFD(s) with existing PFD identifier(s) within the "pfds" attribute if the existing PFD(s) is updated for the application identifier and the "partialFlag" attribute set to true;

-	include the existing PFD identifier(s) without any content within the "pfds" attribute if the existing PFD(s) is removed for the application identifier and the "partialFlag" attribute set to true;

-	not include the "pfds" attribute if the PFD(s) is removed for the application identifier.

NOTE 1:	The PFDF does not include the PfdDataForApp data type for the application identifier whose PFD(s) is not updated since last request.

NOTE 2	If the PFDF determines that the PFDs are changed since the last request but cannot determine what changes have been applied to the individual PFD(s) for an application identifier, the PFDF can include the full list of the PFD(s) in the PfdDataForApp data type.

-	the HTTP "204 No Content" response (as shown in figure 4.2.2.3-1, step 2b) if the PFD(s) for all the requested application identifier(s) are not changed since last request.

When the NF service consumer receives the response with "200 OK" status code, the NF service consumer shall

-	remove the all existing PFD(s) (if available) and install all the new provided PDF(s) for an application identifier if full list of PFD(s) is received but "partialFlag" attribute is not received;

-	delete the existing PFD(s) for an application identifier(s) where no PFD(s) is received;

-	for an application identifier(s) where the PFD(s) is provided and "partialFlag" attribute is also provided and set to true:

-		install a new PFD(s) if the new PFD(s) with a new PFD identifier(s) is received;

-	update an existing PFD(s) if a new PFD(s) with the same PFD identifier(s) is received;

-	delete an existing PFD(s) if the same PFD identifier(s) without any content is received;

-	reserve an existing PFD(s) if the PFD identifier(s) is not received.

### 4.2.3	Nnef_PFDmanagement_Subscribe Service Operation

#### 4.2.3.1	General

The Nnef_PFDmanagement_Subscribe service operation enables the NF service consumer to subscribe to notifications on events when the PFDs for application identifier(s) change.

The following procedures using the Nnef_PFDmanagement_Subscribe service operation are supported:

-	Subscription for event notifications on PFDs change;

-	Subscription update for event notifications on PFD change.

#### 4.2.3.2	Subscription for event notifications on PFDs change

This procedure, as shown in Figure 4.2.3.2-1, is used to subscribe to notifications on events when the PFDs for application identifier(s) change.

Figure 4.2.3.2-1: Creation of a subscription for event notifications on PFDs change

1.	The NF service consumer (e.g. SMF) shall send a POST request to the request URI representing the collection of PFD subscriptions resource "{apiRoot}/nnef-pfdmanagement/v1/subscriptions". The request payload body shall include:

-	subscribed application identifier(s) optionally, and;

-	an URI where to receive the requested notifications as "notifyUri" attribute.

2.	If the request is accepted, the PFDF shall:

-	create a new subscription;

-	assign a subscriptionId;

-	store the subscription; and

-	send an HTTP "201 Created" response, with the payload body containing a representation of the created subscription, and the Location header containing the resource URI of the created subscription "{apiRoot}/nnef-pfdmanagement/v1/subscriptions/{subscriptionId}".

Otherwise, one of the HTTP status codes listed in table 5.3.4.3.1-3 shall be returned.

#### 4.2.3.3	Subscription update for event notifications on PFDs change

This procedure, as shown in Figure 4.2.3.3-1, is used to update an existing subscription to notifications on events when the PFDs for application identifier(s) change.

Figure 4.2.3.3-1: Update of a subscription for event notifications on PFDs change

1.	If the feature PfdChgSubsUpdate is supported, the NF service consumer (e.g. SMF) shall send a PUT request to the resource URI representing the targeted PFD subscription resource "{apiRoot}/nnef-pfdmanagement/v1/subscriptions/{subscriptionId}". The request payload body shall include:

-	subscribed application identifier(s) optionally; and;

-	an URI where to receive the requested notifications as "notifyUri" attribute.

NOTE:	The "notifUri" attribute within the PfdSubscription data structure can be modified to request that subsequent notifications are sent to a new NF service consumer.

2.	If the feature PfdChgSubsUpdate is supported and the request is accepted, the PFDF shall:

-	update the subscription; and

-	send an HTTP "200 OK" response with the payload body containing a representation of the updated subscription.

Otherwise, if errors occur when processing the HTTP PUT request, the PFDF shall send an HTTP error response or, if the feature "ES3XX" is supported, an HTTP redirect response as specified in subclause 5.7.

### 4.2.4	Nnef_PFDmanagement_Notify Service Operation

#### 4.2.4.1	General

The Nnef_PFDmanagement_Notify service operation notifies the NF service consumer to update, delete or retrieve the PFDs for application identifier(s).

The following procedures using the Nnef_PFDmanagement_Notify service operation are supported:

-	Notification of PFD change.

-	Notification PUSH

#### 4.2.4.2	Notification of PFD change

Figure 4.2.4.2-1: Notification of PFD change

1.	The PFDF shall send a POST request to the NF service consumer (e.g. SMF) targeting the URI "{notifyUri}, where {notifyUri} is the notification URI provided during the creation or modification of the subscription resource as specified in subclause 4.2.3.2. The payload body of the POST request shall contain one or more PfdChangeNotification data structure(s).

2	If the notification is accepted, the NF service consumer shall reply with:

-	"204 No Content" indicating the successful provisioning of all PFDs; or

-	"200 OK" and the payload body of the response shall contain "PfdChangeReport" data structure with detailed information of failed application(s).

Otherwise, if errors occur when processing the HTTP POST request, the NF service consumer shall send an HTTP error response or, if the feature "ES3XX" is supported, an HTTP redirect response as specified in subclause 5.7.

#### 4.2.4.3	Notification PUSH

Figure 4.2.4.3-1: Notification PUSH

1.	If the NotificationPush feature defined in subclause 5.8 is supported, and when the PFDF only notifies the NF service consumer to retrieve or remove the PFDs for application identifier(s), then the PFDF shall send a POST request to the NF service consumer (e.g. SMF) with "{notifyUri}/notifypush" as URI (where the "notifyUri" was previously supplied by the NF service consumer) and one or more NotificationPush data structure as request body. Each NotificationPush data structure shall include the application identifier(s) within the "appIds" attribute, the "pfdOp" attribute set to the applicable value and may include the "allowedDelay" attribute containing the allowed delay time if received when the "pfdOp" attribute is set to "RETRIEVE", "FULLPULL" or "PARTIALPULL".

2	If the NF service consumer accepts the received POST request, the NF service consumer shall send an HTTP "204 No Content" response.

After the successful processing of the HTTP POST request,

-	if the PFDF requests the NF service consumer to retrieve the PFD(s) with the "pfdOp" attribute set to the value "RETRIEVE", the NF service consumer shall determine to invoke the full pull procedure defined in subclause 4.2.2 or invoke the partial pull procedure defined in subclause 4.2.2.3 if the "PartialPull" feature is supported to retrieve the PFD(s) for the application identifier(s).

-	if the "PartialPull" feature is supported and if the PFDF requests the NF service consumer to retrieve the PFD(s) with the "pfdOp" attribute set to the value "FULLPULL", the NF service consumer shall invoke the pull full procedure defined in subclause 4.2.2.2.

-	if the "PartialPull" feature is supported and if the PFDF indicates the NF service consumer to retrieve the PFD(s) with the "pfdOp" attribute set to the value "PARTIALPULL", the NF service consumer may invoke the partial full procedure defined in subclause 4.2.2.3.

-	for all above cases, if the "allowedDelay" attribute is provided for one or more application(s), the NF service consumer shall retrieve the PFD within the allowed delay time, if the PFDF requests the NF service consumer to remove the PFD(s) with the "pfdOp" attribute set to the value "REMOVE", the NF service consumer shall remove the PFD(s) for the application identifier(s) and re-apply the pre-configured PFDs.

If errors occur when processing the HTTP POST request, the NF service consumer shall send an HTTP error response or, if the feature "ES3XX" is supported, an HTTP redirect response as specified in subclause 5.7.

### 4.2.5	Nnef_PFDmanagement_Unsubscribe Service Operation

#### 4.2.5.1	General

The Nnef_PFDmanagement_Unsubscribe service operation is used by the NF service consumer to unsubscribe from notifications on PFD change events.

The following procedures using the Nnef_PFDmanagement_Unsubscribe service operation are supported:

-	Unsubscribe from event notifications on PFDs change.

#### 4.2.5.2	Unsubscribe from event notifications on PFDs change

Figure 4.2.5.2-1: Unsubscribe from event notifications on PFDs change

1.	The NF service consumer (e.g. SMF) shall send a DELETE request to the resource URI representing the individual PFD subscription. The request body shall be empty.

2.	If the request is accepted, an HTTP "204 No Content" response shall be returned. The response body shall be empty.

Otherwise, if errors occur when processing the HTTP DELETE request, the PFDF consumer shall send an HTTP error response or, if the feature "ES3XX" is supported, an HTTP redirect response as specified in subclause 5.7.

# 5	Nnef_PFDmanagement API

## 5.1	Introduction

The Packet Flow Description Management Service shall use the Nnef_PFDmanagement API.

The API URI of the Nnef_PFDmanagement API shall be:

{apiRoot}/<apiName>/<apiVersion>/

The request URIs used in HTTP requests from the NF service consumer towards the PFDF shall have the Resource URI structure defined in subclause 4.4.1 of 3GPP TS 29.501 [6], i.e.:

{apiRoot}/<apiName>/<apiVersion>/<apiSpecificResourceUriPart>

with the following components:

-	The {apiRoot} shall be set as described in 3GPP TS 29.501 [6].

-	The <apiName> shall be "nnef-pfdmanagement".

-	The <apiVersion> shall be "v1".

-	The <apiSpecificResourceUriPart> shall be set as described in subclause 5.3.

## 5.2	Usage of HTTP

### 5.2.1	General

HTTP/2, IETF RFC 7540 [7], shall be used as specified in subclause 5.2 of 3GPP TS 29.500 [5].

HTTP/2 shall be transported as specified in subclause 5.3 of 3GPP TS 29.500 [5].

The OpenAPI [9] specification of HTTP messages and content bodies for the Nnef_PFDmanagement service is contained in Annex A.

### 5.2.2	HTTP standard headers

#### 5.2.2.1	General

See subclause 5.2.2 of 3GPP TS 29.500 [5] for the usage of HTTP standard headers.

#### 5.2.2.2	Content type

JSON, IETF RFC 8259 [8], shall be used as content type of the HTTP bodies specified in the present specification, as specified in subclause 5.4 of 3GPP TS 29.500 [5]. The use of the JSON format shall be signalled by the content type "application/json".

"Problem Details" JSON object shall be used to indicate additional details of the error in a HTTP response body and shall be signalled by the content type "application/problem+json", as defined in IETF RFC 7807 [16].

### 5.2.3	HTTP custom headers

#### 5.2.3.1	General

The Nnef_PFDmanagement API shall support HTTP custom header fields specified in subclause 5.2.3.2 of 3GPP TS 29.500 [4] and may support HTTP custom header fields specified in subclause 5.2.3.3 of 3GPP TS 29.500 [4].

In this Release of the specification, no specific custom headers are defined for the Nnef_PFDmanagement API.

## 5.3	Resources

### 5.3.1	Resource Structure

Figure 5.3.1-1: Resource URI structure of the Nnef_PFDmanagement API

Table 5.3.1-1 provides an overview of the resources and applicable HTTP methods.

Table 5.3.1-1: Resources and methods overview

| Resource name | Resource URI | HTTP method or custom operation | Description |
| --- | --- | --- | --- |
| PFD of applications | /applications | GET | Nnef_PFDmanagement_Fetch. Retrieve PFDs for one or multiple applications using query parameters. |
|  | /applications/partial-pull | partialpull (POST) | Request PFDs for one or multiple application identifier(s) by the partial update. |
| Individual application PFD | /applications/{appId} | GET | Nnef_PFDmanagement_Fetch. Retrieve the PFD for an application. |
| PFD subscriptions | /subscriptions | POST | Nnef_PFDmanagement_Subscribe. Subscribe the notification of PFD changes. |
| Individual PFD subscription | /subscriptions/{subscriptionId} | PUT | Update a subscription to PFD change notifications. |
| Individual PFD subscription | /subscriptions/{subscriptionId} | DELETE | Nnef_PFDmanagement_Unsubscribe. Delete a subscription to PFD change notifications. |

### 5.3.2	Resource: PFD of applications

#### 5.3.2.1	Description

This resource represents PFDs for all applications.

#### 5.3.2.2	Resource definition

Resource URI: {apiRoot}/nnef-pfdmanagement/v1/applications

This resource shall support the resource URI variables defined in table 5.3.2.2-1.

Table 5.3.2.2-1: Resource URI variables for this resource

| Name | Data type | Definition |
| --- | --- | --- |
| apiRoot | string | See subclause 5.1 |

#### 5.3.2.3	Resource Standard Methods

##### 5.3.2.3.1	GET

This method shall support the URI query parameters specified in table 5.3.2.3.1-1.

Table 5.3.2.3.1-1: URI query parameters supported by the GET method on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| application-ids | array(ApplicationId) | M | 1..N | The requested application identifier(s) for which PFD(s) shall be returned. |
| supported-features | SupportedFeatures | O | 0..1 | To filter irrelevant responses related to unsupported features. |

This method shall support the request data structures specified in table 5.3.2.3.1-2 and the response data structures and response codes specified in table 5.3.2.3.1-3.

Table 5.3.2.3.1-2: Data structures supported by the GET Request Body on this resource

| Data type | P | Cardinality | Description |
| --- | --- | --- | --- |
| n/a |  |  |  |

Table 5.3.2.3.1-3: Data structures supported by the GET Response Body on this resource

| Data type | P | Cardinality | Response codes | Description |
| --- | --- | --- | --- | --- |
| array(PfdDataForApp) | M | 0..N | 200 OK | The PFDs for one or more application identifier(s) provided in the request URI are returned. |
| NOTE: In addition, the HTTP status codes which are specified as mandatory in table 5.2.7.1-1 of 3GPP TS 29.500 [5] for the GET method shall also apply. |  |  |  |  |

#### 5.3.2.4	Resource Custom Operations

##### 5.3.2.4.1	Overview

Table 5.3.2.4.1-1: Custom operations

| Operation Name | Custom operation URI | HTTP method or custom operation | Description |
| --- | --- | --- | --- |
| Partialpull | /applications/partial-pull | partialpull (POST) | Request PFDs for one or multiple application identifier(s) by the partial update. |

##### 5.3.2.4.2	Operation: Partialpull

###### 5.3.2.4.2.1	Description

###### 5.3.2.4.2.2	Operation Definition

This custom operation requests PFDs for one ore multiple application identifier(s) by the partial update..

This operation shall support the request data structures specified in table 5.3.2.4.2.2-1 and the response data structure and response codes specified in table 5.3.2.4.2.2-2.

Table 5.3.2.4.2.2-1: Data structures supported by the POST Request Body on this resource

| Data type | P | Cardinality | Description |
| --- | --- | --- | --- |
| array(ApplicationForPfdRequest) | M | 1 | Contains the application Identifer(s) for PFDs request. |

Table 5.3.2.4.2.2-2: Data structures supported by the POST Response Body on this resource

| Data type | P | Cardinality | Response codes | Description |
| --- | --- | --- | --- | --- |
| array(PfdDataForApp) | M | 0..N | 200 OK | The PFDs for one or more application identifier(s) are returned. |
| n/a |  |  | 204 No Content |  |
| NOTE: The mandatory HTTP error status codes for the POST method listed in table 5.2.7.1-1 of 3GPP TS 29.500 [4] shall also apply. |  |  |  |  |

### 5.3.3	Resource: Individual application PFD

#### 5.3.3.1	Description

This resource represents the PFD identified by an application identifier.

#### 5.3.3.2	Resource definition

Resource URI: {apiRoot}/nnef-pfdmanagement/v1/applications/{appId}

This resource shall support the resource URI variables defined in table 5.3.3.2-1.

Table 5.3.3.2-1: Resource URI variables for this resource

| Name | Data type | Definition |
| --- | --- | --- |
| apiRoot | string | See subclause 5.1. |
| appId | string | Identifies a set of PFD for an application identifier. |

#### 5.3.3.3	Resource Standard Methods

##### 5.3.3.3.1	GET

This method shall support the URI query parameters specified in table 5.3.3.3.1-1.

Table 5.3.3.3.1-1: URI query parameters supported by the GET method on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| supported-features | SupportedFeatures | O | 0..1 | To filter irrelevant responses related to unsupported features. |

This method shall support the request data structures specified in table 5.3.3.3.1-2 and the response data structures and response codes specified in table 5.3.3.3.1-3.

Table 5.3.3.3.1-2: Data structures supported by the GET Request Body on this resource

| Data type | P | Cardinality | Description |
| --- | --- | --- | --- |
| n/a |  |  |  |

Table 5.3.3.3.1-3: Data structures supported by the GET Response Body on this resource

| Data type | P | Cardinality | Response codes | Description |
| --- | --- | --- | --- | --- |
| PfdDataForApp | M | 1 | 200 OK | A representation of PFDs for an application in the request URI is returned. |
| ProblemDetails | O | 0..1 | 307 Temporary Redirect | Temporary redirection, during Individual PFD subscription retrieval. The response shall include a Location header field containing an alternative URI of the resource located in an alternative PFDF (service) instance. Applicable if the feature "ES3XX" is supported. |
| ProblemDetails | O | 0..1 | 308 Permanent Redirect | Permanent redirection, during Individual PFD subscription retrieval. The response shall include a Location header field containing an alternative URI of the resource located in an alternative PFDF (service) instance. Applicable if the feature "ES3XX" is supported. |
| NOTE: In addition, the HTTP status codes which are specified as mandatory in table 5.2.7.1-1 of 3GPP TS 29.500 [5] for the GET method shall also apply. |  |  |  |  |

Table 5.3.3.3.1-4: Headers supported by the 307 Response Code on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| Location | string | M | 1 | An alternative URI of the resource located in an alternative PFDF (service) instance. |
| 3gpp-Sbi-Target-Nf-Id | string | O | 0..1 | Identifier of the target NF (service) instance towards which the request is redirected |

Table 5.3.3.3.1-5: Headers supported by the 308 Response Code on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| Location | string | M | 1 | An alternative URI of the resource located in an alternative PFDF (service) instance. |
| 3gpp-Sbi-Target-Nf-Id | string | O | 0..1 | Identifier of the target NF (service) instance towards which the request is redirected |

#### 5.3.3.4	Resource Custom Operations

None.

### 5.3.4	Resource: PFD subscriptions

#### 5.3.4.1	Description

This resource represents a collection of subscriptions created by NF service consumers of Nnef_PFDmanagement service.

#### 5.3.4.2	Resource definition

Resource URI: {apiRoot}/nnef-pfdmanagement/v1/subscriptions

This resource shall support the resource URI variables defined in table 5.3.4.2-1.

Table 5.3.4.2-1: Resource URI variables for this resource

| Name | Data type | Definition |
| --- | --- | --- |
| apiRoot | string | See subclause 5.1 |

#### 5.3.4.3	Resource Standard Methods

##### 5.3.4.3.1	POST

This method shall support the URI query parameters specified in table 5.3.4.3.1-1.

Table 5.3.4.3.1-1: URI query parameters supported by the POST method on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| n/a |  |  |  |  |

This method shall support the request data structures specified in table 5.3.4.3.1-2 and the response data structures and response codes specified in table 5.3.4.3.1-3.

Table 5.3.4.3.1-2: Data structures supported by the POST Request Body on this resource

| Data type | P | Cardinality | Description |
| --- | --- | --- | --- |
| PfdSubscription | M | 1 | Create a PfdSubscription resource. |

Table 5.3.4.3.1-3: Data structures supported by the POST Response Body on this resource

| Data type | P | Cardinality | Response codes | Description |
| --- | --- | --- | --- | --- |
| PfdSubscription | M | 1 | 201 Created | The creation of a PfdSubscription resource is confirmed and a representation of that resource is returned. |
| NOTE: In addition, the HTTP status codes which are specified as mandatory in table 5.2.7.1-1 of 3GPP TS 29.500 [5] for the POST method shall also apply. |  |  |  |  |

Table 5.3.4.3.1-4: Headers supported by the 201 Response Code on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| Location | string | M | 1 | Contains the URI of the newly created resource, according to the structure: {apiRoot}/nnef-pfdmanagement/v1/subscriptions/{subscriptionId} |

#### 5.3.4.4	Resource Custom Operations

None.

### 5.3.5	Resource: Individual PFD subscription

#### 5.3.5.1	Description

This resource represents an individual PFD subscription created by an NF service consumer of the Nnef_PFDmanagement service.

#### 5.3.5.2	Resource definition

Resource URI: {apiRoot}/nnef-pfdmanagement/v1/subscriptions/{subscriptionId}

This resource shall support the resource URI variables defined in table 5.3.5.2-1.

Table 5.3.5.2-1: Resource URI variables for this resource

| Name | Data type | Definition |
| --- | --- | --- |
| apiRoot | string | See subclause 5.1 |
| subscriptionId | string | Identifies an individual subscription to the PFD management service |

#### 5.3.5.3	Resource Standard Methods

##### 5.3.5.3.1	DELETE

This method shall support the URI query parameters specified in table 5.3.5.3.1-1.

Table 5.3.5.3.1-1: URI query parameters supported by the DELETE method on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| n/a |  |  |  |  |

This method shall support the request data structures specified in table 5.3.5.3.1-2 and the response data structures and response codes specified in table 5.3.5.3.1-3.

Table 5.3.5.3.1-2: Data structures supported by the DELETE Request Body on this resource

| Data type | P | Cardinality | Description |
| --- | --- | --- | --- |
| n/a |  |  |  |

Table 5.3.5.3.1-3: Data structures supported by the DELETE Response Body on this resource

| Data type | P | Cardinality | Response codes | Description |
| --- | --- | --- | --- | --- |
| n/a |  |  | 204 No Content | The PfdSubscription resource matching the subscriptionId was deleted successfully. |
| ProblemDetails | O | 0..1 | 307 Temporary Redirect | Temporary redirection, during Individual PFD subscription deletion. The response shall include a Location header field containing an alternative URI of the resource located in an alternative PFDF (service) instance. Applicable if the feature "ES3XX" is supported. |
| ProblemDetails | O | 0..1 | 308 Permanent Redirect | Permanent redirection, during Individual PFD subscription deletion. The response shall include a Location header field containing an alternative URI of the resource located in an alternative PFDF (service) instance. Applicable if the feature "ES3XX" is supported. |
| NOTE: In addition, the HTTP status codes which are specified as mandatory in table 5.2.7.1-1 of 3GPP TS 29.500 [5] for the DELETE method shall also apply. |  |  |  |  |

Table 5.3.5.3.1-4: Headers supported by the 307 Response Code on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| Location | string | M | 1 | An alternative URI of the resource located in an alternative PFDF (service) instance. |
| 3gpp-Sbi-Target-Nf-Id | string | O | 0..1 | Identifier of the target NF (service) instance towards which the request is redirected |

Table 5.3.5.3.1-5: Headers supported by the 308 Response Code on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| Location | string | M | 1 | An alternative URI of the resource located in an alternative PFDF (service) instance. |
| 3gpp-Sbi-Target-Nf-Id | string | O | 0..1 | Identifier of the target NF (service) instance towards which the request is redirected |

##### 5.3.5.3.2	PUT

This method shall support the URI query parameters specified in table 5.3.5.3.2-1.

Table 5.3.5.3.2-1: URI query parameters supported by the PUT method on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| n/a |  |  |  |  |

This method shall support the request data structures specified in table 5.3.5.3.2-2 and the response data structures and response codes specified in table 5.3.5.3.2-3.

Table 5.3.5.3.2-2: Data structures supported by the PUT Request Body on this resource

| Data type | P | Cardinality | Description |
| --- | --- | --- | --- |
| PfdSubscription | M | 1 | Update a PfdSubscription resource. |

Table 5.3.5.3.2-3: Data structures supported by the PUT Response Body on this resource

| Data type | P | Cardinality | Response codes | Description |
| --- | --- | --- | --- | --- |
| PfdSubscription | M | 1 | 200 OK | The update of a PfdSubscription resource is confirmed and a representation of that resource is returned. |
| ProblemDetails | O | 0..1 | 307 Temporary Redirect | Temporary redirection, during Individual PFD subscription modification. The response shall include a Location header field containing an alternative URI of the resource located in an alternative PFDF (service) instance. Applicable if the feature "ES3XX" is supported. |
| ProblemDetails | O | 0..1 | 308 Permanent Redirect | Permanent redirection, during Individual PFD subscription modification. The response shall include a Location header field containing an alternative URI of the resource located in an alternative PFDF (service) instance. Applicable if the feature "ES3XX" is supported. |
| NOTE: In addition, the HTTP status codes which are specified as mandatory in table 5.2.7.1-1 of 3GPP TS 29.500 [5] for the PUT method shall also apply. |  |  |  |  |

Table 5.3.5.3.2-4: Headers supported by the 307 Response Code on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| Location | string | M | 1 | An alternative URI of the resource located in an alternative PFDF (service) instance. |
| 3gpp-Sbi-Target-Nf-Id | string | O | 0..1 | Identifier of the target NF (service) instance towards which the request is redirected |

Table 5.3.5.3.2-5: Headers supported by the 308 Response Code on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| Location | string | M | 1 | An alternative URI of the resource located in an alternative PFDF (service) instance. |
| 3gpp-Sbi-Target-Nf-Id | string | O | 0..1 | Identifier of the target NF (service) instance towards which the request is redirected |

#### 5.3.5.4	Resource Custom Operations

None.

## 5.4	Custom Operations without associated resources

None

## 5.5	Notifications

### 5.5.1	General

Notifications shall comply to subclause 6.2 of 3GPP TS 29.500 [5] and subclause 4.6.2.3 of 3GPP TS 29.501 [6].

Table 5.5.1-1: Notifications overview

| Notification | Callback URI | HTTP method or custom operation | Description (service operation) |
| --- | --- | --- | --- |
| PFD Change Notification | {notifyUri} | POST | Notification of PFD change. |
| Notification Push | {notifyUri}/notifypush | notifypush (POST) | Notifies NF service consumer to retrieve the PFDs for application identifier(s). |

### 5.5.2	PFD Change Notification

#### 5.5.2.1	Description

The PFD Change Notification is used by the PFDF to inform the NF service consumer, which has subscribed to this Notification via the PFD subscriptions resource.

#### 5.5.2.2	Target URI

The Callback URI "{notifyUri}" shall be used with the callback URI variables defined in table 5.5.2.2-1.

Table 5.5.2.2-1: Callback URI variables

| Name | Data type | Definition |
| --- | --- | --- |
| notifyUri | Uri | The Notification Uri as assigned within the PFD subscriptions resource and described within the PfdSubscription data type (see table 5.6.2.3-1). |

#### 5.5.2.3	Standard Methods

##### 5.5.2.3.1	POST

This method shall support the URI query parameters specified in table 5.5.2.3.1-1.

Table 5.5.2.3.1-1: URI query parameters supported by the POST method on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| n/a |  |  |  |  |

This method shall support the request data structures specified in table 5.5.2.3.1-2 and the response data structures and response codes specified in table 5.5.2.3.1-3.

Table 5.5.2.3.1-2: Data structures supported by the POST Request Body on this resource

| Data type | P | Cardinality | Description |
| --- | --- | --- | --- |
| array(PfdChangeNotification) | M | 1..N | Provides PFD change information. |

Table 5.5.2.3.1-3: Data structures supported by the POST Response Body on this resource

| Data type | P | Cardinality | Response codes | Description |
| --- | --- | --- | --- | --- |
| n/a |  |  | 204 No Content | The PFD operation in the notification is performed successfully, i.e. all PFD changes are accepted by the NF service consumer. |
| array(PfdChangeReport) | M | 1..N | 200 OK | The PFD operation in the notification is performed and the PfdChangeReport indicates failure reason for each failed application in the partial success. |
| ProblemDetails | O | 0..1 | 307 Temporary Redirect | Temporary redirection, during PFD Change Notification. The response shall include a Location header field containing an alternative URI representing the end point of an alternative NF consumer (service) instance where the notification should be sent. Applicable if the feature "ES3XX" is supported. |
| ProblemDetails | O | 0..1 | 308 Permanent Redirect | Permanent redirection, during PFD Change Notification. The response shall include a Location header field containing an alternative URI representing the end point of an alternative NF consumer (service) instance where the notification should be sent. Applicable if the feature "ES3XX" is supported. |
| ProblemDetails | O | 0..1 | 500 Internal Server Error | The server encountered an unexpected condition that prevented it from fulfilling the request. PFDs for all applications are not accepted by the NF service consumer. (NOTE 2) |
| NOTE 1: In addition, the HTTP status codes which are specified as mandatory in table 5.2.7.1-1 of 3GPP TS 29.500 [5] for the POST method shall also apply. NOTE 2: Failure cases are described in subclause 5.7. |  |  |  |  |

Table 5.5.2.3.1-4: Headers supported by the 307 Response Code on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| Location | string | M | 1 | An alternative URI representing the end point of an alternative NF consumer (service) instance towards which the notification should be redirected. |
| 3gpp-Sbi-Target-Nf-Id | string | O | 0..1 | Identifier of the target NF (service) instance towards which the notification request is redirected |

Table 5.5.2.3.1-5: Headers supported by the 308 Response Code on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| Location | string | M | 1 | An alternative URI representing the end point of an alternative NF consumer (service) instance towards which the notification should be redirected. |
| 3gpp-Sbi-Target-Nf-Id | string | O | 0..1 | Identifier of the target NF (service) instance towards which the notification request is redirected |

### 5.5.3	Notification Push

#### 5.5.3.1	Description

The Notification Push is used by the PFDF to inform the NF service consumer, which has subscribed to this Notification via the PFD subscriptions resource.

#### 5.5.3.2	Target URI

The Callback URI "{notifyUri}/notifypush" shall be used with the callback URI variables defined in table 5.5.3.2-1.

Table 5.5.3.2-1: Callback URI variables

| Name | Data type | Definition |
| --- | --- | --- |
| notifyUri | Uri | The Notification Uri as assigned within the PFD subscriptions resource and described within the PfdSubscription data type (see table 5.6.2.3-1). |

#### 5.5.3.3	Standard Methods

##### 5.5.3.3.1	POST

This method shall support the URI query parameters specified in table 5.5.3.3.1-1.

Table 5.5.3.3.1-1: URI query parameters supported by the POST method on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| n/a |  |  |  |  |

This method shall support the request data structures specified in table 5.5.3.3.1-2 and the response data structures and response codes specified in table 5.5.3.3.1-3.

Table 5.5.3.3.1-2: Data structures supported by the POST Request Body on this resource

| Data type | P | Cardinality | Description |
| --- | --- | --- | --- |
| array(NotificationPush) | M | 1..N | Provides the information for the NF service consumer to retrieve and/or remove the PFDs. |

Table 5.5.3.3.1-3: Data structures supported by the POST Response Body on this resource

| Data type | P | Cardinality | Response codes | Description |
| --- | --- | --- | --- | --- |
| n/a |  |  | 204 No Content | Notification PUSH is accepted. |
| ProblemDetails | O | 0..1 | 307 Temporary Redirect | Temporary redirection, during PFD Change Notification. The response shall include a Location header field containing an alternative URI representing the end point of an alternative NF consumer (service) instance where the notification should be sent. Applicable if the feature "ES3XX" is supported. |
| ProblemDetails | O | 0..1 | 308 Permanent Redirect | Permanent redirection, during PFD Change Notification. The response shall include a Location header field containing an alternative URI representing the end point of an alternative NF consumer (service) instance where the notification should be sent. Applicable if the feature "ES3XX" is supported. |
| NOTE 1: In addition, the HTTP status codes which are specified as mandatory in table 5.2.7.1-1 of 3GPP TS 29.500 [5] for the POST method shall also apply. NOTE 2: Failure cases are described in subclause 5.7. |  |  |  |  |

Table 5.5.3.3.1-4: Headers supported by the 307 Response Code on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| Location | string | M | 1 | An alternative URI representing the end point of an alternative NF consumer (service) instance towards which the notification should be redirected. |
| 3gpp-Sbi-Target-Nf-Id | string | O | 0..1 | Identifier of the target NF (service) instance towards which the notification request is redirected |

Table 5.5.3.3.1-5: Headers supported by the 308 Response Code on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| Location | string | M | 1 | An alternative URI representing the end point of an alternative NF consumer (service) instance towards which the notification should be redirected. |
| 3gpp-Sbi-Target-Nf-Id | string | O | 0..1 | Identifier of the target NF (service) instance towards which the notification request is redirected |

## 5.6	Data Model

### 5.6.1	General

This subclause specifies the application data model supported by the API.

Table 5.6.1-1 specifies the data types defined for the Nnef_PFDmanagement service based interface protocol.

Table 5.6.1-1: Nnef_PFDmanagement specific Data Types

| Data type | Section defined | Description | Applicability |
| --- | --- | --- | --- |
| ApplicationForPfdRequest | 5.6.2.8 | Contains the application identifer(s) for the PFDs request. | PartialPull |
| NotificationPush | 5.6.2.7 | Provides the information for the NF service consumer to retrieve the PFDs and/or remove the PFDs of the applicable application identifier(s). | NotificationPush |
| PfdChangeNotification | 5.6.2.4 | Represents PFD change information. |  |
| PfdChangeReport | 5.6.2.6 | Represents error of PFD change. |  |
| PfdContent | 5.6.2.5 | Represents the content of a PFD for an application identifier. |  |
| PfdDataForApp | 5.6.2.2 | Represents the PFDs for an application identifier. |  |
| PfdOperation | 5.6.3.3 | Indicates the operation of the PFDs. | NotificationPush |
| PfdSubscription | 5.6.2.3 | Represents a PFD subscription. |  |

Table 5.6.1-2 specifies data types re-used by the Nnef_PFDmanagement service based interface protocol from other specifications, including a reference to their respective specifications and when needed, a short description of their use within the Nnef_PFDmanagement service based interface.

Table 5.6.1-2: Nnef_PFDmanagement re-used Data Types

| Data type | Reference | Comments | Applicability |
| --- | --- | --- | --- |
| ApplicationId | 3GPP TS 29.571 [10] |  |  |
| DateTime | 3GPP TS 29.571 [10] |  |  |
| DomainNameProtocol | 3GPP TS 29.122 [11] | Indicates the additional protocol and protocol field for domain names to be matched. | DomainNameProtocol |
| DurationSec | 3GPP TS 29.571 [10] | Identifies a period of time in units of seconds. | NotificationPush |
| SupportedFeatures | 3GPP TS 29.571 [10] |  |  |
| Uri | 3GPP TS 29.571 [10] |  |  |

### 5.6.2	Structured data types

#### 5.6.2.1	Introduction

This subclause defines the structures to be used in resource representations.

Allowed structures are: array, object.

#### 5.6.2.2	Type: PfdDataForApp

Table 5.6.2.2-1: Definition of type PfdDataForApp

| Attribute name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| applicationId | ApplicationId | M | 1 | Identifier of an application. |  |
| pfds | array(PfdContent) | C | 1..N | PFDs for the application identifier. |  |
| cachingTime | DateTime | O | 0..1 | Caching time for an application identifier. |  |
| pfdTimestamp | DateTime | O | 0..1 | The value represents the UTC time set for the PFD(s) of the application identifier provisioning by the PFDF. | PartialPull |
| partialFlag | boolean | O | 0..1 | Indication of partial update of PFDs for an existing application identifier if it is included and set to true. The default value "false" applies, if the attribute is not present. | PartialPull |
| NOTE: The "pfds" attribute shall be include if the "PartialPull" feature is not supported. |  |  |  |  |  |

#### 5.6.2.3	Type: PfdSubscription

Table 5.6.2.3-1: Definition of type PfdSubscription

| Attribute name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| applicationIds | array(ApplicationId) | O | 1..N | Identifiers of applications with PFDs change. |  |
| notifyUri | Uri | M | 1 | Identifies the recipient of notifications sent by PFDF for this subscription. |  |
| supportedFeatures | SupportedFeatures | M | 1 | List of supported features used as described in subclause 5.8. This parameter shall be provided by the NF service consumer in the POST request that request the creation of a subscription, and shall be provided by the PFDF in the response of corresponding request. |  |

#### 5.6.2.4	Type: PfdChangeNotification

Table 5.6.2.4-1: Definition of type PfdChangeNotification

| Attribute name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| applicationId | ApplicationId | M | 1 | Identifier of an application. |  |
| removalFlag | boolean | C | 0..1 | Indication of removal of PFDs for an existing application identifier. |  |
| partialFlag | boolean | C | 0..1 | Indication of partial update of PFDs for an existing application identifier if this operation is supported according to feature negotiation. | PartialUpdate |
| pfds | array(PfdContent) | C | 1..N | PFD creation/update for the application identifier as specified in subclause 6.3.3.5 of 3GPP TS 29.251 [12]. |  |

#### 5.6.2.5	Type: PfdContent

Table 5.6.2.5-1: Definition of type PfdContent

| Attribute name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| pfdId | string | C | 0..1 | Identifies a PDF of an application identifier. If PartialUpdate or PartialPull feature is supported during PFD change event subscription, this attribute shall be provided by the PFDF. |  |
| flowDescriptions | array(string) | O | 1..N | Represents a 3-tuple with protocol, server ip and server port for UL/DL application traffic. (NOTE) |  |
| urls | array(string) | O | 1..N | Indicates a URL or a regular expression which is used to match the significant parts of the URL. (NOTE) |  |
| domainNames | array(string) | O | 1..N | Indicates an FQDN or a regular expression as a domain name matching criteria. (NOTE) |  |
| dnProtocol | DomainNameProtocol | C | 0..1 | Indicates the additional protocol and protocol field for domain names to be matched, it may only be provided when domainNames attribute is present. | DomainNameProtocol |
| NOTE: If a PFD contains multiple filter types, the PFD is only matched when every filter type contained in the PFD has a matching value. |  |  |  |  |  |

#### 5.6.2.6	Type: PfdChangeReport

Table 5.6.2.6-1: PfdChangeReport

| Attribute name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| pfdError | ProblemDetails | M | 1 | More information on the error shall be provided in the "cause" attribute of the "ProblemDetails" structure. The "cause" attribute in the ProblemDetails shall be set to one of following application errors (see table 5.2.7.1-1 of 3GPP TS 29.500 [5]): - SYSTEM_FAILURE - INSUFFICIENT_RESOURCES - UNSPECIFIED_NF_FAILURE |  |
| applicationId | array(ApplicationId) | M | 1..N | Indicates the application identifier(s) which PFD(s) are failed to be added or modified. |  |

#### 5.6.2.7	Type: NotificationPush

Table 5.6.2.7-1: Definition of type NotificationPush

| Attribute name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| appIds | array(ApplicationId) | M | 1..N | Identifiers of one or more applications. |  |
| allowedDelay | DurationSec | O | 0..1 | Indicates the time limit that the NF service consumer shall retrieve the PFD(s). |  |
| pfdOp | PfdOperation | O | 0..1 | Indication the operation of the PFDs. |  |

#### 5.6.2.8	Type: ApplicationForPfdRequest

Table 5.6.2.8-1: Definition of ApplicationForPfdRequest

| Attribute name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| applicationId | ApplicationId | M | 1 | Identifier of an application. |  |
| pfdTimestamp | DateTime | O | 1 | The value represents the UTC time set for the PFD(s) of the application identifier provisioning by the PFDF. |  |

### 5.6.3	Simple data types and enumerations

#### 5.6.3.1	Introduction

This subclause defines simple data types and enumerations that can be referenced from data structures defined in the previous subclauses.

#### 5.6.3.2	Simple data types

The simple data types defined in table 5.6.3.2-1 shall be supported.

Table 5.6.3.2-1: Simple data types

| Type Name | Type Definition | Description | Applicability |
| --- | --- | --- | --- |

#### 5.6.3.3	Enumeration: PfdOperation

Table 5.6.3.3-1: Enumeration PfdOperation

| Enumeration value | Description | Applicability |
| --- | --- | --- |
| RETRIEVE | Indicates to retrieve the PFD(s) for the application identifier(s). The NF service consumer determines which pull procedure can be applied to retrieve the PFD(s). |  |
| FULLPULL | Indicates to retrieve the PFD(s) for the application identifier(s) by the full pull procedure. |  |
| PARTIALPULL | Indicates to retrieve the PFD(s) for the application identifier(s) by the partial pull procedure. The NF consumer may decide to use full pull procedure instead if needed. |  |
| REMOVE | Indicates to remove the PFD(s) for the application identifier(s). |  |

## 5.7	Error handling

### 5.7.1	General

HTTP error handling shall be supported as specified in subclause 5.2.4 of 3GPP TS 29.500 [5].

For the Nnef_PFDmanagement API, HTTP error responses shall be supported as specified in subclause 4.8 of 3GPP TS 29.501 [6].

Protocol errors and application errors specified in table 5.2.7.2-1 of 3GPP TS 29.500 [5] shall be supported for an HTTP method if the corresponding HTTP status codes are specified as mandatory for that HTTP method in table 5.2.7.1-1 of 3GPP TS 29.500 [5].

Protocol errors and application errors specified in table 5.2.7.2-1 of 3GPP TS 29.500 [5] for HTTP redirections shall be supported if the feature "ES3XX" is supported.

In addition, the requirements in the following subclauses shall apply.

### 5.7.2	Protocol Errors

In this Release of the specification, there are no additional protocol errors applicable for the Nnef_PFDmanagement API.

### 5.7.3	Application Errors

The application errors defined for the Nnef_PFDmanagement service are listed in table 5.7.3-1.

Table 5.7.3-1: Application errors

| Application Error | HTTP status code | Description |
| --- | --- | --- |
| SYSTEM_FAILURE | 500 Internal Server Error | Something functions wrongly in PFD provisioning or the PFD provisioning does not function at all. (NOTE) |
| INSUFFICIENT_RESOURCE | 500 Internal Server Error | There is limitation for resource storage. (NOTE) |
| UNSPECIFIED_NF_FAILURE | 500 Internal Server Error | Unspecified reason. (NOTE) |
| NOTE: This application error is included in the responses to the POST request of PFD change notification. |  |  |

## 5.8	Feature negotiation

The optional features in table 5.8-1 are defined for the Nnef_PFDmanagement API. They shall be negotiated using the extensibility mechanism defined in subclause 6.6 of 3GPP TS 29.500 [5].

Table 5.8-1: Supported Features

| Feature number | Feature Name | Description |
| --- | --- | --- |
| 1 | PartialUpdate | The PFDF can use this feature for partial update of PFDs. |
| 2 | DomainNameProtocol | This feature supports the additional protocol matching condition for the domain name in PFD data. |
| 3 | PfdChgSubsUpdate | The NF service consumer can use this feature for updating the PFD change subscription. |
| 4 | ES3XX | Extended Support for 3xx redirections. This feature indicates the support of redirection for any service operation, according to Stateless NF procedures as specified in subclauses 6.5.3.2 and 6.5.3.3 of 3GPP TS 29.500 [5] and according to HTTP redirection principles for indirect communication, as specified in subclause 6.10.9 of 3GPP TS 29.500 [5]. |
| 5 | PartialPull | The NF service consumer can use this feature to retrieve the PFDs by the partial update. |
| 6 | NotificationPush | The PFDF can use this feature to notify the NF service consumer to retrieve the PFDs. |

## 5.9	Security

As indicated in 3GPP TS 33.501 [14] and 3GPP TS 29.500 [5], the access to the Nnef_PFDmanagement API may be authorized by means of the OAuth2 protocol (see IETF RFC 6749 [15]), based on local configuration, using the "Client Credentials" authorization grant, where the NRF (see 3GPP TS 29.510 [13]) plays the role of the authorization server.

If OAuth2 is used, an NF service consumer, prior to consuming services offered by the Nnef_PFDmanagement API, shall obtain a "token" from the authorization server, by invoking the Access Token Request service, as described in 3GPP TS 29.510 [13], subclause 5.4.2.2.

NOTE:	When multiple NRFs are deployed in a network, the NRF used as authorization server is the same NRF that the NF service consumer used for discovering the Nnef_PFDmanagement service.

The Nnef_PFDmanagement API defines a single scope "nnef-pfdmanagement" for OAuth2 authorization (as specified in 3GPP TS 33.501 [14]) for the entire service, and it does not define any additional scopes at resource or operation level.

Annex A (normative):
OpenAPI specification

# A.1	General

The present Annex contains an OpenAPI [9] specification of HTTP messages and content bodies used by the Nnef_PFDmanagement API.

This Annex shall take precedence when being discrepant to other parts of the specification with respect to the encoding of information elements and methods within the API.

NOTE:	The semantics and procedures, as well as conditions, e.g. for the applicability and allowed combinations of attributes or values, not expressed in the OpenAPI definitions but defined in other parts of the specification also apply.

Informative copies of the OpenAPI specification file contained in this 3GPP Technical Specification are available on a Git-based repository that uses the GitLab software version control system (see clause 5B of the 3GPP TR 21.900 [17] and subclause 5.3.1 of the 3GPP TS 29.501 [6] for further information).:

# A.2	Nnef_PFDmanagement API

openapi: 3.0.0

info:

title: Nnef_PFDmanagement Service API

version: 1.2.0-alpha.3

description: |

Packet Flow Description Management Service.

© 2021, 3GPP Organizational Partners (ARIB, ATIS, CCSA, ETSI, TSDSI, TTA, TTC).

All rights reserved.

#

externalDocs:

description: 3GPP TS 29.551 v17.2.0, 5G System; Packet Flow Description Management Service

url: 'http://www.3gpp.org/ftp/Specs/archive/29_series/29.551/'

#

servers:

- url: '{apiRoot}/nnef-pfdmanagement/v1'

variables:

apiRoot:

default: https://example.com

description: apiRoot as defined in subclause 4.4 of 3GPP TS 29.501

security:

- {}

- oAuth2ClientCredentials:

- nnef-pfdmanagement

paths:

/applications:

get:

summary: Retrieve PFDs for all applications or for one or multiple applications with query parameter.

tags:

- PFD of applications

operationId: Nnef_PFDmanagement_AllFetch

parameters:

- name: application-ids

description: The required application identifier(s) for the returned PFDs.

in: query

required: true

schema:

type: array

items:

$ref: 'TS29571_CommonData.yaml#/components/schemas/ApplicationId'

minItems: 1

- name: supported-features

in: query

description: To filter irrelevant responses related to unsupported features

schema:

$ref: 'TS29571_CommonData.yaml#/components/schemas/SupportedFeatures'

responses:

'200':

description: The PFDs for one or more application identifier(s) in the request URI are returned.

content:

application/json:

schema:

type: array

items:

$ref: '#/components/schemas/PfdDataForApp'

minItems: 0

'400':

$ref: 'TS29571_CommonData.yaml#/components/responses/400'

'401':

$ref: 'TS29571_CommonData.yaml#/components/responses/401'

'403':

$ref: 'TS29571_CommonData.yaml#/components/responses/403'

'404':

$ref: 'TS29571_CommonData.yaml#/components/responses/404'

'406':

$ref: 'TS29571_CommonData.yaml#/components/responses/406'

'414':

$ref: 'TS29571_CommonData.yaml#/components/responses/414'

'429':

$ref: 'TS29571_CommonData.yaml#/components/responses/429'

'500':

$ref: 'TS29571_CommonData.yaml#/components/responses/500'

'503':

$ref: 'TS29571_CommonData.yaml#/components/responses/503'

default:

$ref: 'TS29571_CommonData.yaml#/components/responses/default'

/applications/partial-pull:

post:

summary: retrieve the PFD(s) by partial update

operationId: Nnef_PFDmanagement_AppFetchPartialUpdate

tags:

- PFD of applications by partial update

requestBody:

required: true

content:

application/json:

schema:

type: array

items:

$ref: '#/components/schemas/ApplicationForPfdRequest'

minItems: 1

responses:

'200':

description: OK. Changed PFD(s) is returned

content:

application/json:

schema:

type: array

items:

$ref: '#/components/schemas/PfdDataForApp'

minItems: 1

'204':

description: The PFD(s) is not changed

'400':

$ref: 'TS29571_CommonData.yaml#/components/responses/400'

'401':

$ref: 'TS29571_CommonData.yaml#/components/responses/401'

'403':

$ref: 'TS29571_CommonData.yaml#/components/responses/403'

'404':

$ref: 'TS29571_CommonData.yaml#/components/responses/404'

'411':

$ref: 'TS29571_CommonData.yaml#/components/responses/411'

'413':

$ref: 'TS29571_CommonData.yaml#/components/responses/413'

'415':

$ref: 'TS29571_CommonData.yaml#/components/responses/415'

'429':

$ref: 'TS29571_CommonData.yaml#/components/responses/429'

'500':

$ref: 'TS29571_CommonData.yaml#/components/responses/500'

'503':

$ref: 'TS29571_CommonData.yaml#/components/responses/503'

default:

$ref: 'TS29571_CommonData.yaml#/components/responses/default'

/applications/{appId}:

get:

summary: Retrieve the PFD for an application.

tags:

- Individual application PFD

operationId: Nnef_PFDmanagement_IndAppFetch

parameters:

- name: appId

description: The required application identifier(s) for the returned PFDs.

in: path

required: true

schema:

type: string

- name: supported-features

in: query

description: To filter irrelevant responses related to unsupported features

schema:

$ref: 'TS29571_CommonData.yaml#/components/schemas/SupportedFeatures'

responses:

'200':

description: A representation of PFDs for an application in the request URI is returned.

content:

application/json:

schema:

$ref: '#/components/schemas/PfdDataForApp'

'307':

description: Temporary Redirect

content:

application/problem+json:

schema:

$ref: 'TS29571_CommonData.yaml#/components/schemas/ProblemDetails'

headers:

Location:

description: 'An alternative URI of the resource located on an alternative PFDF (service) instance.'

required: true

schema:

type: string

3gpp-Sbi-Target-Nf-Id:

description: 'Identifier of the target NF (service) instance towards which the request is redirected'

schema:

type: string

'308':

description: Permanent Redirect

content:

application/problem+json:

schema:

$ref: 'TS29571_CommonData.yaml#/components/schemas/ProblemDetails'

headers:

Location:

description: 'An alternative URI of the resource located on an alternative PFDF (service) instance.'

required: true

schema:

type: string

3gpp-Sbi-Target-Nf-Id:

description: 'Identifier of the target NF (service) instance towards which the request is redirected'

schema:

type: string

'400':

$ref: 'TS29571_CommonData.yaml#/components/responses/400'

'401':

$ref: 'TS29571_CommonData.yaml#/components/responses/401'

'403':

$ref: 'TS29571_CommonData.yaml#/components/responses/403'

'404':

$ref: 'TS29571_CommonData.yaml#/components/responses/404'

'406':

$ref: 'TS29571_CommonData.yaml#/components/responses/406'

'429':

$ref: 'TS29571_CommonData.yaml#/components/responses/429'

'414':

$ref: 'TS29571_CommonData.yaml#/components/responses/414'

'500':

$ref: 'TS29571_CommonData.yaml#/components/responses/500'

'503':

$ref: 'TS29571_CommonData.yaml#/components/responses/503'

default:

$ref: 'TS29571_CommonData.yaml#/components/responses/default'

/subscriptions:

post:

summary: Subscribe the notification of PFD changes.

tags:

- PFD subscriptions

operationId: Nnef_PFDmanagement_CreateSubscr

requestBody:

description: a PfdSubscription resource to be created.

required: true

content:

application/json:

schema:

$ref: '#/components/schemas/PfdSubscription'

callbacks:

PfdChangeNotification:

'{request.body#/notifyUri}':

post:

summary: Notification of PFD change.

tags:

- PfdChangeNotification data

operationId: Nnef_PFDmanagement_Notify

requestBody:

required: true

content:

application/json:

schema:

type: array

items:

$ref: '#/components/schemas/PfdChangeNotification'

minItems: 1

responses:

'200':

description: The PFD operation in the notification is performed and the PfdChangeReport indicates failure reason.

content:

application/json:

schema:

type: array

items:

$ref: '#/components/schemas/PfdChangeReport'

minItems: 1

'204':

description: The PFD operation in the notification is performed successfully.

'307':

description: Temporary Redirect

content:

application/problem+json:

schema:

$ref: 'TS29571_CommonData.yaml#/components/schemas/ProblemDetails'

headers:

Location:

required: true

description: 'A URI pointing to the endpoint of an alternative NF consumer (service) instance towards which the notification should be redirected.'

schema:

type: string

3gpp-Sbi-Target-Nf-Id:

description: 'Identifier of the target NF (service) instance towards which the notification request is redirected'

schema:

type: string

'308':

description: Permanent Redirect

content:

application/problem+json:

schema:

$ref: 'TS29571_CommonData.yaml#/components/schemas/ProblemDetails'

headers:

Location:

required: true

description: 'A URI pointing to the endpoint of an alternative NF consumer (service) instance towards which the notification should be redirected.'

schema:

type: string

3gpp-Sbi-Target-Nf-Id:

description: 'Identifier of the target NF (service) instance towards which the notification request is redirected'

schema:

type: string

'400':

$ref: 'TS29571_CommonData.yaml#/components/responses/400'

'401':

$ref: 'TS29571_CommonData.yaml#/components/responses/401'

'403':

$ref: 'TS29571_CommonData.yaml#/components/responses/403'

'404':

$ref: 'TS29571_CommonData.yaml#/components/responses/404'

'411':

$ref: 'TS29571_CommonData.yaml#/components/responses/411'

'413':

$ref: 'TS29571_CommonData.yaml#/components/responses/413'

'415':

$ref: 'TS29571_CommonData.yaml#/components/responses/415'

'429':

$ref: 'TS29571_CommonData.yaml#/components/responses/429'

'500':

$ref: 'TS29571_CommonData.yaml#/components/responses/500'

'503':

$ref: 'TS29571_CommonData.yaml#/components/responses/503'

default:

$ref: 'TS29571_CommonData.yaml#/components/responses/default'

NotificationPush:

'{request.body#/notifyUri}/notifypush':

post:

summary: Notification Push.

tags:

- NotificationPush data

operationId: Nnef_PFDmanagement_PushNotify

requestBody:

required: true

content:

application/json:

schema:

type: array

items:

$ref: '#/components/schemas/NotificationPush'

minItems: 1

responses:

'204':

description: Notificaiton PUSH is accepted.

'307':

description: Temporary Redirect

content:

application/problem+json:

schema:

$ref: 'TS29571_CommonData.yaml#/components/schemas/ProblemDetails'

headers:

Location:

required: true

description: 'A URI pointing to the endpoint of an alternative NF consumer (service) instance towards which the notification should be redirected.'

schema:

type: string

3gpp-Sbi-Target-Nf-Id:

description: 'Identifier of the target NF (service) instance towards which the notification request is redirected'

schema:

type: string

'308':

description: Permanent Redirect

content:

application/problem+json:

schema:

$ref: 'TS29571_CommonData.yaml#/components/schemas/ProblemDetails'

headers:

Location:

required: true

description: 'A URI pointing to the endpoint of an alternative NF consumer (service) instance towards which the notification should be redirected.'

schema:

type: string

3gpp-Sbi-Target-Nf-Id:

description: 'Identifier of the target NF (service) instance towards which the notification request is redirected'

schema:

type: string

'400':

$ref: 'TS29571_CommonData.yaml#/components/responses/400'

'401':

$ref: 'TS29571_CommonData.yaml#/components/responses/401'

'403':

$ref: 'TS29571_CommonData.yaml#/components/responses/403'

'404':

$ref: 'TS29571_CommonData.yaml#/components/responses/404'

'411':

$ref: 'TS29571_CommonData.yaml#/components/responses/411'

'413':

$ref: 'TS29571_CommonData.yaml#/components/responses/413'

'415':

$ref: 'TS29571_CommonData.yaml#/components/responses/415'

'429':

$ref: 'TS29571_CommonData.yaml#/components/responses/429'

'500':

$ref: 'TS29571_CommonData.yaml#/components/responses/500'

'503':

$ref: 'TS29571_CommonData.yaml#/components/responses/503'

default:

$ref: 'TS29571_CommonData.yaml#/components/responses/default'

responses:

'201':

description: The creation of a PfdSubscription resource is confirmed and a representation of that resource is returned.

content:

application/json:

schema:

$ref: '#/components/schemas/PfdSubscription'

headers:

Location:

description: 'Contains the URI of the newly created resource, according to the structure: {apiRoot}/nnef-pfdmanagement/v1/subscriptions/{subscriptionId}'

required: true

schema:

type: string

'400':

$ref: 'TS29571_CommonData.yaml#/components/responses/400'

'401':

$ref: 'TS29571_CommonData.yaml#/components/responses/401'

'403':

$ref: 'TS29571_CommonData.yaml#/components/responses/403'

'404':

$ref: 'TS29571_CommonData.yaml#/components/responses/404'

'411':

$ref: 'TS29571_CommonData.yaml#/components/responses/411'

'413':

$ref: 'TS29571_CommonData.yaml#/components/responses/413'

'415':

$ref: 'TS29571_CommonData.yaml#/components/responses/415'

'429':

$ref: 'TS29571_CommonData.yaml#/components/responses/429'

'500':

$ref: 'TS29571_CommonData.yaml#/components/responses/500'

'503':

$ref: 'TS29571_CommonData.yaml#/components/responses/503'

default:

$ref: 'TS29571_CommonData.yaml#/components/responses/default'

/subscriptions/{subscriptionId}:

put:

summary: Updates/replaces an existing subscription resource

tags:

- Individual PFD subscription

operationId: Nnef_PFDmanagement_ModifySubscr

parameters:

- name: subscriptionId

description: Identify the subscription.

in: path

required: true

schema:

type: string

requestBody:

description: Parameters to update/replace the existing subscription

required: true

content:

application/json:

schema:

$ref: '#/components/schemas/PfdSubscription'

responses:

'200':

description: OK (Successful update of the subscription)

content:

application/json:

schema:

$ref: '#/components/schemas/PfdSubscription'

'307':

description: Temporary Redirect

content:

application/problem+json:

schema:

$ref: 'TS29571_CommonData.yaml#/components/schemas/ProblemDetails'

headers:

Location:

description: 'An alternative URI of the resource located on an alternative PFDF (service) instance.'

required: true

schema:

type: string

3gpp-Sbi-Target-Nf-Id:

description: 'Identifier of the target NF (service) instance towards which the request is redirected'

schema:

type: string

'308':

description: Permanent Redirect

content:

application/problem+json:

schema:

$ref: 'TS29571_CommonData.yaml#/components/schemas/ProblemDetails'

headers:

Location:

description: 'An alternative URI of the resource located on an alternative PFDF (service) instance.'

required: true

schema:

type: string

3gpp-Sbi-Target-Nf-Id:

description: 'Identifier of the target NF (service) instance towards which the request is redirected'

schema:

type: string

'400':

$ref: 'TS29571_CommonData.yaml#/components/responses/400'

'401':

$ref: 'TS29571_CommonData.yaml#/components/responses/401'

'403':

$ref: 'TS29571_CommonData.yaml#/components/responses/403'

'404':

$ref: 'TS29571_CommonData.yaml#/components/responses/404'

'411':

$ref: 'TS29571_CommonData.yaml#/components/responses/411'

'413':

$ref: 'TS29571_CommonData.yaml#/components/responses/413'

'415':

$ref: 'TS29571_CommonData.yaml#/components/responses/415'

'429':

$ref: 'TS29571_CommonData.yaml#/components/responses/429'

'500':

$ref: 'TS29571_CommonData.yaml#/components/responses/500'

'503':

$ref: 'TS29571_CommonData.yaml#/components/responses/503'

default:

$ref: 'TS29571_CommonData.yaml#/components/responses/default'

delete:

summary: Delete a subscription of PFD change notification.

tags:

- Individual PFD subscription

operationId: Nnef_PFDmanagement_Unsubscribe

parameters:

- name: subscriptionId

description: Identify the subscription.

in: path

required: true

schema:

type: string

responses:

'204':

description: The PfdSubscription resource matching the subscriptionId was deleted successfully.

'307':

description: Temporary Redirect

content:

application/problem+json:

schema:

$ref: 'TS29571_CommonData.yaml#/components/schemas/ProblemDetails'

headers:

Location:

description: 'An alternative URI of the resource located on an alternative PFDF (service) instance.'

required: true

schema:

type: string

3gpp-Sbi-Target-Nf-Id:

description: 'Identifier of the target NF (service) instance towards which the request is redirected'

schema:

type: string

'308':

description: Permanent Redirect

content:

application/problem+json:

schema:

$ref: 'TS29571_CommonData.yaml#/components/schemas/ProblemDetails'

headers:

Location:

description: 'An alternative URI of the resource located on an alternative PFDF (service) instance.'

required: true

schema:

type: string

3gpp-Sbi-Target-Nf-Id:

description: 'Identifier of the target NF (service) instance towards which the request is redirected'

schema:

type: string

'400':

$ref: 'TS29571_CommonData.yaml#/components/responses/400'

'401':

$ref: 'TS29571_CommonData.yaml#/components/responses/401'

'403':

$ref: 'TS29571_CommonData.yaml#/components/responses/403'

'404':

$ref: 'TS29571_CommonData.yaml#/components/responses/404'

'429':

$ref: 'TS29571_CommonData.yaml#/components/responses/429'

'500':

$ref: 'TS29571_CommonData.yaml#/components/responses/500'

'503':

$ref: 'TS29571_CommonData.yaml#/components/responses/503'

default:

$ref: 'TS29571_CommonData.yaml#/components/responses/default'

components:

securitySchemes:

oAuth2ClientCredentials:

type: oauth2

flows:

clientCredentials:

tokenUrl: '{nrfApiRoot}/oauth2/token'

scopes:

nnef-pfdmanagement: Access to the Nnef_PFDmanagement API

schemas:

#

# STRUCTURED DATA TYPES

#

PfdContent:

description: Represents the content of a PFD for an application identifier.

type: object

properties:

pfdId:

type: string

description: Identifies a PDF of an application identifier.

flowDescriptions:

type: array

items:

type: string

minItems: 1

description: Represents a 3-tuple with protocol, server ip and server port for UL/DL application traffic.

urls:

type: array

items:

type: string

minItems: 1

description: Indicates a URL or a regular expression which is used to match the significant parts of the URL.

domainNames:

type: array

items:

type: string

minItems: 1

description: Indicates an FQDN or a regular expression as a domain name matching criteria.

dnProtocol:

$ref: 'TS29122_PfdManagement.yaml#/components/schemas/DomainNameProtocol'

PfdDataForApp:

description: Represents the PFDs for an application identifier.

type: object

properties:

applicationId:

$ref: 'TS29571_CommonData.yaml#/components/schemas/ApplicationId'

pfds:

type: array

items:

$ref: '#/components/schemas/PfdContent'

minItems: 1

cachingTime:

$ref: 'TS29571_CommonData.yaml#/components/schemas/DateTime'

pfdTimestamp:

$ref: 'TS29571_CommonData.yaml#/components/schemas/DateTime'

partialFlag:

type: boolean

required:

- applicationId

PfdSubscription:

description: Represents a PFD subscription.

type: object

properties:

applicationIds:

type: array

items:

$ref: 'TS29571_CommonData.yaml#/components/schemas/ApplicationId'

minItems: 1

notifyUri:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Uri'

supportedFeatures:

$ref: 'TS29571_CommonData.yaml#/components/schemas/SupportedFeatures'

required:

- notifyUri

- supportedFeatures

PfdChangeNotification:

description: Represents information related to a notification of PFD change.

type: object

properties:

applicationId:

$ref: 'TS29571_CommonData.yaml#/components/schemas/ApplicationId'

removalFlag:

type: boolean

default: false

partialFlag:

type: boolean

default: false

pfds:

type: array

items:

$ref: '#/components/schemas/PfdContent'

minItems: 1

required:

- applicationId

NotificationPush:

description: Represents the information to be used by the NF service consumer to retrieve the PFDs and/or remove the PFDs of the applicable application identifier(s).

type: object

properties:

appIds:

type: array

items:

$ref: 'TS29571_CommonData.yaml#/components/schemas/ApplicationId'

minItems: 1

allowedDelay:

$ref: 'TS29571_CommonData.yaml#/components/schemas/DurationSec'

pfdOp:

$ref: '#/components/schemas/PfdOperation'

required:

- appIds

PfdChangeReport:

description: Represents an error report on PFD change.

type: object

properties:

pfdError:

$ref: 'TS29571_CommonData.yaml#/components/schemas/ProblemDetails'

applicationId:

type: array

items:

$ref: 'TS29571_CommonData.yaml#/components/schemas/ApplicationId'

minItems: 1

required:

- pfdError

- applicationId

PfdOperation:

description: Indicates the operation to be applied on PFD(s).

anyOf:

- type: string

enum:

- RETRIEVE

- FULLPULL

- PARTIALPULL

- REMOVE

- type: string

ApplicationForPfdRequest:

description: Contains the application identifer(s) for the PFD(s) request.

type: object

properties:

applicationId:

$ref: 'TS29571_CommonData.yaml#/components/schemas/ApplicationId'

pfdTimestamp:

$ref: 'TS29571_CommonData.yaml#/components/schemas/DateTime'

required:

- applicationId

Annex B (informative):
Change history

| Change history |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Date | Meeting | TDoc | CR | Rev | Cat | Subject/Comment | New version |
| 2018-01 |  |  |  |  |  | TS skeleton of PFD Management Service specification | 0.0.0 |
| 2018-01 | CT3#94 |  |  |  |  | Includes the following contribution agreed by CT3 at CT3#94: C3-180045, C3-180270, C3-180271. | 0.1.0 |
| 2018-03 | CT3#95 |  |  |  |  | Includes the following contribution agreed by CT3 at CT3#95: C3-181100, C3-181262, C3-181102, C3-181103, C3-181314. | 0.2.0 |
| 2018-04 | CT3#96 |  |  |  |  | Includes the following contribution agreed by CT3 at CT3#96: C3-182153, C3-182411, C3-182412, C3-182413, C3-182414, C3-182477, C3-182478. | 0.3.0 |
| 2018-05 | CT3#97 |  |  |  |  | Includes the following contribution agreed by CT3 at CT3#97: C3-183115, C3-183557, C3-183558, C3-183560. | 0.4.0 |
| 2018-06 | CT#80 | CP-181029 |  |  |  | TS sent to plenary for approval | 1.0.0 |
| 2018-06 | CT#80 | CP-181029 |  |  |  | TS approved by plenary | 15.0.0 |
| 2018-09 | CT#81 | CP-182015 | 0001 | 2 | F | Protocol error statement | 15.1.0 |
| 2018-09 | CT#81 | CP-182015 | 0002 | 1 | F | Description of Structured data types | 15.1.0 |
| 2018-12 | CT#82 | CP-183205 | 0003 |  | F | Default value for apiRoot | 15.2.0 |
| 2018-12 | CT#82 | CP-183205 | 0004 | 3 | F | Cardinality | 15.2.0 |
| 2018-12 | CT#82 | CP-183205 | 0005 | 1 | F | Correct Nnef_PFDmanagement API | 15.2.0 |
| 2018-12 | CT#82 | CP-183205 | 0006 |  | F | Location Header | 15.2.0 |
| 2018-12 | CT#82 | CP-183205 | 0007 | 1 | F | Alignment of openAPI | 15.2.0 |
| 2018-12 | CT#82 | CP-183205 | 0008 |  | F | API version update | 15.2.0 |
| 2018-12 | CT#82 | CP-183205 | 0009 |  | F | Security | 15.2.0 |
| 2018-12 | CT#82 | CP-183205 | 0010 |  | F | Content type | 15.2.0 |
| 2019-03 | CT#83 | CP-190116 | 0011 | 1 | F | Fetch PFD for all applications | 15.3.0 |
| 2019-03 | CT#83 | CP-190116 | 0014 | 1 | F | Correction of resource URIs | 15.3.0 |
| 2019-03 | CT#83 | CP-190130 | 0010 | 1 | B | PUT for PFD change subscription | 16.0.0 |
| 2019-03 | CT#83 | CP-190121 | 0012 | 3 | B | PFD extension | 16.0.0 |
| 2019-03 | CT#83 |  |  |  |  | Open API version update by MCC | 16.0.0 |
| 2019-06 | CT#84 | CP-191083 | 0016 | 2 | A | Precedence of OpenAPI file | 16.1.0 |
| 2019-06 | CT#84 | CP-191083 | 0019 |  | A | Correction to Notification of PFD change | 16.1.0 |
| 2019-06 | CT#84 | CP-191083 | 0021 |  | A | Copyright Note in YAML file | 16.1.0 |
| 2019-06 | CT#84 | CP-191101 | 0023 | 2 | F | API version Update | 16.1.0 |
| 2019-09 | CT#85 | CP-192149 | 0025 | 2 | A | Correct presence condition in PFD definition | 16.2.0 |
| 2020-03 | CT#87e | CP-200215 | 0026 |  | F | Reference of Error code | 16.3.0 |
| 2020-03 | CT#87e | CP-200216 | 0027 |  | F | Update of OpenAPI version and TS version in externalDocs field | 16.3.0 |
| 2020-06 | CT#88e | CP-201244 | 0028 |  | F | Non-unique operation identifiers | 16.4.0 |
| 2020-06 | CT#88e | CP-201244 | 0029 | 1 | F | Storage of YAML files in ETSI Forge | 16.4.0 |
| 2020-06 | CT#88e | CP-201256 | 0030 | 1 | F | URI of the Nnef_PFDmanagement service | 16.4.0 |
| 2020-06 | CT#88e | CP-201244 | 0031 | 1 | F | Optionality of ProblemDetails | 16.4.0 |
| 2020-06 | CT#88e | CP-201244 | 0032 | 1 | F | Supported headers, Resource Data type and yaml mapping | 16.4.0 |
| 2020-06 | CT#88e | CP-201255 | 0033 |  | F | Update of OpenAPI version and TS version in externalDocs field | 16.4.0 |
| 2020-09 | CT#89e | CP-202056 | 0035 |  | A | Correction to the PFD change notification | 16.5.0 |
| 2020-09 | CT#89e | CP-202205 | 0038 | 2 | B | Notification PUSH | 17.0.0 |
| 2020-09 | CT#89e | CP-202085 | 0039 |  | F | Update of OpenAPI version and TS version in externalDocs field | 17.0.0 |
| 2020-12 | CT#90e | CP-203078 | 0041 | 2 | A | Essential corrections and alignments | 17.1.0 |
| 2020-12 | CT#90e | CP-203135 | 0042 | 1 | F | Corrections to Notification Push procedure | 17.1.0 |
| 2020-12 | CT#90e | CP-203120 | 0046 | 1 | A | Correction to PFD retrieval in PULL mode | 17.1.0 |
| 2020-12 | CT#90e | CP-203120 | 0049 |  | A | Correction to notification URI of PFD change notification | 17.1.0 |
| 2020-12 | CT#90e | CP-203135 | 0050 | 1 | B | PULL mode enhancement | 17.1.0 |
| 2020-12 | CT#90e | CP-203135 | 0051 | 1 | B | Remove the EN within the PushNotification | 17.1.0 |
| 2020-12 | CT#90e | CP-203139 | 0053 | 1 | A | Storage of YAML files in 3GPP Forge | 17.1.0 |
| 2020-12 | CT#90e | CP-203135 | 0054 |  | F | NotificationPush data type definition | 17.1.0 |
| 2020-12 | CT#90e | CP-203141 | 0055 | 1 | F | Non-unique operation identifiers | 17.1.0 |
| 2020-12 | CT#90e | CP-203153 | 0056 |  | F | Update of OpenAPI version and TS version in externalDocs field | 17.1.0 |
| 2021-03 | CT#91e | CP-210191 | 0058 | 1 | A | Support of stateless NFs | 17.2.0 |
| 2021-03 | CT#91e | CP-210215 | 0059 | 5 | B | Resolve the FFS for notification push | 17.2.0 |
| 2021-03 | CT#91e | CP-210215 | 0060 | 3 | B | Partial pull update | 17.2.0 |
| 2021-03 | CT#91e | CP-210215 | 0061 | 5 | F | Corrections on partial pull | 17.2.0 |
| 2021-03 | CT#91e | CP-210218 | 0062 |  | F | OpenAPI reference | 17.2.0 |
| 2021-03 | CT#91e | CP-210222 | 0064 | 1 | A | notifyUri used by notification | 17.2.0 |
| 2021-03 | CT#91e | CP-210215 | 0065 |  | F | Invalid Nnef_PFDmanagement OpenAPI file | 17.2.0 |
| 2021-03 | CT#91e | CP-210215 | 0066 |  | B | Correction to the name of custom operation | 17.2.0 |
| 2021-03 | CT#91e | CP-210215 | 0067 | 1 | B | Overall description of notification push | 17.2.0 |
| 2021-03 | CT#91e | CP-210219 | 0069 |  | F | Adding some missing description fields to data type definitions in OpenAPI specification files | 17.2.0 |
| 2021-03 | CT#91e | CP-210200 | 0072 |  | A | Datatype and figure corrections | 17.2.0 |
| 2021-03 | CT#91e | CP-210220 | 0073 |  | F | Custom header clarification | 17.2.0 |
| 2021-03 | CT#91e | CP-210221 | 0074 | 1 | F | Terminology alignment usage of NF service consumer | 17.2.0 |
| 2021-03 | CT#91e | CP-210240 | 0076 |  | F | Update of OpenAPI version and TS version in externalDocs field | 17.2.0 |