3GPP TSG-CT WG3 Meeting #108-e	C3-201168

- E-Meeting, 19th – 28th February 2020

| CR-Form-v12.0 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 29.214 | CR | 1635 | rev | - | Current version: | 16.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | x |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Report of EPS Fallback |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson |  |  |  |  |  |  |  |  |  |
| Source to TSG: | C3 |  |  |  |  |  |  |  |  |  |
| Work item code: | eIMS5G_SBA |  |  |  |  |  | Date: |  |  | 2020-02-14 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | SA2 agreed in S2-2001341 the support of a new event, EPS Fallback, which is met when armed by the PCF and the SMF receives a PDU session modification response indicating that the establishment of the QoS Flow with 5QI=1 is rejected due to EPS Fallback. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Definition of: - a new feature “EPSFallbackSupport”, - a new Specific-Action value, “EPS_FALLBACK”, and - detailed service procedures. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Report of EPS fallback is not supported. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.3.13, 5.4.1, E.x1 (new) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  | X |  | Other core specifications |  |  |  | TS 23.503 CR 0384 |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

- Additional discussion(if needed):W

- Proposed changes:

- *** 1st Change ***

### 5.3.13	Specific-Action AVP

The Specific-Action AVP (AVP code 513) is of type Enumerated.

Within a PCRF initiated Re-Authorization Request, the Specific-Action AVP determines the type of the action.

Within an initial AA request the AF may use the Specific-Action AVP to request any specific actions from the server at the bearer events and to limit the contact to such bearer events where specific action is required. If the Specific-Action AVP is omitted within the initial AA request, no notification of any of the events defined below is requested at this time.

For one time specific actions, as identified in the value descriptions below, the AF may also provide the Specific-Action AVP with the applicable one-time-specific-action value(s) in subsequent AA-Requests. Non-one-time-specific-action value(s) may only be provided in the initial AA-Request and shall then be applicable for the entire lifetime of the Rx session.

NOTE 1:	One time specific actions are reported once the required action is fulfilled and are not reported again unless the AF sends a new request.

NOTE 2:	Unless otherwise stated in the definition of the specific action value, when the AF requests specific actions in the initial AA-Request, the PCRF reports that action whenever new related information is available during the lifetime of the Rx session.

NOTE 2a:	Whether the PCRF decides to report INDICATION_OF_RELEASE_OF_BEARER (4) or INDICATION_OF_FAILED_RESOURCES_ALLOCATION (9) upon receipt of a bearer failure from the PCEF is left to the implementation.

The following values are defined:

Void (0)

- CHARGING_CORRELATION_EXCHANGE (1)

Within a RAR, this value shall be used when the server reports the access network charging identifier to the AF. The Access-Network-Charging-Identifier AVP shall be included within the request. In the AAR, this value indicates that the AF requests the server to provide the access network charging identifier to the AF for each authorized flow, when the access network charging identifier becomes known at the PCRF.

- INDICATION_OF_LOSS_OF_BEARER (2)

Within a RAR, this value shall be used when the server reports a loss of a bearer (in the case of GPRS PDP context bandwidth modification to 0 kbit for GBR bearers) to the AF. The SDFs that are deactivated as a consequence of this loss of bearer shall be provided within the Flows AVP. In the AAR, this value indicates that the AF requests the server to provide a notification at the loss of a bearer.

- INDICATION_OF_RECOVERY_OF_BEARER (3)

Within a RAR, this value shall be used when the server reports a recovery of a bearer (in the case of 3GPP-GPRS or 3GPP-EPS when PGW interoperates with a Gn/Gp SGSN, PDP context bandwidth modification from 0 kbit to another value for GBR bearers) to the AF. The SDFs that are re-activated as a consequence of the recovery of bearer shall be provided within the Flows AVP. In the AAR, this value indicates that the AF requests the server to provide a notification at the recovery of a bearer.

- INDICATION_OF_RELEASE_OF_BEARER (4)

Within a RAR, this value shall be used when the server reports the release of a bearer (e.g. PDP context removal for 3GPP-GPRS or bearer/PDP context removal for 3GPP-EPS) to the AF. The SDFs that are deactivated as a consequence of this release of bearer shall be provided within the Flows AVP. In the AAR, this value indicates that the AF requests the server to provide a notification at the removal of a bearer. The content version corresponding to the affected media component may be provided in the Content-Version AVP included within the Flows AVP.

Void (5)

- IP-CAN_CHANGE (6)

This value shall be used in RAR command by the PCRF to indicate a change in the IP-CAN type or RAT type (if applicable). When used in an AAR command, this value indicates that the AF is requesting subscription to IP-CAN change and RAT change notification. When used in RAR it indicates that the PCRF generated the request because of an IP-CAN or RAT change. IP-CAN-Type AVP, RAT-Type AVP (if applicable) ,AN-Trusted AVP (if applicable) and AN-GW-Address AVP (if applicable) shall be provided in the same request with the new/valid value(s).

If an IP-CAN type or RAT type change is due to IP flow mobility and a subset of the flows within the AF session is affected, the affected service data flows shall be provided in the same request.

- INDICATION_OF_OUT_OF_CREDIT (7)

Within a RAR, this value shall be used when the PCRF reports to the AF that SDFs have run out of credit, and that the termination action indicated by the corresponding Final-Unit-Action AVP applies (3GPP TS 32.240 [23] and 3GPP TS 32.299 [24]. The SDFs that are impacted as a consequence of the out of credit condition shall be provided within the Flows AVP. In the AAR, this value indicates that the AF requests the PCRF to provide a notification of SDFs for which credit is no longer available. Applicable to functionality introduced with the Rel8 feature as described in clause 5.4.1.

- INDICATION_OF_SUCCESSFUL_RESOURCES_ALLOCATION (8)

Within a RAR, this value shall be used by the PCRF to indicate that the resources requested for particular service information have been successfully allocated. The SDFs corresponding to the resources successfully allocated shall be provided within the Flows AVP and the content version within the Content-Version AVP as included when the corresponding media component was provisioned.

In the AAR, this value indicates that the AF requests the PCRF to provide a notification when the resources associated to the corresponding service information have been allocated.

Applicable to functionality introduced with the Rel8 feature as described in clause 5.4.1.

NOTE 3:	This value applies to applications for which the successful resource allocation notification is required for their operation since subscription to this value impacts the resource allocation signalling overhead towards the PCEF/BBERF.

- INDICATION_OF_FAILED_RESOURCES_ALLOCATION (9)

Within a RAR, this value shall be used by the PCRF to indicate that the resources requested for a particular service information cannot be successfully allocated. The SDFs corresponding to the resources that could not be allocated shall be provided within the Flows AVP. In case of session modification failure, the status of the related service information may be reported in the Media-Component-Status AVP included within the Flows AVP and the content version within the Content-Version AVP as included when the corresponding media component was provisioned.

In the AAR, this value indicates that the AF requests the PCRF to provide a notification when the resources associated to the corresponding service information cannot be allocated. Applicable to functionality introduced with the Rel8 feature as described in clause 5.4.1.

NOTE 4:	This value applies to applications for which the unsuccessful resource allocation notification is required for their operation since subscription to this value impacts the resource allocation signalling overhead towards the PCEF/BBERF.

- INDICATION_OF_LIMITED_PCC_DEPLOYMENT (10)

Within a RAR, this value shall be used when the PCRF reports the limited PCC deployment (i.e. dynamically allocated resources are not applicable) as specified at Annex K and Annex L in 3GPP TS 23.203 [2] to the AF. In the AAR, this value indicates that the AF requests the PCRF to provide a notification for the limited PCC deployment. Applicable to functionality introduced with the Rel8 feature as described in clause 5.4.1.

- USAGE_REPORT (11)

In the RA-Request (RAR), this value shall be used by the PCRF to report accumulated usage volume and/or time of usage when the usage threshold provided by the AF has been reached.

In the AA-Request (AAR), this value indicates that the AF requests PCRF to report accumulated usage volume and /or time of usage when it reaches the threshold.

Applicable to functionality introduced with the SponsoredConnectivity feature for volume usage reporting and with SCTimeBased UM feature for time usage reporting as described in clause 5.4.1.

- ACCESS_NETWORK_INFO_REPORT (12)

In the RA-Request (RAR), this value shall be used by the PCRF to report access network information (i.e.user location and/or user timezone information) when the PCRF receiving an Access Network Information report corresponding to the AF session from the PCEF/BBERF.

In the AA-Request (AAR), this value indicates that the AF requests PCRF to report one time access network information when the PCRF receives the first Access Network Information report corresponding to the AF session from the PCEF/BBERF after the AF request for the access network information. The required access information is provided within the Required-Access-Info AVP. Applicable to functionality introduced with the NetLoc feature as described in clause 5.4.1.

The Specific-Action AVP with this value indicates a one time specific action.

- INDICATION_OF_RECOVERY_FROM_LIMITED_PCC_DEPLOYMENT (13)

Within a RAR, this value shall be used when the PCRF reports the recovery from limited PCC deployment (i.e. the UE moves from the VPLMN to the HPLMN as specified at Annex K in 3GPP TS 23.203 [2]) to the AF. In the AAR, this value indicates that the AF requests the PCRF to provide a notification for the recovery from limited PCC deployment. Applicable to functionality introduced with the Rel8 feature as described in clause 5.4.1.

NOTE 5:	This value is optional and only applicable to the scenario where PCC is deployed in the HPLMN but	not in the VPLMN and dynamic policy provisioning only occurs in the home routed roaming cases if no BBERF is employed.

- INDICATION_OF_ACCESS_NETWORK_INFO_REPORTING_FAILURE (14)

In the RAR, this value shall be used when the PCRF reports the access network information reporting failure. When applicable, the NetLoc-Access-Support AVP may be provided as well to indicate the reason for the access network information reporting failure. This specific action does not require to be provisioned by the AF. Applicable to functionality introduced with the NetLoc feature as described in clause 5.4.1.

- INDICATION_OF_TRANSFER_POLICY_EXPIRED (15)

In the RAR, this value shall be used when the PCRF determines that the transfer policy has expired. This specific action does not require to be provisioned by the AF.

PLMN_CHANGE (16)

In the AA-Request (AAR), this value indicates that the AF requests PCRF to report changes of PLMN. In the RA-Request (RAR), this value shall be used by the PCRF to indicate that there was a change of PLMN. 3GPP-SGSN-MCC-MNC AVP shall be provided in the same RAR command with the new value. Applicable to functionality introduced with the PLMNInfo feature as described in subclause 5.4.1.

- EPS_FALLBACK (x1)

In the RA-Request (RAR), this value shall be used by the PCF to report EPS Fallback for the resources requested for a particular service information (media type voice). The SDFs corresponding to the resources for which the access network initiated fallback to EPS shall be provided within the Flows AVP.

In the AA-Request (AAR), this value indicates that the AF requests the PCF to provide a notification when the access network initiates EPS Fallback for the requested resources associated to service information for voice media type.

Applicable to functionality introduced with the EPSFallbackReport feature as described in clause 5.4.1.

- *** 2nd Change ***

### 5.4.1	Use of the Supported-Features AVP on the Rx reference point

The Supported-Features AVP is used during session establishment to inform the destination host about the required and optional features that the origin host supports. The client shall, in the first request of a Diameter session indicate the set of supported features. The server shall, in the first answer within the Diameter session indicate the set of features that it has in common with the client and that the server shall support within the same Diameter session. Any further command messages shall always be compliant with the list of supported features indicated in the Supported-Features AVPs during session establishment. Features that are not advertised as supported shall not be used to construct the command messages for that Diameter session. Unless otherwise stated, the use of the Supported-Features AVP on the Rx reference point shall be compliant with the requirements for dynamic discovery of supported features and associated error handling on the Cx reference point as defined in clause 7.2.1 of 3GPP TS 29.229 [25].

The base functionality for the Rx reference point is the 3GPP Rel-7 standard and a feature is an extension to that functionality. If the origin host does not support any features beyond the base functionality, the Supported-Features AVP may be absent from the Rx commands. As defined in clause 7.1.1 of 3GPP TS 29.229 [25], when extending the application by adding new AVPs for a feature, the new AVPs shall have the M bit cleared and the AVP shall not be defined mandatory in the command ABNF.

As defined in 3GPP TS 29.229 [25], the Supported-Features AVP is of type grouped and contains the Vendor-Id, Feature-List-ID and Feature-List AVPs. On the Rx reference point, the Supported-Features AVP is used to identify features that have been defined by 3GPP and hence, for features defined in this document, the Vendor-Id AVP shall contain the vendor ID of 3GPP (10415). If there are multiple feature lists defined for the Rx reference point, the Feature-List-ID AVP shall differentiate those lists from one another.

On receiving an initial request application message, the destination host shall act as defined in clause 7.2.1 of 3GPP TS 29.229 [25]. The following exceptions apply to the initial and stateless AAR/AAA command pair:

-	If the AF supporting post-Rel-7 Rx functionality is able to interoperate with a PCRF supporting Rel-7, the AAR shall include the features supported by the AF within Supported-Features AVP(s) with the ‘M’ bit cleared. Otherwise, the AAR shall include the supported features within the Supported-Features AVP(s) with the M-bit set.

NOTE 1:	One instance of Supported-Features AVP is needed per Feature-List-ID.

-	If the AAR command does not contain any Supported-Features AVP(s) and the PCRF supports Rel-7 Rx functionality, the AAA command shall not include the Supported-Features AVP. In this case, both AF and PCRF shall behave as specified in the Rel-7 version of this document.

-	If the AAR command contains the Supported-Features AVP(s), the PCRF shall include the Supported-Features AVP(s) in the AAA command, with the ‘M’ bit cleared, indicating only the features that both the PCRF and AF support. In this case, the PCRF should not use the 'M' bit setting of the Supported-Features AVP(s) to determine if the AAR is accepted or rejected.

NOTE 2:	The client will always declare all features that are supported according to table 5.4.1.1. When more than one feature identifying a release is supported by both AF and PCRF, the AF will work according to the latest common supported release.

Once the PCRF and AF have negotiated the set of supported features during session establishment, the set of common features shall be used during the lifetime of the Diameter session.

The table below defines the features applicable to the Rx interfaces for the feature list with a Feature-List-ID of 1.

- Table 5.4.1.1: Features of Feature-List-ID 1 used in Rx

| Feature bit | Feature | M/O | Description |
| --- | --- | --- | --- |
| 0 | Rel8 | M | This feature indicates the support of the base 3GPP Rel-8 functionality, including the AVPs and corresponding procedures supported by the base 3GPP Rel-7 Rx standard, but excluding those features represented by separate feature bits. AVPs introduced with this feature are marked with "Rel8" in Table 5.4.0.1. |
| 1 | Rel9 | M | This feature indicates the support of the base 3GPP Rel-9 functionality, including the AVPs and corresponding procedures supported by the Rel8 feature bit, but excluding those features represented by separate feature bits. |
| 2 | ProvAFsignalFlow | O | This indicates support for the feature of provisioning of AF signalling flow information as described in clause 4.4.5a. If the PCRF supports this feature the AF may provision AF signalling flow information. NOTE: This feature is used by the IMS Restoration Procedures to provide to the PDN-Gateway the address of the P-CSCF selected by the UE, refer to TS 23.380 [28]. |
| 3 | SponsoredConnectivity | O | This feature indicates support for sponsored data connectivity feature. If the PCRF supports this feature, the AF may provide sponsored data connectivity to the subscriber. |
| 4 | Rel10 | M | This feature indicates the support of the base 3GPP Rel-10 functionality, including the AVPs and corresponding procedures supported by the Rel8 and Rel9 feature bit, but excluding those features represented by separate feature bits. AVPs introduced with this feature are marked with "Rel10" in table 5.3.0.1. |
| 5 | NetLoc | O | This feature indicates the support of the Access Network Information Reporting. |
| 6 | ExtendedFilter | O | This feature indicates the support for the local (i.e. UE) address and mask being present in filters signalled between network and UE. |
| 7 | SCTimeBasedUM | O | This feature indicates support for sponsored data connectivity feature with time-based usage monitoring control required. If the PCRF supports this feature, the AF may provide time threshold for the usage monitoring control. |
| 8 | Netloc-Trusted-WLAN | O | This feature indicates the support for the Trusted WLAN access.It requires that NetLoc feature is also supported. |
| 9 | RAN-NAS-Cause | O | This feature indicates the support for the release cause code information (NOTE 1) from the access network. |
| 10 | GroupComService | O | This feature indicates the support of Group Communication services as described in TS 23.468 [36] for unicast services. |
| 11 | ResShare | O | This feature indicates the support of resource sharing among several AF sessions. |
| 12 | DeferredService | O | This feature indicates the support of deferred transfer of service information from the AF. |
| 13 | DSCP | O | This feature indicates that the AF may provide a DSCP value when describing a service flow by supplying the ToS-Traffic-Class AVP. |
| 14 | SponsorChange | O | This feature indicates that the AF provides information on whether it wants to enable or disable/not enable sponsoring a service. It requires that SponsoredConnectivity is also supported. |
| 15 | E2EQOSMTSI | O | This feature indicates that the AF supports QoS End-to-end MTSI extensions as defined in 3GPP TS 26.114 [41] |
| 16 | NetLoc-Untrusted-WLAN | O | This feature indicates the support of the Untrusted WLAN access as described in 3GPP TS 23.203 [2]. It requires that NetLoc feature is also supported. |
| 17 | MCPTT | O | This feature indicates the support of Mission Critical Push To Talk services as described in 3GPP TS 23.179 [44] |
| 18 | PrioritySharing | O | This feature indicates that Priority Sharing is supported as described in 3GPP TS 23.203 [2], subclause 6.1.19. |
| 19 | PLMNInfo | O | This feature indicates that reporting on changes of PLMN info is supported. |
| 20 | MediaComponentVersioning | O | This feature indicates the support of media component versioning as defined in subclause 4.4.9. |
| 21 | MCPTT-Preemption | O | This feature indicates the support of service pre-emption based on the information provided by the AF. It requires that both PrioritySharing and MCPTT features are also supported. |
| 22 | MCVideo | O | This feature indicates the support of Mission Critical Video services as described in 3GPP TS 23.281 [61]. |
| Feature bit: The order number of the bit within the Feature-List AVP where the least significant bit is assigned number "0". Feature: A short name that can be used to refer to the bit and to the feature, e.g. "EPS". M/O: Defines if the implementation of the feature is mandatory ("M") or optional ("O"). Description: A clear textual description of the feature. NOTE 1: In this release, the release cause code information from the access network can include RAN/NAS release cause(s), a TWAN release cause or an untrusted WLAN release cause. |  |  |  |

- Table 5.4.1.2: Features of Feature-List-ID 2 used in Rx

| Feature bit | Feature | M/O | Description |
| --- | --- | --- | --- |
| 0 | PCSCF-Restoration-Enhancement | O | This feature indicates support of P-CSCF Restoration Enhancement. It is used for the PCRF and the P-CSCF to indicate if they support P-CSCF Restoration Enhancement. |
| 1 | Extended-Max-Requested-BW-NR | O | This feature indicates the support of the extended Max-Requested-Bandwidth for NR. |
| 2 | Extended-Min-Requested-BW-NR | O | This feature indicates the support of the extended Min-Requested-Bandwidth for NR. It requires that Rel-10 feature is also supported. |
| 3 | Extended-BW-E2EQOSMTSI-NR | O | This feature indicates the support of the extended E2EQOSMTSI bandwidth values for NR. It requires that E2EQOSMTSI feature and the Extended-Max-Requested-BW-NR are also supported. |
| 4 | VBC | O | This feature indicates the support of Volume Based Charging of IMS services as defined in subclause A.16. |
| 5 | CHEM | O | This feature indicates the support of Coverage and Handover Enhancements for Media (CHEM) |
| 6 | VBCLTE | O | This feature indicates the support of providing the caller and callee information as defined in subclause A.16. |
| x1 | EPSFallbackReport | O | This feature indicates the support of the report of EPS Fallback as defined in subclause E.x1. |
| Feature bit: The order number of the bit within the Feature-List AVP where the least significant bit is assigned number "0". Feature: A short name that can be used to refer to the bit and to the feature, e.g. "EPS". M/O: Defines if the implementation of the feature is mandatory ("M") or optional ("O"). Description: A clear textual description of the feature. |  |  |  |

- *** 3rd Change ***

# E.x1	Reporting EPS Fallback

When the feature "EPSFallbackReport" is supported, if the AF requests the PCF to report EPS Fallback, the AF shall subscribe to the "EPS_FALLBACK" within the Specific-Action AVP. The AF shall request the PCF to report the EPS Fallback in conjuction with providing the PCF with the AF session information for voice media type, as described in subclause 4.4.1 for the initial provisioning of session information, or as described in subclause 4.4.2 for the modification of session information.

When the PCF then receives the EPS Fallback indication from the SMF, the PCF shall provide the corresponding EPS Fallback indication to the AF by sending an RAR (re-authorization request) command. The RAR command shall include the IP flows of the resources for which the access network initiated EPS fallback encoded in the Flows AVP, and the EPS_FALLBACK cause encoded in the Specific-Action AVP.

When the AF receives the RAR command, it shall acknowledge the command by sending an RAA (re-authorization answer) command to the PCF.

*** End of Changes ***