3GPP TSG-CT3 Meeting #112e	C3-205295

- E-Meeting, 04th – 13th November 2020										(Revision of C3-204xyz)

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 29.675 | CR | 0018 | rev | 1 | Current version: | 17.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correct UCMF id |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson |  |  |  |  |  |  |  |  |  |
| Source to TSG: | C3 |  |  |  |  |  |  |  |  |  |
| Work item code: | RACS |  |  |  |  |  |  | Date: |  | 2020-10-14 |
| Category: | A |  |  |  |  |  |  | Release: |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | Clause 4.2.3.2 mentions: In order to modify a UE radio capability provisioning resource, i.e. add new UCMF dictionary entries, update and/or remove the existing UCMF dictionary entries for one or more UCMF ID(s),… It is misleading, the UCMF ID(s) cannot be modified. It should be RACS ID(s) |  |  |  |  |  |  |  |  |
| Summary of change: |  | Change UCMF ID to RACS ID in the resource modification (4.2.3.2). |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Incorrect procedure description may cause interoperability issue. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 4.2.3.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  | TS/TR ... CR ... |  |  |  |
| affected: |  |  | X | Test specifications |  |  | TS/TR ... CR ... |  |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  | TS/TR ... CR ... |  |  |  |
| Other comments: |  | This CR doesn’t have openAPI impact. |  |  |  |  |  |  |  |  |

| This CR's revision history: |  |
| --- | --- |

- Additional discussion(if needed):

…

- Proposed changes:

- *** 1st Change ***

#### 4.2.3.2	Modifying a UE radio capability provisioning resource

Figure 4.2.3.2-1: Modifying a UE radio capability provisioning resource

In order to modify a UE radio capability provisioning resource, i.e. add new UCMF dictionary entries, update and/or remove the existing UCMF dictionary entries for one or more RACS ID(s), the NF service consumer shall send an HTTP PUT or PATCH request to the UCMF with: "{apiRoot}/nucmf-provisioning/<apiVersion>/provisionings/{provisioningId}" as request URI as shown in step 1 of figure 4.2.3.2-1, and the "RacsData" or "RacsDataPatch" data structure as request body, correspondingly. The "RacsData" or "RacsDataPatch" data type shall contain one or more RACS configurations in the "racsConfigs" attribute which include:

-	a RACS ID in the "racsId" attribute;

-	UE radio capability information in the "racsParamEps" and/or "racsParam5Gs" attributes; and

-	the related UE model(s) IMEI-TAC value(s) in the "imeiTacs" attribute.

Upon reception of the HTTP PUT message from the NF service consumer, the UCMF shall start replacing the existing UE radio capability provisioning resource and update the corresponding UCMF dictionary entr(ies).

Upon reception of the HTTP PATCH message from the NF service consumer, the UCMF shall start modifying the existing UE radio capability provisioning resource:

-	remove RACS ID and its UE radio capability information with a NULL map key value, and the corresponding UCMF dictionary entry;

-	replace the UE radio capability information for any existing RACS ID in the resource, and the corresponding UCMF dictionary entry; and/or

-	create a UCMF dictionary entry for any new RACS ID if there is no ID conflict with other existing resources.

After processing all requested RACS IDs, if at least one UCMF dictionary entry is successfully provisioned, the UCMF shall respond "200 OK" as shown in step 2 of figure 4.2.3.2-1 with the successfully provisioned RACS information, the UCMF may include RACS report(s) within attribute "racsReports" with a list of RACS ID(s) and the corresponding failure code for which the provisioning has failed as specified in table 5.16.2.2.3-1 of 3GPP TS 29.122 [15] in the body of the HTTP response; otherwise the UCMF shall respond "500 Internal Server Error" and may include the RACS report(s) to indicate failure details.

*** End of Changes ***