3GPP TSG CT WG3 Meeting #112e	C3-205abc

- E-Meeting, 4th-13th November 2020												(revision of C3-205133)

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 29.251 | CR | 0030 | rev | 1 | Current version: | 16.2.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Optimized Pull mode |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | China Telecom, Ericsson |  |  |  |  |  |  |  |  |  |
| Source to TSG: | CT3 |  |  |  |  |  |  |  |  |  |
| Work item code: | pfdManEnh |  |  |  |  |  | Date: |  |  | 2020-10-21 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | For existing specifications of TS 29.251, when pull mode is initiated, the PFDF will reponse with all PFDs of application identifier(s), which will increase signalling load when partial PFDs are updated. So we propose an optimized method to decrease signalling load by introduce partial pull procedures with partial-flag and/or updated PFDs instead of all PFDs. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Add partial update mechanism in pull mode with procedure descriptions |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | May cause the overload of signalling between PCEF and PFDF. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 4.1, 4.4.1; 6.3.3.1, 6.3.3.x(new), 6.3.5.1, 6.4.y(new), A.1 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | This CR introduces a backwards compatible feature to the OpenAPI file. |  |  |  |  |  |  |  |  |
| This’CR's revision history: |  |  |  |  |  |  |  |  |  |  |

- Additional discussion(if needed):

…

- Proposed changes:

- First change

- 4.1	Overview

The Gw reference point is located between the Packet Flow Description Function (PFDF) and the Policy and Charging Enforcement Function (PCEF). The Gw reference point is used for retrieval, provisioning and removal of PFDs from the PFDF to the PCEF and reporting the handling result of PFDs from the PCEF to the PFDF.

The stage 2 level requirements for the Gw reference point are defined in 3GPP TS 23.203 [2].

Next change

### 4.4.1	Request for PFDs ("Pull mode" and "Combination mode")

At the time a PCC Rule with an application identifier for which PFDs provisioned by the PFDF are not available is activated or provisioned, or when the caching timer for an application identifier elapses and a PCC Rule for the application identifier is still active, the PCEF shall request PFDs for the application identifier from the PFDF.

The PCEF shall send an HTTP GET message to the PFDF to retrieve the PFDs for one or more application identifier(s) as defined in subclauses 6.3.3.2, 6.3.3.3 and 6.3.3.4.The PCEF may send an HTTP POST message to the PFDF for partial retrieval of the PFDs for one or more application identifier(s), if request to update the PFDs existing in PCEF, if the PartialPull feature is supported in the PFDF and PCEF.

Upon receipt of the HTTP request for the pull operation, the PFDF shall respond to the PCEF indicating whether the pull operation was successful or not by using one of the HTTP status codes; if the pull operation is successful, the PFDF:

-	if there are no PFDs changed, shall not provide the specified application identifier(s) in the body of the HTTP response; and

-	if the PFDs of the specified application identifier(s) is removed, shall not provide any PFDs for the specified removed application identifier(s).

-	if there are partial PFDs of the specified application identifier(s) changed, shall provide the partial-flag and set to true and updated PFDs.

-	if all PFDs changed of the specified application identifier(s), shall provide all the new PFD(s).

-	if there are caching time value(s) configured for any of the specified application identifier(s), shall set the caching-time to the caching time value(s) for those application identifier(s) in the body of the HTTP response.

Upon receipt of the HTTP response for the pull operation from the PFDF, The PCEF shall

- for an application identifier(s) not provided, reserve all the existing PFD(s); or

-	for an application identifier(s) where partial-flag is provided and set to true

-	install a new PFD(s) if the new PFD(s) with a new PFD identifier(s) is received,

-	update an existing PFD(s) if a new PFD(s) with the same PFD identifier(s) is recevied, and/or

-	delete an existing PFD(s) if the same PFD identifier(s) without any content is received.

-	for an application identifier(s) where no flag and no PFD(s) is provided, remove all existing PFD(s) and install all the new provided PFD(s) if provided by PFDF.

The PCEF shall also for each application identifier start a caching timer with a value

-	according to the received caching time value for that application identifier, if such a value is received for that application identifier; or

-	otherwise with a default caching time value configured in the PCEF.

NOTE 1:	The PCEF(s) and the PFDF(s) within an operator network are configured with the same default caching time value to be applied for all application identifiers.

NOTE 2:	In the combination mode, the HTTP POST message from the PFDF does not impact the running caching timer of an application identifier.

When the PCEF removes the last PCC rule that refers to the corresponding application identifier, or when the caching timer expires and no PCC rule refers to the application identifier, the PCEF may remove the PFD(s) related with the application identifier.

The PFDs retrieved from PFDF take precedence over any PFDs pre-configured in the PCEF. If all PFDs retrieved from the PFDF are removed for an application identifier, the pre-configured PFDs shall be applied again for the application identifier.

Next change

#### 6.3.3.1	General

The PFDs pull procedure is performed through HTTP transactions consisting of a request initiated by the PCEF/TDF and its corresponding response provided by the PFDF. While the PFDs push procedure is initiated by the PFDF and answered by the PCEF/TDF

Table 6.3.3.1-1 summarizes the content of the requests and responses. More detailed information is specified in the corresponding subclauses as indicated in the table.

Table 6.3.3.1-1: Gw/Gwn requests/response summary table

| Method | Resource URI’s path | Clause Defined | Request body | Initiator | Response body |
| --- | --- | --- | --- | --- | --- |
| GET | /gwapplication/pfds/{application-identifier} (NOTE 1) | 6.3.3.2 | None | PCEF/TDF | Successful response: The PFDF shall include the representation of the corresponding PDFs of the specified application identifier in the body of the response as per Annex A.1 |
| GET | /gwapplication/pfds?{query-parameters} (NOTE 1) | 6.3.3.3 | None | PCEF/TDF | Successful response: The PFDF shall include the representation of the corresponding PDFs for a set of application identifier(s) in the body of the response as per Annex A.1 |
| GET | /gwapplication/pfds (NOTE 1) | 6.3.3.4 | None | PCEF/TDF | Successful response: The PFDF shall include the representation of the corresponding PDFs for all application identifier(s) in the body of the response as per Annex A.1 |
| POST | /gwapplication/provisioning (NOTE 2) | 6.3.3.5 | Content-Type: application/json The PFDF may include PFDs content and/or notification associated with application identifier(s) using the schema defined in Annex A.2. | PFDF | Successful response: The PCEF/TDF may include informational data in the body of the response as per Annex A.3 |
| POST | /gwapplication/partialpull (NOTE 2) | 6.3.3.x(new) | Content-Type: application/json The PCEF/TDF may include application identifier(s) using the schema defined in Annex A.x. | PCEF/TDF | Successful response: The PFDF shall include the representation of the corresponding updated PFD(s) for the requested application identifier(s) in the body of the response as per Annex A.1. If no PFDs is changed, the application identifier is not included. |
| NOTE 1: A different path from /gwapplication/pfds/ may be used when it is configured in the PCEF/TDF. In that case the "path" part set in the different methods should use the configured one. NOTE 2: A different path from /gwapplication/provisioning may be used when it is configured in the PFDF. In that case the "path" part set in the different methods should use the configured one. |  |  |  |  |  |

Next change

- 6.3.3.x(new)		POST /gwapplication/partialpull

The partialpull of the PFDs shall be performed by the PCEF by using the POST method as follows:

-	The request URI formatted as defined in subclause 6.3.2 with the "path" part set to: /gwapplication/partialpull.

-	The Content-Type header field set to "application/json"

-	The body of the message encoded in JSON format as defined in Annex A.x. The body shall include

-	for the PFD(s) partial pull for an existing application identifier(s),

Upon receipt of the HTTP POST, the PFDF shall respond to the PCEF/TDF indicating whether the partialpull was successful or not using one of the HTTP status codes as defined in subclause 6.3.4. If the partialpull was accepted, the PFDF shall respond with an HTTP 200 OK status code if no resource is created, or an HTTP 201 Created status code if one or more resources are created. If the provisioning was rejected, the PFDF shall indicate the reason using an appropriate HTTP status code for as defined in subclause 6.3.4 and optionally additional information in the body of the response as defined in Annex A.3. If the pfd-reports is included, the PCEF/TDF shall identify the failed PFDs by including the application-identifier and specify the failed reason code by including a pfd-failure-code field.

Depending on the value of the pfd-failure-code field, the PCEF/TDF may partialpull the PFDs again.

Editor’s Note:	How to define the schema for partial pull in Annex A.x is for FFS.

Below is an example of an HTTP POST and a corresponding successful response:

POST /gwapplication/partialpull HTTP/1.1

Host: pfdfserver.example.com

Content-Type: application/json

Content-Length: …

[

{

"application-identifier":"test-application-1",

},

{

"application-identifier":"test-application-2",

},

{

"application-identifier":"test-application-3",

},

]

Here is an example of a successful response:

HTTP/1.1 200 OK

Server: pfdfserver.example.com

Content-Type: application/json

Content-Length: …

{

"application-identifier":"test-application-1",

"caching-time":200000,

"pfds":[

{

"pfd-identifier":"pfd1",

"flow-descriptions":[

"permit in ip from 10.68.28.39 80 to any",

"permit out ip from any to 10.68.28.39 80"

]

},

{

"pfd-identifier":"pfd2",

"urls":[

"^http://test.example.com(/\\S*)?$"

]

}

]

Next change

- 6.3.5.1	General

The REST based Gw/Gwn interface needs to provide a mechanism to advertise required and optional features supported by both the PCEF/TDF and PFDF for interoperability reasons as the functionality of the REST Gw/Gwn based interface is augmented.

Feature negotiation shall take place during the first interaction (either HTTP POST in push or HTTP GET in pull) between the PFDF and the PCEF/TDF. The client shall include in the HTTP request the set of supported features as follows:

-	if a feature is required for the proper operation of the application, it shall be included within the 3gpp-Required-Features header;

-	if a feature is optional for the proper operation of the application, it shall be included within the 3gpp-Optional-Features header.

The server shall include, within the 3gpp-Accepted-Features header in the HTTP response, the set of features it supports in common with the client.

If the server does not support any of the required features advertised by the client within the 3gpp-Required-Features header, the server shall reject the HTTP request with an HTTP 412 Precondition Failed status code and shall include the commonly supported features with the client within the 3gpp-Accepted-Features.

If the server requires certain features to be supported that are not advertised by the client, the server shall reject the HTTP request with an HTTP 412 Precondition Failed status code and shall include the commonly supported features with the client within the 3gpp-Accepted-Features and the required features in the 3gpp-required-features.

If the PCEF/TDF and the PFDF successfully negotiate supported features, the list of commonly supported features shall be applicable for the lifetime of the application. Features that are not advertised as supported shall not be used.

The sender may send information that is related to the supported features. Any unrecognized information shall be ignored by the receiver.

The table below defines the features applicable to the Gw interface.

Table 6.3.5.1-1: Features used in Gw Interface

| Feature | M/O | Description |
| --- | --- | --- |
| PartialUpdate | O | The PFDF can use this feature for partial update in PFD push. |
| PartialPull | O | The PCEF can use this feature for partial pull in PFD retrieval. |
| DomainNameProtocol | O | This feature supports the additional protocol matching condition for the domain name in PFD data. |
| Feature: A short name for the feature to which the M/O and description pertain. M/O: Indication on whether the implementation of the feature is mandatory ("M") or optional ("O") in this 3GPP Release. Description: Textual description of the feature. |  |  |

The table below defines the features applicable to the Gwn interface.

Table 6.3.5.1-2: Features used in Gwn Interface

| Feature | M/O | Description |
| --- | --- | --- |
| PartialUpdate | O | The PFDF can use this feature for partial update in PFD push. |
| PartialPull | O | The PCEF can use this feature for partial pull in PFD retrieval. |
| DomainNameProtocol | O | This feature supports the additional protocol matching condition for the domain name in PFD data. |
| Feature: A short name for the feature to which the M/O and description pertain. M/O: Indication on whether the implementation of the feature is mandatory ("M") or optional ("O") in this 3GPP Release. Description: Textual description of the feature. |  |  |

NOTE:	 The base functionality for the Gw/Gwn interface is defined in the Release-14 version of this specification and a feature is an extension of that functionality. The negotiation of supported features allows interworking between the endpoints of the Gw/Gwn interface whereby each entity may support all, some, or none of the features that the Gw/Gwn application can support defined in this specification. Features are defined so that they are independent of each other. Any introduced feature is explicitly defined in this specification.

Next change

- 6.4.y	JSON PartialPull fields

- 6.4.y.1	General

In addition to subclause 6.4.3, Table 6.4.y.1.1 describes the extra JSON fields used within the body of the HTTP messages representing the PFDs partialpull information in pull mode. The table includes the information about the name of the field and the type of the fields.

Table 6.4.y.1.1: Gw/Gwn PartialPull JSON fields

| Field Name | Clause defined | JSON Value Type (NOTE 1) | JCR Type (NOTE 2) | Applicability (NOTE 3) |
| --- | --- | --- | --- | --- |
| partial-flag | 6.4.4.5 | boolean | boolean | PartialPull |
| NOTE 1: The basic JSON value types are defined in IETF RFC 7159 [11]. NOTE 2: The JCR types are defined in IETF draft-newton-json-content-rules [12]. NOTE 3: Fields marked with a supported feature are applicable as described in subclause 6.3.5. |  |  |  |  |

- 6.4.y.2	partial-flag

The partial-flag is of type boolean.

If the value of the flag is true, then it indicates the PCEF/TDF supporting update the existing PFDs, or keep the existing PFDs if no changes of PFDs for the application idenfier(s).in PFDF.

Next change

# A.1	PFDs schema

This subclause defines the JSON schema for the body of HTTP responses providing the state of one or more PFDs resources in pull mode. The schema is based on IETF draft-newton-json-content-rules [12] and is defined below:

# jcr-version 0.7

# ruleset-id 3gpp.gwapplication.pfds

; JCR representing the pfds root resource

$pfds-root = @{root}{

$application-identifier,

$caching-time ?,

$partial-flag ?

$pfds ?

}

; An array list of the PFDs for multiple application identifiers

$pfds-array-root = @{root} [ $pfds-root * ]

; A flag indicates whether this is a partial update or not

$partial-flag = "partial-flag" : boolean

; The caching timer for the PFDs

$caching-time = "caching-time" : uint64

; The PFDs associated with the same application identifier

$pfds = "pfds" : [ $pfd * ]

; The PFD content

$pfd =  {

$pfd-identifier,

( $flow-descriptions | $urls | $domain-names | // : any) ?,

$dn-protocol ?

}

; The PFD identifier

$pfd-identifier = "pfd-identifier" : string

; The flow descriptions

$flow-descriptions = "flow-descriptions" : [ string + ]

; The url matching expressions

$urls = "urls" :  [ string + ]

; The domain name match criteria

$domain-names = "domain-names" : [ string + ]

; The domain name protocol

$dn-protocol =: (

"DNS_QNAME" |

"TLS_SNI" |

"TLS_SAN" |

"TLS_SCN"

)

End of changes