3GPP TSG CT WG3 Meeting #112e	C3-205181

- E-Meeting, 04th-13th November 2020                                               (revision of C3-20xxxx)

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 29.551 | CR | 0050 | rev | - | Current version: | 17.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | PULL mode enhancement |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, Ericsson |  |  |  |  |  |  |  |  |  |
| Source to TSG: | CT3 |  |  |  |  |  |  |  |  |  |
| Work item code: | pfdManEnh |  |  |  |  |  | Date: |  |  | 2020-11-04 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | In the case that the NF service consumer retrieves PFDs for an Application Identifier(s) from the PFDF when the caching timer for an application identifier elapses and a PCC rule for this application identifier is still active, the PFDF does not need to provide the full list of the PFD(s). As the different NF service consumer may request the PFDs based on the caching timer managed itself and have different delay of the PFD provisioning, it is easy for the PFDF to determine whether the PFD(s) has been changed if the NF service consumer can provide the timestamp of the last provisioning. The timestamp is provided by the PFDF when the PFDF provisions the PFD(s). If the PFDF determine that the PFD(s) is not changed based on the timestamp, the PFDF returns the response message with "204 No Content"; otherwise, the PFDF returns the response message with "200 OK"and includes the new or updated PFD(s) or requests to remove the PFD(s) |  |  |  |  |  |  |  |  |
| Summary of change: |  | - Define the new procedure to retrieve the PFD(s) by partial update. - Define the new resource and method to retrieve the PFD(s) by partial update. - Define the new data type ApplicationForPfdRequest to request the PFD(s) - Define the new supported feature. - Update the OpenAPI file. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The PFDF always provisions the full list of the PFD(s) even the PFD(s) is not changed. The signalling load is high. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 4.2.1; 4.2.2.1; 4.2.2.2, 4.2.2.x(new); 5.3.1; 5.3.2.4, 5.3.2.4.x1(new), 5.3.2.4.x2(new), 5.3.2.4.x2.1(new), 5.3.2.4.x2.2(new),5.6.1; 5.6.2.2, 5.6.2.5-1, 5.6.2.x(new); 5.8; A.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | This CR introduces a backwards compatible feature to the OpenAPI file. |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

- Additional discussion(if needed):

…

- Proposed changes:

- *** 1st Change ***

### 4.2.1	Introduction

Service operations defined for the Nnef_PFDmanagement Service are shown in table 4.2.1-1.

Table 4.2.1-1: Nnef_PFDmanagement Service Operations

| Service Operation Name | Description | Initiated by |
| --- | --- | --- |
| Nnef_PFDmanagement_Fetch | Provides the PFDs for application identifier(s) to the NF service consumer by the full pull or partial pull. | SMF |
| Nnef_PFDmanagement_Subscribe | Allows NF service consumers to subscribe the notification of events when the PFDs for application identifier change. | SMF |
| Nnef_PFDmanagement_Notify | Notifies NF service consumer to update and/or delete the PFDs for application identifier(s) or notifies NF service consumer to retrieve the PFDs for application identifier(s). | PFDF |
| Nnef_PFDmanagement_Unsubscribe | Allows NF service consumers to unsubscribe the notification of events. | SMF |

- *** Next Change ***

#### 4.2.2.1	General

The Nnef_PFDmanagement_Fetch service operation provides a means for the NF service consumer to retrieve the PFDs for one or more application identifier(s).

The following procedures using the Nnef_PFDmanagement_Fetch service operation are supported:

-	Retrieval of PFDs by the full pull.

-	Retrieval of PFDs by the partial pull.

- *** Next Change ***

#### 4.2.2.2	Retrieval of PFDs by the full pull

This procedure as shown in Figure 4.2.2.2-1 is used to retrieve PFDs for an application identifier from the PFDF by the full pull for requested application identifier(s).

Figure 4.2.2.2-1: Retrieval of PFDs by the full pull

1.	The NF service consumer (i.e. SMF) shall send a GET request to the resource representing the PFD for application identifier(s) to be required:

-	for PFD of individual application identifier, the request URI shall include "{apiRoot}/nnef-pfdmanagement/v1/applications/{appId}"; and

-	for PFD of a collection of application identifiers, the request URI shall include "{apiRoot}/nnef-pfdmanagement/v1/applications/" with query parameters indicating the requested application identifier(s).

2.	On success, "200 OK" shall be returned; the payload body of GET response shall contain a representation of "Individual application PFD" resource or "PFD of applications" resource for the requested application identifier(s). If the resource of one or more requested application identifier(s) is not provided in the response, the SMF shall remove the PFD(s) of the requested application identifier(s) which is not included in the response.

On failure, one of the HTTP status code listed in table 5.3.2.3.1-3 or table 5.3.3.3.1-3 shall be returned. For "404 Not Found", the NF service consumer shall remove:

-	all the PFD(s) existing in the NF service consumer if the request is for PFD of all application identifiers;

-	the PFD(s) of the requested application identifier(s) in the NF service consumer if the request is for PFD of individual application identifier or a collection of application identifiers.

- *** Next Change ***

#### 4.2.2.x	Retrieval of PFDs by the partial pull

This procedure as shown in Figure 4.2.2.x-1 is used to retrieve PFDs for an application identifier from the PFDF by the partial pull for requested application identifier(s) if the "PartialPull" feature defined in subclause 5.8 is supported both by NF service consumer and PFDF. This procedure enables the NF service consumer to retrieve PFDs for an Application Identifier(s) from the PFDF when the caching timer for an application identifier elapses and a PCC rule for this application identifier is still active.

Figure 4.2.2.x-1: Retrieval of PFDs by the partial pull

1.	The NF service consumer (i.e. SMF) shall send an HTTP POST message to the resource "{apiRoot}/nnef-pfdmanagement/v1/applications/partial-pull". The SMF shall include one or more ApplicationForPfdRequest data structure in the payload body of the HTTP POST. With the ApplicationForPfdReqeust data structure, the NF service consumer shall include the application identifier within the "applicationId" attribute and the timestamp of the PFDs for the application identifier within the "pfdTimestamp" attribute.

2.	If the PFDF accepted the HTTP POST request, the PFDF shall send to the NF service consumer:

-	the HTTP "200 OK" response (as shown in figure 4.2.2.x-1, step 2a) including one or more PfdDataForApp data structure if the PFDF determines that PFD(s) corresponding to one or more application identifier is changed since the last request based on the timestamp received in the request for the application identifier. Within the PfdDataForApp data type, the PFDF shall include the application identifier within the "applicationId" attribute, the new timestamp within the "pfdTimestamp" attirbute and create/update/remove the PFDs as follows:

-	include the full list of PFD(s) within the "pfds" attribute if all the PFDs are newly created for the application identifier;

-	include the new PFD(s) with new PFD identifier(s) within the "pfds" attribute if the new PFD(s) is added for the application identifier;

-	include the new PFD(s) with existing PFD identifier(s) within the "pfds" attribute if the existing PFD(s) is updated for the application identifier;

-	include the existing PFD identifier(s) without any content within the "pfds" attribute if the existing PFD(s) is removed for the application identifier;

-	not include the "pfds" attribute if the PFD(s) is removed for the application identifier.

NOTE 1:	The PFDF does not include the PfdDataForApp data type for the application identifier whose PFD(s) is not updated since last request.

NOTE 2:	How the PFDF determines that PFD(s) is changed since the last request based on the timestamp received in the request for the application identifier is implementation specific, e.g the PFDF keeps the timestamp of the change for each PFD. If the timestamp of the change is later than the timestamp received from the SMF, the PFDF determines that the PFD is changed since the last request.

NOTE 3	If the PFDF determines that the PFDs are changed since the last request but cannot determine what changes have been applied to the individual PFD(s) for an application identifier, the PFDF can include the full list of the PFD(s) in the PfdDataForApp data type.

-	the HTTP "204 No Content" response (as shown in figure 4.2.2.x-1, step 2b) if the PFD(s) for all the requested application identifier(s) are not changed since last request.

When the NF service consumer receives the response with "200 OK" status code, the NF service consumer shall

-	install all the new provided PDF(s) for an application identifier if full list of PFD(s) is received;

-	delete the existing PFD(s) for an application identifier(s) where no PFD(s) is received;

-		install a new PFD(s) if the new PFD(s) with a new PFD identifier(s) is received;

-	update an existing PFD(s) if a new PFD(s) with the same PFD identifier(s) is received;

-	delete an existing PFD(s) if the same PFD identifier(s) without any content is received.

- *** Next Change ***

### 5.3.1	Resource Structure

Figure 5.3.1-1: Resource URI structure of the Nnef_PFDmanagement API

Table 5.3.1-1 provides an overview of the resources and applicable HTTP methods.

Table 5.3.1-1: Resources and methods overview

| Resource name | Resource URI | HTTP method or custom operation | Description |
| --- | --- | --- | --- |
| PFD of applications | //{apiRoot}/ nnef-pfdmanagement/v1 /applications | GET | Nnef_PFDmanagement_Fetch. Retrieve PFDs for all applications or for one or multiple applications with query parameter. |
|  | /applications/partial-pull | partialpull (POST) | Request PFDs for one or multiple application identifier(s) by the partial update. |
| Individual application PFD | //{apiRoot}/ nnef-pfdmanagement/v1 /applications/{appId} | GET | Nnef_PFDmanagement_Fetch. Retrieve the PFD for an application. |
| PFD subscriptions | //{apiRoot}/ nnef-pfdmanagement/v1 /subscriptions | POST | Nnef_PFDmanagement_Subscribe. Subscribe the notification of PFD changes. |
| Individual PFD subscription | {apiRoot}/ nnef-pfdmanagement/v1 /subscriptions/{subscriptionId} | PUT | Update a subscription of PFD change notification. |
| Individual PFD subscription | //{apiRoot}/ nnef-pfdmanagement/v1 /subscriptions/{subscriptionId} | DELETE | Nnef_PFDmanagement_Unsubscribe. Delete a subscription of PFD change notification. |

- *** Next Change ***

#### 5.3.2.4	Resource Custom Operations

5.3.2.4.x1	Overview

Table 5.3.2.4.x1-1: Custom operations

| Operation Name | Custom operation URI | Mapped HTTP method | Description |
| --- | --- | --- | --- |
| partialpull | /applications/partial-pull | partialpull (POST) | Request PFDs for one or multiple application identifier(s) by the partial update. |

##### 5.3.2.4.x2	Operation: request

###### 5.3.2.4.x2.1	Description

###### 5.3.2.4.x2.2	Operation Definition

This custom operation requests PFDs for one ore multiple application identifier(s) by the partial update..

This operation shall support the request data structures specified in table 5.3.2.4.x2.2-1 and the response data structure and response codes specified in table 5.3.2.4.x2.2-2.

Table 5.3.2.4.x2.2-1: Data structures supported by the POST Request Body on this resource

| Data type | P | Cardinality | Description |
| --- | --- | --- | --- |
| array(ApplicationForPfdRequest) | M | 1 | Contains the application Identifer(s) for PFDs request. |

Table 5.3.2.4.x2.2-2: Data structures supported by the POST Response Body on this resource

| Data type | P | Cardinality | Response codes | Description |
| --- | --- | --- | --- | --- |
| array(PfdDataForApp) | M | 0..N | 200 OK | The PFDs for one or more application identifier(s) are returned. |
| n/a |  |  | 204 No Content |  |
| NOTE: The mandatory HTTP error status codes for the POST method listed in table 5.2.7.1-1 of 3GPP TS 29.500 [4] shall also apply. |  |  |  |  |

- *** Next Change ***

### 5.6.1	General

This subclause specifies the application data model supported by the API.

Table 5.6.1-1 specifies the data types defined for the Nnef_PFDmanagement service based interface protocol.

Table 5.6.1-1: Nnef_PFDmanagement specific Data Types

| Data type | Section defined | Description | Applicability |
| --- | --- | --- | --- |
| NotificationPush | 5.6.2.7 | Provides the information for the NF service consumer to retrieve the PFDs. | NotificationPush |
| PfdDataForApp | 5.6.2.2 | Represents the PFDs for an application identifier. |  |
| PfdSubscription | 5.6.2.3 | Represents a PFD subscription. |  |
| PfdChangeNotification | 5.6.2.4 | Represents PFD change information. |  |
| PfdContent | 5.6.2.5 | Represents the content of a PFD for an application identifier. |  |
| PfdChangeReport | 5.6.2.6 | Represents error of PFD change. |  |
| ApplicationForPfdRequest | 5.6.2.x | Contains the application identifer(s) for the PFDs request. | PartialPull |

Table 5.6.1-2 specifies data types re-used by the Nnef_PFDmanagement service based interface protocol from other specifications, including a reference to their respective specifications and when needed, a short description of their use within the Nnef_PFDmanagement service based interface.

Table 5.6.1-2: Nnef_PFDmanagement re-used Data Types

| Data type | Reference | Comments | Applicability |
| --- | --- | --- | --- |
| ApplicationId | 3GPP TS 29.571 [10] |  |  |
| DomainNameProtocol | 3GPP TS 29.122 [11] |  |  |
| DateTime | 3GPP TS 29.571 [10] |  |  |
| SupportedFeatures | 3GPP TS 29.571 [10] |  |  |
| Uri | 3GPP TS 29.571 [10] |  |  |

- *** Next Change ***

#### 5.6.2.2	Type: PfdDataForApp

Table 5.6.2.2-1: Definition of type PfdDataForApp

| Attribute name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| applicationId | ApplicationId | M | 1 | Identifier of an application. |  |
| pfds | array(PfdContent) | C | 1..N | PFDs for the application identifier. |  |
| cachingTime | DateTime | O | 0..1 | Caching time for an application identifier. |  |
| pfdTimestamp | DateTime | O | 0..1 | The value represents the UTC time of the last provisioning by the PFDF. | PartialPull |
| NOTE: The "pfds" attribute shall be include if the "PartialPull" feature is not supported. |  |  |  |  |  |

- *** Next Change ***

#### 5.6.2.5	Type: PfdContent

Table 5.6.2.5-1: Definition of type PfdContent

| Attribute name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| pfdId | string | C | 0..1 | Identifies a PDF of an application identifier. If PartialUpdate or PartialPull feature is supported during PFD change event subscription, this attribute shall be provided by the PFDF. |  |
| flowDescriptions | array(string) | O | 1..N | Represents a 3-tuple with protocol, server ip and server port for UL/DL application traffic. (NOTE) |  |
| urls | array(string) | O | 1..N | Indicates a URL or a regular expression which is used to match the significant parts of the URL. (NOTE) |  |
| domainNames | array(string) | O | 1..N | Indicates an FQDN or a regular expression as a domain name matching criteria. (NOTE) |  |
| dnProtocol | DomainNameProtocol | C | 0..1 | Indicates the additional protocol and protocol field for domain names to be matched, it may only be provided when domainNames attribute is present. | DomainNameProtocol |
| NOTE: If a PFD contains multiple filter types, the PFD is only matched when every filter type contained in the PFD has a matching value. |  |  |  |  |  |

- *** Next Change ***

#### 5.6.2.x	Type: ApplicationForPfdRequest

Table 5.6.2.x-1: Definition of ApplicationForPfdRequest

| Attribute name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| applicationId | ApplicationId | M | 1 | Identifier of an application. |  |
| pfdTimestamp | DateTime | M | 1 | The value represents the UTC time of the last provisioning by the PFDF. |  |

- *** Next Change ***

## 5.8	Feature negotiation

The optional features in table 5.8-1 are defined for the Nnef_PFDmanagement API. They shall be negotiated using the extensibility mechanism defined in subclause 6.6 of 3GPP TS 29.500 [5].

Table 5.8-1: Supported Features

| Feature number | Feature Name | Description |
| --- | --- | --- |
| 1 | PartialUpdate | The PFDF can use this feature for partial update of PFDs. |
| 2 | DomainNameProtocol | This feature supports the additional protocol matching condition for the domain name in PFD data. |
| 3 | PfdChgSubsUpdate | The SMF can use this feature for updating the PFD change subscription. |
| 4 | NotificationPush | The PFDF can use this feature to notify the SMF to retrieve the PFDs. |
| x | PartialPull | The SMF can use this feature to retrieve the PFDs by the partial update. |

- *** Next Change ***

# A.2	Nnef_PFDmanagement API

openapi: 3.0.0

info:

title: Nnef_PFDmanagement Service API

version: 1.2.0.alpha-1

description: |

Packet Flow Description Management Service.

© 2020, 3GPP Organizational Partners (ARIB, ATIS, CCSA, ETSI, TSDSI, TTA, TTC).

All rights reserved.

#

externalDocs:

description: 3GPP TS 29.551 v17.0.0, 5G System; Packet Flow Description Management Service

url: 'http://www.3gpp.org/ftp/Specs/archive/29_series/29.551/'

#

servers:

- url: '{apiRoot}/nnef-pfdmanagement/v1'

variables:

apiRoot:

default: https://example.com

description: apiRoot as defined in subclause 4.4 of 3GPP TS 29.501

security:

- {}

- oAuth2ClientCredentials:

- nnef-pfdmanagement

paths:

/applications:

get:

summary: Retrieve PFDs for all applications or for one or multiple applications with query parameter.

tags:

- PFD of applications

operationId: Nnef_PFDmanagement_AllFetch

parameters:

- name: application-ids

description: The required application identifier(s) for the returned PFDs.

in: query

required: true

schema:

type: array

items:

$ref: 'TS29571_CommonData.yaml#/components/schemas/ApplicationId'

minItems: 1

- name: supported-features

in: query

description: To filter irrelevant responses related to unsupported features

schema:

$ref: 'TS29571_CommonData.yaml#/components/schemas/SupportedFeatures'

responses:

'200':

description: The PFDs for one or more application identifier(s) in the request URI are returned.

content:

application/json:

schema:

type: array

items:

$ref: '#/components/schemas/PfdDataForApp'

minItems: 0

'400':

$ref: 'TS29571_CommonData.yaml#/components/responses/400'

'401':

$ref: 'TS29571_CommonData.yaml#/components/responses/401'

'403':

$ref: 'TS29571_CommonData.yaml#/components/responses/403'

'404':

$ref: 'TS29571_CommonData.yaml#/components/responses/404'

'406':

$ref: 'TS29571_CommonData.yaml#/components/responses/406'

'414':

$ref: 'TS29571_CommonData.yaml#/components/responses/414'

'429':

$ref: 'TS29571_CommonData.yaml#/components/responses/429'

'500':

$ref: 'TS29571_CommonData.yaml#/components/responses/500'

'503':

$ref: 'TS29571_CommonData.yaml#/components/responses/503'

/applications/partial-pull:

post:

summary: retrieve the PFD(s) by partial update

operationId: Nnef_PFDmanagement_AppFetchPartialUpdate

tags:

- PFD of applications by partial update

requestBody:

required: true

content:

application/json:

schema:

type: array

items:

$ref: '#/components/schemas/ApplicationForPfdRequest'

minItems: 1

responses:

'200':

description: OK. Changed PFD(s) is returned

content:

application/json:

schema:

type: array

items:

$ref: '#/components/schemas/PfdDataForApp'

minItems: 1

'204':

description: The PFD(s) is not changed

'400':

$ref: 'TS29571_CommonData.yaml#/components/responses/400'

'401':

$ref: 'TS29571_CommonData.yaml#/components/responses/401'

'403':

$ref: 'TS29571_CommonData.yaml#/components/responses/403'

'404':

$ref: 'TS29571_CommonData.yaml#/components/responses/404'

'411':

$ref: 'TS29571_CommonData.yaml#/components/responses/411'

'413':

$ref: 'TS29571_CommonData.yaml#/components/responses/413'

'415':

$ref: 'TS29571_CommonData.yaml#/components/responses/415'

'429':

$ref: 'TS29571_CommonData.yaml#/components/responses/429'

'500':

$ref: 'TS29571_CommonData.yaml#/components/responses/500'

'503':

$ref: 'TS29571_CommonData.yaml#/components/responses/503'

default:

$ref: 'TS29571_CommonData.yaml#/components/responses/default'

/applications/{appId}:

get:

summary: Retrieve the PFD for an application.

tags:

- Individual application PFD

operationId: Nnef_PFDmanagement_IndAppFetch

parameters:

- name: appId

description: The required application identifier(s) for the returned PFDs.

in: path

required: true

schema:

type: string

- name: supported-features

in: query

description: To filter irrelevant responses related to unsupported features

schema:

$ref: 'TS29571_CommonData.yaml#/components/schemas/SupportedFeatures'

responses:

'200':

description: A representation of PFDs for an application in the request URI is returned.

content:

application/json:

schema:

$ref: '#/components/schemas/PfdDataForApp'

'400':

$ref: 'TS29571_CommonData.yaml#/components/responses/400'

'401':

$ref: 'TS29571_CommonData.yaml#/components/responses/401'

'403':

$ref: 'TS29571_CommonData.yaml#/components/responses/403'

'404':

$ref: 'TS29571_CommonData.yaml#/components/responses/404'

'406':

$ref: 'TS29571_CommonData.yaml#/components/responses/406'

'429':

$ref: 'TS29571_CommonData.yaml#/components/responses/429'

'414':

$ref: 'TS29571_CommonData.yaml#/components/responses/414'

'500':

$ref: 'TS29571_CommonData.yaml#/components/responses/500'

'503':

$ref: 'TS29571_CommonData.yaml#/components/responses/503'

/subscriptions:

post:

summary: Subscribe the notification of PFD changes.

tags:

- PFD subscriptions

operationId: Nnef_PFDmanagement_CreateSubscr

requestBody:

description: a PfdSubscription resource to be created.

required: true

content:

application/json:

schema:

$ref: '#/components/schemas/PfdSubscription'

callbacks:

PfdChangeNotification:

'{request.body#/notifyUri}':

post:

summary: Notification of PFD change.

tags:

- PfdChangeNotification data

operationId: Nnef_PFDmanagement_Notify

requestBody:

required: true

content:

application/json:

schema:

type: array

items:

$ref: '#/components/schemas/PfdChangeNotification'

minItems: 1

responses:

'200':

description: The PFD operation in the notification is performed and the PfdChangeReport indicates failure reason.

content:

application/json:

schema:

type: array

items:

$ref: '#/components/schemas/PfdChangeReport'

minItems: 1

'204':

description: The PFD operation in the notification is performed successfully.

'400':

$ref: 'TS29571_CommonData.yaml#/components/responses/400'

'401':

$ref: 'TS29571_CommonData.yaml#/components/responses/401'

'403':

$ref: 'TS29571_CommonData.yaml#/components/responses/403'

'404':

$ref: 'TS29571_CommonData.yaml#/components/responses/404'

'411':

$ref: 'TS29571_CommonData.yaml#/components/responses/411'

'413':

$ref: 'TS29571_CommonData.yaml#/components/responses/413'

'415':

$ref: 'TS29571_CommonData.yaml#/components/responses/415'

'429':

$ref: 'TS29571_CommonData.yaml#/components/responses/429'

'500':

$ref: 'TS29571_CommonData.yaml#/components/responses/500'

'503':

$ref: 'TS29571_CommonData.yaml#/components/responses/503'

NotificationPush:

'{request.body#/notifyUri}/notifypush':

post:

summary: Notification Push.

tags:

- NotificationPush data

operationId: Nnef_PFDmanagement_Notify

requestBody:

required: true

content:

application/json:

schema:

type: array

items:

$ref: '#/components/schemas/NotificationPush'

minItems: 1

responses:

'204':

description: Notificaiton PUSH is accepted.

'400':

$ref: 'TS29571_CommonData.yaml#/components/responses/400'

'401':

$ref: 'TS29571_CommonData.yaml#/components/responses/401'

'403':

$ref: 'TS29571_CommonData.yaml#/components/responses/403'

'404':

$ref: 'TS29571_CommonData.yaml#/components/responses/404'

'411':

$ref: 'TS29571_CommonData.yaml#/components/responses/411'

'413':

$ref: 'TS29571_CommonData.yaml#/components/responses/413'

'415':

$ref: 'TS29571_CommonData.yaml#/components/responses/415'

'429':

$ref: 'TS29571_CommonData.yaml#/components/responses/429'

'500':

$ref: 'TS29571_CommonData.yaml#/components/responses/500'

'503':

$ref: 'TS29571_CommonData.yaml#/components/responses/503'

responses:

'201':

description: The creation of a PfdSubscription resource is confirmed and a representation of that resource is returned.

content:

application/json:

schema:

$ref: '#/components/schemas/PfdSubscription'

headers:

Location:

description: 'Contains the URI of the newly created resource, according to the structure: {apiRoot}/nnef-pfdmanagement/v1/subscriptions/{subscriptionId}'

required: true

schema:

type: string

'400':

$ref: 'TS29571_CommonData.yaml#/components/responses/400'

'401':

$ref: 'TS29571_CommonData.yaml#/components/responses/401'

'403':

$ref: 'TS29571_CommonData.yaml#/components/responses/403'

'404':

$ref: 'TS29571_CommonData.yaml#/components/responses/404'

'411':

$ref: 'TS29571_CommonData.yaml#/components/responses/411'

'413':

$ref: 'TS29571_CommonData.yaml#/components/responses/413'

'415':

$ref: 'TS29571_CommonData.yaml#/components/responses/415'

'429':

$ref: 'TS29571_CommonData.yaml#/components/responses/429'

'500':

$ref: 'TS29571_CommonData.yaml#/components/responses/500'

'503':

$ref: 'TS29571_CommonData.yaml#/components/responses/503'

/subscriptions/{subscriptionId}:

put:

summary: Updates/replaces an existing subscription resource

tags:

- Individual PFD subscription

operationId: Nnef_PFDmanagement_ModifySubscr

parameters:

- name: subscriptionId

description: Identify the subscription.

in: path

required: true

schema:

type: string

requestBody:

description: Parameters to update/replace the existing subscription

required: true

content:

application/json:

schema:

$ref: '#/components/schemas/PfdSubscription'

responses:

'200':

description: OK (Successful update of the subscription)

content:

application/json:

schema:

$ref: '#/components/schemas/PfdSubscription'

'400':

$ref: 'TS29571_CommonData.yaml#/components/responses/400'

'401':

$ref: 'TS29571_CommonData.yaml#/components/responses/401'

'403':

$ref: 'TS29571_CommonData.yaml#/components/responses/403'

'404':

$ref: 'TS29571_CommonData.yaml#/components/responses/404'

'411':

$ref: 'TS29571_CommonData.yaml#/components/responses/411'

'413':

$ref: 'TS29571_CommonData.yaml#/components/responses/413'

'415':

$ref: 'TS29571_CommonData.yaml#/components/responses/415'

'429':

$ref: 'TS29571_CommonData.yaml#/components/responses/429'

'500':

$ref: 'TS29571_CommonData.yaml#/components/responses/500'

'503':

$ref: 'TS29571_CommonData.yaml#/components/responses/503'

default:

$ref: 'TS29571_CommonData.yaml#/components/responses/default'

delete:

summary: Delete a subscription of PFD change notification.

tags:

- Individual PFD subscription

operationId: Nnef_PFDmanagement_Unsubscribe

parameters:

- name: subscriptionId

description: Identify the subscription.

in: path

required: true

schema:

type: string

responses:

'204':

description: The PfdSubscription resource matching the subscriptionId was deleted successfully.

'400':

$ref: 'TS29571_CommonData.yaml#/components/responses/400'

'401':

$ref: 'TS29571_CommonData.yaml#/components/responses/401'

'403':

$ref: 'TS29571_CommonData.yaml#/components/responses/403'

'404':

$ref: 'TS29571_CommonData.yaml#/components/responses/404'

'429':

$ref: 'TS29571_CommonData.yaml#/components/responses/429'

'500':

$ref: 'TS29571_CommonData.yaml#/components/responses/500'

'503':

$ref: 'TS29571_CommonData.yaml#/components/responses/503'

components:

securitySchemes:

oAuth2ClientCredentials:

type: oauth2

flows:

clientCredentials:

tokenUrl: '{nrfApiRoot}/oauth2/token'

scopes:

nnef-pfdmanagement: Access to the Nnef_PFDmanagement API

schemas:

#

# STRUCTURED DATA TYPES

#

PfdContent:

type: object

properties:

pfdId:

type: string

description: Identifies a PDF of an application identifier.

flowDescriptions:

type: array

items:

type: string

minItems: 1

description: Represents a 3-tuple with protocol, server ip and server port for UL/DL application traffic.

urls:

type: array

items:

type: string

minItems: 1

description: Indicates a URL or a regular expression which is used to match the significant parts of the URL.

domainNames:

type: array

items:

type: string

minItems: 1

description: Indicates an FQDN or a regular expression as a domain name matching criteria.

dnProtocol:

$ref: 'TS29122_PfdManagement.yaml#/components/schemas/DomainNameProtocol'

PfdDataForApp:

type: object

properties:

applicationId:

$ref: 'TS29571_CommonData.yaml#/components/schemas/ApplicationId'

pfds:

type: array

items:

$ref: '#/components/schemas/PfdContent'

minItems: 1

cachingTime:

$ref: 'TS29571_CommonData.yaml#/components/schemas/DateTime'

pfdTimestamp:

$ref: 'TS29571_CommonData.yaml#/components/schemas/DateTime'

required:

- applicationId

PfdSubscription:

type: object

properties:

applicationIds:

type: array

items:

$ref: 'TS29571_CommonData.yaml#/components/schemas/ApplicationId'

minItems: 1

notifyUri:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Uri'

supportedFeatures:

$ref: 'TS29571_CommonData.yaml#/components/schemas/SupportedFeatures'

required:

- notifyUri

- supportedFeatures

PfdChangeNotification:

type: object

properties:

applicationId:

$ref: 'TS29571_CommonData.yaml#/components/schemas/ApplicationId'

removalFlag:

type: boolean

default: false

partialFlag:

type: boolean

default: false

pfds:

type: array

items:

$ref: '#/components/schemas/PfdContent'

minItems: 1

required:

- applicationId

NotificationPush:

type: object

properties:

appIds:

type: array

items:

$ref: 'TS29571_CommonData.yaml#/components/schemas/ApplicationId'

minItems: 1

allowedDelay:

$ref: 'TS29571_CommonData.yaml#/components/schemas/DurationSec'

required:

- appIds

PfdChangeReport:

type: object

properties:

pfdError:

$ref: 'TS29571_CommonData.yaml#/components/schemas/ProblemDetails'

applicationId:

type: array

items:

$ref: 'TS29571_CommonData.yaml#/components/schemas/ApplicationId'

minItems: 1

required:

- pfdError

- applicationId

ApplicationForPfdRequest:

type: object

properties:

applicationId:

$ref: 'TS29571_CommonData.yaml#/components/schemas/ApplicationId'

pfdTimestamp:

$ref: 'TS29571_CommonData.yaml#/components/schemas/DateTime'

required:

- applicationId

- pfdTimestamp

- *** End of Changes ***