3GPP TSG-CT3 Meeting #118e	C3-215028_r1

- E-Meeting, 11th – 15th October 2021

(revision of CP-212163)

3GPP TSG-CT WG4 Meeting #106-e	C4-215150

- E-Meeting, 11th – 15th October 2021	(revision of CP-212163)

- Source:	Ericsson

- Title:	Revised WID on CT aspects on Dynamically Changing AM Policies in the 5GC

- Document for:	Approval

Agenda Item:	17.1.1

3GPP™ Work Item Description

Information on Work Items can be found at http://www.3gpp.org/Work-Items 
See also the 3GPP Working Procedures, article 39 and the TSG Working Methods in 3GPP TR 21.900

# Title: CT aspects on Dynamically Changing AM Policies in the 5GC

## Acronym: TEI17_DCAMP

## Unique identifier: 	910009

Potential target Release: Rel-17

Note that this field above indicates the proposed Release at the time of submission of the WID to TSG approval. It can later be changed without a need to revise the WID. The updated target Release is indicated in the Work Plan.

## 1	Impacts

| Affects: | UICC apps | ME | AN | CN | Others (specify) |
| --- | --- | --- | --- | --- | --- |
| Yes |  |  |  | X |  |
| No | X | X | X |  | X |
| Don't know |  |  |  |  |  |

## 2	Classification of the Work Item and linked work items

### 2.1	Primary classification

This work item is a …

|  | Feature |
| --- | --- |
| X | Building Block |
|  | Work Task |
|  | Study Item |

### 2.2	Parent Work Item

| Parent Work Items |  |  |  |
| --- | --- | --- | --- |
| Acronym | Working Group | Unique ID | Title (as in 3GPP Work Plan) |
| TEI17_DCAMP | SA2 | 870005 | Dynamically Changing AM Policies in the 5GC |

### 2.3	Other related Work Items and dependencies

| Other related Work Items (if any) |  |  |
| --- | --- | --- |
| Unique ID | Title | Nature of relationship |

Dependency on non-3GPP (draft) specification: None

## 3	Justification

- Dynamically Changing AM Policies (DCAMP) in the 5GC is a Rel-17 SA WGs work which impacts the CT WGs (see WID in SP-201137).

Release 16 supports AM policy control in the PCF, which enables to modify the access and mobility policies, as the RFSP index value, assigned by subscription based on operator policies.

For Release 17, SA2 is enhancing the 5GC capabilities to enable the dynamic provisioning/modification of AM Policies, in particular the RFSP index value and the SAR, by defining explicit third-party requests or application detection related functions to influence the Access and Mobility policy decisions, as specified in TS 23.501, 23.502 and 23.503.

## 4	Objective

The objective of this work item is to develop the Stage 3 specifications to enable the dynamic change of AM policies as specified by Stage 2.

The Stage 3 work shall be started only after the applicable normative Stage 2 work is available.

The following areas of work are expected to be covered:

CT3:

-	Definition of a new NEF service, Nnef_AMInfluence service, to allow the AF to influence PCF decisions for AM Policy changes, in particular RFSP index value change and SAR change.

-	UDR application data model impacts to store the AF service parameters that indicate the requirements for AM Policy changes, e.g. RFSP index value change.

-	PCF (for a UE) and PCF (for a PDU session) impacts that enable the PCF (for a UE) to detect when an application starts/stops for the services specified in the AF service parameters.

-	BSF new functionality to enable the discovery of the PCF(for a UE) and to notify about the registration of a new PCF(for a PDU session) for a DNN, S-NSSAI and SUPI combination or a new PCF (for a UE).

-	PCF (for a UE) and PCF (for a PDU session) impacts that enable the PCF (for a UE) to provide to the PCF (for a PDU session), via AMF and SMF, the request to be notified of PDU session established/terminated events.

-	Definition of a new NEF service, Nnef_AMPolicyAuthorization, and PCF (for a UE) service, Npcf_AMPolicyAuthorization to enable the AF to provide an indication to the PCF (for a UE) that a change of AM polices may be required because a service has started or terminated.

CT4:

-	Potential new supported feature(s) or resource(s) for UDR services.

-	Impacts to TS 29.510 to support (e.g. registration, discovery, etc.) the new services associated with the Dynamic Change of AM Policies in 5GC.

-	Impacts in TS 29.510 to update the NF Profile for the BSF to support registration and discovery based on the BSF Group Id and SUPI or GPSI ranges.

-	Impacts in TS 29.502 and TS 29.518 to enable the delivery of the PCF (for a UE) callback URI related information from the AMF to the SMF and to the target AMF at AMF relocation.

## 5	Expected Output and Time scale

| New specifications |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Type | TS/TR number | Title | For info at TSG# | For approval at TSG# | Rapporteur |
| TS | 29.534 | 5G System; Access and Mobility Policy Authorization Service; Stage 3 | TSG#94(December 2021) | TSG#95 (March 2022) | CT3 responsibility Rapporteur: Garcia Azorero, Fuencisla, Ericsson, fuencisla.garcia@ericsson.com |

| Impacted existing TS/TR {One line per specification. Create/delete lines as needed} |  |  |  |
| --- | --- | --- | --- |
| TS/TR No. | Description of change | Target completion plenary# | Remarks |
| 29.513 | Updates in procedures and call flows to support the Dynamic Change of AM Policies in 5GC | TSG#95 (March 2022) | CT3 responsibility |
| 29.507 | Updates to support an application can trigger the Dynamic Change of AM Policies in 5GC. Updates to support he request to the PCF for a PDU session via AMF for the notification of PDU session established/terminated events. | TSG#95 (March 2022) | CT3 responsibility |
| 29.512 | Updates to support an application can trigger the detection of application’s traffic to support the Dynamic Change of AM Policies in 5GC. Impacts to support the reporting of PCF for a UE request of notifications of PDU session established/terminated events by SMF. | TSG#95 (March 2022) | CT3 responsibility |
| 29.519 | Extension of UDR application data model to support DCAMP related information. | TSG#95 (March 2022) | CT3 responsibility |
| 29.521 | Updates to support the Dynamic Change of AM Policies in 5GC. Impacts to support the registration and discovery of the PCF for a UE. Impacts to support the notification of a new registered PCF for a PDU session or PCF for a UE. | TSG#95 (March 2022) | CT3 responsibility |
| 29.523 | New event to report service area coverage change to support the Dynamic Change of AM Policies in 5GC. | TSG#95 (March 2022) | CT3 responsibility |
| 29.514 | Updates to specify new events (application start/ application stop) to support the Dynamic Change of AM Policies in 5GC. Impacts to support the notification of PDU session established/terminated events. | TSG#95 (March 2022) | CT3 responsibility |
| 29.502 | Impacts in TS 29.502 to enable the delivery from the AMF to the SMF of the PCF (for a UE) request of notification of PDU session established/terminated events (delivery of callback URI related information from the AMF to the SMF). | TSG#95 (March 2022) | CT4 responsibility |
| 29.504 | Potential new features and resources to support the extensions of UDR data model to support the DCAMP related information. | TSG#95 (March 2022) | CT4 responsibility |
| 29.510 | Updates to enable the support (e.g. registration, discovery, etc.) of the new services related to the Dynamic Change of AM Policies in 5GC. Updates to support the BSF Group Id, and ranges of GPSI or SUPI within the BSF NF profile. | TSG#95 (March 2022) | CT4 responsibility |
| 29.518 | Impacts in TS 29.518 to enable the delivery, to the target AMF at AMF relocation, of the PCF (for a UE) request of notification of PDU session established/terminated events (delivery of callback URI related information from the AMF to the SMF). | TSG#95 (March 2022) | CT4 responsibility |
| 29.522 | New Nnef services, Nnef_AMInfluence and Nnef_AMPolicyAuthorization, to support the Dynamic Change of AM Policies in 5GC. | TSG#95 (March 2022) | CT3 responsibility |
| 29.571 | Potential updates to support the Dynamic Change of AM Policies in 5GC. | TSG#95 (March 2022) | CT4 responsibility |

## 6	Work item Rapporteur(s)

Garcia Azorero, Fuencisla. Ericsson. fuencisla.garcia@ericsson.com

## 7	Work item leadership

CT3

## 8	Aspects that involve other WGs

None.

## 9	Supporting Individual Members

Supporting IM name

Ericsson

China Telecom

Telstra

Charter Communications

Samsung

Nokia, Nokia Shanghai Bell

Telecom Italia

China Mobile

Huawei