3GPP TSG CT WG3 Meeting #137	C3-245174

# Hefei, CN, 14th – 18th October, 2024

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 29.548 | CR | 0022 | rev | - | Current version: | 19.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Corrections related to updated SEALDD terminology |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei |  |  |  |  |  |  |  |  |
| Source to TSG: | CT3 |  |  |  |  |  |  |  |  |
| Work item code: | SEALDD_Ph2 |  |  |  |  |  | Date: |  | 2024-10-07 |
| Category: | F |  |  |  |  |  | Release: |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  |  | Clauses 3.1, 8.4, 9.2 and 9.3 of TS 23.433 have been updated to clarify the difference between SEALDD-S flow/connection and SEALDD-UU flow/connection and their definition. Cf. the agreed S6-241443 (CR#0057 to TS 23.433). It is hence needed to align with this terminology in stage 3, wherever needed. |  |  |  |  |  |  |
| Summary of change: |  |  | This CR proposes to: - Address the above-detailed stage 2 requirements. |  |  |  |  |  |  |
| Consequences if not approved: |  |  | - The above-detailed stage 2 requirements are not specified in stage 3. |  |  |  |  |  |  |
| Clauses affected: |  | 6.3.6.1, 6.3.6.2.2, 6.3.6.2.3, 6.3.6.2.5, 6.3.6.2.6, A.4 |  |  |  |  |  |  |  |
|  |  | Y |  | N |  |  |  |  |  |
| Other specs |  |  |  | X | Other core specifications |  |  | TS/TR ... CR ... |  |
| affected: |  |  |  | X | Test specifications |  |  | TS/TR ... CR ... |  |
| (show related CRs) |  |  |  | X | O&M Specifications |  |  | TS/TR ... CR ... |  |
| Other comments: |  | This CR introduces backwards compatible corrections to the OpenAPI description of the SDD_DDContext API defined in this specification. |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |

* * * * Start of changes * * * *

#### 6.3.6.1	General

This clause specifies the application data model supported by the API.

Table 6.3.6.1-1 specifies the data types defined for the SDD_DDContext API.

Table 6.3.6.1-1: SDD_DDContext API specific Data Types

| Data type | Section defined | Description | Applicability |
| --- | --- | --- | --- |
| DdContext | 6.3.6.2.2 | Represents the DD context. |  |
| DdContextPushReq | 6.3.6.2.4 | Represents the DD context push request. |  |
| DdContextResp | 6.3.6.2.5 | Represents the DD context push response. |  |
| SddUuContext | 6.3.6.2.6 | Represents the context related to the SEALDD-UU connection. |  |
| SddSContext | 6.3.6.2.7 | Represents the context related to the SEALDD-S connection. |  |
| TranspLayerContext | 6.3.6.2.3 | Represents the transport layer context. |  |

Table 6.3.6.1-2 specifies data types re-used by the SDD_DDContext API from other specifications, including a reference to their respective specifications, and when needed, a short description of their use within the SDD_DDContext API.

Table 6.3.6.1-2: SDD_DDContext API re-used Data Types

| Data type | Reference | Comments | Applicability |
| --- | --- | --- | --- |
| ConnInfo | 6.1.6.2.4 | Represents SEALDD Data transmission connection information. |  |
| DurationSec | 3GPP TS 29.122 [2] | Represents a time duration in seconds. |  |
| ProblemDetails | 3GPP TS 29.122 [2] | Represents error related information. |  |
| QoSInfo | 6.1.6.2.5 | Represents SEALDD related QoS requirements. |  |
| SupportedFeatures | 3GPP TS 29.571 [18] | Represents the list of supported feature(s) and used to negotiate the applicability of the optional features. |  |
| TransportProtocol | 3GPP TS 29.558 [17] | Represents the transport layer protocol. |  |
| ValServBdw | 6.1.6.2.6 | Represents VAL Server related bandwidth information. |  |
| ValTargetUe | 3GPP TS 29.549 [15] | Represents the identifier of a VAL UE or VAL user. |  |
| ValUsersBdw | 6.1.6.2.7 | Represents VAL users related bandwidth information. |  |

* * * * Next changes * * * *

##### 6.3.6.2.2	Type: DdContext

Table 6.3.6.2.2-1: Definition of type DdContext

| Attribute name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| uuContext | SddUuContext | M | 1 | Represents the context related to the SEALDD-UU connection. |  |
| sContext | SddSContext | M | 1 | Represents the context related to the SEALDD-S connection. |  |
| trLayerContext | TranspLayerContext | O | 0..1 | Represents the transport layer context. This attribute is applicable only for the SEALDD-UU connection. |  |

* * * * Next changes * * * *

##### 6.3.6.2.3	Type: TranspLayerContext

Table 6.3.6.2.3-1: Definition of type TranspLayerContext

| Attribute name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| transProtoc | TransportProtocol | M | 1 | Represents the transport layer context for the SEALDD-UU user plane communication. |  |

* * * * Next changes * * * *

##### 6.3.6.2.5	Type: DdContextResp

Table 6.3.6.2.5-1: Definition of type DdContextResp

| Attribute name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| ddContext | DdContext | C | 0..1 | Represents the DD context information. This attribute shall be present only in the response to a DD Context Pull request. |  |
| endPoint | ConnInfo | C | 0..1 | Represents the endpoint information of the SEALDD Server for the new SEALDD-UU user plane communication. This attribute shall be present only in a response to a DD Context Push request. |  |
| policies | array(PolicyConfig) | C | 0..1 | Represents the set of SEALDD Policy(ies) and the related configuration information currently configured at (and being used by) the SEALDD Server. This attribute shall be present only in a response to a DD Context Pull request. |  |
| suppFeat | SupportedFeatures | C | 0..1 | Represents the list of supported features among the ones defined in clause 6.3.8. This attribute shall be present only when feature negotiation needs to take place. |  |

* * * * Next changes * * * *

##### 6.3.6.2.6	Type: SddUuContext

Table 6.3.6.2.6-1: Definition of type SddUuContext

| Attribute name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| sddFlowId | string | M | 1 | Represents the identifier of the SEALDD-UU flow. |  |
| valServiceId | string | O | 0..1 | Contains the identifier of the VAL service. |  |
| valServerId | string | O | 0..1 | Contains the identifier of the VAL Server. |  |
| valServEndPoint | ConnInfo | M | 1 | Represents the endpoint information of the selected VAL Server. |  |
| ddClientConnInfo | ConnInfo | O | 0..1 | Represents the SEALDD Client-side SEALDD-UU data transmission connection information. |  |
| ddServConnInfo | ConnInfo | O | 0..1 | Represents the SEALDD Server-side SEALDD-UU data transmission connection information. |  |
| valTgtUe | ValTargetUe | O | 0..1 | Contains the identifier of the target VAL UE or VAL user. |  |
| comLifetime | DurationSec | O | 0..1 | Represents the SEALDD communication lifetime. |  |
| valUsersBdw | ValUsersBdw | O | 0..1 | Contains the UL/DL bandwidth limits for VAL users. |  |
| pendingTimer | DurationSec | O | 0..1 | Contains the value of the pending timer representing the time duration to be respected before triggering re-connection from SEALDD Client when the bandwidth limit check fails. |  |

* * * * Next changes * * * *

# A.4	SDD_DDContext API

openapi: 3.0.0

info:

title: SEALDD Server Data Delivery Context Relocation

version: 1.0.0

description: |

SEALDD Server Data Delivery Context Relocation.

© 2024, 3GPP Organizational Partners (ARIB, ATIS, CCSA, ETSI, TSDSI, TTA, TTC).

All rights reserved.

externalDocs:

description: >

3GPP TS 29.548 V18.1.0 Service Enabler Architecture Layer for Verticals (SEAL);

SEAL Data Delivery (SEALDD) Server Services; Stage 3.

url: https://www.3gpp.org/ftp/Specs/archive/29_series/29.548/

security:

- {}

- oAuth2ClientCredentials: []

servers:

- url: '{apiRoot}/sdd-ddc/v1'

variables:

apiRoot:

default: https://example.com

description: apiRoot as defined in clause 6.5 of 3GPP TS 29.549

paths:

/contexts:

get:

summary: Pull the DD context from the SEALDD Server.

operationId: PullDdContext

tags:

- DD Contexts (Collection)

parameters:

- name: sealdd-pol-ind

in: query

required: false

schema:

type: boolean

default: false

description: >

Indicates whether the configured SEALDD Policy(ies), if any, is/are requested and

shall hence be provided in addition to the DD Context.

true indicates that the configured SEALDD Policy(ies) is/are requested.

false indicates that the configured SEALDD Policy(ies) is/are not requested.

The default value when this query parameter is omitted is false.

- name: supp-feats

in: query

description: Contains the list of supported features.

required: false

schema:

$ref: 'TS29571_CommonData.yaml#/components/schemas/SupportedFeatures'

responses:

'200':

description: Successful case. The requested DD context is returned.

content:

application/json:

schema:

$ref: '#/components/schemas/DdContextResp'

'400':

$ref: 'TS29122_CommonData.yaml#/components/responses/400'

'401':

$ref: 'TS29122_CommonData.yaml#/components/responses/401'

'403':

$ref: 'TS29122_CommonData.yaml#/components/responses/403'

'404':

$ref: 'TS29122_CommonData.yaml#/components/responses/404'

'406':

$ref: 'TS29122_CommonData.yaml#/components/responses/406'

'429':

$ref: 'TS29122_CommonData.yaml#/components/responses/429'

'500':

$ref: 'TS29122_CommonData.yaml#/components/responses/500'

'503':

$ref: 'TS29122_CommonData.yaml#/components/responses/503'

default:

$ref: 'TS29122_CommonData.yaml#/components/responses/default'

post:

summary: Push the DD context to the SEALDD Server.

tags:

- DD Contexts (Collection)

requestBody:

description: >

Represents the DD context to be pushed to the

SEALDD Server.

required: true

content:

application/json:

schema:

$ref: '#/components/schemas/DdContextPushReq'

responses:

'201':

description: >

Created. Successful case. The DD context is successfully pushed to the SEALDD Server and

the related information is returned in the response body.

content:

application/json:

schema:

$ref: '#/components/schemas/DdContextResp'

'307':

$ref: 'TS29122_CommonData.yaml#/components/responses/307'

'308':

$ref: 'TS29122_CommonData.yaml#/components/responses/308'

'400':

$ref: 'TS29122_CommonData.yaml#/components/responses/400'

'401':

$ref: 'TS29122_CommonData.yaml#/components/responses/401'

'403':

$ref: 'TS29122_CommonData.yaml#/components/responses/403'

'404':

$ref: 'TS29122_CommonData.yaml#/components/responses/404'

'411':

$ref: 'TS29122_CommonData.yaml#/components/responses/411'

'413':

$ref: 'TS29122_CommonData.yaml#/components/responses/413'

'415':

$ref: 'TS29122_CommonData.yaml#/components/responses/415'

'429':

$ref: 'TS29122_CommonData.yaml#/components/responses/429'

'500':

$ref: 'TS29122_CommonData.yaml#/components/responses/500'

'503':

$ref: 'TS29122_CommonData.yaml#/components/responses/503'

default:

$ref: 'TS29122_CommonData.yaml#/components/responses/default'

components:

securitySchemes:

oAuth2ClientCredentials:

type: oauth2

flows:

clientCredentials:

tokenUrl: '{tokenUrl}'

scopes: {}

schemas:

#

# STRUCTURED DATA TYPES

#

DdContext:

description: Represents the DD context.

type: object

properties:

uuContext:

$ref: '#/components/schemas/SddUuContext'

sContext:

$ref: '#/components/schemas/SddSContext'

trLayerContext:

$ref: '#/components/schemas/TranspLayerContext'

required:

- uuContext

- sContext

TranspLayerContext:

description: Represents the transport layer context.

type: object

properties:

transProtoc:

$ref: 'TS29558_Eees_EASRegistration.yaml#/components/schemas/TransportProtocol'

required:

- transProtoc

DdContextPushReq:

description: Represents the DD context push request.

type: object

properties:

ddContext:

$ref: '#/components/schemas/DdContext'

suppFeat:

$ref: 'TS29571_CommonData.yaml#/components/schemas/SupportedFeatures'

required:

- ddContext

DdContextResp:

description: Represents the DD context push response.

type: object

properties:

ddContext:

$ref: '#/components/schemas/DdContext'

endPoint:

$ref: 'TS29548_SDD_Transmission.yaml#/components/schemas/ConnInfo'

policies:

type: array

items:

$ref: 'TS29548_SDD_PolicyConfiguration.yaml#/components/schemas/PolicyConfig'

minItems: 1

suppFeat:

$ref: 'TS29571_CommonData.yaml#/components/schemas/SupportedFeatures'

SddUuContext:

description: Represents the context related to the SEALDD-UU connection.

type: object

properties:

sddFlowId:

type: string

valServiceId:

type: string

description: Contains the identifier of the VAL service.

valServerId:

type: string

description: Contains the identifier of the VAL Server.

valServEndPoint:

$ref: 'TS29548_SDD_Transmission.yaml#/components/schemas/ConnInfo'

ddClientConnInfo:

$ref: 'TS29548_SDD_Transmission.yaml#/components/schemas/ConnInfo'

ddServConnInfo:

$ref: 'TS29548_SDD_Transmission.yaml#/components/schemas/ConnInfo'

valTgtUe:

$ref: 'TS29549_SS_UserProfileRetrieval.yaml#/components/schemas/ValTargetUe'

comLifetime:

$ref: 'TS29122_CommonData.yaml#/components/schemas/DurationSec'

valUsersBdw:

$ref: 'TS29548_SDD_Transmission.yaml#/components/schemas/ValUsersBdw'

pendingTimer:

$ref: 'TS29122_CommonData.yaml#/components/schemas/DurationSec'

required:

- sddFlowId

- valServEndPoint

SddSContext:

description: Represents the context related to the SEALDD-S connection.

type: object

properties:

valServerId:

type: string

description: Contains the identifier of the VAL Server.

valServiceId:

type: string

description: Contains the identifier of the VAL service.

valTargetId:

$ref: 'TS29549_SS_UserProfileRetrieval.yaml#/components/schemas/ValTargetUe'

valServerConnInfo:

$ref: 'TS29548_SDD_Transmission.yaml#/components/schemas/ConnInfo'

ddServerConnInfo:

$ref: 'TS29548_SDD_Transmission.yaml#/components/schemas/ConnInfo'

qosInfo:

$ref: 'TS29548_SDD_Transmission.yaml#/components/schemas/QosInfo'

valServerBdw:

$ref: 'TS29548_SDD_Transmission.yaml#/components/schemas/ValServBdw'

valUsersBdw:

$ref: 'TS29548_SDD_Transmission.yaml#/components/schemas/ValUsersBdw'

required:

- valServerId

- valServerConnInfo

# SIMPLE DATA TYPES

#

#

# ENUMERATIONS

#

# Data types describing alternative data types or combinations of data types:

#

* * * * End of changes * * * *