3GPP TSG-CT WG4 Meeting #119	C4-235468

# Chicago, US; 13th– 17th November 2023                                                                was5200

Source:	China Mobile

Title:	Pseudo-CR on Update the A.3 for Nimsas_MediaControl service API

Spec:	3GPP TS 29.175 v0.4.0

Agenda item:	6.2.18

Document for:	Decision

1. Introduction

<Introduction part (optional)>

2. Reason for Change

The A.3 for Nimsas_MediaControl service API is missing. This pCR propose to complete this part.

3. Conclusions

<Conclusion part (optional)>

4. Proposal

It is proposed to agree the following changes to 3GPP TS 29.175 v0.4.0.

* * * First Change * * * *

A.3	Nimsas_MediaControl API

openapi: 3.0.0

info:

title: 'IMS AS MediaControl Service'

version: '1.0.0-alpha.1'

description: |

Nimsas_MediaControl Service.

© 2023, 3GPP Organizational Partners (ARIB, ATIS, CCSA, ETSI, TSDSI, TTA, TTC).

All rights reserved.

externalDocs:

description: >

3GPP TS 29.175 V0.5.0; IP Multimedia Subsystem; IP Multimedia Subsystem (IMS) Application

Server (AS) Services; Stage 3

url: 'https://www.3gpp.org/ftp/Specs/archive/29_series/29.175'

servers:

- url: '{apiRoot}/nimsas-mc/v1'

variables:

apiRoot:

default: https://example.com

description: apiRoot as defined in clause 4.4 of 3GPP TS 29.501

security:

- {}

- oAuth2ClientCredentials:

- nimsas-mc

paths:

/call-sessions/{sessionId}/media-instruction:

post:

summary: Update

operationId: UpdateCallSession

tags:

- Individual call session (Document)

parameters:

- name: sessionId

in: path

description: Session ID assigned by the IMS AS during the IMS session setup.

required: true

schema:

$ref: 'TS29571_CommonData.yaml#/components/schemas/SessionId'

requestBody:

description: Representation of the updates to apply to the call session.

required: true

content:

application/json:

schema:

$ref: '#/components/schemas/MediaInstructionData'

responses:

'200':

description: >

Successful update of the call session, when the IMS AS needs to return information

in the response.

content:

application/json:

schema:

$ref: '#/components/schemas/MediaInstructionData'

'204':

description: >

Successful update of the call session, when the IMS AS does not need to return

information in the response.

'307':

description: Temporary Redirect

content:

application/json:

schema:

$ref: 'TS29571_CommonData.yaml#/components/schemas/RedirectResponse'

headers:

Location:

description: The URI pointing to the resource located on the redirect target UPF.

schema:

type: string

'308':

description: Permanent Redirect

content:

application/json:

schema:

$ref: 'TS29571_CommonData.yaml#/components/schemas/RedirectResponse'

headers:

Location:

description: The URI pointing to the resource located on the redirect target UPF.

schema:

type: string

'400':

$ref: 'TS29571_CommonData.yaml#/components/responses/400'

'401':

$ref: 'TS29571_CommonData.yaml#/components/responses/401'

'403':

$ref: 'TS29571_CommonData.yaml#/components/responses/403'

'404':

$ref: 'TS29571_CommonData.yaml#/components/responses/404'

'406':

$ref: 'TS29571_CommonData.yaml#/components/responses/406'

'411':

$ref: 'TS29571_CommonData.yaml#/components/responses/411'

'413':

$ref: 'TS29571_CommonData.yaml#/components/responses/413'

'415':

$ref: 'TS29571_CommonData.yaml#/components/responses/415'

'429':

$ref: 'TS29571_CommonData.yaml#/components/responses/429'

'500':

$ref: 'TS29571_CommonData.yaml#/components/responses/500'

'501':

$ref: 'TS29571_CommonData.yaml#/components/responses/501'

'503':

$ref: 'TS29571_CommonData.yaml#/components/responses/503'

default:

$ref: 'TS29571_CommonData.yaml#/components/responses/default'

components:

securitySchemes:

oAuth2ClientCredentials:

type: oauth2

flows:

clientCredentials:

tokenUrl: '{nrfApiRoot}/oauth2/token'

scopes:

nimsas-mc: Access to the Nimsas_MediaControl API.

schemas:

# STRUCTURED DATA TYPES

MediaInstructionData:

description: The media instruction data for a specific session.

type: object

required:

- sessionId

- mediaInstructionSet

properties:

sessionId:

$ref: 'TS29571_CommonData.yaml#/components/schemas/SessionId'

mediaInstructionSet:

description: >

Contains a set of instructions for each media flow to control. The mediaId

attribute within the MediaInstructions data type is the key of the map.

type: object

additionalProperties:

$ref: '#/components/schemas/MediaInstructions'

minProperties: 1

MediaInstructions:

description: The media instructions for a specific media flow.

type: object

required:

- mediaId

- mediaResourceType

properties:

mediaId:

$ref: 'TS29571_CommonData.yaml#/components/schemas/MediaId'

mediaResourceType:

$ref: 'TS29571_CommonData.yaml#/components/schemas/MediaResourceType'

mediaInstruction:

$ref: '#/components/schemas/MediaInstruction'

dcMediaSpecification:

$ref: '#/components/schemas/DcMediaSpecification'

mediaProcessingUrl:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Uri'

DcMediaSpecification:

description: The data channel media specification information.

type: object

required:

- streams

properties:

mediaProxyConfig:

$ref: 'TS29571_CommonData.yaml#/components/schemas/MediaProxy'

replaceHttpUrls:

description: >

Contains a list of replacement HTTP URLs. The streamId

attribute within the ReplaceHttpUrl data type is the key of the map.

type: object

additionalProperties:

$ref: 'TS29571_CommonData.yaml#/components/schemas/ReplaceHttpUrl'

minProperties: 1

mdc1EndpointDcsf:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Endpoint'

mdc1EndpointMf:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Endpoint'

mdc2EndpointDcAs:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Endpoint'

mdc2EndpointMf:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Endpoint'

mdc2Protocol:

type: string

streams:

description: >

Contains a data channel mapping and configuration information. The streamId

attribute within the DcStream data type is the key of the map.

type: object

additionalProperties:

$ref: 'TS29571_CommonData.yaml#/components/schemas/DcStream'

minProperties: 1

# ENUMS

MediaInstruction:

description: The media instruction for handling a specific media flow.

anyOf:

- type: string

enum:

- TERMINATE_MEDIA

- ORIGINATE_MEDIA

- TERMINATE_AND_ORIGINATE_MEDIA

- UPDATE_MEDIA

- DELETE_MEDIA

- REJECT_MEDIA

- type: string

description: >

This string provides forward-compatibility with future extensions to the enumeration

and is not used to encode content defined in the present version of this API.

* * * End of Changes * * * *