3GPP TSG-CT WG4 Meeting #96	C4-200014

# Sophia Antipolis, France; 24th – 28th February 2020

| CR-Form-v12.0 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 29.500 | CR | 0079 | rev | - | Current version: | 16.2.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | 3GPP Rel-16 LOLC support |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei |  |  |  |  |  |  |  |  |  |
| Source to TSG: | CT4 |  |  |  |  |  |  |  |  |  |
| Work item code: | LOLC |  |  |  |  |  | Date: |  |  | 2020-01-07 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | In September 2019, CT plenary approved finalized version of the 3GPP TR 29.843v16.0.0, which outlines the Overload Control solution for the normative work. This CR adds the definition of the new LOLC header to 3GPP TS 29.500, which indicates what kind of LOLC headers the consumer supports. |  |  |  |  |  |  |  |  |
| Summary of change: |  | - 3gpp-Sbi-Lolc_support header added to Table 5.2.3.2-1 - New clause 5.2.3.2.z is added that defines 3gpp-Sbi-Lolc-Support custom header |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  |  |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.2.3.2.1, 5.2.3.2.z (new), 6.6.1 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  | X |  | Other core specifications |  |  |  | TS 29.500 CR 0074 |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

* * * First Change * * * *

### 5.2.3	HTTP custom headers

#### 5.2.3.1	General

The list of custom HTTP headers applicable to 3GPP Service Based NFs are specified below.

#### 5.2.3.2	Mandatory to support custom headers

#### 5.2.3.2.1	General

The 3GPP NF Services shall support the HTTP custom headers specified in Table 5.2.3.2-1 below. A description of each custom header and the normative requirements on when to include them are also provided in Table 5.2.3.2-1.

Table 5.2.3.2.1-1: Mandatory HTTP custom headers

| Name | Reference | Description |
| --- | --- | --- |
| 3gpp-Sbi-Message-Priority | Clause 5.2.3.2.2 | This header is used to specify the HTTP/2 message priority for 3GPP service based interfaces. This header shall be included in HTTP/2 messages when a priority for the message needs to be conveyed (e.g HTTP/2 messages related to Multimedia Priority Sessions). |
| 3gpp-Sbi-Callback | Clause 5.2.3.2.3 | This header is used to indicate if a HTTP/2 message is a callback (e.g notification). This header shall be included in HTTP POST messages for callbacks towards NF service consumer(s) in another PLMN via the SEPP (See 3GPP TS 29.573 [27]). |
| 3gpp-Sbi-Target-apiRoot | Clause 5.2.3.2.4 | This header is used by an HTTP client to indicate the apiRoot of the target URI when communicating indirectly with the HTTP server via an SCP using HTTPS. |
| 3gpp-Sbi-Server-Binding | Clause 5.2.3.2.5 | This header is used in a service request to signal binding information to direct the service request to an HTTP server which has the targeted NF Service Resource context (see clause 6.12). |
| 3gpp-Sbi-Client-Binding | Clause 5.2.3.2.6 | This header is used to signal binding information related to an NF Service Resource to a future consumer (HTTP client) of that resource (see clause 6.12). |
| 3gpp-Sbi-Discovery-* | Clause 5.2.3.2.7 | Headers beginning with the prefix 3gpp-Sbi-Discovery- are used in indirect communication mode for discovery and selection of a suitable producer by the SCP. Such headers may be included in any SBI message and include information allowing an SCP to find a suitable producer as per the consumer's included delegated discovery parameters. |
| 3gpp-Sbi-Producer-Id | Clause 5.2.3.2.8 | This header is used in a service response from the SCP to the NF Service Consumer, when using indirect communication with delegated discovery and the NF Service Producer does not return a binding indication in a service response creating a resource. See clause 6.10.3.4. |
| 3gpp-Sbi-Lolc-Support | Clause 5.2.3.2.x. | This header is used by an HTTP client (service consumer) to indicate to the service producer which types of LOLC headers it supports. |

* * * 2nd Change * * * *

##### 5.2.3.2.z	3gpp-Sbi-Lolc-Support

The header contains the information on the supported LOLC header types.

The encoding of the header follows the ABNF as defined in IETF RFC 7230 [12].

3gpp-Sbi-Lolc-Support = 2*DIGIT

; At least 2 digits long string

The string of digits shall be treated as a binary bitmask, which indicate LOLC header supports as follows:

-	The leftmost first digit shall be set to "1", if the sender supports the "3gpp-Sbi-Lci" header, as specified in 3GPP TS 29.500v16.3.0. Otherwise, it must be set to "0" (if in future a new header replaces this one).

-	The second leftmost digit shall be set to "1", if the sender supports the "3gpp-Sbi-Oci" header, as specified in 3GPP TS 29.500v16.3.0. Otherwise, it must be set to "0" (if in future a new header replaces this one).

3GPP Rel-16 service instance shall support both "3gpp-Sbi-Lci" and "3gpp-Sbi-Oci" header types.

Presence of additional digits in the string indicates support for the extended features. Each digit shall have unique meaning in the 3gpp-Sbi-Lolc-Support header (see Table 5.2.3.2.x-1).

NOTE:	For instance, if two extended features, the third and the fourth one are specified, but the sender supports only the fourth feature, then the ssender shall set the third digit to "0" and the fouth one to "1".

Table 5.2.3.2.x-1: 3gpp-Sbi-Lolc-Support HTTP custom header encoding

| Leftmost digit (bit 1) | Next digit (bit 2) | Next digit (bit 3) | Next digit (bit 4) | … |
| --- | --- | --- | --- | --- |
| Support for the 3GPP Rel-16 Load Control header "3gpp-Sbi-Lci", as specified in 3GPP TS 29.500v16.3.0 | Support for the 3GPP Rel-16 Overload Control header "3gpp-Sbi-Oci", as specified in 3GPP TS 29.500v16.3.0 | Spare, for future new element addition to the 3gpp-Sbi-Lci or 3gpp-Sbi-Oci headers | Spare, for future new element addition to the 3gpp-Sbi-Lci or 3gpp-Sbi-Oci headers | Spare, for future new element addition to the 3gpp-Sbi-Lci or 3gpp-Sbi-Oci headers |

* * * 2nd Change * * * *

## 6.6	Extensibility Mechanisms

### 6.6.1	General

This clause describes the extensibility mechanisms supported in the Service-Based Architecture in 3GPP 5GC, such as feature negotiation, vendor-specific extensions, etc.

In addition to API specific feature negotiations specified in this clause, another HTTP custom header based negotiation mechanism is defined in clause 5.2.3 for the Load Control and the Overload Control mechanisms.

* * * End of Changes * * * *