3GPP TSG-CT WG4 Meeting #98e	C4-203291

- E-meeting, 2nd – 12th June 2020

| CR-Form-v12.0 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 29.518 | CR | 0362 | rev | 1 | Current version: | 16.3.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | x |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Indication of control plane CIoT 5GS optimization to an LMF |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Qualcomm Incorporated |  |  |  |  |  |  |  |  |  |
| Source to TSG: | C4 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5G_eLCS |  |  |  |  |  | Date: |  |  | 2020-06-02 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | For support of Low Power Periodic and Triggered 5GC-MT-LR Procedures in clause 6.7.1 of TS 23.273, an AMF needs to be able to indicate to an LMF when an event report being transferred to the LMF was transferred to the AMF using Control Plane CIoT 5GS Optimisation. This is described in step 5 of clause 6.7.1 in TS 23.273 as follows. 5. The AMF checks the integrity of the NAS message and deciphers its contents. The AMF then invokes an Namf_Communication_N1MessageNotify service operation to forward the event report to either the serving LMF or any suitable LMF as described for step 25 for clause 6.3.1. The AMF includes an indication of Control Plane CIoT 5GS Optimisation in the service operation and the serving cell ID. |  |  |  |  |  |  |  |  |
| Summary of change: |  | The N1MessageNotify service operation is extended to allow inclusion of an indication of Control Plane CIoT 5GS Optimisation. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Low Power Periodic and Triggered 5GC-MT-LR Procedures as defined in clause 6.7.1 of TS 23.273 will not be correctly supported which may result in failure to transfer an event report to an external client or AF or impairment of Control Plane CIoT 5GS Optimisation support for a UE. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.2.2.3.5.1, 5.2.2.3.5.5, 6.1.6.2.16, A.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | This CR introduces backwards-compatible changes in the OpenAPI specification. |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | - Updated cover sheet - Updated description of cIoT5GSOptimisation attribute. |  |  |  |  |  |  |  |  |

******** NEXT CHANGE ********

- 5.2.2.3.5.1	General

The N1MessageNotify service operation is used by an AMF notifying the N1 message received from the UE to a destination CN NF, and it is used in the following procedures:

-	Registration with AMF re-allocation (see clause 4.2.2.2.3 of 3GPP TS 23.502 [3])

-	UE assisted and UE based positioning procedure (see clause 4.13.5.4 of 3GPP TS 23.502 [3])

-	LCS Event Report, LCS Cancel Location and LCS Periodic-Triggered Invoke procedures (see clause 6.3 and clause 6.7 of 3GPP TS 23.273 [42])

-	UE configuration update procedure for transparent UE policy delivery (See clause 4.2.4.3 in 3GPP TS 23.502 [3])

NOTE:	Though in 3GPP TS 23.502 [3] the procedure is called "UE configuration update procedure for transparent UE policy delivery", as per 3GPP TS 24.501 [11] clause 5.4.5.2.1, the UE initiated NAS transport procedure is used.

The AMF shall use HTTP POST method to the N1 Notification URI provided by the NF Service Consumer via N1N2MessageSubscribe service operation (See clause 5.2.2.3.3). See also figure 5.2.2.3.5.1-1.

Figure 5.2.2.3.5.1-1 N1 Message Notify

1.	The AMF shall send a HTTP POST request to the N1 Notification URI, and the payload body of the POST request shall contain an N1MessageNotificatoin data structure with the subscribed N1 message.

2a.	On success, "204 No Content" shall be returned and the payload body of the POST response shall be empty.

2b.	On failure, one of the HTTP status code listed in Table 6.1.5.6.4.1-2 shall be returned. The message body shall contain a ProblemDetails object with "cause" set to one of the corresponding application errors listed in Table 6.1.5.6.4.1-2.

******** NEXT CHANGE ********

- 5.2.2.3.5.5	Using N1MessageNotify in the LCS Event Report, LCS Cancel Location and LCS Periodic-Triggered Invoke Procedures

In the LCS Event Report, LCS Cancel Location and LCS Periodic-Triggered Invoke procedure, the N1MessageNotify service operation is invoked by the AMF, towards the LMF, to notify the N1 UE LCS messages received from the UE.

The requirements specified in clause 5.2.2.3.5.1 shall apply with the following modifications:

1.	If the corresponding N1 notification URI is not available, the AMF shall retrieve the NF profile of the NF Service Consumer (e.g. the LMF) from the NRF using the NF Instance Identifier received during corresponding N1N2MessageTransfer service operation (see clause 5.2.2.3.1), and further identify the corresponding service instance if Service Instance Identifier was also received, and fetch N1 Notification URI from the default notification subscription registered with "N1_MESSAGE" notification type and "LCS" N1 message class (See Table 6.2.6.2.3-1 and Table 6.2.6.2.4-1 of 3GPP TS 29.510 [29]).

2.	Same as step 1 of Figure 5.2.2.3.5.1-1, the request payload shall include the following information:

-	the N1 Uplink LCS Message;

-	LCS correlation identifier;

-	indication of Control Plane CIoT 5GS Optimisation if Control Plane CIoT 5GS Optimisation is being used.

NOTE:	For the EventReport message and UE initiated CancelDeferredLocation message, the AMF includes the deferred routing identifier received from UE in N1 UL NAS TRANSPORT message as LCS correlation identifier. The LCS correlation identifier can assist a serving LMF in identifying the periodic or triggered location session if the same LMF had assigned the deferred routing identifier or can indicate to the LMF that it is acting as a default LMF.

******** NEXT CHANGE ********

- 6.1.6.2.16	Type: N1MessageNotification

Table 6.1.6.2.16-1: Definition of type N1MessageNotification

| Attribute name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| n1NotifySubscriptionId | string | C | 0..1 | Represents the subscription Id for which the notification is generated. The NF Service Consumer uses this to correlate the notification against a corresponding subscription. If the notification is due to an implicit subscription via NRF, then the value shall be set as "implicit". This IE shall be present if the notification is based on a subscription to N1MessgeNotification. An exception is for the case when initial AMF forwards NAS message to target AMF during AMF re-allocation procedure. |
| n1MessageContainer | N1MessageContainer | M | 1 | Contains the N1 message class and N1 message content. |
| lcsCorrelationId | CorrelationID | O | 0..1 | If the N1 message notified is for LCS procedures, the NF Service Producer (e.g. AMF) may include an LCS correlation identifier. |
| registrationCtxtContainer | RegistrationContextContainer | C | 0..1 | If the N1 message notified is of type 5GMM (i.e. during Registration with AMF re-allocation procedure), the NF Service Producer (e.g. AMF) shall include this IE, if available. |
| newLmfIdentification | LMFIdentification | O | 0..1 | If a new LMF is selected by AMF, this IE may include the new selected LMF Identification. |
| cIoT5GSOptimisation | boolean | C | 0..1 | This IE shall be present when the N1 message class is “LPP/LCS” and the N1 message is received from the UE with Control Plane CIoT 5GS Optimisation. When present, it shall be set as follows: - true: Control Plane CIoT 5GS Optimisation was used and no signalling or data is currently pending for the UE at the AMF. - false (default): Control Plane CIoT 5GS Optimisation was not used or signalling or data is currently pending for the UE at the AMF. |

******** NEXT CHANGE ********

- A.2	Namf_Communication API

openapi: 3.0.0

******** Unchanged text omitted ********

#

# STRUCTURED DATA TYPES

#

******** Unchanged text omitted ********

N1MessageNotification:

type: object

properties:

n1NotifySubscriptionId:

type: string

n1MessageContainer:

$ref: '#/components/schemas/N1MessageContainer'

lcsCorrelationId:

$ref: 'TS29572_Nlmf_Location.yaml#/components/schemas/CorrelationID'

registrationCtxtContainer:

$ref: '#/components/schemas/RegistrationContextContainer'

newLmfIdentification:

$ref: 'TS29572_Nlmf_Location.yaml#/components/schemas/LMFIdentification'

cIoT5GSOptimisation:

type: boolean

default: false

required:

- n1MessageContainer

******** END OF CHANGES ********