3GPP TSG-CT WG4 Meeting #99e	C4-204xyz

- E-Meeting, 18th – 28th August 2020	(was C4-204084)

| CR-Form-v12.0 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 29.503 | CR | 0466 | rev | 1 | Current version: | 16.4.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | x |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Providing configured human-readable network name for CAG-ID |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson |  |  |  |  |  |  |  |  |  |
| Source to TSG: | CT4 |  |  |  |  |  |  |  |  |  |
| Work item code: | Vertical_LAN |  |  |  |  |  | Date: |  |  | 2020-08-10 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | When a PLMN does not allow a user of an UE to manually select a CAG-ID at CAG cells of the PLMN (unless the MS is authorized to select the CAG-ID automatically), then only UEs configured with the CAG-ID can select the CAG-ID at the CAG cells of the PLMN. In such case, if the UE is also configured with human-readable network name associated with the CAG-ID, then the CAG cells of the PLMN do not need to broadcast the human-readable network name for the CAG-ID. This would save precious radio resources of the CAG cells needed for the broadcast of the human-readable network name for the CAG-ID. |  |  |  |  |  |  |  |  |
| Summary of change: |  | The UDM can provide the AMF with human-readable network names for CAG-IDs, which the AMF can provide to the UE. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Waste of precious radio resources of the CAG cells needed for the broadcast of the human-readable network name for the CAG-ID. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.1.6.1, 6.1.6.2.37, 6.1.6.2.x (new), A.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  | X |  | Other core specifications |  |  |  | TS 24.501 CR 2009 |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | This CR introduces backwards compatible corrections, with impacts on the following APIs: - TS29503_Nudm_SDM.yaml - TS29504_Nudr_DataRepository.yaml (via TS29505_SubscriptionData.yaml) - TS29518_Namf_Communication.yaml |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

* * * First Change * * * *

#### 6.1.6.1	General

This clause specifies the application data model supported by the API.

Table 6.1.6.1-1 specifies the data types defined for the Nudm_SDM service API.

Table 6.1.6.1-1: Nudm_SDM specific Data Types

| Data type | Clause defined | Description |
| --- | --- | --- |
| Nssai | 6.1.6.2.2 | Network Slice Selection Assistance Information |
| SdmSubscription | 6.1.6.2.3 | A subscription to notifications |
| AccessAndMobilitySubscriptionData | 6.1.6.2.4 | Access and Mobility Subscription Data |
| SmfSelectionSubscriptionData | 6.1.6.2.5 | SMF Selection Subscription Data |
| DnnInfo | 6.1.6.2.6 | Data Network Name and associated information (LBO roaming allowed flag) |
| SnssaiInfo | 6.1.6.2.7 | S-NSSAI and associated information (DNN Info) |
| SessionManagementSubscriptionData | 6.1.6.2.8 | User subscribed session management data |
| DnnConfiguration | 6.1.6.2.9 | User subscribed data network configuration |
| PduSessionTypes | 6.1.6.2.11 | Default/allowed session types for a data network |
| SscModes | 6.1.6.2.12 | Default/allowed SSC modes for a data network |
| SmsSubscriptionData | 6.1.6.2.13 |  |
| SmsManagementSubscriptionData | 6.1.6.2.14 | SMS Management Subscription Data |
| SubscriptionDataSets | 6.1.6.2.15 |  |
| UeContextInSmfData | 6.1.6.2.16 | UE Context In SMF Data |
| PduSession | 6.1.6.2.17 |  |
| IdTranslationResult | 6.1.6.2.18 | SUPI that corresponds to a given GPSI |
| ModificationNotification | 6.1.6.2.21 |  |
| IpAddress | 6.1.6.2.22 | IP address (IPv4, or IPv6, or IPv6 prefix) |
| UeContextInSmsfData | 6.1.6.2.23 |  |
| SmsfInfo | 6.1.6.2.24 |  |
| AcknowledgeInfo | 6.1.6.2.25 |  |
| SorInfo | 6.1.6.2.26 | Steering Of Roaming Information |
| SharedData | 6.1.6.2.27 | Subscription Data shared by multiple UEs |
| PgwInfo | 6.1.6.2.28 | Information about the DNNs/APNs and PGW-C+SMF FQDNs used in interworking with EPS |
| TraceDataResponse | 6.1.6.2.29 | Contains Trace Data or a shared data Id identifying shared Trace Data |
| SteeringContainer | 6.1.6.2.30 |  |
| SdmSubsModification | 6.1.6.2.31 | Modification instruction for a subscription to notifications |
| EmergencyInfo | 6.1.6.2.32 | Information about emergency session |
| UpuInfo | 6.1.6.2.33 | UE Parameters Update Information |
| GroupIdentifiers | 6.1.6.2.34 |  |
| NiddInformation | 6.1.6.2.35 | Non-IP Data Delivery information |
| CagData | 6.1.6.2.36 |  |
| CagInfo | 6.1.6.2.37 |  |
| CagIdHrnn | 6.1.6.2.x | CAG Id to HRNN mapping |
| DataSetName | 6.1.6.3.3 |  |
| PduSessionContinuityInd | 6.1.6.3.7 |  |
| AdditionalSnssaiData | 6.1.6.2.38 | Additional information specific to a slice |
| VnGroupData | 6.1.6.2.39 |  |
| AppDescriptor | 6.1.6.2.40 |  |
| AppPortId | 6.1.6.2.41 | Application Port Id |
| LcsPrivacyData | 6.1.6.2.42 |  |
| Lpi | 6.1.6.2.43 |  |
| UnrelatedClass | 6.1.6.2.44 |  |
| PlmnOperatorClass | 6.1.6.2.45 |  |
| ValidTimePeriod | 6.1.6.2.46 |  |
| LcsMoData | 6.1.6.2.47 |  |
| EcRestrictionDataWb | 6.1.6.2.48 | Enhance Coverage Restriction Data |
| ExpectedUeBehaviourData | 6.1.6.2.49 | Expected UE Behaviour Data |
| MaximumResponseTime | 6.1.6.2.50 | Maximum Response Time |
| MaximumLatency | 6.1.6.2.51 | Maximum Latency |
| SuggestedPacketNumDl | 6.1.6.2.52 | Suggested Number of Downlink Packets |
| SmfRegistrationInfo | 6.1.6.2.53 |  |
| FrameRouteInfo | 6.1.6.2.54 | Frame Route Information |
| SorUpdateInfo | 6.1.6.2.55 |  |
| EnhancedCoverageRestrictionData | 6.1.6.2.56 | Enhanced Coverage Restriction Data |
| EdrxParameters | 6.1.6.2.57 | eDRX Parameters |
| PtwParameters | 6.1.6.2.58 | Paging Time Window Parameters |
| OperationMode | 6.1.6.3.12 | Operation Mode |
| SorUpdateIndicator | 6.1.6.3.13 | SoR Update Indicator |
| ExternalUnrelatedClass | 6.1.6.2.62 |  |
| AfExternal | 6.1.6.2.63 |  |
| LcsClientExternal | 6.1.6.2.64 |  |
| LcsClientGroupExternal | 6.1.6.2.65 |  |
| ServiceTypeUnrelatedClass | 6.1.6.2.66 |  |
| UeId | 6.1.6.2.67 |  |
| DefaultUnrelatedClass | 6.1.6.2.68 |  |
| UeContextInAmfData | 6.1.6.2.70 |  |
| V2xSubscriptionData | 6.1.6.2.71 | V2X Subscription Data |
| LcsBroadcastAssistanceTypesData | 6.1.6.2.72 | LCS Broadcast Assistance Data Types |
| DefaultDnnIndicator | 6.1.6.3.2 |  |
| LboRoamingAllowed | 6.1.6.3.2 |  |
| UeUsageType | 6.1.6.3.2 |  |
| MpsPriorityIndicator | 6.1.6.3.2 |  |
| McsPriorityIndicator | 6.1.6.3.2 |  |
| 3GppChargingCharacteristics | 6.1.6.3.2 | 3GPP Charging Characteristics |
| DlPacketCount | 6.1.6.3.2 |  |
| MicoAllowed | 6.1.6.3.2 |  |
| SmsSubscribed | 6.1.6.3.2 |  |
| SharedDataId | 6.1.6.3.2 |  |
| IwkEpsInd | 6.1.6.3.2 | Interworking with EPS Indication |
| SecuredPacket | 6.1.6.3.2 |  |
| UpuRegInd | 6.1.6.3.2 |  |
| ExtGroupId | 6.1.6.3.2 |  |
| NbIoTUePriority | 6.1.6.3.2 |  |
| CodeWord | 6.1.6.3.2 |  |
| AfId | 6.1.6.3.2 |  |
| LcsClientId | 6.1.6.3.2 |  |
| DataSetName | 6.1.6.3.3 |  |
| PduSessionContinuityInd | 6.1.6.3.7 |  |
| LocationPrivacyInd | 6.1.6.3.8 |  |
| PrivacyCheckRelatedAction | 6.1.6.3.9 |  |
| LcsClientClass | 6.1.6.3.10 |  |
| LcsMoServiceClass | 6.1.6.3.11 |  |
| OperationMode | 6.1.6.3.12 |  |
| SorUpdateIndicator | 6.1.6.3.13 |  |
| CodeWordInd | 6.1.6.3.14 |  |
| MdtUserConsent | 6.1.6.3.15 | MDT User Consent |

Table 6.1.6.1-2 specifies data types re-used by the Nudm_SDM service API from other specifications, including a reference to their respective specifications and when needed, a short description of their use within the Nudm_SDM service API.

Table 6.1.6.1-2: Nudm_SDM re-used Data Types

| Data type |  | Reference |  | Comments |
| --- | --- | --- | --- | --- |
| Dnn |  | 3GPP TS 29.571 [7] |  | Data Network Name with Network Identifier only; this type is used as key in a map of: - DnnConfigurations; see clause 6.1.6.2.8; - EpsIwkPgws; see clause 6.2.6.2.2; - ExpectedUeBehaviourData; see clause 6.1.6.2.8; |
| DurationSec |  | 3GPP TS 29.571 [7] |  | Time value in seconds |
| ProblemDetails |  | 3GPP TS 29.571 [7] |  | Common data type used in response bodies |
| Snssai |  | 3GPP TS 29.571 [7] |  | Single NSSAI |
| Uri |  | 3GPP TS 29.571 [7] |  | Uniform Resource Identifier |
| Gpsi |  | 3GPP TS 29.571 [7] |  | Generic Public Subscription Identifier |
| RatType |  | 3GPP TS 29.571 [7] |  | Radio Access Technology Type |
| Area |  | 3GPP TS 29.571 [7] |  |  |
| ServiceAreaRestriction |  | 3GPP TS 29.571 [7] |  |  |
| CoreNetworkType |  | 3GPP TS 29.571 [7] |  |  |
| SupportedFeatures |  | 3GPP TS 29.571 [7] |  | see 3GPP TS 29.500 [4] clause 6.6 |
| PlmnId |  | 3GPP TS 29.571 [7] |  | PLMN Identity |
| PduSessionType |  | 3GPP TS 29.571 [7] |  |  |
| SubscribedDefaultQos |  | 3GPP TS 29.571 [7] |  | Subscribed Default QoS |
| Ambr |  | 3GPP TS 29.571 [7] |  |  |
| PduSessionId |  | 3GPP TS 29.571 [7] |  | PduSessionId is used as key in a map of PduSessions; see clause 6.1.6.2.16. |
| NfInstanceId |  | 3GPP TS 29.571 [7] |  |  |
| Supi |  | 3GPP TS 29.571 [7] |  |  |
| RfspIndex |  | 3GPP TS 29.571 [7] |  |  |
| SscMode |  | 3GPP TS 29.571 [7] |  |  |
| Ipv4Addr |  | 3GPP TS 29.571 [7] |  |  |
| Ipv6Addr |  | 3GPP TS 29.571 [7] |  |  |
| Ipv6Prefix |  | 3GPP TS 29.571 [7] |  |  |
| SorMac |  | 3GPP TS 29.509 [24] |  |  |
| SteeringInfo |  | 3GPP TS 29.509 [24] |  |  |
| AckInd |  | 3GPP TS 29.509 [24] |  |  |
| CounterSor |  | 3GPP TS 29.509 [24] |  |  |
| UpuMac | 3GPP TS 29.509 [24] |  |  |  |
| UpuData | 3GPP TS 29.509 [24] |  |  |  |
| UpuAckInd | 3GPP TS 29.509 [24] |  |  |  |
| CounterUpu | 3GPP TS 29.509 [24] |  |  |  |
| TraceData |  | 3GPP TS 29.571 [7] |  | Trace control and configuration parameters |
| NotifyItem |  | 3GPP TS 29.571 [7] |  |  |
| UpSecurity |  | 3GPP TS 29.571 [7] |  |  |
| ServiceName |  | 3GPP TS 29.510 [19] |  |  |
| OdbPacketServices |  | 3GPP TS 29.571 [7] |  |  |
| GroupId |  | 3GPP TS 29.571 [7] |  | This type is also used as key of a map in attributes: - vnGroupInfo and sharedVnGroupDataIds; see clause 6.1.6.2.4, 6.1.6.2.8, 6.1.6.2.27; |
| DateTime |  | 3GPP TS 29.571 [7] |  |  |
| CagId |  | 3GPP TS 29.571 [7] |  |  |
| StnSr |  | 3GPP TS 29.571 [7] |  | Session Transfer Number for SRVCC |
| CMsisdn |  | 3GPP TS 29.571 [7] |  | Correlation MSISDN |
| OsId |  | 3GPP TS 29.519 [33] |  |  |
| Uint16 |  | 3GPP TS 29.571 [7] |  |  |
| RgWirelineCharacteristics |  | 3GPP TS 29.571 [7] |  |  |
| GeographicArea |  | 3GPP TS 29.572 [34] |  |  |
| LcsServiceType |  | 3GPP TS 29.572 [34] |  |  |
| ScheduledCommunicationTime |  | 6.5.6.2.9 |  |  |
| LocationArea |  | 6.5.6.2.10 |  |  |
| StationaryIndication |  | 6.5.6.3.3 |  |  |
| TrafficProfile |  | 6.5.6.3.4 |  |  |
| ScheduledCommunicationType |  | 6.5.6.3.5 |  |  |
| BatteryIndication |  | 6.5.6.2.14 |  |  |
| ScheduledCommunicationTime |  | 6.5.6.2.9 |  |  |
| LocationArea |  | 6.5.6.2.10 |  |  |
| StationaryIndication |  | 6.5.6.3.3 |  |  |
| TrafficProfile |  | 6.5.6.3.4 |  |  |
| ScheduledCommunicationType |  | 6.5.6.3.5 |  |  |
| BatteryIndication |  | 6.5.6.3.14 |  |  |
| AcsInfo |  | 3GPP TS 29.571 [7] |  | ACS Information |
| IPv4AddrMask |  | 3GPP TS 29.571 [7] |  |  |
| NefId |  | 3GPP TS 29.510 [19] |  |  |
| PatchResult |  | 3GPP TS 29.571 [7] |  |  |
| NrV2xAuth |  | 3GPP TS 29.571 [7] |  |  |
| LteV2xAuth |  | 3GPP TS 29.571 [7] |  |  |
| BitRate |  | 3GPP TS 29.571 [7] |  |  |
| MdtConfiguration |  | 3GPP TS 29.571 [7] |  |  |
| Uint64 |  | 3GPP TS 29.571 [7] |  |  |

* * * Next Change * * * *

##### 6.1.6.2.37	Type: CagInfo

- Table 6.1.6.2.37-1: Definition of type CagInfo

| Attribute name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| allowedCagList | array(CagId) | M | 1..N | List of allowed CAG Ids. |
| cagOnlyIndicator | boolean | O | 0..1 | true indicates that the UE is restricted to only access 5GS via CAG cells; absence and false indicate that the UE is not restricted to only access 5GS via CAG cells. |
| cagIdHrnnList | map(CagIdHrnn) | O | 1..N | A map of CAG Id to HRNN mappings, where the CagId string value (see 3GPP TS 29.571 [7], Table 5.3.2-1) serves as a key of the map. |

* * * Next Change * * * *

##### 6.1.6.2.x	Type: CagIdHrnn

- Table 6.1.6.2.x-1: Definition of type CagIdHrnn

| Attribute name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| hrnn | string | M | 1 | Human-readable network name for a certain CAG Id |

* * * Next Change * * * *

## A.2	Nudm_SDM API

(… text not shown for clarity …)

CagData:

type: object

required:

- cagInfos

properties:

cagInfos:

description: A map (list of key-value pairs where PlmnId serves as key) of CagInfo

type: object

additionalProperties:

$ref: '#/components/schemas/CagInfo'

provisioningTime:

$ref: 'TS29571_CommonData.yaml#/components/schemas/DateTime'

CagInfo:

type: object

required:

- allowedCagList

properties:

allowedCagList:

type: array

items:

$ref: 'TS29571_CommonData.yaml#/components/schemas/CagId'

cagOnlyIndicator:

type: boolean

cagIdHrnnList:

type: object

additionalProperties:

$ref: '#/components/schemas/CagIdHrnn'

minProperties: 1

CagIdHrnn:

type: object

required:

- hrnn

properties:

hrnn:

type: string

SmfSelectionSubscriptionData:

type: object

properties:

supportedFeatures:

$ref: 'TS29571_CommonData.yaml#/components/schemas/SupportedFeatures'

subscribedSnssaiInfos:

type: object

additionalProperties:

$ref: '#/components/schemas/SnssaiInfo'

sharedSnssaiInfosId:

$ref: '#/components/schemas/SharedDataId'

(… text not shown for clarity …)

* * * End of Changes * * * *