3GPP TSG-CT WG6 Meeting #113	C6-220678

# Toulouse, France, 15th – 18th November 2022

Source:	China Mobile

Title:	Pseudo-CR on Study on  GBA_U Based APIs

Spec:	3GPP TR 31.822 v0.1.0

Agenda item:	7.13.2

Document for:	Approval

1. Introduction

The aim on this document is to introduce the test plan of Method GBA_U_Cipher_init.

2. Reason for Change

Since Classes and Interfaces of GBA_U_APIs are defined, the corresponding testing methods should be studied.

3. Proposal

It is proposed to agree the following changes to 3GPP TS 31.822 v0.1.0.

New section 6 is added to define the test methods of GBA_U_APIs from clause5, and new section 5.1.1 is added to define the test methods of GBA_U_Cipher_init from clause 5.1.2.2 .

* * * First Change * * * *

6	Test plan

6.1	package uicc.usim.gba

6.1.1	Interface GBA_U_Cipher

6.1.1.1          Method GBA_U_Cipher_init

Test Area Reference: Api_ GBA_U_Cipher_init

6.1.1.1.1           Conformance requirement

The method with following header shall be compliant to its definition in the API.

public  Class cipher.inti()

6.1.1.1.1.1         Normal execution

CRRN1: the method is successful if the appropriate parameters are input in the mode of MODE_ENCRYPT.

CRRN2:  the method is successful if the appropriate parameters are input in the mode of MODE_DECRYPT.

6.1.1.1.1.2        Parameter errors

CRRP1: in the mode of MODE_ENCRYPT, if an input parameter is not valid, an exception shall be thrown.

CRRP2: in the mode of MODE_DECRYPT, if an input parameter is not valid, an exception shall be thrown.

CRRP3: if theMode parameter is not valid, an exception shall be thrown.

6.1.1.1.1.3       Context errors

CRRC1: in the mode of MODE_ENCRYPT, if the AID of app which calls the method is not included in the EF of Access Control to GBA_U_APIs , an exception shall be thrown.

CRRC2:  in the mode of MODE_ENCRYPT, if the nafID is not included in the EF of Access Control to GBA_U_APIs, an exception shall be thrown.

CRRC3: in the mode of MODE_ENCRYPT, if the nafID is not corresponding to the app which calls the method, an exception shall be thrown

CRRC4: in the mode of MODE_ENCRYPT, neither the  nafID nor the AID of app which calls the method is included in the EF of Access Control to GBA_U_APIs,  an exception shall be thrown

CRRC5: in the mode of MODE_ENCRYPT, if AUTHENTICATE command in GBA security context (Bootstrapping Mode and ) is not executed before the method is called, an exception shall be thrown.

CRRC6: in the mode of MODE_DECRYPT, if the AID of app which calls the method is not included in the EF of Access Control to GBA_U_APIs , an exception shall be thrown.

CRRC7:  in the mode of MODE_DECRYPT, if the nafID is not included in the EF of Access Control to GBA_U_APIs, an exception shall be thrown.

CRRC8: in the mode of MODE_DECRYPT, if the nafID is not corresponding to the app which calls the method, an exception shall be thrown

CRRC9: in the mode of MODE_DECRYPT, neither the  nafID nor the AID of app which calls the method is included in the EF of Access Control to GBA_U_APIs,  an exception shall be thrown

CRRC10: in the mode of MODE_DECRYPT, if AUTHENTICATE command in GBA security context (Bootstrapping Mode and ) is not executed before the method is called, an exception shall be thrown.

6.1.1.1.2        Test area files

Test Applet:  Test_Api_GBA_Test_01.java

Cap File:  Api_GBA_Test_01.cap

6.1.1.1.3        Test coverage

| CRR number | Test case number |
| --- | --- |
| N1 | 1 |
| N2 | 2 |
| P1 | 3 to 8 |
| P2 | 9 to 14 |
| P3 | 15 |
| C1 | 16 |
| C2 | 17 |
| C3 | 18 |
| C4 | 19 |
| C5 | 20 |
| C6 | 21 |
| C7 | 22 |
| C8 | 23 |
| C9 | 24 |
| C10 | 25 |

6.1.1.1.4       Test procedure

| ID | Description | API Expectation | APDU Expectation |
| --- | --- | --- | --- |
| 0 | The content of EF for Access Control to GBA_U_APIs include NAF_ID1 and the AID of app1, which means NAF_ID1 is corresponding to app1. |  |  |
|  | Normal execution |  |  |
| 1 | Execute GBA_U Procedure with NAF_ID1 and select app1, call init() method with correct parameters in the mode of MODE_ENCRYPT. | Returns 0x9000 |  |
| 2 | Execute GBA_U Procedure with NAF_ID1 and select app1, call init() method with correct parameters in the mode of MODE_DECRYPT. | Returns 0x9000 |  |
|  | Parameter errors |  |  |
| 3 | Execute GBA_U Procedure with NAF_ID1 and select app1, call init() method with Invalid nafID in the mode of MODE_ENCRYPT. | Returns 0x9002 |  |
| 4 | Execute GBA_U Procedure with NAF_ID1 and select app1, call init() method with Invalid nafOff in the mode of MODE_ENCRYPT. | Returns 0x9003 |  |
| 5 | Execute GBA_U Procedure with NAF_ID1 and select app1, call init() method with Invalid nafLen in the mode of MODE_ENCRYPT. | Returns 0x9004 |  |
| 6 | Execute GBA_U Procedure with NAF_ID1 and select app1, call init() method with Invalid bArray in the mode of MODE_ENCRYPT. | Returns 0x9006 |  |
| 7 | Execute GBA_U Procedure with NAF_ID1 and select app1, call init() method with Invalid bOff in the mode of MODE_ENCRYPT. | Returns 0x9007 |  |
| 8 | Execute GBA_U Procedure with NAF_ID1 and select app1, call init() method with Invalid bLen in the mode of MODE_ENCRYPT. | Returns 0x9008 |  |
| 9 | Execute GBA_U Procedure with NAF_ID1 and select app1, call init() method with Invalid nafID in the mode of MODE_DECRYPT. | Returns 0x9001 |  |
| 10 | Execute GBA_U Procedure with NAF_ID1 and select app1, call init() method with Invalid nafOff in the mode of MODE_DECRYPT. | Returns 0x9002 |  |
| 11 | Execute GBA_U Procedure with NAF_ID1 and select app1, call init() method with Invalid nafLen in the mode of MODE_DECRYPT. | Returns 0x9003 |  |
| 12 | Execute GBA_U Procedure with NAF_ID1 and select app1, call init() method with Invalid bArray in the mode of MODE_DECRYPT. | Returns 0x9004 |  |
| 13 | Execute GBA_U Procedure with NAF_ID1 and select app1, call init() method with Invalid bOff in the mode of MODE_DECRYPT. | Returns 0x9006 |  |
| 14 | Execute GBA_U Procedure with NAF_ID1 and select app1, call init() method with Invalid bLen in the mode of MODE_DECRYPT. | Returns 0x9007 |  |
| 15 | Execute GBA_U Procedure with NAF_ID1 and select app1, call init() method with Invalid theMode. | Returns 0x9008 |  |
|  | Context errors |  |  |
| 16 | Execute GBA_U Procedure with NAF_ID1 and select app2, call init() method with NAF_ID1 in the mode of MODE_ENCRYPT. | Returns 0x9001 |  |
| 17 | Execute GBA_U Procedure with NAF_ID2 and select app1, call init() method with NAF_ID2 in the mode of MODE_ENCRYPT. | Returns 0x9001 |  |
| 18 | Execute GBA_U Procedure with NAF_ID1 and select app2, call init() method with NAF_ID2 in the mode of MODE_ENCRYPT. | Returns 0x9001 |  |
| 19 | Execute GBA_U Procedure with NAF_ID2 and select app1, call init() method with NAF_ID1 in the mode of MODE_ENCRYPT. | Returns 0x9001 |  |
| 20 | Select app1, call init() method with NAF_ID1 in the mode of MODE_ENCRYPT. | Returns 0x9001 |  |
| 21 | Execute GBA_U Procedure with NAF_ID1 and select app2, call init() method with NAF_ID1 in the mode of MODE_DECRYPT. | Returns 0x9001 |  |
| 22 | Execute GBA_U Procedure with NAF_ID2 and select app1, call init() method with NAF_ID2 in the mode of MODE_DECRYPT. | Returns 0x9001 |  |
| 23 | Execute GBA_U Procedure with NAF_ID1 and select app2, call init() method with NAF_ID2 in the mode of MODE_DECRYPT. | Returns 0x9001 |  |
| 24 | Execute GBA_U Procedure with NAF_ID2 and select app1, call init() method with NAF_ID1 in the mode of MODE_DECRYPT. | Returns 0x9001 |  |
| 25 | Select app1, call init() method with NAF_ID1 in the mode of MODE_DECRYPT. | Returns 0x9001 |  |

* * * End of Changes * * * *