TSG-CT WG6 #56	C6-1003xx

Venice, Italy, 24-27 August 2010

Title:	Report of SCP discussions & decisions

Source:	CT6 Vice-chairman / SCP TEC Chairman

Contact:	Paul JOLIVET – LG Electronics

# Report of SCP discussions and decisions

# SCP activity

Since the last CT6 meeting, there were 1 REQ, 1 Plenary, 1 TEC and 2 TEST meetings.  The status report of the working groups are available in SCP(10)0115 for TEST, SCP(10)0162 for TEC and SCP(10)0154 for REQ.

The main topics ongoing are:

- TLV coding:  The issue of extending TLV coding was discussed at both TEC and Plenary.  Basically the conclusion is that there is no need at this stage for 3 byte extension and that context specific and non context specific tags will be identified.

- Contactless:

  - The HCI APIs specification (TS 102 705) was approved for Rel-9 with reference to Java Card v3.0.1 Classic.

  - P2P work is still ongoing at REQ level.

- API specifications: Consistently with HCI API specification, all SCP API Rel-9 specifications were updated so that they now reference Java Card 3.0.1 Classic.

- Support of UICC on Notebooks:

  - TR 102 906 (UICC in Mobile Broadband Notebook) was approved.

  - Additional conference calls to address requirements, work is ongoing.

- New runtime environment:  The discussions are still on hold on this topic due to lack of contributions (this has lower priority compared to other urgent topics to be dealt with currently, including contactless API).  Being a SCP document, the draft specification is a publicly available document.

- IC_USB:

  - Discussion on the use of USB descriptors aside the existing Vendor ID and Product ID identification system.  The proposal (initiated by the STF391) was sent back to TEC after discussions.

  - Test STF391 provided the draft test specification for information at this SCP meeting, inline with the approved schedule at the creation of the STF.

- Spike definition (TS 102 221):  A TEC agreed solution was presented at the meeting however it was objected by email with backward compatibility issues argument.  The definition of a spike is proposed to be changed to a pulse.  There were concerns to approve directly the proposal, therefore, it was postponed to SCP #46.

# Set of approved CRs at last SCP meetings

This section only lists approved SCP document, i.e. documents presented at SCP #45.

## CRs likely to have impact on 3GPP

This section only lists approved SCP document, i.e. documents presented at SCP #

| Document number | Document title | Spec | Rel. |
| --- | --- | --- | --- |
| SCPTEC(10)0342 | #101: Context specific tags for COMPREHENSION-TLVs (revision of SCPTEC(10)0318) | 101 220 | Rel-10 |
| SCPTEC(10)0167 | #191: Cleanup of errors in BIP sections (Rel-7 revision of SCPTEC(10)0115) | 102 223 | Rel-7 |
| SCPTEC(10)0168 | #192: Cleanup of errors in BIP sections (Rel-8 revision of SCPTEC(10)0115) | 102 223 | Rel-8 |
| SCPTEC(10)0169 | #193: Cleanup of errors in BIP sections (Rel-9 revision of SCPTEC(10)0115) | 102 223 | Rel-9 |
| SCP(10)0167 | #196r1: Correction of length in registry application data (revision of SCPTEC(10)0361 in SCP(10)0149) | 102 223 | Rel-7 |
| SCP(10)0168 | #197r1: Correction of length in registry application data (revision of SCPTEC(10)0382 in SCP(10)0149) | 102 223 | Rel-8 |
| SCP(10)0169 | #198r1: Correction of length in registry application data (revision of SCPTEC(10)0383 in SCP(10)0149) | 102 223 | Rel-9 |
| SCP(10)0170 | #199r1: Addition of objects for user confirmation (revision of SCPTEC(10)0347 in SCP(10)0149) | 102 223 | Rel-10 |
| SCP(10)0182 | #118: Change reference from 'Java Card 2.2.2' to 'Java Card 3.0.1 Classic Edition' (revision of SCPTEC(10)0388 in SCP(10)0138) | 102 241 | Rel-9 |
| SCP(10)0181 | #002: Change reference from 'Java Card 2.2.2' to 'Java Card 3.0.1 Classic Edition' (revision of SCPTEC(10)0375 in SCP(10)0138) | 102 267 | Rel-9 |
| SCP(10)0180 | #001: Change reference from 'Java Card 2.2.2' to 'Java Card 3.0.1 Classic Edition' (revision of SCPTEC(10)0196 in SCP(10)0138) | 102 569 | Rel-9 |
| SCP(10)0183 | #020: Change reference from 'Java Card 2.2.2' to 'Java Card 3.0.1 Classic Edition' (revision of SCPTEC(10)0389 in SCP(10)0138) | 102 588 | Rel-9 |
| SCPTEC(10)0191 | #011: Minimum value for "bMaxCurrent" after power negotiation | 102 600 | Rel-7 |
| SCPTEC(10)0198 | Clarifications on support of Remote Wake Up (revision of SCPTEC(10)0138) | 102 600 | Rel-7 |
| SCPTEC(10)0379 | #015: clarification on Remote wakeup and resume signalling time (revision of SCPTEC(10)0339) | 102 600 | Rel-7 |
| SCPTEC(10)0380 | #016: Remote wakeup time negotiation (revision of SCPTEC(10)0312) | 102 600 | Rel-10 |

## CRs unlikely to have impact on 3GPP

This section only lists approved SCP document, i.e. documents presented at SCP #

| Document number | Document title | Spec | Rel. |
| --- | --- | --- | --- |
| SCPTEC(10)0390 | #097r1: reservation of a TAR value for OMA BCAST Audience Measurement (replaces SCPTEC(10)0159 - wrong spec version) | 101 220 | Rel-10 |
| SCPTEC(10)0343 | #098: Removal of redundant information in clause 6 (revision of SCPTEC(10)0313) | 101 220 | Rel-10 |
| SCPTEC(10)0344 | #099: Creation of empty specification referencing the next Release (TS 101 220 Rel-10) (revision of SCPTEC(10)0314) | 101 220 | Rel-9 |
| SCPTEC(10)0345 | #100: Correction of status of MM Transfer Status tag (revision of SCPTEC(10)0319) | 101 220 | Rel-10 |
| SCP(10)0165 | #276r3: Application selection on Application-to-application Secure Channel (revision of SCPTEC(10)0394 in SCP(10)0145) | 102 221 | Rel-8 |
| SCP(10)0173 | #277r6: Behaviour for Manage Secure Channel - Terminate SA (revision of SCPTEC(10)0162 in SCP(10)0130) | 102 221 | Rel-7 |
| SCP(10)0196 | #282r6: Application selection on Application-to-application Secure Channel (revision of SCP(10)0164) | 102 221 | Rel-7 |
| SCP(10)0174 | #289r1: Behaviour for Manage Secure Channel - Terminate SA (revision of SCPTEC(10)0210 in SCP(10)0130) | 102 221 | Rel-8 |
| SCP(10)0175 | #290r1: Behaviour for Manage Secure Channel - Terminate SA (revision of SCPTEC(10)0211 in SCP(10)0130) | 102 221 | Rel-9 |
| SCP(10)0166 | #291r2: Application selection on Application-to-application Secure Channel (revision of SCPTEC(10)0395 in SCP(10)0145) | 102 221 | Rel-9 |
| SCPTEC(10)0171 | #194: Correction to NMR multiple technologies (revision of SCPTEC(10)0121) | 102 223 | Rel-8 |
| SCPTEC(10)0172 | #195: Correction to NMR multiple technologies (revision of SCPTEC(10)0121) | 102 223 | Rel-9 |
| SCPTEC(10)0173 | #081: Correction of one statement for response structures using indefinite length coding (revision of SCPTEC(10)0101) | 102 226 | Rel-9 |
| SCPTEC(10)0387 | #082: Correction of scenario 1 (revision of SCPTEC(10)0315) | 102 226 | Rel-9 |
| SCPTest(10)0130 | Correction of maximum allowed rise/fall time on contact C7 in test 5.2.5.2 | 102 230 |  |
| SCPTest(10)0060 | Test case 6.1.4.4 Method getContentType |  |  |
| SCPTest(10)0061 | Test case 6.1.5.4 ID 4 not in line with TS 102 588 |  |  |
| SCPTest(10)0062 | Test case 6.2.1.2 Registration remains valid if applet is not in selectable state |  |  |
| SCP(10)0158 | #045: Addition of a requirement for expanded support of large HTTP messages in SCWS API (TS 102 588) | 102 412 | Rel-10 |
| SCP(10)0159 | #046: Introduction of requirements for a selective secure channel able to handle toolkit commands | 102 412 | Rel-10 |
| SCP(10)0176 | #017r1: TERMINATE SA behaviour for application-to-application Secure Channel (revision of SCPTEC(10)0207 in SCP(10)0130) | 102 484 | Rel-7 |
| SCP(10)0177 | #018r1: TERMINATE SA behaviour for application-to-application Secure Channel (revision of SCPTEC(10)0208 in SCP(10)0130) | 102 484 | Rel-8 |
| SCP(10)0178 | #019r1: TERMINATE SA behaviour for application-to-application Secure Channel (revision of SCPTEC(10)0209 in SCP(10)0130) | 102 484 | Rel-9 |
| SCPTEC(10)0194 | #018: Corrections in the Java files (Rel-7 revision of SCPTEC(10)0103) | 102 588 | Rel-7 |
| SCPTEC(10)0195 | #019: Corrections in the Java files (Rel-8 revision of SCPTEC(10)0103) | 102 588 | Rel-8 |
| SCPTEC(10)0378 | #014: Clarifications of inconsistencies (revision of SCPTEC(10)0199) | 102 600 | Rel-10 |
| SCPTEC(10)0177 | #067: Correction of REJ frame processing (Rel-7 revision of SCPTEC(10)0106) | 102 613 | Rel-7 |
| SCPTEC(10)0178 | #068: Correction of REJ frame processing (Rel-8 revision of SCPTEC(10)0106) | 102 613 | Rel-8 |
| SCPTEC(10)0179 | #069: Correction of REJ frame processing (Rel-9 revision of SCPTEC(10)0106) | 102 613 | Rel-9 |
| SCPTEC(10)0174 | #045: Correction of contactless reader operation (Rel-7 revision of SCPTEC(10)0136) | 102 622 | Rel-7 |
| SCPTEC(10)0175 | #046: Correction of contactless reader operation (Rel-8 revision of SCPTEC(10)0136) | 102 622 | Rel-8 |
| SCPTEC(10)0176 | #047: Correction of contactless reader operation (Rel-9 revision of SCPTEC(10)0136) | 102 622 | Rel-9 |
| SCPTEC(10)0180 | #048: Correction of procedure for contactless card emulation (revision of SCPTEC(10)0140) | 102 622 | Rel-10 |
| SCPTEC(10)0349 | #050: Definition of source gate assignment (Rel-7 revision of SCPTEC(10)0075) | 102 622 | Rel-7 |
| SCPTEC(10)0350 | #051: Definition of source gate assignment (Rel-8 revision of SCPTEC(10)0075) | 102 622 | Rel-8 |
| SCPTEC(10)0351 | #052: Definition of source gate assignment (Rel-9 revision of SCPTEC(10)0075) | 102 622 | Rel-9 |
| SCP(10)0172 | #053r1: Collection of clarifications (revision of SCPTEC(10)0360 in SCP(10)0135) | 102 622 | Rel-10 |
| SCP(10)0187 | #001r1: Correction of automotive vibration and automotive shock clauses (revision of SCPTEC(10)0381 in SCP(10)0136) | 102 671 | Rel-9 |
| SCPTest(10)0129 | Implementation of test case 6.1.4.3 | 102 835 | Rel-7 |
| SCPTest(10)0063 | Corrections to test case 5.9.2.2.2 | 102 694-1 | Rel-7 |
| SCPTest(10)0064 | Modify test case 5.9.2.1.2 | 102 694-1 | Rel-7 |
| SCPTest(10)0135 | Improved testing of idle bits | 102 694-1 | Rel-7 |
| SCPTest(10)0137 | Test case 5.3.2.3.19: added RSET to last step of test case | 102 694-1 | Rel-7 |
| SCPTest(10)0138 | Test case 5.6.2.2.3: clear specification of sequence numbers | 102 694-1 | Rel-7 |
| SCPTest(10)0139 | Correction of specification regarding ACT_INFORMATION | 102 694-1 | Rel-7 |
| SCPTest(10)0140 | Test case 5.3.2.2.2: correction of applicability | 102 694-1 | Rel-7 |
| SCPTest(10)0141 | Test case 5.3.2.5.2: removal due to redundancy | 102 694-1 | Rel-7 |
| SCPTest(10)0142 | Test cases 5.6.3.2.X: Updated to send subsequent frames in order to check for response to invalid frame | 102 694-1 | Rel-7 |
| SCPTest(10)0143 | Change of usage of 10 I-frames throughout specification to 9 I-frames | 102 694-1 | Rel-7 |
| SCPTest(10)0157 | Removal of redundant applicability statements | 102 694-1 | Rel-7 |
| SCP(10)0188 | #012r1: HCP message fragmentation (revision of SCPTEST(10)0145 in SCP(10)0118) | 102 694-1 | Rel-7 |
| SCP(10)0189 | #013r1: Clarification of O_WS_3 (revision of SCPTEST(10)0155 in SCP(10)0118) | 102 694-1 | Rel-7 |
| SCP(10)0190 | #021: Update of SHDLC test cases to align with TS 102 694-1 (revision of SCPTEST(10)0067 in SCP(10)0119) | 102 694-2 | Rel-7 |
| SCP(10)0191 | #028r1: HCP message fragmentation (revision of SCPTEST(10)0146 in SCP(10)0119) | 102 694-2 | Rel-7 |
| SCP(10)0192 | #034r1: Clarification of O_WS_3 (revision of SCPTEST(10)0156 in SCP(10)0119) | 102 694-2 | Rel-7 |
| SCP(10)0195 | #001r1: Correction of card emulation test procedures and initial conditions (revision of SCPTEST(10)0152 in SCP(10)0120) | 102 695-2 | Rel-7 |
| SCP(10)0193 | #007r1: Define the host network initial state (revision of SCPTEST(10)0136 in SCP(10)0121) | 102 695-2 | Rel-7 |

# Work Items

Development of conformance test specifications for internet protocol connectivity

Target date for approval of the document is updated to mid-2011.

Specification of a new Enhanced Runtime Environment based on Java Card™

Target date for approval of the document is updated to SCP #48 (mid 2011).

# Latest version of the specifications

The following table is providing a list of the latest version of the ETSI SCP specification available on the ETSI portal at the time of this meeting.

| Spec | R99 | Rel-4 | Rel-5 | Rel-6 | Rel-7 | Rel-8 | Rel-9 | Rel-10 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| TS 101 220 ETSI numbering system for telecommunication application providers | withdrawn | withdrawn | 5.2.0 | 6.7.0 | 7.13.0 | 8.5.0 | 9.2.0 |  |
| TR 102 151 Measurement of Electromagnetic Emission of SIM Cards |  |  |  | 6.0.0 |  |  |  |  |
| TS 102 124 Transport Protocol for UICC based Applications;Stage 1 |  |  |  | 6.1.0 |  |  |  |  |
| TS 102 127 Transport protocol for CAT applications; Stage 2 |  |  |  | 6.13.0 |  |  |  |  |
| TR 102 216 Vocabulary for Smart Card Platform specifications | 3.0.0 |  |  |  |  |  |  |  |
| TS 102 221 UICC-Terminal interface; Physical and logical characteristics | 3.18.0 | 4.16.0 | 5.10.0 | 6.15.0 | 7.17.0 | 8.4.0 | 9.1.0 |  |
| TS 102 222 Administrative commands for telecommunications applications |  | 4.2.0 | 5.2.0 | 6.11.0 |  |  |  |  |
| TS 102 223 Card Application Toolkit (CAT) |  | 4.13.0 | 5.11.0 | 6.14.0 | 7.14.0 | 8.5.0 | 9.1.0 |  |
| TS 102 224 Security mechanisms for UICC based Applications - Functional requirements |  |  |  | 6.1.0 | 7.1.0 | 8.0.0 |  |  |
| TS 102 225 Secured packet structure for UICC based application |  |  |  | 6.8.0 | 7.4.0 | 8.3.0 | 9.0.0 |  |
| TS 102 226 Remote APDU structure for UICC based applications |  |  |  | 6.18.0 | 7.8.1 | 8.2.1 | 9.2.0 |  |
| TS 102 230 UICC-Terminal interface; Physical, electrical and logical test specification | 3.12.0 | 4.12.0 | 5.10.0 | 6.0.0 | 7.2.0 | 8.0.0 |  |  |
| TS 102 240 UICC Application Programming Interface and Loader Requirements; Service description |  |  |  | 6.2.0 | 7.0.0 | 8.0.0 |  |  |
| TS 102 241 UICC Application Programming Interface (UICC API) for Java CardTM |  |  |  | 6.12.0 | 8.9.0 | 8.0.0 |  |  |
| TR 102 242 Terminal - card interface; Considerations on robustness improvements | 3.0.0 |  |  |  |  |  |  |  |
| TS 102 266 USSM: UICC Security Service Module; Stage 1 |  |  |  |  | 7.1.0 |  |  |  |
| TS 102 267 Connection Oriented Service API for the Java CardTM platform |  |  |  |  | 7.1.0 |  |  |  |
| TS 102 268 Test specification for the UICC Application Programming Interface (AP)I for Java CardTM |  |  |  | 6.0.0 |  |  |  |  |
| TS 102 310 Extensible Authentication Protocol support in the UICC |  |  |  | 6.2.0 | 7.0.0 | 8.2.0 | 9.0.0 |  |
| TS 102 350 Identity Files and Procedures on a UICC; Stage 1 |  |  |  |  | 7.0.0 |  |  |  |
| TS 102 384 UICC-Terminal interface; Card Application Toolkit (CAT) conformance specification |  |  |  | 6.6.0 | 7.1.0 |  |  |  |
| TS 102 412 Smart Card Platform Requirements Stage 1 |  |  |  |  | 7.8.0 | 8.4.0 | 9.1.0 | 10.0.0 |
| TS 102 431 Test specification for the Transport Protocol of CAT Applications (CAT_TP) validation |  |  |  |  | 7.1.0 |  |  |  |
| TR 102 481 Requirements for UICC - external peripherals data exchange |  |  |  |  | 7.0.0 |  |  |  |
| TS 102 483 UICC-Terminal interface;Internet Protocol connectivity between UICC and terminal |  |  |  |  | 7.3.0 | 8.1.0 |  |  |
| TS 102 484 Secure channel between a UICC and an end-point terminal |  |  |  |  | 7.6.0 | 8.0.0 | 9.0.0 |  |
| TS 102 569 UICC Security Service Module (USSM);Stage 2 |  |  |  |  | 7.0.0 |  |  |  |
| TS 102 588 Application invocation Application Programming Interface (API) by a UICC webserver for Java Card™ platform |  |  |  |  | 7.2.0 |  |  |  |
| TS 102 600 UICC-Terminal interface;Characteristics of the USB interface |  |  |  |  | 7.7.0 |  |  |  |
| TR 102 613 UICC - Contactless Front-end (CLF) Interface;Part 1: Physical and data link layer characteristics |  |  |  |  | 7.8.0 | 8.1.0 | 9.1.0 |  |
| TS 102 622 UICC - Contactless Front-end (CLF) Interface;Host Controller Interface (HCI) |  |  |  |  | 7.6.0 | 8.0.0 | 9.0.0 |  |
| TS 102 671 Machine to Machine UICC;Physical and logical characteristics |  |  |  |  |  |  | 9.0.0 |  |
| TS 102 694-1 Test specification for the Single Wire Protocol (SWP) interface;Part 1: Terminal features |  |  |  |  | 7.0.0 |  |  |  |
| TS 102 694-2 Test specification for the Single Wire Protocol (SWP) interface;Part 2: UICC features |  |  |  |  | 7.0.0 |  |  |  |
| TS 102 694-3 |  |  |  |  | 7.0.0 |  |  |  |
| TS 102 694-4 |  |  |  |  | 7.0.0 |  |  |  |
| TS 102 695-1 Test specification for the Host Controller Interface (HCI) Part 1: Terminal features |  |  |  |  | 7.0.0 |  |  |  |
| TS 102 695-2 Test specification for the Host Controller Interface (HCI);Part 2: UICC features |  |  |  |  | 7.0.0 |  |  |  |
| TS 102 695-3 Test specification for the Host Controller Interface (HCI) Part 3: Host Controller features |  |  |  |  | 7.0.0 |  |  |  |
| TS 102 695-4 |  |  |  |  | 7.0.0 |  |  |  |
| TS 102 835 Test Specification for SCWS Application Invocation API for Java CardTM;Tests Environment and Annexes |  |  |  |  | 7.0.0 |  |  |  |

# Next SCP meetings

The following table is a copy of the SCP meetings (and mention of CT/CT6 meetings) as presented by the SCP secretary at the last SCP TEC meeting.  It reflects the latest schedule of meetings.

| Meeting ID | Dates | Location |
| --- | --- | --- |
| 2010 |  |  |
| SCP TEC #34 | 06 - 10 September 2010 | Sophia Antipolis, France |
| SCP TEST #15 | 14 – 16 September 2010 | TBD |
| CT-49 | 15 – 17 September 2010 | TBD |
| SCP TEC #35 | 04 - 08 October 2010 | Sophia Antipolis, France |
| SCP REQ #27 | TBD November 2010 | TBD |
| SCP #46 | TBD November 2010 | TBD |
| CT6-57 | 16 - 19 November 2010 | Lisbon, Portugal |
| CT-50 | 08 – 10 December 2010 | TBD |
| SCP TEC #36 | 13 - 17 December 2010 | Sophia Antipolis, France |
| 2011 |  |  |
| SCP REQ #28 | TBD end of February 2011 | TBD |
| SCP #47 | TBD end of February 2011 | TBD |