3GPP TSG-CT6#56	C6-100400

- Venice, Italy, 24-27 August 2010

| CR-Form-v9.7 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 31.102 | CR | 0444 |  rev | 1 |  Current version: | 8.9.0 |  |
| For HELP on using this form look at the pop-up text over the  symbols. Comprehensive instructions on how to use this form can be found at http://www.3gpp.org/specs/CR.htm. |  |  |  |  |  |  |  |  |

| Proposed change affects:  | UICC apps | X | ME | X | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title:  | Introduction of a DF_TELECOM EF to support Public Service Identity for SMS over IP. |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG:  | Research In Motion, TeliaSonera, Verizon Wireless |  |  |  |  |  |  |  |  |  |
| Source to TSG:  | C6 |  |  |  |  |  |  |  |  |  |
| Work item code:  | TEI8 |  |  |  |  |  |  | Date:  |  | 26/08/2010 |
| Category:  | F |  |  |  |  |  |  | Release:  |  | Rel-8 |
|  | Use one of the following categories: F (correction) A (corresponds to a correction in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: R99 (Release 1999) Rel-4 (Release 4) Rel-5 (Release 5) Rel-6 (Release 6) Rel-7 (Release 7) Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) |  |
| Reason for change:  |  | Currently SMS over IP stage 3 TS 24.341 states: #### 5.3.1.2 Submitting a short message When an SM-over-IP sender wants to submit an SM over IP, the SM-over-IP sender shall send a SIP MESSAGE request with the following information: a) the Request-URI, which shall contain the PSI of the SC of the SM-over-IP sender; NOTE 1: The PSI of the SC can be SIP URI or tel URI based on operator policy. Also in subclause 5.3.1.2 NOTE 4: The address of the SC is included in the RP-DATA message content. The address of the SC is configured in the SM-over-IP sender. However currently there is no means to configure the UE with either the SIP URI or tel URI for the Public Service Identity of the SMS service center (SM-SC) based on operator policy as specified in NOTE 1 of subclause 5.3.1.2 of TS 24.341.This is needed is order to enable the UE to submit an SMS’ over IP network as defined in TS 24.341. |  |  |  |  |  |  |  |  |
| Summary of change:  |  | Added the SMS of IP specification in the reference section along with relevant SIP RFCs. Introduced a new EF in DF_TELECOM to store either a SIP URI or TEL URI of the Public Service Identity of the SM-SC that will be read by the ME. Added the new EF to the DF_TELECOM file structure. Added the the new EF to Annex A. |  |  |  |  |  |  |  |  |
| Consequences if  not approved: |  | There will continue to be no mechanism for UEs to use the appropriate SM-SC Public Service Identity for SMS’ over IP and for the operator to configure the correct SM-SC Public Service Identity. |  |  |  |  |  |  |  |  |
| Clauses affected:  |  | 2, 4.6.1.X, 4.7, 5.3.Y, Annex A, Annex E |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs  |  |  |  | Other core specifications  |  |  |  |  |  |  |
| affected: |  |  |  | Test specifications |  |  |  |  |  |  |
|  |  |  |  | O&M Specifications |  |  |  |  |  |  |
| Other comments:  |  |  |  |  |  |  |  |  |  |  |

# 2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

- References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

- For a specific reference, subsequent revisions do not apply.

- For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TS 21.111: "USIM and IC Card Requirements".

[2]	3GPP TS 22.011: "Service accessibility".

[3]	3GPP TS 22.024: "Description of Charge Advice Information (CAI)".

[4]	3GPP TS 22.030: "Man-Machine Interface (MMI) of the User Equipment (UE)".

[5]	3GPP TS 23.038: "Alphabets and language".

[6]	3GPP TS 23.040: "Technical realization of the Short Message Service (SMS)".

[7]	3GPP TS 23.060: "General Packet Radio Service (GPRS); Service description; Stage 2".

[8]	3GPP TS 22.067: "enhanced Multi Level Precedence and Pre-emption service (eMLPP) - Stage 1".

[9]	3GPP TS 24.008: "Mobile Radio Interface Layer 3 specification; Core Network Protocols; Stage 3".

[10]	3GPP TS 24.011: "Point-to-Point (PP) Short Message Service (SMS) support on mobile radio interface".

[11]	3GPP TS 31.101: "UICC-Terminal Interface, Physical and Logical Characteristics".

[12]	3GPP TS 31.111: "USIM Application Toolkit (USAT)".

[13]	3GPP TS 33.102: "3GPP Security; Security Architecture".

[14]	3GPP TS 33.103: "3GPP Security; Integration Guidelines".

[15]	3GPP TS 22.086: "Advice of charge (AoC) Supplementary Services - Stage 1".

[16]	3GPP TS 23.041: "Technical realization of Cell Broadcast (CB)".

[17]	3GPP TS 02.07: "Mobile Stations (MS) features".

[18]	3GPP TS 51.011 Release 4: "Specification of the Subscriber Identity Module – Mobile Equipment (SIM – ME) interface".

[19]	ISO 639 (1988): "Code for the representation of names of languages".

[20]	ISO/IEC 7816-4: "Integrated circuit cards, Part 4: Organization, security and commands for interchange".

[21]	Void.

[22]	ITU-T Recommendation E.164: "The international public telecommunication numbering plan".

[23]	3GPP TS 23.073: "Support of Localised Service Area (SoLSA); Stage 2".

[24]	3GPP TS 22.101: "Service aspects; service principles".

[25]	3GPP TS 23.003: "Numbering, Addressing and Identification".

[26]	Void.

[27]	3GPP TS 22.022: "Personalisation of Mobile Equipment (ME); Mobile functionality specification".

[28]	3GPP TS 44.018 "Mobile Interface Layer3 Specification, Radio Resource control protocol".

[29]	3GPP TS 23.022: "Functions related to Mobile Station (MS) in idle mode and group receive mode".

[30]	3GPP TS 23.057: "Mobile Execution Environment (MExE);Functional description; Stage 2".

[31]	3GPP TS 23.122: "NAS Functions related to Mobile Station (MS) in idle mode".

[32]	Void.

[33]	3GPP TS 25.101: "UE Radio Transmission and Reception (FDD)".

[34]	3GPP TS 45.005: "Radio Transmission and Reception".

[35]	ISO/IEC 8825 (1990): "Information technology; Open Systems Interconnection; Specification of Basic Encoding Rules for Abstract Syntax Notation One (ASN.1)".

[36]	3GPP TS 23.097: "Multiple Subscriber Profile (MSP)".

[37]	Void.

[38]	3GPP TS 23.140 Release 6: "Multimedia Messaging Service (MMS); Functional description; stage 2".

[39]	ETSI TS 102 222 V7.1.0: "Administrative commands for telecommunications applications".

[40]	3GPP TS 24.234: "3GPP System to WLAN Interworking; UE to Network protocols;Stage 3".

[41]	3GPP TS 33.234: "3G Security; Wireless Local Area Network (WLAN) interworking security".

[42]	3GPP TS 33.220: "Generic Authentication Architecture (GAA); Generic bootstrapping architecture".

[43]	3GPP TS 33.246: "Security of Multimedia Broadcast/Multicast Service".

[44]	3GPP TS 43.020: "Technical Specification Group Services and system Aspects; Security related network functions"

[45]	X.S0016-000-A v1.0: "3GPP2 Multimedia Messaging System MMS Specification Overview, Revision A"

[46]	3GPP TS 43.068: "Technical Specification Group Core Network; Voice Group Call Service (VGCS); Stage 2".

[47]	3GPP TS 33.110: "Key establishment between a Universal Integrated Circuit Card (UICC) and a terminal".

[48]	IETF RFC 3629 (2003): "UTF-8, a transformation format of ISO 10646".

[49]	Open Mobile Alliance; OMA-TS-BCAST_SvcCntProtection
URL: http://www.openmobilealliance.org/

[50]	ETSI TS TS 102 483 release 7: "UICC-Terminal interface; Internet Protocol connectivity between UICC and Terminal".

[51]	3GPP TS 24.301: "Technical Specification Group Core Network and Terminals; Non-Access-Stratum (NAS) protocol for Evolved Packet Systems (EPS): Stage 3".

[52]	3GPP TS 33.401: "3GPP System Architecture Evolution (SAE); Security architecture".

[XX]	3GPP TS 24.341: "Support of SMS over IP networks; Stage 3"

[XY]	IETF RFC 3261: "SIP: Session Initiation Protocol".

[XZ]	IETF RFC 3629 (2003): "UTF-8, a transformation format of ISO 10646".

### 4.2.8	EFUST (USIM Service Table)

This EF indicates which services are available. If a service is not indicated as available in the USIM, the ME shall not select this service.

| Identifier: '6F38' |  | Structure: transparent |  |  | Mandatory |  |
| --- | --- | --- | --- | --- | --- | --- |
| SFI: '04' |  |  |  |  |  |  |
| File size: X bytes, (X ≥ 1) |  |  | Update activity: low |  |  |  |
| Access Conditions: READ PIN UPDATE ADM DEACTIVATE ADM ACTIVATE ADM |  |  |  |  |  |  |
| Bytes | Description |  |  | M/O |  | Length |
| 1 | Services n°1 to n°8 |  |  | M |  | 1 byte |
| 2 | Services n°9 to n°16 |  |  | O |  | 1 byte |
| 3 | Services n°17 to n°24 |  |  | O |  | 1 byte |
| 4 | Services n°25 to n°32 |  |  | O |  | 1 byte |
| etc. |  |  |  |  |  |  |
| X | Services n°(8X-7) to n°(8X) |  |  | O |  | 1 byte |

| -Services |  |  |
| --- | --- | --- |
| Contents: | Service n°1: | Local Phone Book |
|  | Service n°2: | Fixed Dialling Numbers (FDN) |
|  | Service n°3: | Extension 2 |
|  | Service n°4: | Service Dialling Numbers (SDN) |
|  | Service n°5: | Extension3 |
|  | Service n°6: | Barred Dialling Numbers (BDN) |
|  | Service n°7: | Extension4 |
|  | Service n°8: | Outgoing Call Information (OCI and OCT) |
|  | Service n°9: | Incoming Call Information (ICI and ICT) |
|  | Service n°10: | Short Message Storage (SMS) |
|  | Service n°11: | Short Message Status Reports (SMSR) |
|  | Service n°12: | Short Message Service Parameters (SMSP) |
|  | Service n°13: | Advice of Charge (AoC) |
|  | Service n°14: | Capability Configuration Parameters 2 (CCP2) |
|  | Service n°15: | Cell Broadcast Message Identifier |
|  | Service n°16: | Cell Broadcast Message Identifier Ranges |
|  | Service n°17: | Group Identifier Level 1 |
|  | Service n°18: | Group Identifier Level 2 |
|  | Service n°19: | Service Provider Name |
|  | Service n°20: | User controlled PLMN selector with Access Technology |
|  | Service n°21: | MSISDN |
|  | Service n°22: | Image (IMG) |
|  | Service n°23: | Support of Localised Service Areas (SoLSA) |
|  | Service n°24: | Enhanced Multi-Level Precedence and Pre-emption Service |
|  | Service n°25: | Automatic Answer for eMLPP |
|  | Service n°26: | RFU |
|  | Service n°27: | GSM Access |
|  | Service n°28: | Data download via SMS-PP |
|  | Service n°29: | Data download via SMS-CB |
|  | Service n°30: | Call Control by USIM |
|  | Service n°31: | MO-SMS Control by USIM |
|  | Service n°32: | RUN AT COMMAND command |
|  | Service n°33: | shall be set to '1' |
|  | Service n°34: | Enabled Services Table |
|  | Service n°35: | APN Control List (ACL) |
|  | Service n°36: | Depersonalisation Control Keys |
|  | Service n°37: | Co-operative Network List |
|  | Service n°38: | GSM security context |
|  | Service n°39: | CPBCCH Information |
|  | Service n°40: | Investigation Scan |
|  | Service n°41: | MExE |
|  | Service n°42: | Operator controlled PLMN selector with Access Technology |
|  | Service n°43: | HPLMN selector with Access Technology |
|  | Service n°44: | Extension 5 |
|  | Service n°45: | PLMN Network Name |
|  | Service n°46: | Operator PLMN List |
|  | Service n°47: | Mailbox Dialling Numbers |
|  | Service n°48: | Message Waiting Indication Status |
|  | Service n°49: | Call Forwarding Indication Status |
|  | Service n°50: | Reserved and shall be ignored |
|  | Service n°51: | Service Provider Display Information |
|  | Service n°52 | Multimedia Messaging Service (MMS) |
|  | Service n°53 | Extension 8 |
|  | Service n°54 | Call control on GPRS by USIM |
|  | Service n°55 | MMS User Connectivity Parameters |
|  | Service n°56 | Network's indication of alerting in the MS (NIA) |
|  | Service n°57 | VGCS Group Identifier List (EFVGCS and EFVGCSS) |
|  | Service n°58 | VBS Group Identifier List (EFVBS and EFVBSS) |
|  | Service n°59 | Pseudonym |
|  | Service n°60 | User Controlled PLMN selector for I-WLAN access |
|  | Service n°61 | Operator Controlled PLMN selector for I-WLAN access |
|  | Service n°62 | User controlled WSID list |
|  | Service n°63 | Operator controlled WSID list |
|  | Service n°64 | VGCS security |
|  | Service n°65 | VBS security |
|  | Service n°66 | WLAN Reauthentication Identity |
|  | Service n°67 | Multimedia Messages Storage |
|  | Service n°68 | Generic Bootstrapping Architecture (GBA) |
|  | Service n°69 | MBMS security |
|  | Service n°70 | Data download via USSD and USSD application mode |
|  | Service n°71 | Equivalent HPLMN |
|  | Service n°72 | Additional TERMINAL PROFILE after UICC activation |
|  | Service n°73 | Equivalent HPLMN Presentation Indication |
|  | Service n°74 | Last RPLMN Selection Indication |
|  | Service n°75 | OMA BCAST Smart Card Profile |
|  | Service n°76 | GBA-based Local Key Establishment Mechanism |
|  | Service n°77 | Terminal Applications |
|  | Service n°78 | Service Provider Name Icon |
|  | Service n°79 | PLMN Network Name Icon |
|  | Service n°80 | Connectivity Parameters for USIM IP connections |
|  | Service n°81 | Home I-WLAN Specific Identifier List |
|  | Service n°82 | I-WLAN Equivalent HPLMN Presentation Indication |
|  | Service n°83 | I-WLAN HPLMN Priority Indication |
|  | Service n°84 | I-WLAN Last Registered PLMN |
|  | Service n°85 | EPS Mobility Management Information |
|  | Service n°86 | Allowed CSG Lists and corresponding indications |
|  | Service n°87 | Call control on EPS PDN connection by USIM |
|  | Service n°88 | HPLMN Direct Access |
|  | Service n°89 | eCall Data |
|  | Service n°xx | Support for SM-over-IP |

The EF shall contain at least one byte. Further bytes may be included, but if the EF includes an optional byte, then it is mandatory for the EF to also contain all bytes before that byte. Other services are possible in the future and will be coded on further bytes in the EF. The coding falls under the responsibility of the 3GPP.

Coding:

1 bit is used to code each service:
bit = 1: service available;
bit = 0: service not available.

-	Service available means that the USIM has the capability to support the service and that the service is available for the user of the USIM unless the service is identified as "disabled" in EFEST.
Service not available means that the service shall not be used by the USIM user, even if the USIM has the capability to support the service.

First byte:

|  |  | b8 |  | b7 |  | b6 |  | B5 |  | B4 |  | b3 |  | b2 |  | b1 |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | Service n°1 |
|  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | Service n°2 |
|  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | Service n°3 |
|  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | Service n°4 |
|  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | Service n°5 |
|  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | Service n°6 |
|  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | Service n°7 |
|  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | Service n°8 |

Second byte:

|  |  | b8 |  | b7 |  | b6 |  | B5 |  | B4 |  | b3 |  | b2 |  | b1 |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | Service n°9 |
|  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | Service n°10 |
|  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | Service n°11 |
|  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | Service n°12 |
|  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | Service n°13 |
|  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | Service n°14 |
|  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | Service n°15 |
|  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | Service n°16 |

etc.

### 4.6.1.X	EFPSISMSC (Public Service Identity of the SM-SC)

This file shall be present if and only if service n°12 and n°xx are "available".

This EF contains the Public Service Identity of the SM-SC (either a SIP URI or tel URI) that the ME shall use to submit  SMS over IP as defined in 24.341 [XX].

| Identifier: '6FYY' |  | Structure: linear fixed |  |  | Optional |  |
| --- | --- | --- | --- | --- | --- | --- |
| SFI: ‘04’ |  |  |  |  |  |  |
| File size: X bytes |  |  | Update activity: low |  |  |  |
| Access Conditions: READ PIN UPDATE PIN DEACTIVATE ADM ACTIVATE ADM |  |  |  |  |  |  |
| Bytes | Description |  |  | M/O |  | Length |
| 1 to X | URI TLV data object |  |  | M |  | X bytes |

-	URI

Contents:

-	SIP URI or TEL URI of the Public Service Identity of the SM-SC.

Coding:

-	For contents and syntax of URI TLV data object values see IETF RFC 3261 [XY]. The URI shall be encoded to an octet string according to UTF-8 encoding rules as specified in IETF RFC 3629 [XZ]. The tag value of the URI TLV data object shall be '80'.

## 4.7	Files of USIM

This clause contains two figures depicting the file structure of the UICC and the ADFUSIM. ADFUSIM shall be selected using the AID and information in EFDIR.

|  |  |  |  |  |  |  |  |  |  |  | MF |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  |  |  |  |  |  | '3F00' |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  |  |  |  | EFDIR |  |  | EFPL |  |  | EFARR |  |  | EFICCID |  |  |  |  |
|  |  |  |  |  |  |  |  | '2F00' |  |  | '2F05' |  |  | '2F06' |  |  | '2FE2' |  |  |  |  |
|  |  | DFGSM |  |  | see TS |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | '7F20' |  |  | 51.011 [18] |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | DFTELECOM |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | '7F10' |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  |  |  |  | EFARR |  |  | EFADN |  |  | EFFDN |  |  | EFSMS |  |  | EFMSISDN |  |
|  |  |  |  |  |  |  |  | '6F06' |  |  | '6F3A' |  |  | '6F3B' |  |  | '6F3C' |  |  | '6F40' |  |
|  |  |  |  |  |  |  |  | EFSMSP |  |  | EFSMSS |  |  | EFLND |  |  | EFSMSR |  |  | EFSDN |  |
|  |  |  |  |  |  |  |  | '6F42' |  |  | '6F43' |  |  | '6F44' |  |  | '6F47' |  |  | '6F49' |  |
|  |  |  |  |  |  |  |  | EFEXT1 |  |  | EFEXT2 |  |  | EFEXT3 |  |  | EFBDN |  |  | EFEXT4 |  |
|  |  |  |  |  |  |  |  | '6F4A' |  |  | '6F4B' |  |  | '6F4C' |  |  | '6F4D' |  |  | '6F4E' |  |
|  |  |  |  |  |  |  |  | EFECCP |  |  | EFSUME |  |  | EFICE_DN |  |  | EFICE_FF |  |  |  |  |
|  |  |  |  |  |  |  |  | '6F4F' |  |  | '6F54' |  |  | '6FE0' |  |  | '6FE1' |  |  |  |  |
|  |  |  |  |  |  |  |  | EFPSISMSC |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  |  |  |  | '6FYY' |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  | DFGRAPHICS |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  | '5F50' |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  |  |  |  | EFIMG |  |  | EFIIDFn |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  |  |  |  | '4F20' |  |  | '4FXX' |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  | DFPHONEBOOK |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  | '5F3A' |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  |  |  |  | EFPSC |  |  | EFCC |  |  | EFPUID |  |  | EFPBR |  |  | EFUID |  |
|  |  |  |  |  |  |  |  | '4F22' |  |  | '4F23' |  |  | '4F24' |  |  | '4F30' |  |  | '4FXX' |  |
|  |  |  |  |  |  |  |  | EFCCP1 |  |  | EFIAP |  |  | EFADN |  |  | EFEXT1 |  |  | EFPBC |  |
|  |  |  |  |  |  |  |  | '4FXX' |  |  | '4FXX' |  |  | '4FXX' |  |  | '4FXX' |  |  | '4FXX' |  |
|  |  |  |  |  |  |  |  | EFGRP |  |  | EFAAS |  |  | EFGAS |  |  | EFANR |  |  | EFSNE |  |
|  |  |  |  |  |  |  |  | '4FXX' |  |  | '4FXX' |  |  | '4FXX' |  |  | '4FXX' |  |  | '4FXX' |  |
|  |  |  |  |  |  |  |  | EFEMAIL |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  |  |  |  | '4FXX' |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  | DFMULTIMEDIA |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  | '5F3B' |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  |  |  |  | EFMML |  |  | EFMMDF |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  |  |  |  | '4F47' |  |  | '4F48' |  |  |  |  |  |  |  |  |  |  |

NOTE 1:	Files under DFTELECOM with shaded background are defined in TS 51.011 [18].

NOTE 2:	The value '6F65' under ADFUSIM was used in earlier versions of this specification, and should not be re-assigned in future versions.

Figure 4.1: File identifiers and directory structures of UICC

### 5.3.Y	SM-over-IP

Requirement:	Service n°12 and n°xx "available".

Request:	the ME performs the reading procedure with EFPSISMSC.

Update:	The ME performs the updating procedure with EFPSISMSC.

Annex A (informative):
EF changes via Data Download or USAT applications

This annex defines if changing the content of an EF by the network (e.g. by sending an SMS), or by a USAT Application, is advisable. Updating of certain EFs "over the air" such as EFACC could result in unpredictable behaviour of the UE; these are marked "Caution" in the table below. Certain EFs are marked "No"; under no circumstances should "over the air" changes of these EFs be considered.

| File identification | Description | Change advised |
| --- | --- | --- |
| '2F00' | Application directory | Caution |
| '2F05' | Preferred languages | Yes |
| '2F06' | Access rule reference | Caution |
| '2FE2' | ICC identification | No |
| '4F20' | Image data | Yes |
| ‘4F20’ | GSM Ciphering key Kc | No |
| '4FXX' | Image Instance data Files | Yes |
| '4FXX' | Unique identifier | Yes |
| '4F22' | Phone book synchronisation counter | Yes |
| '4F23' | Change counter | Yes |
| '4F24' | Previous unique identifier | Yes |
| '4F30' | Phone book reference file | Yes |
| '4FXX' | Capability configuration parameters 1 | Yes |
| '4F30' | SoLSA Access Indicator | Caution |
| '4F31' | SoLSA LSA List | Caution |
| '4FXX' | LSA Descriptor files | Caution |
| ‘4F52’ | GPRS Ciphring key KcGPRS | No |
| '4F63' | CPBCCH Information | No |
| '4F64’ | Investigation Scan | Caution |
| '4FXX' | Additional number alpha string | Yes |
| '4FXX' | Additional number | Yes |
| '4FXX' | Second name entry | Yes |
| '4FXX' | Grouping information alpha string | Yes |
| '4FXX' | Phone book control | Yes |
| '4FXX' | E-mail addresses | Yes |
| '4FXX' | Index administration phone book | Yes |
| '4FXX' | Extension 1 | Yes |
| '4FXX' | Abbreviated dialling numbers | Yes |
| '4FXX' | Grouping file | Yes |
| '4F41' | Pseudonym | Caution |
| '4F42' | User controlled PLMN selector for I-WLAN | No |
| '4F43' | Operator controlled PLMN selector for I-WLAN | Caution |
| '4F44' | User controlled WSID List | No |
| '4F45' | Operator controlled WSID List | Caution |
| '4F46' | WLAN Reauthentication Identity | No |
| 4F47' | Home I-WLAN Specific Identifier List | Yes |
| '4F47' | Multimedia Messages List | Yes |
| '4F48' | I-WLAN Equivalent HPLMN Presentation Indication | Yes |
| '4F48' | Multimedia Messages Data File | Yes |
| '4F49' | I-WLAN HPLMN Priority Indication | Yes |
| '4F4A' | I-WLAN Last Registered PLMN | Caution |
| '4F4B' | HPLMN Direct Access Indicator | Yes |
| '4F81' | Allowed CSG lists | Yes |
| '4F82' | CSG Type | Yes |
| '4F83' | HNB name | Yes |
| '6F05' | Language indication | Yes |
| ‘6F06’ | Access rule reference (under ADFUSIM and DFTELECOM) | Caution |
| '6F07' | IMSI | Caution (Note 1) |
| '6F08' | Ciphering and integrity keys | No |
| '6F09' | Ciphering and integrity keys for packet switched domain | No |
| '6F2C' | De-personalization Control Keys | Caution |
| '6F31' | Higher Priority PLMN search period | Caution |
| '6F32' | Co-operative network list | Caution |
| '6F37' | ACM maximum value | Yes |
| '6F38' | USIM service table | Caution |
| '6F39' | Accumulated call meter | Yes |
| '6F3B' | Fixed dialling numbers | Yes (Note 2) |
| '6F3C' | Short messages | Yes |
| '6F3E' | Group identifier level 1 | Yes |
| '6F3F' | Group identifier level 2 | Yes |
| '6F40' | MSISDN storage | Yes |
| '6F41' | PUCT | Yes |
| '6F42' | SMS parameters | Yes |
| '6F43' | SMS status | Yes |
| '6F45' | CBMI | Caution |
| '6F46' | Service provider name | Yes |
| '6F47' | Short message status reports | Yes |
| '6F48' | CBMID | Yes |
| '6F49' | Service Dialling Numbers | Yes (Note 2) |
| '6F4B' | Extension 2 | Yes |
| '6F4C' | Extension 3 | Yes |
| '6F4D' | Barred dialling numbers | Yes |
| '6F4E' | Extension 5 | Yes |
| '6F4F' | Capability configuration parameters 2 | Yes |
| '6F50' | CBMIR | Yes |
| '6F54' | SetUp Menu Elements | Yes |
| ‘6F55’ | Extension 4 | Yes |
| '6F56' | Enabled services table | Caution |
| '6F57' | Access point name control list | Yes |
| '6F58' | Comparison method information | Yes |
| '6F5B' | Initialisation value for Hyperframe number | Caution |
| '6F5C' | Maximum value of START | Yes |
| '6F60' | User controlled PLMN selector with Access Technology | No |
| '6F61' | Operator controlled PLMN selector with Access Technology | Caution |
| '6F62' | HPLMN selector with Access Technology | Caution |
| '6F73' | Packet switched location information | Caution (Note 1) |
| '6F78' | Access control class | Caution |
| '6F7B' | Forbidden PLMNs | Caution |
| '6F7E' | Location information | No (Note 1) |
| '6F80' | Incoming call information | Yes |
| '6F81' | Outgoing call information | Yes |
| '6F82' | Incoming call timer | Yes |
| '6F83' | Outgoing call timer | Yes |
| '6FAD' | Administrative data | Caution |
| '6FB1' | Voice Group Call Service | Yes |
| '6FB2' | Voice Group Call Service Status | Yes |
| '6FB3' | Voice Broadcast Service | Yes |
| '6FB4' | Voice Broadcast Service Status | Yes |
| '6FB5' | Enhanced Multi Level Pre-emption and Priority | Yes |
| '6FB6' | Automatic Answer for eMLPP Service | Yes |
| '6FB7' | Emergency Call Codes | Caution |
| '6FC3' | Key for hidden phone book entries | No |
| '6FC4' | Network Parameters | No |
| '6FC5' | PLMN Network Name | Yes |
| '6FC6' | Operator Network List | Yes |
| '6FC7' | Mailbox Dialling Numbers | Yes |
| '6FC8' | Extension 6 | Yes |
| '6FC9' | Mailbox Identifier | Caution |
| '6FCA' | Message Waiting Indication Status | Caution |
| '6FCB' | Call Forwarding Indication Status | Caution |
| '6FCC' | Extension 7 | Yes |
| '6FCD' | Service Provider Display Information | Yes |
| '6FCE' | MMS Notification | Yes |
| '6FCF' | Extension 8 | Yes |
| '6FD0' | MMS Issuer Connectivity Parameters | Yes |
| '6FD1' | MMS User Preferences | Yes |
| '6FD2' | MMS User Connectivity Parameters | Yes |
| '6FD3' | Network's indication of alerting (NIA) | Caution |
| '6FD4' | Voice Group Call Service Ciphering Algorithm | Yes |
| '6FD5' | Voice Broadcast Service Ciphering Algorithm | Yes |
| '6FD6' | GBA Bootstrapping parameters | Caution |
| '6FD7' | MBMS Service Keys List | Caution |
| '6FD8' | MBMS User Key | Caution |
| '6FD9' | EHPLMN | Caution |
| '6FDA' | GBA NAF List | Caution |
| '6FDB' | EHPLMN Presentation Indication | Caution |
| '6FDC' | Last RPLMN Selection Indication | Caution |
| '6FDD' | NAF Key Centre Address | Caution |
| '6FDE' | Service Provider Name Icon | Yes |
| '6FDF' | PLMN Network Name Icon | Yes |
| '6FE0' | In Case of Emergency – Dialling Number | Yes |
| '6FE1' | In Case of Emergency – Free Format | Yes |
| '6FE2' | Network Connectivity Parameters for UICC IP connections | Yes |
| '6FE3' | EPS location information | Caution (Note 1) |
| '6FE4' | EPS NAS Security Context | Caution |
| '6FYY' | Public Service Identity of the SM-SC | Yes |
| NOTE1: If EFIMSI is changed, the UICC should issue REFRESH as defined in TS 31.111 and update EFLOCI, EFPSLOCI and EFEPSLOCI accordingly. NOTE2: This file may contain eCALL related test and reconfiguration numbers. |  |  |

Annex E (informative):
Suggested contents of the EFs at pre-personalization

If EFs have an unassigned value, it may not be clear from the main text what this value should be. This annex suggests values in these cases.

| File Identification | Description | Value |
| --- | --- | --- |
| '2F00' | Application directory | Card issuer/operator dependant |
| '2F05' | Preferred languages | 'FF…FF' |
| '2F06' | Access rule reference | Card issuer/operator dependant |
| '2FE2' | ICC identification | operator dependant |
| '4F20' | Image data | '00FF...FF' |
| ‘4F20’ | GSM Ciphering key Kc | 'FF...FF07' |
| '4FXX' | Image instance data files | 'FF…FF' |
| '4FXX' | Unique identifier | '0000' |
| '4F22' | Phone book synchronisation counter | '00000000' |
| '4F23' | Change counter | '0000' |
| '4F24' | Previous unique identifier | '0000' |
| '4F30' | Phone book reference file | Operator dependant |
| '4F30' | SoLSA Access Indicator | '00FF...FF' |
| '4F31' | SoLSA LSA List | 'FF...FF' |
| '4FXX' | LSA Descriptor files | 'FF…FF' |
| '4FXX' | Capability configuration parameters 1 | 'FF…FF' |
| ‘4F52’ | GPRS Ciphring key KcGPRS | 'FF...FF07' |
| '4F63' | CPBCCH Information | 'FF..FF' |
| '4F64' | Investigation PLMN scan | '00' |
| '4FXX' | E-mail addresses | 'FF…FF' |
| '4FXX' | Additional number alpha string | 'FF…FF' |
| '4FXX' | Second name entry | 'FF…FF' |
| '4FXX' | Abbreviated dialling numbers | 'FF...FF' |
| '4FXX' | Grouping file | '00…00' |
| '4FXX' | Grouping information alpha string | 'FF…FF' |
| '4FXX' | Phone book control | '0000' |
| '4FXX' | Index administration phone book | 'FF…FF' |
| '4FXX' | Additional number | 'FF…FF' |
| '4FXX' | Extension 1 | '00FF...FF' |
| '4F41' | Pseudonym | '00FF…FF' |
| '4F42' | User Controlled PLMN selector for I-WLAN | 'FF…FF' |
| '4F43' | Operator Controlled PLMN selector for I-WLAN | Operator dependant |
| '4F44' | User Controlled WSID list | '00FF…FF' |
| '4F45' | Operator controlled WSID list | Operator dependant |
| '4F46' | WLAN Reauthentication Identity | 'FF…FF' |
| '4F47' | Home I-WLAN Specific Identifier List | Operator dependent |
| '4F47' | Multimedia Messages List | 'FF…FF' |
| '4F48' | I-WLAN Equivalent HPLMN Presentation Indication | Operator dependent |
| '4F48' | Multimedia Messages Data File | 'FF…FF' |
| '4F49' | I-WLAN HPLMN Indication | Operator dependent |
| '4F4A' | I-WLAN Last Registered PLMN | 'FFFFFF' |
| '4F4B' | HPLMN Direct Access Indicator | Operator dependent |
| '4F81' | Allowed CSG lists | Operator dependent |
| '4F82' | CSG Type | Operator dependent |
| '4F83' | HNB name | Operator dependent |
| '6F05' | Language indication | 'FF…FF' |
| '6F06' | Access rule reference (under ADFUSIM and DFTELECOM) | Card issuer/operator dependant |
| '6F07' | IMSI | Operator dependant |
| '6F08' | Ciphering and integrity keys | '07FF…FF' |
| '6F09' | Ciphering and integrity keys for packet switched domain | '07FF…FF' |
| '6F2C' | De-personalization control keys | 'FF…FF' |
| '6F31' | Higher Priority PLMN search period | 'FF' |
| '6F32' | Co-operative network list | 'FF…FF' |
| '6F37' | ACM maximum value | '000000' (see note 1) |
| '6F38' | USIM service table | Operator dependant |
| '6F39' | Accumulated call meter | '000000' |
| '6F3B' | Fixed dialling numbers | 'FF...FF' |
| '6F3C' | Short messages | '00FF...FF' |
| '6F3E' | Group identifier level 1 | Operator dependant |
| '6F3F' | Group identifier level 2 | Operator dependant |
| '6F40' | MSISDN storage | 'FF...FF' |
| '6F41' | PUCT | 'FFFFFF0000' |
| '6F42' | SMS parameters | 'FF...FF' |
| '6F43' | SMS status | 'FF...FF' |
| '6F45' | CBMI | 'FF...FF' |
| '6F46' | Service provider name | Operator dependant |
| '6F47' | Short message status reports | '00FF…FF' |
| '6F48' | CBMID | 'FF...FF' |
| '6F49' | Service Dialling Numbers | 'FF...FF' |
| '6F4B' | Extension 2 | '00FF...FF' |
| '6F4C' | Extension 3 | '00FF...FF' |
| '6F4D' | Barred Dialling Numbers | 'FF...FF' |
| '6F4E' | Extension 5 | '00FF...FF' |
| '6F4F' | Capability configuration parameters 2 | 'FF...FF' |
| '6F50' | CBMIR | 'FF...FF' |
| '6F54' | SetUp Menu Elements | Operator dependant |
| '6F55' | Extension 4 | '00FF...FF' |
| '6F56' | Enabled services table | Operator dependant |
| '6F57' | Access point name control list | '00FF…FF' |
| '6F58' | Comparison method information | 'FF…FF' |
| '6F5B' | Initialisation value for Hyperframe number | 'F0 00 00 F0 00 00' |
| '6F5C' | Maximum value of START | Operator dependant |
| '6F60' | User controlled PLMN selector with Access Technology | 'FFFFFF0000..FFFFFF0000' |
| '6F61' | Operator controlled PLMN selector with Access Technology | 'FFFFFF0000..FFFFFF0000' |
| '6F62' | HPLMN selector with Access Technology | 'FFFFFF0000..FFFFFF0000' |
| '6F73' | Packet switched location information | 'FFFFFFFF FFFFFF xxxxxx 0000 FF 01' (see note 2) |
| '6F78' | Access control class | Operator dependant |
| '6F7B' | Forbidden PLMNs | 'FF...FF' |
| '6F7E | Location information | 'FFFFFFFF xxxxxx 0000 FF 01' (see note 2) |
| '6F80' | Incoming call information | 'FF…FF 000000 00 01FFFF' |
| '6F81' | Outgoing call information | 'FF…FF 000000 01FFFF' |
| '6F82' | Incoming call timer | '000000' |
| '6F83' | Outgoing call timer | '000000' |
| '6FAD' | Administrative data | Operator dependant |
| '6FB1' | Voice Group Call Service | Operator dependant |
| '6FB2' | Voice Group Call Service Status | Operator dependant |
| '6FB3' | Voice Broadcast Service | Operator dependant |
| '6FB4' | Voice Broadcast Service Status | Operator dependant |
| '6FB5' | EMLPP | Operator dependant |
| '6FB6' | AaeM | '00' |
| '6FB7' | Emergency call codes | Operator dependant |
| '6FC3' | Key for hidden phone book entries | 'FF…FF' |
| '6FC4' | Network Parameters | 'FF…FF' |
| '6FC5' | PLMN Network Name | Operator dependant |
| '6FC6' | Operator Network List | Operator dependant |
| '6FC7' | Mailbox Dialling Numbers | Operator dependant |
| '6FC8' | Extension 6 | '00 FF...FF' |
| '6FC9' | Mailbox Identifier | Operator dependant |
| '6FCA' | Message Waiting Indication Status | '00 00 00 00 00' |
| '6FCB' | Call Forwarding Indication Status | 'xx 00 FF...FF' |
| '6FCC' | Extension 7 | '00 FF...FF' |
| '6FCD' | Service Provider Display Information |  |
| '6FCE' | MMS Notification | '00 00 00 FF…FF' |
| '6FCF' | Extension 8 | '00FF...FF' |
| '6FD0' | MMS Issuer Connectivity Parameters | 'FF…FF' |
| '6FD1' | MMS User Preferences | 'FF…FF' |
| '6FD2' | MMS User Connectivity Parameters | 'FF…FF' |
| '6FD3' | Network's Indication of Alerting (NIA) | 'FF...FF' |
| '6FD4' | Voice Group Call Service Ciphering Algorithm | '00…00' |
| '6FD5' | Voice Broadcast Service Ciphering Algorithm | '00…00' |
| '6FD6' | GBA Bootstrapping parameters | 'FF…FF' |
| '6FD7' | MBMS Service Keys List | 'FF…FF' |
| '6FD8' | MBMS User Key | 'FF…FF' |
| '6FD9' | EHPLMN | 'FF…FF' or xxxxxx (see Note 2) |
| '6FDA' | GBA NAF List | 'FF…FF' |
| '6FDB' | EHPLMN Presentation Indication | '00' |
| '6FDC' | Last RPLMN Selection Indication | '00' |
| '6FDD' | NAF Key Centre Address | 'FF…FF' |
| '6FDE' | Service Provider Name Icon | '00 FF…FF' |
| '6FDF' | PLMN Network Name Icon | '00 FF…FF' |
| '6FE0' | In Case of Emergency – Dialling Number | Operator dependant |
| '6FE1' | In Case of Emergency – Free Format | Operator dependant |
| '6FE2' | Network Connectivity Parameters for UICC IP connections | Operator dependant |
| '6FE3' | EPS location information | 'FFFFFFFFFFFFFFFFFFFFFFFF xxxxxx0000 01' (see note 2) |
| '6FE4' | EPS NAS Security Context | 'FF…FF' |
| '6FYY' | Public Service Identity of the SM-SC | Operator dependant |

NOTE 1:	The value '000000' means that ACMmax is not valid, i.e. there is no restriction on the ACM. When assigning a value to ACMmax, care should be taken not to use values too close to the maximum possible value 'FFFFFF', because the INCREASE command does not update EFACM if the units to be added would exceed 'FFFFFF'. This could affect the call termination procedure of the Advice of Charge function.

NOTE 2:	xxxxxx stands for any valid MCC and MNC, coded according to TS 24.008 [9].