3GPP TSG-CT6#56	C6-100401

- Venice, Italy, 24-27 August 2010

| CR-Form-v9.7 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 31.103 | CR | 0066 |  rev | 2 |  Current version: | 8.2.0 |  |
| For HELP on using this form look at the pop-up text over the  symbols. Comprehensive instructions on how to use this form can be found at http://www.3gpp.org/specs/CR.htm. |  |  |  |  |  |  |  |  |

| Proposed change affects:  | UICC apps | X | ME | X | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title:  | Introduction of a DF_TELECOM EF to support Public Service Identity for SMS over IP. |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG:  | Research In Motion, TeliaSonera, Verizon Wireless |  |  |  |  |  |  |  |  |  |
| Source to TSG:  | C6 |  |  |  |  |  |  |  |  |  |
| Work item code:  | TEI8 |  |  |  |  |  |  | Date:  |  | 26/08/2010 |
| Category:  | F |  |  |  |  |  |  | Release:  |  | Rel-8 |
|  | Use one of the following categories: F (correction) A (corresponds to a correction in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: R99 (Release 1999) Rel-4 (Release 4) Rel-5 (Release 5) Rel-6 (Release 6) Rel-7 (Release 7) Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) |  |
| Reason for change:  |  | Currently SMS over IP stage 3 TS 24.341 states: #### 5.3.1.2 Submitting a short message When an SM-over-IP sender wants to submit an SM over IP, the SM-over-IP sender shall send a SIP MESSAGE request with the following information: a) the Request-URI, which shall contain the PSI of the SC of the SM-over-IP sender; NOTE 1: The PSI of the SC can be SIP URI or tel URI based on operator policy. Also in subclause 5.3.1.2 NOTE 4: The address of the SC is included in the RP-DATA message content. The address of the SC is configured in the SM-over-IP sender. However currently there is no means to configure the UE with either the SIP URI or tel URI for the Public Service Identity of the SMS service center (SM-SC) based on operator policy as specified in NOTE 1 of subclause 5.3.1.2 of TS 24.341.This is needed is order to enable the UE to submit an SMS’ over IP network as defined in TS 24.341. |  |  |  |  |  |  |  |  |
| Summary of change:  |  | Added a new reference entry. Added a new entries to the service table. Added a new TELECOMs level section. Added the new the EF PSISMSC mapping to DF TELECOM. Added the legacy SMS EFs to the ISIM application in order to support SMS over IP on ISIM only UICCs. Corrected some minor typos in Annex A. |  |  |  |  |  |  |  |  |
| Consequences if  not approved: |  | There wil continue to be no mechanism for UEs to use the appropriate SM-SC Public Service Identity for SMS’ over IP and for the operator to configure the correct SM-SC Public Service Identity. |  |  |  |  |  |  |  |  |
| Clauses affected:  |  | 2, 4.2.7, 4.2.A, 4.2.B, 4.2.C, 4.2.D, 4.3, 4.X, 4.X.E, Annex A |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs  |  | X |  | Other core specifications  |  |  | 31.102 |  |  |  |
| affected: |  |  |  | Test specifications |  |  |  |  |  |  |
|  |  |  |  | O&M Specifications |  |  |  |  |  |  |
| Other comments:  |  | This CR is conditional on the approval of C6-100400. The file indentifier 6FYY shall get the same value as in document C6-100400. |  |  |  |  |  |  |  |  |

# 2	References

The following documents contain provisions that, through reference in this text, constitute provisions of the present document.

- References are either specific (identified by date of publication and/or edition number or version number) or non-specific.

- For a specific reference, subsequent revisions do not apply.

- For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TS 21.111: "USIM and IC Card Requirements".

[2]	3GPP TS 31.102: "Characteristics of the USIM Application".

[3]	3GPP TS 31.101: "UICC-Terminal Interface, Physical and Logical Characteristics".

[4]	3GPP TS 33.102: "3G Security; Security Architecture".

[5]	3GPP TS 33.103: "3G Security; Integration Guidelines".

[6]	ISO/IEC 7816-4: "Identification cards – Integrated circuit cards,Part 4: Organization, security and  commands for interchange".

[7]	Void.

[8]	Void.

[9]	3GPP TS 23.003: "Numbering, Addressing and Identification".

[10]	Void.

[11]	Void.

[12]	3GPP TS 25.101: "UE Radio Transmission and Reception (FDD)".

[13]	3GPP TS 23.228: "IP Multimedia Subsystem (IMS); Stage 2".

[14]	3GPP TS 33.203: "3G security; Access security for IP-based services".

[15]	3GPP TS 24.228: "Signalling flows for the IP multimedia call control based on SIP and SDP; Stage 3".

[16]	IETF RFC 3261: "SIP: Session Initiation Protocol".

[17]	3GPP TS 23.038: "Alphabets and language-specific information".

[18]	ISO 639 (1988): "Code for the representation of names of languages".

[19]	3GPP TS 51.011 Release 4: "Specification of the Subscriber Identity Module – Mobile Equipment (SIM-ME) interface".

[20]	ISO/IEC 8825(1990): "Information technology – Open Systems Interconnection – Specification of Basic Encoding Rules for Abstract Syntax Notation One (ASN.1)" Second Edition.

[21]	3GPP TS 22.101: "Service aspects; Service principles".

[22]	Void.

[23]	ETSI TS 101 220: "Smart cards; ETSI numbering system for telecommunication application providers".

[24]	IETF RFC 2486: "The Network Access Identifier".

[25]	3GPP TS 33.220: "Generic Authentication Architecture (GAA); Generic bootstrapping architecture".

[26]	IETF RFC 2617: "HTTP Authentication: Basic and Digest Access Authentication". (http://www.ietf.org/rfc/rfc2617.txt)

[27]	IETF RFC 3629 (2003): "UTF-8, a transformation format of ISO 10646".

[28]	3GPP TS 33.110: "Key establishment between a Universal Integrated Circuit Card (UICC) and a terminal".

[XX]	3GPP TS 24.341: "Support of SMS over IP networks; Stage 3"

[XY]	3GPP TS 23.040: "Technical realization of the Short Message Service (SMS)".

[XZ]	3GPP TS 24.011: "Point-to-Point (PP) Short Message Service (SMS) support on mobile radio interface".

### 4.2.7	EFIST (ISIM Service Table)

This EF indicates which optional services are available. If a service is not indicated as available in the ISIM, the ME shall not select this service. The presence of this file is mandatory if optional services are provided in the ISIM.

| Identifier: '6F07' |  | Structure: transparent |  |  | Optional |  |
| --- | --- | --- | --- | --- | --- | --- |
| SFI: '07' |  |  |  |  |  |  |
| File size: X bytes, X >= 1 |  |  | Update activity: low |  |  |  |
| Access Conditions: READ PIN UPDATE ADM DEACTIVATE ADM ACTIVATE ADM |  |  |  |  |  |  |
| Bytes | Description |  |  | M/O |  | Length |
| 1 | Services n°1 to n°8 |  |  | M |  | 1 byte |
| 2 | Services n°9 to n°16 |  |  | O |  | 1 byte |
| 3 | Services n°17 to n°24 |  |  | O |  | 1 byte |
| 4 | Services n°25 to n°32 |  |  | O |  | 1 byte |
| etc. |  |  |  |  |  |  |
| X | Services n°(8X-7) to n°(8X) |  |  | O |  | 1 byte |

| -Services |  |  |
| --- | --- | --- |
| Contents: | Service n°1: | P-CSCF address |
|  | Service n°2 | Generic Bootstrapping Architecture (GBA) |
|  | Service n°3 | HTTP Digest |
|  | Service n°4 | GBA-based Local Key Establishment Mechanism |
|  | Service n°5 | Support of P-CSCF discovery for IMS Local Break Out |
|  | Service n°a | Short Message Storage (SMS) |
|  | Service n°b | Short Message Status Reports (SMSR) |
|  | Service n°d | Support for SM-over-IP |

### 4.2.A	EFSMS (Short messages)

If service n° a and service n° d is "available", this file shall be present.

This EF contains information in accordance with TS 23.040 [XY] comprising short messages (and associated parameters) which have either been received by the UE from the network, or are to be used as an UE originated message.

| Identifier: '6F3C' |  | Structure: linear fixed |  |  | Optional |  |
| --- | --- | --- | --- | --- | --- | --- |
| Record length: 176 bytes |  |  | Update activity: low |  |  |  |
| Access Conditions: READ PIN UPDATE PIN DEACTIVATE ADM ACTIVATE ADM |  |  |  |  |  |  |
| Bytes | Description |  |  | M/O |  | Length |
| 1 | Status |  |  | M |  | 1 byte |
| 2 to 176 | Remainder |  |  | M |  | 175 bytes |

-	Status.

Contents:

Status byte of the record which can be used as a pattern in the SEARCH RECORD command. For UE originating messages sent to the network, the status shall be updated when the UE receives a status report, or sends a successful SMS Command relating to the status report.

Coding:

|  |  | b8 |  | b7 |  | b6 |  | b5 |  | b4 |  | b3 |  | b2 |  | b1 |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  |  |  |  |  |  |  | X |  | X |  | 0 |  | free space |  |  |
|  |  |  |  |  |  |  |  |  |  |  |  | X |  | X |  | 1 |  | used space |  |  |
|  |  |  |  |  |  |  |  |  |  |  |  | 0 |  | 0 |  | 1 |  | message received by UE from network; message read |  |  |
|  |  |  |  |  |  |  |  |  |  |  |  | 0 |  | 1 |  | 1 |  | message received by UE from network; message to be read |  |  |
|  |  |  |  |  |  |  |  |  |  |  |  | 1 |  | 1 |  | 1 |  | UE originating message; message to be sent |  |  |
|  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | RFU (see TS 31.101 [3]) |  |  |

|  |  | b8 |  | b7 |  | b6 |  | b5 |  | b4 |  | b3 |  | b2 |  | b1 |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  |  |  | X |  | X |  | 1 |  | 0 |  | 1 |  | UE originating message; message sent to the network: |  |
|  |  |  |  |  |  |  |  | 0 |  | 0 |  | 1 |  | 0 |  | 1 |  | Status report not requested |  |
|  |  |  |  |  |  |  |  | 0 |  | 1 |  | 1 |  | 0 |  | 1 |  | Status report requested but not (yet) received; |  |
|  |  |  |  |  |  |  |  | 1 |  | 0 |  | 1 |  | 0 |  | 1 |  | Status report requested, received but not stored in EF-SMSR; |  |
|  |  |  |  |  |  |  |  | 1 |  | 1 |  | 1 |  | 0 |  | 1 |  | Status report requested, received and stored in EF-SMSR; |  |
|  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | RFU (see TS 31.101 [3]) |  |

-	Remainder.

Contents:

This data item commences with the TS-Service-Centre-Address as specified in TS 24.011 [XZ]. The bytes immediately following the TS-Service-Centre-Address contain an appropriate short message TPDU as specified in TS 23.040 [XY], with identical coding and ordering of parameters.

Coding:

according to TS 23.040 [XY] and TS 24.011 [XZ]. Any TP-message reference contained in an UE originated message stored in the USIM, shall have a value as follows:

Value of the TP-message-reference:

message to be sent:						'FF'.

message sent to the network:			the value of TP-Message-Reference used in the
										message sent to the network.

Any bytes in the record following the TPDU shall be filled with 'FF'.

It is possible for a TS-Service-Centre-Address of maximum permitted length, e.g. containing more than 18 address digits, to be associated with a maximum length TPDU such that their combined length is 176 bytes. In this case the ME shall store in the USIM the TS-Service-Centre-Address and the TPDU in bytes 2 to 176 without modification, except for the last byte of the TPDU, which shall not be stored.

### 4.2.B	EFSMSS (SMS status)

If service n° a and service n° d is "available", this file shall be present.

This EF contains status information relating to the short message service.

| Identifier: '6F43' |  | Structure: transparent |  |  | Optional |  |
| --- | --- | --- | --- | --- | --- | --- |
| File size: 2+X bytes |  |  | Update activity: low |  |  |  |
| Access Conditions: READ PIN UPDATE PIN DEACTIVATE ADM ACTIVATE ADM |  |  |  |  |  |  |
| Bytes | Description |  |  | M/O |  | Length |
| 1 | Last Used TP-MR |  |  | M |  | 1 byte |
| 2 | SMS "Memory Cap. Exceeded" Not. Flag |  |  | M |  | 1 byte |
| 3 to 2+X | RFU |  |  | O |  | X bytes |

-	Last Used TP-MR.

Contents:

-	the value of the TP-Message-Reference parameter in the last mobile originated short message, as defined in TS 23.040 [XY].

Coding:

-	as defined in TS 23.040 [XY].

-	SMS "Memory Capacity Exceeded" Notification Flag.

Contents:

-	this flag is required to allow a process of flow control, so that as memory capacity in the UE becomes available, the Network can be informed. The process for this is described in TS 23.040 [XY].

Coding:

b1=1 means flag unset; memory capacity available;

b1=0 means flag set;

b2 to b8 are reserved and set to 1.

### 4.2.C	EFSMSR (Short message status reports)

If service n° b  and service n° d is "available", this file shall be present.

This EF contains information in accordance with TS 23.040 [XY] comprising short message status reports which have been received by the UE from the network.

Each record is used to store the status report of a short message in a record of EFSMS. The first byte of each record is the link between the status report and the corresponding short message in EFSMS.

| Identifier: '6F47' |  | Structure: linear fixed |  |  | Optional |  |
| --- | --- | --- | --- | --- | --- | --- |
| Record length: 30 bytes |  |  | Update activity: low |  |  |  |
| Access Conditions: READ PIN UPDATE PIN DEACTIVATE ADM ACTIVATE ADM |  |  |  |  |  |  |
| Bytes | Description |  |  | M/O |  | Length |
| 1 | SMS record identifier |  |  | M |  | 1 |
| 2 to 30 | SMS status report |  |  | M |  | 29 bytes |

-	SMS record identifier.

Contents:

-	this data item identifies the corresponding SMS record in EFSMS, e.g. if this byte is coded '05' then this status report corresponds to the short message in record #5 of EFSMS.

Coding:

-	'00'			- empty record;

-	'01' to 'FF'	- record number of the corresponding SMS in EFSMS.

-	SMS status report:

Contents:

-	this data item contains the SMS-STATUS-REPORT TPDU as specified in TS 23.040 [XY], with identical coding and ordering of parameters.

Coding:

-	according to TS 23.040 [XY]. Any bytes in the record following the TPDU shall be filled with 'FF'.

4.2.D		EFSMSP (Short message service parameters)

If service n° d is "available", this file shall be present.

This EF contains values for Short Message Service header Parameters (SMSP), which can be used by the ME for user assistance in preparation of mobile originated short messages. For example, a service centre address will often be common to many short messages sent by the subscriber.

The EF consists of one or more records, with each record able to hold a set of SMS parameters. The first (or only) record in the EF shall be used as a default set of parameters, if no other record is selected.

To distinguish between records, an alpha-identifier may be included within each record, coded on Y bytes.

The SMS parameters stored within a record may be present or absent independently. When a short message is to be sent from the UE, the parameter in the USIM record, if present, shall be used when a value is not supplied by the user.

| Identifier: '6F42' |  | Structure: linear fixed |  |  | Optional |  |
| --- | --- | --- | --- | --- | --- | --- |
| Record length: 28+Y bytes |  |  | Update activity: low |  |  |  |
| Access Conditions: READ PIN UPDATE PIN DEACTIVATE ADM ACTIVATE ADM |  |  |  |  |  |  |
| Bytes | Description |  |  | M/O |  | Length |
| 1 to Y | Alpha-Identifier |  |  | O |  | Y bytes |
| Y+1 | Parameter Indicators |  |  | M |  | 1 byte |
| Y+2 to Y+13 | TP-Destination Address |  |  | M |  | 12 bytes |
| Y+14 to Y+25 | TS-Service Centre Address |  |  | M |  | 12 bytes |
| Y+26 | TP-Protocol Identifier |  |  | M |  | 1 byte |
| Y+27 | TP-Data Coding Scheme |  |  | M |  | 1 byte |
| Y+28 | TP-Validity Period |  |  | M |  | 1 byte |

Storage is allocated for all of the possible SMS parameters, regardless of whether they are present or absent. Any bytes unused, due to parameters not requiring all of the bytes, or due to absent parameters, shall be set to 'FF'.

-	Alpha-Identifier.

Contents:

Alpha Tag of the associated SMS-parameter.

Coding:

see clause 4.4.2.3 (EFADN).

NOTE:	The value of Y may be zero, i.e. the alpha-identifier facility is not used. By using the command GET RESPONSE the ME can determine the value of Y.

-	Parameter Indicators.

Contents:

each of the default SMS parameters which can be stored in the remainder of the record are marked absent or present by individual bits within this byte.

Coding:

allocation of bits:

bit number	Parameter indicated.

1			TP-Destination Address.

2			TS-Service Centre Address.

3			TP-Protocol Identifier.

4			TP-Data Coding Scheme.

5			TP-Validity Period.

6			reserved, set to 1.

7			reserved, set to 1.

8			reserved, set to 1.

Bit value		Meaning.

0			Parameter present.

1			Parameter absent.

-	TP-Destination Address.

Contents and Coding:
as defined for SM-TL address fields in TS 23.040 [XY].

-	TP-Service Centre Address.

Contents and Coding:
as defined for RP-Destination address Centre Address in TS 24.011 [XZ].

-	TP-Protocol Identifier.

Contents and Coding:
as defined in TS 23.040 [XY].

-	TP-Data Coding Scheme.

Contents and Coding:
as defined in TS 23.038 [17].

-	TP-Validity Period.

Contents and Coding:
as defined in TS 23.040 [XY] for the relative time format.

## 4.3	ISIM file structure

This subclause contains a figure depicting the file structure of the ADFISIM. ADFISIM shall be selected using the AID and information in EFDIR.

|  | ADFISIM |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | EFIST |  |  | EFIMPI |  |  | EFDOMAIN |  |  | EFIMPU |  |  |
|  |  |  |  | '6F07' |  |  | '6F02' |  |  | '6F03' |  |  | '6F04' |  |  |
|  |  |  |  | EFAD |  |  | EFARR |  |  | EFP-CSCF |  |  | EFGBAP |  |  |
|  |  |  |  | '6FAD' |  |  | '6F06' |  |  | '6F09' |  |  | '6FD5' |  |  |
|  |  |  |  | EFGBANL |  |  | EFNAFKCA |  |  |  |  |  |  |  |  |
|  |  |  |  | '6FD7' |  |  | '6FDD' |  |  |  |  |  |  |  |  |
|  |  |  |  | EFSMS |  |  | EFSMSS |  |  | EFSMSR |  |  | EFSMSP |  |  |
|  |  |  |  | '6F3C' |  |  | '6F43' |  |  | '6F47' |  |  | '6F42' |  |  |

- Figure 1: File identifiers and directory structures of ISIM

## 4.X	Contents of EFs at the TELECOM level

The EFs in the Dedicated File DFTELECOM contain service related information as defined in 31.102 [2].

The structure of DF TELECOM is defined in 31.102 [2].

### 4.X.E	EFPSISMSC (Public Service Identity of the SM-SC)

If service n° d is "available", this file shall be present.

Coding and usage of this EF is defined in 31.102 [2].

This EF can be found in DF TELECOM with an identifier equal to '6FYY' as defined in 31.102 [2].

## 5.Y	Subscription related procedures

### 5.Y.Z	SM-over-IP

Requirement:	Service n°d "available".

Request:	the ME performs the reading procedure with EFPSISMSC.

Update:	The ME performs the updating procedure with EFPSISMSC.

Annex A (informative):
EF changes via Data Download or USAT applications

This annex defines if changing the content of an EF by the network (e.g. by sending an SMS) or by a USAT Application is advisable. Updating of certain EFs "over the air" could result in unpredictable behaviour of the UE; these are marked "Caution" in the table below. Certain EFs are marked "No"; under no circumstances should "over the air" changes of these EFs be considered.

| File identification | Description | Change advised |
| --- | --- | --- |
| '6F02' | IMS private user identity | Caution (note) |
| '6F03' | Home Network Domain Name | Caution (note) |
| '6F04' | IMS public user identity | Caution (note) |
| '6FAD' | Administrative Data | Caution |
| '6F06' | Access Rule Reference | Caution |
| ‘6F07’ | ISIM Service Table | Caution |
| ‘6F09’ | P-CSCF address | Caution (note) |
| '6FD5’ | GBA Bootstrapping parameters | Caution |
| '6FD7’ | GBA NAF List | Caution |
| '6FDD’ | NAF Key Centre Address | Caution |
| '6F3C' | Short messages | Yes |
| '6F42' | SMS parameters | Yes |
| '6F43' | SMS status | Yes |
| '6F47' | Short message status reports | Yes |
| '6FYY' | Public Service Identity of the SM-SC | Yes |
| NOTE: If EFIMPI, EFIMPU, EFDOMAIN or P-CSCF are changed, the UICC should issue a REFRESH command. |  |  |