3GPP TSG-CT WG6 Meeting #89Bis	C6-180404

- Sophia Antipolis (France), 10th July – 13th July 2018

| CR-Form-v11.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 31.102 | CR | 0797 | Rev | 3 | Current version: | 15.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps | X | ME | X | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Modify structure of SUCI Calc EF and introduce Routing Indicator |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Qualcomm Incorporated, G+D MS |  |  |  |  |  |  |  |  |  |
| Source to TSG: | C6 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5GS_Ph1-CT |  |  |  |  |  |  | Date: |  | 2018-07-12 |
| Category: | B |  |  |  |  |  |  | Release: |  | Rel-15 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | - Current structure of the EF doesn’t specify a way to indicate Protection Scheme Identifier priority across sub-lists (one sub-list is associated with one public key). - It has deep (three) nesting levels of TLVs, adding complexity to the ME implementation. - It does not include the recently introduced Routing Indicator portion of the SUCI, as specified in 3GPP TS 23.003 clause 2.2B. - Clarification of the open points with regard to the coding of some of the data objects. |  |  |  |  |  |  |  |  |
| Summary of change: |  | - Modified the structure of the EF to keyref based, that allows a simpler way of prioritizing all the Protection Scheme Identifiers, irrespective of the Public Key they are associated with. It also reduces the TLV nesting levels from three to two. - Added Routing Indicator to the EF. - Added editor’s note to highlight the missing parts still to be completed. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | - It is impossible to configure the EF in a way that allows prioritization of all the Protection Scheme Identifiers when the Identifiers belong to different sub-lists. - It is not possible to configure Routing Indicator in the USIM, which is needed by the ME to calculate the SUCI, as specified in 3GPP TS 23.003 clause 2.2B. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 4.2.117, 4.7, Annex A, Annex D, Annex E |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  | TS/TR ... CR ... |  |  |  |
| affected: |  |  | x | Test specifications |  |  | TS/TR ... CR ... |  |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  | TS/TR ... CR ... |  |  |  |
| Other comments: |  | This CR update the definition of the content of EFSUCI_Calc_Info and the EFSUCI_Calc_Info needs to be moved to the DF5GS which is defined in CR#0803. |  |  |  |  |  |  |  |  |

### 4.2.117	EFSUCI_Calc_Info (Subscription Concealed Identifier Calculation Information EF)

If "SUCI calculation is to be performed by the ME" (i.e. service n°124 is "available" and service n°125 is not "available"), this file shall be present.

If "SUCI calculation is to be performed by the USIM" (i.e. service n°124 is "available" and service n°125 is "available"), this file shall not be available to the ME.

If service n°124 is not "available", this file shall not be available to the ME. This EF contains information needed by the ME for the support of subscription identifier privacy as defined in 3GPP TS 33.501[105].

| Identifier: '6F04' |  | Structure: transparent |  |  | Optional |  |
| --- | --- | --- | --- | --- | --- | --- |
| File size: X bytes (X ≥ 2) |  |  | Update activity: low |  |  |  |
| Access Conditions: READ PIN UPDATE ADM DEACTIVATE ADM ACTIVATE ADM |  |  |  |  |  |  |
| Bytes | Description |  |  | M/O |  | Length |
| 1 to Z | Protection Scheme Identifier List data object |  |  | M |  | Z bytes |
| Z+1 to Y+Z | Home Network Public Key List data object |  |  | C |  | Y bytes |
| X-W+1 to X | Routing Information TLV data object |  |  | C |  | W bytes |

Editor’s Note: The presence condition of Routing Information TLV data object is FFS.

-	Protection Scheme Identifier List data object.

Contents:

This data object shall always be present. If Protection Scheme Identifier List data object length is not zero, this data object contains a list of the Protection Scheme Identifier and the corresponding Key Index. The first Protection Scheme Identifier entry has the highest priority and the last Protection Scheme Identifier entry has the lowest priority. The Key Index value indicates the position of the Home Network Public Key in the Home Network Public Key List, that is applicable to the Protection Scheme.

Coding:

| Description | Value | M/O/C | Length (bytes) |
| --- | --- | --- | --- |
| Protection Scheme Identifier List data object tag | 'A0' | M | 1 |
| Protection Scheme Identifier List data object length | L1 | M | Note 1 |
| Protection Scheme Identifier 1 (Highest priority) | -- | O | 1 |
| Key Index 1 | -- | C (Note 2) | 1 |
| Protection Scheme Identifier 2 | -- | O | 1 |
| Key Index 2 | -- | C (Note 2) | 1 |
| … | … | … | … |
| Protection Scheme Identifier N (Lowest priority) | -- | O | 1 |
| Key Index N | -- | C (Note 2) | 1 |
| Note 1: The length is coded according to ISO/IEC 8825-1 [35] Note 2: This field is present only if the corresponding Protection Scheme Identifier field is present |  |  |  |

The Protection Scheme Identifier represents a protection scheme as described in 3GPP TS 33.501 [105] and it is coded in one byte as follows:

|  |  | b8 |  | b7 |  | b6 |  | b5 |  | b4 |  | b3 |  | b2 |  | b1 |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | Protection Scheme identifier coded as described in 3GPP TS 33.501 [105] |
|  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | RFU, bit = 0 |

Editor’s Note: The format may need to be updated depending on the outcome of 3GPP TS 23.003.

The Key Index is coded in one byte such that its value indicates the position of the Home Network Public Key in the Home Network Public Key List data object, that is applicable to the Protection Scheme. A Key Index with a value of "1" refers to the first Network Public Key entry in the Home Network Public Key List, and so on. A Key Index with a value of "0" indicates that there is no Home Network Public Key associated with that Protection Scheme (e.g., in the case of null-scheme).

-	Home Network Public Key List data object.

Contents:

This data object contains a list of the Home Network Public Key and the corresponding Home Network Public Key Identifier that shall be used by the ME to calculate the SUCI.

This data object may not be present if none of the protection scheme profiles identified by the Protection Scheme Identifiers included in the Protection Scheme Identifier List data object use the Home Network Public Key (e.g. null-scheme). If this data object is present, it shall contain at least one Home Network Public Key and the corresponding Home Network Public Key Identifier.

Coding:

| Description | Value | M/O/C | Length (bytes) |
| --- | --- | --- | --- |
| Home Network Public Key List data object | 'A1' | C | 1 |
| Home Network Public Key List data object length | L1 | C | Note 1 |
| Home Network Public Key 1 Identifier tag | '80' | C | 1 |
| Home Network Public Key 1 Identifier length | L2 | C | Note 1 |
| Home Network Public Key 1 Identifier | -- | C | L2 |
| Home Network Public Key 1 tag | '81' | C | 1 |
| Home Network Public Key 1 length | L3 | C | Note 1 |
| Home Network Public Key 1 | -- | C | L3 |
| … | … | … | … |
| Home Network Public Key N Identifier tag | '80' | O | 1 |
| Home Network Public Key N Identifier length | L4 | O | Note 1 |
| Home Network Public Key N Identifier | -- | O | L4 |
| Home Network Public Key N tag | '81' | O | 1 |
| Home Network Public Key N length | L5 | O | Note 1 |
| Home Network Public Key N | -- | O | L5 |
| Note 1: The length is coded according to ISO/IEC 8825-1 [35] |  |  |  |

Editor’s Note: Coding of the Home Network Public Key Identifier and Home Network Public Key are FFS.

-	Routing Information TLV data object.

Contents:

This data object contains Routing Indicator that allows together with the MCC and MNC to route network signalling with SUCI to AUSF and UDM instances capable to serve the subscriber, as specified in 3GPP TS 23.003 [25]. This data object may not be present in the case of null-scheme. If this data object is present, it shall have a valid Routing Indicator.

Coding:

| Description | Value | M/O/C | Length (bytes) |
| --- | --- | --- | --- |
| Routing Information TLV data object tag | 'A2' | C | 1 |
| Routing Information TLV data object length | L1 | C | Note 1 |
| Routing Information TLV data object value | -- | C | L1 |
| Note 1: The length is coded according to ISO/IEC 8825-1 [35] |  |  |  |

Editor’s Note: Coding of the above data object TLV value is FFS.

***** Next change *****

Annex A (informative):
EF changes via Data Download or USAT applications

This annex defines if changing the content of an EF by the network (e.g. by sending an SMS), or by a USAT Application, is advisable. Updating of certain EFs "over the air" such as EFACC could result in unpredictable behaviour of the UE; these are marked "Caution" in the table below. Certain EFs are marked "No"; under no circumstances should "over the air" changes of these EFs be considered.

| File identification | Description | Change advised |
| --- | --- | --- |
| '2F00' | Application directory | Caution |
| '2F05' | Preferred languages | Yes |
| … |  |  |
| '6F01' | 5GS 3GPP Access NAS Security Context | Caution |
| '6F02' | 5GS non-3GPP Access NAS Security Context | Caution |
| '6F03' | Subscriber Concealed Identifier Calculation Information | Yes |
| '6F04' | UAC Access Identities Configuration | Caution |
| '6FFE' | Steering of UE in VPLMN | Yes |
| NOTE1: If EFIMSI is changed, the UICC should issue REFRESH as defined in 3GPP TS 31.111 [12] and update EFLOCI, EFPSLOCI, EFEPSLOCI and EF5GSLOCI accordingly. NOTE2: This file may contain eCALL related test and reconfiguration numbers or URIs. NOTE3: If EFUICCIARI is changed, the UICC shall issue a REFRESH command as defined in TS 31.111. The ME shall read the updated list of IARIs associated with active applications installed on the UICC. NOTE4: Updating EFProSe_UIRC Over-The-Air, especially adding more parameters to the report, may cause a reduction of number of reports to be able to be stored in the UE. |  |  |

***** Next change *****

Annex D (informative):
Tags defined in 31.102

| Tag | Name of Data Element | Usage |
| --- | --- | --- |
| '43' | Full name for network IEI | PLMN Network Name (EFPNN) |
| … |  |  |
| '80' | Home Network Public Key Identifier tag | Home Network Public Key Identifier (EFSUCI_Calc_Info) |
| … |  |  |
| '81' | Home Network Public Key tag | Home Network Public Key (EFSUCI_Calc_Info) |
| … |  |  |
| 'A0' | 5GS 3GPP access NAS security Context tag or 5GS non-3GPP access NAS security Context tag The following tags are encapsulated within 'A0' '80' ngKSI Tag '81' KAMF Tag '82' Uplink NAS count Tag '83' Downlink NAS count Tag '84' Identifiers of selected NAS integrity and encryption algorithms Tag | 5GS 3GPP Access NAS Security Context (EF5GS3GPPSNSC) or 5GS non-3GPP Access NAS Security Context (EF5GSN3GPPSNSC) |
| 'A0' | Protection Scheme Identifier List data object tag | Protection Scheme Identifier List (EFSUCI_Calc_Info) |
| 'A1' | XCAP connection parameters policy part tag The following tags are encapsulated within 'A0' '81' Access Tag '82' Application name Tag '83' Provider ID Tag '84' URI Tag '85' XCAP Aithentication User Name Tag '86' XCAP Authentication password Tag '87'…XCAP Authentication type Tag '88'…Address type Tag '89'…Address Tag '8A'…PDP Authentication type Tag '8B'…PDP Authentication Name Tag '8C'…PDP Authentication Secret Tag | EFXCAPConfigData |
| … |  |  |
| 'A1' | EARFCN List Tag | TV Configuration (EFTVCONFIG) |
| 'A1' | SUCI TLV data object tag | Response to GET IDENTITY |
| 'A1' | Home Network Public Key List data object The following tags are encapsulated under 'A1' '80' Home Network Public Key Identifier tag '81' Home Network Public Key tag | Home Network Public Key List (EFSUCI_Calc_Info) |
| 'A2' | TDD frequency information The following tags are encapsulated within 'A2': '80' TDD Intra Frequency Information data object '81' TDD Inter Frequency Information data object | Network Parameters (EFNETPAR) |
| 'A2' | Routing Information TLV data object tag | Routing Information (EFSUCI_Calc_Info) |
| 'A3' | Service provider display information The following tags are encapsulated within 'A3': '80' Service provider PLMN list | Service Provider Display Information (EFSPDI) |
| … |  |  |
| 'DE' | GBA Security Context NAF Derivation Mode tag | Response to AUTHENTICATE |

NOTE:	the value 'FF' is an invalid tag value. For ASN.1 tag assignment rules see ISO/IEC 8825-1 [35]

***** Next change *****

Annex E (informative):
Suggested contents of the EFs at pre-personalization

If EFs have an unassigned value, it may not be clear from the main text what this value should be. This annex suggests values in these cases.

| File Identification | Description | Value |
| --- | --- | --- |
| '2F00' | Application directory | Card issuer/operator dependent |
| '2F05' | Preferred languages | 'FF…FF' |
| … |  |  |
| '6F03' | 5GS non-3GPP Access NAS Security Context | 'FF…FF' |
| '6F04’ | Subscriber Concealed Identifier Calculation Information | Operator dependent |
| '6F04' | UAC Access Identities Configuration | Operator dependent |
| '6F05' | Steering of UE in VPLMN | Yes |

***** End of change *****