3GPP TSG-CT WG6 Meeting #89Bis	C6-180405

- Sophia Antipolis (France), 10th July – 13th July 2018

| CR-Form-v11.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 31.102 | CR | 0809 | Rev | 1 | Current version: | 15.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps | X | ME | X | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Clarification of GET IDENTITY |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | G+D MS |  |  |  |  |  |  |  |  |  |
| Source to TSG: | C6 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5GS_Ph1-CT |  |  |  |  |  |  | Date: |  | 2018-07-12 |
| Category: | F |  |  |  |  |  |  | Release: |  | Rel-15 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | - It does not refer to the recently introduced Routing Indicator portion of the SUCI, as specified in 3GPP TS 23.003 clause 2.2B. - Clarification of the open points with regard to the coding of some of the data objects. - Coding of P2 may not be clear - Wrong section number [editorial] |  |  |  |  |  |  |  |  |
| Summary of change: |  | - Add references to Routing Indicator. - Added editor’s note to highlight the missing parts still to be completed. - Clarify the coding of P2 - Change section number [editorial] |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | - Misalignment with 3GPP TS 23.003 and 3GPP TS 33.501 - Potential incorrect use of P2 leading to non-interoperable UICC implementations |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 7.5.1.1, 7.5.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  | TS/TR ... CR ... |  |  |  |
| affected: |  |  | x | Test specifications |  |  | TS/TR ... CR ... |  |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  | TS/TR ... CR ... |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |

***** First change *****

#### 7.5.1.1	SUCI context

SUCI context shall be supported if "SUCI calculation is to be performed by the USIM" (i.e. Service n°124 and Service n°125 are "available").

The command returns the SUCI which is a privacy preserving identifier containing the concealed SUPI. The function is used in 5GS in the specific cases described in 3GPP TS 33.501 [105] prior to mutual authentication between the UE and the SN.

The SUCI returned is calculated as described in 3GPP TS 33.501 [105].

For the execution of the command, the following information shall be available in the USIM:

- Home network identifier (i.e. MCC and MNC) (see Note).

- Routing indicator (see Note).

- Home network public key (see Note).

- Home network public key identifier (see Note).

- Protection scheme identifier (see Note).

- SUPI.

NOTE:	Provision and storage of the information in the USIM when "SUCI calculation is to be performed by the USIM" (i.e. Service n°124 and Service n°125 are "available") is out of the scope of the specification.

The SUCI is designed for one-time use, however, the freshness and randomness of SUCI returned upon each call of the command depends on the protection scheme configured. There is the special case where the protection scheme used is null-scheme, in such case SUCI contains the non concealed SUPI.

If the home network public key is not provisioned in the USIM, the SUCI shall be calculated using the null-scheme irrespective of the protection scheme stored in the USIM.

The returned SUCI consists of the concatenation of the following information as described in 3GPP TS 23.003 [25]:

- Home network identifier (i.e. MCC and MNC).

- Routing indicator.

Editor’s Note: Routing indicator coding is FFS.

- Protection scheme identifier.

Editor’s Note: Protection scheme identifier coding is FFS.

- Home network public key identifier.

Editor’s Note: Home network public key identifier format is FFS.

- Scheme output, resulting from the protection scheme profile, identified by the protection scheme identifier. The protection scheme profile shall be one of those defined in Annex C of 3GPP TS 33.501 [105] or one of those specified by the Home network.

Editor’s Note: Scheme output format is not defined yet in 3GPP TS 23.003.

If SUCI context is supported and:

-	Service n°124 is not "available" or:

-	"SUCI calculation is to be performed by the ME” (i.e. Service n°124 is "available", and Service n°125 is not "available")

the status word '6985' (Conditions of use not satisfied) shall be returned

Editor’s Note: It is FFS to specify the behavior in case other parameters (e.g. Home network public key identifier, some necessary data is not provisioned) are not correctly configured

***** Next change *****

### 7.5.2	Command parameters and data

| Code | Value |
| --- | --- |
| CLA | As specified in TS 31.101 [11] |
| INS | '78' |
| P1 | '00' |
| P2 | Identity context, see Table X below' |
| Lc | Length of the subsequent data field or not present, see below |
| Data | See below |
| Le | '00', or maximum length of data expected in response |

Parameter P2 specifies the identity context as follows:

- Table 8: Coding of the reference control P2

| b8 | b7 | b6 | b5 | b4 | b3 | b2 | b1 | Meaning |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | X | X | X | X | X | X | X | Identity Context (See below): |
| 1 | 0 | 0 | 0 | 0 | 0 | 0 | 1 | SUCI |

All other codings are RFU.

#### 7.5.2.1	SUCI context

Command parameters/data: None

Response parameters/data:

| Byte(s) | Description | Length |
| --- | --- | --- |
| 1 to Le | SUCI TLV data object | Le |

Subscription Concealed Identifier TLV data object:

| Description | Value | M/O/C | Length (bytes) |
| --- | --- | --- | --- |
| SUCI TLV data object tag | 'A1' | M | 1 |
| Length | X | M | Note |
| SUCI value | -- | M | X |
| NOTE: The length is coded according to ISO/IEC 8825-1 [35] |  |  |  |

-  SUCI

For contents and coding of the SUCI data object TLV value see TS 3GPP TS 23.003 [25]. The most significant bit of SUCI is the most significant bit of the 1st byte of this TLV value field. The least significant bit of SUCI is the least significant bit of the last byte of this TLV value field.

Editor’s Note: Format of SUCI value is still to be completed in 3GPP TS 23.003.

***** End of change *****