3GPP TSG-CT6 Meeting #98	C6-200028

# Sophia-Antipolis, France, 25th Feb 2020 - 28th Feb 2020

| CR-Form-v12.0 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 31.121 | CR | 0311 | rev | - | Current version: | 15.6.1 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction of UICC interface during eDRX tests |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Comprion GmbH |  |  |  |  |  |  |  |  |  |
| Source to TSG: |  |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI15_Test |  |  |  |  |  | Date: |  |  | 2020-01-21 |
| Category: | C |  |  |  |  |  | Release: |  |  | Rel-15 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | To make use of the extended DRX cycle in 14.x tests, EFAD should be corrected to 4 bytes in accordance to 3GPP TS 31.102 and include a UE operation mode coded '01': normal operation + specific facilities. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Adding an appropriate coding for EFAD to the initial conditions of the 14.x tests |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | UEs do not use the eDRX as intended since the specific facility is not mentioned. A 3 byte EFAD will not allow to determine the MNC length. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 14,2, 14.3, 14.4 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# 14	UICC interface during eDRX

…

## 14.2	UICC interface during eDRX for E-UTRAN – eDRX is not accepted by E-USS/NB-SS

…

### 14.2.4	Method of test

#### 14.2.4.1	Initial conditions

The UE is configured to request the use of eDRX (in the ATTACH REQUEST and TRACKING AREA UPDATE messages).

The E-USS transmits on the BCCH, with the following network parameters:

-	TAI (MCC/MNC/TAC):	246/081/0001.

-	Access control:	unrestricted.

The NB-SS transmits on the BCCH, with the following network parameters:

-	TAI (MCC/MNC/TAC):	246/081/0001.

-	Access control:	unrestricted.

The default E-UTRAN UICC is installed into the Terminal with following exception:

EFAD	(Administrative Data)

Logically:	Normal operation + specific facilities;

Ciphering indicator feature disabled: 	b1=0;

CSG Display Control: for every PLMN not included in EF_OCSGL, or for which a CSG display indicator tag is not present, all available CSGs can be displayed without any restriction;

ProSe services for Public Safety: the ME is not authorized for ProSe services for Public Safety usage without contacting the ProSe Function;

Extended DRX cycle: the ME is authorized to modify the polling interval and/or disable the UICC interface during extended DRX cycle;

Length of MNC in the IMSI: 3.

| Coding: | B1 | B2 | B3 | B4 |
| --- | --- | --- | --- | --- |
| Hex | 01 | 00 | 08 | 03 |

The PIN of the USIM is disabled.

…

## 14.3	UICC interface during eDRX for E-UTRAN – UICC deactivation during eDRX

…

### 14.3.4	Method of test

#### 14.3.4.1	Initial conditions

The UE is configured to request the use of eDRX (in the ATTACH REQUEST and TRACKING AREA UPDATE messages).

The E-USS transmits on the BCCH, with the following network parameters:

-	TAI (MCC/MNC/TAC):	246/081/0001.

-	Access control:	unrestricted.

The NB-SS transmits on the BCCH, with the following network parameters:

-	TAI (MCC/MNC/TAC):	246/081/0001.

-	Access control:	unrestricted.

The default E-UTRAN UICC is installed into the Terminal with following exception:

EFAD	(Administrative Data)

Logically:	Normal operation + specific facilities;

Ciphering indicator feature disabled: 	b1=0;

CSG Display Control: for every PLMN not included in EF_OCSGL, or for which a CSG display indicator tag is not present, all available CSGs can be displayed without any restriction;

ProSe services for Public Safety: the ME is not authorized for ProSe services for Public Safety usage without contacting the ProSe Function;

Extended DRX cycle: the ME is authorized to modify the polling interval and/or disable the UICC interface during extended DRX cycle;

Length of MNC in the IMSI: 3.

| Coding: | B1 | B2 | B3 | B4 |
| --- | --- | --- | --- | --- |
| Hex | 01 | 00 | 08 | 03 |

The PIN of the USIM is disabled.

…

## 14.4	UICC interface during eDRX for E-UTRAN– SUSPEND UICC

…

### 14.4.4	Method of test

#### 14.4.4.1	Initial conditions

The UE is configured to request the use of eDRX (in the ATTACH REQUEST and TRACKING AREA UPDATE messages).

The E-USS transmits on the BCCH, with the following network parameters:

-	TAI (MCC/MNC/TAC):	246/081/0001.

-	Access control:	unrestricted.

The NB-SS transmits on the BCCH, with the following network parameters:

-	TAI (MCC/MNC/TAC):	246/081/0001.

-	Access control:	unrestricted.

The default E-UTRAN UICC is used with the following exceptions:

EFUMPC	(UICC Maximum Power Consumption)

Logically:

UICC maximum power consumption:	60 mA

Operator defined time out (T_OP):	5 seconds

Additional information:	UICC does not require increased idle current

UICC supports the UICC suspension procedure

Byte 4 and byte 5:	RFU

| Byte: | B1 | B2 | B3 | B4 | B5 |
| --- | --- | --- | --- | --- | --- |
| Coding: | 3C | 05 | 02 | 00 | 00 |

EFAD	(Administrative Data)

Logically:	Normal operation + specific facilities;

Ciphering indicator feature disabled: 	b1=0;

CSG Display Control: for every PLMN not included in EF_OCSGL, or for which a CSG display indicator tag is not present, all available CSGs can be displayed without any restriction;

ProSe services for Public Safety: the ME is not authorized for ProSe services for Public Safety usage without contacting the ProSe Function;

Extended DRX cycle: the ME is authorized to modify the polling interval and/or disable the UICC interface during extended DRX cycle;

Length of MNC in the IMSI: 3.

| Coding: | B1 | B2 | B3 | B4 |
| --- | --- | --- | --- | --- |
| Hex | 01 | 00 | 08 | 03 |

The PIN of the USIM is enabled and verified.

...