3GPP TSG-CT WG6 Meeting #114	C6-230113

# Athens, Greece, 28th Feb – 03rd March 2023

Source:	STMicroelectronics srl

Title:	Pseudo-CR on Study on  GBA_U Based APIs

Spec:	3GPP TR 31.822 v0.1.0

Agenda item:	7.13.2

Document for:	Approval

1. Introduction

The aim on this document is to introduce an alternative solution to Solution 2) for access control.

2. Reason for Change

Solution 2) uses file system content for authorize API usage of the GBA_U_APIs. In 3GPP TS 31 116 a different mechanism is used to authorize applications based on installation parameters. It is hence proposed an alternative Solution for access control based on application installation parameters.

3. Proposal

It is proposed to agree the following changes to 3GPP TS 31.822 v0.1.0.

New section 5.x is added to introduce the Solution 3 and new section x.2.3 is added to introduce the related evaluation.

* * * First Change * * * *

# 2	References

- 3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

- 3GPP TS 33.220: "Generic Authentication Architecture (GAA); Generic bootstrapping architecture".

[XX] 3GPP TS 31.116: "Remote APDU Structure for (U)SIM Toolkit applications"

***** Next change *****

## 5.X	Solution 3: Access Control to GBA_U_APIs through install parameters

## 5.X.1	Description

## An alternative approach to Solution 2 for Access Control may be based on application installation parameters. Applications using APIs that require NAA provider or issuer provider – like file system access – are authorized to perform the intended operation by adding specific install parameters as specified in 3GPP TS 31.116 [xx].

According to 3GPP TS 31.116[xx], Depending on issuer configuration, such install parameters may be protected with Remote Applet Management credentials that identify the off-card entity performing the Remote Applet Management operation and also with additional credentials (ETSI DAP key to protect Access and Toolkit install parameters) that can be used as an additional authentication according to the card issuer’s security policy.

It is then proposed to add a new install parameter subject to additional authentication according to the card issuer’s security policy to authorize applications to use GBA_U_APIs.

As an example of the install parameter structure a new access domain parameter as specified in ETSI TS 102 226 § 8.2.1.3.2.5.1 can be requested to be added as “API Access mechanism” (e.g. value ‘03’).

### 5.X.2	Solution Evaluation

This solution addresses the key issue 2 in an alternative way than Solution 2). It provides a mechanism for access control to GBA_U_APIs that is managed by the RAM off-card entity potentially in conjunction with ETSI DAP mechanism according to the card issuer’s security policy.

***** Next change *****

### x.2.3	Solution 3

#### x.2.3.1	Key issues addressed

This solution addresses key issue 2.

Table x.2: Key issue 2 solution evaluation

| Key Issue | Solution | Evaluation (subclause reference) |
| --- | --- | --- |
| Key Issues2: Support for Access Control to GBA_U_APIs | Solution3: Access Control to GBA_U_APIs | 5.x.2 |

#### x.2.3.2	Impact on USIM and ISIM

None.

#### x.2.3.3	Impact on UICC

A new install parameter needs to be defined at UICC level.

#### x.2.3.4	Backward compatibility

This solution is backward compatible.

#### x.2.3.5	Impact on core and RAN networks

This solution has no impact on the core network or RAN.

* * * End of Changes * * * *