| ETSI TC SCP Meeting #76 La Ciotat, France, 15-16 December 2016 |  |  |  |  |  |  |  |  |  |  |  |  |  | SCP(16)000249r1 |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| TC SCP CR-Form-v1.8.0 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 102 695-2 |  |  |  | CR | 044 |  |  |  rev |  |  |  |  Current version: |  |  |  |  | 11.0.0 |  |  |  |
| For HELP on using this form, see bottom of this page or look at the pop-up text over the  symbols. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Proposed change affects:  |  |  |  |  | smart card |  | X | terminal |  |  |  |  | server / network entity |  |  |  |  |  |  |  |  |
| Title:  |  | Correction of test procedure in 5.6.4.4.2.3. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Source:  |  | SCP |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Work item name:  |  | TEI10 |  |  |  |  |  |  |  |  |  |  |  | Date:  |  | 16/12/2016 |  |  |  |  |  |
| Category:  |  | F |  |  |  |  |  |  |  |  |  |  |  | Release:  |  | REL-10 |  |  |  |  |  |
|  |  | Use one of the following categories: F (essential correction) A (corresponds to a correction in an earlier release) B (Addition of feature), C (Functional modification of feature) D (Editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| TC SCP use of the status "Accepted" for change requests: - At the TC SCP working group level, the status "Accepted" means that a change request is accepted for presentation to TC SCP Plenary. - At the TC SCP Plenary level, the status "Accepted" means that a change request is accepted for implementation in the targeted specification. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Reason for change:  |  |  | There is a typo of the type F test command. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Summary of change:  |  |  | Correction of the step 4 in the test procedure. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Consequences if  not accepted: |  |  | The specification might be misunderstood. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| New tag value defined within the CR? |  |  |  | If ticked, add document number of related CR to TS 101 220: |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Clauses affected:  |  |  | 5.6.4.4.2.3 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Other specs  |  |  |  | Other core specifications  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Affected: |  |  |  | Test specifications |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Other comments:  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

##### 5.6.4.4.2	Test case 1: RF error indicator

5.6.4.4.2.1	Test execution

Void.

5.6.4.4.2.2	Initial conditions

- The host controller simulator is configured to support only the Type F RF gate.‬

- For full power mode execution: HCI session initialization has been performed, the HCI interface is idle and the SWP interface is not DEACTIVATED.

- For low power mode execution: the Host is not powered up.

- At the end of the previous activation, the state of the card emulation pipe was open, and the MODE parameter was '02' (as set by the UICC).

5.6.4.4.2.3	Test procedure

| Step | Direction | Description | RQ |
| --- | --- | --- | --- |
| 1 | HCSHUT | For full power mode execution: send EVT_FIELD_ON event. For low power mode execution: power up Host, activate SWP interface and establish SHDLC link. |  |
| 2 | HCSHUT | Send the ISO/IEC 18092 [Error: Reference source not found] 212 kbps/424 kbps frames, where LEN is set to '37', CMD0 to 'D8', CMD1 to '00', and Byte 0 to Byte 51 to '02FE000000000000060708090A0B0C0D0E0F101112131415 161718191A1B1C1D1E1F202122232425262728292A2B2C2D2E2F3031', with EVT_SEND_DATA event in which RF error indicator is set to '01'. |  |
| 3 | HCS | Wait 1 s from the end of sending EOF in SWP frame in step 2. During this time HUT shall not send any ISO/IEC 18092 [Error: Reference source not found] 212 kbps/424 kbps frames. | RQ6 |
| 4 | HCSHUT | Send the ISO/IEC 18092 [Error: Reference source not found] 212 kbps/424 kbps frames, where LEN is set to '37', CMD0 to 'D8', CMD1 to '00', and Byte 0 to Byte 51 to '02FE000000000000060708090A0B0C0D0E0F101112131415 161718191A1B1C1D1E1F202122232425262728292A2B2C2D2E2F3031', with EVT_SEND_DATA event in which RF error indicator is set to '00'. |  |
| 5 | HUTHCS | Send the ISO/IEC 18092 [Error: Reference source not found] 212 kbps/424 kbps frames, where LEN is set to '37', CMD0 to 'D9', CMD1 to '01', and Byte 0 to Byte 51 to '02FE000000000000060708090A0B0C0D0E0F101112131415 161718191A1B1C1D1E1F202122232425262728292A2B2C2D2E2F3031', with EVT_SEND_DATA event. | RQ2, RQ4 |
| 6 | HCSHUT | Send EVT_FIELD_OFF event. |  |