| ETSI TC SCP Meeting #69 Detroit, MI, U.S. , 9th to 10th of July 2015 |  |  |  |  |  |  |  |  |  |  |  |  | SCP(15)000160 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| ETSI TC SCP TEST Meeting #43 Sophia Antipolis, France, 30 March – 2 April 2015 |  |  |  |  |  |  |  |  |  |  |  |  |  |  | SCPTEST(15)043016r1 |  |  |  |  |  |  |  |
| TC SCP CR-Form-v1.8.0 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 103 115 |  |  |  | CR | 066 |  |  |  rev |  |  |  |  |  Current version: |  |  |  |  | 9.4.0 |  |  |  |
| For HELP on using this form, see bottom of this page or look at the pop-up text over the  symbols. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Proposed change affects:  |  |  |  |  | smart card |  | X | terminal |  |  |  |  |  | server / network entity |  |  |  |  |  |  |  |  |
| Title:  |  | Test case 6.1.2.3/ID7: correction of applet to activate EVENT_FIELD_OFF |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Source:  |  | SCP TEST |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Work item name:  |  | TEI9 |  |  |  |  |  |  |  |  |  |  |  |  | Date:  |  | 23/06/2015 |  |  |  |  |  |
| Category:  |  | F |  |  |  |  |  |  |  |  |  |  |  |  | Release:  |  | Rel-9 |  |  |  |  |  |
|  |  | Use one of the following categories: F (essential correction) A (corresponds to a correction in an earlier release) B (Addition of feature), C (Functional modification of feature) D (Editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  |  |  |  |  |  |  | Use one of the following releases: REL-6 (Release 6) REL-7 (Release 7) REL-8 (Release 8) REL-9 (Release 9) REL-10 (Release 10) REL-11 (Release 11) |  |  |  |  |  |  |
| TC SCP use of the status "Accepted" for change requests: - At the TC SCP working group level, the status "Accepted" means that a change request is accepted for presentation to TC SCP Plenary. - At the TC SCP Plenary level, the status "Accepted" means that a change request is accepted for implementation in the targeted specification. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Reason for change:  |  |  | Applet Api_1_Hsr_Ace_5 (used in 6.1.2.3/ID7) expects to be notified of EVENT_FIELD_OFF in onCallback(), but it never activates this event. This means that the required code will not be executed, and the test result will not be valid. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Summary of change:  |  |  | Applet is updated to activate EVENT_FIELD_OFF in its constructor. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Consequences if  not accepted: |  |  | Invalid test results; in particular, invalid UICCs might pass the test case. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| New tag value defined within the CR? |  |  |  | If ticked, add document number of related CR to TS 101 220: |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Clauses affected:  |  |  | Annex D |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Other specs  |  |  |  | Other core specifications  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Affected: |  |  |  | Test specifications |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Other comments:  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

Applet uicc/hci/test/framework/api_1_hsr_ace/Api_1_Hsr_Ace_5.java:

public class Api_1_Hsr_Ace_5 extends Applet implements CardEmulationListener {

[…]

private Api_1_Hsr_Ace_5() {

/*

* JavaCard applet register

*/

register();

ibuffer = JCSystem.makeTransientByteArray((short) 2, JCSystem.CLEAR_ON_RESET);

verificationByte = new byte[1];

verificationByte[0] = 0x00;

/*

* HCI listener register

*/

try {

ceService = (CardEmulationService) HCIDevice.getHCIService(HCIDevice.CARD_EMULATION_SERVICE_ID);

ceService.register(this);

ceService.activateEvent(EVENT_ON_SEND_DATA);

ceService.activateEvent(EVENT_FIELD_OFF);

} catch (HCIException e) {

ISOException.throwIt(ISO7816.SW_DATA_INVALID);

}

}

[…]

public void onCallback(byte event, HCIMessage hcimessage) {

CardEmulationMessage message = (CardEmulationMessage) hcimessage;

switch (event) {

case EVENT_ON_SEND_DATA:

ibuffer[0] = (byte) 0x90;

ibuffer[1] = 0x00;

message.prepareAndSendSendDataEvent(ibuffer, (short) 0, (short) 2);

return;

case EVENT_FIELD_OFF:

if ((verificationByte[0] & 0x02) == 0x00) //assure, that deselect hasn't been called yet

verificationByte[0] |= 0x01;

ceService.deactivateEvent(EVENT_ON_SEND_DATA);

//fall through

default:

return;

}

}