| ETSI TC SCP Meeting #69 Detroit, MI, U.S. , 9th to 10th of July 2015 |  |  |  |  |  |  |  |  |  |  |  |  | SCP(15)000180 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| ETSI TC SCP TEST Meeting #44 Sophia-Antipolis, France 22nd – 25th Jun 2015 |  |  |  |  |  |  |  |  |  |  |  |  |  |  | SCPTEST(15)044012r1 |  |  |  |  |  |  |  |
| TC SCP CR-Form-v1.8.0 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 103 115 |  |  |  | CR | 067 |  |  |  rev |  |  |  |  |  Current version: |  |  |  |  | 9.4.0 |  |  |  |
| For HELP on using this form, see bottom of this page or look at the pop-up text over the  symbols. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Proposed change affects:  |  |  |  |  | smart card |  | X | terminal |  |  |  |  |  | server / network entity |  |  |  |  |  |  |  |  |
| Title:  |  | Correction of test case 6.1.2.3, ID 10 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Source:  |  | SCP TEST |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Work item name:  |  | TEI-9 |  |  |  |  |  |  |  |  |  |  |  |  | Date:  |  | 23/06/2015 |  |  |  |  |  |
| Category:  |  | F |  |  |  |  |  |  |  |  |  |  |  |  | Release:  |  | REL-9 |  |  |  |  |  |
|  |  | Use one of the following categories: F (essential correction) A (corresponds to a correction in an earlier release) B (Addition of feature), C (Functional modification of feature) D (Editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  |  |  |  |  |  |  | Use one of the following releases: REL-6 (Release 6) REL-7 (Release 7) REL-8 (Release 8) REL-9 (Release 9) REL-10 (Release 10) REL-11 (Release 11) |  |  |  |  |  |  |
| TC SCP use of the status "Accepted" for change requests: - At the TC SCP working group level, the status "Accepted" means that a change request is accepted for presentation to TC SCP Plenary. - At the TC SCP Plenary level, the status "Accepted" means that a change request is accepted for implementation in the targeted specification. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Reason for change:  |  |  | 6.1.2.3.4 Method activateEvent ID 10: the test case currently expect that upon reception of EVT_CARD_DEACTIVATED, the test applet shall be deselected and the command shall not be dispatched to this applet. But for some UICCs, upon EVT_CARD_DEACTIVATED reception, the applet proprietary command is dispatched to the default applet (e.g. ISD) and the error code (6D00) may be returned, which will results in UICC failing this test ID. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Summary of change:  |  |  | Annex_D: Change the applet to replace (6D 00) by (6F 01) to be execluded from expected responces for test case 6.1.2.3.4 ID 10 6.1.2.3.4 ID 10: test case ID 10 has been modified to reflect the changes in the applet |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Consequences if  not accepted: |  |  | Correctly implemented UICC might unfairly fails the test case. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| New tag value defined within the CR? |  |  |  | If ticked, add document number of related CR to TS 101 220: |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Clauses affected:  |  |  | 6.1.2.3.4 ID 10, Annex_D |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Other specs  |  |  |  | Other core specifications  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Affected: |  |  |  | Test specifications |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Other comments:  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

#### 6.1.2.3	Method activateEvent

- […]

##### 6.1.2.3.2	Test Suite Files

| Applet Name | Test case ID |
| --- | --- |
| Api_1_Hsr_Ace_1.java | 1 |
| Api_1_Hsr_Ace_3.java | 2 |
| Api_1_Hsr_Ace_2.java | 3 |
| Api_1_Hsr_Ace_1.java | 4 |
| Api_1_Hsr_Ace_7.java | 5-1 |
| Api_1_Hsr_Ace_9.java | 5-2 |
| Api_1_Hsr_Ace_4.java | 6 |
| Api_1_Hsr_Ace_5.java | 7 |
| Api_1_Hsr_Ace_6.java | 8 |
| Api_1_Hsr_Ace_8.java | 9 |
| Api_1_Hsr_Ace_6.java | 10 |

##### 6.1.2.3.3	Initial conditions

- EVT_FIELD_ON has been sent on HCI interface.

- EVT_CARD_ACTIVATED has been sent on HCI interface.

- According applet has been successfully installed and selected using HCI Interface.

##### 6.1.2.3.4	Test procedure

| Test Case |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| ID | HCI Command | Description | API Expectation | HCI Response | CRR |
| [………..] |  |  |  |  |  |
| 9 | Check for Selection |  |  |  |  |
|  | Send EVT_SEND_DATA (INS = '01') | Service = CardEmulationService Listener = CardEmulationListener | Check for invocation of select() | EVT_SEND_DATA (SW - '90 00') | N12, N13 |
| 10 | No triggering after Event Card Deactivate |  |  |  |  |
|  | EVT_CARD_DEACTIVATE - Send EVT_SEND_DATA (INS = '01') | Service = CardEmulationService Listener = CardEmulationListener activateEvent(EVENT_ON_SEND_DATA) during the installation onCallback() activateEvent() event = EVENT_FIELD_OFF | EVT_SEND_DATA shall not be raised | No response or EVT_SEND_DATA withany response except: (SW - '90 00' or 6F 01') | N12, N14 |

#### [..]

Annex_D_TestAppletsSourceCode\Annex_D_TestAppletsSourceCode\src\uicc\hci\test\framework\api_1_hsr_ace\Api_1_Hsr_Ace_6.java

package uicc.hci.test.framework.api_1_hsr_ace;

/*

* Define specific SW

*/

private static final short SW_EVENT_NOT_SUPPORTED = ISO7816.SW_UNKNOWN + (short) 1;

….

public class Api_1_Hsr_Ace_6 extends Applet implements CardEmulationListener {

/**

* Called by the Java Card runtime environment to process an incoming APDU

* command.

*

* @see Applet#process(APDU)

*/

public void process(APDU apdu) throws ISOException {

/*

* Check for SELECT command

*/

if (selectingApplet())

return;

/*

* analyze incoming data

*/

byte buffer[] = apdu.getBuffer();

switch (buffer[ISO7816.OFFSET_INS]) {

default:

ISOException.throwIt(SW_EVENT_NOT_SUPPORTED);

}

}

}