3GPP TSG-CT Meeting #106	CP-243171

# Madrid, Spain; 9th – 10th December 2024

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 31.124 | CR | 0781 | rev | 3 | Current version: | 18.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps | X | ME | X | Radio Access Network | X | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | PLI rejected slice info test case |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Apple AB Denmark, Thales |  |  |  |  |  |  |  |  |  |
| Source to TSG: | Apple AB Denmark, Thales |  |  |  |  |  |  |  |  |  |
| Work item code: | UEConTest_R18 |  |  |  |  |  | Date: |  |  | 2024-11-26 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | No test cases available to verify PLI rejected slices info. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Added two new Test cases 1.XX and 1.YY in clause 27.22.4.15.4.2 to verify PLI rejected slice info (both with mapping and without mapping). |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | No test available to verify PLI rejected slice as per Rel 18 implementation. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 3.4, 27.22.4.15.3, 27.22.4.15.4, Annex B |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | C6-240707 was approved during CT6#120-bis, but an issue was pointed in Table E.1 once meeting closed. Indeed, creation of new Czzz in original version is not needed since this configuration was already defined in Table E.1 with C310. Content of CR 0781 Rev 2 was : In CR 0781 Rev 3, reference Czzz is replaced by C310 and condition Czzz is removed. |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | Was C6-240707 was C6-240695 was C6-240622 |  |  |  |  |  |  |  |  |

##first change###

## 3.4	Applicability table

NOTE:	It is possible that the applicability of some tests indicated in table B.1 does not match with the value in the Release column, due to late definition of the test sequences. Tests should be performed without considering the Release column, but only based on the conditions indicated for each release.

Table B.1: Applicability of tests

| Test | Test sequence | Description | from Rel. | up to Rel. (see note 2) | Applicability | Terminal Profile | Network Dependency | Support | Additional test case execution parameter |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 27.22.1 |  | PROFILE DOWNLOAD |  |  |  |  |  |  |  |
|  | 1 | PROFILE DOWNLOAD | R99 |  | M | E.1/1 | No |  |  |
| 27.22.2 |  | Contents of the TERMINAL PROFILE command |  |  |  |  |  |  |  |
|  | 1 | Contents of the TERMINAL PROFILE command | R99 |  | M | E.1/1 | No |  |  |
| 27.22.3 |  | Servicing of Proactive UICC Commands |  |  |  |  |  |  |  |
|  | 1 | Servicing of Proactive UICC Commands | R99 |  | M |  | No |  |  |
| 27.22.4.1 |  | DISPLAY TEXT |  |  |  |  |  |  |  |
|  | 1.1 | Unpacked | R99 |  | C177 | E.1/17 AND E.1/110 | No |  |  |

…

| 27.22.4.15 |  | PROVIDE LOCAL INFORMATION |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 1.1 | location information | R99 |  | M | E.1/31 | USS only |  | AER003 |
|  | … | … | … | … | … | … | … |  |  |
|  | 1.24 | Slices Information | Rel-16 |  | C231 | E.1/284 | NG-SS only |  |  |
|  | 1.25 | Slices Information, no served Slice | Rel-16 |  | C231 | E.1/284 | NG-SS only |  |  |
|  | 1.26 | Slices Information, several served Slices | Rel-16 |  | C231 | E.1/284 | NG-SS only |  |  |
|  | 1.27 | NG-RAN Timing advance | Rel-16 |  | C231 | E.1/305 | NG-SS only |  |  |
|  | 1.28 | PROVIDE LOCAL INFORMATION, NG-RAN Intra-Frequency Measurements | Rel-16 |  | C231 | E.1/305 | NG-SS only |  |  |
|  | 1.29 | PROVIDE LOCAL INFORMATION, NG-RAN Inter-Frequency Measurements | Rel-16 |  | C231 | E.1/305 | NG-SS only |  |  |
|  | 1.30 | PROVIDE LOCAL INFORMATION, CAG information list | Rel-17 |  | C235 | E.1/287 | NG-SS only |  |  |
|  | 1.31 | Satellite NG-RAN PrimaryTiming advance | Rel-17 |  | C236 | E.1/305 | SAT-NG-SS only |  |  |
|  | 1.32 | Access Technology, Satellite NG-RAN | Rel-17 |  | C236 | E.1/72 | SAT-NG-SS only |  |  |
|  | 1.XX | PROVIDE LOCAL INFORMATION, Rejected slice(s) information) - without S-NSSAI mapping | Rel-18 |  | C231 | E.1/289 | NG-SS only |  |  |
|  | 1.YY | PROVIDE LOCAL INFORMATION, Rejected slice(s) information) - with S-NSSAI mapping | Rel-18 |  | C231 | E.1/289 | NG-SS only |  |  |

#####Second change####

#### 27.22.4.15	PROVIDE LOCAL INFORMATION

##### 27.22.4.15.1	Definition and applicability

See clause 3.2.2.

##### 27.22.4.15.2	Conformance requirement

The ME shall support the PROVIDE LOCAL INFORMATION facility as defined in:

-	TS 31.111 [15] clause 6.4.15.

##### 27.22.4.15.3	Test purpose

To verify that the ME returns the following requested local information within a TERMINAL RESPONSE:

-	location information:

-	Mobile Country Code (MCC);

-	Mobile Network Code (MNC);

-	Location Area Code (LAC); and

-	cell ID of the current serving cell;

-	the IMEI of the ME;

-	the Network Measurement Results and the BCCH channel list;

-	the current date, time and time zone;

-	the current ME language setting;

-	the Timing Advance;

-	the Access Technology;

-	the IMEISV;

-	the Search Mode change;

-	the Battery charge State;

-	the UTRAN intra- and inter-frequency measurements;

-	the E-UTRAN intra- and inter-frequency measurements

-	the CSG ID list and corresponding HNB names of surrounding CSG cells (if class "q" is supported);

-	the list of slice(s) information;

-	the CAG information list and the corresponding CAG Human-readable network name per CAG ID (if available in the broadcasted information to the ME) of detected CAG cells (if class "ag" is supported);

-	the NG-RAN/Satellite NG-RAN Primary Timing Advance Information.

If the local information is stored in the ME; otherwise, sends the correct error code to the UICC in the TERMINAL RESPONSE.

To verify that the ME returns required error information in the TERMINAL RESPONSE in case requested information cannot be provided due to missing network coverage.

To verify that the E-UTRAN cell identifier is correctly transmitted when requesting the location information while accessing an E-UTRAN.

To verify that the NG-RAN cell identifier is correctly transmitted when requesting the location information while accessing an NG-RAN SA mode Cell.

To verify that the NG-RAN cell identifier is correctly transmitted when requesting the location information while accessing an NG-RAN Cell.

To verify that the SAT-NG-SS cell identifier is correctly transmitted when requesting the location information while accessing an SAT-NG-RAN Cell.

To verify that the Served S-NSSAIs are correctly transmitted in the TERMINAL RESPONSE when requesting the list of slice(s) information.

To verify that the ME reports available measurement information when requesting the Network Measurement Results while on an NG-RAN.

To verify that the ME reports available measurement information when requesting the Network Measurement Results while on an NG-RAN.

To verify that the CAG information list and the corresponding CAG Human-readable network name per CAG ID for specified PLMN are correctly transmitted in the TERMINAL RESPONSE when requesting the CAG information list and the corresponding CAG Human-readable network name per CAG ID of detected CAG cells when ME is camped on a CAG cell. When ME is not camped on a CAG cell, the TERMINAL RESPONSE shall contain 'ME currently not able to process command – no service' and optionally may return CAG cell selection status indicating ‘not camped on a CAG cell’ with additional information on the mode of selection.

To verify that ME provides the list of rejected slice(s) information in TERMINAL RESPONSE when the list of rejected slice(s) information is requested. When S-NSSAI mapping information is available for rejected slice(s), the list of rejected slice(s) with S-NSSAI mapping information shall be returned. When S-NSSAI mapping information is not available for rejected slice(s), the list of rejected slice(s) without S-NSSAI mapping shall be provided. When the S-NSSAI is rejected only on a subset of Tracking Area(s) of the current Registration Area, the tracking area identity list where the S-NSSAI is rejected shall be returned.

##### 27.22.4.15.4	Method of tests

27.22.4.15.4.1	Initial conditions

The ME is connected to the USIM Simulator.

For sequences 1.1 to 1.7 and 1.9 to 1.13 the UICC shall contain elementary files coded as defined in the default values for USIM Application Toolkit testing from clause 27.22.2A.

To operate the sequences 1.14 to 1.18 the E-UTRAN/EPC UICC as defined in clause 27.22.2B.1 shall be used.

To operate the sequences 1.22 to 1.29, 1.31 and 1.32 the NG-RAN UICC as defined in clause 27.22.2D.1 shall be used.

Prior to these test sequences the ME shall have been powered on and performed the PROFILE DOWNLOAD procedure.

To operate the sequence 1.30 the NG-RAN UICC supporting CAG, as defined in clause 27.22.2D.4 shall be used.

To operate the sequence 1.XX and 1.YY the NG-RAN UICC as defined in clause 27.22.2D.5 shall be used.

Sequences 1.2, 1.4, 1.5, 1.9 and 1.11 do not need any System Simulator connection.

For sequences 1.1, 1.7, 1.12 and 1.13 the ME is connected to a USS, for sequences 1.15, 1.16 and 1.18 the ME is connected to an E-USS only, for sequences 1.14 and 1.17 the ME is connected to an E-USS or NB-SS. In all listed sequences it has to perform the location update procedure or routing area update or combined update procedure.

For sequences 1.10 the ME is connected to an SS, a USS or an E-USS.

For sequences 1.22 to 1.YY the ME is connected to NG-SS. For all sequences but 1.30 it has to perform the Registration procedure.

The browser's cache shall have been cleared before execution of the test sequence. For sequence 1.24, 1.25, 1.26, 1.XX and 1.YY the URSP rules stored in the ME are:

URSP:

Rule Precedence =1

Traffic Descriptor:

- DNN=TestGp.rs

Route Selection Descriptor:

- Precedence=1

- Network Slice Selection, S-NSSAI: 01 01 01 03 (ST: eMBB, SD: 010103)

- SSC Mode Selection: SSC Mode 1

- Access Type preference: 3GPP access

Rule Precedence =2

Traffic Descriptor:

- DNN=Test12.rs

Route Selection Descriptor:

- Precedence=1

- Network Slice Selection, S-NSSAI: 01 01 01 02 (ST: eMBB, SD: 010102)

- SSC Mode Selection: SSC Mode 1

- Access Type preference: 3GPP access

Rule Precedence = <lowest priority>

Traffic Descriptor: *

Route Selection Descriptor:

- Precedence =1

- SSC Mode Selection: SSC Mode 1

- Access Type preference: 3GPP access

The NG-RAN parameters of the system simulator are:

- -	Mobile Country Code (MCC) = 001;

- -	Mobile Network Code (MNC) = 01;

- -	Tracking Area Code (TAC) = 000001;

- -	NG-RAN/Satellite NG-RAN Cell Identifier (NCI) = 0001 (36 bits);

- -	Allowed S-NSSAIs = 01010103 (SST: eMBB, SD: 010103), 01010102 (SST: eMBB, SD: 010102);

- -	NG-RAN/Satellite NG-RAN Primary Timing Advance = 0;

The NG-RAN parameters of the system simulator for sequence 1.XX is:

- -	Mobile Country Code (MCC) = 001;

- -	Mobile Network Code (MNC) = 01;

- -	Tracking Area Code (TAC) = 000001;

- -	NG-RAN/Satellite NG-RAN Cell Identifier (NCI) = 0001 (36 bits);

- -	Allowed S-NSSAIs = 01010103 (SST: eMBB, SD: 010103);

- -	Rejected S-NSSAIs = 01010102 (SST: eMBB, SD: 010102);

- -	NG-RAN/Satellite NG-RAN Primary Timing Advance = 0;

The NG-RAN parameters of the system simulator for sequence 1.YY is:

- -	Mobile Country Code (MCC) = 001;

- -	Mobile Network Code (MNC) = 02;

- -	Tracking Area Code (TAC) = 000001;

- -	NG-RAN/Satellite NG-RAN Cell Identifier (NCI) = 0001 (36 bits);

- -	Allowed S-NSSAIs with mapping information:

'01010103' (SST: eMBB, SD: 010103) '01010103' (Mapped HPLMN SST: eMBB, Mapped HPLMN SD:010103);

- -	Rejected S-NSSAIs with mapping information:

'01010102' (SST: eMBB, SD: 010102) '01010102' (Mapped HPLMN SST: eMBB, Mapped HPLMN SD:010102);

- -	NG-RAN/Satellite NG-RAN Primary Timing Advance = 0;

The E-UTRAN/NB-IoT parameters of the system simulator are:

-	Mobile Country Code (MCC) = 001;

-	Mobile Network Code (MNC) = 01;

-	Tracking Area Code (TAC) = 0001;

-	E-UTRAN Cell Identity value = 0001 (28 bits);

The UTRAN parameters of the system simulator are:

-	Mobile Country Code (MCC) = 001;

-	Mobile Network Code (MNC) = 01;

-	Location Area Code (LAC) = 0001;

-	Cell Identity value = 0001;

The GERAN parameters of the system simulator are:

-	Mobile Country Code (MCC) = 001;

-	Mobile Network Code (MNC) = 01;

-	Location Area Code (LAC) = 0001;

-	Cell Identity value = 0001;

-	Timing advance = 0;

-	Neighbour allocations = 561, 565, 568, 569, 573, 575, 577, 581, 582 and 585.

The PCS 1900 parameters of the system simulator are:

-	Mobile Country Code (MCC) = 001;

-	Mobile Network Code (MNC) = 011;

-	Location Area Code (LAC) = 0001;

-	Cell Identity value = 0001;

-	Timing advance = 0;

-	Neighbour allocations = 561, 565, 568, 569, 573, 575, 577, 581, 582 and 585.

Expected sequence 1.3 and 1.6 shall be used on a USS setting up only a GERAN or PCS 1900 cell and expected sequences 1.7 and 1.12 shall be used on a USS setting up only a UTRAN cell.

Expected sequence 1.12 requires two UTRA cells on the same frequency and 1.13 requires two UTRA cells on different frequencies.

Expected sequences 1.14 and 1.17 shall be used on a E-USS/NB-SS setting up only a E-UTRAN/NB-IoT cell.

Expected sequences 1.22, 1.23, 1.24, 1.25, 1.26, 1.27, 1.XX and 1.YY shall be used on a NG-SS setting up only a NG-RAN cell.

Expected sequence 1.15 requires two E-UTRA cells on the same frequency and 1.16 requires two E-UTRA cells on different frequencies, with second cell having EARFCN less than maxEARFCN. For both sequences if the USIM request is triggered in the RRC_CONNECTED state, the system simulator shall configure the corresponding frequency measurement for a sufficient period before sending the USIM request.

Expected sequence 1.18 requires two E-UTRAN cells configured in CSG mode.

For sequence 1.18 the default E-UTRAN/EPC UICC is used and the E-USS transmits on two cells with the following parameters:

Network parameters for Cell 1:

-	TAI (MCC/MNC/TAC):	001/01/0001.

-	Access control:	unrestricted.

-	csg-Indication:	TRUE

-	csg-Identity:	01 (27 bits)

- Home (e)NB Name	Home ONE

Network parameters for Cell 2:

-	TAI (MCC/MNC/TAC):	001/01/0002.

-	Access control:	unrestricted.

-	csg-Indication:	TRUE

-	csg-Identity:	02 (27 bits)

- Home (e)NB Name	Home TWO

Expected sequences 1.28 and 1.29 require two NG-RAN cells on the same NR band.

For sequences 1.28 and 1.29, the default NG-RAN UICC is used and the two cells with the following parameters:

Network parameters for Cell 1:

-	Mobile Country Code (MCC) = 001;

-	Mobile Network Code (MNC) = 01;

-	Tracking Area Code (TAC) = 000001;

-	 NG-RAN/Satellite NG-RAN Cell Identifier (NCI) = 0001 (36 bits);

-	Allowed S-NSSAIs = 01010103 (SST: eMBB, SD: 010103), 01010102 (SST: eMBB, SD: 010102);

-	 NG-RAN/Satellite NG-RAN Primary Timing Advance = 0;

Network parameters for Cell 2:

-	Mobile Country Code (MCC) = 001;

-	Mobile Network Code (MNC) = 01;

-	Tracking Area Code (TAC) = 000001;

-	 NG-RAN/Satellite NG-RAN Cell Identifier (NCI) = 0002 (36 bits);For sequence 1.28, the Cell 1 is the serving cell, and Cell 2 is the intra-frequency neighbour cell.

Note – Cell 1 and Cell 2 are configured with proper cell reference power, as well as SIB1 and SIB2 cellReselection Thresholds and Priority values that ensure ME will not reselect to Cell 2.

For sequence 1.29, the Cell 1 is the serving cell, and Cell 2 is the inter-frequency neighbour cell.

Note – Cell 1 is configured with SIB4 interFreqCarrierFreqList includes dl-CarrierFreq of Cell 2 Downlink NR SSB ARFCN; Cell 1 and Cell 2 are configured with proper cell reference power, as well as SIB1 and SIB4 with the cellReselection Thresholds and Priority values that ensure ME will not reselect to Cell 2.

For sequences 1.28 and 1.29, the ME is connected to Cell 1 and it has performed the Registration procedure.

Expected sequence 1.30 requires two NR CAG cells.

For sequence 1.30, the NG-SS transmits two NR CAG cells with following configuration:

Cell 1:

-	Mobile Country Code (MCC) = 244;

-	Mobile Network Code (MNC) = 083;

-	Tracking Area Code (TAC) = 000001;

Cell 2:

-	Mobile Country Code (MCC) = 244;

-	Mobile Network Code (MNC) = 084;

-	Tracking Area Code (TAC) = 000002;

CAG cell 1 has the following parameters configured for NPN-Identity and corresponding HRNN in SIB1 and SIB 10 respectively:

Entry 1:

-	Mobile Country Code (MCC) = 244;

-	Mobile Network Code (MNC) = 083;

-	CAG ID:	00 00 00 01

-	No CAG human-readable network name broadcasted

Entry 2:

-	Mobile Country Code (MCC) = 244;

-	Mobile Network Code (MNC) = 084;

-	CAG ID:	00 00 00 02

-	CAG human-readable network name: ′CAG-00000002′

Expected sequence 1.31 shall be used on a SAT-NG-SS setting up only a Satellite NG-RAN cell with following configuration:

The Satellite NG-RAN parameters of the system simulator are:

-	Mobile Country Code (MCC) = 001;

-	Mobile Network Code (MNC) = 01;

-	Tracking Area Code (TAC) = 000001;

-	NG-RAN/Satellite NG-RAN Cell Identifier (NCI) = 0001 (36 bits);

-	NG-RAN/Satellite NG-RAN Primary Timing Advance = 0.

27.22.4.15.4.2	Procedure

#####Third change####

Expected Sequence 1.XX (PROVIDE LOCAL INFORMATION, Rejected slice(s) information) - without S-NSSAI mapping

| Step | Direction | MESSAGE / Action | Comments |
| --- | --- | --- | --- |
| 1 | USER → ME | Internet PDU session using DNN "TestGp.rs" is configured in the terminal | DNN: "TestGp.rs" for internet PDU |
| 2 | ME NG-SS | ME successfully REGISTER with NG-RAN cell. |  |
| 3 | ME NG-SS | An Internet PDU Session is established successfully. | DNN=TestGp.rs, S-NSSAI = 01010103 |
| 4 | UICC ME | PROACTIVE COMMAND PENDING PROVIDE LOCAL INFORMATION 1.XX.1 |  |
| 5 | ME UICC | FETCH |  |
| 6 | UICC ME | PROACTIVE COMMAND: PROVIDE LOCAL INFORMATION 1.XX.1 |  |
| 7 | ME UICC | TERMINAL RESPONSE: PROVIDE LOCAL INFORMATION 1.XX.1 | [Command performed successfully]. |

PROACTIVE COMMAND: PROVIDE LOCAL INFORMATION 1.XX.1

Logically:

Command details

Command number:	1

Command type:	PROVIDE LOCAL INFORMATION

Qualifier:	"17" rejected slices information

Device identities

Source device:	UICC

Destination device:	ME

Coding:

| BER-TLV: | D0 | 09 | 81 | 03 | 01 | 26 | 17 | 82 | 02 | 81 | 82 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

TERMINAL RESPONSE: PROVIDE LOCAL INFORMATION 1.XX.1

Logically:

Command details

Command number:	1

Command type:	PROVIDE LOCAL INFORMATION

Qualifier:	"17" rejected slices information

Device identities

Source device:	ME

Destination device:	UICC

Result

General Result:	Command performed successfully

Rejected slices information with S-NSSAI mapping: empty (zero length)

Rejected slices information:

Length of rejected S-NSSAI elements: 4

Rejected S-NSSAI:  '01 01 01 01' (SST: eMBB, SD: 010101)

Coding:

| BER-TLV: | 81 | 03 | 01 | 26 | 17 | 82 | 02 | 82 | 81 | 83 | 01 | 00 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 57 | 00 | 31 | 04 | 01 | 01 | 01 | 01 |  |  |  |  |

Expected Sequence 1.YY (PROVIDE LOCAL INFORMATION, Rejected slice(s) information) - with S-NSSAI mapping

| Step | Direction | MESSAGE / Action | Comments |
| --- | --- | --- | --- |
| 1 | USER → ME | Internet PDU session using DNN "TestGp.rs" is configured in the terminal | DNN: "TestGp.rs" for internet PDU |
| 2 | ME NG-SS | ME successfully REGISTER with NG-RAN cell. |  |
| 3 | ME NG-SS | An Internet PDU Session is established successfully. | DNN=TestGp.rs, S-NSSAI = 01010103 |
| 4 | UICC ME | PROACTIVE COMMAND PENDING PROVIDE LOCAL INFORMATION 1.YY.1 |  |
| 5 | ME UICC | FETCH |  |
| 6 | UICC ME | PROACTIVE COMMAND: PROVIDE LOCAL INFORMATION 1.YY.1 |  |
| 7 | ME UICC | TERMINAL RESPONSE: PROVIDE LOCAL INFORMATION 1.YY.1 | [Command performed successfully]. |

PROACTIVE COMMAND: PROVIDE LOCAL INFORMATION 1.YY.1

Same as PROACTIVE COMMAND: PROVIDE LOCAL INFORMATION 1.XX.1 in expected sequence 1.XX.1

TERMINAL RESPONSE: PROVIDE LOCAL INFORMATION 1.YY.1

Logically:

Command details

Command number:	1

Command type:	PROVIDE LOCAL INFORMATION

Qualifier:	"17" rejected slices information

Device identities

Source device:	ME

Destination device:	UICC

Result

General Result:	Command performed successfully

Rejected Slice information with S-NSSAI mapping:

Length of allowed S-NSSAI with mapping information elements: 08

Rejected S-NSSAI with mapping information element 1:  '01010102' (SST: eMBB, SD: 010102) '01010102' (Mapped HPLMN SST: eMBB, Mapped HPLMN SD:010102)

Rejected Slices information: empty (zero length)

Coding:

| BER-TLV: | 81 | 03 | 01 | 26 | 17 | 82 | 02 | 82 | 81 | 83 | 01 | 00 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 57 | 08 | 01 | 01 | 01 | 02 | 01 | 01 | 01 | 02 | 31 | 00 |

##third change###

Annex B (normative):
Details of terminal profile support

Table E.1: TERMINAL PROFILE support

| Item | Byte.bit | Terminal Profile | Ref. | Release | Status | Support | Mnemonic |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 1.1 | Profile Download | TS 31.111 §5.2 | R99 | M |  | PD_Pro_Dwnl |
| 2 | 1.2 | SMS-PP data download | TS 31.111 §5.2 | R99 | C279 |  | PD_SMS_PP |
| … | … | … | … | … | … |  | … |
| … | … | … | … | … | … |  | … |
| 282 | 36.2 | Event: Network Rejection for NG-RAN | TS 31.111 §5.2 | Rel-15 | C310 |  | PD_Event_NW_Rejection_NR |
| 283 | 36.3 | Non-IP Data Delivery support (if class "e" and class "ai" are supported) | TS 31.111 §5.2 | Rel-15 | C311 |  | PD_Non_IP_Data_Delivery |
| 284 | 36.4 | Support of PROVIDE LOCAL INFORMATION, Slice information | TS 31.111 §5.2 | Rel-16 | C310 |  | PD_PLI_Slice_Information |
| 285 | 36.5 | Reserved for 3GPP | TS 31.111 §5.2 | Rel-15 | X |  |  |
| 286 | 36.6 | Reserved for 3GPP | TS 31.111 §5.2 | Rel-15 | X |  |  |
| 287 | 36.7 | Support of CAG feature | TS 31.111 §5.2 | Rel-17 | C314 |  | PD_CAG_feature |
| 288 | 36.8 | Reserved for 3GPP | TS 31.111 §5.2 | Rel-15 | X |  |  |
| 289 | 37.1 | Support of PROVIDE LOCAL INFORMATION, Rejected Slice information | TS 31.111 §5.2 | Rel-18 | C310 |  | PD_PLI_Rejected Slice_Information |
| 290 | 37.2 | Reserved for 3GPP | TS 31.111 §5.2 | Rel-15 | X |  |  |
| 291 | 37.3 | Reserved for 3GPP | TS 31.111 §5.2 | Rel-15 | X |  |  |
| 292 | 37.4 | Reserved for 3GPP | TS 31.111 §5.2 | Rel-15 | X |  |  |
| 293 | 37.5 | Reserved for 3GPP | TS 31.111 §5.2 | Rel-15 | X |  |  |
| 294 | 37.6 | Reserved for 3GPP | TS 31.111 §5.2 | Rel-15 | X |  |  |
| 295 | 37.7 | Reserved for 3GPP | TS 31.111 §5.2 | Rel-15 | X |  |  |
| 296 | 37.8 | Reserved for 3GPP | TS 31.111 §5.2 | Rel-15 | X |  |  |
| 297 | 38.1 | Reserved for 3GPP | TS 31.111 §5.2 | Rel-15 | X |  |  |
| 298 | 38.2 | Reserved for 3GPP | TS 31.111 §5.2 | Rel-15 | X |  |  |
| 299 | 38.3 | Reserved for 3GPP | TS 31.111 §5.2 | Rel-15 | X |  |  |
| 300 | 38.4 | Reserved for 3GPP | TS 31.111 §5.2 | Rel-15 | X |  |  |
| 301 | 38.5 | Reserved for 3GPP | TS 31.111 §5.2 | Rel-15 | X |  |  |
| 302 | 38.6 | Reserved for 3GPP | TS 31.111 §5.2 | Rel-15 | X |  |  |
| 303 | 38.7 | Reserved for 3GPP | TS 31.111 §5.2 | Rel-15 | X |  |  |
| 304 | 38.8 | Reserved for 3GPP | TS 31.111 §5.2 | Rel-15 | X |  |  |
| 305 | 39.1 | Proactive UICC: PROVIDE LOCAL INFORMATION (NG-RAN/Satellite NG-RAN Timing Advance Information) | TS 31.111 §5.2 | Rel-16 | C310 OR C315 |  | PD_Provide_Local_NR_TA |
| 306 | 39.2 | Reserved for 3GPP | TS 31.111 §5.2 | Rel-16 | X |  |  |
| 307 | 39.3 | Reserved for 3GPP | TS 31.111 §5.2 | Rel-16 | X |  |  |
| 308 | 39.4 | Reserved for 3GPP | TS 31.111 §5.2 | Rel-16 | X |  |  |
| 309 | 39.5 | Reserved for 3GPP | TS 31.111 §5.2 | Rel-16 | X |  |  |
| 310 | 39.6 | Reserved for 3GPP | TS 31.111 §5.2 | Rel-16 | X |  |  |
| 311 | 39.7 | Reserved for 3GPP | TS 31.111 §5.2 | Rel-16 | X |  |  |
| 312 | 39.8 | Reserved for 3GPP | TS 31.111 §5.2 | Rel-16 | X |  |  |

| C201 | Void | -- Void |
| --- | --- | --- |
| C202 | Void | -- Void |
| ... | … | … |
| … | … | … |
| C294 | IF A.1/84 AND A.1/164 THEN M ELSE O.1 | -- O_No_Type_ND AND O_Refresh_AlphaIdentifier |
| C295 | IF A.1/165 THEN M ELSE O.1 | -- O_ProSE |
| C296 | IF A.1/166 THEN M ELSE O.1 | -- O_WLAN_Access_Status |
| C297 | IF A.1/167 THEN M ELSE O.1 | -- O_WLAN_Bearer |
| C298 | IF A.1/168 THEN M ELSE O.1 | -- O_I-WLAN_OR_WLAN |
| C299 | IF A.1/150 AND A.1/179 THEN M ELSE O.1 | -- O_IMS AND O_SM-over-IP_without_MSISDN |
| C300 | IF A.1/150 AND A.1/84 AND A.1/85 AND A.1/87 AND A.1/180 THEN M ELSE O.1 | -- O_IMS AND O_No_Type_ND AND O_No_Type_NK AND O_No_Type_NS AND O_Voice_Call_with_URI |
| C301 | IF A.1/169 THEN M ELSE O.1 | -- O_Media_Type_Voice |
| C302 | IF A.1/170 THEN M ELSE O.1 | -- O_Media_Type_Video |
| C303 | Void |  |
| C304 | IF A.1/87 THEN M ELSE O.1 | -- O_No_Type_NS |
| C305 | IF (A.1/64 OR A.1/134) THEN M ELSE O.1 | -- O_GERAN OR O_UTRAN |
| C306 | IF A.1/186 THEN O ELSE M | -- O_NB-IoT_only |
| C307 | IF A.1/189 THEN M ELSE O.1 | -- O_PLI_HeNB_IP_Address |
| C308 | IF A.1/190 THEN M ELSE O.1 | -- O_PLI_HeNB_Sur_Macrocells |
| C310 | IF A.1/187 THEN M ELSE O.1 | -- pc_NG_RAN |
| C311 | IF A.1/192 THEN M ELSE O.1 | -- O_NIDD |
| C312 | Void |  |
| C313 | IF (NOT A.1/135) AND A.1/78 THEN M ELSE O.1 | -- (NOT O_EUTRAN_NO_UTRAN_NO_GERAN) AND O_AddInfo_SS |
| C314 | IF (A.1/187 AND A.1/196) THEN M ELSE O.1 | -- pc_NG_RAN AND pc_CAG |
| C315 | IF A.1/BBB THEN M ELSE O.1 | -- pc_nonTerrestrialNetwork_r17 |
| O.1 | Allowed: Bit value ="0" or bit not present |  |

##end of change###