3GPP TSG CT Meeting #106	CP-243319

Madrid, ES, 9 - 10 December, 2024 	was CP-243279

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 29.561 | CR | 0164 | rev | 4 | Current version: | 18.5.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Supporting transfer of media related information over N6 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Lenovo |  |  |  |  |  |  |  |  |  |
| Source to TSG: | Lenovo, Ericsson |  |  |  |  |  |  |  |  |  |
| Work item code: | XRM_Ph2 |  |  |  |  |  | Date: |  |  | 2024-08-11 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | Stage 2 has defined that the QUIC UDP tunnel is used to convey the media related information from the application server to the UPF. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Description to establish connect UDP tunnel connect-UDP establishment, and media related information transmission over HTTP Datagrams, and Option-UDP. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Stage 3 for the transportation of the media related information remains incomplete. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 2, 22 (new), 22.1 (new), 22.2 (new), 22.3 (new), 22.3.1 (new), 22.3.2 (new), 22.3.3 (new) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  | X |  | Other core specifications |  |  |  | TS23.501 CR 5711 |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | Revision 3: - Original version was self-contained with references and full hierarchy of clauses, but revision 1 removed some clauses and references to avoid clash with CR0167. CR 0167, however, was postponed and this revision 2 restores the references and full hierarchy of clauses for appropriate CR implementation. Revision 4: - Adjustment of “source to TSG”. Addition of an existing reference in clause 22.3.3, to the reference section. Addition of an EN that “General clause is to be added.” In clause 22.3.1, instead of “Void”. |  |  |  |  |  |  |  |  |

********************* First Change *********************

# 2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[2]	3GPP TS 23.501: "System Architecture for the 5G System; Stage 2".

[3]	3GPP TS 23.502: "Procedures for the 5G System; Stage 2".

[4]	3GPP TS 29.281: "General Packet Radio System (GPRS) Tunnelling Protocol User Plane (GTPv1-U)".

[5]	3GPP TS 29.061: "Interworking between the Public Land Mobile Network (PLMN) supporting packet based services and Packet Data Networks (PDN)".

[6]	IETF RFC 3748: "Extensible Authentication Protocol (EAP)".

[7]	IETF RFC 3579: "RADIUS (Remote Authentication Dial In User Service) Support For Extensible Authentication Protocol (EAP)".

[8]	IETF RFC 2865: "Remote Authentication Dial In User Service (RADIUS)".

[9]	IETF RFC 3162: "RADIUS and IPv6".

[10]	IETF RFC 4818: "RADIUS Delegated-IPv6-Prefix Attribute".

[11]	IETF RFC 5216: "The EAP-TLS Authentication Protocol".

[12]	3GPP TS 23.228: "IP Multimedia Subsystem (IMS); Stage 2".

[13]	3GPP TS 24.229: "IP Multimedia Call Control Protocol based on SIP and SDP; Stage 3".

[14]	IETF RFC 2132: "DHCP Options and BOOTP Vendor Extensions".

[15]	IETF RFC 3361: "Dynamic Host Configuration Protocol (DHCP-for-IPv4) Option for Session Initiation Protocol (SIP) Servers".

[16]	IETF RFC 3646: "DNS Configuration options for Dynamic Host Configuration Protocol for IPv6 (DHCPv6)".

[17]	IETF RFC 3319: "Dynamic Host Configuration Protocol (DHCPv6) Options for Session Initiation Protocol (SIP) Servers".

[18]	IETF RFC 2131: "Dynamic Host Configuration Protocol".

[19]	IETF RFC 1542: "Clarification and Extensions for the Bootstrap Protocol".

[20]	IETF RFC 4039: "Rapid Commit Option for the Dynamic Host Configuration Protocol version 4 (DHCPv4)".

[21]	IETF RFC 3315: "Dynamic Host Configuration Protocol for IPv6 (DHCPv6)".

[22]	IETF RFC 3736: "Stateless Dynamic Host Configuration Protocol (DHCP) Service for IPv6".

[23]	IETF RFC 7155: "Diameter Network Access Server Application".

[24]	IETF RFC 6733: "Diameter Base Protocol".

[25]	IETF RFC 4072: "Diameter Extensible Authentication Protocol (EAP) Application".

[26]	IETF RFC 2866: "RADIUS Accounting".

[27]	IETF RFC 5176: "Dynamic Authorization Extensions to Remote Authentication Dial In User Service (RADIUS)".

[28]	3GPP TS 23.003: "Numbering, addressing and identification".

[29]	IETF RFC 1825: "Security Architecture for the Internet Protocol".

[30]	IETF RFC 1826: "IP Authentication Header".

[31]	IETF RFC 1827: "IP Encapsulating Security Payload (ESP)".

[32]	IETF RFC 4291: "IP Version 6 Addressing Architecture".

[33]	IETF RFC 4861: "Neighbor Discovery for IP Version 6 (IPv6)".

[34]	IETF RFC 4862: "IPv6 Stateless Address Autoconfiguration".

[35]	IETF RFC 1027: "Using ARP to Implement Transparent Subnet Gateways".

[36]	802.3-2015 - IEEE Standard for Ethernet.

[37]	IETF RFC 5281: "Extensible Authentication Protocol Tunneled Transport Layer Security Authenticated Protocol Version 0 (EAP-TTLSv0)".

[38]	3GPP TS 23.380: "IMS Restoration Procedures".

[39]	3GPP TS 29.571: "5G System; Common Data Types for Service Based Interfaces; Stage 3".

[40]	3GPP TS 29.502: "5G System; Session Management Services; Stage 3".

[41]	3GPP TS 29.229: "Cx and Dx interfaces based on Diameter protocol; Protocol details".

[42]	3GPP TS 24.501: "Non-Access-Stratum (NAS) protocol for 5G System (5GS); Stage 3".

[43]	3GPP TS 23.316: "Wireless and wireline convergence access support for the 5G System (5GS)".

[44]	IETF RFC 7761: "Protocol Independent Multicast - Sparse Mode (PIM-SM): Protocol Specification (Revised)".

[45]	IETF RFC 3973: "Protocol Independent Multicast - Dense Mode (PIM-DM): Protocol Specification (Revised)".

[46]	3GPP TS 29.571: "5G System; Common Data Types for Service Based Interfaces Stage 3".

[47]	IETF RFC 2132: "DHCP Options and BOOTP Vendor Extensions".

[48]	IETF RFC 3925: "Vendor-Identifying Vendor Options for Dynamic Host Configuration Protocol version 4 (DHCPv4)".

[49]	IETF RFC 8415: "Dynamic Host Configuration Protocol for IPv6 (DHCPv6)".

[50]	3GPP TS 29.274: "3GPP Evolved Packet System. Evolved GPRS Tunnelling Protocol for EPS (GTPv2)".

[51]	CableLabs WR-TR-5WWC-ARCH: "5G Wireless Wireline Converged Core Architecture".

[52]	BBF WT-470: "5G FMC Architecture".

[53]	3GPP TS 23.401: "General Packet Radio Service (GPRS) enhancements for Evolved Universal Terrestrial Radio Access Network (E-UTRAN) access".

[54]	BBF TR-456: "AGF Functional Requirements".

[55]	CableLabs DOCSIS MULPI: "Data-Over-Cable Service Interface Specifications DOCSIS 3.1, MAC and Upper Layer Protocols Interface Specification".

[56]	IETF RFC 7542: "The Network Access Identifier".

[57]	IETF RFC 2661: " Layer Two Tunneling Protocol "L2TP".

[58]	3GPP TS 29.244: "Interface between the Control Plane and the User Plane of EPC Nodes; Stage 3".

[59]	3GPP TS 33.501: "Security architecture and procedures for 5G system".

[60]	3GPP TS 23.247: "Architectural enhancements for 5G multicast-broadcast services; Stage 2".

[61]	3GPP TS 26.502: "5G Multicast-Broadcast User Service Architecture".

[62]	IETF RFC 6603: "Prefix Exclude Option for DHCPv6-based Prefix Delegation".

[63]	3GPP TS 23.434: "Service Enabler Architecture Layer for Verticals (SEAL); Functional architecture and information flows".

[64]	IETF RFC 9298: "Proxying UDP in HTTP"

[65]	IETF RFC 9114: "HTTP/3"

[66]		IETF draft-ietf-tsvwg-udp-options: "Transport Options for UDP".

Editor's note: The above reference will be revised to RFC when finalized by IETF.

********************* Second Change *********************

# 22	Interworking with DN for e2e encrypted traffic

## 22.1	General

Editor's Note:	General clause is to be added.

## 22.2	Media Related Information

Editor's Note:	Details about format, encoding are to be further specified.

## 22.3	Connect-UDP

### 22.3.1	Introduction

The established connect-UDP tunnel may be used for transferring of Media Related Information used for the identification of the following information, defined in 3GPP TS 23.501 [2], as

a)	PDU Set Information;

b)	End of Data Burst Indication;

c)	Expedited Transfer Indication;

d)	Data Burst Size; and

e)	Time to Next Burst,

in-band from the HTTP server to the HTTP client in the UPF.

Editor's Note: Further updates may be needed according to the latest stage 2 requirements.

The format and semantics of the Media Related Information field are defined in clause 22.2.

### 22.3.2	Sending Media Related Information using proxying UDP in HTTP Datagram

To establish a connect-UDP tunnel as defined in IETF RFC 9298 [64], the HTTP client in the UPF, shall include in the HTTP/3 CONNECT request :protocol pseudo-header set to connect-udp. Upon receipt of a successful 2XX response as per IETF RFC 9298 [64], the UPF shall consider that the UDP tunnel is established to receive HTTP datagrams.

If the HTTP client in the UPF, chooses to reuse the QUIC connection to the HTTP server for multiple UEs, the UPF shall open a different QUIC stream for the HTTP/3 CONNECT request issued for each UE.

The HTTP client in the UPF may use different QUIC connections to the HTTP server for multiple UEs, each connection with a separate TLS security context as per IETF RFC 9114 [65].

When the Media Related Information are encapsulated in the HTTP Datagram of the connect-UDP tunnel, then the HTTP server shall include in the HTTP Datagram payload:

Editor's Note: Details about HTTP Datagram is FFS.

NOTE:	The UDP proxying payload can be end-to-end encrypted between a UE and an Application Server.

### 22.3.3	Sending Media Related Information using transport options for UDP

When the Media Related Information are encapsulated in the transport options for UDP draft-ietf-tsvwg-udp-options [66], then the HTTP server shall include the Media Related Information in a UDP option of the outer UDP header of the connect-UDP tunnel. The Media Related Informtion shall be encrypted between the HTTP server and the HTTP client in the UPF.

Editor's Note:	Details about format, encoding and encryption are to be further specified.

Upon receipt of the encrypted UDP option with Media Related Information, the UPF shall extract the Media Related Information from the UDP option for DL transmission with differentiated handling.

********************* End of Changes *********************