3GPP TSG-CT WG4 Meeting #119	C4-235155

Chicago, US; 13th– 17th November 2023	was C4-234622

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 29.518 | CR | 0965 | rev | 2 | Current version: | 17.11.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correct the N2MbsSMinfo in the MBS broadcast service |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson, Nokia, Nokia Shanghai Bell |  |  |  |  |  |  |  |  |  |
| Source to TSG: | CT4 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5MBS |  |  |  |  |  | Date: |  |  | 2023-10-31 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | When the NF service consumer, i.e. MB-SMF, sends a ContextCreate request message to the AMF to create a broadcast MBS session context, it shall contain the N2 MBS Session Management container" n2MbsSmInfo". However, this n2MbsSmInfo can NOT include NG-RAN ID, since it is not available in the MB-SMF. For ContextUpdate Request, if the update is intended for a list of specific NG-RAN nodes, it shall use "ranIdList" instead. However, ranId is defined as Mandatory IE in the N2MbsSmInfo, see below: ##### 6.5.6.2.7 Type: N2MbsSmInfo Table 6.5.6.2.7-1: Definition of type N2MbsSmInfo Attribute name; Data type; P; Cardinality; Description. ngapIeType; NgapIeType; M; 1; This IE shall indicate the NGAP IE type of the ngapData as specified in clause 6.5.6.3.4.. ngapData; RefToBinaryData; M; 1; This IE shall contain the reference to the binary data part carrying the NGAP data.. ranId; GlobalRanNodeId; M; 1; This IE shall indicate the Global RAN ID of the gNB that generated the N2 MBS Session Management related information.. The ranId shall only be included in the Context Create/Update response and and Context Status Notification request from AMF to the MB-SMF. Therefore, the ranId in N2MbsSmUnfo is not mandatory and shall be changed into an conditional property. In additon, the ranId has been defined as randId in the OpenAPI, therefore a correction is also needed. Therefore, we would suggest to change the Cardinatily of the ranId in to conditional and also renaming the ranId in the OpenAPI file. |
| Summary of change: |  | Change the ranId from mandatory into conditional. Correct the ranId in OpenAPI file. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The Broadcast MBS session cannot be created since the Global RAN ID is not avaliable in the MB-SMF. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.5.6.2.7, A.6. |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | This CR introduces backward compatible correction on the OpenAPI Namf_MBSBroadcast. |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | Rev1: wording improvement for the description of ranId in the N2MbsSmInfo IE. Rev2: resubmission for November meeting. |  |  |  |  |  |  |  |  |

# Additional discussion(if needed):

# Proposed changes:

# *** First Change ***

##### 6.5.6.2.7	Type: N2MbsSmInfo

Table 6.5.6.2.7-1: Definition of type N2MbsSmInfo

| Attribute name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| ngapIeType | NgapIeType | M | 1 | This IE shall indicate the NGAP IE type of the ngapData as specified in clause 6.5.6.3.4. |
| ngapData | RefToBinaryData | M | 1 | This IE shall contain the reference to the binary data part carrying the NGAP data. |
| ranId | GlobalRanNodeId | C | 0..1 | This IE shall indicate the Global RAN ID of the gNB that generated the N2 MBS Session Management related information. The IE shall be present when the N2MbsSmInfo is included in the ContextCreate Response, ContextUpdate Response, or ContextStatusNotify Request messages which are sent from the AMF to the NF service consumer (e.g., MB-SMF). |

# *** Next Change ***

# A.6	Namf_MBSBroadcast API

openapi: 3.0.0

info:

version: 1.0.3

title: Namf_MBSBroadcast

description: |

AMF MBSBroadcast Service.

© 2023, 3GPP Organizational Partners (ARIB, ATIS, CCSA, ETSI, TSDSI, TTA, TTC).

All rights reserved.

externalDocs:

description: 3GPP TS 29.518 V17.9.0; 5G System; Access and Mobility Management Services

url: 'https://www.3gpp.org/ftp/Specs/archive/29_series/29.518/'

servers:

- url: '{apiRoot}/namf-mbs-bc/v1'

variables:

apiRoot:

default: https://example.com

description: apiRoot as defined in clause 4.4 of 3GPP TS 29.501

------Skip for clarity------

N2MbsSmInfo:

description: N2 MBS Session Management information

type: object

properties:

ngapIeType:

$ref: '#/components/schemas/NgapIeType'

ngapData:

$ref: 'TS29571_CommonData.yaml#/components/schemas/RefToBinaryData'

ranId:

$ref: 'TS29571_CommonData.yaml#/components/schemas/GlobalRanNodeId'

required:

- ngapIeType

- ngapData

------Skip for clarity------

*** End of Changes ***