3GPP TSG-CT1 Meeting #122-e	C1-200830

# Electronic meeting, 20-28 February 2020                                      revision of C1-200392

| CR-Form-v12.0 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 24.501 | CR | 1893 | rev | 1 | Current version: | 16.3.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Clarification on HPLMN S-NSSAI |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | LG Electronics |  |  |  |  |  |  |  |  |  |
| Source to TSG: | C1 |  |  |  |  |  |  |  |  |  |
| Work item code: | eNS |  |  |  |  |  | Date: |  |  | 2020-02-14 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | For the Network slice-specific authentication and authorization procedure, the AMF initiates the procedure for "HPLMN S-NSSAI" according to the stage 2 and also to the clause 5.4.7.1 of TS 24.501. However it is not clear the definition of the HPLMN S-NSSAI in TS 24.501. Therefore, the HPLMN S-NSSAI can be interpreted in various ways as below. - SST and SD of the serving PLMN - Mapped HPLMN SST and mapped HPLMN SD for HPLMN of the associated with the serving PLMN - SST and SD of the serving PLMN and mapped HPLMN SST and mapped HPLMN SD of the associated with the serving PLMN Since the AMF sets the S-NSSAI IE of the NETWORK SLICE-SPECIFIC AUTHENTICATION COMMAND message to the HPLMN S-NSSAI, the correct interpretation is 2, so this needs to be clarified. |  |  |  |  |  |  |  |  |
| Summary of change: |  | In order to clarify HPLMN S-NSSAI, SST and SD descriptions are added in Table 9.11.2.8-1: S-NSSAI information element. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The incorrect UE behavior can be happened when the UE forwards wrong HPLMN S-NSSAI value to the upper layer. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 9.11.2.8 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | Change of revision 1 - Remove changes in 5.4.7.1 - Add SST and SD descriptions in coding part in table 9.11.2.8.1 : S-NSSAI information element - Add Note5 : if only HPLMN S-NSSAI is included, octets 7 to 10 shall not be included in table 9.11.2.8.1 : S-NSSAI information element - Because the phrase seems to only include when the UE is in the VPLMN, it is removed “to which SST/ST is mapped” phrase in SST/ST description in table 9.11.2.8.1,. |  |  |  |  |  |  |  |  |

***** First change *****

9.11.2.8	S-NSSAI

The purpose of the S-NSSAI information element is to identify a network slice.

The S-NSSAI information element is coded as shown in figure 9.11.2.8.1 and table 9.11.2.8.1.

The S-NSSAI is a type 4 information element with a minimum length of 3 octets and a maximum length of 10 octets.

| 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| S-NSSAI IEI |  |  |  |  |  |  |  | octet 1 |
| Length of S-NSSAI contents |  |  |  |  |  |  |  | octet 2 |
| SST |  |  |  |  |  |  |  | octet 3 |
| SD |  |  |  |  |  |  |  | octet 4* octet 6* |
| Mapped HPLMN SST |  |  |  |  |  |  |  | octet 7* |
| Mapped HPLMN SD |  |  |  |  |  |  |  | octet 8* octet 10* |

Figure 9.11.2.8.1: S-NSSAI information element

Table 9.11.2.8.1: S-NSSAI information element

| Length of S-NSSAI contents (octet 2) |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| This field indicates the length of the included S-NSSAI contents, and it can have the following values. Depending on the value of the length field the following S-NSSAI contents are included: |  |  |  |  |  |  |  |  |
| Bits |  |  |  |  |  |  |  |  |
| 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 |  |  |
| 0 | 0 | 0 | 0 | 0 | 0 | 0 | 1 | SST |  |
| 0 | 0 | 0 | 0 | 0 | 0 | 1 | 0 | SST and mapped HPLMN SST |  |
| 0 | 0 | 0 | 0 | 0 | 1 | 0 | 0 | SST and SD |  |
| 0 | 0 | 0 | 0 | 0 | 1 | 0 | 1 | SST, SD and mapped HPLMN SST |  |
| 0 | 0 | 0 | 0 | 1 | 0 | 0 | 0 | SST, SD, mapped HPLMN SST and mapped HPLMN SD |  |
| All other values are reserved. |  |  |  |  |  |  |  |  |
| Slice/service type (SST) (octet 3) |  |  |  |  |  |  |  |  |
| This field contains the 8 bit SST value. The coding of the SST value part is defined in 3GPP TS 23.003 [4]. If this IE is included during the network slice-specific authentication and authorization procedure, this field contains the 8 bit SST value of an S-NSSAI in the S-NSSAI(s) of the HPLMN. |  |  |  |  |  |  |  |  |
| Slice differentiator (SD) (octet 4 to octet 6) This field contains the 24 bit SD value. The coding of the SD value part is defined in 3GPP TS 23.003 [4]. If this IE is included during the network slice-specific authentication and authorization procedure, this field contains the 24 bit SD value of an S-NSSAI in the S-NSSAI(s) of the HPLMN. |  |  |  |  |  |  |  |  |
| If the SST encoded in octet 3 is not associated with a valid SD value, and the sender needs to include a mapped HPLMN SST (octet 7) and a mapped HPLMN SD (octets 8 to 10), then the sender shall set the SD value (octets 4 to 6) to "no SD value associated with the SST". |  |  |  |  |  |  |  |  |
| mapped HPLMN Slice/service type (SST) (octet 7) |  |  |  |  |  |  |  |  |
| This field contains the 8 bit SST value of an S-NSSAI in the S-NSSAI(s) of the HPLMN to which the SST value is mapped. The coding of the SST value part is defined in 3GPP TS 23.003 [4]. |  |  |  |  |  |  |  |  |
| mapped HPLMN Slice differentiator (SD) (octet 8 to octet 10) This field contains the 24 bit SD value of an S-NSSAI in the S-NSSAI(s) of the HPLMN to which the SD value is mapped. The coding of the SD value part is defined in 3GPP TS 23.003 [4]. |  |  |  |  |  |  |  |  |
| NOTE 1: Octet 3 shall always be included. NOTE 2: If the octet 4 is included, then octet 5 and octet 6 shall be included. NOTE 3: If the octet 7 is included, then octets 8, 9, and 10 may be included. NOTE 4: If the octet 8 is included, then octet 9 and octet 10 shall be included. NOTE 5: If only HPLMN S-NSSAI is included, then octets 7 to 10 shall not be included. |  |  |  |  |  |  |  |  |

***** End change *****