Pseudo CR 45.820 : C-UNB Introduction and Table of Content

Introduction

This document provides text for the 7.3 section for the C-UNB candidate solution in the Technical Report 45.820.

The first part of the proposed text is an introductory text, which is intended to provide the reader with an overview of the key design principles for the C-UNB candidate solution. Text provided for section 7.3.1 is based on contributions [1] and [2]. The remaining parts of the document are titles of sections that we plan to fill progressively.

Proposed text for the TR

Start of Changes

## Cooperative Ultra Narrow Band (C-UNB)

### General

C-UNB is a clean slate approach, where IoT devices are connected to the network without prior synchronization or attachment to a given base station. IoT device transmissions occur on a purely random manner and are received by all base stations in the communication range of the transmitter. Therefore, power consumption in the IoT devices occurs only during transmission or reception.

This medium access, equivalent to the well known ALOHA, may generate many radio packet collisions if the network load is high. The C-UNB radio access technology keeps the network load at very low levels with three mitigations techniques.

IoT devices transmit quite low volume of data compared to legacy GSM/GPRS devices, therefore they can afford very low row data rates. A data rate as low as 250 bps is compatible with Cellular IoT requirements. The use of a such low data rate is very interesting because it occupies an ultra narrow band spectrum, thus leaving large room for other transmission within a 200kHz block. The center frequency of such a "micro-channel" is selected randomly in a 200kHz block of the legacy GSM/GPRS. This feature creates a "frequency diversity", which helps reducing the collision of radio packets. The ultra narrow band transmission improves the MCL with limited complexity in devices.

The second feature for reducing the collision probability is the lack of time synchronization in device-triggered transmissions. Transmissions occur randomly and with no scheduling broadcasted by the network. This creates "time diversity" between IoT devices in a given area.

The third feature, on which C-UNB radio access technology is based on, is the "space diversity" given by the multiple receptions of radio packets in UL. In a given cellular network, all base stations listen for the same 200kHz block continuously and thus may receive multiple copies of the same radio packet. If an UL radio packet experiences collision or interference during reception by a given base station, the probability to have bad reception in all surrounding base stations is very limited.

The C-UNB RAT behaves quite differently compared to legacy GSM/GPRS radio access network, but it can be deployed in a GSM/GPRS infrastructure. To do so, base stations with software-defined radios implement a specific software package. The first part of this C-UNB software manages the C-UNB air interface. The second part is C-UNB agent that makes the C-UNB RAT compatible with the legacy Gb architecture, by re-routing C-UNB data streams to a C-UNB server located in the core network.

### Downlink physical design

FFS

### Uplink physical desing

FFS

### Link layer aspects

#### MAC frames in DL

FFS

#### MAC frames in UL

FFS

#### Segmentation & reassembly

FFS

### Radio Resource management

#### C-UNB architecture

FFS

#### Beacon channel

FFS

#### Cell selection

FFS

#### Paging

FFS

### Concept evaluation

FFS

End of Changes

# References

[1]	GPC150052, "Cooperative ultra narrow band technology for Cellular IoT", Sigfox, GERAN Ad'hoc meeting, 02-05 Feb'15, Sophia Antipolis, France

[2]	GP-150368, "C-UNB architecture", Sigfox, GERAN#66 meeting, 25-29 May'15, Vilnius, Lithuania