pCR for text inclusion in TR 45.820 :

C-UNB Latency Evaluation

Introduction

This document provides text on the C-UNB candidate solution for inclusion in section 7.4 of the Technical Report 45.820 revision 1.4.0.

The proposed text describes the latency evaluation of the C-UNB solution.

Proposed text for the TR

Start of Changes

## Cooperative Ultra Narrow Band

### General description

### Downlink physical layer design

### Uplink physical layer design

### Link layer aspects

### Radio resource management

### Concept evaluation

#### Latency evaluation

##### Evaluation methodology

###### Header

Annex E.2.1 hereafter defines a MAR exception report payload of 20 bytes; the protocol overhead is 29 bytes (compressed) or 65 bytes (full overhead).

In C-UNB, the MAC-PDUs have relatively small payload size, resulting in segmentation when long application data have to be transmitted. In order to reduce the amount of data carried over the air interface, the preferred implementation of the C-UNB protocol is to skip the transmission of the protocol header and let it manage by the MS-avatar hosted in the C-UNB server (see above for system architecture).

Nevertheless, this contribution evaluates latency with and without protocol header for the sake of comparison.

###### Blind repetition

The C-UNB radio access technology assumes a number of blind repetitions for each MAC-PDU. Blind repetitions helps increasing the probability of good reception but reduces the overall network capacity. MAR exception reports require a high quality of service, therefore the present contribution considers that each MAC-PDU is transmitted with three PHY-PDUs (i.e. three repetition of the same radio packet).

Figure 1 depicts the transmission timing for a MAR exception report. Upon reception of the MAR (trigger event in figure 7.4.6.1), the mobile station starts a timer equals to the beacon cycle and schedules to listen for the next occurrence of the beacon channel it knows. Following transmissions occur with Tx power set according to Tx Power adaptation procedure.

Figure 7.4.6.1: Timing for exception report transmission

###### Initial BLER evaluation

C-UNB radio access technology has three sources of BLER: propagation, noise and collision. Appropriate link budget allows keeping the related BLER under a given threshold, whereas multiple transmissions and space diversity helps to mitigate collision-related BLER.

Given the length of the PHY-PDUs in table 1, the propagation-related BLER is always lower than 3.7%, hence lower than 5.10-5 with three repetitions.

Given the three repetitions of each MAC-PDU and the multiple receptions by several base stations, the collision related BLER is less than 3.10-4.

Therefore the latency is evaluated with the assumption that the initial message BLER is ≤ 1% (according to section 5.6 hereabove).

##### Results

Table 7.4.6.1 details latency calculation for a MAR exception report. Figures are valid for all coverage classes, because initial BLER is less than 1% for all cases.

Table 7.4.6.1: latency evaluation (size in bytes and time in ms)

| Item | transmission with 65 byte protocol overhead | transmission with 29 byte protocol overhead | transmission with no protocol overhead (managed by MS-Avatar) |
| --- | --- | --- | --- |
| application data | 20 | 20 | 20 |
| protocol overhead | 65 | 29 | 0 |
| segmentation | 3x 29 | 2x 25 | 1x 20 |
| LL-PDU # and size | 3x 30 | 2x 26 | 1x 20 |
| MAC-PDU # and size | 3x 46,5 | 2x 42,5 | 1x 36,5 |
| PHY-PDU # and duration | 9x 1 488 | 6x 1 360 | 3x 1 168 |
| lead time (beacon cycle) | 1 400 | 1 400 | 1 400 |
| radio wake up time | 50 | 50 | 50 |
| gap between 2 radio packet | 25 | 25 | 25 |
| Latency | 15 042 | 9 735 | 5 014 |

We can see that the C-UNB radio access technology delivers the MAR exception reports in less than 10s when the transmission over the air interface is optimized, that is to say, when protocol overhead is either not transmitted over the air, or transmitted with compressed IP address.

When the full protocol header is transmitted over the air along with the application data, the latency for MAR exception reports is about 15 seconds.

End of Changes