# Impact on EC-CCCH in a reduced BCCH spectrum allocation

# Introduction

At GERAN#67 a new work item named Extended Coverage GSM (EC-GSM) for support of Cellular Internet of Things (WI code: CIoT_EC_GSM) was approved, see [1]. One of the objectives of the work item is the following:

“Support for extended coverage GSM deployment in a reduced BCCH spectrum allocation, provided it is shown to be feasible, from 4/12 (2.4 MHz) frequency re-use to 3/9 (1.8 MHz) or 1/3 (600 kHz) frequency re-use, where legacy CS users might not be supported, and add respective normative changes, if any.”

The BCCH layer serves many purposes, which may be negatively impacted by a tightened reuse, including the following:

- Frequency and time synchronization (FCCH, SCH)

- Cell identification (reading BSIC on SCH) and received signal level measurements e.g. for cell reselection purposes

- System information acquisition (BCCH)

- Common control signaling (RACH, PCH, AGCH)

- User data traffic

In this contribution, item 4 is investigated for EC-EGPRS devices for BCCH layer spectrum allocations using 2.4 MHz down to 600 kHz.

Item 1 is investigated in [4].

Item 5 is investigated in [5].

# Assumptions

## Link model

The link level model used in the simulator is described in [8].

## Blind transmissions

The blind transmissions used in the simulations are those derived by link level analysis in [6] and evaluated on system level in [5], see Table 1.

Table 1: Blind transmissions.

| Logical channel | Coverage Class [CC1, CC2, CC3, CC4] |
| --- | --- |
| EC-AGCH | [1,8,16,32] |
| EC-RACH | [1,4,16,48] |

## Network synchronization

The interference situation modeled by the simulation is limited to a frame and timeslot synchronized network. This means EC-AGCH / EC-RACH channels are only interfered by other EC-AGCH / EC-RACH channels from other cells in the DL and UL respectively.

## Coverage class adaptation

Coverage class adaptation has been applied as described in [7] with two failed attempts before adaptation of the coverage class is allowed. At most two increments in CC from the initially estimated class are allowed.

## BCCH Power Savings

BCCH power savings can be used to reduce interference on the BCCH frequency layer.  With tighter BCCH frequency re-use the importance of this functionality increases. BCCH power savings can be used with various levels of reduction and selections of what timeslots and channels it should be applied to. For the simulator a simple implementation for BCCH PS was used with a reduction of 6 dB for 60% of the dummy bursts transmitted on the EC-CCCH DL. I.e. no power control was applied to Immediate Assignment messages. The choice not to down-regulate all dummy bursts on the EC-CCCH is to also include a more highly loaded network where not only EC-AGCH but also EC-PCH would be transmitted (assumed to be not power regulated).

## Frequency planning

Two different frequency planning procedures have been simulated:

- Flexible planner: The frequency plan includes a flexible planner based on the gain value between cells without any restriction in frequency used by a specific cell in the cell grid, except that the planner is limited to the number of frequencies configured for the simulation. Frequencies configured investigated are 12, 11, 10, 9, 8, 7, 6, 5 and 4.

- Regular planner: Classical frequency plan with regular re-use clusters are used. Frequencies configured investigated are 12, 9, and 3.

# Simulations

## Simulation assumptions

The system level simulation assumptions in [3] have been followed. Other specific assumptions are shown in Table 2.

### System parameters

Table 2. Simulation assumptions, in addition to [3]

| Parameter | Value |
| --- | --- |
| Number of re-use clusters | 9 |
| Direction | UL and DL |
| Frequency band | 900 MHz |
| Layer | BCCH |
| Frequency re-use | Varied from 12,11,..…, 4 with frequency planner, and 12,9,3 with regular frequency planning |
| BTS antenna diversity | MRC |
| BTS output power | 43 dBm |
| Cell radius | 577.33 m |
| MTC arrival rate per cell and second | 6.8 |
| EC-RACH mapping | 2 TS, EC-RACH |
| Coverage class adaptation | See section 2.4 |
| Maximum attempts on EC-RACH per system access attempt | 6 |
| Power control, DL | 6 dB DL on 60% of dummy bursts. |
| Power control, UL | As described in [9] with target received power level of -105 dBm |
| Device output power | 23 dBm or 33 dBm |
| BPL model | Model 1, inter-site correlation 0.5 |

## Results

The results presented are:

- Resource Usage

  - This represents the average amount of bursts used per user, including all transmissions per system access attempt.

- Common control signaling delay

  - The delay includes time from initial EC-RACH transmission to a received matching Immediate Assignment.

- Failed attempts

  - This represents the percentage of the attempts that were not successful, after the maximum attempts.

- Coverage class distribution

  - This shows the % of devices ending up in different coverage classes for 33 dBm and 23 dBm devices respectively, with the coverage class thresholds used in the simulations for the respective frequency re-use factor.

### Resource Usage

#### Flexible planner

The resource usage is shown in Table 3 and Table 4 for the downlink and uplink. On the downlink, the resource usage increases from 2.3 to around 5 when the reuse is changed from 12 to 4.

Table 3: Resource Usage for the downlink and uplink, 33 dBm

| BCCH Re-use | Resource usage DL [#bursts] | Resource usage UL [#bursts] |
| --- | --- | --- |
| 12 | 2.3 | 1.1 |
| 11 | 2.4 | 1.1 |
| 10 | 2.5 | 1.1 |
| 9 | 2.6 | 1.1 |
| 8 | 2.7 | 1.1 |
| 7 | 3.0 | 1.1 |
| 6 | 3.4 | 1.1 |
| 5 | 4.1 | 1.2 |
| 4 | 4.9 | 1.2 |

Table 4: Resource Usage for the downlink and uplink, 23 dBm

| BCCH Re-use | Resource usage DL [#bursts] | Resource usage UL [#bursts] |
| --- | --- | --- |
| 12 | 2.3 | 1.8 |
| 11 | 2.3 | 1.8 |
| 10 | 2.4 | 1.8 |
| 9 | 2.5 | 1.8 |
| 8 | 2.7 | 1.8 |
| 7 | 2.9 | 1.8 |
| 6 | 3.3 | 1.8 |
| 5 | 4.1 | 1.8 |
| 4 | 5.3 | 1.9 |

For the uplink, the TS Utilization increases from 1.1. to 1.2 for 33 dBm devices and 1.8 to 1.9 for 23 dBm devices.

#### Regular planner

Similar effects as for the flexible planner is seen for the regular planner.

Table 5: Resource Usage for the downlink and uplink, 33 dBm

| BCCH Re-use | Resource usage DL [#bursts] | Resource usage UL [#bursts] |
| --- | --- | --- |
| 12 | 2.2 | 1.0 |
| 9 | 2.5 | 1.1 |
| 3 | 4.6 | 1.2 |

Table 6: Resource Usage for the downlink and uplink, 23 dBm

| BCCH Re-use | Resource usage DL [#bursts] | Resource usage UL [#bursts] |
| --- | --- | --- |
| 12 | 2.2 | 1.6 |
| 9 | 2.4 | 1.7 |
| 3 | 5.0 | 1.9 |

### Common control channel delay

#### Flexible planner

In Figure 1 and Figure 2 the common control signaling delay is shown. As expected, the delay is increased when going to a tighter re-use factor. Still, the delay figures are kept at reasonable levels. The main reason for the delay increase is the use of more aggressive CC thresholds, meaning that users will start to use blind repetitions earlier in a tight re-use scenario than a more lose re-use scenario. This is a means to keep the BLER level low enough for users in different conditions, when the system interference level is increased.

One can note the granularity of the delay distribution is 50 ms and hence the first bin in the distribution represents all access attempts ready within this time. This will mainly be represented by CC1 accesses.

Figure 1: Common Control Signaling Delay, 33 dBm

Figure 2: Common Control Signaling Delay, 23 dBm

#### Regular planner

In Figure 3 and Figure 4 the same performance is shown for the regular planner. Similar trends can be seen.

Figure 3: Common Control Signaling Delay, 33 dBm

Figure 4: Common Control Signaling Delay, 23 dBm

### Failed Attempts

#### Flexible planner

The percentage of failed attempts are extremely low in all scenarios investigated indicating that it would be possible to have a more aggressive system settings of for example CC thresholds to minimize resource usage in all scenarios.

Table 7: Failed attempts, 33 dBm

| BCCH Re-use | Failed attempts [%] |
| --- | --- |
| 12 | 0.00% |
| 11 | 0.00% |
| 10 | 0.00% |
| 9 | 0.00% |
| 8 | 0.00% |
| 7 | 0.00% |
| 6 | 0.00% |
| 5 | 0.01% |
| 4 | 0.01% |

Table 8: Failed attempts, 23 dBm

| BCCH Re-use | Failed attempts [%] |
| --- | --- |
| 12 | 0.00% |
| 11 | 0.00% |
| 10 | 0.01% |
| 9 | 0.01% |
| 8 | 0.01% |
| 7 | 0.01% |
| 6 | 0.01% |
| 5 | 0.02% |
| 4 | 0.02% |

#### Regular planner

Table 9: Failed attempts, 33 dBm

| BCCH Re-use | Failed attempts [%] |
| --- | --- |
| 12 | 0.00% |
| 9 | 0.00% |
| 3 | 0.01% |

Table 10: Failed attempts, 23 dBm

| BCCH Re-use | Failed attempts [%] |
| --- | --- |
| 12 | 0.01% |
| 9 | 0.01% |
| 3 | 0.01% |

### Coverage class distribution

The coverage class distribution for the regular planner is shown in Table 11 and Table 12.

Table 11: Coverage class distribution on UL for 33 dBm / 23 dBm [%]

| BCCH Re-use | CC1 | CC2 | CC3 | CC4 |
| --- | --- | --- | --- | --- |
| 12 | 99.5 / 94.6 | 0.4 / 4.0 | 0.1 / 0.8 | <0.1 / 0.7 |
| 9 | 99.5 / 94.0 | 0.5 / 4.4 | 0.1 / 0.9 | <0.1 / 0.7 |
| 3 | 99.3 / 93.1 | 0.6 / 4.9 | 0.1 / 1.0 | <0.1 / 1.0 |

Table 12: Coverage class distribution on DL for 33 dBm / 23 dBm

| BCCH Re-use | CC1 | CC2 | CC3 | CC4 |
| --- | --- | --- | --- | --- |
| 12 | 98.7 / 98.8 | 1.2 / 1.1 | 0.1 / 0.1 | <0.1 / <0.1 |
| 9 | 97.6 / 97.7 | 2.1 / 2.1 | 0.2 / 0.2 | <0.1 / <0.1 |
| 3 | 87.7 / 86.4 | 9.6 / 10.7 | 2.0 / 2.2 | 0.7 / 0.7 |

# Discussion

The paper has investigated the performance of the EC-CCCH in a tight BCCH re-use scenario. Frequency re-use factors from 12 to 3 has been investigated using either what is called a “regular planning”, reflecting the classical cell grid of repeated re-use clusters, and what is referred to as a “flexible planner”, with full freedom to plan the frequencies. The same system deployment, site placement, etc. is assumed in both cases.

One can note that the regular planner is performing better than the flexible planner, indicating that the flexible planner has room for improvements. Hence, the flexible planner results should rather be seen as a means to illustrate the relative impact to the system when gradually reducing the BCCH spectrum allocation. Differences seen between the regular planner and the flexible planner are not of interest for this investigation.

The results should be considered as preliminary. More careful review will be done of the results before the GERAN#69 submission. One can note from the results that the failed rate is extremely low, indicating that a more aggressive system setting in specifically the CC thresholds could be applied resulting in less resources used by the EC-CCCH.

BCCH power savings has been applied, but only on dummy bursts transmitted on the EC-CCCH, and down-regulation has only been allowed in 60% of the bursts. This is to model a higher load on the EC-CCCH, considering also for example EC-PCH traffic would be present in a real network deployment.

It should further be noted that the simulations have assumed a frame and timeslot synchronized network meaning that only EC-CCCH wanted and interferer signals are modeled. On the UL, this will reflect a beneficial interference scenario with only EC-RACH interference. Considering that the EC-RACH load is reasonably low, so will also the interference be.

# Conclusions

The paper has investigated the impact on the EC-CCCH in a tight BCCH spectrum. The results should be considered preliminary, but all in all, the results are encouraging showing extremely low failed rates even in a very tight re-use pattern. The resource usage is increased by roughly 120% when going from 12 to 3 in frequency re-use. The overall common control signaling delay is increased, as expected, but still the 99 percentile is around 1-1.5 sec for the lowest re-use factor.

# References

- GP-151039, “New Work Item on Extended Coverage GSM (EC-GSM) for support of Cellular Internet of Things”, source Ericsson LM, Intel, Gemalto N.V., MediaTek Inc., TeliaSonera AB, Sierra Wireless S.A., Telit Communications S.p.A., ORANGE, Nokia Networks, Alcatel-Lucent. GERAN#67.

- GP-151124, ”GPRS synchronization performance in a tight frequency reuse scenario”, source Ericsson LM. GERAN#68.

- GP-151104, ”Intended scope for reduced spectrum allocation on BCCH evaluation (update of GPE150043)”, source Ericsson LM. GERAN#68.

- GP-160042, “Impact on EC-EGPRS synchronization performance in reduced spectrum allocation”, source Ericsson LM. GERAN #69.

- GP-160026, “Dependency on number of coverage classes for EC-EGPRS on system capacity (update of GP-151125)”, source Ericsson LM. GERAN Telco#4 on EC-GSM and eDRX

- GP-160030, “Alignment of Coverage Classes for EC-EGPRS”, source Ericsson LM. GERAN#69

- GP-160035, “Coverage Class Adaptation on EC-RACH”, source Ericsson LM. GERAN#69.

- GP-150435, “EC-GSM, Link modelling methodology for EC-RACH capacity evaluations (update of GPC150193)”, source Ericsson LM. GERAN#66

- GP-151126, System impact from power control settings on EC-RACH, source Ericsson LM. GERAN#68

- GP-150762, “EC-GSM, Link modeling methodology for EC-PDTCH capacity evaluations (update of GPC150441)”, source Ericsson LM. GERAN#67.

- GP-151183, “Impact on legacy services in a reduced BCCH spectrum allocation”, source Ericsson LM. GERAN#68.