# Pseudo CR 45.820 – Narrowband LTE Cell Search Performance Evaluation

# 1	Introduction

## 1.1	Background Information

A study on Cellular System Support for Ultra Low Complexity and Low Throughput Internet of Things was approved at GERAN#62.

The study allows both for an evolution of GSM, to comply with the objectives of the study, and non-backwards compatible solutions by a new system design.

## 1.2	Reason for change

In GERAN#67, a new system design, i.e., NB LTE, based on the current LTE system but using only 200 kHz bandwidth is proposed in GP-150779. This document contains the cell search performance evaluation of NB LTE.

## 1.3	Summary of change

The cell search evaluation of NB LTE.

# pCR to 3GPP TR 45.820-v1.4.0

### 7.4.6 Concept evaluation

#### 7.4.6.1 Coverage evaluation

##### 7.4.6.1.1 Cell Search and Network synchronization

##### 7.4.6.1.1.1 Simulation settings

Two cases, i.e., initial cell search, and cell-reconfirmation (non-initial cell search), are evaluated.  For each of the cases, three different scenarios are investigated, corresponding to the presence of 0, 1 or 2 interferers. All the interferers are assumed to be transmitting at the same power level. The results are provided for these scenarios at three different MCLs corresponding to 164 dB, 154 dB and 144 dB. The simulation assumptions are taken from Annex C and tabulated in Table 7.4.6.1.1-1.

Table 7.4.6.1.1-1 Simulation Parameters

| Parameter | Value |
| --- | --- |
| Carrier Frequency | 900 MHz |
| Channel Model | TU 1 Hz |
| Subcarrier Spacing | 15 kHz |
| SNR | -4.6 dB, 5.4 dB and 15.4 dB1 |
| Sampling Frequency (Fs) | 1.92 MHz |
| Cyclic Prefix | 10 samples for the 1st and 7th OFDM symbol within a subframe, 9 samples for the rest |
| Timing offset | Uniformly distributed between [0,15) ms in steps of 1/Fs |
| Antenna Configuration | 1 Tx, 1 Rx |
| Frequency Offset | Randomly generated as one of the values {-18 kHz, 18 kHz} |
| Number of realizations | 1000 |
| Max. M-frames used for detection | 40 |
| NOTE1: -4.6 dB corresponds to an MCL of 164 dB. |  |

##### 7.4.6.1.1.2 Simulation results

###### 7.4.6.1.1.2.1  Initial cell search performance

In the initial cell search no time reference is present and the frequency offset is assumed to be 18 kHz. The device will search for all possible cells at the chosen frequency. In order to limit the search time, a successful detection corresponds to the case where the correlation peak from the PSS exceeds the average correlation by a certain threshold. The detection probabilities are provided in Table 7.4.6.1.1-2 for different MCLs and varying number of interferers. Irrespective of the number of interferers, or the investigated coupling loss, the detection probability is 100%.

Table 7.4.6.1.1-2: Detection probability for initial cell search

| Number of Interferers | Detection Probability at different coupling loss |  |  |
| --- | --- | --- | --- |
|  | 164 dB | 154 dB | 144 dB |
| 0 | 100 % | 100 % | 100 % |
| 1 | 100 % | 100 % | 100 % |
| 2 | 100 % | 100 % | 100 % |

The CDF of the synchronization times for timing offset estimation are provided in Figure 7.4.6.1.1-1, for 164 dB, 154 dB and 144 dB MCL respectively. Note that the synchronization time is given in terms of multiples of 15 ms intervals, which is the repetition interval of the PSS. We observe that at higher coupling loss (lower SNR), the performance of synchronization is worst in the presence of no interferers in the system, and is best when equally strong signals from 3 BSs are simultaneously present. The reason is because in the presence of three cells, the mobile station has an improved chance of getting a good signal from one of the three transmitting cells as opposed to only one cell, resulting in a faster synchronization. On moving to lower MCL (higher SNR), this effect is somewhat reduced because the gains due to macro diversity are offset due to the presence of interfering signals from other cells.

| - MCL = 164 dB | - MCL = 154 dB | - MCL = 144 dB |
| --- | --- | --- |

Figure 7.4.6.1.1-1: CDF of the time required for signal detection at different MCL values for initial cell search.

The timing offset is estimated at the time the signal is detected and requires accumulation of the correlation over multiple 15 ms frames (A 15 ms frame consists of 15 subframes). The CDFs of the residual timing estimation error are provided in Figure 7.4.6.1.1-2, for the three MCLs of 164 dB, 154 dB and 144 dB, respectively. We observe that the timing estimation error is within the length of the cyclic prefix for all the realizations.

| - MCL = 164 dB | - MCL = 154 dB | - MCL = 144 dB |
| --- | --- | --- |

Figure 7.4.6.1.1-2: CDF of the timing estimation error after signal detection at different MCL values for initial cell search.

After the timing offset has been found, an accumulation over multiple 15 ms frames is again required for accurate frequency offset estimation. The CDFs of the time required for frequency offset estimation are provided in Figure 7.4.6.1.1-3 for MCLs of 164 dB, 154 dB and 144 dB, respectively.

| - MCL = 164 dB | - MCL = 154 dB | - MCL = 144 dB |
| --- | --- | --- |

Figure 7.4.6.1.1-3: CDF of the time required for frequency offset estimation at different MCL values for initial cell search.

The CDFs for the residual frequency offset estimation error are provided in Figure 7.4.6.1.1-4 for the different MCL values. We observe that the frequency offset estimation error is within +/- 50 Hz for 99 % of the realizations.

| - MCL = 164 dB | - MCL = 154 dB | - MCL = 144 dB |
| --- | --- | --- |

Figure 7.4.6.1.1-4: CDF of the frequency offset estimation error at different MCL values for initial cell search.

The cell ID detection as well as the M-frame timing is performed using the SSS after the subframe timing and frequency offsets are known and compensated for. The corresponding CDFs of the synchronization time for cell ID detection are provided in Figure 7.4.6.1.1-5, for three different MCL values of 164 dB, 154 dB and 144 dB respectively. Note that a successful detection of the cell ID also results in the correct M-frame timing, because of the inherent design of the SSS.

| - MCL 164 dB | - MCL 154 dB | - MCL 144 dB |
| --- | --- | --- |

Figure 7.4.6.1.1-5: CDF of the time required for cell ID detection at different MCL values for initial cell search.

A comparison of the total synchronization time for initial cell search for a desired percentage of the mobile stations is provided in Table 7.4.6.1.1-3, Table 7.4.6.1.1-4 and Table 7.4.6.1.1-5 for the three different MCL values. “Network Synchronization time” refers to the total time required for successful signal detection (including detection of frame timing), correction of frequency offset and detection of cell ID. In obtaining the total synchronization time, we assume that the frequency offset estimation procedure is started during the last 15 ms frame used for signal detection, and the cell ID detection procedure is started in the last 15 ms frame the frequency offset is correctly estimated.

Table 7.4.6.1.1-3: Comparison of network synchronization time (in ms) for initial cell search at 164 dB MCL

| Synchronization Time | Number of Interferers |  |  |
| --- | --- | --- | --- |
|  | 0 | 1 | 2 |
| 50 % | 135 | 90 | 75 |
| 90 % | 570 | 390 | 390 |
| 99 % | 1245 | 1470 | 1110 |
| Average | 236.57 | 167.48 | 153.92 |

Table 7.4.6.1.1-4: Comparison of network synchronization time (in ms) for initial cell search at 154 dB MCL

| Synchronization Time | Number of Interferers |  |  |
| --- | --- | --- | --- |
|  | 0 | 1 | 2 |
| 50 % | 30 | 30 | 30 |
| 90 % | 120 | 105 | 120 |
| 99 % | 465 | 795 | 1035 |
| Average | 53.23 | 61.08 | 71.46 |

Table 7.4.6.1.1-5: Comparison of network synchronization time (in ms) for initial cell search at 144 dB MCL

| Synchronization Time | Number of Interferers |  |  |
| --- | --- | --- | --- |
|  | 0 | 1 | 2 |
| 50 % | 15 | 15 | 30 |
| 90 % | 30 | 75 | 90 |
| 99 % | 75 | 405 | 630 |
| Average | 19.28 | 41.49 | 57.84 |

###### 7.4.6.1.1.2.2  Non-Initial cell search performance

During non-initial cell search, the mobile station needs to reconfirm to a particular cell that it was previously connected to. The frequency offset during non-initial cell search is taken to be 2 ppm. The detection probabilities are provided in Table 7.4.6.1.1-6 for different MCLs and varying number of interferers.

Table 7.4.6.1.1-6: Comparison of the detection probability for non-initial cell search

| Number of Interferers | Detection Probability |  |  |
| --- | --- | --- | --- |
|  | 164 dB MCL | 154 dB MCL | 144 dB MCL |
| 0 | 100 % | 99.8 % | 98.9 % |
| 1 | 100 % | 99.8 % | 99.2 % |
| 2 | 100 % | 99.8 % | 99.2 % |

The CDF of the synchronization times for timing offset estimation are provided in Figure 7.4.6.1.1-6 for 164 dB, 154 dB and 144 dB MCL, respectively. As before, the synchronization time is given in terms of multiples 15 ms intervals, which is the repetition interval of the PSS.

In these and subsequent comparisons, the realizations for which there was no detection are excluded. For these excluded realizations, the signal was not detected even after 40 M-frames. We observe that the performance of synchronization is worst in the presence of multiple interferers in the system. The reason is because in the presence of three cells, more interference is present from the neighbouring two transmitting cells as opposed to only one cell, resulting in a slower synchronization.

| - MCL = 164 dB | - MCL = 154 dB | - MCL = 144 dB |
| --- | --- | --- |

Figure 7.4.6.1.1-6: CDF of the time required for signal detection at different MCL values for non-initial cell search.

The CDFs of the residual timing estimation error are provided in Figure 7.4.6.1.1-7 for the different MCL values.

| - MCL = 164 dB | - MCL = 154 dB | - MCL = 144 dB |
| --- | --- | --- |

Figure 7.4.6.1.1-7: CDF of the timing estimation error after signal detection at different MCL values for non-initial cell search

After the timing offset has been found, the frequency offset is estimated in a similar fashion as in the initial cell search procedure. The CDFs of the time required for frequency offset estimation are provided in Figure 7.4.6.1.1-8 at MCLs of 164 dB, 154 dB and 144 dB, respectively.

| - MCL = 164 dB | - MCL = 154 dB | - MCL = 144 dB |
| --- | --- | --- |

Figure 7.4.6.1.1-8: CDF of the time required for frequency offset estimation at different MCL values for non-initial cell search

The CDFs of the residual frequency offset estimation error are provided in Figure 7.4.6.1.1-9 for the different MCL values.

| - MCL = 164 dB | - MCL = 154 dB | - MCL = 144 dB |
| --- | --- | --- |

Figure 7.4.6.1.1-9: CDF of the frequency offset estimation error at different MCL values for non-initial cell search

The cell ID detection as well as the M-frame timing estimation is performed using the SSS after the subframe timing and frequency offsets are known and compensated for. The corresponding CDFs of the synchronization time for cell ID detection are provided in Figure 7.4.6.1.1-10 respectively for the three different MCLs considered.

| - MCL = 164 dB | - MCL = 154 dB | - MCL = 144 dB |
| --- | --- | --- |

Figure 7.4.6.1.1-10: CDF of the time required for cell ID detection at different MCL values for non-initial cell search

A comparison of the total synchronization time for non-initial cell search for a desired percentage of the mobile stations is provided in Table 7.4.6.1.1-7, Table 7.4.6.1.1-8 and Table 7.4.6.1.1-9 for the three different MCL values.

Table 7.4.6.1.1-7: Comparison of network synchronization time (in ms) for initial cell search at 164 dB MCL

| Synchronization Time | Number of Interferers |  |  |
| --- | --- | --- | --- |
|  | 0 | 1 | 2 |
| 50 % | 150 | 165 | 210 |
| 90 % | 630 | 690 | 1005 |
| 99 % | 1410 | 1815 | 2165 |
| Average | 255.99 | 292.42 | 392.67 |

Table 7.4.6.1.1-8: Comparison of network synchronization time (in ms) for initial cell search at 154 dB MCL

| Synchronization Time | Number of Interferers |  |  |
| --- | --- | --- | --- |
|  | 0 | 1 | 2 |
| 50 % | 30 | 60 | 90 |
| 90 % | 120 | 420 | 540 |
| 99 % | 495 | 1335 | 1665 |
| Average | 59.41 | 157.19 | 215.19 |

Table 7.4.6.1.1-9: Comparison of network synchronization time (in ms) for initial cell search at 144 dB MCL

| Synchronization Time | Number of Interferers |  |  |
| --- | --- | --- | --- |
|  | 0 | 1 | 2 |
| 50 % | 15 | 45 | 75 |
| 90 % | 30 | 315 | 495 |
| 99 % | 135 | 1275 | 1725 |
| Average | 22.37 | 123.67 | 195.74 |