3GPP TR 21.916 V0.6.0 (2020-09)

Technical Report

3rd Generation Partnership Project;

Technical Specification Group Services and System Aspects;

Release 16 Description;

Summary of Rel-16 Work Items

(Release 16)

The present document has been developed within the 3rd Generation Partnership Project (3GPP TM) and may be further elaborated for the purposes of 3GPP.
The present document has not been subject to any approval process by the 3GPP Organizational Partners and shall not be implemented.
This Report is provided for future development work within 3GPP only. The Organizational Partners accept no liability for any use of this Specification.
Specifications and Reports for implementation of the 3GPP TM system should be obtained via the 3GPP Organizational Partners' Publications Offices.

Keywords

Description, Summary, Release 16

3GPP

Postal address

3GPP support office address

650 Route des Lucioles - Sophia Antipolis

Valbonne - FRANCE

Tel.: +33 4 92 94 42 00 Fax: +33 4 93 65 47 16

Internet

http://www.3gpp.org

Copyright Notification

No part may be reproduced except as authorized by written permission.
The copyright and the foregoing restriction extend to reproduction in all media.

© 2020, 3GPP Organizational Partners (ARIB, ATIS, CCSA, ETSI, TSDSI, TTA, TTC).

All rights reserved.

UMTS™ is a Trade Mark of ETSI registered for the benefit of its members

3GPP™ is a Trade Mark of ETSI registered for the benefit of its Members and of the 3GPP Organizational Partners
LTE™ is a Trade Mark of ETSI registered for the benefit of its Members and of the 3GPP Organizational Partners

GSM® and the GSM logo are registered and owned by the GSM Association

Contents

Foreword	6

Introduction	6

1	Scope	7

2	References	7

3	Definitions of terms, symbols and abbreviations	7

3.1	Terms	7

3.2	Symbols	7

3.3	 Abbreviations	7

4	Rel-16 Executive Summary	8

5	Enhancement of Ultra-Reliable (UR) Low Latency Communications (URLLC)	8

5.1	Enhancement of URLLC support in the 5G Core network	8

5.2	Physical Layer Enhancements for NR Ultra-Reliable and Low Latency Communication (URLLC)	9

5.3	Support of NR Industrial Internet of Things	11

6	Support of LAN-type services	12

6.1	LAN support in 5G	12

6.2	5GS Enhanced support of Vertical and LAN Services	12

7	Cellular IoT	13

7.1	Cellular IoT support and evolution for the 5G System	13

7.2	Additional enhancements for NB-IoT	16

7.3	Additional MTC enhancements for LTE	18

8	Advanced V2X support	21

8.1	Improvement of V2X service Handling	21

8.2	Architecture enhancements for 3GPP support of advanced V2X services	21

8.3	Application layer support for V2X services	23

8.4	5G V2X with NR sidelink	23

9	Northbound APIs related items	26

9.1	Usage of CAPIF for xMB API	26

9.2	Enhancement of 3GPP Northbound APIs	26

9.3	Enhancements for Common API Framework for 3GPP Northbound APIs	26

9.4	Service Enabler Architecture Layer for Verticals	26

9.5	Other APIs-related items	26

10	Coexistence with Non-3GPP systems	27

10.1	Wireless and Wireline Convergence Enhancement	27

10.2	Access Traffic Steering, Switch and Splitting support in the 5G system architecture	32

11	Railways and Maritime	35

11.1	Mobile Communication System for Railways 2	35

11.2	Further performance enhancement for LTE in high speed scenario	35

11.3	NR support for high speed train scenario	36

11.4	Maritime Communication Services over 3GPP System	37

12	Mission Critical, Public Warning	38

12.1	Enhancements of Public Warning System	38

12.2	MBMS APIs for Mission Critical Services	39

12.3	Other Mission critical improvements	39

13	Conversational services, Streaming and TV	40

13.1	Conversational services	40

13.1.1	Coverage and Handoff Enhancements for Multimedia	40

13.1.2	Single radio voice continuity from 5GS to 3G	41

13.1.3	Volume Based Charging Aspects for VoLTE	43

13.1.4	EVS Floating-point Conformance for Non Bit-Exact	44

13.1.5	Media Handling Extensions for 5G Conversational Services	45

13.1.6	VR QoE metrics	46

13.1.7	Media Handling Aspects of RAN Delay Budget Reporting in MTSI	46

13.2	Streaming	46

13.2.1	Enhancement of LTE for Efficient delivery of Streaming Service	46

13.2.2	Enhancements to Framework for Live Uplink Streaming	47

13.2.3	Media streaming architecture	47

14	5G Location and Positioning Services	47

14.1	Stage 1 of 5G_HYPOS	47

14.2	Enhancement to the 5GC LoCation Services	47

14.3	NR positioning support	48

15	User Identities, Authentication, multi-device	48

15.1	User Identities and Authentication	48

15.1.2	Multi-device and multi-identity	49

16	Slicing	49

16.1	Enhancement of Network Slicing	49

16.2	Enhancement of 3GPP management system for multiple tenant environment support	51

16.3	Business Role Models for Network Slicing	51

16.4	Enhancement of performance assurance for 5G networks including network slicing	54

17	UE radio capability signalling optimization	54

17.1	Optimisations on UE radio capability signalling	54

18	Other system-wide Features	56

18.1	Enablers for Network Automation Architecture for 5G	56

18.2	Provision of Access to Restricted Local Operator Services by Unauthenticated UEs	58

18.3	Enhancing Topology of SMF and UPF in 5G Networks	59

18.4	Private Network Support for NG-RAN	61

18.5	Service-Based Architecture	63

18.5.1	Enhancements to the Service-Based 5G System Architecture	63

18.5.2	SBA aspects of enhanced IMS to 5GC integration	63

18.6	User data interworking, Coexistence and Migration	63

19	Radio Features	64

19.1	NR-related Release 16 Features	64

19.1.1	NR-based access to unlicensed spectrum	64

19.1.2	2-step RACH for NR	64

19.1.3	UE Power Saving in NR	65

19.1.4	Dual Connectivity (EN-DC) with 3 bands DL and 3 bands UL	67

19.1.5	NR mobility enhancements	68

19.1.6	Rel-16 NR inter-band CA/Dual Connectivity for 2 bands DL with x bands UL (x=1, 2)	70

19.1.7	Rel16 NR inter-band Carrier Aggregation for 3 bands DL with 1 band UL	71

19.1.8	Add support of NR DL 256QAM for frequency range 2 (FR2)	72

19.1.9	SON (Self-Organising Networks) and MDT (Minimization of Drive Tests) support for NR	72

19.1.10	Introduction of NR FDD bands with variable duplex and corresponding framework	75

19.1.11	Cross Link Interference (CLI) handling and Remote Interference Management (RIM) for NR	76

19.2	Release 16 Features impacting both LTE and NR	78

19.2.1	Transfer of Iuant interface specifications from 25-series to 37-series	78

19.2.2	Introduction of GSM, UTRA, E-UTRA and NR capability set(s) (CS(s)) to the multi-standard radio (MSR) specifications	79

19.2.3	Direct data forwarding between NG-RAN and E-UTRAN nodes for inter-system mobility	79

19.2.4	eNB(s) Architecture Evolution for E-UTRAN and NG-RAN	80

19.2.5	High power UE (power class 2) for EN-DC (1 LTE TDD band + 1 NR TDD band)	82

19.3	LTE-related Release 16 Features	82

19.3.1	LTE-based 5G terrestrial broadcast	82

19.3.2	Support for NavIC Navigation Satellite System for LTE	84

19.3.3	Even further mobility enhancement in E-UTRAN	84

19.3.4	DL MIMO efficiency enhancements for LTE	85

19.3.5	Other LTE-only items	86

20	All other Release 16 Features	86

20.1	Service Interactivity	86

20.2	RTCP Verification for Real-Time Services	88

20.3	Stage-3 SAE Protocol Development for Rel16	89

20.4	Reliable Data Service Serialization Indication	89

20.5	Shared Data Handling on Nudm and Nudr	89

20.6	New Services and Markets Technology Enablers – Phase 2	90

20.7	Ambient noise test methodology for evaluation of acoustic UE performance	91

20.8	KPI reporting	91

21	Telecom Management	92

21.1	Network and Service Management	92

21.1.1	5G Management capabilities	92

21.1.2	Energy Efficiency of 5G	92

21.1.3	OAM aspects of LTE and WLAN integration	93

21.1.4	Methodology for 5G management specifications	93

21.1.5	Closed loop SLS Assurance	94

21.1.6	Trace Management in the context of Services Based Management Architecture and Streaming Trace reporting	94

21.2	Charging Management	95

21.2.1	Charging Enhancement of 5GC interworking with EPC	95

21.3	Other charging and management items	96

22	Other items	96

22.1	Items not (fully) completed in Rel-16	96

22.1.1	Remote Identification of Unmanned Aerial Systems	96

22.1.2	5G message service	96

22.1.3	Integration of Satellite Access in 5G	96

Annex A1:	Template for contributing	105

Annex B:	Process to get further information	107

B.1	General	107

B.2	Direct method	108

B.3	Step by step method	108

Annex C:	Change history	111

# Foreword

This Technical Report has been produced by the 3rd Generation Partnership Project (3GPP).

The contents of the present document are subject to continuing work within the TSG and may change following formal TSG approval. Should the TSG modify the contents of the present document, it will be re-released by the TSG with an identifying change of release date and an increase in version number as follows:

Version x.y.z

where:

x	the first digit:

1	presented to TSG for information;

2	presented to TSG for approval;

3	or greater indicates TSG approved document under change control.

y	the second digit is incremented for all changes of substance, i.e. technical enhancements, corrections, updates, etc.

z	the third digit is incremented when editorial only changes have been incorporated in the document.

# Introduction

The present document provides a summary of each and every 3GPP Release 16 Feature, and more generally of all Work Items for which a summary has been agreed to be provided.

These summaries are based on the inputs issued by the Work Item Rapporteurs, slightly rewritten by the TR Rapporteur to ensure overall consistency. The original Work Item Rapporteur inputs can be retrieved as temporary document (tdoc), as stated in the first sentence of each clause.

Editor’s note: INPUTS ARE EXPECTED from rapporteurs. The list of expected contributors is provided in “Annex B: List of contributors”, in the cell highlighted in yellow. Annex A provides the expected rapporteur in term of Working Group. E.g. if “SA2” is indicated, it means: “The SA2 WID rapporteur is expected to provide the summary for the full feature”.
The name of the rapporteur is provided in “Annex C: Full list of Rel-16 Work Items”, in the column “WI_rapporteur_e_mail”.

Editor’s note: to the contributors: 
The template to be used for the contribution is provided in “Annex A: Template for contributing”. The input type in 3GU is “WI Summary”.
The target audience of this document are readers inside and outside the 3GPP community. Contributions should be written as to be understandable by non-specialist. 
The summary should reflect the actual outcome of the Feature - and not the initial intentions given in the WID.
An exhaustive list of all the CRs and TS/TRs is not to be provided in the summary (instead, the procedure on how to retrieve this information is given in section 4). Only the essential CRs and TS/TRs can be mentioned in the Reference. 
When the usual Stage 1/Stage 2/Stage 3 process is not followed -and in particular if a full stage is missing- explain why.

Editor’s note: on the progress of this document: 
All inputs received up to TSG#88e included. The list of the still-missing inputs is provided at the end of this document.
Lot of editing work, including formatting, still needed.
References to CRs to be replaced by URL to 3GU, e.g. :

List of related CRs: select "TSG Status = Approved" in  https://portal.3gpp.org/ChangeRequests.aspx?q=1&workitem=840192,840292

# 1	Scope

The present document provides a summary of each Release 16 Feature or, whenever needed, of each significant Work Item.

The information provided in the present document is limited to an overview of each Feature, explaining briefly its purpose and the main lines of the system's behaviour to execute the Feature.

More information is available by consulting the 3GPP Ultimate web site, as explained in "Annex C: Process to get further information".

The present document presents the "initial state" of the Features introduced in Release 16, i.e. as they are by the time of publication of the present document. Each Feature is subject to be later modified or enhanced, over several years, by the means of Change Requests (CRs). It is therefore recommended to retrieve all the CRs which relate to the given Feature, as explained in Annex C, to further outline a feature at a given time.

# 2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	TR 21.905: "Vocabulary for 3GPP Specifications".

NOTE:	Due to the specificity of the present document, consisting in a collection of independent summaries, the references are given at the end of each clause rather than in this clause.

# 3	Definitions of terms, symbols and abbreviations

## 3.1	Terms

For the purposes of the present document, the terms given in TR 21.905 [1] and the following apply. A term defined in the present document takes precedence over the definition of the same term, if any, in TR 21.905 [1].

## 3.2	Symbols

Void.

## 3.3		Abbreviations

For the purposes of the present document, the abbreviations given in TR 21.905 [1] and the following apply. 
An abbreviation defined in the present document takes precedence over the definition of the same abbreviation, if any, in TR 21.905 [1]. Abbreviations specific to a given clause are provided in the clause they appear.

DL	Downlink (network to mobile)

IoT 	Internet of Things

KPI	Key Performance Indicator

NB	Narrowband

NR	New Radio (the radio interface of 5G)

Rel	3GPP Release

UL	Uplink (mobile to network)

# 4	Rel-16 Executive Summary

Editor's note: To be provided later.

# 5	Enhancement of Ultra-Reliable (UR) and Low Latency Communications (URLLC)

## 5.1	Enhancement of URLLC support in the 5G Core network

| 830098 | Enhancement of URLLC support in the 5G Core network | 5G_URLLC | S2 | SP-181122 | Hui Ni, Huawei |
| --- | --- | --- | --- | --- | --- |
| 790008 | Study on enhancement of URLLC supporting in 5GC | FS_5G_URLLC | S2 | SP-180118 | Hui Ni, Huawei |
| 810036 | Study on the security of URLLC for 5GS | FS_5G_URLLC_SEC | S3 | SP-180910 | Rong Wu, Huawei Technologies |
| 820019 | Stage 2 of 5G_URLLC | 5G_URLLC | S2 | SP-181122 | Hui Ni, Huawei |
| 840026 | Security of URLLC for 5GS | 5G_URLLC_SEC | S3 | SP-190351 | Rong Wu, Huawei Technologies |
| 840005 | CT Aspects of 5G_URLLC | 5G_URLLC | ct | CP-191063 | Qi Caixia, Huawei |
| 840060 | CT3 Aspects of 5G_URLLC | 5G_URLLC | C3 | CP-191063 | Qi Caixia, Huawei |
| 840061 | CT4 Aspects of 5G_URLLC | 5G_URLLC | C4 | CP-191063 | Qi Caixia, Huawei |
| 780030 | Stopped - Study on EPC support for Mobility with Low Latency Communication | FS_LLC_Mob | S2 | SP-171069 | Chris PUDNEY |
| 790009 | Stopped - Study on enht of systems using EPS for UR and Availability using commodity equipment | FS_EPS_URACE | S2 | SP-180119 | Chris Pudney |

Summary based on the input provided by Huawei in SP-200295.

This Feature enhances 5G core network to support ultra-high reliability and low-latency communications. The main functionalities introduced here are the support of redundant transmission, QoS monitoring, dynamic division of the Packet Delay Budget, and enhancements of the session continuity mechanism.

Redundant transmission for high-reliability communication

Some URLLC services request very high reliability that hardly can be supported in an economical way by a single transport path. To support such services, a redundant transmission mechanism is specified. User packets are duplicated and simultaneously transferred to the receiver via two disjoint user plane paths. The redundant packets are then eliminated at the receiver side. With this, service failure can be avoided even in case the packet transmission via one path occasionally fails or exceeds the delay requirement.

Different options are specified to support redundant transmission at different layers:

-	Dual-connectivity-based end-to-end redundant user plane paths. Two redundant PDU Sessions with independent user plane paths are established between UE and DN . Packet replication and elimination are performed by the upper layer of UE and DN, which are not specified in 3GPP.

-	Support of redundant transmission on N3/N9 interfaces. For a PDU Session used for URLLC services, two redundant N3/N9 tunnels with independent user plane paths are established between UPF and NG-RAN to transfer the duplicated user packets. Packet replication and elimination are performed by NG-RAN and UPF.

-	Support of redundant transmission at transport layer. This approach assumes that the backhaul provides two disjoint transport paths between UPF and RAN. The redundancy functionality within NG-RAN and UPF makes use of the independent paths at transport layer, which is not specified in 3GPP.

QoS Monitoring

QoS Monitoring is defined in this release for the measurement of packet delay between UE and PSA UPF. The NG-RAN is required to provide the QoS Monitoring of UL/DL packet delay at the Uu interface. The QoS Monitoring of UL/DL packet delay between NG-RAN and PSA UPF can be performed at different levels of granularities, i.e. per QoS flow level, or per GTP-U path level.

Dynamic division of Packet Delay Budget

The Packet Delay Budget (PDB) of URLLC services is typically more stringent than for traditional services. To obtain a more accurate delay budget for NG-RAN, SA WG2 decided to allow a dynamic value for the core network PDB (CN PDB), so that the SMF or NG-RAN can dynamically calculate delay budget of NG-RAN based on the CN PDB.

Enhancements of session continuity

When a UE moves, the user plane path of low latency services need to be optimized to reduce the latency and to guarantee session continuity. Several related mechanisms are developed or enhanced in this WI.

PSA relocation for Ethernet PDU Session is specified in this release. The target UPF will assist in the update of Ethernet forwarding tables of Ethernet switches in the DN, so that UL/DL traffics will switch to the target UPF once the UE moves.

For ULCL relocation, a forwarding tunnel between the old and new UL CLs is introduced to avoid packet loss during relocation.

AF-influenced traffic routing mechanism is further enhanced to allow flexible coordination between SMF and AF when user plane change events happen.

References

The redundant transmission and QoS monitoring mechanisms are specified in TS 23.501[1], Clause 5.33. Dynamic division of Packet Delay Budget is specified in TS 23.501 [1], Clause 5.7.3.4. Enhancements of session continuity mechanisms are mainly in TS 23.501 [1], Clause 5.6.7, and TS 23.502 [2], Clause 4.3.5 and Clause 4.3.6.

[1]	3GPP TS 23.501: "System Architecture for the 5G System; Stage 2".

[2]	3GPP TS 23.502: "Procedures for the 5G System; Stage 2".

[3]	3GPP TR 23.725: "Study on enhancement of Ultra-Reliable Low-Latency Communication (URLLC) support in the 5G Core network"

## 5.2	Physical Layer Enhancements for NR Ultra-Reliable and Low Latency Communication (URLLC)

| 830074 | Physical Layer Enhancements for NR Ultra-Reliable and Low Latency Communication (URLLC) | NR_L1enh_URLLC | R1 | RP-191584 | Huawei |
| --- | --- | --- | --- | --- | --- |
| 800095 | Study on physical layer enhancements for NR UR Low Latency Cases | FS_NR_L1enh_URLLC | R1 | RP-181477 | Huawei |
| 830174 | Core part: Physical Layer Enhancements for NR Ultra-Reliable and Low Latency Communication (URLLC) | NR_L1enh_URLLC-Core | R1 | RP-190726 | Huawei |
| 830274 | Perf. part: Physical Layer Enhancements for NR Ultra-Reliable and Low Latency Communication (URLLC) | NR_L1enh_URLLC-Perf | R4 | RP-190726 | Huawei |

Summary based on the input provided by Huawei in RP-200887.

In Release 15 the basic support for URLLC was introduced with TTI structures for low latency as well as methods for improved reliability. Use cases with tighter requirements, e.g. higher reliability up to 1E-6 and short latency in the order of 0.5 to 1ms, have been identified as important areas for NR. This work item [1] was approved based on the outcome of the study items as shown in TR 38.824 [2] and TR 38.825 [3].

This work item specifies PDCCH enhancements, UCI enhancements, PUSCH enhancements, enhanced inter UE TX prioritization/multiplexing and enhanced UL configured grant transmission.

The following key functionalities were introduced by this WI:

DCI format 0_2 and DCI format 1_2

DCI format 0_2/1_2 with configurable sizes for most of the DCI fields are introduced, which provides the possibility to improve the reliability by decreasing the DCI size (e.g. ~24 bits) with appropriate RRC configuration of the DCI fields. Details of DCI format 0_2/1_2 can be found in [4].

Enhanced PDCCH monitoring capability

Rel-16 span-based PDCCH monitoring capability is introduced mainly for achieving low latency. A UE can indicate a capability to monitor PDCCH according to one or more of the combinations (X, Y) = (2, 2), (4, 3), and (7, 3) per SCS configuration of μ=0 and μ=1. A span is a number of consecutive symbols in a slot where the UE is configured to monitor PDCCH. If a UE monitors PDCCH on a cell according to combination (X,Y), the UE supports PDCCH monitoring occasions in any symbol of a slot with minimum time separation of X symbols between the first symbol of two consecutive spans, and the number of symbols of a span is up to Y. For each reported combination (X, Y), the UE supports the limit M_PDCCH^(max,(X,Y),μ) on the maximum number of monitored PDCCH candidates per PDCCH monitoring span as defined in Table 10.1-2A in [5] and the limit C_PDCCH^(max,(X,Y),μ) on the maximum number of non-overlapped CCEs for channel estimation per PDCCH monitoring span as defined in Table 10.1-3A in [5]. An example of PDCCH monitoring according to combination (4, 3) is as shown in Figure 1.

Fig. 1. An example of PDCCH monitoring using Rel-16 span based PDCCH monitoring capability.

Sub-slot based HARQ-ACK feedback

Sub-slot based HARQ-ACK feedback is introduced to support more than one PUCCH for HARQ-ACK transmission within a slot, which  is mainly beneficial for achieving low latency.  An UL slot consists of a number of sub-slots. No more than one PUCCH carrying HARQ-ACKs starts in a sub-slot. A UE can indicate the supported sub-slot configuration among the candidate values of {7-symbol*2, 2-symbol*7 and 7-symbol*2} for normal CP or { 6-symbol*2, 2-symbol*6 and 6-symbol*2} for extended CP.

Two HARQ-ACK codebooks constructed simultaneously

This work item specifies the support of two HARQ-ACK codebooks with different priorities to be simultaneously constructed, which is mainly beneficial for improving reliability for service with higher priority. Each of the two HARQ-ACK codebooks can be either slot-based HARQ-ACK codebook or sub-slot-based HARQ-ACK codebook. Separate PUCCH configurations are supported for different HARQ-ACK codebooks. The feature supports two priority levels for HARQ-ACK. Rules are defined for the UE to resolve collisions between UL channels/signals with different priorities.

PUSCH enhancements

This work item specifies PUSCH repetition type B and PUSCH repetition type A for PUSCH enhancements. PUSCH repetition type B is mainly beneficial for achieving low latency. PUSCH repetition type A can improve the spectral efficiency.

For PUSCH repetition type B, for a transport block, one dynamic UL grant or one configured grant schedules two or more PUSCH repetitions that can be in one slot, or across a slot boundary in consecutive available slots. Examples of PUSCH repetition type B are given in Figure 2. Inter-slot frequency hopping and inter-repetition frequency hopping are specified for PUSCH repetition type B. Interaction with DL/UL directions is specified as in Clause 6 in [6].

Fig. 2. Examples of PUSCH repetition type B.

PUSCH repetition type A corresponds to PUSCH transmission with Rel-15 behavior with or without slot aggregation. With slot aggregation, the number of repetitions can be dynamically indicated in Rel-16.

Enhanced inter UE Tx prioritization/multiplexing

This work item specifies UL cancellation scheme and enhanced UL power control scheme for enhanced inter UE Tx prioritization/multiplexing, which are mainly beneficial for achieving low latency.

For UL cancellation scheme, DCI format 2_4 is introduced for notifying the PRB(s) and OFDM symbol(s) where UE cancels the corresponding UL transmission from the UE. An indication by DCI format 2_4 for a serving cell is applicable to a PUSCH transmission or an SRS transmission on the serving cell.

For UL power control scheme, open-loop power control parameter set indication is included in DCI format 0_1/0_2 to indicate the P0 value for PUSCH scheduled dynamically as defined in [5].

Multiple active configured grant configurations for a BWP

Up to 12 configured grant configurations can be configured in a BWP of a serving cell, which is mainly beneficial for achieving high reliability. Separate RRC parameters can be configured for different configured grant configurations. Separate activation/release can be used for different configured grant Type 2 configurations. In addition, joint release for two or more configured grant Type 2 configurations for a given BWP of a serving cell is also supported.

References

RP-191584, “Revised WID: Physical Layer Enhancements for NR Ultra-Reliable and Low Latency Communication (URLLC)”, Newport Beach, CA, June 3-6, 2019.

TR 38.824 v16.0.0.

TR 38.825 v16.0.0.

TS 38.212 v16.2.0.

TS 38.213 v16.2.0.

TS 38.214 v16.2.0.

## 5.3	Support of NR Industrial Internet of Things

| 830080 | Support of NR Industrial Internet of Things (IoT) | NR_IIOT | R2 | RP-191561 | Nokia |
| --- | --- | --- | --- | --- | --- |
| 810060 | Study on channel modeling for indoor industrial scenarios | FS_IIIOT_CM | R1 | RP-182138 | Ericsson |
| 800098 | Study on NR Industrial Internet of Things (IoT) | FS_NR_IIOT | R2 | RP-181479 | Nokia |
| 830180 | Core part: Support of NR Industrial Internet of Things (IoT) | NR_IIOT-Core | R2 | RP-190728 | Nokia |

Summary based on the input provided by Nokia, Nokia Shanghai Bell in RP-200798.

The Work Item on Support of NR Industrial Internet of Things (IoT) [2] aimed at evolving NR system to better support use cases of various vertical markets such as factory automation or electrical power distribution. It introduced transmission reliability enhancements, NR support for Time Sensitive Communications (as defined in 3GPP TS 23.501 [4]), and addressed efficiency of the system where UEs handle a mixture of URLLC and eMBB traffic. This WI focused mainly on Layer 2/3 protocols aspects while PHY layer enhancements for URLLC were introduced as part of Work Item on Physical layer enhancements for NR ultra-reliable and low latency communication (URLLC) [3].

The following enhancements were introduced as part of the Work Item:

1.	PDCP packet duplication enhancements – a possibility of multiplication of the packets related to signalling or data radio bearer over three or four logical channels has been specified for the increased reliability of the transmission over the air interface. This is possible for both Carrier Aggregation based packet duplication where the packet is sent over up to four different serving cells of a single gNB and for Dual Connectivity based packet duplication where the packet is sent over serving cells belonging to two different gNBs (e.g. two serving cells in Master Node and two serving cells in Secondary Node or three serving cells in Master Node and one serving cell in Secondary Node etc.). The network dynamically controls which of the configured logical channels are active for duplication at a certain time by utilizing a dedicated MAC CE command.

2.	RAN support for higher layer multi-connectivity - The feature introduces also higher reliability for the end to end transmission by using duplication of a PDU session. This functionality allows NG-RAN to ensure the data of the PDU session and its redundant one to utilize two independent transmission paths. The NG-RAN may, for example, use dual connectivity principles with one PDU session delivered through the master node and the redundant one via the secondary node, or the NG-RAN may use independent paths within the same NG-RAN node. Besides, the feature offers the possibility to provide redundancy over N3 tunnel between the UPF and the NG-RAN node on a per QoS flow basis.

3.	Support for accurate reference timing delivery – to support strict synchronization accuracy requirements of TSC applications, the delivery of time reference information from the gNB to the UE using unicast or broadcast RRC signalling with a granularity of 10 ns was introduced. UE Assistance Information procedure was extended to allow the UE to indicate its preference to receive such information.

4.	Scheduling enhancements – support for up to eight simultaneously active semi-persistent scheduling (SPS) configurations for a given BWP of a UE was specified. Work Item introduced also new logical channel restriction based on physical layer priority level of the grant and a list of Configured Grant (CG) configurations allowed to be utilized by a certain logical channel (support of up to twelve simultaneously active CG configurations in a BWP of a UE was introduced as part of WI on Physical Layer Enhancements for NR Ultra-Reliable and Low Latency Communication (URLLC) [3]). UE can now be configured with CG and SPS periodicities of any integer multiple of a slot with maximum periodicity of 640 ms. These enhancements allow, e.g., to more efficiently support, deterministic traffic with a wide range of different periodicities.

5.	Time Sensitive Communication Assistance Information (TSCAI): Core Network may provide a gNB with an information about TSC traffic characteristics such as Burst Arrival Time, traffic flow direction and periodicity, to allow for a more efficient scheduling at the gNB.

6.	Ethernet Header Compression (EHC): since TSC traffic is often carried over Ethernet frames with a short size (e.g. 20-50 bytes), EHC protocol was specified within PDCP sublayer to increase efficiency of Ethernet frames transmission over the NR air interface. EHC allows to avoid transmission of Ethernet header fields such as DESTINATION ADDRESS, SOURCE ADDRESS, 802.1Q TAG, and LENGTH/TYPE, between the gNB and the UE. EHC was specified for both NR and EUTRA PDCP.

7.	Prioritization between overlapping uplink resources of one UE: when multiple UL grants provided to a single UE overlap in time on a serving cell, the UE is now able to consider the priority of the grant and/or the priority of the logical channel that can be carried over the grant when making a decision about which grant to utilize. Similarly, the UE may consider the priority of a scheduling request (SR) as well as priority of the logical channel which triggered the SR when deciding whether to transmit PUSCH or SR when they overlap in time. Furthermore, if two PUCCHs of different PHY priorities or a PUCCH and a PUSCH of different PHY priorities are overlapping in time on a serving cell, the UE is able to cancel the lower priority transmission with the specified cancellation behaviour and related timeline to allow for transmission of the PUSCH or PUCCH of higher PHY priority.

Thanks to the introduced enhancements it is possible to support Industrial IOT applications and Time Sensitive Communications in a more efficient way, allow for extra reliability for URLLC traffic as well optimize handling of a mixture of applications with various priorities and QoS requirements by a single UE.

References

[1]	3GPP TR 38.825: “Study on NR industrial Internet of Things (IoT)”

[2]	RP-192590: ”Revised WID: Support of NR Industrial Internet of Things (IoT)”

[3]	RP-190726: “New WID: Physical Layer Enhancements for NR Ultra-Reliable and Low Latency Communication (URLLC)”

[4]	3GPP TS 23.501: “System architecture for the 5G System (5GS)”

# 6	Support of LAN-type services

## 6.1	LAN support in 5G

| 800006 | LAN support in 5G | 5GLAN | S1 | SP-180593 | Jose Almodovar, KPN |
| --- | --- | --- | --- | --- | --- |
| 760007 | Study on LAN Support in 5G | FS_5GLAN | S1 | SP-170456 | Nasielski, Jack, Qualcomm |
| 800047 | Stage 1 of 5GLAN | 5GLAN | S1 | SP-180593 | Jose Almodovar, KPN |

Summary not yet provided

## 6.2	5GS Enhanced support of Vertical and LAN Services

| 750004 | Study on Communication for Automation in Vertical Domains | FS_CAV | S1 | SP-170169 | Walewski, Joachim, Siemens AG |
| --- | --- | --- | --- | --- | --- |
| 800007 | Service requirements for cyber-physical control applications in vertical domains | cyberCAV | S1 | SP-180321 | Joachim W. Walewski (Siemens) |
| 830042 | 5GS Enhanced support of Vertical and LAN Services | Vertical_LAN | S2 | SP-181120 | Devaki Chandramouli, Nokia |
| 800028 | Study on 5GS Enhanced support of Vertical and LAN Services | FS_Vertical_LAN | S2 | SP-180507 | Devaki Chandramouli, Nokia |
| 820017 | Stage 2 of Vertical_LAN | Vertical_LAN | S2 | SP-181120 | Devaki Chandramouli, Nokia |
| 810038 | Study on Security for 5GS Enhanced support of Vertical and LAN Services | FS_Vertical_LAN_SEC | S3 | SP-180697 | Jerichow, Anja (Nokia) |
| 840027 | Security for Vertical_LAN | Vertical_LAN_SEC | S3 | SP-190352 | Jerichow, Anja, Nokia |
| 830010 | CT aspects of Vertical_LAN | Vertical_LAN | ct | CP-191157 | WON, Sung Hwan, Nokia |
| 830035 | CT1 aspects of Vertical_LAN | Vertical_LAN | C1 | CP-191157 | WON, Sung Hwan, Nokia |
| 830036 | CT3 aspects of Vertical_LAN | Vertical_LAN | C3 | CP-191157 | WON, Sung Hwan, Nokia |
| 830037 | CT4 aspects of Vertical_LAN | Vertical_LAN | C4 | CP-191157 | WON, Sung Hwan, Nokia |
| 820027 | Service Enabler Architecture Layer for Verticals | SEAL | S6 | SP-181141 | Basavaraj (Basu) Pattan, Samsung |

Summary based on the input provided by Nokia, Nokia Shanghai Bell in SP-200296.

The Vertical_LAN work item introduces support for three new and distinct features:

1)	Enablers for 5G System deployment in 5G LAN type environment.

2)	Time-sensitive communication with enablers for 5GS to seamlessly integrate with IEEE TSN, and for 5GS to enable support of time synchronization.

3)	Enablers for Standalone Non-Public Network and Public-Network-integrated Non-Public Network.

5G_LAN

The 5G System supports management of 5G VN Group identification and membership as well as 5G VN Group data. The 5G VN Group management can be configured by a network administrator or it can be managed dynamically by an AF. In order to support dynamic management of 5G VN Group identification and membership, the NEF exposes a set of services to manage (e.g. add/delete/modify) 5G VN group and 5G VN members. The NEF also exposes services to dynamically manage 5G VN group data. Furthermore, 5GS supports optimized routing by enabling support for local switching at the UPF without having to traverse the DN for UE-UE communication when the two UE(s) are served by the same UPF.

Integration with IEEE TSN

TSC is a communication service that supports deterministic and/or isochronous communication with high reliability and availability. It provides packet transport with Quality of Service (QoS) characteristics such as bounded latency and reliability, where end systems and relay/transmit nodes can be strictly synchronized. In Release 16, main focus was on integration with IEEE Time-Sensitive Networking (TSN).

The 5G System also enables support for TSN time synchronization. In order to support time synchronization, the entire E2E 5G system can be considered as an IEEE 802.1AS [104] "time-aware system". UE, gNB, UPF, NW-TT and DS- TTs are synchronized with the 5G GM (i.e. the 5G internal system clock), which keeps these network elements synchronized. The TTs located at the edge of 5G system provides all functions related to IEEE 802.1AS.

Furthermore, this feature has also introduced TSC assistance parameters were introduced to facilitate optimized QoS scheduling for time sensitive and deterministic traffic.

Non-Public Network

This work item also addressed the deployment of Non-Public Networks (NPNs). A Non-Public Network (NPN) is a 5GS deployed for non-public use, see TS 22.261 [2]. An NPN may be deployed as:

-	a Stand-alone Non-Public Network (SNPN), i.e. operated by an NPN operator and not relying on network functions provided by a PLMN, or

-	a Public network integrated NPN, i.e. a non-public network deployed with the support of a PLMN.

SNPN 5GS deployments are based on the architecture depicted in clause 4.2.3, the architecture for 5GC with untrusted non-3GPP access (Figure 4.2.8.2.1-1) for access to SNPN services via a PLMN (and vice versa) and the additional functionality covered in clause 5.30.2.

References

[1] 3GPP TS 23.501, System Architecture for 5G System; Stage 2

[2] 3GPP TS 23.502, Procedures for 5G System; Stage 2.

[3] 3GPP TS 23.503, Policy and Charging Control Framework for the 5G System; Stage 2

Service Enabler Architecture Layer for Verticals

# 7	Cellular Internet of Things (IoT)

## 7.1	Cellular IoT support and evolution for the 5G System

| 830043 | Cellular IoT support and evolution for the 5G System | 5G_CIoT | S2 | SP-181118 | Sebastian Speicher |
| --- | --- | --- | --- | --- | --- |
| 770038 | Study on 5G_CIoT | FS_CIoT_5G | S2 | SP-180614 | Sebastian Speicher |
| 820015 | Stage 2 of 5G_CIoT | 5G_CIoT | S2 | SP-181118 | Sebastian Speicher |
| 830013 | CT aspects of 5G_CIoT | 5G_CIoT | ct | CP-111237 | Mahmoud Watfa; Qualcomm |
| 830038 | CT1 aspects of 5G_CIoT | 5G_CIoT | C1 | CP-111237 | Mahmoud Watfa; Qualcomm |
| 830039 | CT3 aspects of 5G_CIoT | 5G_CIoT | C3 | CP-111237 | Mahmoud Watfa; Qualcomm |
| 830040 | CT4 aspects of 5G_CIoT | 5G_CIoT | C4 | CP-111237 | Mahmoud Watfa; Qualcomm |
| 830041 | CT6 aspects of 5G_CIoT | 5G_CIoT | C6 | CP-111237 | Mahmoud Watfa; Qualcomm |
| 800033 | Study on evolution of Cellular IoT security for the 5G System | FS_CIoT_sec_5G | S3 | SP-180440 | Noamen Ben Henda |
| 760040 | Study on MBMS User Services for IoT | FS_MBMS_IoT | S4 | SP-170592 | Cedric thienot, Expway |

Summary based on the input provided by Qualcomm Incorporated in SP-200274.

Substantial E-UTRAN/EPC evolution has been achieved in 3GPP to enable the "Cellular Internet of Things" (CIoT). In particular, eMTC (WB-E-UTRAN) and NB-IoT have been designed in RAN WGs in Rel-13 and enhanced in Rel-14. The corresponding system architecture aspects have been designed for EPC in Rel-13 and Rel-14. These system architecture aspects apply to both NB-IoT and eMTC (WB-E-UTRAN).

The "Cellular IoT support and evolution for the 5G System" work item focused on enabling equivalent functionality for NB-IoT and eMTC connected to 5GC as what has been defined for NB-IoT and eMTC connected to EPC in earlier releases

This section summarizes the key system impacts resulting from the “Cellular IoT support and evolution for the 5G System” work item.

As documented in TS 23.501 [1] clause 5.31, the following CIoT features have been introduced in Rel-16:

-	Control Plane CIoT 5GS Optimisation (CP CIoT 5GS Optimisation) is used to exchange user data between the UE and the SMF as payload of a NAS message in both uplink and downlink directions, avoiding the establishment of a user plane connection for the PDU Session (i.e. avoiding the need for Data Radio Bearer and N3 tunnel). Early Data Transmission (EDT), i.e. sending user data in RRC Message 3 is supported for mobile originated Control Plane CIoT 5GS Optimisation.

-	User Plane CIoT 5GS Optimisation (UP CIoT 5GS Optimisation) supports transfer of user plane data from CM-IDLE without the need for using the Service Request procedure to establish an Access Stratum (AS) context in NG-RAN and UE. UP CIoT 5GS Optimisation is enabled by the Connection Suspend and Connection Resume in CM-IDLE with Suspend procedures. EDT is also supported for UP CIoT 5GS Optimisation.

-	UE and network negotiate whether to use CP CIoT 5GS Optimisation and/or UP CIoT 5GS Optimisation as well as N3 data transport and header compression for CP CIoT 5GS Optimisation during the Registration procedure. The UE indicates its Preferred and Supported Network Behaviour, i.e. the UE indicates which of the aforementioned features the UE supports and whether it prefers to use CP CIoT 5GS Optimisation or UP CIoT 5GS Optimisation. In response, the network indicates which of those features it supports for this UE.

-	Non-IP Data Delivery (NIDD) refers to mobile originated (MO) and mobile terminated (MT) communication between UE and an Application Function (AF) where the user data is considered unstructured (also referred to as "non-IP"). NIDD is enabled using an unstructured PDU session between UE and NEF and the NEF's NIDD API on the N33/Nnef reference point for data delivery from/to AF. Alternatively, non-IP data is delivered using an unstructured PDU session between UE and UPF and a Point-to-Point N6 tunnel between UPF and AF. The NEF also supports distribution of mobile terminated messages to a group of UEs based on the NIDD API.

-	Reliable Data Service (RDS) may be used between UE and NEF or UPF, respectively, for unstructured PDU Sessions. RDS provides a mechanism for the NEF or UPF to determine if the user data was successfully delivered to the UE and for the UE to determine if the data was successfully delivered to the NEF or UPF. When a requested acknowledgement is not received, RDS retransmits the packet.

-	Extended Discontinuous Reception (DRX) for CM-IDLE and CM-CONNECTED with RRC-INACTIVE enables the UE to reduce its power consumption while still being available for MT data and/or network originated procedures within a certain delay dependent on the negotiated DRX cycle value. In CM-IDLE state the following DRX cycles are supported: up to almost 44 minutes (for eMTC) and up to almost 3 hours (for NB-IoT). In CM-CONNECTED with RRC-INACTIVE, DRX cycles of up to 10.24 seconds are supported.

-	Enhancements for the Mobile Initiated Connection Only (MICO) mode of rel.15:

-	MICO mode with Extended Connected Time enables an AMF that is aware of pending or expected MT traffic to keep the UE in CM-CONNECTED state and to request the RAN to keep the UE in RRC-CONNECTED state for an Extended Connected Time period to ensure that the downlink data and/or signalling is delivered to the UE before the UE is released.

-	MICO mode with Active Time is similar to the UE Power Saving Mode (PSM) defined for EPS [2], i.e. UE and AMF negotiate an Active Time value, which dictates for how long the UE is reachable for paging upon entering CM-IDLE. Once the Active Time has elapsed, the UE can enter MICO mode, i.e. become unreachable for paging.

-	MICO mode and Periodic Registration Timer Control enables the network to align a UE's Periodic Registration Updates with an expected DL communication schedule for the UE. This is achieved by the Strictly Periodic Registration Timer Indication which the network can provide to the UE to avoid that the Periodic Registration Timer is restarted by the UE when the UE enters CM-CONNECTED.

-	High latency communication refers to mechanisms that may be used to handle mobile terminated (MT) communication with UEs being unreachable while using power saving functions (e.g. extended DRX or MICO mode with Active or Extended Connected Time). High latency communication is supported by extended buffering of downlink data in the UPF, SMF or NEF when a UE is using power saving functions in CM-IDLE and is not reachable. Alternatively, high latency communication is supported through different AF notifications. An AF may for example subscribe to receive UE reachability notifications so that the AF then waits with sending the data until it gets a notification that the UE has become reachable.

-	Support for Monitoring Events enables AFs to acquire information such as whether a UE is roaming or to determine when a USIM is changed to a different ME. The complete list of monitoring events is documented in TS 23.502 [3] clause 4.15.3.1.

-	External parameter provisioning refers to functionality originally defined in Rel-15 already, which enables an AF to provision information such as Expected UE Behaviour parameters or other service parameters to the network (see TS 23.502 [3] clause 4.15.6.1). In Rel-16, additional Expected UE Behaviour parameters were introduced (e.g. an indication whether the UE is stationary or mobile), which can be used by the AMF and/or SMF for various purposes, e.g. by AMF for paging optimisations. In addition, an AF can provide Network Configuration parameters (e.g. Maximum Response Time) to the network, which in turn can be used to derive parameters for system procedures (e.g. to derive the Active Time or Extended Connected time for MICO mode). The complete list of Expected UE Behaviour parameters and Network Configuration parameters are documented in TS 23.502 [3] clause 4.15.6.3 and 4.15.6.3a, respectively.

-	PDU session handling during inter-RAT idle mode mobility to and from NB-IoT allows the SMF to maintain a PDU session, disconnect a PDU session with a reactivation request or to disconnect a PDU session without reactivation request when the UE moves between a "broadband" RAT (e.g. NR or WB-E-UTRAN) and a "narrowband" RAT (NB-IoT).

-	System aspects of the Enhanced Coverage RAN feature as specified in TS 36.300 [4], in particular storage of Paging Assistance Data for UEs supporting Enhanced Coverage and providing of Paging Assistance Data to RAN during paging has been introduced. Specific subscribers can also be restricted to use the Enhanced Coverage feature through Enhanced Coverage Restricted information that is stored in the UDM. Based on the latter, AMF informs UE, RAN and SMF about the use (or not) of the Enhanced Coverage feature.

-	The rate of user data sent to and from a UE can be controlled in two ways:

-	Serving PLMN rate control allows the serving network to protect its AMF and the Signalling Radio Bearers in the NG-RAN from the load generated by NAS Data PDUs by indicating a limit for the number of NAS Data PDUs per time unit to the UE and UPF/NEF.

-	Small Data Rate Control allows HPLMN operators to offer customer services such as "maximum of Y messages per day" and is also based on indicating a number of packets per time unit limitation to UE and UPF/NEF for enforcement.

-	Congestion control for control plane data transfer enables the AMF to restrict, e.g. during high-load situations, the use of the control plane for data transmission (i.e. for Control Plane CIoT 5GS Optimisation). In particular, AMF may provide a Control Plane data back-off timer to the UE. While the Control Plane data back-off timer is running, the UE is not allowed to initiate any data transfer via Control Plane CIoT 5GS Optimisation.

-	Service Gap Control can be used to control the frequency at which UEs can access the network, e.g. to alleviate peak load situations. Service Gap control is realized by AMF indicating a Service Gap Time to a UE; the UE then stays in CM-IDLE mode for at least the whole duration of the Service Gap timer before triggering Mobile Originated user data transmission.

-	Inter-UE QoS for NB-IoT targets UEs that are using Control Plane CIoT 5GS Optimisation and are accessing the network via NB-IoT. The feature allows NG-RAN to prioritise resource allocation between those UEs based on the subscribed NB-IoT UE Priority that NG-RAN may retrieve from the AMF.

-	Differentiation of Category M UEs enables the network to identify traffic to/from Category M UEs, e.g. for charging differentiation and subscription based access control. This functionality is based on a Category M indication from UE to NG-RAN during RRC Connection Establishment and subsequently, an LTE-M Indication to the AMF in the Initial UE Message. Based on this, AMF considers the RAT type to be LTE-M and informs SMF, SMSF and PCF accordingly for subsequent use of the LTE-M RAT type, e.g. in CDRs. A subscription parameter for subscription based access restriction for LTE-M is also introduced.

-	Selection, steering and redirection between EPS and 5GS allows a network that supports CIoT features in both EPC and 5GC to steer UEs from the CN type that the UE is attempting to register with to the other CN type (e.g. from 5GC to EPC) due to operator policy, e.g., due to roaming agreements, Preferred and Supported Network Behaviour, load redistribution, etc. It is assumed that operators configure the steering policies in EPC and 5GC such that steering UEs back and forth between EPC and 5GC is avoided.

References

[1]	TS 23.501, “System architecture for the 5G System (5GS)”

[2]	TS 23.401, “General Packet Radio Service (GPRS) enhancements for Evolved Universal Terrestrial Radio Access Network (E-UTRAN) access”

[3]	TS 23.502, “Procedures for the 5G System (5GS)”

[4]	TS 36.300, " Evolved Universal Terrestrial Radio Access (E-UTRA) and Evolved Universal Terrestrial Radio Access Network (E-UTRAN); Overall description; Stage 2"

### 7.2	Additional enhancements for NB-IoT

| 800084 | Additional enhancements for NB-IoT | NB_IOTenh3 | R1 | RP-200293 | Huawei |
| --- | --- | --- | --- | --- | --- |
| 800184 | Core part: Additional enhancements for NB-IoT | NB_IOTenh3-Core | R1 | RP-200293 | Huawei |
| 800284 | Perf. part: Additional enhancements for NB-IoT | NB_IOTenh3-Perf | R4 | RP-200293 | Huawei |

Summary based on the input provided by FUTUREWEI in RP-201229.

This Feature builds on the successful base NB-IoT feature in Rel-13, and enhancements in Rel-14 and Rel-15, and adds features such as DL/UL transmission efficiency improvement, UE power consumption improvement, scheduling enhancement, network management tool enhancement, latency improvement, enhancement on coexistence with NR and connection to 5GC.

Improved DL transmission efficiency and UE power consumption

Reduced UE power consumption and improved transmission efficiency are achieved through reduced downlink monitoring and reduced signalling, building on features introduced in earlier releases.

UE-group wake-up signals (GWUS) (FDD):

With the introduction of Rel-15 wake-up signalling (WUS), UE can skip the paging procedures if the wake-up signal is not detected to save power. This feature (UE-group WUS) allows eNB to transmit a UE-group WUS to instruct the UEs in the group that they must monitor NPDCCH for paging. This allows the UE to skip the paging procedures to save more power if eNB sends UE-group WUS to UEs in other groups. The UEs are grouped according to their paging probability and/or their UE ID based on system information configuration.

Mobile-terminated early data transmission (MT-EDT) (FDD):

Rel-15 Mobile Originating Early Data Transmission (MO-EDT) allows one uplink data transmission optionally followed by one downlink data transmission during the random access procedure, avoiding transition to RRC_CONNECTED mode. Rel-16 Mobile Terminating Early Data Transmission (MT-EDT) allows one downlink data transmission during the random access procedure triggered in response to a paging message. This feature allows the eNB to decide whether to initiate Mobile Terminated EDT procedures towards the UE based on the data size received from the core network. Mobile Terminating Early Data Transmission (MT-EDT) is only supported in EPC.

Support for Preconfigured uplink resources (PUR) in idle mode (FDD)

In Rel-15, signalling overhead and power consumption reductions were introduced by the (mobile-originated) early data transmission (EDT) feature, where data can be transmitted already in Msg3 during the random-access procedure.

In Rel-16, the earlier transmission of UL data payload has been further enhanced by introducing UL transmission using preconfigured uplink resources (PUR). This feature allows eNB to configure uplink resources, in which a UE in IDLE mode can send UL transmission without performing random access procedures. The UE can be potentially configured with a cyclic shift of DMRS, which allows sharing of the preconfigured resources under which up to two users can transmit NPUSCH simultaneously when the NPUSCH transmission is larger than or equal to 64ms for 12-tone allocation. By skipping the random access procedures, the uplink transmission efficiency can be improved and UE power consumption is reduced. Before performing a PUR transmission, the UE must evaluate the validity of the timing advance (TA) based on either individual or combined usage of any of the following attributes: a) serving cell change, b) TA timer, c) RSRP change. Additionally, it is possible to configure the TA as always valid within a given cell.

Scheduling of multiple DL/UL transport blocks with single DCI (FDD)

This feature allows the scheduling of up to two transport blocks (TB) with a single DCI for uplink or downlink unicast transmission, where the number of TBs is indicated by DCI. The transmission of multiple TBs can be configured to be contiguous or interleaved. And HARQ bundling can be potentially configured when transmission is configured as interleaved. It also allows the scheduling of up to eight transport blocks with a single DCI for SC-MTCH, where the number of TBs is indicated by DCI. The DCI overhead can be reduced for contiguous UL/DL transmissions.

Network management tool enhancements - SON (FDD and TDD)

Rel-16 introduces SON features: RACH report, RLF report and ANR for network resource optimisation. The ANR measurements are performed when the UE is in RRC_IDLE and reported next time the UE enters RRC_CONNECTED.  The NPRACH configuration of the NB-IoT Cells are exchanged between neighbour eNBs for RACH optimization. And the RLF report from UE is forwarded to the old eNB to determine the nature of the failure.

SON features are only supported in EPC.

Improved multicarrier operations - Quality report in Msg3 and connected mode (FDD)

In cells with interference, the coverage level corresponding to the estimate RSRP may be mismatched with the channel quality. This feature allows the eNB to configure a UE in IDLE mode to report the downlink channel quality in Msg3 for non-anchor access. It also allows the UE to report the downlink channel quality in connected mode other than Msg3 for anchor and non-anchor carriers. This allows the eNB to schedule NPDCCH and NPDSCH more accurately, especially in cases with mismatch between coverage level and channel quality.

Presence of NRS on a non-anchor carrier for paging (FDD)

This feature allows eNB to transmit NRS in subframes on a non-anchor carrier for paging even when no paging NPDCCH is transmitted. The NRS are present in the first M subframes out of the 10 NB-IoT DL subframes before the Paging Occasion (PO), where the PO can be a subset of POs or a whole set of POs, and the values of M depend on the value of nB as defined in TS 36.304.

When NRS is present on a non-anchor paging carrier and the conditions for NRSRP measurement on non-anchor carrier are met as defined in TS 36.133, the UE may perform serving cell measurements on the non-anchor paging carrier.

Mobility enhancements - Idle mode inter-RAT cell selection to/from NB-IoT (FDD and TDD)

With this feature, NB-IoT can provide assistance information for inter-RAT cell selection to E-UTRAN/GERAN and E-UTRAN can provide assistance information for inter-RAT cell selection to NB-IoT. A UE may use the assistance information provided by the network for cell selection to/from NB-IoT.

Improved latency - UE Specific DRX (FDD and TDD)

Rel-16 introduces support for UE specific DRX to reduce paging latency.  The eNB may broadcast a minimum UE specific DRX value shorter than the cell default DRX value.  When UE specific DRX is configured by the upper layers and the minimum UE specific DRX value is broadcast, the UE monitors paging according to the longer of the two values.

Coexistence of NB-IoT with NR (FDD and TDD)

This feature allows the configuration of the DL/UL resource reservation in subframe/slot/symbol-levels on non-anchor carriers for unicast transmission to avoid resource overlapping with NR channels/signals. The configuration can be for 10ms or 40ms duration, with a periodicity from {10ms, 20ms, 40ms, 80ms, 160ms} and a start position in a granularity of 10ms, which is independent from legacy configurations. It also allows dynamic indication whether the resource reservation is applied or not.

Three system scenarios have been studied and captured in TR 37.824:

•	For NB-IoT operation in NR in-band, RB alignment, power boosting and numerologies have been addressed.

•	For NB-IoT operation in NR guard band, RF requirements will not be specified.

•	For NB-IoT standalone operation, based on coexistence study, it is concluded that there is no issue for NB-IoT standalone coexistence with NR.

Connection to 5GC (FDD and TDD)

Rel-16 introduces support for connection to 5GC reusing eLTE as a baseline, including Unified Access Control (UAC). RRC_INACTIVE, NR SDAP and NR PDCP are not supported and a maximum of two PDU sessions mapped to two default DRBs is supported.

Rel-16 also introduces support for the CIoT 5GS optimisation, in particular extended DRX (eDRX) in RRC_IDLE, control plane and user plane CIoT optimisation, MO-EDT for the control plane and user plane CIoT optimisation, RRC Connection Reestablishment for the control plane and restriction of use of Enhanced Coverage.

Similar backhaul signalling to support control plane and user plane CIoT optimisation are introduced over NG interface (between ng-eNB and AMF) and over Xn interface (between ng-eNBs), including e.g. early UE radio capability retrieval from core network after msg3, NB-IoT CP relocation procedures to support connection reestablishment, Paging assistance information exchanging, UE differentiation information exchanging, etc.

### 7.3	Additional MTC enhancements for LTE

| 800083 | Additional MTC enhancements for LTE | LTE_eMTC5 | R1 | RP-191356 | Ericsson |
| --- | --- | --- | --- | --- | --- |
| 800183 | Core part: Additional MTC enhancements for LTE | LTE_eMTC5-Core | R1 | RP-190770 | Ericsson |
| 800283 | Perf. part: Additional MTC enhancements for LTE | LTE_eMTC5-Perf | R4 | RP-190770 | Ericsson |

Summary based on the input provided by Ericsson in RP-201276.

This work item builds on the LTE features for Machine-Type Communications (MTC) introduced in earlier releases (e.g. low-complexity UE categories M1 and M2, and Coverage Enhancement Modes A and B) by further improving network operation and efficiency in a range of areas.

The following sections describe the new MTC features for LTE in Rel-16. All features are optional for the UE and can be supported by Cat-M1 and Cat-M2 and by normal LTE UEs supporting CE mode unless otherwise stated. All features are applicable to both CE modes (A and B) in all duplex modes (HD-FDD, FD-FDD, and TDD) unless otherwise stated.

Improved DL transmission efficiency and UE power consumption

Reduced UE power consumption is achieved through reduced downlink monitoring and reduced signalling, building on features introduced in earlier releases.

•	UE-group wake-up signals (GWUS): Reduced UE power consumption in idle mode was enabled in Rel-15 by the introduction of the wake-up signal (WUS), a compact signal transmitted a configurable time before the paging occasion (PO) when a UE is being paged, allowing the UE to maximize its sleep time during periods when there is no paging. In Rel-16, an enhancement is introduced that allows a WUS to wake up a configurable group of UEs rather than all UEs that happen to monitor the same PO. This helps reduce the power consumption even further. The mapping of GWUS in the time and frequency domains is highly configurable.

•	Mobile-terminated early data transmission (MT-EDT): For scenarios where the UE only needs to transmit a small amount of data, the early data transmission (EDT) feature in Rel-15 enables the UE to transmit up to (slightly more than) 100 bytes of data already in Msg3 during the random-access procedure, and to receive data already in Msg4. If needed, eNB can order fallback to legacy random-access procedure during the EDT procedure. In Rel-16, an enhancement is introduced that allows not only mobile-originated (MO) EDT access but also mobile-terminated (MT) EDT. When the MME triggers MT-EDT, an indication is included in the paging message, after which the UE triggers random access to resume the connection (in case the UP CIoT EPS optimization is used) or initiate MO-EDT (in case the CP CIoT EPS optimization is used). MT traffic is received in Msg4. MT-EDT is only supported when UE is connected to EPC (not 5GC).

•	Improved DL quality reporting: Legacy CE mode A supports both periodic and aperiodic CSI reporting which can be used to assist PDSCH link adaptation. In Rel-16, a new type of DL quality reporting is introduced which reflects MPDCCH quality rather than PDSCH quality. The report represents the required number of MPDCCH subframe repetitions for reliable MPDCCH reception. It can be sent in connected mode, but it can also be sent already in Msg3 during the random access procedure, which means that the report can be used for guiding the UE-specific MPDCCH configuration, which helps optimize power consumption, latency, and spectral efficiency.

•	MPDCCH performance improvement: In legacy LTE-MTC, MPDCCH demodulation is DMRS-based. With this feature, the UE can use a combination of DMRS and CRS for MPDCCH demodulation to improve the MPDCCH performance. The feature takes the configured DMRS-to-CRS power ratio into account. The feature can be used for transmissions in idle mode and/or connected mode. In idle mode, the DMRS-to-CRS mapping is based on precoder cycling, whereas in connected mode, it can be configured to be precoder cycling based, CSI-based, or (in case of TDD) reciprocity-based.

Beside the features listed above, the features described in sections 2.2, 2.3 and 2.6 can also improve UE power consumption and/or transmission efficiency in DL and/or UL.

Preconfigured uplink resources (PUR)

In Rel-15, signalling overhead and power consumption reductions were introduced by the (mobile-originated) early data transmission (EDT) feature, where data can be transmitted already in Msg3 during the random-access procedure.

In Rel-16, the earlier transmission of UL data payload has been further enhanced by introducing UL transmission using preconfigured uplink resources (PUR). When the feature is configured, both the random-access preamble transmission (Msg1) and the random-access response (Msg2) can be omitted, and the data transmission can be completed in only two messages (i.e., Msg3 and Msg4).

The UE is configured with PUR via dedicated RRC signaling while in connected mode. Configuring a UE with PUR can be triggered by the network or requested by the UE. Before performing a PUR transmission, the UE must evaluate the validity of the timing advance (TA) based on either individual or combined usage of any of the following attributes: a) serving cell change, b) TA timer, c) RSRP change. Additionally, it is possible to configure the TA as always valid within a given cell.

There are two schemes for transmitting using PUR, dedicated PUR and shared PUR, the latter allows up to two users to transmit simultaneously when the number of PUSCH repetitions is greater than or equal to 64 for full-PRB allocation.

Scheduling of multiple transport blocks

In legacy LTE-MTC operation, each DCI carried by MPDCCH schedules a single PDSCH or PUSCH transport block (TB). In Rel-16, a possibility to schedule multiple TBs using a single is introduced. This can help improve the resource utilization by reducing the number of physical resource blocks (PRBs) spent on MPDCCH transmission and the number of subframes spent on guard time for DL-to-UL and UL-to-DL transition (in half-duplex FDD operation).

•	Unicast multi-TB scheduling: When the feature is configured, a single DCI can schedule multi TBs for PDSCH or PUSCH (up to 8 TBs in CE mode A, or up to 4 TBs in CE mode B). The number of TBs is dynamically controlled by the DCI. The TBs can be configured to be transmitted consecutively or subframe interleaved (in case of subframe repetition). For PDSCH multi-TB scheduling, HARQ-ACK bundling can optionally be used to improve the resource utilization further for UEs in good coverage. For PUSCH multi-TB scheduling, early termination of the PUSCH transmission is supported through indication of positive HARQ-ACK in the DCI.

•	Multicast multi-TB scheduling: When the feature is configured a single DCI can schedule up to 8 TBs for PDSCH for a SC-MTCH, with configurable time gaps between the TBs if desired. The number of TBs is dynamically controlled by the DCI.

CE mode improvements for non-Cat-M UEs

The features in this work item can be supported both by Cat-M UEs and non-Cat-M UEs that support CE mode A or B. In addition, the following features have been specified specifically for non-Cat-M UEs that support CE mode A or B.

•	Enhancements to idle mode mobility: A possibility is introduced for a non-Cat-M UE in a non-standalone LTE-MTC cell to use enhanced coverage functionality to camp in the cell even if the S-criterion indicates that the UE is in normal coverage. This functionality is enabled/disabled by a configuration provided in SIB1. (This is the default behavior for the standalone LTE-MTC case described in the next section in this document.)

•	CSI feedback based on CSI-RS: In legacy CE mode A, periodic and aperiodic CSI feedback is based on up to 4 CRS antenna ports. This feature introduces support for periodic CSI feedback based on 8 CSI-RS antenna ports in TM9 for non-Cat-M UEs in CE mode A. The feature can help improve the DL link adaptation and hence the DL performance. As a separate UE capability, the feature can also optionally be supported in combination with codebook subset restriction.

•	ETWS/CMAS in connected mode: In legacy LTE-MTC, ETWS/CMAS notification indication is supported using DCI format 6-2 in MPDCCH common search space Type-1 in idle mode. This feature introduces ETWS/CMAS notification indication using DCI format 6-1A/B in MPDCCH common search space Type-0 in connected mode for non-Cat-M UEs in CE mode A/B. This means that a UE can be notified without releasing the UE to idle mode.

Stand-alone deployment

In legacy LTE-MTC operation, the first few OFDM symbols in each DL subframe are unused by LTE-MTC since they are assumed to be occupied by LTE control channels for normal LTE UEs (PCFICH, PDCCH, PHICH). This feature enables transmission of MPDCCH and/or PDSCH to UEs in CE mode A/B in the “LTE control channel region” on carriers that are not used for normal LTE. The feature can be used for transmissions in idle mode and/or connected mode. The potential DL transmission efficiency gain is about 14% (corresponding to 2 out of 14 OFDM symbols) for 1.4 MHz carriers and about 7% (corresponding to 1 out of 14 OFDM symbols) for wider carriers.

Mobility enhancements

In Rel-15, two new LTE-MTC signals were introduced, the resynchronization signal (RSS) and the wake-up signal (WUS), and in Rel-16 the following mobility enhancements are introduced which make use of the Rel-15 signals.

•	RSS-based measurements: In Rel-15, support for a resynchronization signal (RSS) was introduced and its configuration is provided by the serving cell. In Rel-16, signaling of RSS configurations for neighbor cells is introduced. Both broadcasted and dedicated signaling can be used to provide the configurations. The primary purpose of RSS is to improved synchronization performance, but with the Rel-16 signaling, the UE may also use RSS for improved measurement performance for intra-frequency RSRP measurements for neighbor cells in both idle and connected mode.

•	RRM measurement relaxation: The legacy LTE-MTC UE behavior requires the UE to measure on the serving cell and evaluate the cell selection criterion at least every DRX cycle. The wake-up signal (WUS) introduced in Rel-15 would allow the UE to sleep for multiple paging cycles and wake up to receive paging after a configurable time duration, but the UE power saving gain from WUS cannot be fully utilized since the UE is still required to wake up for measurements. Therefore, an RRM measurement relaxation is introduced in Rel-16, which allows the UE meet the requirements using a longer measurement cycle to save power, where the cycle is configurable under certain conditions.

Performance improvement for NR coexistence

Spectrum sharing with legacy (Rel-13/14/15) LTE-MTC is already supported in Rel-15 NR, and the RF coexistence aspects described in TR 37.823. The following features are introduced in Rel-16 LTE-MTC in order to further improve the performance of the coexistence with NR.

•	DL/UL resource reservation: Legacy LTE-MTC supports configuration of invalid DL/UL subframes, which can be used in order to avoid mapping LTE-MTC transmissions to subframes that are needed for NR transmissions. Rel-16 takes a step further by introducing finer-granularity LTE-MTC resource reservation in both the time domain (with subframe, slot, or symbol level granularity) and the frequency domain (with LTE RBG level granularity) for unicast MPDCCH/PDSCH/PUSCH/PUCCH transmissions in connected mode in CE mode A/B. The resource reservation patterns are configurable using parameter combinations based on bitmaps, periodicities and offsets. For PDSCH/PUSCH, the DCI can indicate that the resource reservation should be overridden, in which case the PDSCH/PUSCH transmission becomes continuous.

•	DL subcarrier puncturing: In order to achieve PRB alignment between LTE-MTC and NR, a possibility to puncture 1 or 2 DL subcarriers at the lower or higher edge of each 6-PRB narrowband is introduced. The puncturing affects MPDCCH/PDSCH transmissions in connected mode in CE mode A/B. The performance loss from the puncturing should typically be insignificant.

Connection to 5GC

In Rel-16, support for connecting LTE-MTC UEs to 5GC is introduced. It resembles the Rel-15 functionality for connecting LTE UEs to 5GC. The RRC_INACTIVE state is supported and additionally the User Plane CIoT 5GS optimisation is supported in RRC_IDLE (similar to the corresponding EPC feature). Some features, such as EDT and PUR are supported only in RRC_IDLE using the UP-optimisation solution and are not supported in RRC_INACTIVE. Long extended DRX in RRC_IDLE is supported, and RAN paging cycles of 5.12 s and 10.24 s are supported in RRC_INACTIVE.

References

[1]	RP-192875, Rel-16 LTE-MTC work item description

[2]	RP-201275, Rel-16 LTE-MTC work item status report

[3]	RP-192647 & RP-192648 & RP-200196 & RP-200698, RAN1 CR packs

[4]	RP-200360 & RP-201192 & RP-201193, RAN2 CR packs

[5]	RP-201086 & RP-201087, RAN3 CR packs

[6]	RP-193023 & RP-200418 & RP-200962, RAN4 CR packs

# 8	Advanced V2X support

## 8.1	Improvement of V2X service Handling

| 820024 | Improvement of V2X service Handling | V2XIMP | S1 | SP-181013 | SungDuck Chun; LG Electronics |
| --- | --- | --- | --- | --- | --- |

Summary based on the input provided by LG Electronics France in SP-200126.

This WI introduced requirements related to vehicle quality of service support, which enables a V2X application to be timely notified of expected or estimated change of quality of service (QoS). For example, when the communication packet error is expected to increase or decrease, the V2X application such as platooning application can increase or decrease inter-vehicle distance.

Followings are key requirements out of this WI:

- A standardized interface to external V2X application servers are provided.

- Using the standardized interface, the V2X application servers can request a specific QoS requirements and, optionally, related alternative QoS requirements.

- Using the standardized interface, the V2X application servers can request to be notified when the requested QoS requirements and/or the alternative QoS requirements can or cannot be met.

- The information that delivered over the interface can be past statistics or future prediction.

With this information, the V2X applications can adapt its behaviour according to the QoS that can be provided by 3GPP System.

Stage-2/3 works related to this WI were progressed within eV2XARC WI (see below).

Reference

[1]	TS 22.186: "Enhancement of 3GPP support for V2X scenarios; Stage 1".

## 8.2	Architecture enhancements for 3GPP support of advanced V2X services

| 840078 | Architecture enhancements for 3GPP support of advanced V2X services | eV2XARC | S2 | SP-181121 | LaeYoung Kim, LG Electronics |
| --- | --- | --- | --- | --- | --- |
| 760043 | Study on eV2XARC | FS_eV2XARC | S2 | SP-180733 | LaeYoung Kim, LG Electronics |
| 820018 | Stage 2 of eV2XARC | eV2XARC | S2 | SP-181121 | LaeYoung Kim, LG Electronics |
| 840011 | CT aspects of eV2XARC | eV2XARC | ct | CP-191154 | Herrero Veron, Christian (Huawei) |
| 840079 | CT1 aspects of eV2XARC | eV2XARC | C1 | CP-191154 | Herrero Veron, Christian (Huawei) |
| 840080 | CT3 aspects of eV2XARC | eV2XARC | C3 | CP-191154 | Herrero Veron, Christian (Huawei) |
| 840081 | CT4 aspects of eV2XARC | eV2XARC | C4 | CP-191154 | Herrero Veron, Christian (Huawei) |
| 840082 | CT6 aspects of eV2XARC | eV2XARC | C6 | CP-191154 | Herrero Veron, Christian (Huawei) |

Summary based on the input provided by LG Electronics in SP-200058.

Architecture enhancements to the 5G System are specified in TS 23.287 [1] in order to facilitate vehicular communications for Vehicle-to-Everything (V2X) services, over the following reference points, based on service requirements defined in TS 22.185 [2] and TS 22.186 [3]:

-	PC5 reference point: NR PC5 RAT, LTE PC5 RAT.

-	Uu reference point: NR, E-UTRA.

Interworking between EPS V2X and 5GS V2X is also specified.

The following architectural reference models are specified:

-	5G System architectures for V2X communication over PC5 and Uu reference points

-	5G System architecture for AF-based service parameter provisioning for V2X communications

-	Architecture reference model for interworking with EPS V2X

The various parameters for V2X communications over PC5 and Uu reference points are specified and these parameters may be made available to the UE in following ways:

-	pre-configured in the ME; or

-	configured in the UICC; or

-	preconfigured in the ME and configured in the UICC; or

-	provided/updated by the V2X Application Server via PCF and/or V1 reference point; or

-	provided/updated by the PCF to the UE.

In addition to PCF initiated Policy Provisioning procedure, the UE may perform UE triggered Policy Provisioning procedure to the PCF when the UE determines the V2X Policy/Parameter is invalid (e.g. Policy/Parameter is outdated, missing or invalid).

Regarding V2X communication over PC5 reference point, two types of PC5 reference points exist: the LTE based PC5 reference point as defined in TS 23.285 [4], and the NR based PC5 reference point as defined in TS 23.287 [1]. A UE may use either type of PC5 or both for V2X communication depending on the services the UE supports. The V2X communication over PC5 reference point supports roaming and inter-PLMN operations. V2X communication over PC5 reference point is supported when UE is "served by NR or E-UTRA" or when the UE is "not served by NR or E-UTRA".

V2X communication over NR based PC5 reference point supports broadcast mode, groupcast mode and unicast mode. For unicast mode, Layer-2 link establishment, Link identifier update, Layer-2 link release, Layer-2 link modification and Layer-2 link maintenance procedures are specified. Per-Flow PC5 QoS Model is introduced for V2X communication over NR based PC5 reference point.

Architecture enhancements for EPS to support V2X communication over NR PC5 reference point are specified in TS 23.285 [4].

For V2X communication over Uu reference point, only unicast is supported. Latency reduction for V2X message transfer via unicast may be achieved by using various mechanisms, including via e.g., edge computing defined in TS 23.501 [5].

Notification on QoS Sustainability Analytics to the V2X Application Server is specified so that the V2X Application Server may request notifications on QoS Sustainability Analytics for an indicated geographic area and time interval in order to adjust the application behaviour in advance with potential QoS change.

To support V2X applications that can operate with different configurations (e.g. different bitrates or delay requirements), the V2X Application Server, acting as the Application Function, can provide, in addition to the requested level of service requirements, Alternative Service Requirements to the 5G System. This enables the 5G System to act on the Alternative Service Requirements and apply them for the extended NG-RAN notification (i.e. Alternative QoS Profiles are provided from SMF to NG-RAN), as described in TS 23.501 [5] and TS 23.503 [6].

In order to facilitate deployment of dedicated network slice for use of, for example, automotive industry and to facilitate roaming support, a new standardized Slice/Service Type (SST) value dedicated for V2X services, i.e. 4 is defined in TS 23.501 [5].

Security aspects of 3GPP support for advanced V2X services are specified in TS 33.536 [7].

Stage 3 normative works made by CT working groups are specified in TS 24.587 [8] and TS 24.588 [9] that are new specifications for V2X as well as many specifications listed in the WID on CT aspects of architecture enhancements for 3GPP support of advanced V2X services [10].

References

[1] 3GPP TS 23.287: "Architecture enhancements for 5G System (5GS) to support Vehicle-to-Everything (V2X) services".

[2] 3GPP TS 22.185: "Service requirements for V2X services; Stage 1".

[3] 3GPP TS 22.186: "Enhancement of 3GPP support for V2X scenarios; Stage 1".

[4] 3GPP TS 23.285: "Architecture enhancements for V2X services".

[5] 3GPP TS 23.501: "System Architecture for the 5G System; Stage 2".

[6] 3GPP TS 23.503: "Policy and Charging Control Framework for the 5G System; Stage 2".

[7] 3GPP TS 33.536: "Security aspects of 3GPP support for advanced Vehicle-to-Everything (V2X) services".

[8] 3GPP TS 24.587: "Vehicle-to-Everything (V2X) services in 5G System (5GS); Stage 3".

[9] 3GPP TS 24.588: "Vehicle-to-Everything (V2X) services in 5G System (5GS); User Equipment (UE) policies; Stage 3".

[10] 3GPP CP-192078: "WID: CT aspects of architecture enhancements for 3GPP support of advanced V2X services".

## 8.3	Application layer support for V2X services

| 840074 | Application layer support for V2X services | V2XAPP | S6 | SP-180898 | Niranth Amogh, Huawei Tel.India |
| --- | --- | --- | --- | --- | --- |
| 780025 | Study on V2XAPP | FS_V2XAPP | S6 | SP-171071 | Niranth Amogh, Huawei Tel.India |
| 810020 | Stage 2 of V2XAPP | V2XAPP | S6 | SP-180898 | Niranth Amogh, Huawei Tel.India |
| 840010 | CT aspects of V2XAPP | V2XAPP | ct | CP-191153 | Herrero Veron, Christian (Huawei) |
| 840075 | CT1 aspects of V2XAPP | V2XAPP | C1 | CP-191153 | Herrero Veron, Christian (Huawei) |
| 840076 | CT3 aspects of V2XAPP | V2XAPP | C3 | CP-191153 | Herrero Veron, Christian (Huawei) |
| 840077 | CT4 aspects of V2XAPP | V2XAPP | C4 | CP-191153 | Herrero Veron, Christian (Huawei) |

Summary not yet provided.

## 8.4	5G V2X with NR sidelink

| 830078 | 5G V2X with NR sidelink | 5G_V2X_NRSL | R1 | RP-190984 | LG Electronics |
| --- | --- | --- | --- | --- | --- |
| 830178 | Core part: 5G V2X with NR sidelink | 5G_V2X_NRSL-Core | R1 | RP-190766 | LG Electronics |
| 830278 | Perf. part: 5G V2X with NR sidelink | 5G_V2X_NRSL-Perf | R4 | RP-190766 | LG Electronics |

Summary based on the input provided by LG Electronics in RP-200855.

3GPP RAN technology for sidelink communication based on 5G NR was specified through this WI to define the means for providing the advanced V2X services identified by 3GPP SA1. This WI corresponds to 3GPP V2X phase 3, which is the evolution of LTE V2X in Release 14 (phase 1) and 15 (phase 2).

This section provides a summary of the key functionalities of NR sidelink.

Physical layer structure

Sidelink bandwidth part (BWP) is defined to support the flexible numerologies in operating on various spectrum band such as the intelligent transport system (ITS) dedicated band and the licensed band of frequency range 1 (FR1) and FR2. For sidelink synchronization, GNSS, gNB/eNB and the NR sidelink UE can be used as a synchronization reference source of a UE.

The NR V2X sidelink uses the following physical channels and signals:

-	Physical sidelink broadcast channel (PSBCH) and its de-modulation reference signal (DMRS)

-	Physical sidelink control channel (PSCCH) and its DMRS

-	Physical sidelink shared channel (PSSCH) and its DMRS

-	Physical sidelink feedback channel (PSFCH)

-	Sidelink primary and secondary synchronization signals (S-PSS and S-SSS)

-	Phase-tracking reference signal (PT-RS) in FR2

-	Channel state information reference signal (CSI-RS)

Sidelink control information (SCI) in NR V2X is transmitted in two stages. The first-stage SCI is carried on PSCCH and contains information to enable sensing operations, as well as information about the resource allocation of the PSSCH. PSSCH transmits the second-stage SCI and the sidelink shared channel (SL-SCH) transport channel. The second-stage SCI carries information needed to identify and decode the associated SL-SCH, as well as control for hybrid automatic repeat request (HARQ) procedures, and triggers for channel state information (CSI) feedback, etc. SL-SCH carries the transport block (TB) of data for transmission over SL.

PSCCH and PSSCH are multiplexed in time and frequency within a slot for short latency and high reliability. DRMS is frequency multiplexed with PSCCH or PSSCH in the corresponding DMRS symbols. PSFCH, which is used for sidelink HARQ feedback for unicast and groupcast, is transmitted at the end of a slot, which is preceded by an additional guard symbol and an automatic gain control (AGC) symbol. Two multiplexing examples are shown in Figure 1(a) and 1(b).

Figure 1(a) Example slot format of 2-symbol PSCCH, 2-symbol PSSCH-DMRS, and no PSFCH.

Figure 1(b) Example slot format of 3-symbol PSCCH, 3-symbol PSSCH-DMRS, and PSFCH.

Resource allocation

There are two resource allocation modes: mode 1 and mode 2. Mode 1 for resource allocation by gNB and Mode 2 for UE autonomous resource selection are very similar to Mode 3 and Mode 4 in LTE sidelink respectively. For mode 1, gNB schedules to UE the dynamic grant resources by downlink control information (DCI), or the configured grant resource type 1 and type 2 by radio resource control (RRC) signalling and DCI respectively.

In Mode 2, the sensing operation to determine transmission resources by UE comprises 1) sensing within a sensing window, 2) exclusion of the resources reserved by other UEs, and 3) select the final resources within a selection window. In Mode 2, shortly before transmitting in a reserved resource, a sensing UE re-evaluates the set of resources to check whether its intended transmission is still suitable, considering a possible aperiodic transmission after the resource reservation. If the reserved resources would not be part of the set for selection at this time, then new resources are selected from the updated resource selection window. In addition to the re-evaluation, pre-emption is also introduced such that a UE selects new resources even after it announces the resource reservation when it observes resource collision with a higher priority transmission from another UE.

Sidelink HARQ feedback, sidelink CSI and PC5-RRC for unicast and groupcast

NR sidelink supports sidelink HARQ-ACK for sidelink unicast and groupcast services for improved reliability. Two sidelink HARQ feedback operations are defined, HARQ-ACK with ACK and NACK, and HARQ-ACK with NACK only. When ACK/NACK operation is used, the sidelink HARQ-ACK procedure is similar to that of Uu for non-codeblock group feedback, i.e. the HARQ-ACKis transmitted based on the success or failure of the whole transport block. NACK-only operation is defined for groupcast to allow a a larger number of Rx UEs to share a single PSFCH resource by sending feedback only when a Rx UE receives SCI but fails to decode the transport block. The transmission of NACK-only feedback can be restricted to UEs within given a radius, and any UE beyond it does not provide any HARQ-ACK. This minimum range requirement of a service is provided together with the associated QoS parameters from service layers. For mode 1, sidelink HARQ-ACK information is reported to gNB to indicate whether additional retransmission resources are required or not.

In sielink unicast transmission, Tx UE can configure aperiodic sidelink CSI reporting from the Rx UE to get information it can use for sidelink link adaptation and rank adaptation. CQI and RI are reported via MAC layer signalling, in a PSSCH transmission for this purpose. In addition, radio link monitoring is adopted to manage a sidelink connection.

Figure 2 PC5 control plane (PC5-C) protocol stack for RRC.

To support exchange of the AS layer configuration and UE capability information between UEs, PC5-RRC is defined for unicast sidelink communication. The AS protocol stacks of the control plane for RRC is depicted in Figure 2.

Cross-RAT and in-device coexistence between LTE V2X and NR V2X sidelinks

Depending on the NR V2X and LTE V2X deployment, it is envisaged that an optional UE design can be supported where a device has both an LTE-V2X RAT and an NR-V2X RAT which are able to inter-communicate. 5G V2X defines two Cross-RAT operations. LTE Uu can control NR resource allocation mode 1 by providing configured grant Type 1 configurations via LTE RRC signalling, and resource allocation mode 2 by LTE Uu RRC providing the semi-static configurations relevant to resource pools, sensing, etc. NR Uu can control LTE resource allocation mode 3 by transmitting an NR DCI which contains the information needed to dynamically control the LTE sidelink, and resource allocation mode 4 by NR Uu RRC providing the necessary semi-static configurations within which the LTE-V2X RAT autonomously selects resources for sidelink transmission.

It is envisaged that there will exist devices that support both LTE-V2X and NR-V2X, and which will be operating in both systems concurrently. If the two RATs are widely spaced in frequency, e.g. being in different bands, then there need be no particular issues to consider since it is assumed that a separate RF chain will be provided for each band. If, however, a sufficiently close frequency spacing is deployed, then it is desirable to enable a single RF chain to be used in the implementation. In this case, the simultaneous transmission on both RATs is prevented by the UE's single power budget, and one RAT cannot be received/transmitted while the other RAT is doing the opposite. In this case, one of the RATs may be dropped at times when both occur simultaneously, but that in some cases where the priority of the V2X service on both RATs is known, the higher priority one is automatically selected.

References

Documents related to this WI are listed as follows:

-	Last approved work item description: RP-200129, “Revised WID on 5G V2X with NR sidelink”

-	Technical report: TR37.985, “Overall description of Radio Access Network (RAN) aspects for Vehicle-to-everything (V2X) based on LTE and NR (Release 16).”

-	Technical report: TR38.886, “V2X Services based on NR; User Equipment (UE) radio transmission and reception.”

-	Last status report: RP-200854

# 9	Northbound APIs related items

## 9.1	Usage of CAPIF for xMB API

| 790019 | Usage of CAPIF for xMB API | CAPIF4xMB | S4 | SP-180031 | Thorsten Lohmar, Ericsson LM |
| --- | --- | --- | --- | --- | --- |

Summary based on the input provided by Ericsson in SP-181187.

The CAPIF4xMB work item resulted in the creation of the xMB specification in TS.26.348 [1]. This TS contains the reference point specification for the external interface towards content providers or other 3GPP defined API invokers.

3GPP SA4 aligned the xMB reference point to the Common API Framework (CAPIF) within the CAPIF4xMB work item targeting Release 16. The xMB reference point exists between Content Providers (as API invoker) and the BM-SC. An API invoker can be a 3rd party provider or other 3GPP defined services.

xMB offers a simple interface to control the MBMS delivery procedure and to ingest the data to be delivered via MBMS. The MBMS usage complexity, e.g. handling of packet losses, etc is handled by the BM-SC.

xMB offers control procedures for different session types, namely generic file delivery, application (timed file streaming like DASH over MBMS), streaming and transport-only (access to a plain IP multicast transport). The xMB reference point offers an extended set of control features for MC Services.

As part of the CAPIF4xMB work item, the xMB Stage 2 architecture and procedures are moved from TS 26.346 to a new TS 26.348, containing the xMB Stage 2 and necessary extensions for CAPIF. The new TS also contains more elaborative guidelines around the usage of the different xMB API features.

References

[1]	TS 26.348, "Northbound Application Programming Interface (API) for Multimedia Broadcast/Multicast Service (MBMS) at the xMB reference point"

## 9.2	Enhancement of 3GPP Northbound APIs

| 840013 | Enhancement of 3GPP Northbound APIs | eNAPIs | C3 | CP-191233 | Yali Yan, Huawei |
| --- | --- | --- | --- | --- | --- |

Summary not yet provided

## 9.3	Enhancements for Common API Framework for 3GPP Northbound APIs

| 830069 | Enhancements for Common API Framework for 3GPP Northbound APIs | eCAPIF |  | SP-181137 | Basavaraj Pattan, Samsung |
| --- | --- | --- | --- | --- | --- |
| 790022 | Stage 2 for eCAPIF | eCAPIF | S6 | SP-181137 | Basavaraj Pattan, Samsung |
| 830022 | Security aspects of eCAPIF | eCAPIF | S3 | SP-1901240 | Rajavelsamy Rajadurai, Samsung |

Summary not yet provided

## 9.4	Service Enabler Architecture Layer for Verticals

| 820027 | Service Enabler Architecture Layer for Verticals | SEAL | S6 | SP-181141 | Basavaraj (Basu) Pattan, Samsung |
| --- | --- | --- | --- | --- | --- |

Summary not yet provided

## 9.5	Other APIs-related items

See also:

Cellular IoT support and evolution for the 5G System

Business Role Models for Network Slicing

Enhancing Topology of SMF and UPF in 5G Networks

MBMS APIs for Mission Critical Services

# 10	Coexistence with Non-3GPP systems

## 10.1	Wireless and Wireline Convergence Enhancement

| 760044 | Study on the Wireless and Wireline Convergence for the 5G system architecture | FS_5WWC | S2 | SP-170380 | Marco Spini, Huawei Technologies |
| --- | --- | --- | --- | --- | --- |
| 800031 | Study on the security of the Wireless and Wireline Convergence for the 5G system architecture | FS_5WWC_SEC | S3 | SP-180438 | He Li, Huawei Technologies |
| 830050 | Wireless and Wireline Convergence for the 5G system architecture | 5WWC |  | SP-181117 | Marco Spini, Huawei Technologies |
| 820014 | Stage 2 of 5WWC | 5WWC | S2 | SP-181117 | Marco Spini, Huawei Technologies |
| 829910 | CT aspects of 5WWC | 5WWC | ct | CP-191156 | Herrero Veron, Christian (Huawei) |
| 829911 | CT1 aspects of 5WWC | 5WWC | C1 | CP-191156 | Herrero Veron, Christian (Huawei) |
| 829912 | CT3 aspects of 5WWC | 5WWC | C3 | CP-191156 | Herrero Veron, Christian (Huawei) |
| 829914 | CT4 aspects of 5WWC | 5WWC | C4 | CP-191156 | Herrero Veron, Christian (Huawei) |
| 830082 | NG interface usage for WWC | 5WWC-NG_interface | R3 | RP-190772 | Huawei |
| 830182 | Core part: NG interface usage for WWC | 5WWC-NG_interface-Core | R3 | RP-190999 | Huawei |
| 860019 | Charging Aspect for 5WWC | 5WWC | S5 | SP-191188 | ZHU, Lei, Huawei |

Summary based on the inputs provided by Huawei in SP-200253 for the overall aspects; by Huawei and HiSilicon in RP-200678 for the radio aspects; and by Huawei in SP-200525 for the charging aspects.

[Editor's note: three different summaries (SP-200253, RP-200678 and SP-200525) have been provided to cover 5WWC. These are to be further merged]

Overall aspects (from SP-200253)

This WI aimes to enhance 5G core network to support connection of residential gateway connected via wireline access network and via 3GPP RAN access. Furthermore the WI aimes to support Non-3GPP network as Trusted network in contrast with Untrusted network. The main features introduced by the WI includes the specification of new access network node, called W-AGF (Wireline Access Gateway function), the improvement of registration and session management procedures, policy, QoS etc,  tailored to the specific characteristic of wireline access network. The main future for supporting Trusted network includes the definition of architecture with new Trusted Gateway Network Function, selection procedure of such gateway, improvement and extension of registration and session procedures, policy , etc for supporting such scenario.

The description of support of Wireline Access Network and Trusted Non-3GPP Access Network are described in the following clauses.

Support of wireline access network

The architecture for non-roaming is shown in figure 1, where the Wireline Access Gateway Function (W-AGF) is the access node performing the termination of N2 and N3 reference point, termination of access network interface Y4 and all access network specify functionalities, the relay of N1 to/from the UE., QoS enforcement, etc. The customer device, the UE, is replaced by the Residential Gateway which is augmented to support the 5G functionalities required to connect to 5G systems, such as NAS, URSP, PDU session, etc, called 5G-RG. The specification in TS 23.316 [1] defines the modification to system architecture, procedure and flows, Policy and Charging Control for the 5G System in TS 23.501 [2], TS 23.502 [3] and TS 23.503 [4].

The 5G-RG can also be connected via 3GPP Access basically by means of supporting the specification defined for UE. This scenario is called Fixed Wireless Access (FWA). Furthermore the 5G-RG may simultaneously connect to 3GPP Access and to wireline access by using the Single Access PDU session or supporting ATSSS feature. This scenario is called Hybrid scenario, using a terminology common on wireline access network. The ATSSS is supported as specified in TS 23.501, 23.502 and TS 23.503 where UE is replaced by 5G-RG and the Non-3GPP access (es) is specifically referred to wireline access. In this latter case, TS 23.316 has also specified the support of interworking with EPC via 3GPP Access via a MA PDU session with a PDN Connection as user-plane resource associated with a MA PDU Session.

The support of legacy Residential Gateway not supporting 5G capability (FN-RG) is supported via W-AGF terminating the N1 NAS on behalf of UE and acting as a UE in respect the 5G core. In this scenario the

In the case of Wireline Access Network defined in Broadband Forum the W-AGF functionalities is specified in BBF TR-470 [5], BBF TR-456 [6] and BBF TR-457 [7], the 5G-RG is defined in BBF TR-124issue6 [8]. In the case of Wireline Access network defined in Cablelabs the W-AGF and 5G-RG functionalities are defined in CableLabs WR-TR-5WWC-ARCH [9].

Main impacts on the system by the WWC Work Item for wireline support are the following:

-	W-AGF: the access network function which performs the termination of N2 and N3 reference point, termination of access network interface Y4 and all access network specify functionalities, the relay of N1 to/from the UE. QoS enforcement, etc. When the W-AGF facing the FN-RG the W-AGF is supporting the termination of N1 NAS and performs the interworking between 5GC and the legacy wireline access network.

-	5G-RG: end user device replacing the UE which supports 5G capabilities (NAS protocol and procedure, USRP, IMSI, ATSSS) and extension of wireline access layer specific functionalities defined by Broadband forum and CableLabs. The 5G-RG may also support UE capability when connects via 3GPP Access.

-	FN-RG: end user device replacing the UE which does not support 5G capabilities.

-	Global Line Identifier (GLI): in case of wireline access based on BBF specifications this parameter uniquely identifies the line at which the 5G-RG in connected to within an operator domain.

-	Global Cable identifier (GCI): in case of wireline access based on CableLabs specification this parameter uniquely identifies the line at which the 5G-RG in connected to within an operator domain.

-	SUPI for FN-RG based on GCI and GLI.

-	All procedures defined in TS 23.502 have been modified to introduce the new network elements. The procedures are focused mainly on the part of specification that required improvements and to point out the access network interaction involving the W-AGF, 5G-RG and FN-RG to allow the Broadband Forum and CableLabs to develop the specifications under their responsibility.

-	IPTV support: The specification TS 23.316 in clauses 4.9.1 and 7.7.1 defines the support of IPTV via the support of multicast over unicast PDU session by using IGMP/MLD message send by STB via 5G-RG on PDU session and managed by UPF for adding the requiring 5G-RG to a multicast group and replicating the traffic received on N6 interface to the PDU session. The SMF is improved to control the support of IPTV by the UPF acting as PSA using PDR, FAR, QER, URR. This includes control of which IGMP and MLD requests the UPF is to accept or to deny.

-	QoS: the QoS model for wireline network is based on a subscription maximum aggregate bitrate including both GBR and Non-GBR traffic, hence the new parameter RG Total Maximum Bit Rate (RG-TMBR) has been defined. The RG-TMBR limits the aggregate bit rate that can be expected to be provided across all GBR and Non-GBR QoS Flows of a RG. The RG-TMBR is a parameter provided to the W-AGF by the AMF based on the value of the Subscribed RG-TMBR retrieved from UDM. The QoS control on wireline access network (i.e scheduling, rate limiting and traffic class management) is based on the line characteristic included in user subscription, for example different priority of service, different traffic class support by line of the single user, etc, for such reason the new parameter RG Level Wireline Access Characteristics (RG-LWAC) has been introduced. The format and content of RG LWAC is specified by BBF and it is transparently provided by UDM to AMF which may provide to the W-AGF at the time of the RG registration

-	mobility restriction based on GLI and GCI

-	support of BBF interaction with the Access Configuration System (ACS) to support the provisioning of configuration and remote management of 5G-RG as described in BBF TR-069 [12] or in BBF TR-369 [13].

Figure 1: Non- roaming architecture for 5G Core Network for 5G-RG with Wireline 5G Access network and NG RAN

Figure 2: Non- roaming architecture for 5G Core Network for FN-RG with Wireline 5G Access network and NG RAN

Support of Trusted Access network

The support of Trusted Network addresses the scenario where the Non-3GPP access network has a tighter relationship with 5GC in respect the untrusted scenario. However how the network is considered Trusted or Untrusted is not in the scope of this WID. The architecture for non-roaming is shown in figure 3, where the Trusted Non-3GPP Access Network (TNAN) is the access node performing the termination of N2 and N3 reference point, termination of access network interface, relay of N1 to/from the UE. From 3GPP point of view the TNAN network is composed by the TNGF and the Trusted Non-3GPP Access Point (TNAP) which are interconnected via the reference point Ta. However the detailed definition of TNAN and of Ta is beyond the WID scope. The reference point between the UE and the TNG, the NWt, is specified leveraging the IKEv2 defined for Untrusted. The main difference in contrast to Untrusted is in registration procedure, where it is assumed that EAP-5G can be carried between UE and TNAP directly on access layers, such on IEEE 802.11x and between TNAP and TNGF via Ta and not as part of IKEv2 establishment.  From other the point of view of other procedures, such as session management, the same procedure specified for Untrusted Non-3GPP access network can be used with basically the TNGF replacing the N3IWF, and modification that IKEv2 Child SA establishment is requested by TNGF and not by UE side.

Within the context of Trusted Non-3GP network, also the scenario of devices not supporting NAS connected via WLAN is specified. The role of TNGF is replaced the Trusted WLAN Interworking Function (TWIF) with the main difference that TWIF terminates the N1 NAS interface and it play the role of UE in respect the 5GC.

The specification is addressed in TS 23.501 [2], TS 23.502 [3] and TS 23.503 [4]

Figure 3: Non-roaming architecture for 5G Core Network with trusted non-3GPP access

References (for the general (non-radio) aspects)

[1]	3GPP TS 23.316: "Wireless and wireline convergence access support for the 5G System (5GS)".

[2]	3GPP TS 23.501: "System Architecture for the 5G System; Stage 2".

[3]	3GPP TS 23.502: "Procedures for the 5G System; Stage 2".

[4]	3GPP TS 23.203: "Policies and Charging control architecture; Stage 2"..

[5]	BBF WT-470: "5G FMC Architecture".

[6]	BBF WT-456: "AGF Functional Requirements".

[7]	BBF WT-457: "FMIF Functional Requirements".

[8]	BBF TR-124 issue 5: "Functional Requirements for Broadband Residential Gateway Devices".

[9]	CableLabs WR-TR-5WWC-ARCH: "5G Wireless Wireline Converged Core Architecture".

[10]	CableLabs WR-TR-5WWC-ARCH: "5G Wireless Wireline Converged Core Architecture".

[11]	CableLabs WR-TR-5WWC-ARCH: "5G Wireless Wireline Converged Core Architecture".

[12]	BBF TR-069: "CPE WAN Management Protocol".

[13]	BBF TR-369: "User Services Platform (USP)".

Radio aspects (from RP-200678)

In Rel-16, the WI “NG interface usage for WWC (Wireless Wireline Convergence)” was approved (The latest WID in RP-190999 [1]). The objective of this WID is to enhance the NG interface protocols specified in TS 29.413 [2], to fulfil the Stage 2 requirements allowing the Trusted Non-3GPP Access and the Wireline Access connectivity with the 5GC as described in the Release 16 of TS 23.316 [3], TS 23.501 [4], TS 23.502 [5] and TS 23.503.

The detailed objective includes:

•	The description and enhancement of NG protocols to support the interface between the Trusted Non-3GPP Access Network and the 5GC;

•	The description and enhancement of NG protocols to support the interface between the Wireline 5G Access Network and the 5GC.

It is also noted that the possible impact on TS 38.413 and others NG protocols is not precluded.

This document is the summary of the outcome of the WI, and the final agreed BL CRs for TS 29.413 and TS 38.413 can be found in [5, 6].

General radio aspects

-	Introduce the Trusted Non-3GPP Gateway Function (TNGF), Trusted WLAN Interworking Function (TWIF) to support the Trusted Non-3GPP Access, and Wireline Access Gateway Function (W-AGF) to support Wireline Access in TS 29.413 and TS 38.413.

-	Add the Global TNGF ID in the applicable NGAP messages between the TNGF and the AMF; add the Global TWIF ID in the applicable NGAP messages between the TWIF and the AMF; add the Global W-AGF ID in the applicable NGAP messages between the W-AGF and the AMF.

-	Add the selected PLMN Identity for trusted non-3GPP access and wireline access in Initial UE Message for Key derivation.

-	Add procedural texts that the Security Key IE may include KTNGF, or KTWIF, or KWAGF in TS 29.413.

Supporting the Trusted Non-3GPP Access with the 5GC – specific aspects

-	Add TNGF Identity Information, TWIF Identity Information in the UPLINK NAS TRANSPORT message containing a list of identifiers of NG-U terminations at TNGF/TWIF for UPF selection.

-	Add TNGF related and TWIF related User Location Information in the User Location Information IE.

Supporting the Wireline Access connectivity with the 5GC – specific aspects

-	Add W-AGF Identity Information in the UPLINK NAS TRANSPORT message containing a list of identifiers of NG-U terminations at W-AGF for UPF selection.

-	Add W-AGF related User Location Information in the User Location Information IE.

-	Add procedural texts to clarify the UE-AMBR is not used for wireline access in TS 29.413.

-	Add RG Level Wireline Access Characteristics in INITIAL CONTEXT SETUP REQUEST messages stored in the UE context by the W-AGF, indicating the wireline access technology specific QoS information corresponding to a specific wireline access subscription.

-	Add the Authenticated Indication in INITIAL UE MESSAGE to indicate that the FN-RG has been authenticated by the wireline 5G access network.

References (for the radio aspects)

[1]	RP-190999 Revised WID on NG interface usage for WWC (Wireless Wireline Convergence), Huawei, HiSilicon

[2]	3GPP TS 23.316: "Wireless and wireline convergence access support for the 5G System (5GS)".

[3]	3GPP TS 23.501: "System Architecture for the 5G System".

[4]	3GPP TS 23.502: "Procedures for the 5G System".

[5]	R3-204444	CR for introducing WWC in RAN, Huawei

[6]	R3-204445	CR for introducing WWC in RAN, Huawei

Charging aspects (from SP-200525)

The work item is to specify charging aspect on Wireless and Wireline Convergence for 5G system architecture (5WWC). The 5WWC is specified in TS 23.501, TS 23.502, TS 23.503 and TS 23.316. The enhancement to charging aspect for 5WWC is considered as part of this series specifications for this 5WWC.

Following charging scenarios are included in charging aspect of 5WWC as following:

-	UE Connects to 5G Core via Trusted Non-3GPP access

-	5G-RG connects to 5G Core via NR-RAN and via W-5GAN

-	FN-RG connects via W-5GAN

This work item specifies charging requirements, procedures related to charging, charging information and related triggers for chargeable event for 5WWC scenarios.

The specifications related to 5WWC charging include TS 32.255, TS 32.291 and TS 32.298. The subscriber’s identifiers and PEI in 5G-RG and FN RG scenarios specified in TS 23.501 and TS 23.361 are used in charging information. The procedures and related triggers in 5WWC charging scenarios are also specified in charging aspect for 5WWC. The related changes to OpenAPI are specified in TS 32.291.

## 10.2	Access Traffic Steering, Switch and Splitting support in the 5G system architecture

| 820044 | Access Traffic Steering, Switch and Splitting support in the 5G system architecture | ATSSS |  | SP-181124 | So, Tricci, ZTE |
| --- | --- | --- | --- | --- | --- |
| 760052 | Study on ATSSS | FS_ATSSS | S2 | SP-180732 | So, Tricci, ZTE |
| 820021 | Stage 2 of ATSSS | ATSSS | S2 | SP-181124 | So, Tricci, ZTE |
| 830016 | CT aspects of ATSSS | ATSSS | ct | CP-190201 | ZHOU, Xingyue (Joy), ZTE |
| 830057 | CT1 aspects of ATSSS | ATSSS | C1 | CP-190201 | ZHOU, Xingyue (Joy), ZTE |
| 830058 | CT3 aspects of ATSSS | ATSSS | C3 | CP-190201 | ZHOU, Xingyue (Joy), ZTE |
| 830059 | CT4 aspects of ATSSS | ATSSS | C4 | CP-190201 | ZHOU, Xingyue (Joy), ZTE |
| 840999 | (IETF) TCP Extensions for Multipath Operation with Multiple Addresses (draft-ietf-mptcp-rfc6824bis) | ATSSS | S2-IETF |  |  |
| 840998 | (IETF) 0-RTT TCP Convert Protocol (draft-ietf-tcpm-converters) | ATSSS | S2-IETF |  |  |

Summary based on the input provided by ZTE Wistron Telecom AB in SP-200124.

The ATSSS feature enables a multi-access PDU Connectivity Service, which can exchange PDUs between the UE and a data network by simultaneously using one 3GPP access network and one non-3GPP access network and two independent N3/N9 tunnels between the PSA and RAN/AN. The multi-access PDU Connectivity Service is realized by establishing a Multi-Access PDU (MA PDU) Session, i.e. a PDU Session that may have user-plane resources on two access networks, as shown on the figure below, extracted from TR 23.793 [1].

Figure 1: MA PDU session

These following procedures are defined in the context of this Feature:

-	Access Traffic Steering: it selects an access network for a new data flow and transfers the traffic of this data flow over the selected access network. Access traffic steering is applicable between one 3GPP access and one non-3GPP access.

-	Access Traffic Switching: it moves all traffic of an ongoing data flow from one access network to another access network in a way that maintains the continuity of the data flow. Access traffic switching is applicable between one 3GPP access and one non-3GPP access.

-	Access Traffic Splitting: it splits the traffic of a data flow across multiple access networks. When traffic splitting is applied to a data flow, some traffic of the data flow is transferred via one access and some other traffic of the same data flow is transferred via another access. Access traffic splitting is applicable between one 3GPP access and one non-3GPP access.

Key concepts of ATSSS supported in Release 16 include the following:

-	Multi-access PDU Session is a PDU Session that provides a PDU connectivity service, which can use one access network at a time, or simultaneously one 3GPP access network and one non-3GPP access network and two independent N3/N9 tunnels between the PSA and RAN/AN.

-	After the establishment of a MA PDU Session:

-	When there are user-plane resources on both access networks:

-	The UE applies network-provided policy (i.e. ATSSS rules derived by UE’s serving SMF based on ATSSS policy from serving PCF) and considers local conditions (such as network interface availability, signal loss conditions, user preferences, etc.) for deciding how to distribute the uplink traffic across the two access networks.

-	Similarly, the UPF anchor of the MA PDU Session applies network-provided policy (i.e. N4 rules derived by UE’s serving SMF based on ATSSS policy from serving PCF) and the feedback information received from the UE via the user-plane (such as access network Unavailability or Availability), the UPF then decides how to distribute the downlink traffic across the two N3/N9 tunnels and two access networks.

-	When there are user-plane resources on only one access network, the UE applies the ATSSS rules and considers local conditions for triggering the establishment or activation of the user plane resources over another access.

-	The type of a MA PDU Session may be one of the following types: i.e. IPv4, IPv6, IPv4v6, and Ethernet. The Unstructured type is not supported in Release 16.

-	The ATSSS feature can be supported over 3GPP and non-3GPP accesses, including untrusted and trusted non-3GPP access networks, wireline 5G access networks, etc., as long as a MA PDU Session can be established over the given type of access network

-	Two ATSSS steering functionalities are supported:

-	MPTCP functionality, for TCP traffic, with MPTCP proxy in UPF, by using the MPTCP protocol over the 3GPP and/or the non-3GPP user plane; and

-	ATSSS-LL functionality for all types of traffic, including TCP traffic, UDP traffic, Ethernet traffic, etc. ATSSS-LL functionality is mandatory for MA PDU Session of type Ethernet.

The following presents the example of the ATSSS traffic steering functionality within the UE.

Figure 2: Steering functionalities in an example UE model

-	The Performance Measurement Function (PMF) is supported by UPF and is specific for ATSSS-LL functionality, if enabled.  In Release 16, PMF supports two types of measurements between UE and UPF to assist access selection and they are:

-	UE and UPF make RTT measurements per access when the "Smallest Delay" steering mode is used; and

-	UE reports access availability/unavailability to UPF

The following presents the protocol stacks of the PMF for the user plane measurements over 3GPP and non-3GPP accesses respectively.

Figure 3: UE/UPF measurements related protocol stack for 3GPP access and for an MA PDU Session with type IP

In the case of an MA PDU Session with type Ethernet, the protocol stack over 3GPP access is that same as the one in the above figure, but the PMF protocol operates on top of Ethernet, instead of UDP/IP.

Figure 4: UE/UPF measurements related protocol stack for non-3GPP access and for an MA PDU Session with type IP

In the case of an MA PDU Session with type Ethernet, the protocol stack over non-3GPP access is that same as the one in the above figure, but the PMF protocol operates on top of Ethernet, instead of UDP/IP.

-	An ATSSS-capable UE may decide to request a MA PDU Session based on the provisioned URSP rules. In particular, the UE should request a MA PDU Session when the UE applies a URSP rule, which triggers the UE to establish a new PDU Session and the Access Type Preference component of the URSP rule indicates "Multi-Access".

-	The 5G QoS model for the Single-Access PDU Session is also applied to the MA PDU Session, i.e. the QoS Flow is the finest granularity of QoS differentiation in the MA PDU Session. One difference compared to the Single-Access PDU Session is that in a MA PDU Session there can be separate user-plane tunnels between the AN and the PSA, each one associated with a different access. The SMF shall provide the same QFI in 3GPP and non-3GPP accesses so that the same QoS is supported in both accesses. Non GBR QoS Flow can be distributed over 3GPP access and non 3GPP access, but GBR QoS Flow is transferred over single access.

-	ATSSS is currently not supported when moving to EPC from 5GC, except for the specific case with wireline access integrated to EPC/5GC with 5G-RG; ATSSS with one User Plane leg in E-UTRA/EPC and one User Plane leg in wireline/5GC is supported.

References

[1]	3GPP TR 23.793: "Study on ATSSS".

[2]	3GPP TS 23.501: "System Architecture for the 5G System; Stage 2".

[3]	3GPP TS 23.502: "Procedures for the 5G System; Stage 2".

[4]	3GPP TS 23.503: "Policy and Charging Control Framework for the 5G System; Stage 2".

# 11	Railways and Maritime

## 11.1	Mobile Communication System for Railways 2

| 760054 | Mobile Communication System for Railways 2 | MONASTERY2 |  | SP-170451 | Merkel, Juergen, Nokia |
| --- | --- | --- | --- | --- | --- |
| 760005 | Study on MONASTERY2 | FS_FRMCS2 | S1 | SP-170588 | Merkel, Juergen, Nokia |
| 790023 | Study on application architecture for the Future Railway Mobile Communication System (FRMCS) Phase 2 | FS_FRMCS2_SA6 | S6 | SP-181133 | Oettl, Martin, Nokia |
| 760004 | Stage 1 of MONASTERY2 | MONASTERY2 | S1 | SP-170451 | Merkel, Juergen, Nokia |
| 830007 | Application Architecture for MONASTERY2 | MONASTERY2 | S6 | SP-190064 | Oettl, Martin, Nokia |
| 840009 | CT aspects of MONASTERY2 | MONASTERY2 | ct | CP-191152 | Gkatzikis, Lazaros (Nokia) |
| 840072 | CT1 aspects of MONASTERY2 | MONASTERY2 | C1 | CP-191152 | Gkatzikis, Lazaros (Nokia) |
| 840073 | CT3 aspects of MONASTERY2 | MONASTERY2 | C3 | CP-191152 | Gkatzikis, Lazaros (Nokia) |

Summary not yet provided

## 11.2	Further performance enhancement for LTE in high speed scenario

| Unique_ID | Name | Acronym | WG | WID | WI Rapporteur |
| --- | --- | --- | --- | --- | --- |
| 800079 | Further performance enhancement for LTE in high speed scenario | LTE_high_speed_enh2 | R4 | RP-181482 | NTT DOCOMO |

Summary based on the input provided by NTT DOCOMO, INC. in RP-192721.

In Rel-13 and 14, the mobility and throughput performance were enhanced to cover high speeds (up to 350 km/h) by specifying the requirements for UE RRM, UE demodulation and base station demodulation, considering the two types of operator’s practical deployments shown in Figures 1 and 2. Figure 1 shows the case where no specific installation is deployed to handle high-speed trains, i.e. UEs in the train use the "standard" LTE eNBs. Alternatively, figure 2 shows the case where Single Frequency Network (SFN) are deployed. SFNs use so-called "Remote Radio Heads" (RRH), which are dedicated antennas deployed along the train track. In this case, the baseband unit (BBU) is connected to the RRH, e.g. using fiber.

Figure 1. Non-Single Frequency Network (SFN) high speed scenario

Figure 2. SFN high speed scenario

These Rel-13 and 14 enhancements were conducted both for non-SFN and for SFN, but only for LTE single carrier, i.e. not covering Carrier Aggregation (CA).

This Rel-16 work item further improves the mobility and throughput performance, now considering CA and speeds up to 500 km/h. To this aim, it enhances RRM, UE demodulation and base station demodulation: it specifies enhanced RRM core requirements and corresponding RRC signals in respectively TS 36.133 [2] and TS 36.331 [3].

Part 1: RRM requirements enhancements:

In Release 14 cases (limited to 350 km/h and single carrier), the latency requirements under DRX configuration up to 1.28s DRX cycle were enhanced by reducing the cell identification delay in connected mode and cell reselection delay in idle mode [1].

In Rel-16, considering Carrier Aggregation and speeds up to 500km/h, the following enhanced requirements were introduced to achieve good mobility performance and less paging outage:

- Enhanced RRM requirements for active SCells (for 350km/h velocity):

The same requirements specified in Rel-14 high speed WI are applied to active SCells.

- Enhanced RRM requirements for deactivated SCells (for 350km/h velocity):

The cell identification delay and measurement period are reduced.

- Enhanced RRM requirements in DRX in connected mode (for 500km/h velocity):

The cell identification delay and measurement period on 1.28s DRX cycle are further reduced from those in Rel-14 high speed WI.

- Enhanced RRM requirements in idle mode (for 500km/h velocity):

The cell detection delay is further reduced from those in Rel-14 high speed WI.

- Enhanced UL timing adjustment requirements in connected mode (for 500km/h velocity):

The larger maximum autonomous time adjustment step is applied when the downlink bandwidth is wider than 10MHz.

Part 2: Demodulation enhancements

- For UE and base station demodulation enhancements

In Release 14, UE and base station demodulation requirements were enhanced [1], for both cases of operator’s practical deployments shown in figures 1 and 2.

In Release 16, regarding the CA case in SFN (figure 2), the requirements specified in Rel-14 are expanded to Dual Connectivity's Secondary Cells (SCells) as defined in TS 36.331 [3]. Regarding further high speed up to 500 km/h, additional requirements are introduced to ensure the PDSCH/PUSCH/PRACH demodulation performance with larger Doppler shift.

The impacts on RRC signals to meet the new requirements above are also specified.

References

[1]	RP-160172, Rel-14 WID on "Performance enhancements for high speed scenario in LTE"

[2]	TS 36.133, “Requirements for support of radio resource management.”

[3]	TS 36.331, “Radio Resource Control.”

## 11.3	NR support for high speed train scenario

| 840092 | NR support for high speed train scenario | NR_HST | R4 | RP-191512 | CMCC |
| --- | --- | --- | --- | --- | --- |
| 840192 | Core part: NR support for high speed train scenario | NR_HST-Core | R4 | RP-191512 | CMCC |
| 840292 | Perf. part: NR support for high speed train scenario | NR_HST-Perf | R4 | RP-191512 | CMCC |

Summary based on the input provided by CMCC in RP-200776.

This WI targets to specify the UE RRM requirements, UE/BS demodulation requirements for high speed train scenario. The supported velocity is up to 500km/h and the carrier frequency is up to 3.6GHz covering both TDD and FDD.

In Rel-16 WI on NR support of high speed train scenarios, the enhanced RRM requirements and demodulation requirements were specified to support the speed of up to 500km/h and carrier frequency is up to 3.6GHz.

For RRM, to guarantee the mobility performance for the scenario with velocity up to 500km/h，both enhanced requirements for NR intra-RAT measurement and enhanced requirements for inter-RAT measurement between NR and EUTRA are specified. The enhanced requirements for NR intra-RAT include NR cell re-selection requirements, NR cell identification requirements, beam management requirements. The enhanced requirements for inter-RAT measurement between NR and EUTRA include EUTRA inter-RAT measurement requirements and NR inter-RAT measurement requirements, both idle mode and connected mode are considered.

For UE demodulation, HST-SFN (multiple RRHs are connected to one BBU with fiber), HST single tap and multi-path fading channel are considered. For HST-SFN, the maximum doppler shift is 1667Hz (500km/h + 3.6GHz) and 870Hz for 30KHz SCS and 15KHz SCS respectively. For HST single tap, the maximum doppler shift is 1667Hz and 972Hz for 30KHz SCS and 15KHz SCS respectively. For multi-path fading channel, the maximum doppler shift is 1200Hz and 600Hz for 30KHz SCS and 15KHz SCS respectively. For BS demodulation, at least HST single tap is considered. And the maximum doppler shift is 3334Hz and 1740Hz for 30KHz SCS and 15KHz SCS respectively.

References

List of related CRs: select "TSG Status = Approved" in: https://portal.3gpp.org/ChangeRequests.aspx?q=1&workitem=840192,840292

## 11.4	Maritime Communication Services over 3GPP System

| 800011 | Maritime Communication Services over 3GPP System | MARCOM | S1 | SP-180594 | Hyounhee Koo, SyncTechno Inc |
| --- | --- | --- | --- | --- | --- |
| 730007 | Study on Maritime Communication Services over 3GPP system | FS_MARCOM | S1 | SP-170453 | SyncTechno Inc., Hyounhee Koo |
| 800051 | Stage 1 of MARCOM | MARCOM | S1 | SP-180594 | Hyounhee Koo, SyncTechno Inc |

Summary based on the input provided by SyncTechno Inc. in SP-200344.

The MARCOM feature introduces service requirements to enable maritime communication services to be supported over 3GPP system in 3GPP TS 22.119 [1]. With the MARCOM feature, the work on maritime communication services is officially included in the scope of 3GPP standardization so 3GPP enabling technologies can be applicable to for maritime usage from 3GPP Release 16 onwards.

The first 3GPP Technical Specification (TS) 22.119 [1] covering service requirements (Stage 1) is specified for the support of maritime communication (MARCOM) over 3GPP systems.

The maritime domain, one of the 5G vertical domains in 3GPP, is moving forward with the digitalisation and mobilisation of commercial as well as safety fields. Legacy 3GPP-based technologies and solutions can be beneficial to the digitalisation and mobilisation of the maritime domain though some of the legacy 3GPP enabling technologies and solutions may not be able to fully support the performances required by the maritime domain. The maritime radio environment was not originally considered by 3GPP when the technical specifications and solutions were standardised for LTE and 5G.

However, most of the legacy mobile services and IoT services based on capabilities of EPS and 5GS specified in 3GPP specifications are applicable to maritime usage for the support of mobile broadband services, and for the support of IoT services or machine-type communication services in a vessel at sea.

In addition, there are service scenarios and requirements specified in 3GPP specifications based on requirements of other related vertical domains (e.g. public safety domain, automotive domain, factory automation domain, and satellite industrial domain). Some requirements derived by service scenarios from these related vertical domains are applicable to the maritime domain. Thus, it is beneficial to use 3GPP enabling technologies developed to satisfy those requirements for the maritime domain in terms of the economy of scale.

For example, satellite access is one of the 3GPP radio access networks supported over the 5G system, so it is possible to provide seamless maritime mobile services by integrating multiple access technologies including satellite access depending on the service scenarios. In addition, Vertical LAN that can replace Ethernet-based access are applicable to indoor maritime mobile services inside a vessel.

MC Services specified in 3GPP specifications are applicable to commercial and maritime safety fields. Some similarities exist between the public safety domain and the maritime domain in terms of service scenarios that are essentially the same. For example, in some situations, mobile communication services are supported in spite of disconnected networks, i.e. off-network mode, or under isolated conditions.

However, the maritime domain also has specific situations that do not happen in other vertical domains or in the legacy ICT industrial domain. New 3GPP enabling technologies dedicated to the maritime domain can be used to address such specific situations based on requirements derived from maritime use cases. Other vertical domains may benefit from such new 3GPP enabling technologies that consider maritime domain scenarios and may need more robust technologies or solutions than those that currently exist for those vertical domains.

The stage 1 requirements in 3GPP TS 22.119 [1] are specific to maritime usage over 3GPP system for commercial as well as safety purposes. The stage 1 requirements derived from specific maritime usage but also related to MCX Services are specified in 3GPP technical specifications dedicated to MCX Services [2] [3] [4] [5].

References

[1] TS 22.119, Maritime communication services over 3GPP system

[2] TS 22.179, Mission Critical Push to Talk (MCPTT); Stage 1

[3] TS 22.280, Mission Critical (MC) services common requirements

[4] TS 22.281, Mission Critical (MC) video

[5] TS 22.282, Mission Critical (MC) data

# 12	Mission Critical, Public Warning

## 12.1	Enhancements of Public Warning System

| 780003 | Enhancements of Public Warning System | ePWS |  | SP-170998 | Hyounhee Koo, SyncTechno Inc. |
| --- | --- | --- | --- | --- | --- |
| 730005 | Study on enhancements of Public Warning System | FS_ePWS | S1 | SP-160733 | SyncTechno Inc., Hyounhee Koo |
| 800052 | Stage 1 of ePWS | ePWS | S1 | SP-170998 | Hyounhee Koo, SyncTechno Inc. |
| 810012 | CT aspects of enhancements of Public Warning System | ePWS | ct | CP-191155 | Hyounhee Koo, SyncTechno Inc. |
| 810053 | Study on stages 2 and 3 of enhancements of ePWS | ePWS | C1 | CP-191155 | Hyounhee Koo, SyncTechno Inc. |
| 810047 | CT1 aspects of ePWS | ePWS | C1 | CP-191155 | Hyounhee Koo, SyncTechno Inc. |
| 810048 | CT4 aspects of ePWS (Possible impacts) | ePWS | C4 | CP-191155 | Hyounhee Koo, SyncTechno Inc. |

Summary based on the input provided by SyncTechno Inc. in SP-200343.

The ePWS feature enhances the Public Warning System by defining behaviours for UEs with no user interface or with a user interface that is incapable of displaying text-based Warning Notifications and providing how to improve the comprehension of a Warning Notification to users with disabilities who have UEs supporting assistive technologies beyond text assistive technologies and users who are not fluent in the language of the Warning Notifications. In addition, additional requirements are specified for PWS-UEs and ePWS-UEs that play the role of a relay UE or a remote UE to conform behaviours when receiving a Warning Notification via the relay functionality. The requirements can be found in 3GPP TS 22.268 [1] and corresponding solutions in 3GPP TS 23.041 [2].

Additional requirements for an enhanced Public Warning System (ePWS) are specified as an update to Technical Specification (TS) 22.268 [1].

3GPP Public Warning Systems were first specified in Release 8, allowing for direct warnings to be sent to mobile users on conventional User Equipment (PWS-UE), capable of displaying a text-based and language-dependent Warning Notification.

Since that time, there has been a growth in the number of mobile devices with little or no user interface - including wrist bands, sensors and cameras – many of which are not able to display Warning Notifications. The recent growth in the number of IoT devices - not used by human users – also highlights the need for an alternative to text based Warning Notifications. If those devices can be made aware of the type of incident (e.g. a fire or flood) in some other way than with a text message, then they may take preventive actions (e.g. lift go to ground floor automatically).

The ePWS feature also specifies how graphical symbols or images can now be used to better disseminate Warning Notifications, specifically aimed at the following categories of users:

-	Users with disabilities who have UEs supporting assistive technologies beyond text assistive technologies; and

-	Users who are not fluent in the language of the Warning Notifications.

Much of the work on enhancing the Public Warning System is set out in the ePWS requirements in 3GPP TS 22.268 [1] and in ePWS protocol solutions in 3GPP TS 23.041 [2] in Release 16, covering:

-	Specifying Message Identifiers for ePWS-UE, especially IoT devices that are intended for machine type communications

-	Enabling language-independent content to be included in Warning Notifications

The work on ePWS in 3GPP TS 22.268 [1] and 3GPP TS 23.041 (Release 16) is expected to help manufacturers of User Equipment meet any future regulatory requirements dedicated to such products.

Requirements defined for PWS-UEs in clause 4 of 3GPP TS 22.268 [1] are applicable for ePWS-UEs unless dedicated ePWS-UE requirements described in clause 9 of 3GPP TS 22.268 [1] supersede them.

References

[1] TS 22.268, Public Warning System (PWS) requirements

[2] TS 23.041, Technical realization of Cell Broadcast Service (CBS)

## 12.2	MBMS APIs for Mission Critical Services

| 800053 | MBMS APIs for Mission Critical Services | MBMSAPI_MCS | S6 | SP-180380 | Ling Zhang, TD Tech Ltd. |
| --- | --- | --- | --- | --- | --- |
| 760051 | Study on MBMS APIs for MC Services | FS_MBMSAPI_MC | S6 | SP-180237 | Ling Zhang, TD Tech Ltd |
| 800020 | Stage 2 of MBMS APIs for MC Services | MBMSAPI_MCS | S6 | SP-180380 | Ling Zhang, TD Tech Ltd. |

Summary not yet provided

## 12.3	Other Mission critical improvements

| 810004 | MCData File Distribution support over xMB | MC_XMB | S4 | SP-180665 | Thiénot Cédric, Expway |
| --- | --- | --- | --- | --- | --- |
| 800053 | MBMS APIs for Mission Critical Services | MBMSAPI_MCS | S6 | SP-180380 | Ling Zhang, TD Tech Ltd. |
| 800019 | Enhanced Mission Critical System Migration and Interconnection | eMCSMI | S6 | SP-180379 | Chater-Lea, David; Motorola Solutions |
| 800021 | Enhanced Mission Critical Communication Interworking with Land Mobile Radio Systems | eMCCI | S6 | SP-180680 | Monnes, Peter, Harris Corporation |
| 800016 | Stage 3 for MC Communication Interworking with Land Mobile Radio Systems | MCCI_CT | C1 | CP-190203 | Monnes, Peter, Harris Corporation |
| 820040 | Mission Critical system migration and interconnection | MCSMI_CT | C1 | CP-190143 | Dom Lazara, Motorola Solutions |
| 800032 | Mission Critical Services Security Enhancements | MCXSec | S3 | SP-180596 | Woodward, Tim, Motorola Solutions, Inc |
| 780003 | Enhancements of Public Warning System | ePWS |  | SP-170998 | Hyounhee Koo, SyncTechno Inc. |

Summaries not yet provided.

# 13	Conversational services, Streaming and TV

## 13.1	Conversational services

### 13.1.1	Coverage and Handoff Enhancements for Multimedia (CHEM)

| 810003 | Coverage and Handoff Enhancements for Multimedia | CHEM | S4 | SP-180664 | Nikolai Leung |
| --- | --- | --- | --- | --- | --- |

Summary based on the input provided by Qualcomm Incorporated in SP-190865.

The Coverage and Handoff Enhancements for Multimedia (CHEM) feature enables the network to delay or reduce handoffs of a Multimedia Telephony Service for IMS ( MTSI) terminal by providing the eNB/gNB additional information about the robustness to packet losses of the negotiated media configurations.  The eNB/gNB can set handoff thresholds that allow the MTSI terminal to remain on the current sector, cell, access technology, or domain (packet-switched vs. circuit-switched) and avoid a handoff even when the MTSI terminal experiences higher packet loss on the traffic channel as the media configuration robustness will help mitigate the effects of the additional packet losses.

The feature is structured with a baseline core functionality and some optional enhancements as illustrated in Figure 1.  The optional enhancements can be independently used with the core CHEM feature and each other. TR 26.959 [1] has also been updated to explain the operation and interaction of these options and the core feature.

Figure 1: Core function and optional enhancements of the CHEM feature 
(references are to clauses in TS 26.114 [6]).

The core CHEM functionality specified in clauses W.1 and W.2 of TS 26.114 [2] introduces the ‘PLR_adapt’ SDP attribute that is used to negotiate the CHEM feature between the MTSI clients.  MNOs choosing not to enable the feature can remove the attribute in the SDP negotiation.  The attribute is also used by the PCRF/PCF to determine whether it can expect the MTSI clients to adapt to the most robust configurations that have been negotiated without relying in application layer redundancy.  The PCRF/PCF uses this information to determine the appropriate PLR thresholds, i.e., choosing the PLR of the most robust mode when ‘PLR_adapt’ is negotiated, and communicate the PLR thresholds to the eNB/gNB.

An optional enhancement specified in clause W.4 of TS 26.114 [2] enables the MTSI clients to indicate the maximum tolerable end-to-end PLRs for particular codec configurations and then negotiate how this end-to-end PLR can be budgeted across their respective uplinks and downlinks.  Indicating maximum tolerable end-to-end PLR thresholds can be useful to account for differing client implementations of features, e.g., such as the de-jitter buffer and Packet-Loss-Concealment (PLC) in the media receiver, which can affect the maximum tolerable PLR for achieving a certain Quality of Experience (QoE).  This enhancement uses the defined ‘MAX-e2e-PLR’ attribute and associated parameters to perform the necessary indication and negotiation.  The PCRF/PCF uses the ‘MAX-e2e-PLR’ attribute information in the SDP answer to determine and communicate the appropriate PLR thresholds to the eNB/gNB.

Another optional enhancement for speech media specified in clause W.3 of TS 26.114 [2] is to support use of application layer redundancy to improve media robustness that, when used with the CHEM feature, can delay or avoid handing off an MTSI terminal.  The optional ‘ALR’ parameter for the ‘PLR_adapt’ attribute has been specified to enable this option and allows the MTSI clients to use newly specified in-band RTP CMR codepoints to request application layer redundancy without having to use RTCP-APP messages.  The PCRF/PCF uses the presence of the ‘PLR_adapt: ALR’ attribute and parameter in the SDP answer to determine the appropriate PLR thresholds based on the maximum tolerable PLRs of the codec configurations (among those that both use, and do not use, application layer redundancy), then communicates this to the eNB/gNB.

An Annex of TS 26.114 [2] provides example PLR values that can be used for different speech codecs and their configurations.  The annex also provides SDP examples for using the specified SDP attributes and parameters.

References

[1] 3GPP TR 26.959 “Study on enhanced Voice over LTE (VoLTE) performance”

[2] 3GPP TS 26.114 “IP Multimedia Subsystem (IMS); Multimedia telephony; Media handling and interaction”

CRs related to this Feature: Select "TSG Status = approved" in: https://portal.3gpp.org/ChangeRequests.aspx?q=1&workitem=810003

### 13.1.2	Single radio voice continuity from 5GS to 3G

| 810041 | Single radio voice continuity from 5GS to 3G | 5G_SRVCC |  | SP-180737 | Chi Ren, China Unicom |
| --- | --- | --- | --- | --- | --- |
| 790010 | Study for single radio voice continuity from 5GS to 3G | FS_5G-SRVCC | S2 | SP-180239 | Chi Ren, China Unicom |
| 810007 | Single radio voice continuity from 5GS to 3G | 5G_SRVCC | S2 | SP-180897 | Chi Ren, China Unicom |
| 820008 | Security aspects of single radio voice continuity from 5GS to UTRAN | 5GS_UTRAN_SEC | S3 | SP-181037 | Feng Gao, China Unicom |
| 820069 | RAN aspects of 5G_SRVCC | SRVCC_NR_to_UMTS | R2 | RP-200151 | China Unicom |
| 820169 | Core part: Single Radio Voice Call Continuity from 5G to 3G | SRVCC_NR_to_UMTS-Core | R2 | RP-200151 | China Unicom |
| 830270 | Perf. part: Single Radio Voice Call Continuity from 5G to 3G | SRVCC_NR_to_UMTS-Perf | R4 | RP-200151 | China Unicom |
| 840004 | CT aspect of 5G_SRVCC | 5G_SRVCC | ct | CP-191062 | Chi Ren, China Unicom |
| 840058 | CT1 aspect of 5G_SRVCC | 5G_SRVCC | C1 | CP-191062 | Chi Ren, China Unicom |
| 840059 | CT4 aspect of 5G_SRVCC | 5G_SRVCC | C4 | CP-191062 | Chi Ren, China Unicom |

Summary based on the inputs provided by China Unicom in SP-200129 for global aspects and in RP-200152 for the radio aspects.

[Editor's Note: merging needed for the summaries of the two aspects (non radio in SP-200129 and radio in RP-200152).]

The work item of 5G_SRVCC introduced a mechanism to support single radio voice call continuity (SRVCC) from 5GS to UTRAN for the following scenarios:

-	Operators with both 5G Voice over IMS and LTE enabled, but no VoLTE.

-	Operators with no LTE (nor VoLTE).

-	Operators with both 5G Voice over IMS and VoLTE enabled, but the voice service continuity may not be guaranteed if the VoLTE coverage provided by the operators is not (ideal) complete, i.e. there are some “holes” of VoLTE coverage.

This work item implemented the conclusion of the study for single radio voice continuity from 5GS to 3G (FS_5G-SRVCC) which is specified in TR 23.756[4]. The main enhancements introduced by this work item are specified in TS 23.216[3].

The architecture of 5G-SRVCC is illustrated in Figure 2-1. A simplified MME function (MME_SRVCC) was introduced to facilitate session transfer (SRVCC) of the voice component to the CS domain. The support of N26 interface and Sv interface is required for the MME_SRVCC.

Figure 1: 5G-SRVCC architecture

The 5G-SRVCC mechanism defined by this work item is an indirect SRVCC from NG-RAN to UTRAN. When an UE moves out of the coverage of NG-RAN while a 5G voice session over IMS (either a normal voice session or an emergency session) is ongoing, a PS to CS handover procedure between 5GS and 3G CS is triggered by the NG-RAN. The 5G-SRVCC procedure including two main parts:

-	5GS to EPS handover using N26 procedure triggered by the NG-RAN.

-	PS-CS handover procedure initiated by the selected MME_SRVCC.

No E-UTRAN involved during the whole process. All the PDU sessions is released by the request of AMF at the end of the 5G-SRVCC procedure.

References

[1] SP-180897, Single radio voice continuity from 5GS to 3G.

[2] CP-193014, CT aspect of 5G_SRVCC.

[3] TS 23.216, Single Radio Voice Call Continuity (SRVCC); Stage 2.

[4] TR 23.756, Study for single radio voice continuity from 5G to 3G.

Radio part

In Rel-16, the WI “Single Radio Voice Call Continuity from 5G to 3G” was approved (latest WID in RP-200151). This work item aims to specify the support of SRVCC from 5GS to UTRAN in RAN. More specifically, the main objectives are as follows:

-	Inter-RAT measurement to support voice service continuity from 5GS to UTRAN [RAN2, RAN1].

-	Specify that the NG-RAN and UE in NR RRC connected mode support the UTRAN cell measurement procedure, e.g. measurement configuration of target UTRAN cells, and the measurement performing and reporting by UE.

-	Indirect Inter-RAT handover procedure to support voice service continuity from 5GS to UTRAN [RAN2, RAN3].

-	Specify the procedures of SRVCC (including emergency call) in RAN, which includes handover preparation between gNB and AMF, gNB and UE, and Handover execution between UE to RNS etc.

-	Signalling of source RAT to target RAT at incoming SRVCC [RAN3].

-	UE capability reporting for supporting SRVCC [RAN2, RAN3].

-	Specify the transfer of UE capability information between UE, NG-RAN and AMF.

-	RRM requirements to support voice service continuity from 5GS to UTRAN [RAN4].

-	Requirements of NR handover to UMTS.

-	Requirements of NR- UMTS inter-RAT measurements.

The key functionalities are:

-	Support of the UTRAN cell measurement procedure for NR node and UE in NR RRC connected mode. A new measurement object for UTRA is defined to include only UTRA-FDD. -	The UE is only required to measurement and report the listed cells. Periodic and event triggered (event B1/B2) reporting for UTRA measurement are both supported. The same measurement gap configuration for UTRA is reused in measurement in NR as in LTE.

-	Support of inter-RAT handover procedure of SRVCC. MobilityFromNRCommand message is reused for SRVCC from NR to UTRAN. SRVCC to UTRAN is also supported for UEs configured with NR-DC and NE-DC, network can trigger SRVCC to UTRAN procedure directly irrespective of the whether the bearer for the voice call is anchored in SN or MN. When and how to perform the return procedure from UTRAN to NR after the UE completes the voice service depends on UE implementation.

-	Support of Signalling of source RAT to target RAT at incoming SRVCC. Add 5G-SRVCC possible into Initial Context Setup Request, HO Request, Path Switch Request Ack. Clarify the source to target transparent container is source RNC to target RNC Transparent Container in 5G-SRVCC. Clarify the target to source transparent container is Target RNC to Source RNC Transparent Container in 5G-SRVCC.

-	Support of UE capability reporting for supporting SRVCC, including SRVCC handover capability with FR1/FR2 and TDD/FDD split, and UTRA-FDD band list is also reported.

-	RRM Requirements have also been specified, including measurement gap applicability requirement for SRVCC, Measurement capability, CSSF, UMTS inter-RAT measurement requirements and handover requirements for SRVCC.

References

[1]	R2-2000325	Introduction of SRVCC from 5G to 3G	Ericsson, ZTE

[2]	R2-2000335	Introduction of SRVCC from 5G to 3G	Ericsson

[3]	R2-2000542	Introduction of SRVCC from 5G to 3G	Huawei, HiSilicon, China Unicom

[4]	R2-2000651	Introduction of SRVCC from 5G to 3G	China Unicom, Huawei, HiSilicon

[5]	R3-194169	Support SRVCC from 5G to 3G	Ericsson

[6]	R3-194171	Support SRVCC from 5G to 3G	Ericsson

[7]	R3-200071	Support SRVCC from 5G to 3G	Ericsson

[8]	R3-200073	Support SRVCC from 5G to 3G	Huawei, China Unicom, Ericsson, Nokia, Nokia Shanghai Bell, ZTE, Qualcomm Incorporated

### 13.1.3	Volume Based Charging Aspects for VoLTE

| 840066 | Volume Based Charging Aspects for VoLTE | VBCLTE | S5 | SP-180813 | Chen, Ai, China Mobile |
| --- | --- | --- | --- | --- | --- |

Summary based on the input provided by China Mobile in SP-191277.

Based on the Study Item FS_VBCLTE (3GPP TR 32.848) which has investigated potential charging requirements and possible mechanisms to support volume based charging for VoLTE, the WI VBCLTE is the enhancement of online and offline charging aspects of volume based charging for VoLTE.

Stage 2 work on WI VBCLTE for TS 32.251 [1] and TS 32.260 [4]:

-	Clarify that IMS charging only supports duration based charging for VoLTE.

-	Introduce volume based charging for VoLTE in PS offline and online charging.

-	Introduce VoLTE Information parameter for volume based charging for VoLTE in PGW-CDR.

-	Add Bindings of CDR parameter, Information Element and AVP for VoLTE Information parameter.

Stage 3 work on WI VBCLTE for 32.298 [3] and 32.299 [4]:

-	Introduce two new values for Change-Condition AVP to indicate the status of VoLTE service delivery.

-	Introduce two new reasons of Cause for Record Closing field in the CDR to indicate the status of VoLTE service delivery.

-	Introduce VoLTE Information parameter including Caller Information and Callee Information for volume based charging for VoLTE in PS domain CDRs.

-	Introduce VoLTE-Information AVP including Calling-Party-Address AVP and Callee-Information AVP for volume based charging for VoLTE in PS domain online charging messages.

References

[1]	TS 32.251: "Packet Switched (PS) domain charging"

[2] 	TS 32.260: "IP Multimedia Subsystem (IMS) charging"

[3] 	TS 32.298: "Charging Data Record (CDR) parameter description"

[4]	TS 32.299: "Diameter charging application"

### 13.1.4	EVS Floating-point Conformance for Non Bit-Exact

| 820001 | EVS Floating-point Conformance for Non Bit-Exact | EVS_FCNBE | S4 | SP-180983 | Fabrice, Plante, Intel |
| --- | --- | --- | --- | --- | --- |

Summary based on the input provided by Intel in SP-191328.

The EVS_FCNBE work item provides improvements related to the 3GPP EVS codec: this codec is standardized in three code versions such that it can be implemented on processors with either fixed-point (TS 26.442, TS 26.452) or floating-point arithmetic (TS 26.443). Implementations can be conformance-tested using “bit-exact” methods that have been specified already when the EVS codec was first standardized. These methods are suitable for fixed-point implementations but their applicability is limited for floating-point implementations. EVS_FCNBE provides methods which enable conformance testing also for “non bit-exact” floating-point implementations.

During Rel-15, the FS_EVS_FCNBE study item investigated tools, conformance process, and criteria for EVS floating-point non bit-exact conformance, and documented in TR 26.843 [1] several recommendations. Based on the agreed conclusions and recommendations of TR 26.843, the EVS_FCNBE work item conducted normative work on TS 26.444 to introduce the non bit-exact conformance process and criteria in the CR 26.444-0027.

The conformance process uses three specific tests:

-	Decoder test comparing the implementation decoder with TS 26.443 decoder.

-	Encoder test comparing the implementation encoder with TS 26.443 encoder.

-	MOS-LQO verification comparing the coder implementation with TS 26.442 coder.

These three tests allow to compare the implementation with the reference floating point code in TS 26.443, and interoperability with the fixed-point code in TS 26.442. All three tests shall pass for the implementation to be declared conformant.

The decoder test relies on a set of signal-based metrics (RMS, SNR, and Spectral Distortion). The encoder test uses a loudness metric.  The MOS-LQO test uses a set of metrics using objective quality measurements.

To define a range of acceptable variation for the various metrics, a set of 10 reference implementations (based on mainstream compilers and platforms) has been used to define reference values.

Annex B of  CR 26.444-0027 provides details on the tests as well as the conformance process and criteria.

The results obtained during the course of the normative work indicate that this conformance process correctly discriminates implementations with functional code changes and aggressive compiler optimization.

References

[1] 3GPP TR 26.843 “Study on non-bit-exact conformance criteria and tools for floating-point EVS codec”

List of related CRs: select "TSG Status = Approved" in https://portal.3gpp.org/ChangeRequests.aspx?q=1&workitem=820001

### 13.1.5	Media Handling Extensions for 5G Conversational Services

| 810040 | Media Handling Extensions for 5G Conversational Services | 5G_MEDIA_MTSI_ext | S4 | SP-180663 | Oyman, Ozgur, Company: Intel |
| --- | --- | --- | --- | --- | --- |

Summary based on the input provided by Intel in SP-200036.

During Rel-15, the FS_5G_MEDIA_MTSI study item investigated media handling aspects of conversational services in 5G, and documented in TR 26.919 several recommendations on Multimedia Telephony Service over IMS (MTSI) in TS 26.114 around media rate adaptation, NR access and profiles for 5G deployments.

Based on the agreed conclusions and recommendations of TR 26.919, 5G_MEDIA_MTSI_ext conducted normative work, mostly on TS 26.114, to introduce the following functionality:

-	For media rate adaptation, the CR 26.114-0436 [2] recommended additional speech and video adaptation capabilities based on access network bitrate recommendation (ANBR) (a.k.a. RAN-assisted codec adaptation) support. In particular, this CR makes ANBR-triggered speech adaptation a recommended feature in MTSI clients for Rel-16 and beyond. Moreover, it provides additional recommendations for MTSI receivers to generate CMR/RTCP-APP (for speech) and TMMBR (for video) and for MTSI senders to generate TMMBN (for video) messages in response to receiving ANBR information.

-    On profiles for 5G deployments, the CR 26.114-0438 [3] defined the constrained MTSI client terminal to address codec requirements for IMS/VoLTE/ViLTE/VoNR conversational services in the low end 5G verticals such as wearables and IoT. In particular, this CR relaxed the Rel-15 speech and video codec requirements for these constrained terminals, e.g., resulting in optional support for super-wideband for speech and HEVC for video, exceptionally only for this special category of constrained terminals (support for SWB for speech and HEVC for video is otherwise mandatory for Rel-15 and beyond).

-	To realize further RAN-assisted codec adaptation enhancements, the CR 26.114-0461 [4] and CR 26.114-0480 [5] introduced ANBR capability signalling into MTSI. In particular, the CRs standardized a new SDP attribute ‘anbr’ to enable end-to-end signaling and coordination of ANBR capabilities (both at the radio level and application level) across the UEs, Access Networks, and PCRF/PCF. According to the adopted solution, the MTSI client in terminal supporting "anbr" signals this attribute in the SDP only if: (i) MTSI client in terminal supports ANBR at the application layer, including the use of ANBR with dynamic bitrate adaptation. (ii) The UE of the MTSI client in terminal is capable of RAN-assisted codec adaptation at the radio layer, including the ability to query and receive ANBR information (both downlink and uplink ANBR) from its eNB/gNB. (iii) The P-CSCF has indicated to the UE of the MTSI client in terminal its ability to handle this SDP attribute through the respective SIP registration procedures specified in TS 24.229 via the corresponding feature capability indicator g.3gpp.anbr.

-	The low and ultra-low latency of 5G will enable new types of use cases with real-time communication and interaction needs where it will be desirable to not only have voice, text and video calling connectivity but also delivery of any kind of data stream and real-time interaction within the same IMS multimedia session. This is already enabled as part of the IMS-based Tele-Presence service in TS 26.223 toward the exchange of CLUE messages and has been introduced to MTSI in TS 26.114 as part of this work item to provide a generic and flexible data channel support for IMS-based multimedia telephony. In particular, CR 26.114-0496 [6] adds necessary UNI procedures to support MTSI data channel based on WebRTC data channel and web scripting techniques as a new MTSI media. This provides a generic and flexible data channel for MTSI in TS 26.114 that allows both UE-to-UE and UE-to-network generic data transfers, in addition to and in parallel with the existing MTSI media such as audio, video, and text. Accordingly, TS 26.223 was also updated via CR 26.223-0013 [7] to align the data channel support of a Tele-Presence (TP) UE with that specified in MTSI.

References

[1] Tdoc SP-180663, New WID on “Media Handling Extensions for 5G Conversational Services” (5G_MEDIA_MTSI_ext)

List of related CRs: select "TSG Status = Approved" in  https://portal.3gpp.org/ChangeRequests.aspx?q=1&workitem=810040

This includes:

[2] Tdoc SP-180975, CR 26.114-0436, “Recommendations on Media Rate Adaptation”

[3] Tdoc SP-180975, CR 26.114-0438, “MTSI Client Profiles”

[4] Tdoc S4-190489, CR 26.114-0461, “Signaling of ANBR Capabilities”

[5] Tdoc S4-190603, CR 26.114-0480, “Updates on ANBR Capability Signaling”

[6] Tdoc S4-200266, CR 26.114-0496, “Addition of MTSI Data Channel Media”

[7] Tdoc S4-200269, CR 26.223-0013, “Alignment with MTSI on IMS Data Channel Support”

### 13.1.6	VR QoE metrics

| 840000 | VR QoE metrics | VRQoE | S4 | SP-190331 | Gunnar HEIKKILÄ , Ericsson |
| --- | --- | --- | --- | --- | --- |

Summary based on the input provided by Ericsson in SP-200128.

The experienced Virtual Reality (VR) quality is dependent on a good service implementation as well as a fast and consistent transport network. To help service providers and operators to measure and optimize aspects related to the delivered VR service quality, specific VR-related metrics are useful.

The VRQoE Work Item added the following functionality:

-	Defined VR metrics observation points.

-	Metrics describing the characteristics of the VR device, such as resolution, FOV etc.

-	Metrics describing the interaction delay, related to the time between head movements and update of the visible content.

-	Extensions to the DASH Metrics configuration and reporting to support the new metrics.

Also, the following metrics have been added into TS 26.118 [1]:

-	Comparable quality viewport switching latency: Reports the delay from head movement until the media quality is restored to a similar quality level as before the move.

-	Rendered viewports: Reports where the user mostly looks during the VR session.

-	VR device information: Reports characteristics for the VR device.

The metrics are building on and extending the existing metrics in TS 26.247 [2], and are well aligned with the current proposal in MPEG [3]. Furthermore, a requirement has been added to TS 26.234 [4] specifying that a VR-capable PSS client should also support these metrics.

References

[1]	3GPP TS 26.118 " Virtual Reality (VR) profiles for streaming applications"

[2] 	3GPP TS 26.247 "Transparent end-to-end Packet-switched Streaming Service (PSS); Progressive Download and Dynamic Adaptive Streaming over HTTP (3GP-DASH)"

[3]	ISO/IEC CD 23090-6 "Immersive Media Metrics"

[4] 	3GPP TS 26.234 "Transparent end-to-end Packet-switched Streaming Service (PSS); Protocols and codecs"

### 13.1.7	Media Handling Aspects of RAN Delay Budget Reporting in MTSI

| 810039 | Media Handling Aspects of RAN Delay Budget Reporting in MTSI | E2E_DELAY | S4 | SP-180662 | Oyman, Ozgur, Company: Intel |
| --- | --- | --- | --- | --- | --- |
| 780022 | Study on E2E_DELAY | FS_E2E_DELAY | S4 | SP-170837 | Oyman, Ozgur, Company: Intel, |
| 810001 | Stage 2 of E2E_DELAY | E2E_DELAY | S4 | SP-180662 | Oyman, Ozgur, Company: Intel |
| 830004 | CT Aspects of E2E_DELAY | E2E_DELAY | ct | CP-190193 | Luetzenkirchen, Thomas, Intel |
| 830066 | CT1 Aspects of E2E_DELAY | E2E_DELAY | C1 | CP-190193 | Luetzenkirchen, Thomas, Intel |
| 830067 | CT3 Aspects of E2E_DELAY | E2E_DELAY | C3 | CP-190193 | Luetzenkirchen, Thomas, Intel |
| 830068 | CT4 Aspects of E2E_DELAY | E2E_DELAY | C4 | CP-190193 | Luetzenkirchen, Thomas, Intel |

Summary based on the input provided by Intel in SP-190322.

This summary reports on the normative specification progress accomplished during the course of the E2E_DELAY work item [1]. The related agreed CRs can be found in Tdocs S4-181176 (CR 26.114-0441) [2], S4-181452 (CR 26.114-0443) [3], S4-190167 (CR 26.114-0448) [4], S4-190497 (CR 26.114-0462) [5] and S4-190498 (CR 26.114-0463) [6].

RAN delay budget reporting is specified in TS 36.331 for E-UTRA and TS 38.331 for NR. RAN delay budget reporting through the use of RRC signalling to eNB / gNB allows UEs to locally adjust air interface delay. Based on the reported delay budget information, a good coverage UE on the receiving end (i.e., the UE that contains the MTSI receiver) can reduce its air interface delay, e.g., by turning off CDRX or via other means. This additional delay budget can then be made available for the sending UE (i.e., the UE that contains the MTSI sender), and can be quite beneficial for the sending UE when it suffers from poor coverage. When the sending UE is in bad coverage, it would request the additional delay from its local eNB / gNB, and if granted, it would utilize the additional delay budget to improve the reliability of its uplink transmissions in order to reduce packet loss, e.g., via suitable repetition or retransmission mechanisms, and thereby improve end-to-end delay and quality performance.

While RAN-level delay budget reporting as defined in TS 36.331 and TS 38.331 allows UEs (i.e., MTSI sender and MTSI receiver) to locally adjust air interface delay, such a mechanism does not provide coordination between the UEs on an end-to-end basis. To alleviate this issue, this work item defined a new RTCP feedback message type in clause 7.3.8 of TS 26.114 to realize the following capabilities on signalling of Delay Budget Information (DBI) across UEs as per CRs 26.114-0443 in Tdoc S4-181452, 26.114-0448 in Tdoc S4-190167 and 26.114-0462 in Tdoc S4-190497:  (i) an MTSI receiver can indicate available delay budget to an MTSI sender, and (ii) an MTSI sender can explicitly request delay budget from an MTSI receiver. Such RTCP-based signaling of DBI can also be used by an MTSI receiver to indicate delay budget availability created via other means such as jitter buffer size adaptation. The recipient UE of the RTCP feedback message carrying DBI may then use this information in determining what delay budget adjustments it may request from its eNB / gNB over the RAN interface, e.g. by using RRC signalling based on UEAssistanceInformation as defined in TS 36.331 and TS 38.331.

In addition, the work item specified a few further media handling aspects of RAN delay budget reporting for MTSI in TS 26.114 to address the following:

1.	   SDP-based exchange of RAN capabilities in regards to delay budget reporting, this is specified in clause 6.2.8 and Annex V.3 of TS 26.114 as per CR 26.114-0441 in Tdoc S4-181176 and CR 26.114-0463 in Tdoc S4-190498. Accordingly, an MTSI client supporting DBI can offer this capability in SDP for all media streams containing speech and/or video by including the a=rtcp-fb attribute with the DBI type under the relevant media line scope as expressed with the parameter 3gpp-delay-budget.

2.	   Recommendations on when and how the UEs in an MTSI session should use RAN-based delay adjustment mechanisms in an end-to-end fashion also accounting other factors such as local radio conditions, various RAN capabilities and configurations, jitter buffer considerations, and UE battery constraints. These are provided as part of the informative signalling flows in Annex V of TS 26.114, as per CR 26.114-0443 in S4-181452.

3.	   Recommendations on when and how the various kinds of end-to-end quality metrics and other relevant information in the MTSI client could be used to trigger RAN delay budget reporting. These are provided as part of the informative signalling flows in Annex V of TS 26.114, as per CR 26.114-0443 in S4-181452.

References

[1] Tdoc SP-180662, New WID on “Media Handling Aspects of RAN Delay Budget Reporting in MTSI” (E2E_DELAY)

[2] Tdoc S4-181176, CR 26.114-0441, “Signaling of Delay Budget Information”

[3] Tdoc S4-181452, CR 26.114-0443, “Recommendations on use of RAN Delay Budget Reporting in MTSI”

[4] Tdoc S4-190167, CR 26.114-0448, “DBI Signaling Recommendations”

[5] Tdoc S4-190497, CR 26.114-0462, “Additional DBI Signaling Recommendations”

[6] Tdoc S4-190498, CR 26.114-0463, “SDP Examples on DBI Signaling”

[7] TS 36.331: "E-UTRA; Radio Resource Control (RRC); Protocol specification".

[8] TS 38.331: "NR; Radio Resource Control (RRC); Protocol Specification".

## 13.2	Streaming

### 13.2.1	Enhancement of LTE for Efficient delivery of Streaming Service

| 800008 | Enhancement of LTE for Efficient delivery of Streaming Service | eLSTR | S1 | SP-180322 | Xia, Xu, China Telecom |
| --- | --- | --- | --- | --- | --- |
| 740001 | Study on eLSTR | FS_eLESTR | S1 | SP-160960 | China Telecommunications, XIA Xu |
| 800054 | Stage 1 of eLSTR | eLSTR | S1 | SP-180322 | Xia, Xu, China Telecom |

Summary not yet provided

### 13.2.2	Enhancements to Framework for Live Uplink Streaming

| 800001 | Enhancements to Framework for Live Uplink Streaming | E_FLUS | S4 | SP-180285 | Lo, Charles, Qualcomm |
| --- | --- | --- | --- | --- | --- |

Summary not yet provided

### 13.2.3	Media streaming architecture

| 820002 | Media streaming architecture | 5GMSA | S4 | SP-180984 | Frédéric GABIN |
| --- | --- | --- | --- | --- | --- |

Summary not yet provided

# 14	5G Location and Positioning Services

## 14.1	Stage 1 of 5G_HYPOS

| 800049 | Stage 1 of 5G_HYPOS | 5G_HYPOS | S1 | SP-180329 | Ries, Lionel, ESA |
| --- | --- | --- | --- | --- | --- |

Summary not yet provided

## 14.2	Enhancement to the 5GC LoCation Services

| 830102 | Enhancement to the 5GC LoCation Services | 5G_eLCS | S2 | SP-181119 | Ming Ai, CATT |
| --- | --- | --- | --- | --- | --- |
| 780028 | Study on Enhancement to the 5GC Location Services | FS_eLCS | S2 | SP-180734 | Ming Ai, CATT |
| 810035 | Study on Security of the enhancement to the 5GC location services | FS_eLCS_Sec | S3 | SP-180694 | Wei Zhou, CATT |
| 820016 | Stage 2 of 5G_eLCS | 5G_eLCS | S2 | SP-181119 | Ming Ai, CATT |
| 830003 | CT aspects of 5G_eLCS | 5G_eLCS | ct | CP-190014 | Tingfang Tang, CATT |
| 830044 | CT1 aspects of 5G_eLCS | 5G_eLCS | C1 | CP-191064 | Tingfang Tang, CATT |
| 830045 | CT3 aspects of 5G_eLCS | 5G_eLCS | C3 | CP-191064 | Tingfang Tang, CATT |
| 830046 | CT4 aspects of 5G_eLCS | 5G_eLCS | C4 | CP-191064 | Tingfang Tang, CATT |

Summary based on the input provided by CATT in SP-191223.

This work item defines the stage 2 of the service-based architecture used for location services in the 5G system, and corresponding Network Functions (NFs), NF services and procedures, to meet the service requirements defined in TS 22.261 [1] and TS 22.071 [2].

The Location Services, specified in TS 23.273 [3], include aspects of both regulatory and commercial nature.

The architecture and signalling procedures in NG-RAN are defined in TS 38.305 [4].

Following aspects have been specified for 5G Location Services:

-	Service based 5G location architecture, including roaming and non-roaming, Function description of per Network Functions, etc.

-	General Concepts, e.g. Type of Location Requests, LCS Quality of services;

-	High Level Features, e.g. LMF selection, UE LCS privacy handling;

-	Location Service Procedures, which includes

-	5GC-MT-LR Procedure

-	5GC-MO-LR Procedure

-	Deferred 5GC-MT-LR Procedure for Periodic, Triggered and UE Available Location Events

-	LMF Change Procedure

-	Unified Location Service Exposure Procedure

-	Low Power Periodic and Triggered 5GC-MT-LR Procedure

-	Bulk Operation of LCS Service Request Targeting to Multiple UEs

-	Procedures to Support Non-3GPP Access

-	Procedures dedicated to Support Regulatory services

-	UE Assisted and UE Based Positioning Procedure

-	Network Assisted Positioning Procedure

-	Obtaining Non-UE Associated Network Assistance Data

-	UE Location Privacy Setting Procedure

-	Procedures with interaction between 5GC and EPC

-	Support of Concurrent Location Request;

-	Network Function Services, e.g. LMF services, GMLC services.

References

[1] 	TS 22.261: "5G system; Services, operations and procedures of charging using Service Based Interface (SBI)"

[2]	TS 22.071: "Service requirements for next generation new services and markets; Stage 1"

[3] 	TS 23.273: "5G System (5GS) Location Services (LCS); Stage 2"

[4]     	TS 38.305: "Stage 2 functional specification of User Equipment (UE) positioning in NG-RAN".

## 14.3	NR positioning support

| 830077 | NR positioning support | NR_pos | R1 | RP-191156 | Intel |
| --- | --- | --- | --- | --- | --- |
| 800092 | Study on NR positioning support | FS_NR_pos | R1 | RP-181399 | Intel |
| 830073 | Study on local NR positioning in NG-RAN | FS_NR_local_pos | R3 | RP-190754 | CMCC |
| 830177 | Core part: NR positioning support | NR_pos-Core | R1 | RP-190752 | Intel |
| 830277 | Perf. part: NR positioning support | NR_pos-Perf | R4 | RP-190752 | Intel |

Summary not yet provided

# 15	User Identities, Authentication, multi-device

## 15.1	User Identities and Authentication

| 800012 | User Identities and Authentication | UIA | S1 | SP-180328 | Bischinger, Kurt; Deutsche Telekom |
| --- | --- | --- | --- | --- | --- |

Summary based on the input provided by Deutsche Telekom AG in SP-200131.

The feature allows the use of a User Identifier [1], which is independent of existing identifiers relating to a 3GPP subscription or UE, for Network Slice-Specific Authentication and Authorization. [1][2][3]

The actual identity provisioning service with creation, managing and authentication of identities is not specified by 3GPP, but the interworking of the 3GPP system with some external entity to authenticate the User Identity and to authorize the user for access to a specific network slice, in addition to the 3GPP network slice selection and authorization.

Rel. 15 Network slice selection (i.e. the procedure to select an AMF that supports the required Network Slices and establishing PDU Session(s) to the required Data network via the Network Slice instance(s) with its Control Plane and User Plane Network Functions) is based on the NSSAI, which consists of one or more S-NSSAIs.

Now with the Rel. 16 feature, Subscription information allows the indication which S-NSSAIs are subject to Network Slice-Specific Authentication and Authorization (NSSAA). For those S-NSSAIs the AMF invokes an EAP-based Network Slice-Specific authentication procedure in which the AUSF exchanges AAA protocol messages with a potentially external AAA Server (AAA-S) via an optional AAA Proxy (AAA-P) to authenticate and authorize a UE for the network slice. Depending on the result of the procedure a UE is either authorized for a network slice, re-allocated to a different one or deregistered. A re-authentication and re-authorization procedure may be triggered by the AAA Server at any time. [2][3]

Security aspects are specified in detail in [4], UE and network capabilities to support and perform NSSAA in [5], message flows for interworking to the AAA-S in [6], flags to indicate that a slice is subject to NSSAA are defined in [7] as well as [8] and the impact on the AUSF is captured in [9].

References

[1] TS 22.101, Service aspects; Service principles

[2] TS 23.501, System architecture for the 5G System (5GS)

[3] TS 23.502, Procedures for the 5G System (5GS)

[4] TS 33.501, Security architecture and procedures for 5G System

[5] TS 24.501, Non-Access-Stratum (NAS) protocol for 5G System (5GS); Stage 3

[6] TS 29.561, 5G System; Interworking between 5G Network and external Data Networks; Stage 3

[7] TS 29.503, 5G System; Unified Data Management Services; Stage 3

[8] TS 29.505, 5G System; Usage of the Unified Data Repository services for Subscription Data; Stage 3

[9] TS 29.509, 5G System; Authentication Server Services; Stage 3

## 15.1.2	Multi-device and multi-identity

| 810044 | Multi-device and multi-identity | MuD | S1 | SP-180315 | Peter Bleckert, Ericsson |
| --- | --- | --- | --- | --- | --- |
| 800004 | Stage 1 of Multi-device and multi-identity | MuD | S1 | SP-180315 | Peter Bleckert, Ericsson |
| 810010 | CT aspects of Multi-device and multi-identity | MuD | ct | CP-182227 | Axell, Jörgen, Ericsson |
| 810045 | CT1 aspects of Multi-device and multi-identity | MuD | C1 | CP-182227 | Axell, Jörgen, Ericsson |
| 810046 | CT3 aspects of Multi-device and multi-identity | MuD | C3 | CP-182227 | Axell, Jörgen, Ericsson |

Summary not yet provided.

# 16	Slicing

## 16.1	Enhancement of Network Slicing

| 830103 | Enhancement of Network Slicing | eNS |  | SP-181232 | ZTE, Tricci So |
| --- | --- | --- | --- | --- | --- |

Summary based on the input provided by ZTE Wistron Telecom AB in SP-200125.

The objective of this work item is to address the following aspects:

-	Enhance the network slicing interworking support from EPC to 5GC, when UE moves from EPC to 5GC by selecting the proper target serving AMF as well as V-SMF.  Note that, the 5GC may not be able to serve all the PDU sessions that the UE intends to move from EPC.

-	Provide the network slice based authentication and authorization that uses User Identities and Credentials different from the 3GPP SUPI and that takes place after the primary authentication.

Release 16 Network Slicing addresses two major limitations of Release 15 in 5GC:

(1)	Enhancement of interworking between EPC and 5GC when UE moves from EPC to 5GC, the target serving AMF may not be able to serve all the PDU sessions that the UE intends to move to the 5GC.  More specifically, the following aspects needs to be addressed:

-	Selecting an AMF based on the slices associated to the active PDU connections that serve the UE in the EPC in Connected mode and during the Idle mode mobility

-	Selecting an appropriate serving V-SMF based on the slices associated to the active PDN connections that serve the UE in the EPC in Connected Mode

(2) Support for Network Slice Specific Authentication and Authorization (NSSAA)

-	Enable the support for separate authentication and authorization per Network Slice.  The trigger of NSSAA in the 5GC is based on UE subscription information from UDM and also operator’s policy.  However, the UE shall indicate its support for NSSAA to its serving 5GC.

-	The AMF performs the role of the EAP Authenticator and communicates with the AAA-S via the AUSF. The AUSF undertakes any AAA protocol interworking with the AAA protocol supported by the AAA-S.

Enhancement of Interworking Between EPC and 5GC

During the mobility from EPS to 5GS, in case of CM-IDLE state, the PGW-C+SMF sends PDU Session IDs and related S-NSSAIs to AMF in Registration procedure. The AMF derives S-NSSAI values for the Serving PLMN and determines whether the current AMF is appropriate to serve the UE. If not, the AMF reallocation may need to be triggered. For each PDU Session the AMF determines whether the V-SMF need to be reselected based on the associated S-NSSAI value for the Serving PLMN. If the V-SMF need be reallocated, the AMF trigger the V-SMF reallocation.

In case of CM-CONNECTED state, during handover preparation phase the PGW-C+SMF sends PDU Session IDs and related S-NSSAIs to AMF. Based on the received S-NSSAIs values, the target AMF derives the S-NSSAI values for the Serving PLMN, the target AMF reselects a final target AMF if necessary and forwards the handover request to the final target AMF. When the Handover procedure completes successfully, the UE proceeds with the Registration procedure. For each PDU Session based on the associated derived S-NSSAI values, if the V-SMF need be reallocated, the final target AMF triggers the V-SMF reallocation. The final target AMF sends the S-NSSAI value for the Serving PLMN to V-SMF to update the SM context. The V-SMF updates NG RAN with the S-NSSAI value for the Serving PLMN via N2 SM message.

Network Slice-Specific Authentication and Authorization (NSSAA)

In Release-16, based on UE’s 5GMM Core Network Capability and subscription information, the serving AMF will trigger Network Slice-Specific Authentication and Authorization for the S-NSSAIs of the HPLMN. If a UE does not support this feature but requests these S-NSSAIs that are subject to Network Slice-Specific Authentication and Authorization, these S-NSSAIs will be rejected by the PLMN.

If a UE supports this feature and requests these S-NSSAIs, which are subject to Network Slice-Specific Authentication and Authorization, the UE shall leverage the corresponding credentials for these S-NSSAIs for the Network Slice-Specific Authentication and Authorization. As for how to these credentials in the UE are not specified.

To perform the Network Slice-Specific Authentication and Authorization for an S-NSSAI, the AMF invokes an EAP- based Network Slice-Specific authorization procedure for the S-NSSAI.

This procedure can be invoked for a supporting UE by an AMF at any time, e.g. when:

a.	The UE registers with the AMF and one of the S-NSSAIs of the HPLMN which maps to an S-NSSAI in the Requested NSSAI is requiring Network Slice-Specific Authentication and Authorization; or

b.	The Network Slice-Specific AAA Server triggers a UE re-authentication and re-authorization for an S-NSSAI; or

c.	The AMF, based on operator policy or a subscription change, decides to initiate the Network Slice-Specific Authentication and Authorization procedure for a certain S-NSSAI which was previously authorized.

Based on the outcome of the Network Slice-Specific Authentication and Authorization, the Allowed NSSAI for each Access Type will be updated accordingly.   It is network policies to decide for which Access Type to be used if both Access Types are subject for the Network Slice-Specific Authentication and Authorization. However, if the Network Slice-Specific Authentication and Authorization fails for all S-NSSAIs in the Allowed NSSAI, the AMF shall execute the Network-initiated Deregistration procedure with the appropriate rejection cause value for each Rejected S-NSSAI.

After a successful or unsuccessful UE Network Slice-Specific Authentication and Authorization, the UE context in the AMF shall retain the authentication and authorization status for the UE for the related specific S-NSSAI of the HPLMN while the UE remains RM-REGISTERED in the PLMN, so that the AMF is not required to execute a Network Slice-Specific Authentication and Authorization for a UE at every Periodic Registration Update or Mobility Registration procedure with the PLMN.

A Network Slice-Specific AAA server may revoke the authorization or challenge the authentication and authorization of a UE at any time. When authorization is revoked for an S-NSSAI that is in the current Allowed NSSAI for an Access Type, the AMF shall provide a new Allowed NSSAI to the UE and trigger the release of all PDU sessions associated with the S-NSSAI, for this Access Type.

The AMF provides the GPSI of the UE related to the S-NSSAI to the AAA Server to allow the AAA server to initiate the Network Slice-Specific Authentication and Authorization, or the Authorization revocation procedure, where the UE current AMF needs to be identified by the system, so the UE authorization status can be challenged or revoked.

The Network Slice-Specific Authentication and Authorization requires that the UE Primary Authentication and Authorization of the SUPI has successfully completed. If the SUPI authorization is revoked, then also the Network Slice-Specific authorization is revoked.

References

[1]	3GPP TS 23.501: "System Architecture for the 5G System; Stage 2".

[3]	3GPP TS 23.502: "Procedures for the 5G System; Stage 2".

## 16.2	Enhancement of 3GPP management system for multiple tenant environment support

| 850031 | Enhancement of 3GPP management system for multiple tenant environment support | MEMTANE | S5 | SP-190786 | Zhu, Lei, Huawei |
| --- | --- | --- | --- | --- | --- |

Summary based on the input provided by Huawei Tech in SP-200526.

The work item on enhancement of 3GPP management system for multiple tenant environment support has concluded conceptual descriptions, specific information for tenant and other potential changes to management services. This release 16 work item is a feature to enhance management and orchestration to 5G network and network slicing. This is also a follow up work item to study on tenancy concept in 5G networks and network slicing management, which concluded to enhance management system as following:

-	Conception description

-	Performance measurement in multiple tenant environment

-	Network slice fault supervision in multiple tenant environment

In 3GPP management system, tenant is type of communication service consumer and tenant represents a group of MnS consumers associated with the management capabilities they are allowed to access and consume. The 3GPP management system provides multi-tenancy support, by associating different tenants with different sets of management capabilities. Every tenant may be authorized to access and consume those MnSs that the operator makes available to this tenant based on SLA.

The MnS consumer, acting on behalf of a tenant, may get the performance measurements of a network slice. Performance measurements specified in TS 28.552 can be split into sub-counters per S-NSSAI. 3GPP management system can use these sub-counters to distinguish performance measurements for different tenants, which might be required when performance measurements are exposed as part of Network Slice as a Service (NSaaS) specified in TS 28.530.

## 16.3	Business Role Models for Network Slicing

| 810051 | Business Role Models for Network Slicing | BRMNS | S1 | SP-180773 | Covell, Betsy, Nokia |
| --- | --- | --- | --- | --- | --- |

Summary based on the input provided by Nokia in SP-200294.

This WI adds requirements to enable a 3GPP system to adequately support the variety of business role models for network slicing that are possible in a 5G system.  The requirements address service exposure giving additional control to 3rd parties, impact of SLAs on network slice management, and non-public network access to MNO spectrum. Also addressed are security relationships between

- a UE and a private slice,

- a private slice and a network, and

- a private slice and other slices of the same network.

The requirements are captured in 3GPP TS 22.261 [1].

The requirements from the BRMNS WI provide the means to support new business role models in vertical markets introduced for supporting vertical markets.  These role models extend the possible interactions between Mobile Network Operators (MNOs) and 3rd parties who use the network and/or own/manage part of the network infrastructure.  In particular, the introduction of network slicing allows new opportunities for 3rd parties to exercise additional control over the portions of the network, i.e., the network slice, supporting their network needs.  Four key business relationship models were considered.

-	Model 3a: MNO provides the virtual/physical infrastructure and V/NFs; a 3rd party uses the functionality provided by the MNO,

-	Model 3b: MNO provides the virtual/physical infrastructure and V/NFs; a 3rd party manages some V/NFs via APIs exposed by the MNO,

-	Model 3c: MNO provides virtual/physical infrastructure; a 3rd party provides some of the V/NFs,

-	Model 3d: a 3rd party provides and manages some of the virtual/physical infrastructure and V/NFs.

Models 3a and 3b are supported by the Rel-15 network slicing requirements, with some enhancements in the Rel-16 APIs and management functions to extend 3rd party access and control of capabilities provided by the MNO, and to do so in a secure manner. In these two models, the 3rd party has greater control over the network capabilities that support its service. However, this control is limited to what is allowed by the MNO through the provided APIs. The Rel-16 enhancements allow the 3rd party to monitor the activities of their UEs when using the network slice and to monitor resource usage within the network slice. These capabilities are added to support SLAs between the MNO and the 3rd party.

As an example of the additional monitoring functionality, a network slice supporting a local smart grid needs additional APIs to monitor activities of many sensors throughout the grid and receive network status information that can be used to detect and resolve any communications problems that arise within the grid.  Figure 2-1 from TR 22.830 [2] illustrates the smart grid role models.

Figure 2-1: role model scenario between MNO and power grid company

Models 3c and 3d resulted in the addition of new functionality to support three new network management models.

- MNO manages all virtual/physical infrastructure and all V/NFs including 3rd party’s ones,

- 3rd party manages its own virtual/physical infrastructure and/or its own V/NFs; MNO manages the others.

- 3rd party manages virtual/physical infrastructure and/or V/NFs including its own virtual/physical infrastructure and/or V/NFs and some MNO’s virtual/physical infrastructure and/or V/NFs; MNO manages the others.

From the 3rd party perspective, the management role models 2 and 3 support the 3rd party management function and provide extended management for the MNO to coordinate with the 3rd party management. The 3rd party may use suitable APIs provided by the MNO to directly manage the V/NFs as well as the infrastructure resources so that it can properly handle when their business requirements are changed.

For models 3c and 3d, additional consideration is needed on the mechanisms to provide the isolation and interfaces that give the 3rd party the appropriate level of control while securing the PLMN. Specifically, where the 3rd party provides V/NFs or provides and manages some of the virtual/physical infrastructure and V/NFs, SLAs may be used to address some trust issues, such as what each party will provide and manage. However, the principles of trust and verification also come into play. The 3rd party must be able to verify, though the management interface, that all terms of the SLA are being met by the MNO. Similarly, the MNO must be able to verify that the 3rd party is managing resources appropriately so that there is no adverse impact to the rest of the network. Appropriate management interfaces are needed to support both the network operator and 3rd party.

Models 3c and 3d allow for additional interactions between non-public networks, private slices, and PLMNs as shown in the following figures from [2].

Figure 2-2: Example of enterprise communications using a combination of 5G PLMNs, a private slice, and a non-public network

Figure 2-3: Example of enterprise communications using a geographically constrained private slice

Figure 2-4: Example of enterprise communications using a private slice throughout the PLMN

Support for enterprise communications may be provided by a variety of network configurations, including multiple PLMNs, private slices, and non-public networks.  New requirements allow for managing the enterprise communications in a coordinated manner across these various implementation options. New requirements also provide for isolation of private network resources which may be constrained for access within a geographic area such as a factory site or restricted for use only by UEs belonging to the enterprise. As a counterpart, additional requirements provide for restricting some enterprise UEs from accessing a PLMN, this may be useful for example to keep factory equipment connected to the private slice or non-public network when a PLMN covers the same area.

References

[1] TS 22.261 Service requirements for the 5G system

[2] TR 22.830 Feasibility Study on Business Role Models for Network Slicing

## 16.4	Enhancement of performance assurance for 5G networks including network slicing

| 810031 | Enhancement of performance assurance for 5G networks including network slicing | 5G_SLICE_ePA | S5 | SP-190247 | Xiaowei Sun (China Mobile) |
| --- | --- | --- | --- | --- | --- |

Summary not yet provided

# 17	UE radio capability signalling optimization

## 17.1	Optimisations on UE radio capability signalling

| 840054 | Optimisations on UE radio capability signalling | RACS | S2 | RP-191460 | Haris Zisimopoulos |
| --- | --- | --- | --- | --- | --- |
| 800025 | Study on RACS | FS_RACS | S2 | SP-180599 | Haris Zisimopoulos, Qualcomm |
| 830027 | Stage 2 of RACS | RACS | S2 | SP-190180 | Haris Zisimopoulos |
| 840003 | CT aspects of RACS | RACS | ct | CP-191061 | Chaponniere, Lena, Qualcomm |
| 840055 | CT1 aspects of RACS | RACS | C1 | CP-191061 | Chaponniere, Lena, Qualcomm |
| 840056 | CT3 aspects of RACS | RACS | C3 | CP-191061 | Chaponniere, Lena, Qualcomm |
| 840057 | CT4 aspects of RACS | RACS | C4 | CP-191061 | Chaponniere, Lena, Qualcomm |
| 830079 | Optimisations on UE radio capability signalling – NR/E-UTRA Aspects | RACS-RAN | R2 | RP-190657 | MediaTek |
| 800097 | Study on optimisations on UE radio capability signalling – NR/E-UTRA Aspects | FS_RACS_RAN | R2 | RP-181459 | MediaTek |
| 830179 | Core part: Optimisations on UE radio capability signalling – NR/E-UTRA Aspects | RACS-RAN-Core | R2 | RP-191088 | MediaTek |

Summary based on the input provided by Qualcomm Incorporated, MediaTek and CATT in SP-200273 (SP-200273 superseds RP-201235, which provides the summary only on the RAN part).

This section summarizes the feature “Optimisations of UE radio capability signalling” across 3GPP specifications (RAN, SA, CT).

Background

With the increase in the size of UE radio capabilities driven by additional supported bands, the size of the UE Radio Capabilities will significantly grow from Rel-15 onwards, therefore an efficient approach to signal UE Radio Capability information is needed.

SA2 concluded that additional optimisation of UE radio capabilities handling should be investigated as part of a dedicated study towards Release 16. A study item [1] was drafted accordingly, along with a corresponding RAN study item [13]. The SA2 study item conclusions are documented in TR 23.743 [2]. The conclusions of the RAN study item are documented in TR 37.873 [14].  Subsequent to the study items, work items were drafted in SA ([5]) and RAN ([10]) to implement optimisations to the UE radio capability signalling.

The work is divided into two subfeatures, UE radio capability ID and UE capability segmentation.

UE Radio Capability ID

The SA2 work item [5] specified system optimisations for the 5GS (documented in TS 23.501 [3]) and for the EPS (documented in TS 23.401 [4]), that apply to both NR and E-UTRA, but not NB-IoT, consisting of using UE Radio Capability IDs as an alternative to signaling the UE Radio Capabilities container in system procedures:

-	between the UE and the CN (over Uu)

-	between the CN and the RAN (impacting N2/S1 interfaces)

-	within the RAN in e.g. the handover procedures (impacting Xn/X2/S1/N2 interfaces)

-	within the CN.

Work was done in collaboration with RAN WGs for the related RAN interfaces and CT WGs for the CN interfaces and NFs. Interworking with nodes that support the feature and those that do not support the feature was also defined.

The UE Radio Capability ID format is defined in TS 23.003 [7]. The UE Radio Capability ID is signaled by the UE in NAS as specified in TS 24.501 [8] for the 5GS and as specified in TS 24.301 [9] for the EPS. Two possible options for the assignment of UE Radio Capability ID exist:

-	Manufacturer-assigned: The UE Radio Capability ID may be assigned by the UE manufacturer in which case it includes a UE manufacturer identification (i.e. a Vendor ID). In this case, the UE Radio Capability ID uniquely identifies a set of UE radio capabilities for a UE by this manufacturer in any network.

-	Network-assigned: If a manufacturer-assigned UE Radio Capability ID is not used by the UE or the serving network, or it is not recognised by the serving network’s UE Capability Management Function (UCMF), the UCMF may allocate UE Radio Capability IDs for the UE corresponding to each different set of UE radio capabilities which the network may receive from the UE at different times. In this case, the UE Radio Capability IDs which the UE receives are applicable to the serving network and uniquely identify the corresponding sets of UE radio capabilities in this network. The network-assigned UE Radio Capability ID includes a Version ID in its format. The value of the Version ID is the one configured in the UCMF, at the time when the UE Radio Capability ID value is assigned. The Version ID value makes it possible to detect whether a UE Radio Capability ID is current or outdated.

UE Radio Capability IDs and the mapping to the corresponding UE radio capabilities are stored in a new function called the UE Capability Management Function (UCMF) in the CN. The UCMF is used for:

-	storage of dictionary entries corresponding to either Network-assigned or Manufacturer-assigned UE Radio Capability IDs.

-	assigning Network-assigned UE Radio Capability ID values.

-	provisioning of Manufacturer-assigned UE Radio Capability ID entries in the UCMF performed from an AF that interacts with the UCMF either directly or via the NEF/SCEF (or via Network Management).

Figure 1: UCMF architecture and related reference points in 5GS (left) and EPS (right)

System procedures are defined for 5GS in TS 23.502 [6] and for EPS in TS 23.401 [4].

UE Capability Segmentation

The RAN work item [10] calls for specification of a segmentation mechanism, so that in cases of excessively large UE capability signalling (e.g. capability information messages exceeding the maximum size of a PDCP SDU), the capability can be segmented into multiple RRC messages.  Segmentation applies to both NR and E-UTRA and is captured respectively in [11] and [12].

Segmentation is performed in the RRC protocol layer, with a separate RRC PDU for each segment.  The UE encodes the capability information message, then divides the encoded message into segments such that the size of each segment does not exceed the maximum size of a PDCP SDU (8188 octets in E-UTRA, 9000 octets in NR); the RAN node (eNB or gNB) receives the segments and reassembles them to reconstruct the original capability information message.  Segmentation is applied only in case the size of the encoded capability information message exceeds the maximum size of a PDCP SDU.  The signalling formats support up to 16 segments for a single capability information message.

References

[1]	SP-180599, “Study on optimisations on UE radio capability signaling”

[2] 	TR 23.743, “Study on optimisations on UE radio capability signaling”

[3]	TS 23.501, “System architecture for the 5G System (5GS)”

[4]	TS 23.401, “General Packet Radio Service (GPRS) enhancements for Evolved Universal Terrestrial Radio Access Network (E-UTRAN) access”

[5]	SP-190180, “Work item on optimisations on UE radio capability signaling”

[6]	TS 23.502, “Procedures for the 5G System (5GS)”

[7]	TS 23.003, “Numbering, addressing and identification”

[8]	TS 24.501, “Non-Access-Stratum (NAS) protocol for 5G System (5GS); Stage 3”

[9]	TS 24.301, “Non-Access-Stratum (NAS) protocol for Evolved Packet System (EPS); Stage 3”

[10]	RP-191088, “Updated WID: Core part: Optimisations on UE radio capability signalling – NR/E-UTRA Aspects”

[11]	TS 38.331, “NR; Radio Resource Control (RRC); Protocol specification”

[12] TS 36.331, “Evolved Universal Terrestrial Radio Access (E-UTRA); Radio Resource Control (RRC); Protocol specification”

[13]	RP-181459, “Study on optimisations on UE radio capability signalling – NR/E-UTRA Aspects”

[14]	TR 37.873, “Study on optimizations of UE radio capability signalling; NR / Evolved Universal Terrestrial Radio Access Network (E-UTRAN) aspects”

# 18	Other system-wide Features

## 18.1	Enablers for Network Automation Architecture for 5G

| 830047 | Enablers for Network Automation for 5G | eNA | S2 | SP-181123 | Xiaobo Wu, Huawei Technologies |
| --- | --- | --- | --- | --- | --- |
| 760047 | Study of enablers for Network Automation for 5G | FS_eNA | S2 | SP-180792 | Xiaobo Wu, Huawei Technologies |
| 820020 | Stage 2 of eNA | eNA | S2 | SP-181123 | Xiaobo Wu, Huawei Technologies |
| 830009 | CT aspects of eNA | eNA | ct | CP-191111 | Yali Yan, Huawei |
| 830048 | CT3 aspects of eNA | eNA | C3 | CP-191111 | Yali Yan, Huawei |
| 830049 | CT4 aspects of eNA | eNA | C4 | CP-191111 | Yali Yan, Huawei |

Summary based on the input provided by Huawei in SP-200254.

In order to improve the NWDAF initiated in Rel 15, the eNA (Enablers for Network Automation for 5G) feature specifies the data collected by NWDAF and the NWDAF output (i.e. statistics and predictions) to support network automation.

The eNA feature includes:

-	Architecture enhancements of 5G System to support network data analytics service

-	A framework to enable data collection and provide analytics to consumers

-	Extensions to existing Nnwdaf services to support the analytics that are required.

In addition, the eNA Work Item is applicable to the eV2XARC Work Item in which the V2X Application Server acting as an Application Function (AF) may consume relevant network data analytics provided by NWDAF for the purposes of adjustment of the application.

Main impacts on the system by the eNA Work Item are as follows:

-	Non-roaming reference architecture for data analytics:

-	Data Collection architecture from any NF:

Figure 1: Data Collection architecture from any NF

-	Network Data Analytics Exposure architecture:

Figure 2: Network Data Analytics Exposure architecture

-	General data collection procedures and network data analytics exposure procedures

-	Collection method of data from 5G NFs, from AF via NEF and from OAM;

-	Analytics exposure to the registered Consumer NF, when the NF is an AF located outside the MNO domain, analytics are provided via NEF.

-	For each Analytics ID, the following was specified:

-	General description;

-	Input data consumed by NWDAF to derive the network data analytics;

-	Output Analytics including details of the parameters;

-	Procedure for NWDAF providing the network data analytics.

-	Possible usage of analytics by some consumer NFs is defined for the following Analytics ID:

-	Analytics ID = "Service experience"

-	If the Consumer NF is PCF, the PCF may check the 5QI values assigned to the Application, and may use this as input to calculate and update the authorized QoS for a service data flow template.

-	Analytics ID = "NF load information"

-	If the Consumer NF is AMF, based on the SMF Load, AMF could select a suitable SMF during the PDU Session Establishment or Modification procedure.

-	If the Consumer NF is SMF, based on the UPF Load, SMF could select a suitable UPF during the PDU Session Establishment or Modification procedure.

-	Analytics ID = "Network Performance"

-	If the Consumer NF is PCF, then PCF could help determine suitable background data transfer policies that fulfills Network Performance requirements t.

-	Analytics ID = "UE mobility"

-	If the Consumer NF is AMF, then AMF may use it to optimize the UE's paging strategy or to learn expected UE behavior parameters for deriving appropriate MICO mode configuration.

-	If the Consumer NF is SMF, based on the UE Moving Trajectory, the SMF could select a suitable UPF during the PDU Session Establishment or Modification procedure.

-	If the Consumer NF is UDM, the UDM may store the UE Mobility analytics as the subscription data for the UE and provision it to the AMF to help monitoring the UE's mobility behavior.

-	Analytics ID = "UE Communication"

-	If the Consumer NF is SMF, based on the UE Communication analytics, the SMF could select a suitable UPF during the PDU Session Establishment or Modification procedure.

-	If the Consumer NF is UDM, the UDM may store the UE Communication analytics as the subscription data for the UE and provision it to the SMF to help monitoring the UE's communication behavior.

-	Analytics ID = "Abnormal behaviour"

-	If the Consumer NF is PCF, based on different Exception IDs in the analytics, the PCF could make different policies, depending on operator defined policies. Some examples are provided for illustration purposes such as the PCF may use "Unexpected UE location" as input to adjust the Service Area Restrictions, "Suspicion of DDoS attack" to request the SMF to terminate the PDU session, "Wrong destination address" to perform gating of a service data flow and "Unexpected long-live/large rate flows" to perform QoS related policies such as gating or policing. In another abnormal behavior example, the AMF based on “Ping-Pong UE” Exception ID, may adjust the UE registration area.

-	Analytics ID = "User Data Congestion"

-	If the Consumer NF is an AF interfacing through NEF, the NEF could provide the User Data Congestion analytics to the AF to help optimize the application information.

-	Analytics ID = "QoS Sustainability"

-	If the Consumer is V2X Application Server acting as an AF, the V2X Application Server can use this analytics for the purposes of adjustment of the application, e.g. adjust inter-vehicle gap, change video codec parameters, etc.

-	NWDAF services to expose Network Data Analytics to the Consumer NFs are specified. Two models are defined:

-	Subscribe-Notify model, i.e. Nnwdaf_AnalyticsSubscription_Subscribe / Nnwdaf_analyticsSubscription_Notify, to allow provide continuous data analytics exposure from NWDAF to the Consumer NF;

-	Request-Response model, i.e. Nnwdaf_AnalyticsInfo_Request / Nnwdaf_AnalyticsInfo_Request response, to provide a one-time data analytics exposure response from the NWDAF to the requesting Consumer NF.

References

[1]	3GPP TS 23.501: "System Architecture for the 5G System; Stage 2".

[2]	3GPP TS 23.502: "Procedures for the 5G System; Stage 2".

[3]	3GPP TS 23.503: "Policy and Charging Control Framework for the 5G System; Stage 2".

[4]	3GPP TS 23.288: "Architecture enhancements for 5G System (5GS) to support network data analytics services".

[5]	3GPP TS 28.550: "Management and orchestration; Performance assurance".

[6]	3GPP TS 29.520: "5G System; Network Data Analytics Services; Stage 3".

[7]	3GPP TS 29.517: "5G System; Application Function Event Exposure Service; Stage 3".

## 18.2	Provision of Access to Restricted Local Operator Services by Unauthenticated UEs

| 760003 | Provision of Access to Restricted Local Operator Services by Unauthenticated UEs | PARLOS |  | SP-170449 | Covell, Betsy Nokia |
| --- | --- | --- | --- | --- | --- |
| 740002 | Study on Stage 1 of PARLOS | FS_PARLOS | S1 | SP-160904 | Covell, Betsy |
| 760046 | Study on Stage 2 for PARLOS | FS_PARLOS_SA2 | S2 | SP-180501 | Nokia (Nicolas Drevon |
| 800035 | Study on Security Aspects of PARLOS | FS_PARLOS_Sec | S3 | SP-180442 | Greg Schumacher, Sprint |
| 760071 | Stage 1 of PARLOS | PARLOS | S1 | SP-170449 | Covell, Betsy Nokia |
| 810008 | Stage 2 of PARLOS | PARLOS | S2 | SP-180738 | Nokia (Nicolas Drevon |
| 830012 | CT aspects of PARLOS | PARLOS | ct | CP-190197 | Liu, Jennifer; Nokia |
| 830062 | CT1 aspects of PARLOS | PARLOS | C1 | CP-190197 | Liu, Jennifer; Nokia |
| 830063 | CT3 aspects of PARLOS | PARLOS | C3 | CP-190197 | Liu, Jennifer; Nokia |
| 830064 | CT4 aspects of PARLOS | PARLOS | C4 | CP-190197 | Liu, Jennifer; Nokia |
| 830065 | CT6 aspects of PARLOS | PARLOS | C6 | CP-190197 | Liu, Jennifer; Nokia |

Summary based on the input provided by Nokia in SP-200215.

This WI adds requirements to enhance the 3GPP PS Domain to provide an optional capability to allow unauthenticated UE's to access restricted local operator services based on operator policy and regional regulatory requirements. The requirements address identifying when restricted local operator services are available and enabling a UE to attach to a network for the purpose of accessing a restricted local operator service even if the UE is not able to be authenticated by the network. The requirements can be found in 3GPP TS 22.101 [1], TS 22.115 [2], and TS 22.228 [3].

The PARLOS WI was driven primarily by FCC regulations for manual roaming as described in [4] and [5]. The ability to provide access to such local services has been available to U.S. operators on a proprietary basis. However, the wide deployment of LTE and corresponding introduction of VoLTE creates demand for a standardized mechanism to allow a UE to access these services such as manual roaming without necessarily being successfully authenticated for access. This optional functionality of supporting access to RLOS services to unauthenticated UEs can be deployed based on operator policy. The RLOS services themselves are out of scope of 3GPP.  PARLOS is an LTE capability.

The PARLOS requirements provide for a network to inform UEs using a 3GPP access technology that RLOS services are available and to allow access to the RLOS services when requested by the UE without requiring successful authentication of the UE first. The services are provided in an isolated manner that prevents an unauthenticated UE from accessing any other service or functionality in the network. Additional requirements also provide for collection of charging information related to the use of RLOS services.

References

[1] TS 22.101 Requirements for access to restricted local operator services

[2] TS 22.115 Charging for restricted local operator services

[3] TS 22.228 Requirements for access to restricted local operator services

[4] Code of Federal Regulations (CFR) Title 47 Chapter 1 Subchapter B Part 20 Section 20.3

[5] Code of Federal Regulations (CFR) Title 47 Chapter 1 Subchapter B Part 20 Section 20.12 (Resale and Roaming)

## 18.3	Enhancing Topology of SMF and UPF in 5G Networks

| 820043 | Enhancing Topology of SMF and UPF in 5G Networks | ETSUN |  | SP-181116 | Laurent Thiebaut (Nokia) |
| --- | --- | --- | --- | --- | --- |
| 770039 | Study on ETSUN | FS_ETSUN | S2 | SP-180731 | Laurent Thiebaut (Nokia) |
| 820013 | Stage 2 of ETSUN | ETSUN | S2 | SP-181116 |  |
| 830002 | CT aspects of ETSUN | ETSUN | ct | CP-190192 | Landais, Bruno, Nokia |
| 830055 | CT3 aspects of ETSUN | ETSUN | C3 | CP-190192 | Landais, Bruno, Nokia |
| 830056 | CT4 aspects of ETSUN | ETSUN | C4 | CP-190192 | Landais, Bruno, Nokia |
| 850024 | Charging aspects of ETSUN | ETSUN | S5 | SP-190880 | Dong, Jia, China Mobile |

Summary based on the input provided by Nokia in SP-200256.

Enhancing Topology of SMF and UPF in 5G Networks (ETSUN) has two goals:

1. Enable the 3GPP system to support deployments where a SMF is not able / allowed to control UPF(s) throughout the same PLMN.

-	This is mostly meant for very big networks which are subdivided into geographical areas with each their own management;

-	This relates with the addition of I-SMF in the architecture but defines also the possibility of changing the V-SMF in case of Home-Routing.

2. Enhance the capability of 5GS architecture for a UPF to be controlled by multiple SMF's (and many UPF's to be controlled by many SMFs) especially for the UE IP address / Prefix allocation

These features couldn’t be specified as part of 3GPP R15 Session Management due to lack of time.

Addition of I-SMF in the architecture

When a UE with an established PDU Session is not in a TA served by the SMF of this PDU Session, an intermediate SMF (called I-SMF) is inserted by the AMF in the signalling path of the PDU session to control the intermediate UPF (called I-UPF) terminating the N3 interface with the 5G Access Network.

Figure 1: Non-roaming architecture with I-SMF insertion to the PDU Session in reference point representation, with no UL-CL/BP

The I-SMF acts in a similar way than the V-SMF of a Home Routed PDU Session:  as long as the UE remains in its own I-SMF service area, the I-SMF handles the transitions between User Plane Active and Inactive for the PDU session, the Hand-Over and controls the local UPF(s) with sometimes relaying of N4 commands received from the SMF. The SMF remains responsible of the interfaces with the PCF and the CHF, of the interface with the UDM and of the overall control of the PDU Session.

When the I-SMF is inserted into a PDU Session, the I-SMF provides to the SMF the list of DNAI it supports. Based on this information received from I-SMF and on PCC rules for the PDU Session, the SMF may provide the DNAI(s) of interest for local traffic steering to the I-SMF for this PDU Session. The I-SMF is thus responsible for the insertion, modification and removal of UPF(s) to ensure local traffic steering: the SMF does not need to know the mapping between DNAI(s) for local traffic offload and local UPF(s). Then the SMF provides N4 rules to the I-SMF for how the traffic for local offload shall be detected, enforced, monitored in UPF(s) controlled by the I-SMF.

When the UE moves out of the service area of the I-SMF, the I-SMF may be changed or the I-SMF is simply removed. Likewise, the V-SMF may be changed upon UE mobility in case of Home-Routing.

UE IP address / Prefix allocation

ETSUN adds the possibility for the SMF to defer to the UPF the UE IP address / Prefix allocation or to indicate to the DHCP/DN-AAA server the range (corresponding to the UPF) of IP address / Prefix to be allocated : this allows many SMF(s) to control the same UPF without having to synchronise or divide the UE IP address / Prefix space between these SMF(s).

Charging aspects (Summary provided by China Mobile in SP-200266)

SA2, CT3 and CT4 have studied the enhanced topology deployments of SMF and UPF in 3GPP TR 23.726, TS 23.501 and 23.502. S5 WI ETSUN specify charging support for deployments topologies with specific SMF service areas to line up with the other NF interfaces.

Charing Stage 2 work on WI ETSUN for TS 32.255 [3]:

-	Specify charging procedures and functionality enhancement for PDU sessions with Intermediate SMF (I-SMF) and anchor SMF (SMF), based on existing Nchf to SMF.

-	Introduction of V-SMF change in Roaming HR principles and message flows.

-	Enhance procedures for addition /change/removal of PSA for UL CL or BP controlled by I-SMF.

-	Update charging procedures with I-SMF insertion/change/removal.

-	Add I-SMF related triggers in SMF.

Charging Stage 3 work on WI ETSUN for 32.291 [4] and 32.298 [5]:

-	Add relevant parameters in CHF-CDR to support I-SMF as serving network function.

-	Add I-SMF related trigger, trigger types and data type.

-	Update Nchf_ ConvergedCharging API.

References

For the addition of I-SMF in the architecture, ETSUN is mostly specified in TS 23.501 [1] clause 5.34 and in TS 23.502 [2] clause 4.23 (both clauses are dedicated to ETSUN).

For the aspects related with UE IP address allocation, ETSUN is mostly specified in a CR 0931/0954 (SP-190164) to TS 23.501 [1] clause 5.8.2.2.1.

[1]	3GPP TS 23.501: "System Architecture for the 5G System; Stage 2".

[2]	3GPP TS 23.502: "Procedures for the 5G System; Stage 2".

[3]	TS 32.255: "5G data connectivity domain charging"

[4] 	TS 32.291: "5G system, charging service"

[5] 	TS 32.298: "Charging Data Record (CDR) parameter description"

## 18.4	Private Network Support for NG-RAN

| 830081 | Private Network Support for NG-RAN | NG_RAN_PRN | R3 | RP-200122 | China Telecom |
| --- | --- | --- | --- | --- | --- |
| 830181 | Core part: Private Network Support for NG-RAN | NG_RAN_PRN-Core | R3 | RP-200122 | China Telecom |

Summary based on the input provided by China Telecom in RP-200733.

Introduction

In Rel-16, this work item specified the Private Network (i.e. Stand-alone Non-Public Network and PLMN Integrated Non-Public Network) features for gNB with the following objectives:

Support NPN functionality in NG-RAN:

-	CAG/SNPN relevant parameter broadcast from SIB

-	CAG/SNPN cell selection/reselection

-	CAG/SNPN cell access control

-	For CAG, in the case of Intra-RAT intra-system and inter-RAT intra-system, the connected mode mobility support

-	The connected mode mobility support within SNPN

For CAG/SNPN, necessary modifications to NG-C and Xn interfaces to communicate the CAG-ID/NID related parameters to NG-RAN nodes, respectively

Support CAG/SNPN functionality with CU-DU split

Support CAG/SNPN functionality with CP-UP split, if any

The key functionalities of this work item include the following.

NPN relevant parameter broadcast from SIB

The NPN IDs (i.e. SNPNs (identified by PLMN ID + NID) and PNI-NPNs (identified by PLMN ID + CAG ID)) were introduced into SIB1 to indicate UEs whether a cell is an NPN cell. Up to 12 different SNPNs or PNI-NPNs or mixed networks can be broadcasted in a cell. SIB1 allows indication of TAC, RANAC, cell Identity per SNPN or per PNI-NPN.

The names of NPNs (HRNN) are broadcasted in SIB10 and the On-demand SI in connected is not supported for SIB10. The HRNN is associated with the Network ID implicitly. The SIB for HRNN shall have the same amount of HRNN elements as the number of CAGs and NIDs in SIB1.

New defined IE ‘manualCAGselectionAllowed-r16’ is used to indicate whether it is allowed users to manually select a CAG-ID supported by the CAG cell but outside the UE’s allowed CAG list.

Only cells supporting CAG(s), including CAG only cells and shared CAG cells, may be listed in the new CAG PCI lists, UE may use knowledge of the CAG PCIs to improve implementation dependent search procedures for CAGs.

NPN cell selection/reselection

NPN selection functions similar to normal PLMN selection, when a cell broadcasts any CAG IDs or NIDs, NPN-capable Rel-16 UE can treat the cell with cellReservedForOtherUse = true as a candidate cell during cell selection and cell reselection. UE AS reports the found NPN IDs to NAS. In case of manual selection, the human readable network name (if broadcasted) may also be provided from AS to NAS.

The UE shall scan all RF channels in the NR bands according to its capabilities to find available NPNs. On each carrier, the UE shall at least search for the strongest cell, read its system information and report available NPN identifiers together with their HRNN (if broadcast) to the NAS.

For a UE in NPN access mode, if the highest ranked cell or best cell according to absolute priority reselection rules is a cell which is not suitable for UE access, for licensed spectrum, the UE shall not consider this cell and other cells on the same frequency as candidates for reselection for a maximum of 300 seconds; for unlicensed spectrum, the UE shall not consider this cell as candidate for cell reselection but should continue to consider other cells on the same frequency for cell reselection.

The Rel-15 UEs and Non-NPN-capable Rel-16 UEs treat a cell with cellReservedForOtherUse=true as barred cell.

For Rel-16 and later NPN-capable UEs, if the npn-IdentityInfoList-r16 IE is present in CellAccessRelatedInfo and the cellReservedForOtherUse = true while, a cell is an NPN-only cell that is only available for normal service for NPNs’ subscriber.

All the R16 UEs will treat the cell as barred when the legacy IE cellReservedForOtherUse=True and this cell does not broadcast any CAG-IDs or NIDs.

For SNPN, Once the UE has selected an SNPN, cell selection/re-selection is only performed within the SNPN, i.e. a cell is only considered suitable if the broadcasted SNPN identifier matches the selected SNPN.

NPN cell access control

There is no preliminary access check for NPN cells in CONNECTED mode. For SNPN, the UAC parameters per SNPN are configured by reusing the existing uac-BarringPerPLMN-List, and for a PNI-NPN the UAC parameter set is selected based on the PLMN ID of PNI-NPNs. The current measurement reporting procedures is extended to include NPN information to support ANR.

During the access procedure, for SNPN, the SNPN ID is included in the RRCSetupComplete and Initial UE Message message, and for PNI-NPN the Allowed CAG list is provided to the gNB by the AMF. A single Cause “NPN access denied” will be sent to users when they fail to access NPN cells.

NPN relevant mobility support

At mobility, the source NG-RAN node knows the NPN information supported by the candidate target cells, the target RAN node needs to be informed of the serving SNPN ID or UE allowed CAG ID list which are included in the mobility restriction list. If the serving SNPN ID or UE allowed CAG ID list does not match any of the target cell supported list of SNPN IDs or CAG IDs, target RAN node shall fail the handover.

For mobility in inactive state, the last serving NG-RAN node performs access control check upon the reception of RETRIEVE UE CONTEXT REQUEST, the new NG-RAN node may perform access control check upon the reception of RETRIEVE UE CONTEXT RESPONSE which is implementation dependent.

NPN Support over NG/Xn

The list of supported SNPN IDs could be exchanged between NG-RAN node and AMF via NG setup and configuration update procedures while the list of cell supported SNPN IDs and CAG IDs could be exchanged via Xn setup and configuration update procedures.

NPN Support over F1/E1

Over F1, the list of supported SNPN IDs can be exchanged between DU and CU, the cell supported list of CAG IDs can be signaled from DU to CU. The NID as part of the UAC Assistance Information could be signalled from gNB-CU to gNB-DU. The gNB-DU is responsible for SIB10 encoding, valueTag and areaScope associated with SIB10, and need to signal the HRNN (SIB 10) to gNB-CD. The general cause value "NPN not supported" is introduced for interface related messages and the general cause value "NPN access denied” is introduced for UE-associated messages. Over E1, the list of SNPN IDs supported by CU-UP could be signalled to CU-CP.

NPN support for RAN sharing and Dual connectivity

If NR access is shared, system information broadcast in a shared cell indicates a TAC and a Cell Identity for each subset of PLMNs, PNI NPNs and SNPNs. NR access provides only one TAC and one Cell Identity per cell per PLMN, SNPN or PNI NPN. In Rel-16, a Cell Identity can only belong to one network type among PLMN, PNI-NPN or SNPN.

The NR-DC within a single SNPN or within PNI-NPN or across PLMN and PNI-NPN is supported, EN-DC is not supported for NPN.

Emergency Services

For SNPN, Emergency services are not supported.

For a CAG-only cell in PNI-NPN, if the cellReservedForOtherUse = false in gNB, the access attempts of Rel-15 UEs or Rel-16 non-NPN capable UEs for emergency services could be allowed; if the cellReservedForOtherUse = true, the access attempts of Rel-16 NPN capable UEs for emergency services could be allowed.

## 18.5	Service-Based Architecture

### 18.5.1	Enhancements to the Service-Based 5G System Architecture

| 820045 | Enhancements to the Service-Based 5G System Architecture | 5G_eSBA | S2 | SP-181125 |  |
| --- | --- | --- | --- | --- | --- |
| 790007 | Study on Enhancements to the Service-Based 5G System Architecture | FS_eSBA | S2 | SP-180231 | Tao Sun (China Mobile) |
| 820022 | Stage 2 of 5G_eSBA | 5G_eSBA | S2 | SP-181125 | Tao Sun (China Mobile) |
| 830001 | CT aspects of 5G_eSBA | 5G_eSBA | ct | CP-190191 | Song Yue (China Mobile) |
| 830060 | CT3 aspects of 5G_eSBA | 5G_eSBA | C3 | CP-190191 | Song Yue (China Mobile) |
| 830061 | CT4 aspects of 5G_eSBA | 5G_eSBA | C4 | CP-190191 | Song Yue (China Mobile) |
| 780029 | Study on Enhanced IMS to 5GC Integration | FS_eIMS5G | S2 | SP-180736 | Joul, Chris, T-Mobile USA |

Summary not yet provided

### 18.5.2	SBA aspects of enhanced IMS to 5GC integration

| 840062 | SBA aspects of enhanced IMS to 5GC integration | eIMS5G_SBA | S2 | SP-190181 | T-Mobile USA, Christopher Joul |
| --- | --- | --- | --- | --- | --- |
| 830028 | Stage 2 of eIMS5G_SBA | eIMS5G_SBA | S2 | SP-190181 | T-Mobile USA, Christopher Joul |
| 840006 | CT aspects of eIMS5G_SBA | eIMS5G_SBA | ct | CP-191065 | de Gregorio, Jesus. Ericsson |
| 840063 | CT3 aspects of eIMS5G_SBA | eIMS5G_SBA | C3 | CP-191065 | de Gregorio, Jesus. Ericsson |
| 840064 | CT4 aspects of eIMS5G_SBA | eIMS5G_SBA | C4 | CP-191065 | de Gregorio, Jesus. Ericsson |

Summary not yet provided

## 18.6	User data interworking, Coexistence and Migration

| 840065 | User data interworking, Coexistence and Migration | UDICOM | S2 | SP-190182 | Alessio Casati, Nokia |
| --- | --- | --- | --- | --- | --- |
| 800055 | Study on UDICOM | FS_UDICoM | S2 | SP-190148 | Susana Sabater (Vodafone |
| 830029 | Stage 2 of UDICOM | UDICOM | S2 | SP-190182 | Alessio Casati, Nokia |
| 840007 | CT aspects of UDICOM | UDICOM | C4 | CP-191066 | Wiehe, Ulrich, Nokia |

Summary not yet provided

# 19	Radio Features

## 19.1	NR-related Release 16 Features

### 19.1.1	NR-based access to unlicensed spectrum

| 820067 | NR-based access to unlicensed spectrum | NR_unlic | R1 | RP-191575 | Qualcomm |
| --- | --- | --- | --- | --- | --- |
| 750045 | Study on NR-based access to unlicensed spectrum | FS_NR_unlic | R1 | RP-181339 | Qualcomm |
| 820167 | Core part: NR-based access to unlicensed spectrum | NR_unlic-Core | R1 | RP-190706 | Qualcomm |
| 820267 | Perf. part: NR-based access to unlicensed spectrum | NR_unlic-Perf | R4 | RP-190706 | Qualcomm |

Summary not yet provided

### 19.1.2	2-step RACH for NR

| 820068 | 2-step RACH for NR | NR_2step_RACH | R1 | RP-190711 | ZTE |
| --- | --- | --- | --- | --- | --- |
| 820168 | Core part: 2-step RACH for NR | NR_2step_RACH-Core | R1 | RP-190711 | ZTE |
| 820268 | Perf. part: 2-step RACH for NR | NR_2step_RACH-Perf | R4 | RP-190711 | ZTE |

Summary based on the input provided by ZTE Corporation in RP-200087 revised to RP-200623 revised to RP-201225.

The Rel-16 Work Item 2-step RACH for NR achieves the following objectives:

-	A simplified random access procedure was developed. This reduces the number of interactions between the UE and network during the connection setup and connection resume, thereby enabling a lower control plane latency. In case of connected mode, a small amount of data can be sent via 2-step RACH procedure thus also enabling a lower latency for UL UP data for connected mode UEs.

-	Channel structure of transmitting PRACH and PUSCH in one step (i.e. without an intermediate message from the network) was developed. The PRACH and PUSCH are separated by a pre-configured guard period.

-	The above enhancements are applicable to both licensed spectrum and shared spectrum (i.e. NR-U).

The general procedure of 4-step RACH and 2-step RACH are depicted in Figure 1. The first step of 2-step RACH comprises an UL MSGA transmission which includes the equivalent contents of msg1 and msg3 of 4-step RACH. The second step of 2-step RACH is a DL MSGB reception which includes the equivalent content of msg2 and/or msg4 of 4-step RACH, depending on the detection of UL MSGA.

(a) 4-step RACH	                                                (b) 2-step RACH

Figure 1 General procedure of 4-step RACH and 2-step RACH

RA type selection

For contention based random access (CBRA), all the triggers for Rel-15 NR 4-step RACH are also applicable to 2-step RACH except when CA is configured, the 2-step RACH is only applicable on PCell. Contention free random access (CFRA) procedure with 2-step RACH is only supported for handover.

The UE selects the type of random access at initiation of the random access procedure based on network configuration:

-	when CFRA resources are not configured, an RSRP threshold is used by the UE to select between 2-step RA type and 4-step RA type;

-	when CFRA resources for 4-step RA type are configured, UE performs random access with 4-step RA type;

-	when CFRA resources for 2-step RA type are configured, UE performs random access with 2-step RA type.

In case of random access in a cell configured with SUL, UE performs carrier selection (between SUL and NUL) before selecting between 2-step and 4-step RA type.

MSGA structure: PRACH

The MSGA in 2-step RACH comprise a PRACH and a PUSCH. The PRACH resources for 2-step RACH in time/frequency domain can be either shared with 4-step RACH or can be configured to be separate. All the preamble formats and the PRACH configuration indexes defined in Rel-15 and in Rel-16 NR-U and TEI can be used. In case of shared time domain PRACH resources between 4-step RACH and 2-step RACH, different preambles are allocated to differentiate the RA types.  The mapping between SSB and PRACH occasion reuses that for 4-step RACH.

MSGA structure: PUSCH

2-step RACH uses a specified mapping rule to determine the PUSCH resource of MSGA that is associated with the selected PRACH resource. Each PRACH slot is mapped to a number of PUSCH occasions with associated DMRS resource, once the UE selects a preamble in a PRACH occasion, the corresponding PUSCH occasion and DMRS resource can be determined by a predefined mapping order.

MSGB

After MSGA transmission, the UE monitors the downlink for a response from the network within a configured window. This response from the network is called the MSGB. The contents of MSGB depend on whether or not the gNB is able to successfully detect both the PRACH and the PUSCH parts.

-	If the PRACH is detected but the decoding of PUSCH fails, network will include a fallback indication in MSGB and the subsequent UE procedure will be similar to that for a UE monitoring msg2 in the 4 step RACH.

-	If both preamble and PUSCH are decoded, network will include a successRAR and reception of this at the UE completes the contention resolution. HARQ feedback is enabled for the successful reception of the successRAR.

References

[1]	RP-200085, Revised WID on 2 step RACH for NR

[2]	RP-200622, Status report for WI - NR 2-step RACH

### 19.1.3	UE Power Saving in NR

| 830075 | UE Power Saving in NR | NR_UE_pow_sav | R1 | RP-191607 | CATT |
| --- | --- | --- | --- | --- | --- |
| 800094 | Study on UE power saving in NR | FS_NR_UE_pow_sav | R1 | RP-181463 | CATT |
| 830175 | Core part: UE Power Saving in NR | NR_UE_pow_sav-Core | R1 | RP-190727 | CATT |
| 830275 | Perf. part: UE Power Saving in NR | NR_UE_pow_sav-Perf | R4 | RP-190727 | CATT |

Summary based on the input provided by CATT in RP-200912.

UE battery life is an important aspect of the user’s experience.   The RAN1 study of the Rel-16 UE power saving had shown substantial power saving gain comparing to considered Rel-15 NR features such as DRX operation, with UE adaptation in frequency domain, time domain, antenna domain, tight control of DRX operations, and reducing PDCCH monitoring with different traffic types.

The work item of UE power saving in NR includes the power saving techniques, such as DRX adaptation, cross-slot scheduling, and maximum MIMO layer adaptation in CONNECTED state, fast transition out of CONNECTED state, and reduced RRM measurements in idle/inactive states.  The UE assistance information is part of the work to enable the UE to feedback its preferred configuration to achieve desired power saving.

The UE power saving work in Rel-16 focuses on the power saving techniques  in CONNECTED state, which includes DRX adaptation, cross-slot scheduling, maximum MIMO layer adaptation, and fast transition out of CONNECTED state.  The RRM measurement reductions are the power saving techniques specified in idle/inactive states.   UE assistance information is supported for the UE to feedback its preferred configuration of the specific power saving technique.

Power Saving Techniques in CONNECTED state

The power saving techniques are dynamically triggered by L1 signaling indicated from PDCCH-based power saving signal/channel or semi-statically configured by RRC signaling.   The PDCCH-based power saving signal/channel reuses the existing PDCCH search space and CORESET configurations with dynamic TCI states with DCI field indicating the adaptation to achieve UE power saving, such as UE wakeup in the DRX operation, cross-slot scheduling, and maximum MIMO layer adaptation through BWP switching.

-	DRX adaptation

The DRX adaptation power saving technique is to configure the PDCCH-based power saving signal/channel at the active BWP before the beginning of DRX ON for UE monitoring with the indication of UE wakeup or not depending on whether there is data for UE to receive.    A new DCI format 2_6 is introduced with CRC scrambled by PS-RNTI (DCP) which contains the wakeup indication as well as SCell dormancy indication if configured.   A PS-offset is semi-statically configured before DRX ON defining the start of the interval for the DCP monitor occasion as shown in Figure 1.   More than one monitoring occasions could be configured for DCP on PCell for CA and SpCell for DC based on the search space and CORESET configurations.   Minimum time gap is specified as the UE processing time as shown in Figure 1.  UE is not required to monitor DCP at the interval of minimum time gap and within Active Time.

Figure 1: DCP Monitoring occasion for DRX adaptation

When DCP monitoring occasion collides with other procedures with higher priority in PDCCH monitoring, the monitoring occasion is considered invalid.   UE follows legacy behavior when all the configured monitoring occasions are invalid.   UE is configured by RRC to wake up or not when no DCP is detected with valid monitoring occasions.   One DCP can be configured to control PDCCH monitoring during on-duration for one or more UEs independently.  UE is also configured by RRC whether to report periodic L1-RSRP or periodic CSI/L1-SINR when UE is not indicated to wake up at the DRX ON.

-	Cross slot scheduling

Power saving technique with cross-slot scheduling facilitates UE to achieve power saving with the assumption that it won’t be scheduled to receive PDSCH, triggered to receive A-CSI or transmit a PUSCH at the scheduling slot within Active Time.  A 1-bit minimum scheduling offset in DCI format 1_1 and 0_1 enables dynamic switching of DL and UL minimum scheduling offset values.

-	Maximum MIMO Layer Adaptation

UE power saving techniques with the adaptation to the DL maximum number of MIMO layers could be achieved by dynamic switching of BWPs, which the DL maximum number of MIMO layers are configured to be different.

-	   Fast transition out of CONNECTED state

UE can feed back the assistance information of its preference to be released/suspended for gNB to get UE transitioning out of CONNECTED state quickly when there is no further data arrival.

Power Saving Techniques in idle/inactive state

-	Reduced RRM measurements in idle/inactive state

Power saving in RRC_IDLE and RRC_INACTIVE can also be achieved by UE relaxing neighbour cells RRM measurements when it meets the criteria determining it is in low mobility and/or not at cell edge.

UE assistance information

UE assistance information allows the UE to feedback its preferred configuration, such as c-DRX configuration, aggregated bandwidth, SCell configuration, MIMO configuration, RRC state, minimum scheduling offset values in order for network to assist UE achieving power saving gain.

References

[1]	RP-200494, “WID: UE power saving in NR”, CATT, CAICT

[2]	TR 38.840 v15.0.0. CATT

[3]	R1-1913657, 38.202CR0014 Introduction of UE power savings, Qualcomm

[4]	R1-1913658, 38.212CR0028   Introduction of UE power savings in 38.212, Huawei

[5]	R1-1913659, 38.213CR0076 Introduction of UE power savings, Samsung

[6]	R1-1913660, 38.214CR0056 Introduction of UE power savings, Nokia

[7]	R2-2002369, 38.300 CR0193 Introduction of UE Power Saving in NR, CATT

[8]	R2-2005949, 38.304 CR 0158, CR on 38.304 for UE Power saving in NR, vivo

[9]	R2-2005869, 38.321CR0719 MAC CR for Rel-16 UE power savings, Huawei, HiSilicon, Nokia, Nokia  Shanghai Bell

[10]	R2-2004943, 38.331 CR1540  CR for 38.331 for power savings. MediaTek

[11]	R2-2004944,  36.331 CR4245  CR for 36.331 for power savings, MediaTek

[12]	 R2-2005856, 38.306CR0134  UE capabilities for Rel-16 Power Saving WI,  Intel

[13]	 R2-2005857, 38.331CR1618  UE capabilities for Rel-16 Power Saving WI, Intel

[14]	 R2-2005866, 37.340CR0189 SRB3 for reporting UAI for power saving,  OPPO, MediaTek

[15]	R4-2009133  38.133CR0854 Measurement requirements for UEs under power saving mode 	Ericsson

[16]	R4-2009244  38.133CR0635  CR on minimum requirement at transition period for UE power saving	CATT

[17]	R4-2007440   38.133 CR0736  CR for maximum MIMO layer adaptation vivo, CATT

### 19.1.4	Dual Connectivity (EN-DC) with 3 bands DL and 3 bands UL

| 841000 | Dual Connectivity (EN-DC) with 3 bands DL and 3 bands UL | DC_R16_LTE_NR_3DL3UL | R4 | RP-191598 | ZTE |
| --- | --- | --- | --- | --- | --- |

Summary based on the input provided by ZTE Corporation in RP-200112.

In Release 15, the operating bands are specified for these eligible operations with EN-DC configured. The EN-DC band combinations include at least one E-UTRA operating band and a NR operation band. In moving forward to Release 16, the configuration of EN-DC operation needs to be expanded for more simultaneous uplink and downlink configurations, including band combination of LTE, FR1 and FR2 bands.

New configurations still emerge from existing bands and whenever new band is specified, it will create a potential for several new EN-DC configurations consisting of 3 different bands DL with 3 different bands UL (1 LTE band UL and 2 NR different band UL(i.e. 1CC LTE band +  1CC NR FR1 band +  NR FR2 band), or 2 LTE different band UL and 1 NR FR2 band UL).

The EN-DC configurations including NR CA will be introduced in a release independent manner based on TS38.307, which will be updated depending on newly introduced EN-DC configurations including NR CA.

All new ENDC configurations including NR with 3 bands  will be defined under this WI, where :

-	For only 1 NR band included, only NR FR2 band is applied

-	For only 2 NR bands included, 1 NR FR1 band  and 1 NR FR2 band are included and operated as inter-band CA

The companies can request their interesting ENDC configurations before each 3GPP RAN4 meeting. The WID was started from RAN #84 meeting, and will completed at RAN #88 meeting.

Usually these configurations are the potential NR NSA deployment scenarios for the operators.  Each of the requested ENDC configurations includes at least applicable frequencies if necessary, applicable bandwidths and bandwidth sets if necessary, and Downlink ENDC configurations and Uplink ENDC configuration, respectively. More importantly, each ENDC configuration shall be requested with at least three other supporting companies. In addition,  these combinations will be introduced in a Rel-independent way starting from Rel-15.

All of the ENDC configurations requested by the companies are all captured in two tables in the WID, shown as follow.

1. EN-DC of 3 different bands DL(LTE 1 band + NR 2 bands) with 3 different bands UL

2. EN-DC of 3 different bands DL(LTE 2 bands + NR 1 band) with 3 different bands UL

For the ENDC configuration in the WID, the RF requirements shall be specified, including:

Analyse combinations that have self-desensitization due to following reasons:

-	TX Harmonic and/or intermodulation overlap of receive band

-	TX signal overlap of receiver harmonic frequency

-	TX frequency being in close proximity of one of the receive bands

-	Any other identified reasons such that insufficient cross band isolation, harmonic mixing

For the combination where self-desensitization exists, specify at least needed

-	∆TIB, c and ∆RIB, c

-	Reference sensitivity exceptions including MSD test cases

All of the study results are captured into TR37.716-33, and for the ENDC configurations will be included in TS38.101-3 specification in the case of these configurations are approved in RAN4 meeting.

References

[1]	RP-192537,  Revised WID for Dual Connectivity (EN-DC) with 3 bands DL and 3 bands , ZTE Corporation

[2]	R4-1913627, CR to reflect the completed ENDC combinations for 3 bands DL with 3 bands UL into Rel16 TS 38.101-3, ZTE Corporation

[3]	R4-2000501, CR to reflect the completed ENDC combinations for 3 bands DL with 3 bands UL into Rel16 TS 38.101-3, ZTE Corporation

[4]	R4-1913631, TR 37.716-33 v0.1.0 Dual Connectivity (EN-DC) with 3 bands DL  and 3 bands UL, ZTE Corporation

[5]	RP-200110, Revised WID for Dual Connectivity (EN-DC) with 3 bands DL and 3 bands , ZTE Corporation

### 19.1.5	NR mobility enhancements

| 800087 | NR mobility enhancements | NR_Mob_enh | R2 | RP-190489 | Intel |
| --- | --- | --- | --- | --- | --- |
| 800187 | Core part: NR mobility enhancements | NR_Mob_enh-Core | R2 | RP-190489 | Intel |
| 800287 | Perf. part: NR mobility enhancements | NR_Mob_enh-Perf | R4 | RP-190489 | Intel |

Summary based on the input provided by ZTE Corporation in RP-200170.

The work item on NR mobility enhancements specifies solutions to reduce interruption time during HO (by Dual Active Protocol Stack (DAPS) handover), and to improve HO/SCG change reliability and robustness (by Conditional Handover (CHO), Conditional PSCell Change (CPC) and T312 based fast failure recovery).

The corresponding changes are captured into TS 38-series specifications and TS37.340/TS36.331 in [2]-[5].

Dual Active Protocol Stack (DAPS) handover

DAPS Handover is a handover procedure that maintains the source gNB connection after reception of RRC message for handover and until releasing the source cell after successful random access to the target gNB.

-	The UE maintains DL reception and UL transmission for user data with source upon receiving DAPS HO command before successful RACH in target (UL switching);

-	Upon receiving the indication on UL switching, for UL the PDCP layer will only forward the user data to target path;

-	The UE will continue the reception of DL from both source and target, and provides UL to both source (PDCP will not forward the user data to low layer) and target node before release of source.

-	Upon HO failure, the UE can use source link for recovery instead of reestablishment if the source link is still valid;

Conditional Handover (CHO)

Conditional Handover (CHO) is a handover procedure that is executed only when the configured execution condition(s) are met.

To improve the robustness, the network can provide the up to 8 candidate cell configuration(s) associated with execution condition (s) to UE. The UE maintains connection with source gNB after receiving CHO configuration, and starts evaluating the CHO execution conditions for the candidate cell(s). If at least one CHO candidate cell satisfies the corresponding CHO execution condition, the UE detaches from the source gNB, applies the stored corresponding configuration for that selected candidate cell, synchronises to that candidate cell and completes the RRC handover procedure by sending RRCReconfigurationComplete message to the target gNB. The UE releases stored CHO configurations after successful completion of RRC handover procedure.

Upon failure, the UE will perform CHO if the selected cell is CHO candidate cell and it is the first time of recovery and if the network allows CHO based recovery.

Conditional PSCell Change (CPC)

Conditional PSCell Change is a PSCell change procedure that is executed only when PSCell execution condition(s) are met.

To improve the robustness, the network can provide the up to 8 candidate cell configuration(s) associated with execution condition (s) to UE. If CPC is configured in the RRCReconfiguration, the UE maintains connection with source PSCell after receiving CPC configuration, and starts evaluating the CPC execution conditions for the candidate PSCell(s). If at least one CPC candidate PSCell satisfies the corresponding CPC execution condition, the UE detaches from the source PSCell, applies the stored corresponding configuration for the selected candidate PSCell and synchronises to that candidate PSCell. The UE completes the CPC execution procedure by an ULInformationTransferMRDC message to the MN which includes an embedded RRCReconfigurationComplete message to the new PSCell if SRB3 is not configured, otherwise the UE sends RRCReconfigurationComplete message to the new PSCell directly.

Upon SCG failure, same as MR-DC, the UE will transmit the SCG Failure Information message to the MN.

In Rel-16, conditional handover based NR PSCell addition/change for any architecture option with NR PSCell is supported, but limit to intra SN change without MN involvement.

T312 based fast failure recovery

T312 based solution (same as LTE) is used for both PCell and PSCell. The motivation of T312 is to speed up RLF recovery procedure by triggering re-establishment procedure sooner using a shorter timer than T310. T312 on PSCell can be configured for SCG measurement configurations provided over SRB3 or SRB1.

References

[1]	RP-192534, Revised WID on NR mobility enhancements, Intel Corporation, RAN#86

[2]	RAN1 CR packets

[3]	RAN2 CR packets

[4]	RAN3 CR packets

[5]	RAN4 CR packets

### 19.1.6	Rel-16 NR inter-band CA/Dual Connectivity for 2 bands DL with x bands UL (x=1, 2)

| 800074 | Rel-16 NR inter-band CA/Dual Connectivity for 2 bands DL with x bands UL (x=1, 2) | NR_CADC_R16_2BDL_xBUL | R4 | RP-191565 | ZTE |
| --- | --- | --- | --- | --- | --- |

Summary based on the input provided by ZTE Corporation in RP-200170.

This WI is to introduce the band combinations for inter-band carrier aggregation and dual connectivity for 2 bands DL with up to 2 bands UL into Rel-16 specifications and specify the corresponding configurations and RF requirements.

The study results for each band combination are captured in TR 38.716-02-00, and the configurations and RF requirements are added into the corresponding technical specifications.

This WI includes all target band combinations for both NR CA and DC including the following configurations:

•	New NR CA configurations including inter band CA for 2 different bands DL with up to 2 different bands UL

o	new configurations from exiting bands

o	new configurations from new band

o	The NR CA configurations will be introduced in a release independent manner based on TS38.307

•	New NR DC configurations including inter band CA for 2 different bands DL with up to 2 different bands UL

o	Accommodated only until the outcome of Rel-15 late drop handling becomes clear.

The preconditions to propose NR CA/DC configurations including Inter band CA for 2 different bands DL with up to 2 different bands UL in rel-16 are as follows.

o	Constituent NR band and NR Intra band CA shall be completed and specified in advance.

o	NR Band n1 and n2 requirements shall be completed and specified in advanced.

Specification for each band combination includes:

o	Applicable frequencies if necessary

o	Applicable bandwidth and bandwidth sets if necessary

o	Analysis on potential self-desensitization due to following reasons:

o	TX Harmonic and/or intermodulation overlap of receive band

o	TX signal overlap of receiver harmonic frequency

o	TX frequency being in close proximity of one of the receive bands

o	Any other identified reasons

o	At least the following for the combination where self-desensitization exists

o	∆TIB, c

o	∆RIB, c

o	Reference sensitivity exceptions including MSD test cases

o	Add conformance testing in RAN5 specifications (to follow at a later stage)

The target band combinations are captured in four tables in this WI respectively:

o	CA configurations for 2 different bands DL with 1 band UL

o	CA configurations for 2 different bands DL with 2 band UL

o	CA configurations for 2 different bands DL with up to 2 band UL within FR2

o	DC configurations for 2 different bands DL with 2 band UL within FR1, or within FR2 or including FR2

References

[1]	R4-2000502 Revised WID on Rel-16 NR Inter-band Carrier Aggregation/Dual Connectivity  for 2 bands DL with x bands UL (x=1,2), ZTE Corporation

[2]	R4-2000497 CR to reflect the completed NR inter band CA DC combinations for 2 bands DL with up to 2 bands UL into Rel16 TS 38.101-1, ZTE Corporation

[3]	R4-2000498 CR to reflect the completed NR inter band CA DC combinations for 2 bands DL with up to 2 bands UL into Rel16 TS 38.101-3, ZTE Corporation

[4]	R4-2000803 TR 38.716-02-00 v0.9.0, ZTE Corporation

[5]	RP-200168, Revised WID for,  ZTE Corporation

### 19.1.7	Rel16 NR inter-band Carrier Aggregation for 3 bands DL with 1 band UL

| 830095 | Rel16 NR inter-band Carrier Aggregation for 3 bands DL with 1 band UL | NR_CA_R16_3BDL_1BUL | R4 | RP-191153 | CATT |
| --- | --- | --- | --- | --- | --- |

Summary based on the input provided by ZTE Corporation in RP-200173.

This WI is to introduce the band combinations for inter-band carrier aggregation and dual connectivity for 3 bands DL with 2 bands UL into Rel-16 specifications and specify the corresponding configurations and RF requirements.

The study results for each band combination are captured in TR 38.716-03-02, and the configurations and RF requirements are added into the corresponding technical specifications.

This WI includes all target band combinations for both NR CA and DC including the following configurations:

•	New NR CA configurations including inter band CA for 3 different bands DL with 2 different bands UL

o	new configurations from exiting bands

o	new configurations from new band

o	The NR CA configurations will be introduced in a release independent manner based on TS38.307

•	New NR DC configurations including inter band CA for 3 different bands DL with 2 different bands UL

The preconditions to propose NR CA/DC configurations including Inter band CA for 3 different bands DL with 2 different bands UL in rel-16 are as follows.

o	Requirements for all concerning bands shall be completed and specified in advanced

o	Constituent NR band and NR intra band CA shall be completed and specified in advance

Specification for each band combination includes:

o	Applicable frequencies if necessary

o	Applicable bandwidth and bandwidth sets if necessary

o	Analysis on potential self-desensitization due to following reasons:

o	TX Harmonic and/or intermodulation overlap of receive band

o	TX signal overlap of receiver harmonic frequency

o	TX frequency being in close proximity of one of the receive bands

o	Any other identified reasons

o	At least the following for the combination where self-desensitization exists

o	∆TIB, c

o	∆RIB, c

o	Reference sensitivity exceptions including MSD test cases

o	Add conformance testing in RAN5 specifications (to follow at a later stage)

The target band combinations are captured in two tables in this WI respectively:

o	CA configurations for 3 different bands DL with 2 band UL

o	DC configurations for 3 different bands DL with 2 band UL

References

[1]	R4-2000503 Revised WID on Rel-16 NR Inter-band Carrier Aggregation/Dual Connectivity for 3 bands DL with 2 bands UL, ZTE Corporation

[2]	R4-2000499 CR to reflect the completed NR inter band CA DC combinations for 3 bands DL with 2 bands UL into Rel16 TS 38.101-1, ZTE Corporation

[3]	R4-2000500 CR to reflect the completed NR inter band CA DC combinations for 3 bands DL with 2 bands UL into Rel16 TS 38.101-3, ZTE Corporation

[4]	R4-2000804 TR 38.716-03-02 v0.4.0, ZTE Corporation

[5]	RP-200171, Revised WID for Rel-16 NR Inter-band Carrier Aggregation/Dual Connectivity for 3 bands DL with 2 bands UL,  ZTE Corporation

### 19.1.8	Add support of NR DL 256QAM for frequency range 2 (FR2)

| 830088 | Add support of NR DL 256QAM for frequency range 2 (FR2) | NR_DL256QAM_FR2 | R4 | RP-200124 | China Telecom |
| --- | --- | --- | --- | --- | --- |
| 830188 | Core part: Add support of NR DL 256QAM for frequency range 2 (FR2) | NR_DL256QAM_FR2-Core | R4 | RP-200124 | China Telecom |
| 830288 | Perf. part: Add support of NR DL 256QAM for frequency range 2 (FR2) | NR_DL256QAM_FR2-Perf | R4 | RP-200124 | China Telecom |

Summary based on the input provided by China Telecom in RP-200727.

In NR_DL256QAM_FR2 WI, the RF requirements include BS modulation quality, UE maximum input level and BS conformance test requirements were specified to make RAN4 specification support FR2 DL 256QAM.  This will enable the commercial deployment and also guarantee the transceiver performance for the feature of FR2 DL 256QAM from both BS and UE sides.

This WI firstly evaluated the feasibility of FR2 DL 256QAM based on link level simulation, system level simulation and implementation study, and concluded that FR2 DL 256QAM is feasible and beneficial in certain of identified applicable scenarios (see TR 38.883 on "Study on FR2 DL 256QAM").

After the feasibility confirmation, the WI introduced corresponding FR2 DL 256QAM specific RF requirements listed as

-	For 38.104: BS modulation quality expressed as Error Vector Magnitude (EVM) requirement which is specified as 3.5%, same required value with FR1.

-	For 38.101-2: UE Maximum input level for both single carrier and intra-band CA which reflects the receiver linearity capability, and the related Fixed Reference Channel which is used to test the Maximum input level.

-	For 38.141-2: Manufacturers declarations for the output power, Test models, Test procedures and Test EVM requirement.

### 19.1.9	SON (Self-Organising Networks) and MDT (Minimization of Drive Tests) support for NR

| 840091 | SON (Self-Organising Networks) and MDT (Minimization of Drive Tests) support for NR | NR_SON_MDT | R3 | RP-192610 | CMCC |
| --- | --- | --- | --- | --- | --- |
| 840191 | Core part: SON (Self-Organising Networks) and MDT (Minimization of Drive Tests) support for NR | NR_SON_MDT-Core | R3 | RP-192610 | CMCC |

Summary based on the input provided by CMCC in RP-200774.

This work item introduces SON and MDT features support in NR, including MRO (Mobility Robustness Optimisation), MLB (Mobility Load Balancing), RACH optimization, MDT (Minimization of Drive Test) and L2 measurements. Specification of these features will help mobile operators reduce the CAPEX/OPEX and improve the use experience.

This work item produces a new specification TS 38.314: L2 measurements for NR.

The key functionalities of this WI are described as below:

Mobility Robustness Optimisation (MRO) aims at detecting and enabling correction of mobility related problems including Connection failure due to intra-system or inter-system mobility, Inter-system Unnecessary HO and Inter-system HO ping-pong. MRO also provides means to distinguish the above problems from NR coverage related problems and other problems, not related to mobility.

-	Connection failure due to intra-system mobility

-	Intra-system Too Late Handover

-	Intra-system Too Early Handover

-	Intra-system Handover to Wrong Cell

-	Connection failure due to inter-system mobility

-	Inter-system/ Too Late Handover

-	Inter-system/ Too Early Handover

-	Inter-system Unnecessary HO

-	Inter-system Ping-pong

Mobility load balancing (MLB) is to distribute load evenly among cells and among areas of cells, or to transfer part of the traffic from congested cell or from congested areas of cells, or to offload users from one cell, cell area, carrier or RAT to achieve network energy saving.

Intra-RAT and intra-system inter-RAT load balancing scenarios are specified and includes the functions of   Load reporting, Load balancing action based on handovers and Adapting handover and/or reselection configuration.

The load reporting function is executed by exchanging load information over the Xn/X2/F1/E1 interfaces via Resource Status Reporting Initiation & Resource Status Reporting procedures.

The load metrics that have been specified consist of

-	Radio resource usage (per-cell and per SSB area PRB usage: DL/UL GBR PRB usage, DL/UL non-GBR PRB usage, DL/UL total PRB usage, and DL/UL scheduling PDCCH CCE usage);

-	TNL capacity indicator (UL/DL TNL offered capacity and available capacity);

-	Cell Capacity Class value (UL/DL relative capacity indicator);

-	Capacity value (per cell, per SSB area and per slice: UL/DL available capacity);

-	HW capacity indicator (offered throughput and available throughput over E1, percentage utilisation over F1);

-	RRC connections (number of RRC connections, and available RRC Connection Capacity);

-	Number of active UEs.

RACH optimization is supported by UE reported information made available at the NG RAN node and by PRACH parameters exchange between NG RAN nodes.

The functionalities supported over the interfaces consist of

-	PRACH configuration per served cell is signalled from DU to CU and over Xn

-	PRACH configuration of Served Cells over X2

-	RACH report from UE over Uu

-	RACH report from CU to DU over F1

NR MDT takes LTE as baseline and takes NR new features into account, including beam, RRC_INACTIVE and EN-DC.

The following functionalities are specified:

-	MDT network signaling support over S1, X2, NG, Xn, F1 and E1

-	Logged MDT for RRC_IDLE and RRC_INACTIVE UEs, support event-trigged or periodic logged MDT

-	Immediate MDT

-	CEF (Connection Establishment Failure) report

-	Resume failure report

-	Immediate MDT for EN-DC

L2 measurements. In this work item, a new spec TS 38.314 was produced to capture the measurements performed by network or the UE that are transferred over the standardised interfaces in order to support NR radio link operations, radio resource management (RRM), network operations and maintenance (OAM), minimization of drive tests (MDT) and self-organising networks (SON).

The specified measurements includes

-	Received Random Access Preambles

-	Packet delay

-	Number of active UEs

-	Number of stored inactive UE contexts

-	UL PDCP Packet Average Delay by the UE

References

[1]	RP-192610, Revised WID on SON/MDT support for NR, CMCC

[2]	RP-200773, Status report for WI on SON_MDT support for NR, CMCC

[3]       CRs and new specs

R2-2006195	draft TS 38.314	CMCC

R2-2006342 	CR to 37.320 to support NR MDT	CMCC, Nokia, Nokia Shanghai Bell	CR	Rel-16	37.320	16.0.0	0082	1	F	NR_SON_MDT-Core

R2-2006336	Corrections on MDT and SON in NR  Huawei, Ericsson, HiSilicon CR  Rel-16    38.331    16.0.0     1669       2     F       NR_SON_MDT-Core

R2-2006337	Corrections on MDT and SON	Huawei, Ericsson, HiSilicon CR Re-16    36.331    16.0.0     4323       2      F       NR_SON_MDT-Core

R2-2006197	UE capabilities for NR MDT and SON	vivo, CMCC	38.306CR

R2-2006198	UE capabilities for NR MDT and SON	vivo, CMCC	36.306CR

R3-204414	BLCR to 38.420: Addition of MDT feature	CMCC	38.420

R3-204415	BLCR to 38.420: Addition of SON feature	CMCC	38.420

R3-204416	BLCR to 38.470: Addition of SON feature	CMCC	38.470

R3-204417	BLCR to 38.460: Addition of SON feature	CMCC	38.460

R3-204421	MDT support for EN-DC	Huawei	36.413

R3-204423	Addition of SON features	CMCC, Huawei	38.401

R3-204425	Addition of SON features	Huawei	36.413

R3-204426	Addition of MDT feature	Huawei	38.413

R3-204427	Addition of MDT features	ZTE	38.463

R3-204428	Addition of MDT features	Samsung	38.473

R3-204478	Addition of SON feature	Huawei	36.300

R3-204479	Addition of SON feature	CATT	36.423

R3-204480	Addition of SON features	CMCC, Huawei	38.300

R3-204481	Addition of SON features	Huawei	38.413

R3-204482	Addition of SON features	Samsung	38.423

R3-204483	Addition of SON features	Huawei	38.473

R3-204484	Addition of MDT features	Samsung	38.401

R3-204485	MDT Configuration support for XnAP	Ericsson	38.423

R3-204496	MDT support for EN-DC	Huawei, SAMSUNG	36.423

R3-204502	Addition of SON features	Ericsson	38.463

### 19.1.10	Introduction of NR FDD bands with variable duplex and corresponding framework

| 841001 | Introduction of NR FDD bands with variable duplex and corresponding framework | NR_FDD_bands_varduplex | R4 | RP-191567 | Huawei |
| --- | --- | --- | --- | --- | --- |
| 841101 | Core part: Introduction of NR FDD bands with variable duplex and corresponding framework | NR_FDD_bands_varduplex-Core | R4 | RP-191567 | Huawei |

Summary based on the input provided by Huawei, HiSilicon in RP-201019.

Supplementary downlink (SDL) frequency bands are defined within NR specifications in accordance with regulated licensed bands such as in the 1400-1500MHz range in Europe, and Uplink Sharing is defined as a means to operate NR uplink on the uplink resources of other bands, e.g. sharing the uplink carrier with that of an LTE carrier. Such a configuration may be particularly useful when there is not a desire to allocate downlink NR resources in a 2nd downlink carrier, e.g. for an EN-DC configuration with the SDL band as part of the secondary cell group (SCG).

This work item allows the uplink and downlink of the introduced bands to be from 2 different licensed spectrum blocks, and new paired bands are defined in the RF specifications as n91, n92, n93 and n94.

System parameters

System parameters for the new variable duplex FDD bands are defined in all the related RAN4 specifications including, operating bands, channel bandwidth, rasters, Tx-Rx separation, etc.

Operating bands and NR-ARFCN

Newly introduced variable duplex bands have the same UL frequency parts as in SUL bands either n81 or n82, while they also have the same DL frequency parts as in SDL bands n75 or n76.

| n91 | 832 MHz – 862 MHz | 1427 MHz – 1432 MHz | FDD9 |
| --- | --- | --- | --- |
| n92 | 832 MHz – 862 MHz | 1432 MHz – 1517 MHz | FDD9 |
| n93 | 880 MHz – 915 MHz | 1427 MHz – 1432 MHz | FDD9 |
| n94 | 880 MHz – 915 MHz | 1432 MHz – 1517 MHz | FDD9 |

A note 9 is specified in the table that says ‘Variable duplex operation does not enable dynamic variable duplex configuration by the network, and is used such that DL and UL frequency ranges are supported independently in any valid frequency range for the band’. This comes from the concern that the operator may want to dynamically configure the duplex distance used in the network.

Channel bandwidth

Since the frequency parts of the variable duplex FDD bands are acquired independently, asymmetric bandwidths must be introduced to accommodate most of the desired deployments.

| NR Band | Channel bandwidths for UL (MHz) | Channel bandwidths for DL (MHz) |
| --- | --- | --- |
| n911 | 10 | 5 |
| n921 | 5 | 10, 15, 20 |
|  | 10 | 15, 20 |
| n931 | 10 | 5 |
| n941 | 5 | 10, 15, 20 |
|  | 10 | 15, 20 |
| NOTE 1: The assignment of the paired UL and DL channels are subject to a TX-RX separation as specified in clause 5.4.4. |  |  |

Hence other bandwidth configurations of the variable duplex FDD bands follow what was defined for the corresponding UL or DL bands that are consisted of.

Tx-Rx separation

It is worth noting that Tx-Rx separation is defined in a different way for the variable duplex bands since the default separation no longer applies with them.

| NR Operating Band | TX – RX carrier centre frequency separation |
| --- | --- |
| n91 | 570 MHz – 595 MHz (NOTE 2) |
| n92 | 575 MHz – 680 MHz (μ = 0) 580 MHz – 675 MHz (μ = 1) (NOTE 2) |
| n93 | 517 MHz – 547 MHz (NOTE 2) |
| n94 | 522 MHz – 632 MHz (μ = 0) 527 MHz – 627 MHz (μ = 1) (NOTE 2) |

Note 2 says that the range of Tx-Rx frequency separation given paired UL and DL channel bandwidths BWUL and BWDL is given by the respective lower and upper limit FDL_low – FUL_high + 0.5(BWDL + BWUL) and FDL_high – FUL_low – 0.5(BWDL + BWUL). The UL and DL channel bandwidth combinations specified in Table 5.3.5-1 and 5.3.6-1 depend on the subcarrier spacing configuration μ.

UE requirements

UE requirements considering the characteristics when operating in the variable duplex bands and band combinations are defined including REFSENS and related configurations.

REFSENS and UL configurations

REFSENS requirements follow the ones defined for n75 and n76 correspondingly for the variable duplex bands but UL configurations are not necessarily the same with those defined for n8 and n20. For n20, additional restrictions on the UL RBs are specified so that the same REFSENS value can be achieved with such a narrow duplex distance. Since the variable duplex bands have much larger expected Tx-Rx separation, additional restrictions on the UL RB allocations are not specified.

BS requirements

In this work item, BS requirements span among a series of specifications such as 38.104, 38.141-1, 37.104, 37.105, 37.141, 37.145-1, 37.145-2, 36.104, 36.141, etc. Mainly coexistence requirements are defined in these specs.

Additional spurious emission requirements

Respective requirements which correspond to BS spurious limits for co-existence with systems operating in the new variable duplex FDD bands are introduced in the mentioned spec, with maximum measured spurious levels and bandwidth configurations.

Out of band blocking requirements

Another set of Tx requirements is also defined with the similar manner as the above one. The requirements apply when co-location with other BS-s operating in the variable duplex FDD bands is observed.

Receiver response under co-location requirements

Receiver response requirements are defined in the mentioned specifications that the BS operating in the introduced bands should have the ability to block unwanted interband interferences from any other collocated BS.

For all the above mentioned BS requirements, both conducted and radiated requirements are defined.

References

[1]	RP-192820, “Revised WID: Introduction of NR FDD bands with variable duplex and corresponding framework”, Huawei, HiSilicon.

[2]	RP-192818, “Status report for WI: Introduction of NR FDD bands with variable duplex and corresponding framework”, Huawei, HiSilicon.

### 19.1.11	Cross Link Interference (CLI) handling and Remote Interference Management (RIM) for NR

| 800082 | Cross Link Interference (CLI) handling and Remote Interference Management (RIM) for NR | NR_CLI_RIM | R1 | RP-191546 | LG Electronics |
| --- | --- | --- | --- | --- | --- |
| 800182 | Core part: Cross Link Interference (CLI) handling and Remote Interference Management (RIM) for NR | NR_CLI_RIM-Core | R1 | RP-190700 | LG Electronics |
| 800282 | Perf. part: Cross Link Interference (CLI) handling and Remote Interference Management (RIM) for NR | NR_CLI_RIM-Perf | R4 | RP-190700 | LG Electronics |

Summary based on the input provided by Huawei in RP-190144.

Rel-14 NR study showed that duplexing flexibility with cross-link interference mitigation shows better user throughput compared to static UL/DL operation or dynamic UL/DL operation without interference mitigation in indoor hotspot (4GHz and 30GHz) and urban macro scenarios (4GHz and 2GHz). Furthermore, semi-static and/or dynamic DL/UL resource assignments should also consider coexistence issues particularly among different operators where tight coordination are challenged. For efficient coexistence, not only coexistence requirements need to be understood but also advanced mechanisms to mitigate interference such as TRP-to-TRP measurement and adaptation based on measurements should be considered. The Rel-16 Work Item Cross Link Interference (CLI) handling to support flexible resource adaptation for unpaired NR cells achieves the following objectives [1]:

•	CLI measurements and reporting at a UE (i.e., CLI-RSSI and SRS-RSRP), and network coordination mechanism(s) (i.e., exchange of intended DL/UL configuration) are developed.

•	Perform coexistence study to identify conditions of coexistence among different operators in adjacent channels is accomplished.

In NR deployment on lower TDD frequency, the impact of the troposphere bending will continue existing if no special mechanisms are introduced. In the RIM SI, the frameworks for mechanisms for gNBs to start and terminate the transmission/detection of reference signal(s), the functionalities and requirements of the corresponding RS(s) as well as the design of the RS(s), and the backhaul-based coordination mechanisms among gNBs have been studied. It is recommended to specify RIM RS(s) to support identifying remote interference related information, it is also recommended to specify the inter-set RIM backhaul signalling via the core network for backhaul-based solution. The Rel-16 Work Item Remote Interference Management (RIM) to deal with mitigation of the remote interference caused by gNBs achieves the following objectives [1]:

•	RIM RS resource and configurations, and the inter-set RIM backhaul signalling via the core network to convey the messages of “RIM-RS detected” and “RIM-RS disappeared are developed.

•	Corresponding OAM functions to support RIM operation is identified.

CLI handling

Once different TDD UL/DL configurations are applied among neighboring cells, UL transmission from a UE in a cell causes interference to DL reception of some other UEs in the rest of the neighboring cells. The interference is referred as inter-cell UE-to-UE cross link interference (CLI). To mitigate an inter-cell UE-to-UE CLI, gNBs can exchange and coordinate the intended TDD UL/DL configuration over Xn and F1 interfaces. Taking into account the exchanged information, a gNB may decide the transmission and reception pattern in order to avoid CLI to neighboring cell or from neighboring cell.

For CLI handling, two types of CLI measurements and reporting (i.e., CLI-Received Signal Strength Indicator (RSSI) measurement and SRS-Reference Signal Received Power (RSRP) measurement) are specified. For CLI-RSSI measurement, the victim UE measures the total received power over configured CLI-RSSI measurement resource. For SRS-RSRP measurement, the victim UE measures the RSRP over configured SRS resource(s) which is/are transmitted from one or multiple aggressor UE(s). Then, Layer 3 filtering can be applied to the measurement result for both CLI-RSSI measurement and SRS-RSRP measurement. For measurement result reporting for both CLI-RSSI measurement and SRS-RSRP measurement, event triggered and periodic reporting are supported. Furthermore, CLI measurement and reporting can be configured for NR cells in multi-carrier option.

Semi-static and/or dynamic DL/UL resource assignment causes interferers between networks on adjacent channels. It has been tasked to investigate the adjacent channel co-existence effects arising when CLI, or more generically dynamic TDD is operated in an aggressor network. The technical report captures a description of the adjacent channel interference effects that arise with dynamic TDD as well as a simulation investigation of adjacent channel interference in a number of different deployment scenarios [2].

RIM

Due to the atmospheric ducting phenomenon, the DL signals of an aggressor cell in unpaired spectrum can interfere with the UL signals of a victim cell in the same unpaired spectrum bandwidth that is far away from the aggressor cell. Such interference is termed as remote interference. A remote interference scenario may involve a number of victim and aggressor cells, where the gNBs may execute Remote Interference Management (RIM) coordination on behalf of their respective cells.

To mitigate remote interference, RIM frameworks for coordination between victim and aggressor gNBs are specified. The coordination communication in RIM frameworks can be wireless- or backhaul-based. In both frameworks, all gNBs in a victim set can simultaneously transmit an identical RIM reference signal carrying the victim set ID over the air.

In the wireless framework, RIM-RS (RIM reference signal) type 1 and RIM-RS type 2 are specified. Upon reception of the RIM reference signal (RIM-RS type 1) from the victim set, aggressor gNBs undertake RIM measurement, and may send back a RIM reference signal (RIM-RS type 2) carrying the aggressor set ID if configured, and may undertake interference mitigation actions. The RIM reference signal (RIM-RS type 2) sent by the aggressor is able to facilitate estimation whether the atmospheric ducting phenomenon between victim and aggressor sets exists.

In the RIM backhaul framework, upon reception of the RIM reference signal (RIM-RS type 1) from the victim set, aggressor gNBs undertake RIM measurement, establish backhaul coordination towards the victim gNB set. The backhaul messages which carry the detection or disappearance indication are aggregated at gNB-CU via F1 interface and sent from individual aggressor gNBs to individual victim gNBs via NG interface, where the signalling is transparent to the core network.

References

[1] RP-193190	“Revised WID on Cross Link Interference (CLI) handling and Remote Interference Management (RIM) for NR”

[2] 3GPP TR38.828	“Cross Link Interference (CLI) handling and Remote Interference Management (RIM) for NR”

## 19.2	Release 16 Features impacting both LTE and NR

### 19.2.1	Transfer of Iuant interface specifications from 25-series to 37-series

| 820071 | Transfer of Iuant interface specifications from 25-series to 37-series | Iuant_transfer | R3 | RP-190160 | Huawei |
| --- | --- | --- | --- | --- | --- |

Summary based on the input provided by Huawei in RP-190144.

This feature transfers the contents of the TSs 25.46x into TSs 37.46x (x=0, 1, 2, 6) for releases 8/9/10/11/12/13/14/15, considering the fact theses specifications apply to different RAT i.e. UTRA, E-UTRA and NR.

Indeed, 25 series of specifications are dedicated to UMTS only, when the material which was previous there referred to multiple RATs, i.e. UTRA, E-UTRA and NR.

Rel-6/7: UMTS only (no changes needed, also no changes possible as these releases are no longer maintained)

Rel-8/9/10/11/12/13/14: UMTS and E-UTRA/LTE

Rel-15: UMTS, E-UTRA/LTE and NR

Note: The TSs 25.4.6x are turned into empty specifications referring to the new created TSs 37.46x.

The TSs 25.46x are not withdrawn so references to these specs will lead to the correct specs.

Additional remarks:

1. This transfer of data over 8 releases is an exceptional case: Instead of creating Rel-8 specs and step-by-step upgrade to higher releases via CRs (which would be the normal approach), it was decided with the specs manager to bring v1.N.0 of TS 37.46x for 1-step approval to RAN #83 and instead of approving them to vN.0.0 it was decided to approve them the same version as the last 25.46x version that is created via the CRs.

2. The text shown in TS 37.46x is taken 1:1 from the corresponding TS 25.46x showing changes of the contents via revision marks (note: spec cover and history adaptations are done without revision marks). The changes are mainly to indicate the considered RATs in the scope, to replace 25.46x references by 37.46x references and to correct obvious problems. Further technical changes of the contents of the specifications may rather be considered in a future cat.F CR (and probably just focusing on the latest releases).

3. There is also a TS 25.463 which exists for Rel-6 and Rel-7 only. TS 25.466 replaced TS 25.463 in Rel-7 and future releases. This means for Rel-6 TS 25.463 is still valid and cannot be withdrawn. Rel-7 is no longer maintained i.e. no CRs are possible i.e. 3GPP internal references to TS 25.463 could not be corrected anymore, so also 25.463 Rel-7 will not be withdrawn but it will be clarified on the 3GPP webpage that for Rel-7 TS 25.466 should be considered instead of TS 25.463.

4. References of Rel-16 and higher should refer to TS 37.46x instead of TS 25.46x. Changes of existing references of Rel-8 to Rel-15 specifications of TS 25.46x may be corrected to references to TS 37.46x but such a change is not mandatory as 25.46x Rel-8 to Rel-15 will remain. A further upgrade of 25.46x Iuant specs to Rel-16 and higher will not be considered. Rel-16 and higher specifications shall refer to TS 37.46x.

Following this transfer, RAN3 agreed to transfer the ownership of the clauses currently owned by RAN4 in TS 37.461 back to RAN3. RAN3 informed RAN4 by LS (R3-191145).

References

[1]	R3-190104	Presentation of Specifications to TSG: TS 37.460 v1.N.0, TS 37.461 v1.N.0, TS 37.462 v1.N.0, TS 37.466 v1.N.0 (N=8/9/a/b/c/d/e/f)	ETSI MCC, Huawei, Ericsson

### 19.2.2	Introduction of GSM, UTRA, E-UTRA and NR capability set(s) (CS(s)) to the multi-standard radio (MSR) specifications

| 830086 | Introduction of GSM, UTRA, E-UTRA and NR capability set(s) (CS(s)) to the multi-standard radio (MSR) specifications | MSR_GSM_UTRA_LTE_NR | R4 | RP-190642 | Ericsson |
| --- | --- | --- | --- | --- | --- |

Summary based on the input provided by Ericsson in RP-191816.

The WI added support for MSR BS including GSM, UTRA, E-UTRA and NR combinations by means of creating two new CS, CS 18 (GSM+E-UTRA+NR) and CS19 (UTRA+E-UTRA+NR).

The WI created two new capability sets, CS 18 (GSM+E-UTRA+NR) and CS19 (UTRA+E-UTRA+NR). A BS with multiple connectors can support all 4 RATs by transmitting CS18 on at least one connector and CS19 on at least one connector. To add these CS, the following was needed:

- Updating of the MSR core specification to include correct OBUE emissions tables for GSM/UTRA when operating together with MSR as well as some fixes to the blocking and RX IM requirements.

- Introduction in the conformance specification of:

  - The new capability sets

  - Test configurations relating to the new capability sets

  - Test applicability rules relating to the new capability sets and test configurations

  - Updated test requirements corresponding to the core requirement updates.

All of the new functionality was introduced in the two approved CRs in [1] and [2].

References

[1] 	R4-1908049	Introduction of requirements for NR + UTRA/GSM combinations, Ericsson, Nokia, Nokia Shanghai Bell, Vodafone, ZTE

[2]	R4-1910476	Introduction of requirements for NR + UTRA/GSM combinations, Ericsson, Nokia, Nokia Shanghai Bell, Vodafone, Huawei

List of related CRs: select "TSG Status = Approved" in: https://portal.3gpp.org/ChangeRequests.aspx?q=1&workitem=830086,830186,830286

### 19.2.3	Direct data forwarding between NG-RAN and E-UTRAN nodes for inter-system mobility

| 820072 | Direct data forwarding between NG-RAN and E-UTRAN nodes for inter-system mobility | Direct_data_fw_NR | R3 | RP-182886 | Ericsson |
| --- | --- | --- | --- | --- | --- |

Summary based on the input provided by Ericsson in RP-192162.

This WI specified direct data forwarding per bearer during inter-system mobility between NG-RAN and E-UTRAN nodes in addition to the solution for indirect data forwarding specified in Rel-15 during the NR WI.

This WI continued discussions on direct data forwarding between NG-RAN and E-UTRAN nodes during the Rel-15 discussions on the NR WI. Direct data forwarding encompasses the following two main features:

-	While data forwarding between NG-RAN and E-UTRAN nodes in Rel-15 foresees establishing the data forwarding path via the EPC and the 5GC, direct data forwarding allows establishing the data forwarding path directly between the involved RAN nodes, without the need to interact with user plane functions of the EPC and the 5GC.

-	Indirect data forwarding for inter-system handover in Rel-15 foresees transporting to be forwarded user plane data between the NG-RAN node and the UPF via PDU Session tunnels. As forwarding paths on the E-UTRAN side are established on a per-E-RAB basis, the Rel-15 solution requires interworking functions at the UPF and the NG-RAN node in order to map per-DRB / per-E-RAB packets to the per-PDU Session tunnel and vice versa. Such interworking is not necessary in case of direct data forwarding.

In the course of the WI RAN3 realised that direct data forwarding tunnel related information may be communicated between the NG-RAN node and the 5GC in a way that the inter-system handover signalling performance benefits from such approach, which was realised in the agreed CRs in [3] and [4].

Further, SA2 and CT4 where contacted to perform necessary work in their areas of responsibility [5].

References

[1]    RP-182886	"New WID: Direct data forwarding for inter-system mobility", approved at RAN#82

[2]    RP-191369	"Status report of WI: Core part: Direct data forwarding between NG-RAN and E-UTRAN nodes for inter-system mobility"

[3]    R3-192626	"Support of Direct Data forwarding for handover between 4G and 5G", agreed CR0041r4 38.413

[4]    R3-193272	"Introduction of direct and indirect data forwarding for inter-system HO between EPS and 5GS", endorsed CR 38.300

[5]    R3-193284	"LS on Finalization of Direct Data Forwarding feature", agreed LS out to SA2 and CT4

### 19.2.4	eNB(s) Architecture Evolution for E-UTRAN and NG-RAN

| Unique_ID | Name | Acronym | WG | WID | WI Rapporteur |
| --- | --- | --- | --- | --- | --- |
| 790054 | eNB(s) Architecture Evolution for E-UTRAN and NG-RAN | LTE_NR_arch_evo | R3 | RP-180531 | China Unicom |

Summary based on the input provided by China Unicom in RP-192964.

In Rel-15, the F1 interface was standardized in the gNB (LTE) to support gNB-CU and gNB-DU split, i.e. to split the Control Unit from the Data Unit (supporting the user data).

Similarly, in Rel-16, the W1 interface is standardized in the ng-eNB (5G) to support ng-eNB-CU and ng-eNB-DU split.

These two interfaces adopt the same higher layer split architecture for between PDCP and RLC, with similar functionalities, such as control plan functionalities.

The ng-eNB-CU and gNB-CU could be physically collocated, deployed in a same physical "box". This unification of the , as to unify the ng-eNB and gNB architectures would simplify the network deployment complexity and facilitate hardware upgrades.

The W1 interface specifications includes the general aspects and principles (TS 37.470), W1 layer 1 (TS 37.471), signalling transport (TS 37.472) and W1AP aspects (TS 37.473). The general aspects and principles and W1 layer1 are aligned with F1 interface. For signalling transport, the W1-C signalling bearer aligned with F1 interface, function and protocol stack in figure 1 are defined.

Figure 1: W1-C signalling bearer protocol stack

The functionalities of W1 interface are supported over W1-C and W1-U (in a similar way as for the F1 interface).

The W1-C functionalities are the following:

- Interface management: W1 setup, eNB-CU/eNB-DU configuration update, error indication function, reset function coordination function.

- system information management: system information is critical for high layer split architecture, eNB-DU is responsible for transmitting MIB, SIB1, SIB2, SIB3, SIB8 and SIB 16. Other system information is responsible by ng-eNB-DU.

- Paging function: The eNB-DU is responsible for transmitting the paging information. While, the eNB-CU provides paging information to enable the eNB-DU to calculate the exact PO and PF.

- UE context management function: The W1 UE context management function supports the establishment and modification of the necessary overall UE context.

- RRC message transfer function:  RRC message transfer function supports the establishment and modification of the necessary overall UE context. This function allows to transfer RRC messages between eNB-CU and eNB-DU.

- Warning message information transfer function: This function allows to cooperate with the warning message transmission procedures over NG interface.

The W1-U functionalities are specified as below:

- Transfer of user data: The functionalities are used to transfer of user data between ng-eNB-CU and ng-eNB-DU.

- flow control function: It is designed for control the downlink user data flow to the ng-eNB-DU.

More detail procedures and IEs for W1 can be founded in TS 37.473 on W1AP specification.

References

[1]	 The WID

[2]	RP-191972  Status report for WI Core part: eNB(s) Architecture Evolution for E-UTRAN and NG-RAN WI (China Unicom)

[3]	R3-187270	pCR to 37.470 on W1AP functions and procedures (Huawei, China Unicom, TIM)

[4]	R3-191163	pCR to 37.470 on W1AP functions and procedures(Huawei, TIM, China Unicom)

[5]	R3-193174	pCR to 37.470 on W1AP functions and procedures (Huawei, China Unicom, Orange, TIM)

[6]	R3-194676	pCR to 37.470 on W1 general aspects and principles (China Unicom)

[7]	R3-194678	pCR to 37.472 on W1 signalling transport (China Unicom)

[8]	R3-194425	pCR to 37.470 on W1AP functions (Huawei, China Unicom, TIM)

[9]	R3-194426	pCR for 37.473 on UE context management procedures (Huawei, China Unicom, TIM)

[10]	R3-194427	pCR for 37.473 on Interface Management procedures (Huawei, China Unicom, TIM)

[11]	R3-194565	Structure pCR for TS 37.473 (Huawei, China Unicom, TIM)

[12]	R3-194562	pCR to 37.470 on W1AP functions (Huawei, China Unicom, TIM, Orange)

[13]	R3-194563	pCR for 37.473 on UE context management procedures (Huawei, China Unicom, TIM, Orange)

[14]	R3-194564	pCR for 37.473 on Interface Management procedures (Huawei, China Unicom, TIM, Orange)

[15]	R3-197544	pCR to 37.473 on ASN.1 completition (Huawei, China Unicom, TIM)

[16]	R3-197636	pCR to 37.473 on miscellaneous correction to contexts (Huawei, China Unicom, Orange, TIM)

[17]	R3-197544	CR to 38.401 on corrections to ng-eNB deployment (Huawei, China Unicom, Orange, TIM)

### 19.2.5	High power UE (power class 2) for EN-DC (1 LTE TDD band + 1 NR TDD band)

| 820079 | High power UE (power class 2) for EN-DC (1 LTE TDD band + 1 NR TDD band) | ENDC_UE_PC2_TDD_TDD | R4 | RP-190315 | CMCC |
| --- | --- | --- | --- | --- | --- |

Summary based on the input provided by CMCC in RP-200214.

This WI contains a general part and band specific combination part for high power UE (power class 2) for EN-DC (1 LTE TDD band + 1 NR TDD band). The purpose is to introduce high power UE (power class2) for EN-DC (1 LTE TDD band +1 NR TDD band). The actual requirements are added to the corresponding technical specifications.

The purpose of this WI on High power UE (power class 2) for EN-DC (1 LTE TDD band + 1 NR TDD band) is to improve the UP coverage of 5G UE. In order to reduce the big imbalance between 5G NR uplink and downlink coverage, we can improve uplink coverage by defining high power UE for SA and NSA, and it is a feasible way to improve the uplink coverage. This PC2 EN-DC (TDD+TDD) feature will meet the operator's deployment request for uplink coverage enhancement.

In this work item, we focus on the PC2 EN-DC RF requirements including UE maximum output power, Tx power tolerance, MPR, A-MPR, configured output power, ACLR, SAR and n41-B40 UE-UE co-existence for Power Class 2 EN-DC.

There are two important issues that have been addressed in this topic, and in the end we have resolved them through appropriate solutions

For SAR issue, RAN4 specified UE Tx duty cycle requirements sufficient to prevent exceeding local regulatory limits such as SAR for inter-band PC2 EN-DC UE.

For PC2 EN-DC (TDD+TDD) release independent issue, RAN4 reached a conclusion that PC2 inter-band EN-DC (LTE TDD PC3 + NR TDD PC3) can be supported from Rel-15 in release independence manner.

This WI was completed in the RAN#86 meeting and the complete version of NR Technical Report 37.825 has been approved.

References

[1]R4-1913208	    TR 37.825 v0.4.0 Finalization, CMCC, RAN4#93

[2]R4-1915992     CR for adding solutions for addressing SAR issue for EN-DC PC2 UE, CATT，CMCC, RAN4#93

[3]R4-1913209    CR for REL-16 TS 38.307 for PC2 EN-DC TDD+TDD, CMCC, RAN4#93

[4]R4-1913210    CR for REL-15 TS 38.307 for PC2 EN-DC TDD+TDD, CMCC, RAN4#93

[5]RP-192583      Presentation of TR 37.825 V1.0.0，CMCC，RAN#86

## 19.3	LTE-related Release 16 Features

### 19.3.1	LTE-based 5G terrestrial broadcast

| 830076 | LTE-based 5G terrestrial broadcast | LTE_terr_bcast | R1 | RP-190732 | Qualcomm |
| --- | --- | --- | --- | --- | --- |
| 800091 | Study on LTE-based 5G terrestrial broadcast | FS_LTE_terr_bcast | R1 | RP-181342 | Qualcomm |
| 830176 | Core part: LTE-based 5G terrestrial broadcast | LTE_terr_bcast-Core | R1 | RP-190732 | Qualcomm |
| 830276 | Perf. part: LTE-based 5G terrestrial broadcast | LTE_terr_bcast-Perf | R4 | RP-190732 | Qualcomm |

Summary based on the input provided by Qualcomm Incorporated in RP-200850.

The work item on “LTE-based 5G terrestrial broadcast” [1] specified enhancements on top of Rel-14 MBMS [2] to meet the 5G requirements for broadcast systems [3]. These enhancements, obtained as a result of the study in [4], are summarized as follows:

-	Introduction of a new numerology for PMCH with 100us cyclic prefix and 2.5kHz subcarrier spacing for support of high mobility scenarios (up to 250km/h).

-	Introduction of a new numerology for PMCH with 300us cyclic prefix and approximately 0.37kHz subcarrier spacing for support of rooftop reception.

-	Enhancements to PBCH and PDCCH to increase robustness in low SINR scenarios.

This work item also produced a TR [5] summarizing the overall aspects of LTE-based 5G terrestrial broadcast.

Numerology for high speed reception

A new numerology for PMCH with 100us cyclic prefix, 400us OFDM core symbol duration, and 2.5kHz subcarrier spacing is introduced to support high mobility scenarios (up to 250km/h). With this numerology, each OFDM symbol uses one slot (0.5ms), and a transport block is mapped to two OFDM symbols (1ms).

Associated to this new numerology, a single reference signal pattern (MBSFN-RS) is introduced with a time-stagger length of 2, and a frequency separation (after de-staggering) of 4 subcarriers, which gives a maximum theoretical equalization interval of 200us.

Numerology for rooftop reception

A new numerology for PMCH with 300us cyclic prefix, 2700us OFDM core symbol duration, and approximately 0.37kHz subcarrier spacing is introduced to support rooftop reception, especially tailored for deployments with large inter-site distance (e.g. HPHT-1 as defined in [4], with an inter-site distance of 125km).

With this new numerology, a new variation of frame structure type 1 is introduced. This frame structure consists of, every 40ms:

-	One subframe (1ms) with 15kHz SCS, containing synchronization and system information, also named “cell acquisition subframe” (CAS)

-	13 slots (3ms each) carrying the PMCH with 0.37kHz SCS. Note that the time-transmission interval (TTI) for a transport block is extended to 3ms.

This frame structure is depicted in Figure 1

Four radio frames = 40ms

One 3ms slot, one symbol per slot

#0

One 1 ms subframe

#12

…………

#2

#1

#1

CAS

Figure 1: Frame structure type 1 for transmissions using f = 0.370 kHz.

Two reference signal patterns are introduced associated with this new numerology, with a stagger length of 2 and 4 OFDM symbols. Both of them have a frequency separation (after de-staggering) of 3 subcarriers, which gives a maximum theoretical equalization interval of 900us.

Enhancements to cell acquisition subframes

As shown in [4], in some deployment scenarios the CAS will experience significantly lower SINR than PMCH due to the former being based on 15kHz subcarrier spacing. Several enhancements are introduced to enhance the performance of the CAS, and are detailed as follows:

-	Semistatic CFI: A semistatic value for CFI (which determines the number of symbols to be used for PDCCH in a given subframe) is indicated in MIB. This allows the UE to skip PCFICH decoding, which can become the bottleneck in low SINR conditions.

-	Aggregation level 16: Before Rel-16, the maximum aggregation level for PDCCH was 8. Rel-16 introduces aggregation level 16 (only for MBMS dedicated cells) to enhance performance in low SINR conditions.

-	PBCH repetition: Similar to the scheme defined for eMTC (but with additional symbol-level rotation for interference randomization), PBCH is repeated in CAS.

References

[1]	RP-193050: “Revised WID on LTE-based 5G terrestrial broadcast”, RAN#86

[2] 	RP-160675, “New WID: eMBMS enhancements for LTE”

[3] 	3GPP TR 38.913: "Study on scenarios and requirements for next generation access technologies"

[4]	3GPP TR 36.776: “Evolved Universal Terrestrial Radio Access (E-UTRA); Study on LTE-based 5G terrestrial broadcast”

[5]	3GPP TR 36.976: “Overall description of LTE-based 5G broadcast”

### 19.3.2	Support for NavIC Navigation Satellite System for LTE

| 850072 | Support for NavIC Navigation Satellite System for LTE | LCS_NAVIC | R2 | RP-192408 | Reliance Jio |
| --- | --- | --- | --- | --- | --- |
| 850172 | Core part: Support for NavIC Navigation Satellite System for LTE | LCS_NAVIC-Core | R2 | RP-192408 | Reliance Jio |
| 850272 | Perf. part: Support for NavIC Navigation Satellite System for LTE | LCS_NAVIC-Perf | R4 | RP-192408 | Reliance Jio |

Summary based on the input provided by Reliance Jio in RP-200604.

The LCS_NAVIC feature introduces NavIC satellite system specific assistance data support used by the location server to enable UE-based and UE-assisted A-GNSS positioning methods in E-UTRAN. This is applicable to both Control plane and User plane positioning. Before this work item EUTRAN A-GNSS methods supported assistance data for only for GPS, Galileo, GLONASS, BDS, SBAS & QZSS constellations.

Introduction of NavIC satellite system assistance data speeds up positioning performance, improves receiver sensitivity and helps to conserve battery power.in UEs supporting NavIC RNSS for positioning.

The LCS_NAVIC feature is applicable only to LTE UEs using L5 band GNSS reception for positioning and has been standardized starting from Rel-16.

3GPP TSG RAN has introduced A-GNSS support for BDS, Galileo, GLONASS, GPS, SBAS & QZSS to cellular positioning systems. The LCS_NAVIC feature introduces NavIC constellation support under the existing framework of A-GNSS.

The key updates introduced under this feature are as follows:

1)	The support for NavIC satellite system has been added to the A-GNSS specifications by assignment of new ‘GNSS ID’, ‘GNSS Signal ID’, and ‘GNSS Time ID’, together with ASN.1 extensions to existing assistance data elements.

2)	NavIC specific New Clock Model, Orbit Model, Almanac Model, UTC model update, Differential corrections and Ionospheric Model have been added to existing assistance data elements.

3)	The GNSS-GenericAssistData used by the location server to aid data for a specific GNSS has been updated by introducing NavIC satellite system specific Differential corrections & Grid model.

4)	Positioning assistance data broadcast has been extended to include NavIC satellite system specific generic assistance data by introduction of two new Positioning System Information Block Type 2 as listed below:

|  | posSibType | assistanceDataElement |
| --- | --- | --- |
| GNSS Generic Assistance Data | posSibType2-24 | NavIC-DifferentialCorrections |
|  | posSibType2-25 | NavIC-GridModelParameter |

References

[1]	3GPP TS 37.355: LTE Positioning Protocol (LPP); Protocol specification

[2]	RP-200439: SR LCS_NAVIC, Reliance Jio

### 19.3.3	Even further mobility enhancement in E-UTRAN

| 800089 | Even further mobility enhancement in E-UTRAN | LTE_feMob | R2 | RP-200148 | China Telecom |
| --- | --- | --- | --- | --- | --- |
| 800189 | Core part: Even further mobility enhancement in E-UTRAN | LTE_feMob-Core | R2 | RP-200148 | China Telecom |
| 800289 | Perf. part: Even further mobility enhancement in E-UTRAN | LTE_feMob-Perf | R4 | RP-200148 | China Telecom |

Summary based on the input provided by China Telecom in RP-200738.

The work item on Even further mobility enhancement in E-UTRAN [1] studies solutions to meet both the reliability and very low HO interruption time requirements in the Study Phase, and specifies the chosen solutions, i.e., Dual Active Protocol Stack (DAPS) handover to reduce interruption time during HO and Conditional Handover (CHO) to improve HO reliability and robustness in the Work Phase.

The corresponding changes are captured into TS 36-series specifications.

Dual Active Protocol Stack (DAPS) handover

DAPS Handover is a handover procedure that maintains the source eNB connection after reception of RRC message for handover and until releasing the source cell after successful random access to the target eNB.

-	If DAPS handover is configured, the UE continues the downlink user data reception from the source eNB until releasing the source cell and continues the uplink user data transmission to the source eNB until successful random access procedure to the target eNB.

-	Upon reception of the handover command, the UE:

-	Creates a MAC entity for target cell;

-	Establishes the RLC entity and an associated DTCH logical channel for target cell for each DRB configured with DAPS;

-	For the DRB(s) configured with DAPS, reconfigures the PDCP entity to configure DAPS with separate security and ROHC functions for source and target and associates them with the RLC entities configured for source and target respectively;

-	Retains rest of the source link configurations until release of the source.

-	When DAPS handover fails, the UE falls back to source cell configuration, resumes the connection with source cell, and reports the DAPS handover failure via the source without triggering RRC connection re-establishment if the source link is still available; Otherwise, RRC re-establishment is performed;

Conditional Handover (CHO)

A Conditional Handover (CHO) is defined as a handover that is executed by the UE when one or more handover execution conditions are met.

-	UE maintains connection with source eNB after receiving CHO configuration, and starts evaluating the CHO execution condition(s) for the CHO candidate cell(s) and executes the HO command once the execution condition(s) are met for a CHO candidate cell.

-	To improve the robustness, the network can provide the up to 8 candidate cell configuration(s) associated with execution condition (s) to UE. If at least one CHO candidate cell satisfies the corresponding CHO execution condition, the UE detaches from the source eNB, applies the stored corresponding configuration for that candidate cell and synchronises to that candidate cell. UE stops evaluating the execution condition(s) for other candidate cells once the handover is triggered.

-	The UE accesses to the target eNB and completes the handover procedure by sending RRCConnectionReconfigurationComplete message to target eNB. The UE releases stored CHO configurations after successful completion of RRC handover procedure.

-	When initial CHO execution attempt fails or HO fails, if network configured the UE to try CHO after HO/CHO failure and the UE performs cell selection to a CHO candidate cell, the UE attempts CHO execution to that cell; Otherwise, RRC re-establishment is performed.

### 19.3.4	DL MIMO efficiency enhancements for LTE

| 800086 | DL MIMO efficiency enhancements for LTE | LTE_DL_MIMO_EE | R1 | RP-182901 | Huawei |
| --- | --- | --- | --- | --- | --- |
| 800186 | Core part: DL MIMO efficiency enhancements for LTE | LTE_DL_MIMO_EE-Core | R1 | RP-182901 | Huawei |
| 800286 | Perf. part: DL MIMO efficiency enhancements for LTE | LTE_DL_MIMO_EE-Perf | R4 | RP-182901 | Huawei |

Summary based on the input provided by Huawei, HiSilicon in RP-200885.

MIMO is an effective technique to improve spectral efficiency and increase overall network capacity. SRS can be utilized to improve DL MIMO performance, especially for massive MIMO in TDD. In this WI [1], SRS capacity and coverage are enhanced by introducing more than one SRS symbol in a UL normal subframe and introducing virtual cell ID for SRS [2].

More than one symbol for SRS in a UL normal subframe

With the introduction of more than one SRS symbol in a UL normal subframe, the SRS capacity and coverage can be increased. These additional SRS symbol(s) are referred to as trigger type 2 SRS.

1 to 13 symbols of the first 13 symbols of a UL normal subframe can be configured to a UE for aperiodically triggered SRS transmission. Intra-subframe repetition, frequency hopping and antenna switching of the additional SRS symbols can be supported. A guard period of one SC-FDMA symbol can be configured for frequency hopping and antenna switching.

The number of repetitions can be configured from the set {1, 2, 3, 4, 6, 7, 8, 9, 12, 13}. If a UE has more receive antennas than transmit chains (e.g. 1T2R), the UE can be configured to transmit the additional SRS with antenna switching. And a UE can additionally be configured with frequency hopping for the additional SRS. The number of antennas (pairs) to switch is:

-	2 for 1T2R, or the number of pairs is configured as 2 for 2T4R

-	3 if the number of pairs is configured as 3 for 2T4R

-	4 for 1T4R

Both legacy SRS and additional SRS can be configured to the same UE, and transmission of legacy SRS and additional SRS symbol(s) in the same subframe for the UE is supported.

For sequence generation, per-symbol group hopping and sequence hopping are supported.

Independent open loop and close loop power control is supported for additional SRS, and DCI formats 3/3A/3B are used for close loop power control.

UEs not configured with SPUCCH/SPUSCH are not expected to be triggered with additional SRS in the same subframe as PUSCH/PUCCH in the same serving cell. UEs configured with SPUCCH/SPUSCH drop the SRS transmission in the symbols colliding with SPUCCH/SPUSCH in the same serving cell.

The additional SRS transmission in the symbols colliding with PUSCH/PUCCH of another serving cell in the same TAG, the same band and with the same CP, is dropped.

The additional SRS transmission in the symbols colliding with PUSCH/PUCCH of another serving cell in the different TAG is dropped if the total transmission power exceeds the PCMAX in any overlapping portion.

Virtual cell ID

Virtual cell ID within the range from 0 to 503 can be configured for SRS to increase SRS capacity.

The virtual cell ID can be configured to only additional SRS symbol(s) or both legacy and additional SRS symbol(s). If virtual cell ID is not configured, the physical cell ID is used.

References

[1]	RP-192766, “WID Revision: DL MIMO efficiency enhancements for LTE”, Huawei, HiSilicon, RAN#86, Sitges, Spain, December, 2019.

[2]	R1-1913596, RAN1 agreements for DL MIMO efficiency enhancements for LTE, WI rapporteur (Huawei), RAN1#99, Reno, USA, November, 2019.

### 19.3.5	Other LTE-only items

Further performance enhancement for LTE in high speed scenario: Covered in the section on Railways.

# 20	All other Release 16 Features

## 20.1	Service Interactivity

| 770020 | Service Interactivity | SerInter | S4 | SP-170796 | Lo, Charles, Qualcomm |
| --- | --- | --- | --- | --- | --- |

Summary based on the input provided by Qualcomm Incorporated in SP-191247.

This summarizes the progress of the normative specifications accomplished during the course of the SerInter work item [1]. The related agreed CRs can be found in the Tdocs [2], [3], [4], [5] and [6].

The service interactivity feature enables dynamic user engagement and resulting auxiliary content presentation during the consumption of a streaming service or content item, received over unicast or broadcast.  Interactive service capabilities can be further personalized to the end user consuming the service. Examples of service interactivity functionality include voting, rating, purchasing, online chats, and reception of targeted advertisements and other content, in real-time during the viewing of a streaming program.

Technical functionality to enable and support dynamic and personalized service interactivity are added to 3GPP TS 26.247 [7], TS 26.346 [8] and TS 26.347 [9] and comprise the following components:

- Signaling of upcoming interactivity events to native or Web app based interactivity applications. Such interactivity event signaling, which could be sent at specific and potentially arbitrary times during the consumption of an associated 3GPP User Service, contain metadata of upcoming interactivity events and which enable an interactivity application to perform its intended task. The interactivity event signaling is defined by the DASH Industry Forum upon request from 3GPP, and is implemented as either  a DASH Event stream, or as samples of a ISOBMFF timed metadata track.

- Processing model and rules for the 3GP-DASH Client to extract metadata contained in interactivity event signaling   .

- WebIDL APIs exposed by the 3GP-DASH Client to an interactivity application, for the subscription by and delivery to, the application, of interactivity event metadata.

(The above functionalities are defined by the DASH Industry Forum, based on request from 3GPP to support the Service Interactivity work item, in the specification “DASH Player’s Application Events and Timed Metadata Processing Model and APIs” [10].)

- For DASH-formatted streaming services, signaling via the DASH MPD of the intended measurement and reporting of interactivity-related usage by the user/device. This signaling enables the service provider to: a) specify the parameters and criteria regarding interactivity consumption reporting; b) specify the type of interactivity usage report to be submitted by the DASH client, and c) employ either random or selective control of the user/device population to perform the reporting. In addition, the XML-based interactivity usage report format and HTTP POST based reporting protocol are defined. The above functionality are specified in TS 26.247 [7].

- Clarifications to the MBMS APIs specification TS 26.347 [9] that an MBMS-Aware Application providing an auxiliary service interactivity feature is expected to acquire, from the MBMS client, a multiplicity of Application User Services. These Application User Services provide the media content of the DASH-over-MBMS application service as well as media content and/or metadata of adjunct application service(s) that provide the service interactivity functionality.

An architecture illustrating the high level interactions between the relevant entities, in the delivery of DASH-formatted content and interactivity event signaling from the network to the 3GP-DASH Client, forwarding of event signaling data from the 3GP-DASH Client to the interactivity application, and subsequent execution by the application of its interactivity task, is shown in the diagram below, which is copied from TR 26.953 [11].

References

[1] 	Tdoc SP-170796, “New WID on Service Interactivity” (SerInter).

[2]	Tdoc S4-180854, CR 26.247-0145, “Signaling and Reporting of Interactivity Usage in 3GP-DASH”.

[3] 	Tdoc S4-190182, CR 26.346-0625, “TS 26.346 Changes Pertaining to Service Interactivity Feature”.

[4] 	Tdoc S4-190196, CR 26.347-0006, “Changes Pertaining to Service Interactivity”.

[5] 	Tdoc S4-191314, CR 26.247-0163, “Support for Service Interactivity via Event Signaling and DASH APIs”.

[6]	Tdoc S4-191318, CR 26.346-0629, “Service Interactivity Support in DASH-over-MBMS Service”.

[7] 	3GPP TS 26.247 “Transparent end-to-end Packet-switched Streaming Service (PSS); Progressive Download and Dynamic Adaptive Streaming over HTTP (3GP-DASH)”.

[8] 	3GPP TS 26.346 “Multimedia Broadcast/Multicast Service (MBMS); Protocols and codecs”.

[9]	3GPP TS 26.347 “Multimedia Broadcast/Multicast Service (MBMS); Application Programming Interface and URL”.

[10]	DASH Industry Forum specification “DASH Player’s Application Events and Timed Metadata Processing Model and APIs”.

[11]	3GPP TR 26.953 “(MBMS and PSS) Interactivity Support for 3GPP-Based Streaming and Download Services”.

## 20.2	RTCP Verification for Real-Time Services

| 850003 | RTCP Verification for Real-Time Services | RTCPVer | S4 | SP-190639 | Burman, Bo, Ericsson LM |
| --- | --- | --- | --- | --- | --- |

Summary based on the input provided by Ericsson LM in SP-200034.

This summarizes the progress of the new, normative specification accomplished during the course of the RTCPVer work item [1].

The RTCP verification for real-time services feature enables verification of the most important parts of the RTP/RTCP and SRTP/SRTCP protocols used by current 3GPP conversational and real-time services.

Technical functionality to enable and support this verification are added to a new 3GPP TS 26.139 [2] and comprise the following aspects:

-	Explicitly allowing for and describing examples of different, possible test architectures.

-	Test cases needed to ensure an adequate level of RTP operation and RTP stream monitoring.

-	Test methods capable to verify that information contained in the RTP header and in RTCP is correct and consistent with the observed characteristics of the related RTP streams:

o	Between RTP/RTCP within the scope of a single RTP stream (e.g. between an RTP stream and the corresponding RTCP reporting from the remote party, or between an RTP stream and the corresponding RTCP metadata, e.g. for sampling clock accuracy compensation between RTP sender and RTP receiver).

o	Between RTP/RTCP across RTP streams in the same RTP session (e.g. between sent and received RTP streams, or between audio RTP streams and video RTP streams).

-	Requirements on what constitutes acceptable RTP/RTCP protocol field values, including RTP payload header and RTP payload length, based on the observed characteristics of the related RTP streams.

-	A method for an RTP/RTCP implementation to announce that it has passed the necessary tests and conforms to the new specification at call setup and during the call.

References

[1] 	Tdoc SP-190639, "New WID on ‘RTP/RTCP Verification for Real-Time Services’" (RTCPVer).

[2] 	3GPP TS 26.139 "Real-time Transport Protocol (RTP) / RTP Control Protocol (RTCP) verification procedures".

## 20.3	Stage-3 SAE Protocol Development for Rel16

| 820042 | Stage-3 SAE Protocol Development for Rel16 | SAES16 | C1 | CP-183088 | Aghili, Behrouz, InterDigital Communications |
| --- | --- | --- | --- | --- | --- |
| 820038 | IMS Stage-3 IETF Protocol Alignment | IMSProtoc16 | C1 | CP-183084 | Leis, Peter, Nokia |

The description of these WIDs has not changed at all from its predecessors, e.g. SAES6, SAES5, etc.

The contents of the WID are exact copies of the ones before as it is an “ongoing” WID at CT1, meaning that it keeps existing for every new release until CT1 makes a decision that it is no longer needed. The only thing that happened this time, i.e. for the Rel-16 version, was that we decided to align the “x” in SAESx with the corresponding release so, as of Rel-16, the WID is called SAES16 and it will be SAES17 for Rel-17.

Same with IMSProtocolx.

## 20.4	Reliable Data Service Serialization Indication

| 840017 | Reliable Data Service Serialization Indication | RDSSI | S2 | SP-190446 | Michael Starsinic, Convida Wireless LLC |
| --- | --- | --- | --- | --- | --- |

Summary based on the input provided by Convida Wireless LLC in SP-200057.

This work item enhances the Reliable Data Service by adding support to the Reliable Data Service for indicating the serialization format of the data that will be sent in a NIDD session

The Reliable Data Service was added to EPS in Rel-14 as part of the “Extended architecture support for Cellular Internet of Things” work item and added to 5GS in Rel-16 as part of the “Cellular IoT support and evolution for the 5G System” work item.

The work was motivated by an LS from oneM2M and resulted in TS 23.682, TS 23.501, and TS 23.502 CRs which added support to the Reliable Data Service for indicating the serialization format of the data that will be sent in a NIDD session. The CRs also updated the NIDD Configuration procedure so that the SCS/AS can indicate to the SCEF or NEF the serialization format(s) that are supported by the SCS/AS.

References

[1] S2-1903031, NIDD RDS API enhancement, LS from oneM2M Technical Plenary to CT3

[2] S2-1903039, LS Reply on NIDD RDS API enhancement, LS from CT3 to SA2 and oneM2M Technical Plenary

[3] S2-1906025, Adding Support for Indicating Serialization Format in RDS, TS 23.682 CR, Convida Wireless LLC, Intel, Deutsche Telekom, AT&T

[4] S2-1906027, Adding Support for Indicating Serialization Format in RDS, TS 23.502 CR, Convida Wireless LLC, Intel, Deutsche Telekom, AT&T

[5] S2-1906162, Adding Support for Indicating Serialization Format in RDS, TS 23.501 CR, Convida Wireless LLC, Intel, Deutsche Telekom, AT&T

## 20.5	Shared Data Handling on Nudm and Nudr

| 800046 | Shared Data Handling on Nudm and Nudr | Shared_Data | C4 | CP-181136 | Wiehe, Ulrich, Nokia |
| --- | --- | --- | --- | --- | --- |

Summary based on the input provided by Nokia, Nokia Shanghai Bell in CP-200143.

The problem of potential signalling floods on Nudm and Nudr service based interfaces that are caused by (identical) subscription data changes for a huge set of subscriptions (e.g. MTC devices) is solved by the concept of shared data.

Subscription data that are retrieved by serving nodes (AMF, SMF, SMSF) from the UDM (and by the UDM from the UDR) may contain a subset that is shared by a huge number of subscriptions (e.g. 1000 specific MTC devices). A simple example for such a subset is e.g. a subscribed periodic registration time with a value of 300 seconds. When the operator decides to modify the value for all these MTC devices to e.g. 400 seconds, a single notification per serving node is sent to update the shared subset; there is no need to send 1000 notifications addressing each single subscription.

The solution is a pure stage 3 signalling optimization issue and is not based on specific stage1 or stage 2 requirements.

CT4#84 decided to implement the solution already in Rel-15.

References

Impacted TSs:

3GPP TS 29.503: "Unified Data Management Services".

3GPP TS 29.504: "5G System; Unified Data Repository Services; Stage 3".

3GPP TS 29.505: "5G System; Usage of the Unified Data Repository Services for Subscription Data; Stage 3".

Main CRs:

29.503 CR 0008r4 C4-186497

29.504 CR 0010r1 C4-187507

29.505 CR 0019r1 C4-187506

## 20.6	New Services and Markets Technology Enablers – Phase 2

| 790001 | New Services and Markets Technology Enablers – Phase 2 | SMARTER_Ph2 | S1 | SP-180589 | Li, Alice, Vodafone |
| --- | --- | --- | --- | --- | --- |

Summary based on the input provided by Vodafone in SP-200224.

There are strong interests among 3GPP network operators not only on addressing various existing and emerging markets and services to increase and diversify revenue streams, but also on enabling new business models and different operational schemes to maximise the use of the operators' networks.

TS 22.261 [1] specifies the requirements that define a 5G system in order to support new deployment scenarios to address diverse market segments, while the requirements for 5G E-UTRA-NR Dual Connectivity in E-UTRAN connected to EPC (i.e. “option 3”) are included in TS 22.278 [2]. This WI allows refinement of the identified stage 1 requirements in both specifications, including clarifications and extension due to the stage 2 developments in other WGs.

The clarifications and alignments are made with stage 2/3 progress in other WGs on the requirements carried forward from Release 15. The main updates include:

- For the 5G system service requirements specified in TS 22.261

  - A statement is added to the Scope to clarify that TS 22.261 provides requirements related to a 5G Core, i.e., specifically excluding Option 3.

  - Clarifications are added on the performance requirements for low-latency and high-reliability scenarios.

  - Clarifications are added on unified access control requirements.

  - Support of legacy USIM in 5G is added.

  - Clarifications are added on communication service availability and reliability.

  - Alignments are added on higher-accuracy positioning.

- For the requirements for 5G E-UTRA-NR Dual Connectivity in E-UTRAN connected to EPC (i.e. “option 3”) specified in TS 22.278

  - A statement is added to the Scope to clarify that TS 22.278 provides requirements related to 5G Option 3.

  - Added the 5G URLLC KPIs from Release 15 onwards to align with stage 2 agreements.

  - New service requirements are added corresponding to the enhancements developed in CIoT to align with stage2/3 progress.

  - Added the 5G requirements on service continuity.

  - Added the 5G requirements on context aware network.

References

[1]	TS 22.261, "Service requirements for the 5G system"

[2]	TS 22.278, "Service requirements for the Evolved Packet System (EPS)"

## 20.7	Ambient noise test methodology for evaluation of acoustic UE performance

| 870010 | Ambient noise test methodology for evaluation of acoustic UE performance | ANTeM | S4 | SP-200051 | Jan Reimes, HEAD acoustics GmbH |
| --- | --- | --- | --- | --- | --- |

Summary based on the input provided by China Mobile in SP-200265.

The work item ANTeM [1] was subsequently initiated to the feasibility study FS_ANTeM [2] in order to follow the conclusion of TR 26.921 [3] regarding handset speech quality testing under ambient noise conditions. Results of the round robin test conducted in FS_ANTeM indicated that an alternative noise field simulation provides test results equivalent to the currently specified method, while having a more efficient equalization procedure and less variance across labs at the same time. To include this new method in TS 26.132 [4], a corresponding change request [5] was agreed.

For speech quality evaluation of UEs under ambient noise conditions, the technical specification TS 26.132 [4] provides test methodologies. For measurements in handset mode, the background noise playback system according to ETSI ES 202 396-1 [6] is used. Within the study item FS_ANTeM, the applicability of the more recent ambient noise simulation according to ETSI TS 103 224 [7] was investigated, which provides the advantage of a fully automated calibration routine.

The so-called "flexible configurations" of ETSI TS 103 224 [7] were found to be useful, since they can be used in conjunction with the currently specified binaural noise types of TS 26.132 [4] to provide backward compatibility with the currently used simulation according to ETSI ES 202 396-1 [6].

The equivalency of the two noise field simulation systems was also shown by almost identical measurement results in the round robin test (comprehensive results of this study are provided in TR 26.921 [3]). The resulting CR [5] specified the flexible configurations of ETSI TS 103 224 [7] as an additional and preferred simulation method.

References

[1]	S4-200192: "New WID on Ambient noise test methodology for evaluation of acoustic UE performance".

[2]	S4-181230: "New SID on ambient noise test methodology for acoustic performance evaluation of UEs".

[3]	3GPP TR 26.921: "Investigations on ambient noise reproduction systems for acoustic testing of terminals".

[4]	3GPP TS 26.132: "Speech and video telephony terminal acoustic test specification".

[5]	S4-200305: "Alternative noise field simulation method for terminal testing" (CR 0102 to TS 26.132).

[6]	ETSI ES 202 396-1: "Speech and multimedia Transmission Quality (STQ); Speech quality performance in the presence of background noise; Part 1: Background noise simulation technique and background noise database".

[7]	ETSI TS 103 224: "Speech and multimedia Transmission Quality (STQ); A sound field reproduction method for terminal testing including a background noise database".

## 20.8	KPI reporting

| 810031 | Enhancement of performance assurance for 5G networks including network slicing | 5G_SLICE_ePA | S5 | SP-190247 | Xiaowei Sun (China Mobile) |
| --- | --- | --- | --- | --- | --- |
| 850055 | Overall aspects of 5G_SLICE_ePA | 5G_SLICE_ePA | S5 | SP-190247 | Xiaowei Sun (China Mobile) |
| 850029 | KPI reporting | 5G_SLICE_ePA-KPI | S5 | SP-190881 | ZHU, Weihong, ZTE Corporation |

Summary based on the input provided by ZTE in SP-200520.

The work item KPI reporting is the building block of work item 5G_SLICE_ePA (Enhancement of performance assurance for 5G networks including network slicing), it defined the use cases and requirements of KPI reporting, and enhanced the performance measurement job control related operations, performance measurement control NRM fragment and KPI template to enable the capability of KPI reporting.

The work item KPI reporting finished the enhancement on the performance measurement mechanism to enable the capability of KPI reporting, which includes:

-	add the use cases and requirements of KPI reporting

-	enhance the performance assurance Management Service (MnS) to support KPI collection and reporting

-	enhance the KPI template and definitions to support KPI collection and reporting

-	enhance the performance measurement control NRM fragment to support KPI collection and reporting

The work item resulted in a number of CRs on TS 28.550, 28.554, 28.622 and 28.623.

References

[1]	WID: SP-190881

[2]	CRs: S5-196410, S5-196748, S5-197571, S5-197572, S5-197575, S5-201522, S5-201581, S5-201582

# 21	Telecom Management

## 21.1	Network and Service Management

### 21.1.1	5G Management capabilities

| 860023 | 5G Management capabilities | 5GMNC | S5 | SP-191195 | Jean-Michel Cornily, Orange |
| --- | --- | --- | --- | --- | --- |

Summary based on the input provided by Orange in SP-200535.

In addition to generic management services, 5G management capabilities are needed to support the management and orchestration of 5G networks. This Rel-16 work item focuses on the heartbeat management capability.

This Work Item has produced the TS 28.537: Management and orchestration; Management capabilities.

TS 28.537 [1] is a placeholder for the specification of 5G management capabilities. In Rel-16, only the heartbeat management capability is addressed.

The heartbeat management capability is needed to monitor the communication between Management Service (MnS) producers and MnS consumers, and to discover communication link breaks between them as early as possible.

TS 28.537 [1] captures use cases, requirements and procedures for the heartbeat management capability.

References

[1] TS 28.537: Management and orchestration; Management capabilities

### 21.1.2	Energy Efficiency of 5G

| 810023 | Energy Efficiency of 5G | EE_5G | S5 | SP-191193 | Jean-Michel Cornily, Orange |
| --- | --- | --- | --- | --- | --- |

Summary based on the input provided by Orange in SP-200228.

The term ‘Energy Efficiency’ (EE) is to be understood as covering two areas:

1. Definition of EE KPI, required performance measurements and method(s) to collect them;

2. Definition of use cases, requirements and solutions to optimize Energy Efficiency – this is also sometimes referred to as Energy Saving.

Both aspects have been addressed in SA5 for pre-5G radio access technologies, focusing on how OA&M can provide support to them. This work item (EE_5G) addresses NG-RAN.

This Work Item has produced the Rel-16 TS 28.310 "Management and orchestration; Energy efficiency of 5G", as well as CRs 0055 and 0111 to TS 28.552 and CR 0032 to TS 28.554.

TS 28.310 [1] captures use cases, requirements and procedures for collecting performance measurements (namely data volume measurements and PEE (Power, Energy and Environment) parameters from 5G base stations. The EE KPI for 5G base stations is obtained thanks to these measurements. Those measurements are collected by the OSS thanks to the usage of generic management services defined in TS 28.532. Data Volume (DV) measurements, at PDCP layer, have been introduced in TS 28.552 [2], as well as PEE parameters definition (amongst which the Energy Consumption (EC) of the base station). A new KPI, named EE KPI, calculated by dividing Data Volume (DV) by Energy Consumption (EC) has been defined in TS 28.554 [5]. All this is fully aligned with baseline ETSI TC EE (Environmental Engineering) specifications ES 203 228 [7] and ES 202 336-12 [8].

Some intra- and inter-RAT energy saving use cases and requirements have been specified in TS 28.310 [1]. The 5G Network Resource Model (TS 28.541 [9]) has been augmented with an Energy Saving NRM fragment enabling the management of both centralized and distributed energy saving modes.

References

[1] TS 28.310: Management and orchestration; Energy efficiency of 5G

[2] TS 28.552: Management and orchestration; 5G performance measurements

[3] SP-190755: Add Power, Energy and Environmental (PEE) measurements and related use case description

[4] SP-190119: Add PDCP data volume measurements for EE

[5] TS 28.554: Management and orchestration; 5G end to end Key Performance Indicators (KPI)

[6] SP-191165: Add 5G Energy Efficiency KPI

[7] ETSI ES 203 228: Environmental Engineering (EE); Assessment of mobile network energy efficiency

[8] ETSI ES 202 336-12: Environmental Engineering (EE); Monitoring and control interface for infrastructure equipment (power, cooling and building environment systems used in telecommunication networks); Part 12: ICT equipment power, energy and environmental parameters monitoring information model

[9] TS 28.541: Management and orchestration; 5G Network Resource Model (NRM); Stage 2 and stage 3

### 21.1.3	OAM aspects of LTE and WLAN integration

| 810024 | OAM aspects of LTE and WLAN integration | OAM_LTE_WLAN | S5 | SP-180820 | Yizhi Yao, Intel Corporation |
| --- | --- | --- | --- | --- | --- |

Summary based on the input provided by Intel Corporation in SP-191252.

The OAM_LTE_WLAN work item defined the requirements and solutions for management aspects of LWA (LTE-WLAN aggregation) and LWIP (LTE WLAN integration with IPsec tunnel) features defined by 3GPP RAN WGs. The management aspects of LWA and LWIP addressed by this work item include:

-   Configurations to eNB to support LWA and LWIP;

-   Management model for WT for co-located and non-co-located LWA deployment scenarios; and

-   Performance measurements to monitor the LWA (including co-located and non-co-located scenarios) and LWIP.

The work item resulted in a number of CRs on TS 32.425, 28.657, 28.658, and 28.659 based on the preceding TR 32.868.

### 21.1.4	Methodology for 5G management specifications

| 810026 | Methodology for 5G management specifications | METHOGY | S5 | SP-180822 | Jan Groenendijk, Ericsson |
| --- | --- | --- | --- | --- | --- |

Summary based on the input provided by Ericsson in SP-200531.

This work item documents the methodology used to document the various specification artefacts, such as requirements and solutions, of the network management services. The target group for the methodology specification is the specification authors for the OAM specifications.

The methodologies include template(s) and network resource model repertoire.

This work item resulted in several CR’s to update network resource model repertoire TS 32.156 [1] and a new specification;

- TS 32.160: Management and orchestration: Management service template

Reference:

[1] TS 32.156: Telecommunication management; Fixed Mobile Convergence (FMC) model repertoire

### 21.1.5	Closed loop SLS Assurance

| 850026 | Closed loop SLS Assurance | COSLA | S5 | SP-190781 | Jan Groenendijk, Ericsson |
| --- | --- | --- | --- | --- | --- |

Summary based on the input provided by Ericsson in SP-200532.

This work item specifies a closed loop assurance solution that helps an operator to continuously deliver the expected level of communication service quality. The closed loop assurance solution allows an operator to create a closed loop management service that automatically adjusts and optimizes the services provided by NG-RAN and 5GC based on the various performance management and QoE input data, and the state of the 5G network, using data analytics. To be able to deploy SLS assurance solutions number of areas need are addressed:

- 	key management control loops in SLS assurance, key entities in the loops (e.g. MDAS)

-	describe important data and enable efficient data collection [for SLS assurance] from NG-RAN and 5GC (includes NWDAF information) to consumers in OAM, e.g. performance management and configuration data

-	describe coordination and management of the management functions involved in SLS assurance loops.

-	placement and role of management analytics functions in the OAM framework.

This work item resulted in several CR’s to update architecture framework TS 28.533 [1] and two new specifications:

- TS 28.535: Management and orchestration: Management Services for Communication Service Assurance; Requirements

- TS 28.536: Management and orchestration: Management Services for Communication Service Assurance; Stage 2 and stage 3

Reference:

[1] TS 28.533: Management and orchestration: Architecture framework

### 21.1.6	Trace Management in the context of Services Based Management Architecture and Streaming Trace reporting

| 820036 | Trace Management in the context of Services Based Management Architecture | TB_SBMA | S5 | SP-181073 | Anatoly Andrianov, Nokia |
| --- | --- | --- | --- | --- | --- |
| 850027 | Streaming Trace reporting | OAM_RTT | S5 | SP-190782 | Anatoly Andrianov, Nokia |

Summary based on the input provided by Ericsson in SP-200532.

The combination of the 3GPP Rel-16 Trace features ("Trace management in the context of Services Based Management Architecture (SBMA)" and "Streaming Trace reporting") transformed the legacy, partially outdated trace functionality into a modern flexible and powerful toolset.

As SA5 no longer mandates the functional management architecture focused on interactions between IRPManager and IRPAgent over Itf-N, a new way of trace management was necessary. The concept of "configurable trace" has been introduced where the management trace activation is represented by the act of Managed Object Instance (MOI) creation. The new TraceJob Information Object Class (IOC) also known as "Trace control NRM fragment" has been specified for this purpose in the Generic NRM TSs 28.622 [1] and 28.623 [2]. The creation, deletion and modification of the MOIs based on the new IOC does not require a dedicated API and is performed by the means of Provisioning Management Service (Provisioning MnS) specified in TS 28.532 [3]. The corresponding content (concepts, requirements, definitions) has been added to the family of trace specifications (TSs 32.421 [4] and 32.422 [5]), while all the dependencies on the legacy Trace Management IRP and legacy functional architecture have been eliminated.

TR 28.806 [8] identified and documented use cases for collecting Cell and UE data for analytics, and for Collecting subscriber and equipment trace data for near-real-time diagnostics and troubleshooting. It established that enabling the capabilities for trace data to be delivered to the consumer(s) while it's still relevant to the analytics, diagnostics and troubleshooting tasks performed by the consumer is important. The normative work on streaming trace data reporting leveraged the model-driven approach to the trace management and SBMA framework (TS 28.533 [7]). It added the new use cases and requirements for streaming trace to the TS 32.421 [4], addressed all relevant trace control and configuration management aspects in TS 32.422 [5] and specified the serialization format for the streaming trace records in TS 32.423 [6]. The initial work on a dedicated Streaming Trace data reporting MnS has been further extended and generalized into support of additional streaming data types such as Streaming PM, Streaming Analytics and proprietary data. The new Streaming data reporting MnS has been fully specified in TS 28.532 [3] it includes a RESTful HTTP based solution set for streaming connection establishment and connection/stream meta-data exchange and an efficient protocol stack for high volume/high speed streaming data reporting based on WebSockets. The use of SBMA framework does not limit the consumers and producers of the new MnS to 3GPP Management Functions and supports the use cases where 3GPP Network Element (NE) reports management data (Trace, PM, Analytics and proprietary) to any authorized MnS consumer.

References

[1]	3GPP TS 28.622 "Telecommunication management; Generic Network Resource Model (NRM) Integration Reference Point (IRP); Information Service (IS)"

[2]	3GPP TS 28.623 "Telecommunication management; Generic Network Resource Model (NRM) Integration Reference Point (IRP); Solution Set (SS) definitions"

[3]	3GPP TS 28.532 "Management and orchestration; Generic management services"

[4]	3GPP TS 32.421 "Telecommunication management; Subscriber and equipment trace; Trace concepts and requirements"

[5]	3GPP TS 32.422 "Telecommunication management; Subscriber and equipment trace; Trace control and configuration management"

[6]	3GPP TS 32.423 "Telecommunication management; Subscriber and equipment trace; Trace data definition and management"

[7]	3GPP TS 28.533 "Management and orchestration; Architecture framework"

[8]	3GPP TS 28.806 "Study on non-file-based trace reporting"

## 21.2	Charging Management

### 21.2.1	Charging Enhancement of 5GC interworking with EPC

| 820031 | Charging Enhancement of 5GC interworking with EPC | 5GIEPC_CH | S5 | SP-181067 | Chen Shan, Huawei |
| --- | --- | --- | --- | --- | --- |

Summary based on the input provided by Huawei in SP-190855.

The WI 5GIEPC_CH is the enhancement of the WI 5GS_Ph1-SBI_CH and 5GS_Ph1-DCH (handover with N26 scenario) on charging aspect of 5GC interworking with EPC, to specify procedure, parameter and Open API impact for the handover cancel and handover without N26 scenarios.

Stage 2 work on WI 5GIEPC_CH for TS 32.255 [1] and TS 32.290 [2]:

-	New message flow for EPS to 5GS handover for roaming in Home routed scenario, reflecting the "Visited created Charging Id, Home provided Charging ID" conveyed between VPLMN and HPLMN, and their use within each PLMN.

-	Definition of charging information for interworking with EPC: use of 5G attributes and CDR fields extended with EPC specific ones in the new Interworking Annex.

-	Handover triggers for handover cancel and handover without N26 scenarios.

-	Stage 3 work on WI 5GIEPC_CH for 32.291 [3] and 32.298 [4]:

-	TS 32.291 Nchf_ConvergedCharging API

-	Extensions of Data model

-	Update OpenAPI

-	TS 32.298 ASN.1 CHF CDR with "charging Ids" and extended EPC parameters

References

[1]	TS 32.255: "5G data connectivity domain charging"

[2] 	TS 32.290: "5G system; Services, operations and procedures of charging using Service Based Interface (SBI)"

[3]	TS 32.291: "5G system; Charging service, stage 3"

[4] 	TS 32.298: "Charging Data Record (CDR) parameter description"

## 21.3	Other charging and management items

The other charging and management items are reported under the Feature they relate to.

E.g. " Charging Aspect for 5WWC" is reported under "5WWC" rather than under "Charging".

# 22	Other items

## 22.1	Items not (fully) completed in Rel-16

These items have not been officially moved to the next Release at the time of writing this document but are not implementable in Rel-16.

### 22.1.1	Remote Identification of Unmanned Aerial Systems

| 810049 | Remote Identification of Unmanned Aerial Systems | ID_UAS |  | SP-180771 | Eddy HALL, Qualcomm |
| --- | --- | --- | --- | --- | --- |

This item was moved to Rel-17.

### 22.1.2	5G message service

| 810050 | 5G message service | 5GMSG | S1 | SP-180772 | Zheng, Jianping, China Mobile |
| --- | --- | --- | --- | --- | --- |
| 770004 | Study on 5G message service for MIoT | FS_5GMSG | S1 | SP-170704 | Zheng, Jianping, China Mobile |
| 810014 | Message Service Within the 5G System requirements | MSGin5G | S1 | SP-180920 | Zheng, Jianping, China Mobile |

Summary based on the input provided by China Mobile in SP-200265.

This work item specifies the service level requirements of the MSGin5G Service and the new requirements of the 5G System to support the MSGin5G Service [1].

The MSGin5G Service is basically designed and optimized for massive IoT device communication including thing-to-thing communication and person-to-thing communication. The MSGin5G Service enables various message communication models including point-to-point, application-to-point, group and broadcast message communication, with advanced service capabilities and performance, e.g. very low end-to-end latency and high reliability of message delivery, in a resource efficient manner to optimize the resource usage of the both control plane and user plane in the network, and power saving in the user devices.

Stage 2 work of the MSGin5G Service is undergoing in SA6 with a SI [2] [3].

References

[1] TS 22.262, Message Service within the 5G System; Stage 1

[2] SP-190478, SID on Study on support of the 5GMSG Service

[3] TR23.700-24, Study on support of the 5GMSG Service

### 22.1.3	Integration of Satellite Access in 5G

| 0 | Satellite Access in 5G | - |  | - | - |
| --- | --- | --- | --- | --- | --- |
| 800010 | Integration of Satellite Access in 5G | 5GSAT | S1 | SP-180326 | MICHEL, Cyril, Thales |
| 770002 | Study on using Satellite Access in 5G | FS_5GSAT | S1 | SP-170702 | MICHEL, Cyril |
| 800048 | Stage 1 of 5GSAT | 5GSAT | S1 | SP-180326 | MICHEL, Cyril, Thales |
| 800026 | Study on architecture aspects for using satellite access in 5G | FS_5GSAT_ARCH | S2 | SP-181253 | MICHEL, Cyril |
| 800099 | Study on solutions for NR to support non-terrestrial networks (NTN) | FS_NR_NTN_solutions | R3 | RP-190710 | Thales |
| 830025 | Study on management and orchestration aspects with integrated satellite components in a 5G network | FS_5GSAT_MO | S5 | SP-190138 | MICHEL, Cyril, THALES |

Only Stage 1 was defined in the Rel-16 time-frame. This is not consistent with the 3GPP methodology, where all aspects of a given Feature (Stages 1, 2, 3, charging, security, etc) have to be completed withing the same Release. As a consequence, the work completed for Stage 1 in Rel-16 will be moved to a future Release.

Annex A:
Full structure of the Work Plan as of Dec 2019. This is a non-updated guide to construct this TR.

| UID | Name | Acronym | WG | WID | Rapporteur |
| --- | --- | --- | --- | --- | --- |
| 0 | Release 16 Features and related studies | - |  | - | - |
| 0 | Enhancement of Ultra-Reliable (UR) Low Latency Communications (URLLC) | - |  | - | - |
| 830098 | Enhancement of URLLC support in the 5G Core network | 5G_URLLC | S2 | SP-181122 | Hui Ni, Huawei |
| 790008 | Study on enhancement of URLLC supporting in 5GC | FS_5G_URLLC | S2 | SP-180118 | Hui Ni, Huawei |
| 810036 | Study on the security of URLLC for 5GS | FS_5G_URLLC_SEC | S3 | SP-180910 | Rong Wu, Huawei Technologies |
| 820019 | Stage 2 of 5G_URLLC | 5G_URLLC | S2 | SP-181122 | Hui Ni, Huawei |
| 840026 | Security of URLLC for 5GS | 5G_URLLC_SEC | S3 | SP-190351 | Rong Wu, Huawei Technologies |
| 840005 | CT Aspects of 5G_URLLC | 5G_URLLC | ct | CP-191063 | Qi Caixia, Huawei |
| 840060 | CT3 Aspects of 5G_URLLC | 5G_URLLC | C3 | CP-191063 | Qi Caixia, Huawei |
| 840061 | CT4 Aspects of 5G_URLLC | 5G_URLLC | C4 | CP-191063 | Qi Caixia, Huawei |
| 780030 | Stopped - Study on EPC support for Mobility with Low Latency Communication | FS_LLC_Mob | S2 | SP-171069 | Chris PUDNEY |
| 790009 | Stopped - Study on enht of systems using EPS for UR and Availability using commodity equipment | FS_EPS_URACE | S2 | SP-180119 | Chris Pudney |
| 830074 | Physical Layer Enhancements for NR Ultra-Reliable and Low Latency Communication (URLLC) | NR_L1enh_URLLC | R1 | RP-191584 | Huawei |
| 800095 | Study on physical layer enhancements for NR UR Low Latency Cases | FS_NR_L1enh_URLLC | R1 | RP-181477 | Huawei |
| 830174 | Core part: Physical Layer Enhancements for NR Ultra-Reliable and Low Latency Communication (URLLC) | NR_L1enh_URLLC-Core | R1 | RP-190726 | Huawei |
| 830274 | Perf. part: Physical Layer Enhancements for NR Ultra-Reliable and Low Latency Communication (URLLC) | NR_L1enh_URLLC-Perf | R4 | RP-190726 | Huawei |
| 830080 | Support of NR Industrial Internet of Things (IoT) | NR_IIOT | R2 | RP-191561 | Nokia |
| 810060 | Study on channel modeling for indoor industrial scenarios | FS_IIIOT_CM | R1 | RP-182138 | Ericsson |
| 800098 | Study on NR Industrial Internet of Things (IoT) | FS_NR_IIOT | R2 | RP-181479 | Nokia |
| 830180 | Core part: Support of NR Industrial Internet of Things (IoT) | NR_IIOT-Core | R2 | RP-190728 | Nokia |
| 820999 | 5GS Enhanced support of Vertical and LAN Services | - |  | - | - |
| 750004 | Study on Communication for Automation in Vertical Domains | FS_CAV | S1 | SP-170169 | Walewski, Joachim, Siemens AG |
| 800007 | Service requirements for cyber-physical control applications in vertical domains | cyberCAV | S1 | SP-180321 | Joachim W. Walewski (Siemens) |
| 830042 | 5GS Enhanced support of Vertical and LAN Services | Vertical_LAN | S2 | SP-181120 | Devaki Chandramouli, Nokia |
| 800028 | Study on 5GS Enhanced support of Vertical and LAN Services | FS_Vertical_LAN | S2 | SP-180507 | Devaki Chandramouli, Nokia |
| 820017 | Stage 2 of Vertical_LAN | Vertical_LAN | S2 | SP-181120 | Devaki Chandramouli, Nokia |
| 810038 | Study on Security for 5GS Enhanced support of Vertical and LAN Services | FS_Vertical_LAN_SEC | S3 | SP-180697 | Jerichow, Anja (Nokia) |
| 840027 | Security for Vertical_LAN | Vertical_LAN_SEC | S3 | SP-190352 | Jerichow, Anja, Nokia |
| 830010 | CT aspects of Vertical_LAN | Vertical_LAN | ct | CP-191157 | WON, Sung Hwan, Nokia |
| 830035 | CT1 aspects of Vertical_LAN | Vertical_LAN | C1 | CP-191157 | WON, Sung Hwan, Nokia |
| 830036 | CT3 aspects of Vertical_LAN | Vertical_LAN | C3 | CP-191157 | WON, Sung Hwan, Nokia |
| 830037 | CT4 aspects of Vertical_LAN | Vertical_LAN | C4 | CP-191157 | WON, Sung Hwan, Nokia |
| 820027 | Service Enabler Architecture Layer for Verticals | SEAL | S6 | SP-181141 | Basavaraj (Basu) Pattan, Samsung |
| 820067 | NR-based access to unlicensed spectrum | NR_unlic | R1 | RP-191575 | Qualcomm |
| 750045 | Study on NR-based access to unlicensed spectrum | FS_NR_unlic | R1 | RP-181339 | Qualcomm |
| 820167 | Core part: NR-based access to unlicensed spectrum | NR_unlic-Core | R1 | RP-190706 | Qualcomm |
| 820267 | Perf. part: NR-based access to unlicensed spectrum | NR_unlic-Perf | R4 | RP-190706 | Qualcomm |
| 800006 | LAN support in 5G | 5GLAN | S1 | SP-180593 | Jose Almodovar, KPN |
| 760007 | Study on LAN Support in 5G | FS_5GLAN | S1 | SP-170456 | Nasielski, Jack, Qualcomm |
| 800047 | Stage 1 of 5GLAN | 5GLAN | S1 | SP-180593 | Jose Almodovar, KPN |
| 830024 | Study on non-public networks management | FS_OAM_NPN | S5 | SP-190137 | ZHANG, Kai, Huawei |
| 0 | Cellular IoT support and evolution | - |  | - | - |
| 830043 | Cellular IoT support and evolution for the 5G System | 5G_CIoT | S2 | SP-181118 | Sebastian Speicher |
| 770038 | Study on 5G_CIoT | FS_CIoT_5G | S2 | SP-180614 | Sebastian Speicher |
| 820015 | Stage 2 of 5G_CIoT | 5G_CIoT | S2 | SP-181118 | Sebastian Speicher |
| 830013 | CT aspects of 5G_CIoT | 5G_CIoT | ct | CP-111237 | Mahmoud Watfa; Qualcomm |
| 830038 | CT1 aspects of 5G_CIoT | 5G_CIoT | C1 | CP-111237 | Mahmoud Watfa; Qualcomm |
| 830039 | CT3 aspects of 5G_CIoT | 5G_CIoT | C3 | CP-111237 | Mahmoud Watfa; Qualcomm |
| 830040 | CT4 aspects of 5G_CIoT | 5G_CIoT | C4 | CP-111237 | Mahmoud Watfa; Qualcomm |
| 830041 | CT6 aspects of 5G_CIoT | 5G_CIoT | C6 | CP-111237 | Mahmoud Watfa; Qualcomm |
| 800033 | Study on evolution of Cellular IoT security for the 5G System | FS_CIoT_sec_5G | S3 | SP-180440 | Noamen Ben Henda |
| 760040 | Study on MBMS User Services for IoT | FS_MBMS_IoT | S4 | SP-170592 | Cedric thienot, Expway |
| 800083 | Additional MTC enhancements for LTE | LTE_eMTC5 | R1 | RP-191356 | Ericsson |
| 800183 | Core part: Additional MTC enhancements for LTE | LTE_eMTC5-Core | R1 | RP-190770 | Ericsson |
| 800283 | Perf. part: Additional MTC enhancements for LTE | LTE_eMTC5-Perf | R4 | RP-190770 | Ericsson |
| 800084 | Additional enhancements for NB-IoT | NB_IOTenh3 | R1 | RP-191576 | Huawei |
| 800184 | Core part: Additional enhancements for NB-IoT | NB_IOTenh3-Core | R1 | RP-190757 | Huawei |
| 800284 | Perf. part: Additional enhancements for NB-IoT | NB_IOTenh3-Perf | R4 | RP-190757 | Huawei |
| 810050 | 5G message service | 5GMSG | S1 | SP-180772 | Zheng, Jianping, China Mobile |
| 770004 | Study on 5G message service for MIoT | FS_5GMSG | S1 | SP-170704 | Zheng, Jianping, China Mobile |
| 810014 | Message Service Within the 5G System requirements | MSGin5G | S1 | SP-180920 | Zheng, Jianping, China Mobile |
| 0 | Advanced V2X support | - |  | - | - |
| 790002 | Study on Improvement of V2X Service Handling | FS_V2XIMP | S1 | SP-180247 | SungDuck Chun; LG Electronics |
| 820024 | Improvement of V2X service Handling | V2XIMP | S1 | SP-181013 | SungDuck Chun; LG Electronics |
| 840074 | Application layer support for V2X services | V2XAPP | S6 | SP-180898 | Niranth Amogh, Huawei Tel.India |
| 780025 | Study on V2XAPP | FS_V2XAPP | S6 | SP-171071 | Niranth Amogh, Huawei Tel.India |
| 810020 | Stage 2 of V2XAPP | V2XAPP | S6 | SP-180898 | Niranth Amogh, Huawei Tel.India |
| 840010 | CT aspects of V2XAPP | V2XAPP | ct | CP-191153 | Herrero Veron, Christian (Huawei) |
| 840075 | CT1 aspects of V2XAPP | V2XAPP | C1 | CP-191153 | Herrero Veron, Christian (Huawei) |
| 840076 | CT3 aspects of V2XAPP | V2XAPP | C3 | CP-191153 | Herrero Veron, Christian (Huawei) |
| 840077 | CT4 aspects of V2XAPP | V2XAPP | C4 | CP-191153 | Herrero Veron, Christian (Huawei) |
| 840078 | Architecture enhancements for 3GPP support of advanced V2X services | eV2XARC | S2 | SP-181121 | LaeYoung Kim, LG Electronics |
| 760043 | Study on eV2XARC | FS_eV2XARC | S2 | SP-180733 | LaeYoung Kim, LG Electronics |
| 820018 | Stage 2 of eV2XARC | eV2XARC | S2 | SP-181121 | LaeYoung Kim, LG Electronics |
| 840011 | CT aspects of eV2XARC | eV2XARC | ct | CP-191154 | Herrero Veron, Christian (Huawei) |
| 840079 | CT1 aspects of eV2XARC | eV2XARC | C1 | CP-191154 | Herrero Veron, Christian (Huawei) |
| 840080 | CT3 aspects of eV2XARC | eV2XARC | C3 | CP-191154 | Herrero Veron, Christian (Huawei) |
| 840081 | CT4 aspects of eV2XARC | eV2XARC | C4 | CP-191154 | Herrero Veron, Christian (Huawei) |
| 840082 | CT6 aspects of eV2XARC | eV2XARC | C6 | CP-191154 | Herrero Veron, Christian (Huawei) |
| 800096 | Study on NR Vehicle-to-Everything (V2X) | FS_NR_V2X | R1 | RP-190224 | LG Electronics |
| 770029 | Study on V2X Media Handling and Interaction | FS_mV2X | S4 | SP-170799 | Jung, Kyunghun, Samsung Electronics Co., Ltd |
| 830023 | Study on Security Aspects of 3GPP support for Advanced V2X Services | FS_eV2X_Sec | S3 | SP-190108 | Joonwoong Kim, LG Electronics |
| 830078 | 5G V2X with NR sidelink | 5G_V2X_NRSL | R1 | RP-190984 | LG Electronics |
| 830178 | Core part: 5G V2X with NR sidelink | 5G_V2X_NRSL-Core | R1 | RP-190766 | LG Electronics |
| 830278 | Perf. part: 5G V2X with NR sidelink | 5G_V2X_NRSL-Perf | R4 | RP-190766 | LG Electronics |
| 0 | 5G Location and Positionning Services | - |  | - | - |
| 800013 | 5G positioning services | 5G_HYPOS | S1 | SP-180329 | Ries, Lionel, ESA |
| 760006 | Study on positioning use cases | FS_5G_HYPOS | S1 | SP-170589 | Ries, Lionel, ESA |
| 800049 | Stage 1 of 5G_HYPOS | 5G_HYPOS | S1 | SP-180329 | Ries, Lionel, ESA |
| 830102 | Enhancement to the 5GC LoCation Services | 5G_eLCS | S2 | SP-181119 | Ming Ai, CATT |
| 780028 | Study on Enhancement to the 5GC Location Services | FS_eLCS | S2 | SP-180734 | Ming Ai, CATT |
| 810035 | Study on Security of the enhancement to the 5GC location services | FS_eLCS_Sec | S3 | SP-180694 | Wei Zhou, CATT |
| 820016 | Stage 2 of 5G_eLCS | 5G_eLCS | S2 | SP-181119 | Ming Ai, CATT |
| 830003 | CT aspects of 5G_eLCS | 5G_eLCS | ct | CP-190014 | Tingfang Tang, CATT |
| 830044 | CT1 aspects of 5G_eLCS | 5G_eLCS | C1 | CP-191064 | Tingfang Tang, CATT |
| 830045 | CT3 aspects of 5G_eLCS | 5G_eLCS | C3 | CP-191064 | Tingfang Tang, CATT |
| 830046 | CT4 aspects of 5G_eLCS | 5G_eLCS | C4 | CP-191064 | Tingfang Tang, CATT |
| 830077 | NR positioning support | NR_pos | R1 | RP-191156 | Intel |
| 800092 | Study on NR positioning support | FS_NR_pos | R1 | RP-181399 | Intel |
| 830073 | Study on local NR positioning in NG-RAN | FS_NR_local_pos | R3 | RP-190754 | CMCC |
| 830177 | Core part: NR positioning support | NR_pos-Core | R1 | RP-190752 | Intel |
| 830277 | Perf. part: NR positioning support | NR_pos-Perf | R4 | RP-190752 | Intel |
| 0 | UE radio capability signalling optimization | - |  | - | - |
| 840054 | Optimisations on UE radio capability signalling | RACS | S2 | RP-191460 | Haris Zisimopoulos |
| 800025 | Study on RACS | FS_RACS | S2 | SP-180599 | Haris Zisimopoulos, Qualcomm |
| 830027 | Stage 2 of RACS | RACS | S2 | SP-190180 | Haris Zisimopoulos |
| 840003 | CT aspects of RACS | RACS | ct | CP-191061 | Chaponniere, Lena, Qualcomm |
| 840055 | CT1 aspects of RACS | RACS | C1 | CP-191061 | Chaponniere, Lena, Qualcomm |
| 840056 | CT3 aspects of RACS | RACS | C3 | CP-191061 | Chaponniere, Lena, Qualcomm |
| 840057 | CT4 aspects of RACS | RACS | C4 | CP-191061 | Chaponniere, Lena, Qualcomm |
| 830079 | Optimisations on UE radio capability signalling – NR/E-UTRA Aspects | RACS-RAN | R2 | RP-190657 | MediaTek |
| 800097 | Study on optimisations on UE radio capability signalling – NR/E-UTRA Aspects | FS_RACS_RAN | R2 | RP-181459 | MediaTek |
| 830179 | Core part: Optimisations on UE radio capability signalling – NR/E-UTRA Aspects | RACS-RAN-Core | R2 | RP-191088 | MediaTek |
| 0 | Satellite Access in 5G | - |  | - | - |
| 800010 | Integration of Satellite Access in 5G | 5GSAT | S1 | SP-180326 | MICHEL, Cyril, Thales |
| 770002 | Study on using Satellite Access in 5G | FS_5GSAT | S1 | SP-170702 | MICHEL, Cyril |
| 800048 | Stage 1 of 5GSAT | 5GSAT | S1 | SP-180326 | MICHEL, Cyril, Thales |
| 800026 | Study on architecture aspects for using satellite access in 5G | FS_5GSAT_ARCH | S2 | SP-181253 | MICHEL, Cyril |
| 800099 | Study on solutions for NR to support non-terrestrial networks (NTN) | FS_NR_NTN_solutions | R3 | RP-190710 | Thales |
| 830025 | Study on management and orchestration aspects with integrated satellite components in a 5G network | FS_5GSAT_MO | S5 | SP-190138 | MICHEL, Cyril, THALES |
| 0 | Enablers for Network Automation Architecture for 5G | - |  | - | - |
| 830047 | Enablers for Network Automation for 5G | eNA | S2 | SP-181123 | Xiaobo Wu, Huawei Technologies |
| 760047 | Study of enablers for Network Automation for 5G | FS_eNA | S2 | SP-180792 | Xiaobo Wu, Huawei Technologies |
| 820020 | Stage 2 of eNA | eNA | S2 | SP-181123 | Xiaobo Wu, Huawei Technologies |
| 830009 | CT aspects of eNA | eNA | ct | CP-191111 | Yali Yan, Huawei |
| 830048 | CT3 aspects of eNA | eNA | C3 | CP-191111 | Yali Yan, Huawei |
| 830049 | CT4 aspects of eNA | eNA | C4 | CP-191111 | Yali Yan, Huawei |
| 0 | Wireless and Wireline Convergence Enhancement | - |  | - | - |
| 760044 | Study on the Wireless and Wireline Convergence for the 5G system architecture | FS_5WWC | S2 | SP-170380 | Marco Spini, Huawei Technologies |
| 800031 | Study on the security of the Wireless and Wireline Convergence for the 5G system architecture | FS_5WWC_SEC | S3 | SP-180438 | He Li, Huawei Technologies |
| 830050 | Wireless and Wireline Convergence for the 5G system architecture | 5WWC |  | SP-181117 | Marco Spini, Huawei Technologies |
| 820014 | Stage 2 of 5WWC | 5WWC | S2 | SP-181117 | Marco Spini, Huawei Technologies |
| 829910 | CT aspects of 5WWC | 5WWC | ct | CP-191156 | Herrero Veron, Christian (Huawei) |
| 829911 | CT1 aspects of 5WWC | 5WWC | C1 | CP-191156 | Herrero Veron, Christian (Huawei) |
| 829912 | CT3 aspects of 5WWC | 5WWC | C3 | CP-191156 | Herrero Veron, Christian (Huawei) |
| 829914 | CT4 aspects of 5WWC | 5WWC | C4 | CP-191156 | Herrero Veron, Christian (Huawei) |
| 830082 | NG interface usage for WWC | 5WWC-NG_interface | R3 | RP-190772 | Huawei |
| 830182 | Core part: NG interface usage for WWC | 5WWC-NG_interface-Core | R3 | RP-190999 | Huawei |
| 0 | Mission Critical, Public Warning, Railways and Maritime | - |  | - | - |
| 790024 | Study on MC services access aspects | FS_MCSAA | S6 | SP-181135 | Joakim Åkesson, Ericsson |
| 800024 | Study into discreet listening and logging for mission critical services | FS_MCLOG | S6 | SP-181138 | Chater-Lea, David, Motorola Solutions |
| 810055 | Study on location enhancements for mission critical services | FS_enhMCLoc | S6 | SP-180921 | Dom Lazara, Motorola Solutions |
| 800022 | Enhanced Mission Critical Push-to-talk architecture phase 2 | enh2MCPTT | S6 | SP-190068 | Dom Lazara; Motorola Solutions |
| 830051 | Enhancements to Functional architecture and information flows for Mission Critical Data | eMCData2 | S6 | SP-180378 | Shih, Jerry, AT&T |
| 800018 | Stage 2 of eMCData2 | eMCData2 | S6 | SP-180378 | Shih, Jerry, AT&T |
| 830014 | CT aspects of eMCData2 | eMCData2 | C1 | CP-190199 | Val Oprescu (AT&T) |
| 800023 | Study on Mission Critical services support over 5G System | FS_MCOver5GS | S6 | SP-181136 | Verweij, Kees, The Police of the Netherlands |
| 810004 | MCData File Distribution support over xMB | MC_XMB | S4 | SP-180665 | Thiénot Cédric, Expway |
| 810011 | Protocol enhancements for Mission Critical Services | MCProtoc16 | C1 | CP-182110 | AXELL, Jörgen |
| 800053 | MBMS APIs for Mission Critical Services | MBMSAPI_MCS | S6 | SP-180380 | Ling Zhang, TD Tech Ltd. |
| 760051 | Study on MBMS APIs for MC Services | FS_MBMSAPI_MC | S6 | SP-180237 | Ling Zhang, TD Tech Ltd |
| 800020 | Stage 2 of MBMS APIs for MC Services | MBMSAPI_MCS | S6 | SP-180380 | Ling Zhang, TD Tech Ltd. |
| 840013 | Enhancement of 3GPP Northbound APIs | eNAPIs | C3 | CP-191233 | Yali Yan, Huawei |
| 800019 | Enhanced Mission Critical System Migration and Interconnection | eMCSMI | S6 | SP-180379 | Chater-Lea, David; Motorola Solutions |
| 810054 | Enhanced Mission Critical Communication Interworking with Land Mobile Radio Systems | eMCCI |  |  | Monnes, Peter, Harris Corporation |
| 800021 | Enhanced Mission Critical Communication Interworking with Land Mobile Radio Systems | eMCCI | S6 | SP-180680 | Monnes, Peter, Harris Corporation |
| 800016 | Stage 3 for MC Communication Interworking with Land Mobile Radio Systems | MCCI_CT | C1 | CP-190203 | Monnes, Peter, Harris Corporation |
| 820040 | Mission Critical system migration and interconnection | MCSMI_CT | C1 | CP-190143 | Dom Lazara, Motorola Solutions |
| 800032 | Mission Critical Services Security Enhancements | MCXSec | S3 | SP-180596 | Woodward, Tim, Motorola Solutions, Inc |
| 780003 | Enhancements of Public Warning System | ePWS |  | SP-170998 | Hyounhee Koo, SyncTechno Inc. |
| 730005 | Study on enhancements of Public Warning System | FS_ePWS | S1 | SP-160733 | SyncTechno Inc., Hyounhee Koo |
| 800052 | Stage 1 of ePWS | ePWS | S1 | SP-170998 | Hyounhee Koo, SyncTechno Inc. |
| 810012 | CT aspects of enhancements of Public Warning System | ePWS | ct | CP-191155 | TaeHyeon Kim, SyncTechno Inc. |
| 810053 | Study on stages 2 and 3 of enhancements of ePWS | ePWS | C1 | CP-191155 | TaeHyeon Kim, SyncTechno Inc. |
| 810047 | CT1 aspects of ePWS | ePWS | C1 | CP-191155 | TaeHyeon Kim, SyncTechno Inc. |
| 810048 | CT4 aspects of ePWS (Possible impacts) | ePWS | C4 | CP-191155 | TaeHyeon Kim, SyncTechno Inc. |
| 760054 | Mobile Communication System for Railways 2 | MONASTERY2 |  | SP-170451 | Merkel, Juergen, Nokia |
| 760005 | Study on MONASTERY2 | FS_FRMCS2 | S1 | SP-170588 | Merkel, Juergen, Nokia |
| 790023 | Study on application architecture for the Future Railway Mobile Communication System (FRMCS) Phase 2 | FS_FRMCS2_SA6 | S6 | SP-181133 | Oettl, Martin, Nokia |
| 760004 | Stage 1 of MONASTERY2 | MONASTERY2 | S1 | SP-170451 | Merkel, Juergen, Nokia |
| 830007 | Application Architecture for MONASTERY2 | MONASTERY2 | S6 | SP-190064 | Oettl, Martin, Nokia |
| 840009 | CT aspects of MONASTERY2 | MONASTERY2 | ct | CP-191152 | Gkatzikis, Lazaros (Nokia) |
| 840072 | CT1 aspects of MONASTERY2 | MONASTERY2 | C1 | CP-191152 | Gkatzikis, Lazaros (Nokia) |
| 840073 | CT3 aspects of MONASTERY2 | MONASTERY2 | C3 | CP-191152 | Gkatzikis, Lazaros (Nokia) |
| 840092 | NR support for high speed train scenario | NR_HST | R4 | RP-191512 | CMCC |
| 840192 | Core part: NR support for high speed train scenario | NR_HST-Core | R4 | RP-191512 | CMCC |
| 840292 | Perf. part: NR support for high speed train scenario | NR_HST-Perf | R4 | RP-191512 | CMCC |
| 800011 | Maritime Communication Services over 3GPP System | MARCOM | S1 | SP-180594 | Hyounhee Koo, SyncTechno Inc |
| 730007 | Study on Maritime Communication Services over 3GPP system | FS_MARCOM | S1 | SP-170453 | SyncTechno Inc., Hyounhee Koo |
| 800051 | Stage 1 of MARCOM | MARCOM | S1 | SP-180594 | Hyounhee Koo, SyncTechno Inc |
| 0 | Streaming and TV | - |  | - | - |
| 800008 | Enhancement of LTE for Efficient delivery of Streaming Service | eLSTR | S1 | SP-180322 | Xia, Xu, China Telecom |
| 740001 | Study on eLSTR | FS_eLESTR | S1 | SP-160960 | China Telecommunications, XIA Xu |
| 800054 | Stage 1 of eLSTR | eLSTR | S1 | SP-180322 | Xia, Xu, China Telecom |
| 800001 | Enhancements to Framework for Live Uplink Streaming | E_FLUS | S4 | SP-180285 | Lo, Charles, Qualcomm |
| 810003 | Coverage and Handoff Enhancements for Multimedia | CHEM | S4 | SP-180664 | Nikolai Leung |
| 770027 | Study on QoE metrics for VR | FS_QoE_VR | S4 | SP-170614 | Ericsson (Gunnar Heikkilä |
| 760039 | Study on 5G enhanced Mobile Broadband Media Distribution | FS_5GMedia_Distribution | S4 | SP-170334 | Bouazizi, Imed, Samsung Electronics Co., Ltd., |
| 820002 | Media streaming architecture | 5GMSA | S4 | SP-180984 | Frédéric GABIN |
| 810040 | Media Handling Extensions for 5G Conversational Services | 5G_MEDIA_MTSI_ext | S4 | SP-180663 | Oyman, Ozgur, Company: Intel |
| 760041 | Study on Media Handling Aspects of Conversational Services in 5G Systems | FS_5G_MEDIA_MTSI | S4 | SP-170336 | Oyman, Ozgur, Company: Intel |
| 810002 | Media Handling Extensions for 5G Conversational Services | 5G_MEDIA_MTSI_ext | S4 | SP-180663 | Oyman, Ozgur, Company: Intel |
| 770031 | Study on EVS Float Conformance Non Bit-Exact | FS_EVS_FCNBE | S4 | SP-170618 | Fabrice Plante, Intel |
| 820001 | EVS Floating-point Conformance for Non Bit-Exact | EVS_FCNBE | S4 | SP-180983 | Fabrice, Plante, Intel |
| 800003 | Addition of HLG HDR to TV Video Profiles | HLG_HDR | S4 | SP-180287 | Paul Szucs, Sony Europe Limited |
| 0 | User Identities, Authentication, multi-device | - |  | - | - |
| 800012 | User Identities and Authentication | UIA | S1 | SP-180328 | Bischinger, Kurt; Deutsche Telekom |
| 780004 | Study on a Layer for User Centric Identifiers and Authentication | FS_LUCIA | S1 | SP-170995 | Bischinger, Kurt; Deutsche Telekom |
| 800050 | Stage 1 of User Identities and Authentication | UIA | S1 | SP-180328 | Bischinger, Kurt; Deutsche Telekom |
| 810044 | Multi-device and multi-identity | MuD | S1 | SP-180315 | Peter Bleckert, Ericsson |
| 800004 | Stage 1 of Multi-device and multi-identity | MuD | S1 | SP-180315 | Peter Bleckert, Ericsson |
| 810010 | CT aspects of Multi-device and multi-identity | MuD | ct | CP-182227 | Axell, Jörgen, Ericsson |
| 810045 | CT1 aspects of Multi-device and multi-identity | MuD | C1 | CP-182227 | Axell, Jörgen, Ericsson |
| 810046 | CT3 aspects of Multi-device and multi-identity | MuD | C3 | CP-182227 | Axell, Jörgen, Ericsson |
| 0 | Slicing | - |  | - | - |
| 830103 | Enhancement of Network Slicing | eNS |  | SP-181232 | ZTE, Tricci So |
| 790011 | Study on Enhancement of Network Slicing | FS_eNS | S2 | SP-180503 | ZTE, Tricci So |
| 820023 | Stage 2 of eNS | eNS | S2 | SP-181232 | ZTE, Tricci So |
| 830011 | CT aspects of eNS | eNS | ct | CP-190196 | Fei Lu, ZTE |
| 830052 | CT1 aspects of eNS | eNS | C1 | CP-190196 | Fei Lu, ZTE |
| 830053 | CT3 aspects of eNS | eNS | C3 | CP-190196 | Fei Lu, ZTE |
| 830054 | CT4 aspects of eNS | eNS | C4 | CP-190196 | Fei Lu, ZTE |
| 810051 | Business Role Models for Network Slicing | BRMNS | S1 | SP-180773 | Covell, Betsy, Nokia |
| 770005 | Study on Business Role Models for Network Slicing | FS_BMNS | S1 | SP-180169 | Covell, Betsy, Nokia |
| 810015 | Business Role Models for Network Slicing | BRMNS | S1 | SP-180773 | Covell, Betsy, Nokia |
| 810034 | Study on Security aspects of Enhancement of Network Slicing | FS_eNS_SEC | S3 | SP-180692 |  |
| 810022 | Study on tenancy concept in 5G networks and network slicing management | FS_TENANCYC | S5 | SP-180815 | ZHU, Lei (Huawei |
| 810031 | Enhancement of performance assurance for 5G networks including network slicing | 5G_SLICE_ePA | S5 | SP-190247 | Xiaowei Sun (China Mobile) |
| 820029 | Study on Charging Aspects of Network Slicing | FS_NETSLICE_CH | S5 | SP-181065 | Chen Shan, Huawei |
| 0 | Other cross-TSG Release 16 Features | - |  | - | - |
| 760003 | Provision of Access to Restricted Local Operator Services by Unauthenticated UEs | PARLOS |  | SP-170449 | Covell, Betsy Nokia |
| 740002 | Study on Stage 1 of PARLOS | FS_PARLOS | S1 | SP-160904 | Covell, Betsy |
| 760046 | Study on Stage 2 for PARLOS | FS_PARLOS_SA2 | S2 | SP-180501 | Nokia (Nicolas Drevon |
| 800035 | Study on Security Aspects of PARLOS | FS_PARLOS_Sec | S3 | SP-180442 | Greg Schumacher, Sprint |
| 760071 | Stage 1 of PARLOS | PARLOS | S1 | SP-170449 | Covell, Betsy Nokia |
| 810008 | Stage 2 of PARLOS | PARLOS | S2 | SP-180738 | Nokia (Nicolas Drevon |
| 830012 | CT aspects of PARLOS | PARLOS | ct | CP-190197 | Liu, Jennifer; Nokia |
| 830062 | CT1 aspects of PARLOS | PARLOS | C1 | CP-190197 | Liu, Jennifer; Nokia |
| 830063 | CT3 aspects of PARLOS | PARLOS | C3 | CP-190197 | Liu, Jennifer; Nokia |
| 830064 | CT4 aspects of PARLOS | PARLOS | C4 | CP-190197 | Liu, Jennifer; Nokia |
| 830065 | CT6 aspects of PARLOS | PARLOS | C6 | CP-190197 | Liu, Jennifer; Nokia |
| 820043 | Enhancing Topology of SMF and UPF in 5G Networks | ETSUN |  | SP-181116 | Laurent Thiebaut (Nokia) |
| 770039 | Study on ETSUN | FS_ETSUN | S2 | SP-180731 | Laurent Thiebaut (Nokia) |
| 820013 | Stage 2 of ETSUN | ETSUN | S2 | SP-181116 |  |
| 830002 | CT aspects of ETSUN | ETSUN | ct | CP-190192 | Landais, Bruno, Nokia |
| 830055 | CT3 aspects of ETSUN | ETSUN | C3 | CP-190192 | Landais, Bruno, Nokia |
| 830056 | CT4 aspects of ETSUN | ETSUN | C4 | CP-190192 | Landais, Bruno, Nokia |
| 820044 | Access Traffic Steering, Switch and Splitting support in the 5G system architecture | ATSSS |  | SP-181124 | So, Tricci, ZTE |
| 760052 | Study on ATSSS | FS_ATSSS | S2 | SP-180732 | So, Tricci, ZTE |
| 820021 | Stage 2 of ATSSS | ATSSS | S2 | SP-181124 | So, Tricci, ZTE |
| 830016 | CT aspects of ATSSS | ATSSS | ct | CP-190201 | ZHOU, Xingyue (Joy), ZTE |
| 830057 | CT1 aspects of ATSSS | ATSSS | C1 | CP-190201 | ZHOU, Xingyue (Joy), ZTE |
| 830058 | CT3 aspects of ATSSS | ATSSS | C3 | CP-190201 | ZHOU, Xingyue (Joy), ZTE |
| 830059 | CT4 aspects of ATSSS | ATSSS | C4 | CP-190201 | ZHOU, Xingyue (Joy), ZTE |
| 840999 | (IETF) TCP Extensions for Multipath Operation with Multiple Addresses (draft-ietf-mptcp-rfc6824bis) | ATSSS | S2-IETF |  |  |
| 840998 | (IETF) 0-RTT TCP Convert Protocol (draft-ietf-tcpm-converters) | ATSSS | S2-IETF |  |  |
| 810041 | Single radio voice continuity from 5GS to 3G | 5G_SRVCC |  | SP-180737 | Chi Ren, China Unicom |
| 790010 | Study for single radio voice continuity from 5GS to 3G | FS_5G-SRVCC | S2 | SP-180239 | Chi Ren, China Unicom |
| 810007 | Single radio voice continuity from 5GS to 3G | 5G_SRVCC | S2 | SP-180897 | Chi Ren, China Unicom |
| 820008 | Security aspects of single radio voice continuity from 5GS to UTRAN | 5GS_UTRAN_SEC | S3 | SP-181037 | Feng Gao, China Unicom |
| 820069 | RAN aspects of 5G_SRVCC | SRVCC_NR_to_UMTS | R2 | RP-190713 | China Unicom |
| 820169 | Core part: Single Radio Voice Call Continuity from 5G to 3G | SRVCC_NR_to_UMTS-Core | R2 | RP-190713 | China Unicom |
| 830270 | Perf. part: Single Radio Voice Call Continuity from 5G to 3G | SRVCC_NR_to_UMTS-Perf | R4 | RP-190713 | China Unicom |
| 840004 | CT aspect of 5G_SRVCC | 5G_SRVCC | ct | CP-191062 | Chi Ren, China Unicom |
| 840058 | CT1 aspect of 5G_SRVCC | 5G_SRVCC | C1 | CP-191062 | Chi Ren, China Unicom |
| 840059 | CT4 aspect of 5G_SRVCC | 5G_SRVCC | C4 | CP-191062 | Chi Ren, China Unicom |
| 810049 | Remote Identification of Unmanned Aerial Systems | ID_UAS |  | SP-180771 | Eddy HALL, Qualcomm |
| 790005 | Study on Remote Identification of Unmanned Aerial Systems | FS_ID_UAS | S1 | SP-180172 | Eddy Hall; Qualcomm |
| 810013 | Stage 1 of Remote Identification of Unmanned Aerial Systems | ID_UAS | S1 | SP-180771 | Eddy HALL, Qualcomm |
| 800034 | Study on Security Aspects of the 5G Service Based Architecture | FS_SBA_Sec | S3 | SP-180598 | Hans Christian Rudolph, Deutsche Telekom AG |
| 820045 | Enhancements to the Service-Based 5G System Architecture | 5G_eSBA | S2 | SP-181125 |  |
| 790007 | Study on Enhancements to the Service-Based 5G System Architecture | FS_eSBA | S2 | SP-180231 | Tao Sun (China Mobile) |
| 820022 | Stage 2 of 5G_eSBA | 5G_eSBA | S2 | SP-181125 | Tao Sun (China Mobile) |
| 830001 | CT aspects of 5G_eSBA | 5G_eSBA | ct | CP-190191 | Song Yue (China Mobile) |
| 830060 | CT3 aspects of 5G_eSBA | 5G_eSBA | C3 | CP-190191 | Song Yue (China Mobile) |
| 830061 | CT4 aspects of 5G_eSBA | 5G_eSBA | C4 | CP-190191 | Song Yue (China Mobile) |
| 780029 | Study on Enhanced IMS to 5GC Integration | FS_eIMS5G | S2 | SP-180736 | Joul, Chris, T-Mobile USA |
| 840062 | SBA aspects of enhanced IMS to 5GC integration | eIMS5G_SBA | S2 | SP-190181 | T-Mobile USA, Christopher Joul |
| 830028 | Stage 2 of eIMS5G_SBA | eIMS5G_SBA | S2 | SP-190181 | T-Mobile USA, Christopher Joul |
| 840006 | CT aspects of eIMS5G_SBA | eIMS5G_SBA | ct | CP-191065 | de Gregorio, Jesus. Ericsson |
| 840063 | CT3 aspects of eIMS5G_SBA | eIMS5G_SBA | C3 | CP-191065 | de Gregorio, Jesus. Ericsson |
| 840064 | CT4 aspects of eIMS5G_SBA | eIMS5G_SBA | C4 | CP-191065 | de Gregorio, Jesus. Ericsson |
| 830081 | Private Network Support for NG-RAN | NG_RAN_PRN | R3 | RP-190729 | China Telecom |
| 830181 | Core part: Private Network Support for NG-RAN | NG_RAN_PRN-Core | R3 | RP-191563 | China Telecom |
| 810039 | Media Handling Aspects of RAN Delay Budget Reporting in MTSI | E2E_DELAY | S4 | SP-180662 | Oyman, Ozgur, Company: Intel |
| 780022 | Study on E2E_DELAY | FS_E2E_DELAY | S4 | SP-170837 | Oyman, Ozgur, Company: Intel, |
| 810001 | Stage 2 of E2E_DELAY | E2E_DELAY | S4 | SP-180662 | Oyman, Ozgur, Company: Intel |
| 830004 | CT Aspects of E2E_DELAY | E2E_DELAY | ct | CP-190193 | Luetzenkirchen, Thomas, Intel |
| 830066 | CT1 Aspects of E2E_DELAY | E2E_DELAY | C1 | CP-190193 | Luetzenkirchen, Thomas, Intel |
| 830067 | CT3 Aspects of E2E_DELAY | E2E_DELAY | C3 | CP-190193 | Luetzenkirchen, Thomas, Intel |
| 830068 | CT4 Aspects of E2E_DELAY | E2E_DELAY | C4 | CP-190193 | Luetzenkirchen, Thomas, Intel |
| 830069 | Enhancements for Common API Framework for 3GPP Northbound APIs | eCAPIF |  | SP-181137 | Basavaraj Pattan, Samsung |
| 790022 | Stage 2 for eCAPIF | eCAPIF | S6 | SP-181137 | Basavaraj Pattan, Samsung |
| 830022 | Security aspects of eCAPIF | eCAPIF | S3 | SP-1901240 | Rajavelsamy Rajadurai, Samsung |
| 790019 | Usage of CAPIF for xMB API | CAPIF4xMB | S4 | SP-180031 | Thorsten Lohmar, Ericsson LM |
| 840065 | User data interworking, Coexistence and Migration | UDICOM | S2 | SP-190182 | Alessio Casati, Nokia |
| 800055 | Study on UDICOM | FS_UDICoM | S2 | SP-190148 | Susana Sabater (Vodafone |
| 830029 | Stage 2 of UDICOM | UDICOM | S2 | SP-190182 | Alessio Casati, Nokia |
| 840007 | CT aspects of UDICOM | UDICOM | C4 | CP-191066 | Wiehe, Ulrich, Nokia |
| 0 | NR-related Release 16 Features | - |  | - | - |
| 830075 | UE Power Saving in NR | NR_UE_pow_sav | R1 | RP-191607 | CATT |
| 800094 | Study on UE power saving in NR | FS_NR_UE_pow_sav | R1 | RP-181463 | CATT |
| 830175 | Core part: UE Power Saving in NR | NR_UE_pow_sav-Core | R1 | RP-190727 | CATT |
| 830275 | Perf. part: UE Power Saving in NR | NR_UE_pow_sav-Perf | R4 | RP-190727 | CATT |
| 820070 | Integrated access and backhaul for NR | NR_IAB |  | RP-191558 | Qualcomm |
| 750047 | Study onNR_IAB | FS_NR_IAB | R2 | RP-181349 | Qualcomm |
| 820170 | Core part: NR_IAB | NR_IAB-Core | R2 | RP-190712 | Qualcomm |
| 820270 | Perf. Part: NR_IAB | NR_IAB-Perf | R4 | RP-190712 | Qualcomm |
| 830021 | Study on Security for NR_IAB | FS_NR_IAB_Sec | S3 | SP-190106 | Rajavelsamy Rajadurai, Samsung, |
| 800072 | Band combinations for SA NR supplementary uplink (SUL), NSA NR SUL, NSA NR SUL with UL sharing from the UE perspective (ULSUP) | NR_SUL_combos_R16 | R4 | RP-191126 | Huawei |
| 800172 | Core part: Band combinations for SA NR supplementary uplink (SUL), NSA NR SUL, NSA NR SUL with UL sharing from the UE perspective (ULSUP) | NR_SUL_combos_R16-Core | R4 | RP-190714 | Huawei |
| 800272 | Perf. part: Band combinations for SA NR supplementary uplink (SUL), NSA NR SUL, NSA NR SUL with UL sharing from the UE perspective (ULSUP) | NR_SUL_combos_R16-Perf | R4 | RP-190714 | Huawei |
| 800057 | NR intra-band Carrier Aggregation |  |  |  |  |
| 800073 | Rel-16 NR intra band CA for xCC DL/yCC UL including contiguous and non-contiguous spectrum (x>=y) | NR_CA_R16_intra | R4 | RP-191194 | Ericsson |
| 800173 | Core part: Rel-16 NR intra band CA for xCC DL/yCC UL including contiguous and non-contiguous spectrum (x>=y) | NR_CA_R16_intra-Core | R4 | RP-190689 | Ericsson |
| 800273 | Perf. part: Rel-16 NR intra band CA for xCC DL/yCC UL including contiguous and non-contiguous spectrum (x>=y) | NR_CA_R16_intra-Perf | R4 | RP-190689 | Ericsson |
| 800074 | Rel-16 NR inter-band CA/Dual Connectivity for 2 bands DL with x bands UL (x=1, 2) | NR_CADC_R16_2BDL_xBUL | R4 | RP-191565 | ZTE |
| 800174 | Core part: Rel-16 NR inter-band CA/Dual Connectivity for 2 bands DL with x bands UL (x=1, 2) | NR_CADC_R16_2BDL_xBUL-Core | R4 | RP-190494 | ZTE |
| 800274 | Perf. part: Rel-16 NR inter-band CA/Dual Connectivity for 2 bands DL with x bands UL (x=1, 2) | NR_CADC_R16_2BDL_xBUL-Perf | R4 | RP-190494 | ZTE |
| 830099 | NR inter-band Carrier Aggregation |  |  |  |  |
| 830083 | UE Conformance Test Aspects - Rel-16 NR CA and DC; and NR and LTE DC Configurations | NR_CADC_NR_LTE_DC_R16-UEConTest | R5 | RP-190321 | CMCC |
| 830095 | Rel16 NR inter-band Carrier Aggregation for 3 bands DL with 1 band UL | NR_CA_R16_3BDL_1BUL | R4 | RP-191153 | CATT |
| 830195 | Core part: Rel16 NR inter-band Carrier Aggregation for 3 bands DL with 1 band UL | NR_CA_R16_3BDL_1BUL-Core | R4 | RP-190692 | CATT |
| 830295 | Perf. part: Rel16 NR inter-band Carrier Aggregation for 3 bands DL with 1 band UL | NR_CA_R16_3BDL_1BUL-Perf | R4 | RP-190692 | CATT |
| 830096 | Rel-16 NR Inter-band Carrier Aggregation/Dual Connectivity for 3 bands DL with 2 bands UL | NR_CADC_R16_3BDL_2BUL | R4 | RP-191566 | ZTE |
| 830196 | Core part: Rel-16 NR Inter-band Carrier Aggregation/Dual Connectivity for 3 bands DL with 2 bands UL | NR_CADC_R16_3BDL_2BUL-Core | R4 | RP-190693 | ZTE |
| 830296 | Perf. part: Rel-16 NR Inter-band Carrier Aggregation/Dual Connectivity for 3 bands DL with 2 bands UL | NR_CADC_R16_3BDL_2BUL-Perf | R4 | RP-190693 | ZTE |
| 831000 | New bands for New Radio |  | R4 |  | Dish Network |
| 800077 | Band 65 for New Radio | NR_band_n65 | R4 | RP-190360 | Dish Network |
| 800177 | Core part: Band 65 for New Radio | NR_band_n65-Core | R4 | RP-190360 | Dish Network |
| 800277 | Perf. part: Band 65 for New Radio | NR_band_n65-Perf | R4 | RP-190360 | Dish Network |
| 820073 | Introduction of NR band n18 | NR_n18 | R4 | RP-191556 | KDDI |
| 820173 | Core part: Introduction of NR band n18 | NR_n18-Core | R4 | RP-190173 | KDDI |
| 820273 | Perf. part: Introduction of NR band n18 | NR_n18-Perf | R4 | RP-190173 | KDDI |
| 820077 | Introduction of NR band n48 | NR_n48 | R4 | RP-190908 | US Cellular |
| 820177 | Core part: Introduction of NR band n48 | NR_n48-Core | R4 | RP-182865 | US Cellular |
| 830271 | Perf. part: Introduction of NR band n48 | NR_n48-Perf | R4 | RP-190140 | Nokia |
| 830090 | Introduction of NR band n14 | NR_n14 | R4 | RP-190965 | AT&T |
| 830190 | Core part: Introduction of NR band n14 | NR_n14-Core | R4 | RP-190165 | AT&T |
| 830290 | Perf. part: Introduction of NR band n14 | NR_n14-Perf | R4 | RP-190165 | AT&T |
| 830091 | Introduction of NR band n30 | NR_n30 | R4 | RP-190967 | AT&T |
| 830191 | Core part: Introduction of NR band n30 | NR_n30-Core | R4 | RP-190166 | AT&T |
| 830291 | Perf. part: Introduction of NR band n30 | NR_n30-Perf | R4 | RP-190166 | AT&T |
| 830097 | Introduction of NR band n259 | NR_n259 | R4 | RP-191416 | Ericsson |
| 830197 | Core part: Introduction of NR band n259 | NR_n259-Core | R4 | RP-190765 | Ericsson |
| 830297 | Perf. part: Introduction of NR band n259 | NR_n259-Perf | R4 | RP-190765 | Ericsson |
| 840097 | Introduction of NR supplementary DL band n29 | NR_n29 | R4 | RP-191474 | Dish Network |
| 840197 | Core part: Introduction of NR supplementary DL band n29 | NR_n29-Core | R4 | RP-191474 | Dish Network |
| 840297 | Perf. part: Introduction of NR supplementary DL band n29 | NR_n29-Perf | R4 | RP-191474 | Dish Network |
| 831001 | New Radio Band-related topics |  | R4 |  | Nokia |
| 830087 | RF requirements for NR frequency range 1 (FR1) | NR_RF_FR1 | R4 | RP-191114 | Huawei |
| 830187 | Core part: RF requirements for NR frequency range 1 (FR1) | NR_RF_FR1-Core | R4 | RP-191114 | Huawei |
| 830287 | Perf. part: RF requirements for NR frequency range 1 (FR1) | NR_RF_FR1-Perf | R4 | RP-191114 | Huawei |
| 830088 | Add support of NR DL 256QAM for frequency range 2 (FR2) | NR_DL256QAM_FR2 | R4 | RP-190925 | China Telecom |
| 830188 | Core part: Add support of NR DL 256QAM for frequency range 2 (FR2) | NR_DL256QAM_FR2-Core | R4 | RP-190760 | China Telecom |
| 830288 | Perf. part: Add support of NR DL 256QAM for frequency range 2 (FR2) | NR_DL256QAM_FR2-Perf | R4 | RP-190760 | China Telecom |
| 830089 | NR RF requirement enhancements for frequency range 2 (FR2) | NR_RF_FR2_req_enh | R4 | RP-191290 | Nokia |
| 830189 | Core part: NR RF requirement enhancements for frequency range 2 (FR2) | NR_RF_FR2_req_enh-Core | R4 | RP-190761 | Nokia |
| 830092 | Introduction of an NR SUL (supplemental uplink) band with same uplink frequency range as NR band n5 | NR_SUL_UL_n5 | R4 | RP-191116 | Huawei |
| 830192 | Core part: Introduction of an NR SUL (supplemental uplink) band with same uplink frequency range as NR band n5 | NR_SUL_UL_n5-Core | R4 | RP-191116 | Huawei |
| 830292 | Perf. part: Introduction of an NR SUL (supplemental uplink) band with same uplink frequency range as NR band n5 | NR_SUL_UL_n5-Perf | R4 | RP-191116 | Huawei |
| 820074 | Adding wider channel bandwidths in NR band n7 | NR_n7_BW | R4 | RP-190137 | BT |
| 820174 | Core part: Adding wider channel bandwidths in NR band n7 | NR_n7_BW-Core | R4 | RP-190137 | BT |
| 830093 | Addition of wider channel bandwidth in NR band n38 | NR_n38_BW | R4 | RP-191472 | Huawei |
| 830193 | Core part: Addition of wider channel bandwidth in NR band n38 | NR_n38_BW-Core | R4 | RP-190392 | Huawei |
| 830293 | Perf. part: Addition of wider channel bandwidth in NR band n38 | NR_n38_BW-Perf | R4 | RP-190392 | Huawei |
| 820075 | Addition of 30MHz channel bandwidth for NR band n41 | NR_n41_BW | R4 | RP-191569 | KDDI |
| 820175 | Core part: Addition of 30MHz channel bandwidth for NR band n41 | NR_n41_BW-Core | R4 | RP-190174 | KDDI |
| 820275 | Perf. part: Addition of 30MHz channel bandwidth for NR band n41 | NR_n41_BW-Perf | R4 | RP-190174 | KDDI |
| 820076 | Adding wider channel bandwidths in NR band n50 | NR_n50_BW | R4 | RP-182829 | Huawei |
| 820176 | Core part: Adding wider channel bandwidths in NR band n50 | NR_n50_BW-Core | R4 | RP-182829 | Huawei |
| 800085 | Enhancements on MIMO for NR | NR_eMIMO | R1 | RP-182863 | Samsung |
| 800185 | Core part: Enhancements on MIMO for NR | NR_eMIMO-Core | R1 | RP-182863 | Samsung |
| 800285 | Perf. part: Enhancements on MIMO for NR | NR_eMIMO-Perf | R4 | RP-182863 | Samsung |
| 800087 | NR mobility enhancements | NR_Mob_enh | R2 | RP-190489 | Intel |
| 800187 | Core part: NR mobility enhancements | NR_Mob_enh-Core | R2 | RP-190489 | Intel |
| 800287 | Perf. part: NR mobility enhancements | NR_Mob_enh-Perf | R4 | RP-190489 | Intel |
| 820068 | 2-step RACH for NR | NR_2step_RACH | R1 | RP-190711 | ZTE |
| 820168 | Core part: 2-step RACH for NR | NR_2step_RACH-Core | R1 | RP-190711 | ZTE |
| 820268 | Perf. part: 2-step RACH for NR | NR_2step_RACH-Perf | R4 | RP-190711 | ZTE |
| 840091 | SON (Self-Organising Networks) and MDT (Minimization of Drive Tests) support for NR | NR_SON_MDT | R3 | RP-191594 | CMCC |
| 840191 | Core part: SON (Self-Organising Networks) and MDT (Minimization of Drive Tests) support for NR | NR_SON_MDT-Core | R3 | RP-191594 | CMCC |
| 840093 | RRM requirement for CSI-RS based L3 measurement in NR | NR_CSIRS_L3meas | R4 | RP-191580 | CATT |
| 840193 | Core part: RRM requirement for CSI-RS based L3 measurement in NR | NR_CSIRS_L3meas-Core | R4 | RP-191580 | CATT |
| 840293 | Perf. part: RRM requirement for CSI-RS based L3 measurement in NR | NR_CSIRS_L3meas-Perf | R4 | RP-191580 | CATT |
| 840094 | NR performance requirement enhancement | NR_perf_enh | R4 | RP-191587 | China Telecom |
| 840294 | Perf. part: NR performance requirement enhancement | NR_perf_enh-Perf | R4 | RP-191587 | China Telecom |
| 840095 | NR RRM enhancement | NR_RRM_enh | R4 | RP-191601 | Intel |
| 840195 | Core part: NR RRM enhancement | NR_RRM_enh-Core | R4 | RP-191601 | Intel |
| 840295 | Perf. part: NR RRM enhancement | NR_RRM_enh-Perf | R4 | RP-191601 | Intel |
| 840096 | Rel-16 NR inter-band Carrier Aggregation for 4 bands DL with 1 band UL | NR_CA_R16_4BDL_1BUL | R4 | RP-191196 | Ericsson |
| 840196 | Core part: Rel-16 NR inter-band Carrier Aggregation for 4 bands DL with 1 band UL | NR_CA_R16_4BDL_1BUL-Core | R4 | RP-191196 | Ericsson |
| 840296 | Perf. part: Rel-16 NR inter-band Carrier Aggregation for 4 bands DL with 1 band UL | NR_CA_R16_4BDL_1BUL-Perf | R4 | RP-191196 | Ericsson |
| 840098 | Addition of wider channel bandwidth in NR band n1 | NR_n1_BW | R4 | RP-191595 | Ericsson |
| 840198 | Core part: Addition of wider channel bandwidth in NR band n1 | NR_n1_BW-Core | R4 | RP-191595 | Ericsson |
| 840298 | Perf. part: Addition of wider channel bandwidth in NR band n1 | NR_n1_BW-Perf | R4 | RP-191595 | Ericsson |
| 840099 | Addition of channel bandwidths in Band n77 and n78 | NR_n77_n78_BW | R4 | RP-191606 | Huawei |
| 840199 | Core part: Addition of channel bandwidths in Band n77 and n78 | NR_n77_n78_BW-Core | R4 | RP-191606 | Huawei |
| 841000 | Dual Connectivity (EN-DC) with 3 bands DL and 3 bands UL | DC_R16_LTE_NR_3DL3UL | R4 | RP-191598 | ZTE |
| 841100 | Core part: Dual Connectivity (EN-DC) with 3 bands DL and 3 bands UL | DC_R16_LTE_NR_3DL3UL-Core | R4 | RP-191598 | ZTE |
| 841200 | Perf. part: Dual Connectivity (EN-DC) with 3 bands DL and 3 bands UL | DC_R16_LTE_NR_3DL3UL-Perf | R4 | RP-191598 | ZTE |
| 841001 | Introduction of NR FDD bands with variable duplex and corresponding framework | NR_FDD_bands_varduplex | R4 | RP-191567 | Huawei |
| 841101 | Core part: Introduction of NR FDD bands with variable duplex and corresponding framework | NR_FDD_bands_varduplex-Core | R4 | RP-191567 | Huawei |
| 841002 | Common RF requirement configured output power for EN-DC with 3 uplink CC and 2 different bands (2CC LTE, 1CC NR FR1) | DC_Pcmax_3UL_CC | R4 | RP-191604 | Huawei |
| 841102 | Core part: Common RF requirement configured output power for EN-DC with 3 uplink CC and 2 different bands (2CC LTE, 1CC NR FR1) | DC_Pcmax_3UL_CC-Core | R4 | RP-191604 | Huawei |
| 800082 | Cross Link Interference (CLI) handling and Remote Interference Management (RIM) for NR | NR_CLI_RIM | R1 | RP-191546 | LG Electronics |
| 800182 | Core part: Cross Link Interference (CLI) handling and Remote Interference Management (RIM) for NR | NR_CLI_RIM-Core | R1 | RP-190700 | LG Electronics |
| 800282 | Perf. part: Cross Link Interference (CLI) handling and Remote Interference Management (RIM) for NR | NR_CLI_RIM-Perf | R4 | RP-190700 | LG Electronics |
| 0 | Release 16 Features impacting both LTE and NR | - |  | - | - |
| 800088 | LTE-NR & NR-NR Dual Connectivity and NR Carrier Aggregation enhancements | LTE_NR_DC_CA_enh | R2 | RP-191600 | Ericsson |
| 800188 | Core part: LTE-NR & NR-NR Dual Connectivity and NR CA enhancements | LTE_NR_DC_CA_enh-Core | R2 | RP-190452 | Ericsson |
| 800288 | Perf. part: LTE-NR & NR-NR Dual Connectivity and NR CA enhancements | LTE_NR_DC_CA_enh-Perf | R4 | RP-190452 | Ericsson |
| 820079 | High power UE (power class 2) for EN-DC (1 LTE TDD band + 1 NR TDD band) | ENDC_UE_PC2_TDD_TDD | R4 | RP-190315 | CMCC |
| 820061 | Study on high power UE (power class 2) for EN-DC (1 LTE FDD band + 1 NR TDD band) | FS_ENDC_UE_PC2_FDD_TDD | R4 | RP-190167 | China Unicom |
| 820179 | Core part: High power UE (power class 2) for EN-DC (1 LTE TDD band + 1 NR TDD band) | ENDC_UE_PC2_TDD_TDD-Core | R4 | RP-190315 | CMCC |
| 820279 | Perf. part: Power class 2 UE for EN-DC (1 LTE TDD band + 1 NR TDD band) | ENDC_UE_PC2_TDD_TDD-Perf | R4 | RP-190315 | CMCC |
| 790054 | eNB(s) Architecture Evolution for E-UTRAN and NG-RAN | LTE_NR_arch_evo | R3 | RP-180531 | China Unicom |
| 790154 | Core part: eNB(s) Architecture Evolution for E-UTRAN and NG-RAN | LTE_NR_arch_evo-Core | R3 | RP-181375 | China Unicom |
| 820080 | LTE/NR spectrum sharing in Band 41/n41 frequency range | NR_n41_LTE_41_coex | R4 | RP-191588 | KDDI |
| 820180 | Core part: LTE/NR spectrum sharing in Band 41/n41 frequency range | NR_n41_LTE_41_coex-Core | R4 | RP-190175 | KDDI |
| 800078 | 29 dBm UE Power Class for LTE band 41 and NR Band n41 | LTE_NR_B41_Bn41_PC29dBm | R4 | RP-190502 | Sprint |
| 800178 | Core part: 29 dBm UE Power Class for LTE band 41 and NR Band n41 | LTE_NR_B41_Bn41_PC29dBm-Core | R4 | RP-190502 | Sprint |
| 800058 | Dual Connectivity (EN-DC) |  | R4 |  | Ericsson |
| 800066 | Dual Connectivity (EN-DC) of 1 LTE band (1UL/1DL) and 1 NR band (1DL/1UL) | DC_R16_1BLTE_1BNR_2DL2UL | R4 | RP-191568 | NTT DOCOMO |
| 800166 | Core part: Dual Connectivity (EN-DC) of 1 LTE band (1UL/1DL) and 1 NR band (1DL/1UL) | DC_R16_1BLTE_1BNR_2DL2UL-Core | R4 | RP-190705 | NTT DOCOMO |
| 800266 | Perf. part: Dual Connectivity (EN-DC) of 1 LTE band (1UL/1DL) and 1 NR band (1DL/1UL) | DC_R16_1BLTE_1BNR_2DL2UL-Perf | R4 | RP-190705 | NTT DOCOMO |
| 800067 | Dual Connectivity (EN-DC) of 2 bands LTE inter-band CA (2DL/1UL) and 1 NR band (1DL/1UL) | DC_R16_2BLTE_1BNR_3DL2UL | R4 | RP-191124 | Huawei |
| 800167 | Core part: Dual Connectivity (EN-DC) of 2 bands LTE inter-band CA (2DL/1UL) and 1 NR band (1DL/1UL) | DC_R16_2BLTE_1BNR_3DL2UL-Core | R4 | RP-190379 | Huawei |
| 800267 | Perf. part: Dual Connectivity (EN-DC) of 2 bands LTE inter-band CA (2DL/1UL) and 1 NR band (1DL/1UL) | DC_R16_2BLTE_1BNR_3DL2UL-Perf | R4 | RP-190379 | Huawei |
| 800068 | Dual Connectivity (EN-DC) of 3 bands LTE inter-band CA (3DL/1UL) and 1 NR band (1DL/1UL) | DC_R16_3BLTE_1BNR_4DL2UL | R4 | RP-191195 | Ericsson |
| 800168 | Core part: Dual Connectivity (EN-DC) of 3 bands LTE inter-band CA (3DL/1UL) and 1 NR band (1DL/1UL) | DC_R16_3BLTE_1BNR_4DL2UL-Core | R4 | RP-190159 | Ericsson |
| 800268 | Perf. part: Dual Connectivity (EN-DC) of 3 bands LTE inter-band CA (3DL/1UL) and 1 NR band (1DL/1UL) | DC_R16_3BLTE_1BNR_4DL2UL-Perf | R4 | RP-190159 | Ericsson |
| 800069 | Dual Connectivity (EN-DC) of 4 bands LTE inter-band CA (4DL/1UL) and 1 NR band (1DL/1UL) | DC_R16_4BLTE_1BNR_5DL2UL | R4 | RP-191205 | Nokia |
| 800169 | Core part: Dual Connectivity (EN-DC) of 4 bands LTE inter-band CA (4DL/1UL) and 1 NR band (1DL/1UL) | DC_R16_4BLTE_1BNR_5DL2UL-Core | R4 | RP-190294 | Nokia |
| 800269 | Perf. part: Dual Connectivity (EN-DC) of 4 bands LTE inter-band CA (4DL/1UL) and 1 NR band (1DL/1UL) | DC_R16_4BLTE_1BNR_5DL2UL-Perf | R4 | RP-190294 | Nokia |
| 800070 | Dual Connectivity (EN-DC) of 5 bands LTE inter-band CA (5DL/1UL) and 1 NR band (1DL/1UL) | DC_R16_5BLTE_1BNR_6DL2UL | R4 | RP-180997 | Samsung |
| 800170 | Core part: Dual Connectivity (EN-DC) of 5 bands LTE inter-band CA (5DL/1UL) and 1 NR band (1DL/1UL) | DC_R16_5BLTE_1BNR_6DL2UL-Core | R4 | RP-180997 | Samsung |
| 800270 | Perf. part: Dual Connectivity (EN-DC) of 5 bands LTE inter-band CA (5DL/1UL) and 1 NR band (1DL/1UL) | DC_R16_5BLTE_1BNR_6DL2UL-Perf | R4 | RP-180997 | Samsung |
| 800071 | Dual Connectivity (EN-DC) of x bands (x=1, 2, 3, 4) LTE inter-band CA (xDL/1UL) and 2 bands NR inter-band CA (2DL/1UL) | DC_R16_xBLTE_2BNR_yDL2UL | R4 | RP-190949 | LG Electronics |
| 800171 | Core part: Dual Connectivity (EN-DC) of x bands (x=2, 3, 4) LTE inter-band CA (xDL/1UL) and 2 bands NR inter-band CA (2DL/1UL) | DC_R16_xBLTE_2BNR_yDL2UL-Core | R4 | RP-190151 | LG Electronics |
| 800271 | Perf. part: Dual Connectivity (EN-DC) of x bands (x=2, 3, 4) LTE inter-band CA (xDL/1UL) and 2 bands NR inter-band CA (2DL/1UL) | DC_R16_xBLTE_2BNR_yDL2UL-Perf | R4 | RP-190151 | LG Electronics |
| 0 | LTE-related Release 16 Features | - |  | - | - |
| 790057 | E-UTRA 2.4 GHz TDD Band for US | LTE_TDD_2400_US | R4 | RP-182405 | Globalstar |
| 790157 | Core part: E-UTRA 2.4 GHz TDD Band for US | LTE_TDD_2400_US-Core | R4 | RP-182405 | Globalstar |
| 790257 | Perf. part: E-UTRA 2.4 GHz TDD Band for US | LTE_TDD_2400_US-Perf | R4 | RP-182405 | Globalstar |
| 820063 | UE Conformance Test Aspects - E-UTRA 2.4 GHz TDD Band for US (Band 53) | LTE_TDD_2400_US-UEConTest | R5 | RP-182406 | Globalstar |
| 800059 | LTE intra-band Carrier Aggregation (CA) |  | R4 |  | Qualcomm |
| 800060 | Rel-16 LTE intra-band CA for x CC DL/y CC UL including contiguous and non-contiguous spectrum (x>=y) | LTE_CA_R16_intra | R4 | RP-191193 | Ericsson |
| 800160 | Core part: Rel-16 LTE intra-band CA for x CC DL/y CC UL including contiguous and non-contiguous spectrum (x>=y) | LTE_CA_R16_intra-Core | R4 | RP-190157 | Ericsson |
| 800260 | Perf. part: Rel-16 LTE intra-band CA for x CC DL/y CC UL including contiguous and non-contiguous spectrum (x>=y) | LTE_CA_R16_intra-Perf | R4 | RP-190157 | Ericsson |
| 800061 | Rel-16 LTE inter-band CA for 2 bands DL with 1 band UL | LTE_CA_R16_2BDL_1BUL | R4 | RP-191085 | Qualcomm |
| 800161 | Core part: Rel-16 LTE inter-band CA for 2 bands DL with 1 band UL | LTE_CA_R16_2BDL_1BUL-Core | R4 | RP-191085 | Qualcomm |
| 800261 | Perf. part: Rel-16 LTE inter-band CA for 2 bands DL with 1 band UL | LTE_CA_R16_2BDL_1BUL-Perf | R4 | RP-191085 | Qualcomm |
| 800062 | Rel-16 LTE inter-band CA for 3 bands DL with 1 band UL | LTE_CA_R16_3BDL_1BUL | R4 | RP-191120 | Huawei |
| 800162 | Core part: Rel-16 LTE inter-band CA for 3 bands DL with 1 band UL | LTE_CA_R16_3BDL_1BUL-Core | R4 | RP-191120 | Huawei |
| 800262 | Perf. part: Rel-16 LTE inter-band CA for 3 bands DL with 1 band UL | LTE_CA_R16_3BDL_1BUL-Perf | R4 | RP-191120 | Huawei |
| 800063 | Rel-16 LTE inter-band CA for x bands DL (x=4, 5) with 1 band UL | LTE_CA_R16_xBDL_1BUL | R4 | RP-190906 | Nokia |
| 800163 | Core part: Rel-16 LTE inter-band CA for x bands DL (x=4, 5) with 1 band UL | LTE_CA_R16_xBDL_1BUL-Core | R4 | RP-190138 | Nokia |
| 800263 | Perf. part: Rel-16 LTE inter-band CA for x bands DL (x=4, 5) with 1 band UL | LTE_CA_R16_xBDL_1BUL-Perf | R4 | RP-190138 | Nokia |
| 800064 | Rel-16 LTE inter-band CA for 2 bands DL with 2 band UL | LTE_CA_R16_2BDL_2BUL | R4 | RP-190377 | Huawei |
| 800164 | Core part: Rel-16 LTE inter-band CA for 2 bands DL with 2 band UL | LTE_CA_R16_2BDL_2BUL-Core | R4 | RP-190377 | Huawei |
| 800264 | Perf. part: Rel-16 LTE inter-band CA for 2 bands DL with 2 band UL | LTE_CA_R16_2BDL_2BUL-Perf | R4 | RP-190377 | Huawei |
| 800065 | Rel-16 LTE inter-band CA for x bands DL (x= 3, 4, 5) with 2 band UL | LTE_CA_R16_xBDL_2BUL | R4 | RP-190946 | LG Electronics |
| 800165 | Core part: Rel-16 LTE inter-band CA for x bands DL (x= 3, 4, 5) with 2 band UL | LTE_CA_R16_xBDL_2BUL-Core | R4 | RP-190150 | LG Electronics |
| 800265 | Perf. part: Rel-16 LTE inter-band CA for x bands DL (x= 3, 4, 5) with 2 band UL | LTE_CA_R16_xBDL_2BUL-Perf | R4 | RP-190150 | LG Electronics |
| 810061 | UE Conformance Test Aspects – Rel-16 LTE CA configurations | LTE_CA_R16-UEConTest | R5 | RP-182648 | Ericsson |
| 830076 | LTE-based 5G terrestrial broadcast | LTE_terr_bcast | R1 | RP-190732 | Qualcomm |
| 800091 | Study on LTE-based 5G terrestrial broadcast | FS_LTE_terr_bcast | R1 | RP-181342 | Qualcomm |
| 830176 | Core part: LTE-based 5G terrestrial broadcast | LTE_terr_bcast-Core | R1 | RP-190732 | Qualcomm |
| 830276 | Perf. part: LTE-based 5G terrestrial broadcast | LTE_terr_bcast-Perf | R4 | RP-190732 | Qualcomm |
| 830094 | Power class 2 UE for LTE bands 31 and 72 | LTE_PC2_B31_B72 | R4 | RP-191285 | Nokia |
| 830194 | Core part: Power class 2 UE for LTE bands 31 and 72 | LTE_PC2_B31_B72-Core | R4 | RP-190665 | Nokia |
| 830294 | Perf. part: Power class 2 UE for LTE bands 31 and 72 | LTE_PC2_B31_B72-Perf | R4 | RP-190665 | Nokia |
| 800056 | Additional LTE bands |  | R4 |  | Ericsson |
| 800075 | Additional LTE bands for UE category M1 and/or NB1 in Rel-16 | LTE_bands_R16_M1_NB1 | R4 | RP-190332 | Ericsson |
| 800175 | Core part: Additional LTE bands for UE category M1 and/or NB1 in Rel-16 | LTE_bands_R16_M1_NB1-Core | R4 | RP-190332 | Ericsson |
| 800275 | Perf. part: Additional LTE bands for UE category M1 and/or NB1 in Rel-16 | LTE_bands_R16_M1_NB1-Perf | R4 | RP-190332 | Ericsson |
| 800076 | Additional LTE bands for UE category M2 and/or NB2 in Rel-16 | LTE_bands_R16_M2_NB2 | R4 | RP-190333 | Ericsson |
| 800176 | Core part: Additional LTE bands for UE category M2 and/or NB2 in Rel-16 | LTE_bands_R16_M2_NB2-Core | R4 | RP-190333 | Ericsson |
| 800079 | Further performance enhancement for LTE in high speed scenario | LTE_high_speed_enh2 | R4 | RP-181482 | NTT DOCOMO |
| 800179 | Core part: Further performance enhancement for LTE in high speed scenario | LTE_high_speed_enh2-Core | R4 | RP-181482 | NTT DOCOMO |
| 800279 | Perf. part: Further performance enhancement for LTE in high speed scenario | LTE_high_speed_enh2-Perf | R4 | RP-181482 | NTT DOCOMO |
| 800086 | DL MIMO efficiency enhancements for LTE | LTE_DL_MIMO_EE | R1 | RP-182901 | Huawei |
| 800186 | Core part: DL MIMO efficiency enhancements for LTE | LTE_DL_MIMO_EE-Core | R1 | RP-182901 | Huawei |
| 800286 | Perf. part: DL MIMO efficiency enhancements for LTE | LTE_DL_MIMO_EE-Perf | R4 | RP-182901 | Huawei |
| 820071 | Transfer of Iuant interface specifications from 25-series to 37-series | Iuant_transfer | R3 | RP-190160 | Huawei |
| 820171 | Core part: Transfer of Iuant interface specifications from 25-series to 37-series | Iuant_transfer-Core | R3 | RP-190160 | Huawei |
| 820072 | Direct data forwarding between NG-RAN and E-UTRAN nodes for inter-system mobility | Direct_data_fw_NR | R3 | RP-182886 | Ericsson |
| 820172 | Core part: Direct data forwarding between NG-RAN and E-UTRAN nodes for inter-system mobility | Direct_data_fw_NR-Core | R3 | RP-182886 | Ericsson |
| 800089 | Even further mobility enhancement in E-UTRAN | LTE_feMob | R2 | RP-190921 | China Telecom |
| 800189 | Core part: Even further mobility enhancement in E-UTRAN | LTE_feMob-Core | R2 | RP-190272 | China Telecom |
| 800289 | Perf. part: Even further mobility enhancement in E-UTRAN | LTE_feMob-Perf | R4 | RP-190272 | China Telecom |
| 820078 | 410 - 430 MHz E-UTRA FDD Band(s) for LTE PPDR and PMR/PAMR in Europe | LTE410_Europe_PPDR | R4 | RP-191287 | Nokia |
| 820178 | Core part: 410 - 430 MHz E-UTRA FDD Band(s) for LTE PPDR and PMR/PAMR in Europe | LTE410_Europe_PPDR-Core | R4 | RP-182898 | Nokia |
| 820278 | Perf. part: 410 - 430 MHz E-UTRA FDD Band(s) for LTE PPDR and PMR/PAMR in Europe | LTE410_Europe_PPDR-Perf | R4 | RP-182898 | Nokia |
| 0 | All other Release 16 Features | - |  | - | - |
| 830086 | Introduction of GSM, UTRA, E-UTRA and NR capability set(s) (CS(s)) to the multi-standard radio (MSR) specifications | MSR_GSM_UTRA_LTE_NR | R4 | RP-190642 | Ericsson |
| 830186 | Core part: Introduction of GSM, UTRA, E-UTRA and NR capability set(s) (CS(s)) to the multi-standard radio (MSR) specifications | MSR_GSM_UTRA_LTE_NR-Core | R4 | RP-190642 | Ericsson |
| 830286 | Perf. part: Introduction of GSM, UTRA, E-UTRA and NR capability set(s) (CS(s)) to the multi-standard radio (MSR) specifications | MSR_GSM_UTRA_LTE_NR-Perf | R4 | RP-190642 | Ericsson |
| 840066 | Volume Based Charging Aspects for VoLTE | VBCLTE | S5 | SP-180813 | Chen, Ai, China Mobile |
| 810021 | Stage 2 of VBCLTE | VBCLTE | S5 | SP-180813 | Chen, Ai, China Mobile |
| 840012 | CT aspects of VBCLTE | VBCLTE | ct | CP-191110 | Huang Zhenning (China Mobile) |
| 840067 | CT1 aspects of VBCLTE | VBCLTE | C1 | CP-191110 | Huang Zhenning (China Mobile) |
| 840068 | CT3 aspects of VBCLTE | VBCLTE | C3 | CP-191110 | Huang Zhenning (China Mobile) |
| 790003 | Enhancements to IMS for new real time communication services | enIMS | S1 | SP-180137 | China Mobile, Hui Cai |
| 820028 | Enhancement of 5G PCC related services | en5GPccSer | C3 | CP-183246 | Xiaoyun Zhou, Huawei |
| 810023 | Energy efficiency of 5G | EE_5G | S5 | SP-180819 | CORNILY, Jean-Michel, ORANGE |
| 820039 | Signalling Improvements for Network Efficiency in 5GS | SINE_5G | C1 | CP-190142 | Shu, Lin |
| 790001 | New Services and Markets Technology Enablers – Phase 2 | SMARTER_Ph2 | S1 | SP-180589 | Li, Alice, Vodafone |
| 780055 | 5G Voice Service Continuity | 5GVSC | S1 | SP-171077 | Qun Wei, China Unicom |
| 790015 | Security Assurance Specification for 5G | SCAS_5G | S3 | SP-180689 | Marcus Wong, Huawei Technologies |
| 840069 | 5GS Transfer of Policies for Background Data Transfer | xBDT | S2 | SP-180509 | Takeshi Usui KDDI |
| 800030 | Stage 2 of xBDT | xBDT | S2 | SP-180509 | Takeshi Usui KDDI |
| 840014 | CT aspects of xBDT | xBDT | ct | CP-191113 | Yan Xiaojian, ZTE |
| 840070 | CT1 aspects of xBDT | xBDT | C1 | CP-191113 | Yan Xiaojian, ZTE |
| 840071 | CT3 aspects of xBDT | xBDT | C3 | CP-191113 | Yan Xiaojian, ZTE |
| 770020 | Service Interactivity | SerInter | S4 | SP-170796 | Lo, Charles, Qualcomm |
| 770024 | EVS Codec Extension for Immersive Voice and Audio Services | IVAS_Codec | S4 | SP-170611 | Gibbs, Jon, Huawei Technologies Co Ltd |
| 800002 | Alternative EVS implementation using updated fixed-point basic operators | Alt_FX_EVS | S4 | SP-180286 | Pawate, Raj, Cadence Design Systems Inc |
| 800046 | Shared Data Handling on Nudm and Nudr | Shared_Data | C4 | CP-181136 | Wiehe, Ulrich, Nokia |
| 800009 | Policy delivery to UE for background data transfer | PDBDT | S1 | SP-180324 | Takatsugu Kito, KDDI |
| 790004 | QoS Monitoring | QoS_MON | S1 | SP-180141 | Amar Deol |
| 800005 | Inter-RAT Mobility requirements for real time service | MOBRT | S1 | SP-180592 | Yang Xu, Oppo |
| 760058 | Management of QoE measurement collection | QOED | S5 | SP-181069 | Petersen, Robert, Ericsson |
| 760059 | Study on network policy management for mobile networks based on NFV scenarios | FS_NETPOL | S5 | SP-170593 | China Mobile Jinglei Liu |
| 810025 | Network policy management for mobile networks based on NFV scenarios | NETPOL | S5 | SP-180821 | China Mobile Hao Zhang |
| 810027 | Intent driven management service for mobile network | IDMS_MN | S5 | SP-180899 | Zou Lan, Huawei |
| 810024 | OAM aspects of LTE and WLAN integration | OAM_LTE_WLAN | S5 | SP-180820 | Yizhi Yao, Intel Corporation |
| 810026 | Methodology for 5G management specifications | METHOGY | S5 | SP-180822 | Groenendijk, Jan, Ericsson |
| 820030 | Nchf Online and Offline charging services | OFSBI_CH | S5 | SP-181066 | Zhu Lei, Huawei |
| 820031 | Charging Enhancement of 5GC interworking with EPC | 5GIEPC_CH | S5 | SP-181067 | Chen Shan, Huawei |
| 820032 | NRM enhancements | eNRM | S5 | SP-190140 | Jing Ping, Nokia |
| 820033 | Network Exposure Charging in 5G System Architecture | 5GS_Ph1_NEFCH | S5 | SP-181070 | Tornkvist, Robert, Ericsson |
| 820034 | Charging AMF in 5G System Architecture Phase 1 | 5GS_Ph1_AMFCH | S5 | SP-181071 | Gardella, Maryse, Nokia |
| 820035 | Discovery of management services in 5G | 5GDMS | S5 | SP-181072 | Attila Horvat, Huawei Technologies Sweden AB |
| 820036 | Trace Management in the context of Services Based Management Architecture | TM_SBMA | S5 | SP-181073 | Andrianov, Anatoly, Nokia |
| 820005 | Lawful Interception Rel-16 | LI16 | S3LI | SP-181210 | Alex Leadbeater, BT |
| 820038 | IMS Stage-3 IETF Protocol Alignment | IMSProtoc16 | C1 | CP-183084 | Leis, Peter, Nokia |
| 820041 | Stage-3 5GS NAS protocol development | 5GProtoc16 | C1 | CP-183087 | Sedlacek, Ivo, Ericsson |
| 820049 | Stage-3 5GS NAS protocol development general aspects | 5GProtoc16 | C1 | CP-183087 | Sedlacek, Ivo, Ericsson |
| 820050 | Stage-3 5GS NAS protocol development for support for non-3GPP accesses | 5GProtoc16-non3GPP | C1 | CP-183087 | Sedlacek, Ivo, Ericsson |
| 820042 | Stage-3 SAE Protocol Development for Rel16 | SAES16 | C1 | CP-183088 | Aghili, Behrouz, InterDigital Communications |
| 820046 | Stage-3 SAE Protocol Development for Rel16 general aspects | SAES16 | C1 | CP-183088 | Aghili, Behrouz, InterDigital Communications |
| 820047 | Stage-3 SAE Protocol Development for Rel16 for CS Fallback in EPS | SAES16-CSFB | C1 | CP-183088 | Aghili, Behrouz, InterDigital Communications |
| 820048 | Stage-3 SAE Protocol Development Rel16 for support for non-3GPP accesses | SAES16-non3GPP | C1 | CP-183088 | Aghili, Behrouz, InterDigital Communications |
| 840017 | Reliable Data Service Serialization Indication | RDSSI | S2 | SP-190446 | Starsinic, Michael, Convida Wireless LLC |
| 840018 | S6b Optional for ePDG connected to 5GS | 5GS_S6b_Optional | S2 | SP-190447 | Irfan Ali, Cisco |
| 840002 | Service Based Interface Protocol Improvements | SBIProtoc16 | ct | CP-191060 | Yue Song, China Mobile |
| 840052 | CT3 aspects of SBIProtoc16 | SBIProtoc16 | C3 | CP-191060 | Yue Song, China Mobile |
| 840053 | CT4 aspects of SBIProtoc16 | SBIProtoc16 | C4 | CP-191060 | Yue Song, China Mobile |
| 840008 | Withdrawal of TS 24.323 from Rel-11, Rel-12, Rel-13 | ISAT-MO-WITHDRAW | C1 | CP-191151 | Dawes, Peter, Vodafone |
| 840051 | Lawful Interception Report Rel-16 | LIR16 | S3LI | SP-190342 | Nag Rao, Nokia |
| 770050 | (Small) Technical Enhancements and Improvements for Rel-16 | TEI16 |  |  |  |
| 800100 | (IETF) The Transport Layer Security (TLS) Protocol Version 1.3 (RFC 8446) | TEI16 | S3-IETF |  |  |

Annex A1:
Template for contributing

Editor's note: To contribute to this document, please copy-paste the following template into a document to be submitted to the TSG plenary as "Work Item Summary". This will be further endorsed by the plenary.
Each summary is expected to be one page long maximum. 
The target audience of this document are readers inside and outside the 3GPP community. Contributions should be written as to be understandable by non-specialist. 
The summary should reflect the actual outcome of the Feature - and not the initial intentions.

---

Summary based on the input provided by [company name] in [tdoc numbed].

| Unique_ID | Name | Acronym | WG | WID | WI_rapporteur_name |
| --- | --- | --- | --- | --- | --- |
| AABBCC | Title here | XXX | e.g. S4 | xP-19XXXX | Firstname Familyname, Company |

1	Introduction

<guidance:
- summarize in one to a few sentences what was achieved by this WI and how: what problems it solves, what new functionality it adds, etc.;

- remember that only what ended up in the specs is of interest;

- clarify what is the relation of this WI to other WIs (e.g. is this an enhancement of a former WI? Is there a related WI?) and what was the status before this WI

2	Description

<guidance:

- Explain here in a couple of paragraphs the main impacts on the system, mostly: the new and/or impacted entities, and the big lines of the system's behaviour;

- remember that only what ended up in the specs shall be covered here;

- this it not intended to be a copy and paste from the specifications, the WID or status report;

- the reader shall understand how the key functionalities are working without running too far into details;

- use figures, tables, references to illlustrate and  facilitate the understanding;

- remember that you do not write the WI summary for people who contributed to this WI so make sure that you use terminology and abbreviations that can be understood by any engineer or make sure you explain them;

- When the usual Stage 1/Stage 2/Stage 3 process is not followed -and in particular if a full stage is missing- explain why.

- make the following test: give this summary to a colleague who was not at all involved in this topic and let this colleague explain in her/his own words how your WI works after reading the summary>

3	References

<guidance: It is recommended to add some references where the interested reader can find further documentation, e.g.

- new/impacted TRs and TSs (if any)

- last status report (if any)

- main CRs

NOTE: Please provide references that can be found easily, e.g. indicating Tdoc numbers.>

[X] TS XX.XXX, Spec title

Annex B:
Process to get further information

# B.1	General

Since the present document is limited to provide an overview of each Feature, this chapter explains how to get additional information, in particular how to retrieve all the Specifications (TSs) and Reports (TRs) as well as all the CRs which relate to a given Work Item.

The Unique Identifier (UID) is the key to get additional information on a given Work Item. It can be found in the table located just below the clause's header. The table has the following format:

Table B-1: table format

| Unique Identifier (UID) | Name | Acronym | Outline Level (1=Feature, 2=Building Block, 3=Work Task) | Responsible Working Group | Work Item Description |
| --- | --- | --- | --- | --- | --- |

For readability reasons, the table headers are omitted in the continuation of the present document.

For instance, for the "Mission Critical Push to Talk over LTE Realignment", the table has to be understood as:

Table B-2: Example of table at the introduction of each Feature

| Unique Identifier (UID) | Name | Acronym | Outline Level | Responsible Working Group | Work Item Description |
| --- | --- | --- | --- | --- | --- |
| 700029 | Mission Critical Push to Talk over LTE Realignment | MCImp-MCPTTR | 2 | S1 | SP-150821 |

Thus, the UID for this Work Item is 700029.

Two methods are now possible to retrieve more information on a given feature: the "Step by step method" and the "Direct method". The "direct method" is faster but implies to know the hierarchical structure of the Work Items. The "step by step method" is slower but is easier to use, in particular when the hierarchical structure is unknown.

For instance, for retrieving all the CRs that relate to "Enhancements for Mission Critical Push To Talk", the search has to be done on UID 740022 but also potentially on its children Work Items (UID 720056, 740023 and 740024).

Table B-3: Example of a hierarchical structure and its consequences on the search procedure

| 740022 | Enhancements for Mission Critical Push To Talk | MCImp-eMCPTT |  | SP-160490 |
| --- | --- | --- | --- | --- |
| 720056 | Stage 2 of Enhancements for Mission Critical Push To Talk | MCImp-eMCPTT | S6 | SP-160490 |
| 740023 | Stage 3 of Enhancements for Mission Critical Push To Talk | MCImp-eMCPTT-CT | CT | CP-160824 |
| 740024 | CT1 aspects of Enhancements for Mission Critical Push To Talk | MCImp-eMCPTT-CT | C1 | CP-160824 |

These two methods are described in the following clauses.

# B.2	Direct method

The links below lead to the pages containing respectively all the Specifications and all the Change Requests (CRs) linked to a given Feature:

https://portal.3gpp.org/Specifications.aspx?q=1&WiUid=[UID]

https://portal.3gpp.org/ChangeRequests.aspx?q=1&workitem=[UID]

where "[UID]" has to be preplaced by the UID value.

Using the example provided in the table 4.1-3, the specification linked to "Stage 2 of Enhancements for Mission Critical Push To Talk" can be found in:

https://portal.3gpp.org/Specifications.aspx?q=1&WiUid=720056

And all the related Change Requests are listed in:

https://portal.3gpp.org/ChangeRequests.aspx?q=1&workitem=720056

# B.3	Step by step method

This method is to be used when the hierarchical structure is not known or when the "direct method" above does not show the expected results.

In this case, the 3GPP Ultimate web site has to be used:

https://portal.3gpp.org

As a preliminary step, it is essential that the "Customized Selection" is set to "All TSGs" (otherwise, a filter would be applied).

Figure B.3-1: Selecting "All TSGs" in "Customized Selection" as to remove any potential filter on the Search

Then select the "Work Plan" tab (upper red arrow in the figure below).

Figure B.3-2: Using the 3GU Portal to retrieve more information on a given Feature

Then the search might be performed by either typing the Acronym (as shown by the left red arrow on the figure above, using the example "MCPTT"), or by the name or UID (right box) then by clicking on the "Search" button. Watch the "Granularity (Level)" field, which is a filter to return only the Items which level is specified here.

In the results, the icon depicting some binoculars has to be hit (lower right red arrow on the figure above).

This will lead to the page shown in the figure below:

Figure B.3-3: Window resulting from a "Search"

On this window, the "Related" tab has to be clicked, as pointed by the red arrow in the figure above. This will lead to the window depicted in the figure below.

Figure B.3-4: "Related" tab in a Work Item search, with links to all related Specifications and Change Requests

The two links pointed by the red arrows in the figure above lead to the pages containing respectively all the Specifications and all the Change Requests (CRs) linked to this Feature.

Annex C:
Change history

| Change history |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Date | Meeting | TDoc | CR | Rev | Cat | Subject/Comment | New version |
| 2019-09 | TSG#85 |  |  |  |  | Initital draft, call for contributions (Note: in TSG CT and RAN, up to meeting 88e included, summaries were minuted as "Noted" instead of "Endorsed") | 0.1.0 |
| 2019-12 | TSG#86 |  |  |  |  | Summaries received: RP-190144 : Transfer of Iuant interface specifications from 25-series to 37-series RP-191816 Introduction of GSM, UTRA, E-UTRA and NR capability set(s) (CS(s)) to the multi-standard radio (MSR) specifications RP-192162 Direct data forwarding between NG-RAN and E-UTRAN nodes for inter-system mobility SP-190855 Charging Enhancement of 5GC interworking with EPC SP-190865 Coverage and Handoff Enhancements for Multimedia SP-191223 Summary for 5G_eLCS CATT SP-191229 Summary for RDSSI Convida Wireless LLC SP-191247 Summary for 'Service Interactivity' Qualcomm Incorporated SP-191252 Summary for 'OAM aspects of LTE and WLAN integration' Intel SP-191277 Summary for Volume Based Charging Aspects for VoLTE China Mobile SP-191328 Summary for 'EVS Float Conformance for Non Bit-exact EVS_FCNBE Rapporteur (Intel) | 0.2.0 |
| 2020-01 | SA1#89 |  |  |  |  | Summaries received from RAN#86: RP-192721 Summary for: Further performance enhancement for LTE in high speed scenario DOCOMO Communications Lab. RP-192964 Summary for:eNB(s) Architecture Evolution for E-UTRAN and NG-RAN China Unicom | 0.3.0 |
| 2020-03 | SA1#90 |  |  |  |  | Summaries received from SA#86: SP-191223 Summary for 5G_eLCS CATT (SP-191229 Summary for RDSSI Convida Wireless LLC-> replaced by SP-200057) SP-191247 Summary for 'Service Interactivity' Qualcomm Incorporated SP-191252 Summary for 'OAM aspects of LTE and WLAN integration' Intel SP-191277 Summary for Volume Based Charging Aspects for VoLTE China Mobile SP-191328 Summary for 'EVS Float Conformance for Non Bit-exact EVS_FCNBE Rapporteur (Intel) Summaries received from TSG#87e: RP-200087 Summary for 2-step RACH ZTE Corporation RP-200112 Summary for Dual Connectivity (EN-DC) with 3 bands DL and 3 bands UL ZTE Corporation RP-200152 Summary for: Single Radio Voice Call Continuity from 5G to 3G China Unicom RP-200170 Summary for inter-band CA/DC for 2 bands DL with up to 2 bands UL ZTE Corporation RP-200173 Summary for inter-band CA/DC for 3 bands DL with 2 bands UL ZTE Corporation RP-200214 Summary for High power UE (power class 2) for EN-DC (1 LTE TDD band + 1 NR TDD band) CMCC SP-200034 Summary for 'RTCP Verification for Real-Time Services' Ericsson LM SP-200036 Summary for "Media Handling Extensions for 5G Conversational Services” (5G_MEDIA_MTSI_ext) Intel (Rapporteur) SP-200057 Summary for RDSSI Convida Wireless LLC SP-200058 Summary for Rel-16 eV2XARC LG Electronics SP-200124 Summary for ATSSS ZTE Wistron Telecom AB SP-200125 Summary for eNS ZTE Wistron Telecom AB SP-200126 Summary for Rel-16 V2XIMP LG Electronics France SP-200128 Summary for "VR QoE Metrics" Ericsson LM=> NOT INCLUDED, SAID TO BE REL-17 in the Work Plan SP-200129 Summary for 5G_SRVCC China Unicom SP-200131 Summary for User Identities and Authentication Deutsche Telekom AG SP-200217 Summary for Business Role Models for Network Slicing Nokia CP-200143 Summary for Shared Data Handling on Nudm and Nudr Nokia, Nokia Shanghai Bell | 0.4.0 |
| 2020-07 | SA#88e |  |  |  |  | Summaries endorsed (marked as "noted") by RAN#87e: Already included in v.0.4.0 (note that RP-201225 "Summary for: 2-step RACH for NR" from ZTE Corporation was superseded at following RAN meeting) Summaries endorsed by CT#87e: Already included in v.0.4.0 Summaries endorsed by SA#87e: Already included in v.0.4.0: SP-200034; SP-200036; SP-200057; SP-200058; SP-200124; SP-200125; SP-200126; SP-200128(not included, Rel-17); SP-200129; SP-200131 New SA#87e summaries in v.0.5.0: SP-200215 Summary for Provision of Access to Restricted Local Operator Services by Unauthenticated UEs Nokia SP-200224 Summary for SMARTER-Ph2 VODAFONE Group Plc SP-200228 Summary for Energy Efficiency of 5G Orange SP-200253 Summary for 5WWC Huawei SP-200254 Summary for eNA Huawei SP-200256 Summary for 'ETSUN' Nokia Japan SP-200265 Summary for 5GMSG China Mobile SP-200266 Summary for Charging aspects of ETSUN China Mobile SP-200273 Summary for RACS Qualcomm Incorporated, MediaTek SP-200274 Summary for 'Cellular IoT support and evolution for the 5G System' Qualcomm Incorporated SP-200294 Summary for Business Role Models for Network Slicing Nokia SP-200295 Summary for 5G_URLLC Huawei SP-200296 Summary for Vertical_LAN Nokia Japan Summaries endorsed (marked as "noted") by RAN#88e: RP-200604 Summary for: Support for NavIC Navigation Satellite System in LTE Reliance Jio RP-200678 Summary for: NG interface usage for WWC (Wireless Wireline Convergence) Huawei, HiSilicon RP-200727 Summary for Add support of NR DL 256QAM for frequency range 2 (FR2) China Telecom RP-200733 Summary for Private Network Support for NG-RAN China Telecom RP-200738 Summary for Even further mobility enhancement in E-UTRAN China Telecom RP-200774 Summary for SON (Self-Organising Networks) and MDT (Minimization of Drive Tests) support for NR CMCC RP-200776 Summary for NR support for high speed train scenario CMCC RP-200798 Summary for Support of NR Industrial Internet of Things (IoT) Nokia, Nokia Shanghai Bell RP-200850 Summary for LTE-based 5G terrestrial broadcast QUALCOMM Europe Inc. - Italy RP-200855 Summary for 5G V2X with NR sidelink LG Electronics RP-200885 Summary for DL MIMO efficiency enhancements for LTE Huawei, HiSilicon RP-200887 Summary for Physical layer enhancements for NR ultra-reliable and low latency case (URLLC) Huawei, HiSilicon RP-200912 Summary for UE Power Saving in NR CATT RP-201019 Summary for Introduction of NR FDD bands with variable duplex and corresponding framework Huawei, HiSilicon RP-201225 Summary for 2-step RACH for NR ZTE Corporation RP-201229 Summary for Additional enhancements for NB-IoT Futurewei Technologies RP-201235 Summary for Core part: Optimisations on UE radio capability signalling – NR/E-UTRA Aspects Mediatek RP-201274 Summary for NR mobility enhancements Intel Corporation RP-201276 Summary for Additional MTC enhancements for LTE Ericsson RP-201280 Summary for Cross Link Interference (CLI) handling and Remote Interference Management (RIM) for NR LG Electronics Summaries endorsed by CT#88e: None Summaries endorsed by SA#88e: SP-200343 Summary for Enhancements of Public Warning System (ePWS) SyncTechno Inc. (Rapporteur) SP-200344 Summary for Maritime Communication Services over 3GPP System (MARCOM) SyncTechno Inc. (Rapporteur) SP-200403 Summary for ANTeM HEAD acoustics GmbH SP-200520 Summary for KPI reporting ZTE Corporation SP-200524 Add Telecom management section to TR 21.916 SA WG5 Vice Chairman (Huawei), SA WG5 Chairman, SA WG5 Vice Chairman (Nokia) SP-200525 Add summary of Charging Aspect for 5WWC to TR 21.916 Huawei Tech.(UK) Co., Ltd SP-200526 Add summary of management enhancement for tenant environment support to TR 21.916 Huawei Tech.(UK) Co., Ltd SP-200531 Summary for Methodology for 5G management specifications Ericsson LM SP-200532 Summary for Closed loop SLS Assurance Ericsson LM SP-200534 Summary for the SBMA trace and Streaming trace work items Nokia SP-200535 Summary for 5G management capabilities Orange Spain SP-200544 WI_summary_MA5SLA China Mobile Com. Corporation=> Not INCLUDED (Rel-17 In the Work Plan) SP-200128 Summary for "VR QoE Metrics" Ericsson LM (Included after clarification that it was Rel-16 and not Rel-17) | 0.5.0 |
| 2020-09 | SA#88e |  |  |  |  | Summaries endorsed by SA#86: SP-190322 WI_summary E2E_DELAY (was unintentionally skipped earlier) Intel Summaries received at TSG#89e, to be included in next version: CT#89e: CP-202027 Work Item Summary for eNAPIs Huawei SA#89e: SP-200776 Work Item Summary for 5G_eSBA China Mobile SP-200777 Work Item Summary for Rel-16 UDICOM Nokia Korea SP-200801 Summary for Work Item on “Removal of H.263 and MPEG-4 Visual from 3GPP Services” Qualcomm Incorporated SP-200808 Work Item Summary for Rel-16 eIMS5G_SBA T-Mobile USA Inc. SP-200848 Work item Summary for 5GMS3 Sony (Rapporteur) SP-200849 Work item Summary for Enhancement of performance assurance for 5G Intel (Rapporteur) SP-200851 Work Item Summary for Rel-16 eCAPIF Samsung SP-200852 Work Item Summary for Rel-16 SEAL Samsung RAN#89e: RP-201531 Summary of WI on Multi-RAT Dual-Connectivity and Carrier Aggregation enhancements (LTE, NR) Ericsson RP-201718 WI summary: RF requirements for NR frequency range 1 (FR1) Huawei, HiSilicon RP-201757 WI summary for WI: Integrated Access and Backhaul for NR (NR_IAB) Qualcomm Incorporated RP-201829 Summary for WI UE Conformance Test Aspects - Mission Critical Improvements NIST RP-201836 WI summary for WI: NR positioning support Intel Corporation RP-201840 WI summary for NR-based Access to Unlicensed Spectrum QUALCOMM Europe Inc. - Spain RP-201884 WI summary: NR RRM enhancement in R16 Intel Corporation RP-201905 WI Summary for 29 dBm UE Power Class for LTE Band 41and NR Band n41 T-Mobile USA RP-201942 WI summary on RRM requirement for CSI-RS Based L3 measurement CATT Receiveed by e-mail but not submitted to TSG#8e: eNAPI |  |

List of missing summaries:

| 800006 | LAN support in 5G | 5GLAN | S1 | SP-180593 | Jose Almodovar, KPN |
| --- | --- | --- | --- | --- | --- |
| 840074 | Application layer support for V2X services | V2XAPP | S6 | SP-180898 | Niranth Amogh, Huawei Tel.India |
| 800049 | Stage 1 of 5G_HYPOS | 5G_HYPOS | S1 | SP-180329 | Ries, Lionel, ESA |
| 810004 | MCData File Distribution support over xMB | MC_XMB | S4 | SP-180665 | Thiénot Cédric, Expway |
| 800019 | Enhanced Mission Critical System Migration and Interconnection | eMCSMI | S6 | SP-180379 | Chater-Lea, David; Motorola Solutions |
| 800021 | Enhanced Mission Critical Communication Interworking with Land Mobile Radio Systems | eMCCI | S6 | SP-180680 | Monnes, Peter, Harris Corporation |
| 800016 | Stage 3 for MC Communication Interworking with Land Mobile Radio Systems | MCCI_CT | C1 | CP-190203 | Monnes, Peter, Harris Corporation |
| 820040 | Mission Critical system migration and interconnection | MCSMI_CT | C1 | CP-190143 | Dom Lazara, Motorola Solutions |
| 800032 | Mission Critical Services Security Enhancements | MCXSec | S3 | SP-180596 | Woodward, Tim, Motorola Solutions, Inc |
| 800053 | MBMS APIs for Mission Critical Services | MBMSAPI_MCS | S6 | SP-180380 | Ling Zhang, TD Tech Ltd. |
| 760054 | Mobile Communication System for Railways 2 | MONASTERY2 |  | SP-170451 | Merkel, Juergen, Nokia |
| 800008 | Enhancement of LTE for Efficient delivery of Streaming Service | eLSTR | S1 | SP-180322 | Xia, Xu, China Telecom |
| 800001 | Enhancements to Framework for Live Uplink Streaming | E_FLUS | S4 | SP-180285 | Lo, Charles, Qualcomm |
| 820002 | Media streaming architecture | 5GMSA | S4 | SP-180984 | Frédéric GABIN |
| 810044 | Multi-device and multi-identity | MuD | S1 | SP-180315 | Peter Bleckert, Ericsson |

+ SA6's enh2MCPTT and eMCData2.