3GPP TSG RAN meeting #100										RP-230914

Taipei, June 12-14, 2023

## Status Report to TSG

Agenda item:			9.3.1.8

| WI / SI Name | Network energy savings for NR |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| included in this status report | Study Item: No | Core part: Yes | Performance part: Yes |  | Testing part: No |
| Acronym | Netw_Energy_NR |  |  |  |  |
| Unique ID | 981037 |  |  |  |  |
| TSG Tdoc of latest approved WI/SI description (if any) | RP-230566 |  |  |  |  |
| Target Completion Date (indicate if changed) | Study Item: | Core part: 12/2023 | Performance part: 06/2024 | Testing part: |  |
| Overall Completion level | Study Item: | Core part: 55% | Performance Part: 0% | Testing part: |  |

Note: Overall completion level percentage numbers should use one of the colors below:

- xx%: Normal progress, no RAN plenary action needed

- xx%: Progress behind schedule, may need RAN plenary intervention. If so, SR should clearly define requested action

- xx%: Progress critically behind, RAN plenary shall intervene. SR should define requested action

Source:

| Leading WG |  | RAN1 |
| --- | --- | --- |
| Rapporteur | Name | Yi Wang |
|  | Company | Huawei |
|  | Email | wangyi6@huawei.com |

## 1	Work plan related evaluation

| Do you want to modify the time budget for this WI/SI compared to what was endorsed at the last RAN meeting? | No |
| --- | --- |

If you answered No:	Then please remove the Excel file from the zip file of this status report.

If you answered Yes:	Then please fill out the attached Excel template to request a modification of the time 		budgets for your WI /SI. The Excel table has to be filled out for all affected RAN WGs and 		up to the target date of the WI/SI. The basis are the endorsed time budgets of the last 		RAN meeting. Please highlight all changes of the values.
		One time unit (TU) corresponds to ~ 2 hours in the meeting.
		If this status report covers a WI with Core and Performance part, then please have one 		line for each in the attached Excel table.
		Note: If no Excel table is attached, then this means no time budget change.

Additional explanations/motivations for the time budget changes in the attached Excel table:

## 2.	Detailed progress in RAN WGs since last TSG meeting (for all involved WGs)

NOTE: Agreements and Open issues impacted cross-TSG aspects shall be explicitly highlighted

## 2.1	RAN1

#### 2.1.1	Agreements

##### RAN1#112bis-e

Agreement

Define necessary enhancements to support both types of spatial adaptation cases (as defined in RAN1#112) in Rel-18.

- Note: This does not imply explicit definition in specifications for adaptation types.

- Note: This does not imply explicit specification changes are made for both cases

Agreement

Support configurability of NZP CSI-RS resource(s) for channel measurement within one resource setting corresponding to more than one spatial adaptation patterns with at least one of the following

- A1-1-revised: a resource set with multiple resources is configured within a resource setting, where each resource is associated with only one spatial adaptation pattern

- A1-2-revised: For a resource configured in a resource set within a resource setting, the resource can be associated with more than one spatial adaptation patterns

  - One or more resources can be configured in the resource set for channel measurement.

Agreement

At least support A2-2, i.e. one CSI report configuration contains multiple CSI report sub-configurations where each sub-configuration corresponds to one spatial adaptation pattern.

- FFS: impact on CSI processing requirement

Agreement

For a CSI report config with L sub-configuration(s), support a framework that enables a UE to report N CSI(s) in one reporting instance where the N CSI(s) are associated with N sub-configuration(s) from L (where 1≤N≤L) and each CSI corresponds to one sub-configuration.

- For discussion purpose, N=1 refers to single-CSI while N>1 refers to multi-CSI.

- For Semi-persistent/Aperiodic CSI reporting, support gNB trigger/indicate/activate report of N≤L CSIs where N>=1

- The maximum value of N and L are subject to UE capability

- Further study how to address/minimize additional UE complexity

The following bullet not agreed due to objection from Apple and vivo

- For Periodic CSI reporting, at least the case of N=L is supported where N>=1

Conclusion

New CSI-RS resource (RE mapping) pattern is not introduced for R18 network energy savings purpose.

- Note: CSI-RS resource (RE mapping) pattern above refers to a row in TS 38.211 Table 7.4.1.5.3-1 determining CSI-RS locations within a slot.

Agreement

For power domain adaptation, for CSI(s) reporting, support configuration of more than one power offset values for PDSCH relative to CSI-RS

- FFS: impact on CSI processing requirement

- FFS: details on configuration/indication of the power offset values

- FFS: whether/how to additionally consider the case where CSI-RS power is changed

Agreement

- For CSI feedback with CSI overhead/report payload reduction, further study whether/how to report a common value and/or a differential and/or joint coded value across same CSI quantity of different sub-configurations/adaptation patterns, at least for the following

  - CRI

  - RI

  - PMI

  - CQI

  - FFS: L1-RSRP

  - Other (new) report quantity, if any

- Further study whether/how it is feasible/possible for the UE to skip the evaluations of some sub-configurations/adaptation patterns to reduce the burden at the UE

Agreement

For CSI report configuration, if L>1 in a CSI report configuration, at least the following can be included for each sub-configuration for Type 1 SD adaptation

- N1, N2 for single-panel and N1, N2, Ng for multi-panel

  - FFS: details on explicit indication or implicit derivation

- Port subset indication when A1-2 is used (if A1-2 is supported)

  - FFS: details on explicit indication or implicit derivation

- FFS: rank restriction

- FFS: codebook subset restriction

- FFS: supported codebook types for PMI, e.g., Type-I or Type-II

- FFS: report quantity

- FFS: reportFreqConfiguration

- FFS: Group identity of NZP CSI-RS resource(s) in a resource set for channel measurement when A1-1 is used

For CSI report configuration for type 2 SD adaptation, further study under which cases sub-configurations may or may not be needed including sub-configuration content

Agreement

For power domain adaptation, support the following configuration(s) for CSI-RS resource configuration,

- A1-2-power: one or more resources can be configured in a resource set within a resource setting and each resource can be associated with one or more power offset values

- FFS: A1-1-power: a resource set with multiple resources is configured within a resource setting, where resources can have different power offset values

- FFS: Details of how the different power offset values(s) are configured/indicated.

Working Assumption

Al-1-revised and A1-2-revised are supported

    - FFS: Which Type of SD adaptation A1-1-revised and A1-2-revised are applicable for

Agreement

    - For R18 NES, only legacy port configuration values (N1, N2) or (Ng, N1, N2) are supported.

    - FFS: Whether/what restriction for A1-1-revised and A-1-2-revised w.r.t number of ports

Agreement

For Semi-persistent/Aperiodic CSI reporting with 1≤N≤L, study what enhancements to the current DCI and MAC-CE mechanisms are needed for gNB triggering/indication/activation of the N CSI(s) in a reporting instance, where the N CSI(s) are associated with N sub-configuration(s) from L in a report config.

Conclusion

From RAN1 perspective, there is no action needed for the LS R1-2302288 from SA5 this time.

Agreement

From RAN1 point of view, Rel-18 UE supporting cell DTX does not expect to receive and/or process the following signals/channels from the gNB, during non-active periods of cell DTX. The list of signals/channels may be updated based on RAN2/RAN4 input and other signals/channels are not precluded from further discussions.

- Periodic/Semi-persistent CSI-RS configured in CSI report configuration in CSI-ReportConfig with reportQuantity including RI (for CSI reporting)

- FFS:

  - PDCCH in USS

    - UE behaviour for retransmission

    - if some specific RNTI scrambled PDCCH in USS will be excluded from cell DTX operation

  - PDCCH in Type-3 CSS

    - UE behaviour for retransmission

    - if some specific RNTI scrambled PDCCH in Type-3 CSS will be excluded from cell DTX operation

  - PRS

  - CSI-RS configured by measObjectNR (for RRM)

  - CSI-RS associated with RadioLinkMonitoringConfig and BeamFailureDectection (for RLM and BFD)

  - Periodic CSI-RS configured with trs-Info ‘true’ (for tracking)

  - Periodic/Semi-persistent CSI-RS (for BM)

    - FFS on how to differentiate (if needed) with other CSI-RS used for CSI reports for BM

- FFS: Whether the same or different UE behaviour is applicable with or without C-DRX

- FFS: Whether the list of impacted signals/channels can be configurable

- FFS: Whether there will be exception case(s) for UE receiving and/or processing listed signals/channels during non-active periods of DTX

- FFS: RAN1 to consider impact on system if the channels/signals are not transmitted during non-active period

Agreement

Study L1 signalling for enhancing cell DTX/DRX including activation/deactivation for a single configuration which will have the following characteristics:

- PDCCH based signaling

  - FFS: Whether enhancing legacy DCI or introducing new DCI

  - FFS: DCI content

  - FFS: Whether L1 signaling is UE specific DCI or group common DCI

  - FFS: Timer or validity duration based activation/deactivation of cell DTX/DRX

  - FFS: whether to specify a reference time for activation/deactivation of cell DTX/DRX

  - FFS: If multiple Cell DTX/DRX patterns are to be supported

- FFS on detailed UE behavior upon reception of L1 signaling at least including application delay

- FFS how to guarantee reliability of the L1 signaling

- FFS whether the L1 signal can be monitored in non-active periods.

Agreement

From RAN1 point of view, Rel-18 UE supporting cell DRX is not expected to transmit the following signals/channels to the gNB during non-active periods of cell DRX. The list of signals/channels may be updated based on RAN2/RAN4 input and other signals/channels are not precluded from further discussions.

- Periodic/Semi-persistent CSI report

- Periodic/Semi-persistent SRS

  - FFS: SRS for positioning

- FFS:

  - HARQ feedback for SPS PDSCH

- FFS whether there will be exception case(s) for UE transmitting listed signals/channels during non-active periods of DRX

- FFS Whether the listed signals/channels can be configurable by gNB

- FFS: Whether the same or different UE behavior is applicable with or without C-DRX

- FFS: RAN1 to consider impact on system if the channels/signals are not transmitted during non-active period

Further study the following in RAN1:

- Handling of HARQ-ACK codebook generation when configured with cell DTX/DRX

- Handling of PUCCH deferral operation during non-active periods of cell DRX

- Handling of overlapping channels where a least a channel overlaps with non-active periods of cell DTX/DRX

- Handling of signals/channels that can be received/transmitted repeatedly during non-active periods of cell DTX/DRX

- Handling of PUCCH switching during non-active period to an active cell

- Other enhancements are not precluded.

Agreement

For PDDCH monitoring, further work on Rel-18 NES in RAN1 is to follow the RAN2 agreement below:

10.	The understanding for the gNB scheduling behaviour for new transmissions during Cell DTX non-active period is that the gNB does not schedule UE-specific dynamic grants/assignments, even if the UE is in C-DRX Active Time. UE doesn’t monitor PDCCH for dynamic grants/assignments for new transmissions during Cell DTX non-active period, even if the UE is in C-DRX Active time. FFS how to deal with any exceptions (e.g. SR if agreed and RACH).

Working Assumption

Support of L1 signaling at least for activation/deactivation of a cell DTX and/or DRX configuration is feasible (e.g., in terms of enabling/disenabling the feature) from RAN1 perspective.

- This does not imply that L1 activation/deactivation is supported in Rel-18\

- Note: Reliability, overhead, and benefits are FFS

##### RAN1#113

R1-2306030	FL summary#1 for SD and PD adaptation for R18 NES	Moderator (Huawei)

Agreement

Confirm the working assumption with the following update (in red)

    - Al-1-revised and A1-2-revised are supported

      - For Type 1 SD adaptation

        - A1-2-revised is supported

      - For Type 2 SD adaptation

        - A1-1-revised is supported.

Agreement

For a CSI report configuration with L>1, for Type 1 SD, at least when A1-2-revised is used for the associated codebook configuration,

- Only common codebook type for PMI across sub-configurations is supported

  - Codebook type-1 for PMI is supported

Agreement

For N>=1 CSI reporting corresponding to N out of L sub-configurations in one reportConfig where each sub-configuration corresponding to an SD adaptation pattern or/[and] a powerControlOffset value,

- For A-CSI and SP-CSI on PUSCH report, support DCI-based triggering

  - For A-CSI-RS, CPU and CSI-RS resource/port counting depend on N indicated sub-configurations

    - FFS: How to do the counting

  - FFS: For P-CSI-RS/SP-CSI-RS, CPU and CSI-RS resource/port counting depend on L or N sub-configurations

- For SP-CSI on PUCCH report, support MAC-CE-based triggering

  - FFS: For P-CSI-RS/SP-CSI-RS, CPU and CSI-RS resource/port counting depend on L or N sub-configurations

Note: UE complexity reduction is not precluded

- For DCI-based triggering,

  - Alt 1: A triggering state corresponding to N sub-configurations is indicated via the existing CSI request field in DCI. Different triggering states could represent different subsets of L sub-configurations.

    - The DCI is UE specific (in this case, legacy DCI format applies)

- For MAC-CE based triggering

  - Opt 2: An indication to select to N sub-configurations in a MAC-CE is supported

    - It is up to RAN2 to decide the signaling designs of the MAC-CE (including whether it is a new MAC CE or an existing MAC CE)

    - Only one MAC CE is used for this triggering

R1-2306031	FL summary#2 for SD and PD adaptation for R18 NES	Moderator (Huawei)

Agreement

- For A1-1-revised for Type 2, one or more CSI-RS resources from a CSI-RS resource set for channel measurement can be associated with the same sub-configuration provided in a CSI report configuration

  - Resources in the resource set for channel measurement have the same number of antenna ports

- For A1-2-revised for Type 1, all CSI-RS resource(s) (which can be one or more) in the CSI-RS resource set for channel measurement are associated with each sub-configuration provided in a CSI report configuration

  - i.e. each CSI-RS resource is associated with all the sub-configurations

  - Resources in the resource set for channel measurement have the same number of antenna ports

- FFS: restriction on total number of CSI-RS resources for channel measurement in a CSI-ReportConfig and/or sub-configuration.

R1-2306032	FL summary#3 for SD and PD adaptation for R18 NES	Moderator (Huawei)

Agreement

For Type 1 adaptation, for each sub-configuration,

- Port subset indication is based bitmap is supported

    - One bit per port for single panel case (i.e. turning off in a port level)

    - FFS: One bit per panel for multi-panel case (i.e. turning off in panel level)

    - Note: It is up to the gNB to ensure the mapping of the bit to a uniform x-pol rectangular array

Agreement

For Type 1 adaptation, for each sub-configuration, for multi-panel case,

- One bit per port based on bitmap is supported

- Note: It is up to the gNB to ensure the mapping of the bit to a uniform x-pol rectangular array for each of the activated panel(s). Additionally, if more than one panel is activated, uniformity across panels is ensured by the gNB (i.e., the same N1, N2 across multiple activated panels)

Agreement

For both spatial domain NES, when UE reports CSIs corresponding to one or more sub-configurations provided in a CSI report configuration,

- At least support baseline: Report CSI for each indicated sub-configuration, according to reportQuantity configuration

  - FFS: details on how to map CSI(s) in a CSI report

- Further enhancement on CSI payload reduction is not precluded

Agreement

For spatial domain adaptation or power domain adaptation, for CSIs reporting corresponding to N indicated sub-configurations from L sub-configurations in a CSI report, for the case without CSI payload reduction

- OCPU=i=1[N or L]Ksi, where Ksi is the total number of CSI-RS resources corresponding to i-th sub-configuration in the CSI-RS resource set for channel measurement.

  - the summation is over N for A-CSI RS

  - This is for CSI processing criteria for NES in Clause 5.2.1.6 of TS 38.214

Agreement

For the sub-configuration(s) in a CSI report configuration with L>1,

- for Type 1 SD with A1-2-revised, the following is configured in each sub-configuration

    - codebook subset restriction,

    - rank restriction

    - N1, N2 and Ng

    - FFS: the case when the number of ports is less than 4

- for Type 2 SD adaptation with A1-1-revised, for each sub-configuration

    - a list of CSI-RS resource ID

    - FFS: codebookConfig (including codebookSubsetRestriction/ ri-Restriction)

    - FFS: CQI table indication

    - FFS: reportFreqConfiguration

    - FFS: report quantity

Above is agreed in addition to what was agreed in previous RAN1 agreements

Agreement

Joint operation of SD and PD adaptation is supported.

Agreement

- Downselect one of the following for BM enhancements in RAN1#114

    - Case 1: Support scaling the threshold of beam failure detection and threshold of candidate beam identification for power domain network energy saving

    - Case 2: Support UE to send hypothetical beam failure and/or radio link failure (RLF) reports for the indicated hypothetical power offset values.

    - Case 3: No further work on BM enhancements

- Downselect one of the following for TCI configuration enhancement in RAN1#114

    - Method 1: Configure multiple candidate CSI-RS resources as reference signal for QCL information or for spatial relation information, and switch one of them based on L1/L2 signaling

    - Method 2: Configure multiple candidate sets of TCI state(s) associated with DL/UL signal/channel and switch one of them based on L1/L2 signaling

    - Method 3: No further work on TCI configuration enhancement

Agreement

Alt 2: For P-CSI reporting from L configured sub-configurations, support:

- All L configured sub-configurations are reported in every periodic occasion.

- The maximum value of L can be different for A-CSI, SP-CSI, and P-CSI.

- OCPU=i=1[N or L]Ksi, where Ksi is the total number of CSI-RS resources corresponding to i-th sub-configuration in the CSI-RS resource set for channel measurement. (N=L in the equation)

- FFS: Details on active CSI-RS resource / port counting

Agreement

RAN1 supports the group common L1 signaling using PDCCH for cell DTX/DRX activation and deactivation without HARQ feedback

- Send an LS to RAN2 to consider the additional support of a MAC CE based indication

- Subject to UE capability

Agreement

Confirmation of WA from previous meeting with removal of the two sub-bullets.

Working Assumption

  - Support of L1 signaling at least for activation/deactivation of a cell DTX and/or DRX configuration is feasible (e.g., in terms of enabling/disenabling the feature) from RAN1 perspective.

    - This does not imply that L1 activation/deactivation is supported in Rel-18\

    - Note: Reliability, overhead, and benefits are FFS

Agreement

DCI format for group common L1 signaling using PDCCH for cell DTX/DRX activation and deactivation (downselect just one among alternatives)

- Alt 1) DCI Format 2_6 (power saving information outside DRX Active Time)

  - FFS: Monitoring within DRX active time

  - FFS: Field content

- Alt 2) Based on new DCI format 2_X

  - Field content format

    - Block number 1, block number 2, …, block number N

    - For each block should at least support the following:

      - DTX configuration activation/deactivation

      - DRX configuration activation/deactivation

    - FFS: other field details, mapping of UE and each blocks

  - DCI size indicated by higher layers

  - FFS: RNTI

- FFS: application delay, timers for activation/deactivation

- FFS: handling of multiple cells including when UE supports different number of cells

- FFS: details on PDCCH monitoring aspects, including but not limited to:

  - Search Space

  - PDCCH monitoring occasion

  - slots to monitor (during cell DTX/DRX non-active periods, and active periods)

  - BD/CE aspects

- FFS: UE behavior upon reception of the group common PDCCH (during cell DTX/DRX non-active periods, and active periods), including fallback behavior (if any)

R1-2306169	Draft reply LS on cell DTX/DRX activation/deactivation	Moderators (Huawei, Intel)

Draft LS in R1-2306169 is endorsed in principle with the following revision. Further RAN1 agreements can be added in final LS if necessary.

“With respect to questions asked by RAN2, RAN1 has made the following agreements:”

Final LS to include the agreement on the introduction of new DCI format. Final LS is in R1-2306246.

Agreement

For the group common L1 signaling using PDCCH for cell DTX/DRX activation and deactivation

- Alt 2) Based on new DCI format 2_X

  - DCI size budget is not increased

  - Number of required BDs is not increased

  - FFS: PDCCH monitoring configuration for the new DCI format is identical to PDCCH monitoring configuration for DCI format 2_6 if the UE monitors both DCI formats

    - FFS: New RNTI is used

#### 2.1.2	Remaining Open issues

Remaining issues for techniques in spatial and power domains.

Remaining issues for enhancement on cell DTX/DRX mechanism.

## 2.2	RAN2

#### 2.2.1	Agreements

##### RAN2#121bis-e

Cell DTX/DRX:

Agreements

- A periodic cell DTX/DRX configuration is explicitly signalled to the UEs.

- A periodic cell DTX/DRX pattern is configured by UE specific RRC signalling.

- The Cell DTX/DRX configuration contains at least: periodicity, start slot/offset, on duration.

- As a baseline Cell DTX/DRX is activated/deactivated implicitly by RRC signalling, i.e. activated immediately once configured by RRC and deactivated once the RRC configuration is released.

- From RAN2 point of view, majority companies see a benefit with L1 signalling for Cell DTX/DRX activation/deactivation, send a LS to RAN1 (email 308) with our preference and ask about feasibility and design details.   Ask about feasibility and reliability of using L1 signaling.  Clarify that the question is about activation/deactivation copy the agreement from last meeting that we are focusing on single configuration.  Extract a few key benefits of dynamic signaling from email discussion and online discussions

- As baseline, UE doesn’t monitor SPS occasions during Cell DTX non-active period. As baseline, gNB is assumed to be not transmitting PDSCH to that UE on such SPS occasions during the Cell DTX non-active period

- As baseline, UE does not transmit on CG occasions during Cell DRX non-active periods

- As baseline, UE does not transmit SR occasions overlapping with Cell DRX non-active periods, e.g. SR transmissions are dropped during the non-active period

FFS: whether we will allow to configure the UE per SR configuration with whether SR can be transmitted during Cell DRX non-active period to to support high priority traffic

- (for the SRs that will be dropped) If SR is not to be transmitted on an PUCCH occasion during Cell DRX non-active time, the UE keep the SR pending, i.e., the UE delays the SR transmission till the Cell DRX active period without triggering RACH.  For the FFS case there may be some exceptions.

- The understanding for the gNB scheduling behaviour for new transmissions during Cell DTX non-active period is that the gNB does not schedule UE-specific dynamic grants/assignments, even if the UE is in C-DRX Active Time.   UE doesn’t monitor PDCCH for dynamic grants/assignments for new transmissions during Cell DTX non-active period, even if the UE is in C-DRX Active time.   FFS how to deal with any exceptions (e.g. SR if agreed and RACH).

FFS how to deal with retransmissions

Connected mode mobility:

Agreements

-	RAN2 agree to make enhancement in CHO procedure based on that the source cell entering “NES mode”.  FFS further details

-	For source cell CHO framework, RAN2 assumes a reference scenario where the UE has already performed CHO conditions evaluation by the time the source cell starts some “NES-mode”

-	As a baseline, UE initiates CHO evaluation upon receiving the CHO configuration.  FFS what trigger is used for execution of CHO

##### RAN2#122

Cell DTX/DRX:

Agreements:

1	UE monitors PDCCH for RAR during Cell DTX non-active time. The ra-ResponseWindow could be started as legacy.

2	UE monitors PDCCH for msg4 during Cell DTX non-active time. The ra-ContentionResolutionTimer could be started as legacy.

3	Working assumption:  When the retransmission timer is running (if C-DRX is configured), the UE is expected to monitor PDCCH, like in legacy.  It is up to the network whether it schedules retransmissions out of the Cell DTX active period, i.e., when the DRX retransmission timer is running, the UE should monitor PDCCH regardless of the Cell DTX.

4	Once gNB recognizes there is an emergency call or public safety related service (e.g. MPS/MCS), the NW should ensure there is no impact to the emergency call (e.g. may deactivate Cell DTX/DRX).  The behavior is captured in stage 2 spec

5	When an DG grant is received, by the gNB during cell DRX/DTX, the UE follows the grant assignment (i.e. like in legacy).  This includes DL HARQ feedback.

SSB-less SCell operation:

Agreements:

- If RAN4 conclude SSB-less SCell for inter-band CA for FR1 and co-located cells is feasible, the signaling of intra-band CA (including RRC change on timing of SSB-less SCell and capability signaling) can be considered as its baseline. Whether other new signaling is required depends on RAN4 input.

- If RAN4 concludes it is feasible, RAN2 can further work on at leaest the following specification impacts:

-	RRC configuration of the frequency of the SSB to be used for the UE to obtain the timing reference for the inter-band SCell.

-	UE capability reporting to indicate whether UE supports configuration of inter-band SCell that does not transmit SS/PBCH block.

Cell selection/re-selection:

Agreements:

- We will define UE capabilities with signaling.  Details are FFS and will be discussed later during the WI phase.

- Separate camping restrictions for NES-capable and non-NES UEs will be defined.  FFS if it is a single bit or more.

Connected mode mobility:

Agreements

1.	We will have a CHO solution that considers NES mode of at least source cell.

2.	We can have a specific NES CHO execution condition based on source cell NES mode.   FFS how the UE determines is in NES mode.   FFS on how this is achieved in RRC

3.	We will not introduce new L1 signalling for the purpose of CHO

4.	Event A3, A4, A5 can be configured as a CHO execution condition in the NES scenario.   We will study the time based mechanism

#### 2.2.2	Remaining Open issues

Remaining issues for cell DTX/DRX (e.g. configuration, activation/deactivation, gNB/UE behaviour, alignment).

Remaining issues for SSB-less and fast SCell activation (e.g. further RAN2 impacts).

Remaining issues for cell barring and cell selection/reselection (e.g. camping restrictions).

Remaining issues for connected mode mobility (e.g. CHO execution conditions).

## 2.3	RAN3

#### 2.3.1	Agreements

##### RAN3#119bis-e

Inter-node beam activation:

For inter-node beam activation, the XnAP CELL ACTIVATION procedure, and the F1AP GNB-CU CONFIGURATION UPDATE procedure are reused.

- Over Xn interface, the CELL ACTIVATION REQUEST message may include the SSB beam list that is requested to be activated, and the CELL ACTIVATION RESPONSE message may include SSB beam list that are activated. When the receiving NG-RAN node cannot activate any of the SSB beams, it should respond with the CELL ACTIVATION FAILURE message with an appropriate cause value.

- Over F1 interface, the GNB-CU CONFIGURATION UPDATE message may include the SSB beam list that is requested to be activated, and the GNB-CU CONFIGURATION UPDATE ACKNOWLEDGE may include SSB beam list that are activated. In case the gNB-DU cannot activate any of the requested SSB beams, it should respond with the GNB-CU CONFIGURATION UPDATE FAILURE message with an appropriate cause value.

Enhancements on restricting paging in a limited area:

It’s up to gNB’s implementation to decide to which UEs should apply the paging enhancement technique.

The paging enhancement technique is applicable for UEs in RRC inactive state.

Introduce the recommended SSB beam list in the F1AP paging message.

Introduce a list of last few served SSB beam s/recommended SSB beam list as paging assistance information to the gNB-CU in UE CONTEXT RELEASE COMPLETE message over F1AP.

Send a LS to RAN2 asking feedback containing the agreements for RRC inactive UE, and to SA2 asking feedback for RRC idle UE. The above agreements can be revisited based on reply from RAN2.

BLCRs:

Endorse BLCRs for F1AP and XnAP in R3-232082 and R3-232083.

BLCR assignment:

| 38.423 | Huawei |
| --- | --- |
| 38.473 | Ericsson |
| 38.413 if needed | Samsung |
| 38.401 if needed | Nok |
| 38.300 if needed | ZTE |
| 38.470 if needed | Qualcomm |
| 38.420 if needed | CATT |
| 38.410 if needed | Lenovo |
| 37.340 if needed | Intel |

##### RAN3#120

Cell DTX/DRX:

WA: Support the exchange of the Cell DTX/DRX configuration over Xn.

#### 2.3.2	Remaining Open issues

Address FFS(es) in the BLCR including the criticality of the cell to be Activated List IE included in the GNB-CU CONFIGURATION UPDATE message, the maximum number of the SSB areas etc.

Address the working assumption on the exchange of cell DTX/DRX over Xn.

Address the restricting paging in a limited area for RRC idle UE based on the reply LS from other group(s).

## 2.4	RAN4

#### 2.4.1	Agreements

##### RAN4#106bis (BS RF part)

Agreement

The revised work plan (R4-2306640) was approved.

A WF R4-2306639 was approved.

Issue 2-1-1: Priorities for feasibility study

- Agreement：

  - RAN4 prioritize the discussion on SSB-less Scell operation for inter-band CA for FR1 and co-located cell.

Issue 2-2-6: How to define the RF requirements for inter-band CA for SSB-less Scell operation

- Agreement:

  - No impact to the legacy requirements of inter-band CA TAE defined in TS38.104. Whether or not to define feature-specific TAE requirements for SSB-less operation (NW enabling energy savings) for FR1 co-located inter-band CA is FFS.

Issue 2-3-1: RF requirements for Cell DTX

- Agreement:

  - FFS on the necessity to define ON-OFF power requirement waiting for more inputs from RAN1/2.

Issue 2-4-1: RF requirements for spatial domain techniques

- Agreement:

  - FFS on the necessity to the updates in manufacturer declarations, switching period and DL EVM requirement waiting for more inputs from RAN1/2.

##### RAN4#106bis (RRM part)

Agreement

A WF R4-2306369 was approved.

Issue 2-1-1: Which scenario to be considered for SSB-Less SCell operation for FR1 inter-band collocated CA

- Agreements

  - The following scenarios are considered for inter-band SSB-less SCell requirements definition

    - Scenario 1: No SSB but with TRS transmission configured on the SSB-less SCell

      - FFS whether to consider requirements based on TRS transmission during and/or after SCell activation

    - Scenario 2: No SSB and no TRS transmission configured on the SSB-less SCell

    - Scenario 2a: No DL transmission but with UL reception at the NW side on the SSB-less SCell.

    - FFS if requirements will be defined for all or a subset of scenarios subject to feasibility analysis

Issue 2-1-2: From RRM perspective from UE side, under which conditions SSB-less SCell operation can work in scenario-1?

Agreements

  - Conditions for SSB-less inter-band CA SCell operation for scenario-1

    - Received time difference (RTD) between the SSB-less SCell and the FR1 inter-band active serving cell

      - Option 1-1: 260ns

      - Option 1-2: 3us

      - Option 1-3: smaller than 3us

      - Other values are not precluded

    - The difference of the reception power with the FR1 inter-band active serving cell

      - Option 2-1: 6dB

      - Option 2-2: 6dB+offset

      - Other values are not precluded

    - FFS: TCI state (using TCI state information of inter-band co-located SSB less SCell from active serving cell)

    - FFS: frequency domain separation between the SSB-less SCC and the FR1 inter-band active serving CC; or Certain band combinations.

    - FFS if requirements will be defined subject to feasibility analysis

    - Note: different conditions and values may be applicable scenario-1, scenario-2 and scenario-2a

Issue 2-1-3: From RRM perspective from UE side, under which conditions SSB-less SCell operation can work in scenario-2?

Agreements

  - Conditions for SSB-less inter-band CA SCell operation for scenario-2

    - Received time difference (RTD) between the SSB-less SCell and the FR1 inter-band active serving cell

      - Option 1-1: 260ns

      - Option 1-2: 3us

      - Option 1-3: smaller than 3us

      - Other values are not precluded

    - The difference of the reception power with the FR1 inter-band active serving cell

      - Option 2-1: 6dB

      - Option 2-2: 6dB+offset

      - Other values are not precluded

    - FFS: TCI state (using TCI state information of inter-band co-located SSB less SCell from active serving cell)

    - FFS: frequency domain separation between the SSB-less SCC and the FR1 inter-band active serving CC; or Certain band combinations.

    - FFS if requirements will be defined subject to feasibility analysis

    - Note: different conditions and values may be applicable scenario-1, scenario-2 and scenario-2a

Issue 2-1-4: From RRM perspective from UE side, under which conditions SSB-less SCell operation can work in scenario-2a?

Agreements

  - Conditions for SSB-less inter-band CA SCell operation for scenario-2a

    - Received time difference (RTD) between the SSB-less SCell and the FR1 inter-band active serving cell

      - Option 1-1: 260ns

      - Option 1-2: 3us

      - Option 1-3: smaller than 3us

      - Other values are not precluded

    - The difference of the reception power with the FR1 inter-band active serving cell

      - Option 2-1: 6dB

      - Option 2-2: 6dB+offset

      - Other values are not precluded

    - FFS: TCI state (using TCI state information of inter-band co-located SSB less SCell from active serving cell)

    - FFS: frequency domain separation between the SSB-less SCC and the FR1 inter-band active serving CC; or Certain band combinations.

    - FFS if requirements will be defined subject to feasibility analysis

    - Note: different conditions and values may be applicable scenario-1, scenario-2 and scenario-2a

##### RAN4#107 (BS RF part)

Agreement

A WF (R4-2310381) was approved.

Issue 1-1: TAE

- Further discuss the following values for TAE to guarantee the SSB-less feature performance

  - Option 1: 2.x us, i.e., CP size for 30kHz SCS

  - Option 2: 260ns

  - Option 3: 65ns

  - Option 4: 3us

- In the next meeting, encourage companies to provide the achievable values.

##### RAN4#107 (RRM part)

Agreement

A WF (R4-2310086) was approved.

Issue 1-1-1/2/3: Scenario 1 / 2 / 2a

- Agreements

  - Continue RAN4 work on the following SSB-less SCell scenarios

    - Scenario 1: SCell without SSB transmission and with TRS transmission

    - Scenario 2a: SCell without SSB transmission and without any other DL transmissions, but with UL reception at the NW side

      - Note: No RAN1 impacts are expected, and no RAN4 requirements will be defined if the scenario is not supported from RAN1 specification perspective.

  - Deprioritize RAN4 work on the following SSB-less SCell scenario

    - Scenario 2: SCell without SSB transmission and without TRS transmission

  - Send LS to RAN1/2 to check on support of Scenario 2a from RAN1/2 specifications perspective

Issue 1-2-1: RTD conditions for scenario 1

- Agreements

  - Further consider the following cases for requirements definition

    - Set 1: RTD ≤ 3us + X (X is FFS)

    - Set 2: 260ns < RTD < min(CP, 3us)

      - note: the CP corresponding to the largest SCS across CCs

    - Set 3: RTD ≤ 260ns

    - FFS whether all subsets are feasible from UE implementation perspective

#### 2.4.2	Remaining Open issues

All remaining issues are captured in WF R4-2310381 and R4-2310086.

## 2.5	RAN5

#### 2.5.1	Agreements

#### 2.5.2	Remaining Open issues

#### 2.5.3	Remaining Open issues with cross-WG dependencies

## 2.6	RAN6

#### 2.6.1	Agreements

#### 2.6.2	Remaining Open issues

## 3.	Detailed progress in SA/CT WGs since last TSG meeting (for all involved WGs)

NOTE: This section only needs to be filled in for WI/SIs where there is a corresponding relevant WI/SI in SA/CT.

## 3.1	SAx/CTs

#### 3.1.1	Agreements with cross-TSG impacts

#### 3.1.2	Remaining Open issues with cross-TSG impacts

NOTE: This section should also flag any critical dependencies that need TSG attention.

## 4.	References

NOTE:	This can be e.g. a list of all related Tdocs in the affected WGs since last TSG, references to LSs, produced TRs/TSs, the work/study item description or status reports of previous TSGs.

##### RAN1#112bis-e

| R1-2302333 | Spatial and Power Adaptations for Network Energy Savings | FUTUREWEI |
| --- | --- | --- |
| R1-2302334 | Cell DTX/DRX for NES | FUTUREWEI |
| R1-2302337 | CSI enhancements for network energy saving | Huawei, HiSilicon |
| R1-2302338 | Cell DTX/DRX mechanism for network energy saving | Huawei, HiSilicon |
| R1-2302389 | Spatial and power domain adaptation for network energy saving | Panasonic |
| R1-2302390 | Cell DTX/DRX enhancement for network energy saving | Panasonic |
| R1-2302393 | Techniques in spatial and power domains | Nokia, Nokia Shanghai Bell |
| R1-2302394 | Enhancements on cell DTX/DRX mechanism | Nokia, Nokia Shanghai Bell |
| R1-2302498 | Discussions on NES techniques in spatial and power domain | vivo |
| R1-2302499 | Discussions on enhancements on cell DTX/DRX mechanism | vivo |
| R1-2302561 | Discussion on techniques in spatial and power domains | OPPO |
| R1-2302562 | Discussion on enhancements on cell DTX/DRX mechanism | OPPO |
| R1-2302613 | Discussion on NES techniques in spatial and power domains | Spreadtrum Communications |
| R1-2302614 | Discussion on enhancements on cell DTXDRX mechanism | Spreadtrum Communications |
| R1-2302716 | Network Energy Saving techniques in spatial and power domain | CATT |
| R1-2302717 | DTX/DRX for network Energy Saving | CATT |
| R1-2302747 | Cell DTX/DRX Configuration for Network Energy Saving | NEC |
| R1-2302751 | Discussion on network energy saving techniques in spatial and power domains | NEC |
| R1-2302809 | Discussion on NWES techniques in spatial and power domain | Intel Corporation |
| R1-2302810 | Discussion on enhancements on cell DTX/DRX mechanism | Intel Corporation |
| R1-2302912 | Discussion on NW energy saving techniques in spatial and power domains | Fujitsu |
| R1-2302913 | Discussion on cell DTX/DRX mechanism | Fujitsu |
| R1-2302944 | Discussion on NES techniques in spatial and power domains | ZTE, Sanechips |
| R1-2302945 | Discussion on cell DTX/DRX | ZTE, Sanechips |
| R1-2302995 | Discussion on techniques in spatial and power domains | Xiaomi |
| R1-2302996 | Discussions on cell DTX-DRX for network energy saving | xiaomi |
| R1-2303024 | Discussion on techniques in spatial and power domains | InterDigital, Inc. |
| R1-2303025 | Discussion on enhancements on cell DTX/DRX mechanism | InterDigital, Inc. |
| R1-2303030 | Discussion on spatial/power domain adaptation for network energy saving | China Telecom |
| R1-2303031 | Discussion on mechanism of cell DTX/DRX for network energy saving | China Telecom |
| R1-2303056 | Network Energy Saving in Spatial and Power Domain | Google |
| R1-2303057 | Network Energy Saving on Cell DTX and DRX | Google |
| R1-2303141 | Techniques in spatial and power domains | Samsung |
| R1-2303142 | Enhancements on cell DTX/DRX mechanism | Samsung |
| R1-2303202 | Network energy saving techniques in spatial and power domains | ETRI |
| R1-2303203 | Enhancements on cell DTX/DRX mechanism | ETRI |
| R1-2303247 | Discussion on network energy saving techniques in spatial and power domains | CMCC |
| R1-2303248 | Discussion on cell DTX DRX enhancements | CMCC |
| R1-2303309 | Discussion on spatial and power adaptations for network energy savings | CEWiT |
| R1-2303310 | Discussion on cell DTX/DRX mechanism for network energy saving | CEWiT |
| R1-2303344 | On NW energy saving techniques in spatial and power domains | MediaTek Inc. |
| R1-2303345 | On NW energy saving enhancements for cell DTX/DRX mechanism | MediaTek Inc. |
| R1-2303379 | Discussion of NES techniques in spatial domain and power domain | Transsion Holdings |
| R1-2303380 | Discussion on Enhancement on cell DTX DRX mechanism | Transsion Holdings |
| R1-2303426 | Discussion on NES techniques in spatial and power domains | LG Electronics |
| R1-2303427 | Discussion on cell DTX/DRX mechanism | LG Electronics |
| R1-2303496 | Discussion on spatial and power domain enhancements to support network energy saving | Apple |
| R1-2303497 | Discussion on cell DTX/DRX mechanisms | Apple |
| R1-2303531 | Network energy saving techniques in spatial and power domains | Lenovo |
| R1-2303532 | Enhancements on cell DTX/DRX mechanism | Lenovo |
| R1-2303603 | Techniques in spatial and power domains | Qualcomm Incorporated |
| R1-2303604 | Enhancements on cell DTX and DRX mechanism | Qualcomm Incorporated |
| R1-2303647 | Discussion on cell DTX/DRX mechanism | Rakuten Mobile, Inc |
| R1-2303651 | Network energy savings techniques in spatial and power domains | AT&T |
| R1-2303722 | Discussion on spatial and power domain enhancements for NW energy savings | NTT DOCOMO, INC. |
| R1-2303723 | Discussion on enhancements on Cell DTX/DRX mechanism | NTT DOCOMO, INC. |
| R1-2303757 | NW energy saving techniques in spatial and power domains | Ericsson |
| R1-2303758 | RAN1 aspects of cell DTX/DRX | Ericsson |
| R1-2303780 | Discussion on techniques in spatial and power domains | ITRI |
| R1-2303781 | Discussion on potential enhancements on cell DTX/DRX mechanism for NR | ITRI |
| R1-2303813 | Spatial Domain Adaptation for NES | Fraunhofer IIS, Fraunhofer HHI |
| R1-2303815 | RAN1 Considerations for Cell DTX and DRX | Fraunhofer IIS, Fraunhofer HHI |
| R1-2303850 | Discussion on spatial domain adaptation for NES | KT Corp. |
| R1-2303895 | Summary of issues for enhancements on cell DTX/DRX mechanism | Moderator (Intel Corporation) |
| R1-2303896 | Discussion summary #1 for enhancements on cell DTX/DRX mechanism | Moderator (Intel Corporation) |
| R1-2303910 | Discussions on NES techniques in spatial and power domain | vivo |
| R1-2303913 | FL summary#1 for spatial and power domain techniques for R18 NES | Moderator (Huawei) |
| R1-2303914 | FL summary#2 for spatial and power domain techniques for R18 NES | Moderator (Huawei) |
| R1-2303915 | FL summary#3 for spatial and power domain techniques for R18 NES | Moderator (Huawei) |
| R1-2303916 | FL summary#4 for spatial and power domain techniques for R18 NES | Moderator (Huawei) |
| R1-2303917 | FL summary#5 for spatial and power domain techniques for R18 NES | Moderator (Huawei) |
| R1-2303955 | CSI enhancements for network energy saving | Huawei, HiSilicon |
| R1-2303985 | Discussion on NES techniques in spatial and power domains | ZTE, Sanechips |
| R1-2304014 | Discussion summary #2 for enhancements on cell DTX/DRX mechanism | Moderator (Intel Corporation) |
| R1-2304015 | Discussion summary #3 for enhancements on cell DTX/DRX mechanism | Moderator (Intel Corporation) |
| R1-2304119 | Discussion summary #4 for enhancements on cell DTX/DRX mechanism | Moderator (Intel Corporation) |
| R1-2304120 | Discussion summary #5 for enhancements on cell DTX/DRX mechanism | Moderator (Intel Corporation) |
| R1-2304239 | Discussion summary #6 for enhancements on cell DTX/DRX mechanism | Moderator (Intel Corporation) |
| R1-2304270 | Final FL summary for spatial and power domain techniques for R18 NES | Moderator (Huawei) |

##### RAN1#113

| R1-2304357 | Spatial and Power Adaptations for Network Energy Savings | FUTUREWEI |
| --- | --- | --- |
| R1-2304358 | Cell DTX/DRX for NES | FUTUREWEI |
| R1-2304406 | Techniques in spatial and power domains | Nokia, Nokia Shanghai Bell |
| R1-2304407 | Enhancements on cell DTX/DRX mechanism | Nokia, Nokia Shanghai Bell |
| R1-2304492 | Discussions on NES techniques in spatial and power domain | vivo |
| R1-2304493 | Discussions on enhancements on cell DTX/DRX mechanism | vivo |
| R1-2304527 | Discussion on NES techniques in spatial and power domains | ZTE, Sanechips |
| R1-2304528 | Discussion on cell DTX/DRX | ZTE, Sanechips |
| R1-2304570 | Discussion on NES techniques in spatial and power domains | Spreadtrum Communications |
| R1-2304571 | Discussion on enhancements on cell DTXDRX mechanism | Spreadtrum Communications |
| R1-2304644 | CSI enhancements for network energy saving | Huawei, HiSilicon |
| R1-2304645 | Cell DTX/DRX mechanism for network energy saving | Huawei, HiSilicon |
| R1-2304743 | Network Energy Saving techniques in spatial and power domain | CATT |
| R1-2304744 | DTX/DRX for network Energy Saving | CATT |
| R1-2304772 | Discussion on NW energy saving techniques in spatial and power domains | Fujitsu |
| R1-2304773 | Discussion on cell DTX/DRX mechanism | Fujitsu |
| R1-2304810 | Discussion on NWES techniques in spatial and power domain | Intel Corporation |
| R1-2304811 | Discussion on enhancements on cell DTX/DRX mechanism | Intel Corporation |
| R1-2304848 | Network Energy Saving in Spatial and Power Domain | Google |
| R1-2304849 | Network Energy Saving on Cell DTX and DRX | Google |
| R1-2304861 | Discussion on the spatial and power techniques for NES | China Telecom |
| R1-2304862 | Discussion on the Cell-DTX/DRX | China Telecom |
| R1-2304913 | Discussion on techniques in spatial and power domains | xiaomi |
| R1-2304914 | Discussions on cell DTX-DRX for network energy saving | xiaomi |
| R1-2304946 | Network energy savings techniques in spatial domain | AT&T |
| R1-2305071 | Discussion on network energy saving techniques in spatial and power domains | NEC |
| R1-2305073 | Cell DTX/DRX Configuration for Network Energy Saving | NEC |
| R1-2305106 | Discussion on spatial and power domains enhancements for network energy saving | CMCC |
| R1-2305107 | Discussion on cell DTX DRX enhancements | CMCC |
| R1-2305143 | Discussion on NES techniques in spatial and power domains | LG Electronics |
| R1-2305144 | Discussion on cell DTX/DRX mechanism | LG Electronics |
| R1-2305176 | Discussion on techniques in spatial and power domains | InterDigital, Inc. |
| R1-2305181 | Discussion on enhancements on cell DTX/DRX mechanism | InterDigital, Inc. |
| R1-2305209 | Network energy saving techniques in spatial and power domains | Lenovo |
| R1-2305210 | Enhancements on cell DTX/DRX mechanism | Lenovo |
| R1-2305255 | Discussion on spatial and power domain enhancements to support network energy saving | Apple |
| R1-2305256 | Discussion on cell DTX/DRX enhancements | Apple |
| R1-2305349 | Techniques in spatial and power domains | Qualcomm Incorporated |
| R1-2305350 | Enhancements on cell DTX and DRX mechanism | Qualcomm Incorporated |
| R1-2305432 | Discussion on techniques in spatial and power domains | OPPO |
| R1-2305433 | Discussion on enhancements on cell DTX/DRX mechanism | OPPO |
| R1-2305526 | Techniques in spatial and power domains | Samsung |
| R1-2305527 | Enhancements on cell DTX/DRX mechanism | Samsung |
| R1-2305551 | Discussion on spatial/power domain adaptation | ASUSTeK |
| R1-2305552 | On network energy saving techniques in spatial and power domains | KT Corp. |
| R1-2305553 | Discussion on cell DTX/DRX | ASUSTeK |
| R1-2305573 | NW energy saving techniques in spatial and power domains | Ericsson |
| R1-2305574 | RAN1 aspects of cell DTX/DRX | Ericsson |
| R1-2305608 | Discussion on spatial and power domain enhancements for NW energy savings | NTT DOCOMO, INC. |
| R1-2305609 | Discussion on enhancements on Cell DTX/DRX mechanism | NTT DOCOMO, INC. |
| R1-2305676 | On NW energy saving techniques in spatial and power domains | MediaTek Inc. |
| R1-2305677 | On NW energy saving enhancements for cell DTX/DRX mechanism | MediaTek Inc. |
| R1-2305710 | Discussion of NES techniques in spatial domain and power domain | Transsion Holdings |
| R1-2305711 | Discussion on Enhancement on cell DTX DRX mechanism | Transsion Holdings |
| R1-2305765 | Spatial and power domain adaptation for network energy saving | Panasonic |
| R1-2305766 | Cell DTX/DRX enhancement for network energy saving | Panasonic |
| R1-2305773 | Discussion on techniques in spatial and power domains | ITRI |
| R1-2305774 | Discussion on potential enhancements on cell DTX/DRX mechanism for NR | ITRI |
| R1-2305780 | Discussion on NES for signaling of spatial adaptation | FGI |
| R1-2305798 | Discussion on NES techniques in spatial and power domains | ETRI |
| R1-2305799 | Discussion on cell DTX/DRX mechanism | ETRI |
| R1-2305867 | Spatial Domain Adaptation for NES | Fraunhofer IIS, Fraunhofer HHI |
| R1-2305874 | Signaling and Reliability for Cell DTX and DRX | Fraunhofer IIS, Fraunhofer HHI |
| R1-2305904 | Discussion on spatial and power adaptations for network energy savings | CEWiT |
| R1-2305905 | Discussion on cell DTX/DRX mechanism for network energy saving | CEWiT |
| R1-2306006 | Summary of issues for enhancements on cell DTX/DRX mechanism | Moderator (Intel Corporation) |
| R1-2306007 | Discussion Summary #1 for enhancements on cell DTX/DRX mechanism | Moderator (Intel Corporation) |
| R1-2306011 | Discussion on NES techniques in spatial and power domains | ZTE, Sanechips |
| R1-2306030 | FL summary#1 for SD and PD adaptation for R18 NES | Moderator (Huawei) |
| R1-2306031 | FL summary#2 for SD and PD adaptation for R18 NES | Moderator (Huawei) |
| R1-2306032 | FL summary#3 for SD and PD adaptation for R18 NES | Moderator (Huawei) |
| R1-2306033 | FL summary#4 for SD and PD adaptation for R18 NES | Moderator (Huawei) |
| R1-2306034 | Final FL summary for SD and PD adaptation for R18 NES | Moderator (Huawei) |
| R1-2306110 | Discussion Summary #2 for enhancements on cell DTX/DRX mechanism | Moderator (Intel Corporation) |
| R1-2306111 | Discussion Summary #3 for enhancements on cell DTX/DRX mechanism | Moderator (Intel Corporation) |
| R1-2306169 | Draft reply LS on cell DTX/DRX activation/deactivation | Moderators (Huawei, Intel) |
| R1-2306172 | Discussion Summary #4 for enhancements on cell DTX/DRX mechanism | Moderator (Intel Corporation) |
| R1-2306173 | Discussion Summary #5 for enhancements on cell DTX/DRX mechanism | Moderator (Intel Corporation) |
| R1-2306246 | Reply LS on cell DTX/DRX activation/deactivation | RAN1, Huawei, Intel |

##### RAN2#121bis-e

| TDoc | Title | Source |
| --- | --- | --- |
| R2-2302763 | Cell DTX/DRX impact on C-DRX | CATT, Dell Technologies, Turkcell |
| R2-2302764 | CHO enhancement for NES | CATT, Turkcell |
| R2-2302796 | Outcome of [POST121][312][NES] DTX/DRX - Configuration/ activation/ deactivation and alignment (Huawei) | Huawei, HiSilicon |
| R2-2302797 | Discussion on cell DTX and DRX | Huawei, HiSilicon |
| R2-2302835 | Further discussion on cell DTX and DRX | ZTE Corporation, Sanechips |
| R2-2302837 | Further discussion on connected mode mobility | ZTE Corporation, Sanechips |
| R2-2302914 | Cell DTX-DRX Mechanism | Qualcomm Incorporated |
| R2-2302915 | Barring legacy UEs for NES Cells | Qualcomm Incorporated, T-Mobile US |
| R2-2302925 | NES Connected mode mobility | Qualcomm Incorporated |
| R2-2302976 | Further considerations on Cell DTX and DRX | Intel Corporation |
| R2-2303077 | CHO for NES | Ericsson |
| R2-2303080 | Handover enhancement for NES | Sony |
| R2-2303101 | Work plan for NR network energy savings | Huawei, HiSilicon |
| R2-2303102 | Discussion on CHO enhancement for NES | Huawei, HiSilicon |
| R2-2303128 | CHO on NES | Nokia, Nokia Shanghai Bell |
| R2-2303146 | Discussion on CHO enhancements for NES | Sharp |
| R2-2303152 | Discussion on DTX/DRX mechanism | LG Electronics Inc. |
| R2-2303161 | Triggering conditions and other aspects of the Handover to/from DTX/DRX cells | Vodafone España SA, Apple |
| R2-2303247 | Cell selection/re-selection in NES | Lenovo |
| R2-2303257 | On Cell DTX and DRX | Fraunhofer IIS, Fraunhofer HHI |
| R2-2303259 | Discussion on Connected mode mobility for network energy savings | Fujitsu Limited |
| R2-2303310 | Discussion on DTX/DRX mechanism | OPPO |
| R2-2303311 | Discussion on connected mode mobility | OPPO |
| R2-2303316 | UE and gNB behaviors to support cell DTX/DRX | NEC Telecom MODUS Ltd. |
| R2-2303317 | CHO procedure enhancement to support NES mode | NEC Telecom MODUS Ltd. |
| R2-2303369 | Further discussion on Cell DTX / DRX | Apple |
| R2-2303370 | Discussion on CHO enhancement in NES | Apple |
| R2-2303444 | Expected cell - UE behaviour during cell DTX/DRX | BT plc |
| R2-2303481 | DRAFT LS for Enhanced handovers towards cells with activated cell DTX/DRX or cells which are going to be switched off | Vodafone |
| R2-2303512 | CHO procedure enhancements for NES | Intel Corporation |
| R2-2303514 | Discussion on cell barring and reselection for NES | CMCC |
| R2-2303600 | Cell DTX/DRX mechanism | InterDigital |
| R2-2303601 | Cell selection and resection for NES | InterDigital |
| R2-2303602 | NES mobility aspects | InterDigital |
| R2-2303603 | SSB-less Scell operation | InterDigital |
| R2-2303604 | Report of [POST121][311][NES] DTX/DRX - gNB and UE behaviours | InterDigital |
| R2-2303653 | Alignment to Cell DRX and cell DTX | Lenovo |
| R2-2303654 | CHO Procedure in NES Mode | Lenovo |
| R2-2303663 | Further aspects on cell DTX/DRX | Ericsson |
| R2-2303748 | Discussion on DTX/DRX for NES | Samsung |
| R2-2303749 | Discussion on Connected mode mobility for NES | Samsung |
| R2-2303773 | Discussion on Cell DTX/DRX configuration and operation | Xiaomi |
| R2-2303792 | Discussion on cell DTX/DRX | CMCC |
| R2-2303793 | Discussion on Connected mode mobility enhancement for NES | CMCC |
| R2-2303823 | discussion on cell DTX-DRX mechanism | vivo |
| R2-2303824 | Conditional handover enhancement for network energy saving | vivo |
| R2-2303827 | Issues on Cell DTX/DRX | ETRI |
| R2-2303853 | Discussion on UE mobility due to NES cell | Xiaomi |
| R2-2303860 | Remaining issues on DTX/DRX | Nokia, Nokia Shanghai Bell |
| R2-2303978 | Considerations on Cell DTX/DRX | KDDI Corporation |
| R2-2303984 | Discussion on Cell DRX/DTX | Rakuten Mobile, Inc |
| R2-2304070 | Discussion on Cell selection | NTT DOCOMO INC. |
| R2-2304080 | Discussion on Cell DTX/DRX | NTT DOCOMO INC. |
| R2-2304155 | Discussion on CHO procedure enhancements in case source/target cell is in NES mode | Turkcell |
| R2-2304180 | Connected Mode Mobility | LG Electronics Inc. |
| R2-2304181 | Further considerations on the Cell DTX/DRX | MediaTek Inc. |
| R2-2304353 | Summary of EMAIL DISC [121bis#xxx] on AI 7.3.5 Connected Mode Mobility | Lenovo (Rapporteur) |
| R2-2304357 | DRAFT LS on Cell DTX/DRX activation/deactivation | Huawei |
| R2-2304568 | LS on Cell DTX/DRX activation/deactivation | RAN2 |

##### RAN2#122

| TDoc | Title | Source |
| --- | --- | --- |
| R2-2304627 | LS on the enhancements to restricting paging in a limited area (R3-232084; contact: Nokia) | RAN3 |
| R2-2304691 | Discussion on UE access control in NES cell | Xiaomi |
| R2-2304692 | Discussion on Cell DTX/DRX configuration and operation | Xiaomi |
| R2-2304693 | Discussion on UE mobility due to NES cell | Xiaomi |
| R2-2304694 | Discussion on inter-band SSB-less Scell | Xiaomi |
| R2-2304862 | Enhancements of SBB/SIB-less NES solutions | Dell Technologies |
| R2-2305013 | Remaining issues for Cell DTX_DRX | Samsung Electronics Co., Ltd |
| R2-2305081 | Support high priority traffic in Cell DTX / DRX | Apple, InterDigital, T-Mobile USA, MediaTek Inc., Intel |
| R2-2305082 | Discussion on key open issues of Cell DTX / DRX | Apple |
| R2-2305083 | Discussion on RAN2 work of inter-band SSB-less CA | Apple |
| R2-2305084 | Discussion on CHO enhancement in NES | Apple |
| R2-2305120 | Cell DTX-DRX Mechanism | Qualcomm Incorporated |
| R2-2305121 | Barring legacy UEs for NES Cells | Qualcomm Incorporated, T-Mobile US |
| R2-2305122 | NES Connected mode mobility | Qualcomm Incorporated |
| R2-2305123 | Discussion of RAN3 LS on Restricting Paging | Qualcomm Incorporated |
| R2-2305205 | Discussion on Cell DTX/DRX | Fujitsu |
| R2-2305206 | Discussion on Connected mode mobility for network energy savings | Fujitsu |
| R2-2305250 | Discussion on SSB/SIB-less Solutions for NES | Samsung |
| R2-2305251 | Discussion on Cell Selection and Reselection for NES | Samsung |
| R2-2305252 | Discussion on Connected mode mobility for NES | Samsung |
| R2-2305320 | Discussion on SSB-less SCell operation for NES | ZTE corporation, Sanechips |
| R2-2305321 | Further discussion on cell DTX and DRX | ZTE corporation, Sanechips |
| R2-2305322 | Further discussion on connected mode mobility | ZTE corporation, Sanechips |
| R2-2305323 | Consideration on preventing legacy UEs camping on NES cell | ZTE corporation, Sanechips |
| R2-2305335 | Discussion on cell DTX-DRX mechanism | vivo |
| R2-2305336 | RAN2 impact on supporting inter-band SSB-less Scell operation | vivo |
| R2-2305337 | Discussion on cell selection/re-selection | vivo |
| R2-2305338 | Conditional handover enhancement for network energy saving | vivo |
| R2-2305389 | Discussion on cell DTX and DRX | Huawei, HiSilicon |
| R2-2305390 | Discussion on cell selection/reselection for NES | Huawei, HiSilicon |
| R2-2305435 | Emergency calls, Voice, Scheduling Requests and RACH | Vodafone |
| R2-2305455 | Definition of NES and barring on cell DTX/DRX cells | Vodafone |
| R2-2305461 | Triggering conditions and other aspects of the Handover to/from DTX/DRX cells | Vodafone GmbH |
| R2-2305511 | Handover enhancement for NES | Sony |
| R2-2305512 | Skip monitoring of CSI-RS during non-active periods | Sony |
| R2-2305529 | Discussion on DTX/DRX mechanism | OPPO |
| R2-2305530 | Discussion on cell selection reselection | OPPO |
| R2-2305531 | Discussion on connected mode mobility | OPPO |
| R2-2305628 | Discussion on cell DTX/DRX | CMCC |
| R2-2305629 | Discussion on Connected mode mobility | CMCC |
| R2-2305651 | Remaining issues on DTX/DRX | Nokia, Nokia Shanghai Bell |
| R2-2305718 | Cell selection/re-selection in NES | Lenovo |
| R2-2305721 | Discuss on SSB-less SCell operation in NES | Lenovo |
| R2-2305775 | Discussion on SSB-less SCell operation | CMCC |
| R2-2305776 | Discussion on cell barring and reselection for NES | CMCC |
| R2-2305840 | Further aspects on cell DTX/DRX | Ericsson |
| R2-2305841 | SSB-less Scell operation on inter-band CA for FR1 | Ericsson |
| R2-2305842 | NES Cell selection/reselection | Ericsson |
| R2-2305853 | DL considerations for cell DTX/DRX | NEC Telecom MODUS Ltd. |
| R2-2305855 | UL considerations for Cell DTX/DRX | NEC Telecom MODUS Ltd. |
| R2-2305858 | Procedure for legacy UEs camping on NES cells | NEC Telecom MODUS Ltd. |
| R2-2305860 | CHO for NES | Ericsson |
| R2-2305864 | CHO procedure enhancement to support NES mode | NEC Telecom MODUS Ltd. |
| R2-2305870 | Alignment between Cell DTX/DRX and C-DRX | CATT, Turkcell |
| R2-2305871 | Consideration on Cell Selection/Re-selection on NES cells | CATT, Turkcell |
| R2-2305872 | CHO enhancement for NES | CATT, Turkcell |
| R2-2305890 | CHO Enhancements Supporting NES | Google Inc. |
| R2-2305892 | Cell Reselection Enhancements Supporting NES | Google Inc. |
| R2-2305907 | On NES SSB-less SCell operation | Nokia, Nokia Shanghai Bell |
| R2-2305925 | Cell DTX/DRX mechanism | InterDigital |
| R2-2305926 | Cell selection and resection for NES | InterDigital |
| R2-2305927 | NES mobility aspects | InterDigital |
| R2-2305928 | SSB-less Scell operation | InterDigital |
| R2-2305941 | Various alignment aspects | Lenovo |
| R2-2305942 | CHO Procedure in NES Mode | Lenovo |
| R2-2305974 | Legacy UE Handling for NES | ETRI |
| R2-2305975 | Remaining issues for Cell DTX/DRX | ETRI |
| R2-2306044 | Discussion on DTX/DRX mechanism | LG Electronics Inc. |
| R2-2306052 | Discussion on CHO enhancements for NES | Sharp |
| R2-2306059 | Considerations on Cell selection/re-selection | KDDI Corporation |
| R2-2306067 | Work plan for NR network energy savings | Huawei, HiSilicon |
| R2-2306068 | Discussion on SSB-less SCell operation | Huawei, HiSilicon |
| R2-2306069 | Discussion on CHO enhancement for NES | Huawei, HiSilicon |
| R2-2306074 | Considerations on Cell DTX/DRX | KDDI Corporation |
| R2-2306222 | Cell DTX/DRX NES Techniques | III |
| R2-2306240 | Mobility enhancement: mobility triggering by light handover command | LG Electronics Inc. |
| R2-2306276 | Access control enhancement for NES | LG Electronics France |
| R2-2306329 | Discussion on Cell selection | NTT DOCOMO INC. |
| R2-2306330 | Discussion on SR transmission during the Cell DRX non-active period | NTT DOCOMO INC. |
| R2-2306361 | Reselection and Paging handling for NES | Nokia, Nokia Shanghai Bell |
| R2-2306362 | CHO on NES | Nokia, Nokia Shanghai Bell |
| R2-2306403 | Discussion on cell DTX/DRX mechanisms - configuration and behaviour | BT plc, KDDI |
| R2-2306406 | Identify NES capable UEs by network | BT plc |
| R2-2306407 | Cell DTX and DRX Enhancements | Fraunhofer IIS, Fraunhofer HHI |
| R2-2306410 | Cell Selection and Re-Selection for NES | Fraunhofer IIS, Fraunhofer HHI |
| R2-2306500 | Reminding issues on stage 2 of the Cell DTX/DRX | MediaTek Inc. |
| R2-2306538 | Access control enhancement for NES | LG Electronics France |

##### RAN3-119bis-e

| R3-231122 | LS on 3GPP work on Energy Efficiency | SA5, Huawei |
| --- | --- | --- |
| R3-231211 | Discussion on network energy saving | Samsung |
| R3-231212 | CR to 38.473 for paging enhancements on NES | Samsung |
| R3-231266 | Introduction of Network Energy Saving | Ericsson |
| R3-231277 | (TP for BLCR) Discussion on Inter-node Beam Activation | CATT |
| R3-231278 | (TP for BLCR) Discussion on Paging Enhancement | CATT |
| R3-231297 | (TP for NES WI for 38.473) Considerations on network energy savings | Intel Corporation |
| R3-231298 | Baseline CR to 38.423 on NES | Intel Corporation |
| R3-231348 | Paging enhancements and Inter-node beam activation | Qualcomm Incorporated |
| R3-231407 | Introduction of Network Energy Saving | Ericsson |
| R3-231415 | WI Work plan for R18 network energy savings | Huawei |
| R3-231416 | (TP to TS 38.473, 38.413 and 38.470) Network energy saving techniques | Huawei |
| R3-231494 | Network energy saving techniques | Huawei |
| R3-231540 | Baseline CR to 38.423 on NES | Intel Corporation |
| R3-231595 | Discussion on NES related issues | Lenovo |
| R3-231630 | Discussion on network energy saving techniques | Nokia, Nokia Shanghai Bell |
| R3-231631 | Introduction of Network Energy Saving | Nokia, Nokia Shanghai Bell |
| R3-231781 | (TP to BL CR of 38.473) Paging enhancement | ZTE |
| R3-231782 | (TPs to BL CRs of 38.423_38.473_38.300) Inter-node beam activation | ZTE |
| R3-231897 | CB: # NetworkES_Solutions - Summary of email discussion | Huawei - moderator |
| R3-231983 | Introduction of Network Energy Saving | Ericsson |
| R3-231984 | Network energy saving techniques | Huawei, Samsung, Nokia, Nokia Shanghai Bell, ZTE, Ericsson, Samsung, CATT |
| R3-231985 | LS on the enhancements on restricting paging in a limited area | Huawei |
| R3-232052 | BL CR of 38.300 on Network energy saving techniques | ZTE |
| R3-232082 | Introduction of Network Energy Saving | Ericsson, Huawei, Samsung, ZTE, CATT, Intel |
| R3-232083 | Network energy saving techniques | Huawei, Samsung, Nokia, Nokia Shanghai Bell, ZTE, Ericsson, CATT, Intel |
| R3-232084 | LS on the enhancements on restricting paging in a limited area | Nokia |
| R3-232085 | (BL CR for TS38.300) Network energy saving techniques | ZTE |

##### RAN3-120

| ##### R3-232519 | ##### (BLCR to 38.423) Network energy saving techniques | ##### Huawei, Samsung, Nokia, Nokia Shanghai Bell, ZTE, Ericsson, CATT, Intel |
| --- | --- | --- |
| ##### R3-232577 | ##### LS on Cell DTX/DRX activation/deactivation | ##### RAN2(Huawei) |
| ##### R3-232589 | ##### (TP to TS 38.423) Support of network energy saving techniques | ##### Nokia, Nokia Shanghai Bell |
| ##### R3-232594 | ##### (TP to TS 38.473) Beam deactivation decision and signalling for energy saving | ##### Nokia, Nokia Shanghai Bell |
| ##### R3-232813 | ##### Discussion on network energy saving | ##### Samsung |
| ##### R3-232814 | ##### (CR to 38.413) Introduction of Network Energy Saving for Paging IDLE UE | ##### Samsung |
| ##### R3-232853 | ##### Paging enhancements, Inter-node beam activation and Cell DTX/DRX | ##### Qualcomm Incorporated |
| ##### R3-232889 | ##### WI Work plan for R18 network energy savings | ##### Huawei |
| ##### R3-232890 | ##### (TP to Netw_Energy_NR BLCR for TS 38.423) Network energy saving techniques | ##### Huawei |
| ##### R3-232902 | ##### (TP to Netw_Energy_NR BLCR for TS 38.473, 38.470 and 38.300) Network energy saving techniques | ##### Huawei |
| ##### R3-232968 | ##### Introduction of Network Energy Saving | ##### Ericsson |
| ##### R3-232969 | ##### Text Proposal on F1AP: Introduction of Network Energy Saving | ##### Ericsson |
| ##### R3-233045 | ##### Discussion on NES related issues | ##### Lenovo |
| ##### R3-233246 | ##### Discussion on Paging Enhancement (TP for 38.423) | ##### CATT |
| ##### R3-233273 | ##### (BLCR to 38.473) Introduction of Network Energy Saving | ##### Ericsson, Huawei, Samsung, ZTE, CATT, Intel |
| ##### R3-233284 | ##### Discussion on network energy saving | ##### ZTE |
| ##### R3-233285 | ##### TPs to BL CRs for network energy saving | ##### ZTE |

##### RAN4#106bis

| TDoc | Title | Source |
| --- | --- | --- |
| R4-2304155 | On RRM requirements for NES | Apple |
| R4-2304180 | Feasibility study of SSB-less SCell operation for FR1 inter-band co-located CA | Nokia, Nokia Shanghai Bell |
| R4-2304181 | SSB-less operation for FR1 inter-band co-located CA | Nokia, Nokia Shanghai Bell |
| R4-2304212 | Discussion on synchronization and RF requirements | CMCC |
| R4-2304221 | Discussion on feasibility of SSB-less SCell operation for inter-band CA for FR1 and co-located cells | Huawei, Hisilicon |
| R4-2304229 | NR network energy saving RRM aspects | Intel Corporation |
| R4-2304387 | Discussion on requirements of SSBless Scell at inter-band CA for FR1 | Qualcomm Incorporated |
| R4-2304448 | Discussion on BS RF requirement for network energy saving for NR | CATT |
| R4-2304534 | RF requirements for Network energy savings for NR | Nokia, Nokia Shanghai Bell |
| R4-2304647 | Discussion on RRM impaction for network energy saving | CMCC |
| R4-2304816 | Discussion on SSB-less SCell operation for network energy saving | vivo |
| R4-2304893 | Discussion on general aspects in NES | Ericsson |
| R4-2305002 | Discussion on RRM aspects of Network energy saving for NR | ZTE Corporation |
| R4-2305025 | RF aspects on SSB-less SCell operation for FR1 inter-band CA | China Telecom |
| R4-2305026 | RRM requirements on SSB-less SCell operation for FR1 inter-band CA | China Telecom |
| R4-2305288 | Work plan on network energy savings for NR | Huawei, HiSilicon |
| R4-2305289 | Discussion on RRM impact for network energy savings for NR | Huawei, HiSilicon |
| R4-2305529 | Discussion on RF requirement impacts from NES perspective | ZTE Corporation |
| R4-2305544 | Expected RRM requirements for network energy saving | MediaTek Inc. |
| R4-2306177 | Topic summary for [106-bis-e][229] Netw_Energy_NR | Moderator (Huawei) |
| R4-2306227 | Topic summary for [106-bis-e][148] Netw_Energy_NR | Moderator (Huawei) |
| R4-2306260 | Topic summary for [106-bis-e][229] Netw_Energy_NR | Moderator (Huawei) |
| R4-2306310 | Topic summary for [106-bis-e][148] Netw_Energy_NR | Moderator (Huawei) |
| R4-2306369 | WF on RRM requirements for NR network energy saving | Huawei, HiSilicon |
| R4-2306639 | WF on RF feasibility study of NR Network Energy Saving | Huawei, HiSilicon |
| R4-2306640 | Work plan on network energy savings for NR | Huawei, HiSilicon |

##### RAN4#107

| TDoc | Title | Source |
| --- | --- | --- |
| R4-2307328 | On RRM requirements for NES | Apple |
| R4-2307362 | SSB-less operation for FR1 inter-band co-located CA | Nokia, Nokia Shanghai Bell |
| R4-2307386 | Further discussion on BS RF requirement for network energy saving for NR | CATT |
| R4-2307419 | Discussion on RRM requirements of network energy saving | CATT |
| R4-2307529 | Discussion on BS RF requirement of network Energy savings | Huawei, Hisilicon |
| R4-2307600 | Discussion on RRM impaction for network energy saving | CMCC |
| R4-2307804 | NR network energy saving RRM aspects | Intel Corporation |
| R4-2307823 | Discussion on Rel-18 RF requirement for NES | LG Electronics Inc. |
| R4-2307893 | Discussion on RRM requirements for Network energy saving for NR | MediaTek inc. |
| R4-2307901 | Discussion on Rel-18 RRM requirement for NES | LG Electronics UK |
| R4-2308020 | RF aspects on SSB-less SCell operation for FR1 inter-band CA | China Telecom |
| R4-2308021 | RRM requirements on SSB-less SCell operation for FR1 inter-band CA | China Telecom |
| R4-2308222 | Discussion on SSB-less SCell operation for network energy saving | vivo |
| R4-2308339 | Discussion on RRM impact for network energy savings for NR R17 | Huawei, HiSilicon |
| R4-2308621 | On BS RF requirements for Network energy savings for NR | Nokia, Nokia Shanghai Bell |
| R4-2308730 | Discussion on RRM aspects of Network energy saving for NR | ZTE Corporation |
| R4-2308750 | Discussion on NES BS RF impact | Ericsson |
| R4-2309184 | Discussion on BS RF requirement impacts from NES perspective | ZTE Corporation |
| R4-2309185 | Discussion on UE RF requirement impacts from NES perspective | ZTE Corporation |
| R4-2309428 | Discussion on RRM requirements of SSBless Scell for inter-band CA | Qualcomm Incorporated |
| R4-2309442 | Discussion on network energy saving RF requirements | CMCC |
| R4-2309599 | Discussion on SSB less SCell activation | Ericsson |
| R4-2309979 | Topic summary for [107][234] Netw_Energy_NR | Moderator (Huawei) |
| R4-2310034 | Topic summary for [107][151] Netw_Energy_NR | Moderator (Huawei) |

26.04.2023		minor adaptations for RAN #100

01.02.2023		minor adaptations for RAN #99

27.10.2022		minor adaptations for RAN #98e

01.08.2022		minor adaptations for RAN #97e

21.05.2022		minor adaptations for RAN #96

10.01.2022		minor adaptations for RAN #95e

04.10.2021		minor adaptations for RAN #94e

08.08.2021		minor adaptations for RAN #93e

17.05.2021		minor adaptations for RAN #92e

28.01.2021		minor adaptations for RAN #91e

09.11.2020		minor adaptations for RAN #90e

31.08.2020		minor adaptations for RAN #89e

20.04.2020		minor adaptations for RAN #88e

18.02.2020		minor adaptations for RAN #87e

14.11.2019		minor adaptations for RAN #86

18.08.2019		minor adaptations for RAN #85

12.05.2019		minor adaptations for RAN #84

27.02.2019		minor adaptations for RAN #83

21.11.2018		completion levels with colours added (for RAN #82)

v04.81	31.07.2018		simplification of template and addition of cross-TSG aspects (for RAN #81)

v04.80	21.05.2018		minor adaptations for RAN #80

v04.79	26.02.2018		minor adaptations for RAN #79

v04.78	18.11.2017		minor adaptations for RAN #78

v04.77	06.08.2017		minor adaptations for RAN #77

v04.76	15.05.2017		minor adaptations for RAN #76

v04.75	31.01.2017		minor adaptations for RAN #75

v04.74	28.10.2016		minor adaptations for RAN #74

v04.73	01.09.2016		adaptations for RAN #73 (time units in extra Excel table, RAN6 reporting included)

v04.72	26.05.2016		adaptations for RAN #72 (introduction of NR & GERAN TUs)

v04.71	10.02.2016		minor adaptations for RAN #71

v04.70	30.10.2015		minor adaptations for RAN #70

v04.69	12.08.2015		minor adaptations for RAN #69

v04.68	21.05.2015		minor adaptations for RAN #68

v04.67	01.02.2015		minor adaptations for RAN #67

v04.66	16.11.2014		minor adaptations for RAN #66

v04.65	16.08.2014		minor adaptations for RAN #65

v04.64	22.05.2014		minor adaptations for RAN #64

v04.63	24.01.2014		restructuring for RAN #63 to cover Core & Perf. in one doc file

v03.62	11.11.2013		section 1.2.3 adapted for RAN #62

v03	11.08.2013		section 1.2.3 added on time budget

v02	07.05.2010		history added, some spelling corrections

v01	13.11.2009		First version of the template