3GPP TSG RAN Meeting #100       	                                                     RP-231015

Taipei, June 12 – 14, 2023

Agenda Item:	9.2.2

Source:	Spreadtrum Communications

Title:	Further discussion on Ambient IoT in RAN

Document for:	Discussion and decision

Introduction

According to the status report from the last meeting [1], there are still the following remaining issues with this topic:

- To conclude on device characteristics and categorization

- To conclude on entries and descriptions of deployment scenario characteristics

- Complete the set of RAN design targets based on the identified deployment scenarios and their characteristics for the relevant use cases

- Compare and assess the feasibility of meeting the design targets for relevant use case on the basis of the deployment scenario(s) appropriate to it, and identify assumptions on required functionality to be supported.

Based on the above, we will mainly discuss the following topics in this contribution:

- Deployment scenario characteristics

- Device categorization

- RAN design targets

- Comparison and assessment

Deployment scenarios

The following proposal has been endorsed in the last meeting [2].

Proposal 3.5#2 (WED EVE offline): The deployment scenario tables are updated as shown below. Deployment 1 (WED EVE offline consensus): Device indoors, base station indoors Applicable representative use cases; Characteristics; Description. indoor inventory indoor sensor indoor positioning indoor command; Environment (of device); Indoor. Basestation characteristic (if any); Micro- or pico- cell BS. Connectivity topology; Topology (1),[(2)],[(3)]. Spectrum; Licensed FDD, Licensed TDD or Unlicensed. Coexistence with existing 3GPP technologies; Co-site or new site. Traffic assumption; Device terminated and originated. Device characteristic; Device A or B or [C]. Deployment 2: Device indoors, base station outdoors Applicable representative use cases; Characteristics; Description. indoor inventory indoor sensor indoor positioning indoor command; Environment (of device); Indoor. Basestation characteristic (if any); Macro- or Micro- cell BS. Connectivity topology; Topology (1),(2),(3) Note: topology (3) where either the uplink or the downlink coverage is extended. Spectrum; Licensed FDD, or Licensed TDD or unlicensed (for topology (2) and (3)). Coexistence with existing 3GPP technologies; Co-site or new site. Traffic assumption; Device terminated and originated. Device characteristic; Device [A or B] or C. Deployment 3: Device indoors, UE indoors based reader Applicable representative use cases; Characteristics; Description. indoor inventory [indoor sensor] indoor positioning indoor command; Environment (of device); Indoor. Basestation characteristic (if any); No base station in the topology. Connectivity topology; Topology (4). Spectrum; Licensed FDD, Licensed TDD or Unlicensed. Coexistence with existing 3GPP technologies; FFS-[Legacy UE or new UE]. Traffic assumption; Device terminated and originated. Device characteristic; Device A or B or [C] Note: a device C could also support such deployment. Deployment 4: Device outdoors, Outdoor base station outdoors Applicable representative use cases; Characteristics; Description. outdoor inventory outdoor sensor outdoor positioning outdoor command; Environment (of device); Outdoor. Basestation characteristic (if any); Macro- or Micro- cell BS. Connectivity topology; Topology (1),(2),(3). Spectrum; Licensed FDD or Licensed TDD. Coexistence with existing 3GPP technologies; Co-site or new site. Traffic assumption; Device terminated and originated. Device characteristic; Device [A or B] or C. Deployment 5: Device outdoors, UE outdoors based reader Applicable representative use cases; Characteristics; Description. outdoor inventory [outdoor sensor] outdoor positioning outdoor command; Environment (of device); Outdoor. Basestation characteristic (if any); No base station in the topology. Connectivity topology; Topology (4). Spectrum; Licensed FDD or Licensed TDD or Unlicensed. Coexistence with existing 3GPP technologies; FFS-[Legacy UE or new UE]. Traffic assumption; Device terminated and originated. Device characteristic; Device A or B or C.

For deployment 2, combined with the simulation evaluation in section 5.1.2.2, Topo1 and Topo3 cannot work for devices A and B in this scenario. Even for device C, it is necessary to improve the sensitivity of the downlink receiver by over -80 dbm, but this will significantly increase the receiver's energy consumption. Therefore, it is recommended to lower the research priority of this scenario.

Proposal 1: De-prioritize the study on the deployment 2.

In deployment scenarios 3 and 5, for the issue of coexistence with existing 3GPP technologies, it is recommended to refer to the compatibility considerations in NR V2X study. The WID [3] of NR V2X has the following description:

- UE Tx and Rx RF requirement [RAN4] - This requirement should ensure - coexistence between sidelink and Uu interface in the same and adjacent channels in licensed spectrum - coexistence with other V2X technologies in the adjacent channel in ITS spectrum in 5.9 GHz, without assuming that 5.9 GHz spectrum will be universally available nor that it will be universally available in sufficient quantity to support NR V2X advanced use cases

According to the above, the deployment 3 and 5 also need to consider the compatibility of two aspects. One is compatible with the Uu interface in the same and adjacent channels in licensed spectrum, and the other is compatible with other sidelink technologies in the same and adjacent channels.

Proposal 2: Fill the Coexistence with existing 3GPP technologies attribute of deployment 3 and 5 with ‘NR Uu interface and/or other NR sidelink technologies’.

For deployment 4, the choice of topology mainly depends on the size of the station distance. When the distance between stations is small, it is not only possible to use base station direct connection, but also to use RF power supply by increasing the bandwidth of energy collection. However, when the distance between stations increases, not only does it need to improve the sensitivity of the receiver, but it may also need to consider other energy supply methods, such as solar energy. Therefore, considering that the actual deployment environment mostly has station distance of over 300m and the environment is also more harsh, it is recommended to prioritize the study of Device C in this scenario.

Proposal 3: Prioritize the study of Device C in deployment 4.

Device characteristics and categorization

In the last meeting, the moderator provided the following preliminary proposal based on the offline discussion [4].

Proposal 5.1-1-v3: The TR captures energy storage levels as: - Option 1: Two levels - Storage 1: Without energy storage - Storage 2: With limited energy storage. - Option 2: Three levels - Storage 1: Without energy storage - Storage 2 and 3: Up to E1/E2 joules - For this case, 2 exemplary traffic models are chosen, and E1, E2 calculated accordingly. - Model 1: power = per-device design target, rate = 5 kbps, message size = {as agreed under design target}. - Model 2: power = per-device design target, traffic = continuously ON until end of an inventory process. - Option 3: Status quo, i.e. TR reports the three storage levels, and that E2 > E1. (If no agreement, this option results).

The purpose of discussing device storage levels is to provide guidance and reference for future discussions in the RAN subgroup. The classification of storage capacity in Option 1 is too vague, thus providing limited value. The storage level classification method of Option 2 has the following advantages: firstly, it can provide a basis for device classification methods and their complexity evaluation in design targets; secondly, it can provide reference for implementation; In addition, it can also provide some guidance for future standard design, and so on. Option 3 is a compromise solution. Therefore, we prefer the classification of option 2 and use a model similar to Model 2 to calculate E1/E2 as follows:

For E1, assuming that the power consumption of the device is 10uw and the latency of the service is 10s, the storage energy required to complete a service is 10μW * 10s=100uJ.

For E2, assuming that the power consumption of the device is 1mw and the latency of the service is 10s, the storage energy required to complete a service is 1mW * 10s=1mJ.

Table 1. Storage capacity threshold (unit: joules)

|  | Capacitor/Super capacitor |
| --- | --- |
| Storage 1 | no |
| Storage 2 | 100uJ |
| Storage 3 | 1mJ |

Proposal 4: Option 2 is adopted in TR to capture energy storage levels with E1 as 100uJ and E2 as 1mJ.

RAN design targets

Regarding design targets, the following agreements have been reached in the RAN 99th meeting [2].

Agree to set at least the design targets below in Ambient IoT in the RAN SI. - Device power consumption - Device complexity - Coverage - Data rate - Maximum message size (or maximum ‘TB’ size) - Latency - Positioning accuracy - Connection/device density - Device speed (FFS absolute or relative or both) Proposal 6-3a-v2 (Wed offline consensus): Device design target for power consumption during transmitting/receiving is: - [Device A ≤ 10 μW] or [Device A ≤ 1 μW] - Device A ≪ Device B < Device C, or Device A ≤ Device B < Device C - Device C ≤ 1 mW to ≤ 10 mW Proposed working assumption (Wed offline consensus): Device complexity design target is: - Device A: Comparable to UHF RFID ISO18000-6C (EPC C1G2) - Device A < Device B < Device C - Device C: Orders-of-magnitude lower than NB-IoT Proposal 6-3d-v2 (Wed offline consensus): User experienced data rate design target is, at least for the uplink: - Maximum not less than: 5 kbps - Minimum not less than 0.1 kbps

For the design goal of power consumption, it is recommended that the power consumption target of Device A is 10uw. There are several reasons for this. Firstly, the design target of the existing Passive RFID is 10uw [5], and our target is to exceed its performance. Therefore, it is not recommended to lower the power consumption than it. Secondly, we also need to support FDD, which will have a slightly higher implementation complexity compared to existing passive RFID. Therefore, based on the above two points, it is recommended that the power consumption target for device A is 10uw.

For the design complexity, the current working assumption fully conforms to the SI targets of Ambient IOT. Therefore, we suggest confirming the working assumption of the last meeting.

Proposal 5: Adopt Device A ≤ 10 μW as the power consumption target of Device A, and confirm the working assumption of device complexity.

For other design targets, no agreements were formed in the last meeting, but there are preliminary discussion results as follows [4].

Proposal 6-3c-v3: A coverage design target is set per connectivity topology. - Design target(s) can be the same or different among the topologies. - For topology (1), (2), and (3), the coverage reference points are the BS and the tag. - For topology (4), the coverage reference points are the UE and the tag. Proposal 6-3e-v2: Maximum message size design target is: - Hundreds of bits, and < 1000 bits RAN1 to refine as needed e.g. for TB size design. Proposal 6-3f-v3: Latency is defined as the end-to-end latency. - FFS: Value/values - FFS: Whether to define a minimum and a maximum latency, or only one - FFS: Whether to differentiate latency among DO and DT traffic. Assume that RAN WGs would refine this overall target into e.g. user plane latency, control plane latency, or etc. Proposal 6-3g: Positioning accuracy design target is: - Indoor: 3 m @ 90% - Outdoor: several 10 m @ 90% Proposal 6-3h: Connection/device density design target is: - 150 connections/devices per 100 m2

Based on the original requirement data of use cases in the TR 22.840 of SA1, we suggest confirming the design targets of maximum message size, positioning accuracy, and Connection/device density. In addition, considering the diversity of coverage targets under different topologies, it is also recommended to confirm the proposal of the coverage design target.

Proposal 6: The following proposals from RAN#99 Moderator’ summary can be agreed.

  - Proposal 6-3c-v3: A coverage design target is set per connectivity topology.

  - Design target(s) can be the same or different among the topologies.

  - For topology (1), (2), and (3), the coverage reference points are the BS and the tag.

  - For topology (4), the coverage reference points are the UE and the tag.

  - Proposal 6-3e-v2: Maximum message size design target is:

  - Hundreds of bits, and < 1000 bits

  - Proposal 6-3g: Positioning accuracy design target is:

  - Indoor: 3 m @ 90%

  - Outdoor: several 10 m @ 90%

  - Proposal 6-3h: Connection/device density design target is:

  - 150 connections/devices per 100 m2

When discussing the design targets of latency and speed, it is necessary to consider that our objective is to design a standard with extremely low complexity and low power consumption. Therefore, we need to reduce the requirements for latency and speed. Therefore, for the design target of latency, considering the use cases of SA1, we suggest defining a second level latency without distinguishing between traffic types, with a specific value of around 10 seconds. For the design target of speed, it is best not to introduce too many additional designs. Therefore, we recommend a target of indoor speed of 6km/h and outdoor speed of 20km/h.

Proposal 7: Adopt the following targets values for latency and speed:

- Max E2E latency:10s;

- Device speed

- 6 km/h (indoor)

- 20 km/h (outdoor)

Comparison and assessment

According to the requirements of SID [6], we need to compare and assess the feasibility of meeting the design targets for the relevant use case based on the deployment scenario.

- Compare and assess the feasibility of meeting the design targets for relevant use case on the basis of the deployment scenario(s) appropriate to it, and identify assumptions on required functionality to be supported. NOTE: This is not to require a detailed WG-level of analysis. Note: This study shall target for an IoT segment well below the existing 3GPP IoT technologies, e.g. NB-IoT, eMTC, RedCap, etc. The study shall not aim to replace existing 3GPP LPWA technologies.

Among the design targets in Chapter 4, the main one to be assessed is the coverage design target, while other design targets are relatively easy to meet the requirements. The main reasons for the difficulty in achieving the coverage design objectives are as follows:

(1) In some scenarios, it needs to rely on RF to collect energy, while the current industry's RF energy collection threshold requirements are high, about - 20~- 30 dB;

(2) The decrease of terminal capability leads to the increase of sensitivity in the downlink, and the transmission power in the uplink based on backscatter is significantly reduced.

Therefore, in this section, we mainly assess the coverage design targets under various deployment scenarios. The assessment is divided into two types of scenarios: base station outdoor and base station indoor. Large-scale assessment is mainly adopted, and the large-scale model refers to 3GPP TR 38.901[7].

Base station outdoor scenario evaluation

This scenario is to evaluate the outdoor deployment of the base station, including two deployment scenarios of tag: outdoor and indoor.

Link budget

The outdoor evaluation mainly focuses on UMA NLOS/LOS and free space path loss model. The former is for 2C urban scenario, and the latter is mainly for 2B outdoor scenario. The specific formula of path loss is as follows:

UMA LOS:    PL=28+22*log（d3D)+20*log(fc)

UMA NLOS: PL=13.54+39.08*log（d3D)+20*log(fc)-0.6*(hUT-1.5)

Free space path loss model: PL=32.4+20*log（d3D)+20*log(fc)

Table 2. Comparison of coverage for outdoor deployments（Macro station）

| Tx Parameters/Assumptions | UMA NLOS | UMA LOS | Free space path loss model |
| --- | --- | --- | --- |
| Carrier frequency (GHz) | 0.8 | 0.8 | 0.8 |
| Bandwidth(MHz) | 20M | 20M | 20M |
| Tx power (dBm) | 48 | 48 | 48 |
| Tx antenna gain (dB) | 18 | 18 | 18 |
| RF source->IoT device Distance (m) | 200 | 200 | 200 |
| path loss (dB) | 101.78 | 76.78 | 76.57 |
| Received At Tag (dB) |  |  |  |
| RSRP | -66.57 | -41.57 | -41.36 |
| Return loss | 8 | 8 | 8 |
| Ambient IoT device Ant Gain | 0 | 0 | 0 |
| Ambient IoT device Tx EIRP (dBm) | -74.57 | -49.57 | -49.36 |
| Basestation Parameters/Assumptions |  |  |  |
| Basestation Ant gain (dBi) | 18 | 18 | 18 |
| Receiver sensitivity (dBm) | -100 | -100 | -100 |
| MCL/Coverage For Backscatter Link |  |  |  |
| MCL (backscatter link (dB)) | 43.43 | 68.43 | 68.64 |
| Coverage (backscatter link (meters)) | 1 | 75 | 76 |

Table 3. Comparison of coverage for outdoor deployments（Micro station）

| Tx Parameters/Assumptions | UMA LOS | UMA LOS | UMA LOS |
| --- | --- | --- | --- |
| Carrier frequency (GHz) | 0.8 | 0.8 | 0.8 |
| Bandwidth(MHz) | 20M | 20M | 20M |
| Tx power (dBm) | 43 | 43 | 43 |
| Tx antenna gain (dB) | 8 | 8 | 8 |
| RF source->IoT device Distance (m) | 20 | 50 | 100 |
| path loss (dB) | 58.11 | 64.75 | 70.54 |
| Received At Tag (dB) |  |  |  |
| RSRP | -37.9 | -44.54 | -50.33 |
| Return loss | 8 | 8 | 8 |
| Ambient IoT device Ant Gain | 0 | 0 | 0 |
| Ambient IoT device Tx EIRP (dBm) | -45.9 | -52.54 | -58.33 |
| Basestation Parameters/Assumptions |  |  |  |
| Basestation Ant gain (dBi) | 8 | 8 | 8 |
| Receiver sensitivity (dBm) | -100 | -100 | -100 |
| MCL/Coverage For Backscatter Link |  |  |  |
| MCL (backscatter link (dB)) | 62.1 | 55.46 | 49.67 |
| Coverage (backscatter link (meters)) | 40 | 15 | 0 |

Observation 1: Based on the above analysis, it is difficult for outdoor cellular coverage to meet the RF energy collection threshold.

Observation 2: Outdoor uplink coverage may still be a bottleneck in macro NLOS and some micro LOS conditions.

Table 4. Comparison of coverage for indoor deployments

| Tx Parameters/Assumptions | Home |
| --- | --- |
|  | UMA NLOS+ O2I penetration loss |
| Carrier frequency (GHz) | 0.8 |
| Bandwidth(MHz) | 20M |
| Tx power (dBm) | 48 |
| Tx antenna gain (dB) | 18 |
| Penetration loss(dB) | 20 |
| RF source->IoT device Distance (m) | 200 |
| path loss (dB) | 101.78 |
| Received At Tag (dB) |  |
| RSRP | -94.57 |
| Return loss | 8 |
| Ambient IoT device Ant Gain | 0 |
| Ambient IoT device Tx EIRP (dBm) | -112.57 |
| Basestation Parameters/Assumptions |  |
| Basestation Ant gain (dBi) | 18 |
| Receiver sensitivity (dBm) | -100 |
| MCL/Coverage For Backscatter Link |  |
| MCL (backscatter link (dB)) | 5.43 |
| Coverage (backscatter link (meters)) | 0 |

Observation 3: In deployment 2, RSRP cannot meet the requirements of energy harvesting and demodulation threshold.

System simulation

Simulation assumptions

Table 5. Simulation Assumptions for Outdoor Deployment of Base Stations

| Tx Parameters/Assumptions | UMa | UMa |
| --- | --- | --- |
| Carrier frequency (GHz) | 800 MHz | 2.6/3.5 GHz |
| Bandwidth(MHz) | 20 MHz | 100 MHz |
| SCS | 15 kHz | 30 kHz |
| Number of Sites | 19 | 19 |
| ISD(:meter) | 500/400/300/200/100 | 500/400/300/200/100 |
| BS Tx power | 44 dBm | 53 dBm |
| BS antenna configurations | [M,N,P,Mg,Ng]=[4,2,2,1,1], 3-sector antenna | [M,N,P,Mg,Ng]=[12,8,2,1,1], 3 sector antenna |
| BS port mapping | [K,L]=[4,1], 4TXRU | [K,L]=[3,1], 64TXRU |
| BS antenna electrical downtilting | 102 | 102 |
| UT antenna configurations | 1 elements, Isotropic antenna gain pattern | 1 elements, Isotropic antenna gain pattern |
| UT distribution | 3D dropping 80% indoor/100% indoor/100%/outdoor | 3D dropping 80% indoor/100% indoor/100%/outdoor |
| UT attachment | Based on RSRP | Based on RSRP |
| O2I penetration loss | 50% low loss and 50% high loss | 50% low loss and 50% high loss |

Simulation result

In the simulation results, we first divide the users into indoor and outdoor scenarios for simulation, and then provide two parts of results for each scenario. The first part of the results is the CDF results of RSRP under normal UE random scattering, which are used to determine the demodulation performance of the users. The second part is to use full bandwidth power to calculate RSRP, in order to estimate the energy that users can collect.

- TAG Outdoor

  - Normal RSCP CDF diagram

Figure 3: 800M/2.6G/3.5G UE outdoor distribution RSCP CDF

Observation 4: In outdoor scenarios, when the distance between stations increases, the downlink demodulation threshold is difficult to meet.

  - RSCP CDF diagram for full bandwidth reception

Figure 2: 800M/2.6G/3.5G UE outdoor distribution RSCP CDF with full bandwidth reception

Observation 5: By increasing the RF bandwidth for energy harvesting, energy intensity can be significantly improved, but the threshold for energy harvesting still cannot be met when the distance between stations is increased.

Proposal 8: Consider other energy harvesting in addition to RF in outdoor conditions, for example, solar energy, etc.

Proposal 9: Relay topology should be considered in the outdoor environment.

- TAG Indoor

  - Normal RSCP CDF diagram

Figure 3: 800M/2.6G/3.5G UE indoor distribution RSCP CDF

Observation 6: In the scenario where outdoor base stations cover indoor users, due to penetration loss, RSRP cannot meet the sensitivity requirements of receivers even when the distance between base stations is small.

  - RSCP CDF diagram for full bandwidth reception

Figure 4: 800M/2.6G/3.5G UE indoor distribution RSCP CDF with full bandwidth reception

Observation 7: In the scenario where outdoor base stations cover indoor users, even full bandwidth reception cannot meet the energy harvesting threshold requirements.

Base station indoor scenario evaluation

Link budget

The indoor evaluation mainly focuses on the path loss model in the factory and home. Refer to 38.901 for the specific formula of path loss.

Table 6. Comparison of coverage for indoor deployments

| Tx Parameters /Assumptions | NLOS |  |  |  | InF-LOS |
| --- | --- | --- | --- | --- | --- |
|  | InF-SL | InF-DL | InF-SH | InF-DH |  |
| Carrier frequency (GHz) | 0.8 | 0.8 | 0.8 | 0.8 | 0.8 |
| Bandwidth(MHz) | 20M | 20M | 20M | 20M | 20M |
| Tx power (dBm) | 30 | 30 | 30 | 30 | 30 |
| Tx antenna gain (dB) | 10 | 10 | 10 | 10 | 10 |
| Penetration loss(dB) | 0 | 0 | 0 | 0 | 0 |
| RF source->IoT device Distance (m) | 20 | 20 | 20 | 20 | 20 |
| path loss (dB) | 65.47 | 64.84 | 63.85 | 63.48 | 59.01 |
| Received At Tag (dB) |  |  |  |  |  |
| RSRP | -66.27 | -65.63 | -64.64 | -64.27 | -59.80 |
| Return loss | 8 | 8 | 8 | 8 | 8 |
| Ambient IoT device Ant Gain | 0 | 0 | 0 | 0 | 0 |
| Ambient IoT device Tx EIRP (dBm) | -74.27 | -73.63 | -72.64 | -72.27 | -67.8 |
| Basestation Parameters/Assumptions |  |  |  |  |  |
| Basestation Ant gain (dBi) | 10 | 10 | 10 | 10 | 10 |
| Receiver sensitivity (dBm) | -100 | -100 | -100 | -100 | -100 |
| MCL/Coverage For Backscatter Link |  |  |  |  |  |
| MCL (backscatter link (dB)) | 35.73 | 36.37 | 37.36 | 37.73 | 42.2 |
| Coverage (backscatter link (meters)) | 0 | 0 | 0 | 0 | 0 |

Observation 8: In Deployment 1, RSRP can meet the threshold requirements of downlink demodulation, but cannot meet the threshold requirements of energy harvesting due to the height of the base station and the limitation of RF transmission power.

System simulation

The indoor simulation only simulates the Indoor Office scenario, and the site layout refers to the layout in [7]. The simulation results also include two parts. The first part is the normal user distribution RSCP CDF graph, and the second part is the RSCP CDF graph calculated based on full bandwidth transmission power, with the aim of estimating the energy that can be collected by the device.

Simulation assumptions

Table 7. Simulation Assumptions for Indoor Deployment of Base Stations

| Tx Parameters/Assumptions | Indoor Office |
| --- | --- |
| Carrier frequency (GHz) | 0.8/2.6/3.5 GHz |
| Bandwidth(MHz) | 20/100/100 MHz |
| SCS | 15/30/30 kHz |
| Number of sites | 12 |
| ISD | 20 |
| BS Tx power | 24 dBm |
| BS antenna configurations | [M,N,P,Mg,Ng]=[4,2,2,1,1] ceiling antenna |
| BS port mapping | [K,L]=[4,1], 4TXRU |
| BS antenna electrical downtilting | 90 |
| UT antenna configurations | 1 elements, Isotropic antenna gain pattern |
| UT distribution | 2D dropping 100% indoor |
| UT attachment | Based on RSRP |

Simulation result

- Normal RSCP CDF diagram

Figure 5: 800M/2.6G/3.5G RSCP CDF

Observation 9: In indoor environments, low band FDD can meet coverage requirement, while for high band, due to lack of high antenna gain, it cannot meet coverage requirement.

- RSCP CDF diagram for full bandwidth reception

Figure 6: 800M/2.6G/3.5G UE RSCP CDF with full bandwidth reception

Observation 10: In indoor FDD spectrum scenarios, the threshold for RF energy harvesting can be met by increasing the bandwidth of energy harvesting.

Proposal 10: Consider increasing the bandwidth of energy harvesting as an implementation option.

Proposal 11: Adopt the following priority order (from high to low) for study spectrum: licensed FDD, licensed TDD, unlicensed.

Conclusion

In this contribution, we further discuss on Ambient IoT with the the following observations and proposals.

Observation 1: Based on the above analysis, it is difficult for outdoor cellular coverage to meet the RF energy collection threshold.

Observation 2: Outdoor uplink coverage may still be a bottleneck in macro NLOS and some micro LOS conditions.

Observation 3: In deployment 2, RSRP cannot meet the requirements of energy harvesting and demodulation threshold.

Observation 4: In outdoor scenarios, when the distance between stations increases, the downlink demodulation threshold is difficult to meet.

Observation 5: By increasing the RF bandwidth for energy harvesting, energy intensity can be significantly improved, but the threshold for energy harvesting still cannot be met when the distance between stations is increased.

Observation 6: In the scenario where outdoor base stations cover indoor users, due to penetration loss, RSRP cannot meet the sensitivity requirements of receivers even when the distance between base stations is small.

Observation 7: In the scenario where outdoor base stations cover indoor users, even full bandwidth reception cannot meet the energy harvesting threshold requirements.

Observation 8: In Deployment 1, RSRP can meet the threshold requirements of downlink demodulation, but cannot meet the threshold requirements of energy harvesting due to the height of the base station and the limitation of RF transmission power.

Observation 9: In indoor environments, low band FDD can meet coverage requirement, while for high band, due to lack of high antenna gain, it cannot meet coverage requirement.

Observation 10: In indoor FDD spectrum scenarios, the threshold for RF energy harvesting can be met by increasing the bandwidth of energy harvesting.

Proposal 1: De-prioritize the study on the deployment 2.

Proposal 2: Fill the Coexistence with existing 3GPP technologies attribute of deployment 3 and 5 with ‘NR Uu interface and/or other NR sidelink technologies’.

Proposal 3: Prioritize the study of Device C in deployment 4.

Proposal 4: Option 2 is adopted in TR to capture energy storage levels with E1 as 100uJ and E2 as 1mJ.

Proposal 6: The following proposals from RAN#99 Moderator’ summary can be agreed.

  - Proposal 6-3c-v3: A coverage design target is set per connectivity topology.

  - Design target(s) can be the same or different among the topologies.

  - For topology (1), (2), and (3), the coverage reference points are the BS and the tag.

  - For topology (4), the coverage reference points are the UE and the tag.

  - Proposal 6-3e-v2: Maximum message size design target is:

  - Hundreds of bits, and < 1000 bits

  - Proposal 6-3g: Positioning accuracy design target is:

  - Indoor: 3 m @ 90%

  - Outdoor: several 10 m @ 90%

  - Proposal 6-3h: Connection/device density design target is:

  - 150 connections/devices per 100 m2

Proposal 7: Adopt the following targets values for latency and speed:

- Max E2E latency:10s;

- Device speed

- 6 km/h (indoor)

- 20 km/h (outdoor)

Proposal 8: Consider other energy harvesting in addition to RF in outdoor conditions, for example, solar energy, etc.

Proposal 9: Relay topology should be considered in the outdoor environment.

Proposal 10: Consider increasing the bandwidth of energy harvesting as an implementation option.

Proposal 11: Adopt the following priority order (from high to low) for study spectrum: licensed FDD, licensed TDD, unlicensed.

References

- RP-230669, Status Report to TSG, 3GPP TSG RAN Meeting #99,

- Meeting report for RAN # 99th meeting, draft_MeetingReport_RAN_99_230323_eom

- RP-200129 Revised WID on 5G V2X with NR sidelink,3GPP TSG RAN Meeting #87e, Electronic Meeting, March 16-19, 2020

- RP-230801, Results of offline discussions on ambient IoT, 3GPP TSG RAN Meeting #99

- A −20 dBm Passive UHF RFID Tag IC With MTP NVM in 0.13-μm Standard CMOS Process, Songting Li , Cong Li, Lei Cai, Yu Xiao, Zhipeng Luo, Dun Yan, Shunqiang Xu, and Hongyi Wang, IEEE TRANSACTIONS ON CIRCUITS AND SYSTEMS—I: REGULAR PAPERS, VOL. 67, NO. 12, DECEMBER 2020

- RP-222685 New SID: Study on Ambient IoT

- 3GPP TR 38.901