3GPP TSG RAN meeting #100										RP-231178

Taipei, Taiwan, June 12-16, 2023

## Status Report to TSG

Agenda item:			9.4.1.3

| WI / SI Name | Introduction of the satellite L-/S-band for NR |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| included in this status report | Study Item: No | Core part: Yes | Performance part: Yes |  | Testing part: No |
| Acronym | NR_NTN_LSband |  |  |  |  |
| Unique ID | 981043 |  |  |  |  |
| TSG Tdoc of latest approved WI/SI description (if any) | RP-230706 |  |  |  |  |
| Target Completion Date (indicate if changed) | Study Item: | Core part: 12/2023 | Performance part: 12/2023 | Testing part: |  |
| Overall Completion level | Study Item: | Core part: 50% | Performance Part: 90% | Testing part: |  |

Note: Overall completion level percentage numbers should use one of the colors below:

- xx%: Normal progress, no RAN plenary action needed

- xx%: Progress behind schedule, may need RAN plenary intervention. If so, SR should clearly define requested action

- xx%: Progress critically behind, RAN plenary shall intervene. SR should define requested action

Source:

| Leading WG |  | RAN WG4 |
| --- | --- | --- |
| Rapporteur | Name | Alexander Sayenko |
|  | Company | Apple Inc. |
|  | Email | asayenko@apple.com |

## 1	Work plan related evaluation

| Do you want to modify the time budget for this WI/SI compared to what was endorsed at the last RAN meeting? | No |
| --- | --- |

If you answered No:	Then please remove the Excel file from the zip file of this status report.

If you answered Yes:	Then please fill out the attached Excel template to request a modification of the time 		budgets for your WI /SI. The Excel table has to be filled out for all affected RAN WGs and 		up to the target date of the WI/SI. The basis are the endorsed time budgets of the last 		RAN meeting. Please highlight all changes of the values.
		One time unit (TU) corresponds to ~ 2 hours in the meeting.
		If this status report covers a WI with Core and Performance part, then please have one 		line for each in the attached Excel table.
		Note: If no Excel table is attached, then this means no time budget change.

Additional explanations/motivations for the time budget changes in the attached Excel table:

## 2.	Detailed progress in RAN WGs since last TSG meeting (for all involved WGs)

NOTE: Agreements and Open issues impacted cross-TSG aspects shall be explicitly highlighted

## 2.1	RAN1

## 2.2	RAN2

## 2.3	RAN3

## 2.4	RAN4

#### 2.4.1	Agreements

UE RF agreements

-	General aspects and system parameters

-	To be discussed further whether there are any co-existence issues with bands n53 and n41/n90.

-	All the related regulatory requirements should be taken into account.

-	Identify the countries where the co-existence requirement and regulatory requirements apply

-	FFS on whether to apply for GEO only or for other types of satellites

-	n254 is reserved for the band defined for WI NR_NTN_LS band

-	n254 is defined as UE transmitting at 1610-1626.5MHz and SAN transmitting at 2483.5-2500MHz

-	5MHz, 10MHz and [15MHz] channel bandwidths are specified for the band

NOTE: The 15MHz channel bandwidth may need further technical discussions.

-	The following channel raster points are specified for the band, UL channel raster points at 322000 – 325300 and the DL channel raster points at 496700 –500000.

-	"Case A" sync pattern is specified for the band with the sync raster points at 6215 – 6244 in steps of <1>.

-	15MHz channel bandwidths is specified for the band n254

FFS: On whether it should be optional or mandatory

-	TX requirements

-	Consider PC3 power class for the new NTN band

-	Existing MPR (same as for other NTN bands) is re-used for this band.

-	UE co-existence requirements:

-	Use L-band UE co-existence requirements as a baseline considering additional bands as follows:

-	NR band n54, n77, n78, n105; E-UTRA band 31, 54, 72, 73, 87, 88, 103;

-	Other bands can be considered during next meetings.

-	FCC out-of-band emission requirements, as presented in the table below, shall be applied for band n254.

| Frequency band (MHz) | Channel bandwidth / Spectrum emission limit1 (dBm) | Measurement bandwidth | NOTE |
| --- | --- | --- | --- |
|  | 5 MHz, 10 MHz, 15 MHz |  |  |
| 1559 ≤ f ≤ 1605 | -50 | 700 Hz | Discreet emissions averaged over any 2 millisecond active transmission interval |
| 1605 ≤ f ≤ 1610 | -50 + 60/5 (f-1605) | 700 Hz |  |
| 1559 ≤ f ≤ 1605 | -40 | 1MHz | Averaged over any 2 millisecond active transmission interval |
| 1605 ≤ f ≤ 1610 | -40 + 60/5 (f-1605) | 1MHz |  |
| NOTE: The EIRP requirement in regulation is converted to conducted requirement using a 0dBi antenna. |  |  |  |

UE RRM agreements

-	No new RRM requirements are needed

BS RF agreements

- 	TS 38.108 specification does not require any updates for conducted and OTA, RX and TX requirements (sections 6, 7, 9, 10), with the introduction of [n254] operating band. All the existing SAN type 1-H and SAN type 1-O requirements are applicable to [n254].

-	5MHz, 10MHz and [15MHz] channel bandwidths are specified for the band.

-	The following channel raster points are specified for the band, UL channel raster points at 322000 – 325300 and the DL channel raster points at 496700 –500000.

-	"Case A" sync pattern is specified for the band with the sync raster points at 6215 – 6244 in steps of <1>.

-	15MHz channel bandwidths are specified for the band.

#### 2.4.2	Remaining Open issues

UE RF open issues

-	System parameters (asymmetric band combinations, Tx-Rx separation distance)

-	Tx requirements (ETSI in-band and out-of-band emission requirements, A-MPR, NS values)

-	Rx requirements (REFSENS, in-band and out-of-band blocking requirements)

-	Final CRs

BS RF open issue

-	Final CRs

## 2.5	RAN5

## 2.6	RAN6

## 3.	Detailed progress in SA/CT WGs since last TSG meeting (for all involved WGs)

NOTE: This section only needs to be filled in for WI/SIs where there is a corresponding relevant WI/SI in SA/CT.

## 3.1	SAx/CTs

#### 3.1.1	Agreements with cross-TSG impacts

#### 3.1.2	Remaining Open issues with cross-TSG impacts

NOTE: This section should also flag any critical dependencies that need TSG attention.

## 4.	References

NOTE:	This can be e.g. a list of all related Tdocs in the affected WGs since last TSG, references to LSs, produced TRs/TSs, the work/study item description or status reports of previous TSGs.

## 4.1	RAN WG4#106 meeting

- R4-2300300, "Work plan for the NTN L-/S-band", Apple Inc., Globalstar

- R4-2302354, "Draft TR for the NTN L-/S-band", Apple Inc., Globalstar

- R4-2300301, "System parameters for the NTN L-/S-band", Apple Inc., Globalstar

- R4-2301672, "Discussion on new FR1 NTN band", Nokia, Nokia Shanghai Bell

- R4-2300302, "RF requirements for the NTN L-/S-band", Apple Inc., Globalstar

- R4-2300304, "Draft running CR on Introduction of the NTN L-/S-band to TS 38.101-5", Apple Inc., Globalstar

- R4-2301100, "Discussion on UE RF for NTN L-S- bands", Xiaomi

- R4-2301394, "Draft CR to TS 38.101-5:Introduction of a new NTN FDD band n254", ZTE Corp.

- R4-2302710, "UE additional spurious emission requirements for the L-/S-band", Qualcomm Inc.

- R4-2301754, "draft CR to TS38.108 Introduction of the satellite L-band and S-band", ZTE Corp.

- R4-2301755, "Discussion on SAN RF requirements for the satellite L-band and S-band", ZTE Corp.

- R4-2302469, "Initial discussion on SAN RF requirements for the satellite L-/S-band", Huawei, HiSilicon

- R4-2300303, "RRM requirements for the NTN L-/S-band", Apple Inc., Globalstar

- R4-2300305, "Draft running CR on Introduction of the NTN L-/S-band to TS 38.133", Apple Inc., Globalstar

- R4-2301392, "Draft CR to TS 38.133:Introduction of a new NTN FDD band n254", ZTE Corp.

## 4.2	RAN WG4#106bis meeting

- R4-2306562, "Draft TR for the NTN L-/S-band", Apple Inc., Globalstar

- R4-2306563, "Draft running CR on Introduction of the NTN L-/S-band to TS 38.101-5", Apple Inc., Globalstar

- R4-2306564, "Draft running CR on Introduction of the NTN L-/S-band to TS 38.133", Apple Inc., Globalstar

- R4-2304330, "System parameters for the NTN L-/S-band", Apple Inc., Globalstar

- R4-2304939, "Discussion on new FR1 NTN band (n254)”, Nokia, Nokia Shanghai Bell

- R4-2304331, "RF requirements for the NTN L-/S-band”, Apple Inc., Globalstar

- R4-2304794, "Discussion on UE RF requirements for the satellite L-/S-band”, ZTE

- R4-2304795, "CR to TS38.101-5 Introduction of the satellite L-/S-band", ZTE

- R4-2305297, "Discussion on UE RF for NTN L-S- bands", Xiaomi

- R4-2305392, "Discussion on UE RF requiremetns for NTN LS bands", Huawei, HiSilicon

- R4-2305827, "Compatibility between EN 301 441 and 3GPP requirements for MES", Qualcomm Inc.

- R4-2306565, "CR to TS38.108 Introduction of the satellite L-band and S-band", ZTE

- R4-2305037, "Draft CR to TS38.133: Introduction of a new NTN FDD band n254", ZTE

- R4-2306561, "WF on NTN L-/S-band", Apple Inc.

## 4.3	RAN WG4#107 meeting

- R4-2307551, "NR NTN LSband SystemR4-2306561", CAICT

- R4-2309600, "Skeleton TR 38.741 for the NTN L-/S-band", Apple Inc., Globalstar

- R4-2310288, "TP for the TR 38.741 for the NTN L-/S-band", Apple Inc., Globalstar

- R4-2309486, "TP for the TR 38,741 for the NTN L-/S-band (power back-off values)", Apple Inc., Globalstar

- R4-2309045, "System parameters for the NTN L-/S-band", Apple Inc., Globalstar

- R4-2309511, "Specification of wider channel bandwidths for FR1 NTN UE", Qualcomm Inc.

- R4-2308585, "Discussion on UE RF requiremetns for NTN LS bands", Huawei, HiSilicon

- R4-2308586, "Discussion on UE RF requirements for the satellite L-band and S-band", ZTE

- R4-2309012, "Discussion on UE RF for NTN L-S- bands", Xiaomi

- R4-2309046, "RF requirements for the NTN L-/S-band", Apple Inc., Globalstar

- R4-2308587, "CR to TS38.101-5 Introduction of the satellite L-/S-band", ZTE

- R4-2309047, "Draft running CR on Introduction of the NTN L-/S-band to TS 38.101-5", Apple Inc., Globalstar

- R4-2310289, "CR to TS38.108 Introduction of the satellite L-/S-band", ZTE

- R4-2309048, "Draft running CR on Introduction of the NTN L-/S-band to TS 38.133", Apple Inc., Globalstar, ZTE

- R4-2310290, "WF on LTE_NR_NTN_LSband", Apple

01.02.2023		minor adaptations for RAN #99

27.10.2022		minor adaptations for RAN #98e

01.08.2022		minor adaptations for RAN #97e

21.05.2022		minor adaptations for RAN #96

10.01.2022		minor adaptations for RAN #95e

04.10.2021		minor adaptations for RAN #94e

08.08.2021		minor adaptations for RAN #93e

17.05.2021		minor adaptations for RAN #92e

28.01.2021		minor adaptations for RAN #91e

09.11.2020		minor adaptations for RAN #90e

31.08.2020		minor adaptations for RAN #89e

20.04.2020		minor adaptations for RAN #88e

18.02.2020		minor adaptations for RAN #87e

14.11.2019		minor adaptations for RAN #86

18.08.2019		minor adaptations for RAN #85

12.05.2019		minor adaptations for RAN #84

27.02.2019		minor adaptations for RAN #83

21.11.2018		completion levels with colours added (for RAN #82)

v04.81	31.07.2018		simplification of template and addition of cross-TSG aspects (for RAN #81)

v04.80	21.05.2018		minor adaptations for RAN #80

v04.79	26.02.2018		minor adaptations for RAN #79

v04.78	18.11.2017		minor adaptations for RAN #78

v04.77	06.08.2017		minor adaptations for RAN #77

v04.76	15.05.2017		minor adaptations for RAN #76

v04.75	31.01.2017		minor adaptations for RAN #75

v04.74	28.10.2016		minor adaptations for RAN #74

v04.73	01.09.2016		adaptations for RAN #73 (time units in extra Excel table, RAN6 reporting included)

v04.72	26.05.2016		adaptations for RAN #72 (introduction of NR & GERAN TUs)

v04.71	10.02.2016		minor adaptations for RAN #71

v04.70	30.10.2015		minor adaptations for RAN #70

v04.69	12.08.2015		minor adaptations for RAN #69

v04.68	21.05.2015		minor adaptations for RAN #68

v04.67	01.02.2015		minor adaptations for RAN #67

v04.66	16.11.2014		minor adaptations for RAN #66

v04.65	16.08.2014		minor adaptations for RAN #65

v04.64	22.05.2014		minor adaptations for RAN #64

v04.63	24.01.2014		restructuring for RAN #63 to cover Core & Perf. in one doc file

v03.62	11.11.2013		section 1.2.3 adapted for RAN #62

v03	11.08.2013		section 1.2.3 added on time budget

v02	07.05.2010		history added, some spelling corrections

v01	13.11.2009		First version of the template