3GPP TSG- RAN4 Meeting # 107	R4-2310142

# Incheon, KR, May 22 – May 26, 2023

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.133 | CR | 3219 | rev | 1 | Current version: | 18.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | x | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | DL interruption for Tx switching across 3/4 bands |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, HiSilicon |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_MC_enh-Core |  |  |  |  |  | Date: |  |  | 2023-05-08 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | In DL interruption requirements for Tx switching across 3/4 bands, if one downlink carrier is indicated to be interrupted by two band pairs for dynamic switching simultaneously, the DL interruption length and location on the victim carrier is determined by the maximum of uplink switching periods of the two band pairs. If [uplinkTxSwitchingPeriod1T1Tto2T] is reported, DL interruption length is determined by the uplink Tx switching period, which is indicated by [uplinkTxSwitchingPeriod1T1Tto2T]. If [TBD on-unaffeacted-band-invovled] is reported, DL interruption length is determined by the uplink Tx switching period, which is the next larger value from the set {35 us, 140 us, 210 us} w.r.t. the length of the switching period of the band pair, and the uplink Tx switching period is 210 us when the length of the switching period for the band pair is 210 us. |  |  |  |  |  |  |  |  |
| s |  |  |  |  |  |  |  |  |  |  |
| Summary of change: |  | In DL interruption requirements for Tx switching across 3/4 bands, if one downlink carrier is indicated to be interrupted by two band pairs for dynamic switching simultaneously, the DL interruption length and location on the victim carrier is determined by the maximum of uplink switching periods of the two band pairs. If [uplinkTxSwitchingPeriod1T1Tto2T] is reported, DL interruption length is determined by the uplink Tx switching period, which is indicated by [uplinkTxSwitchingPeriod1T1Tto2T]. If [TBD on-unaffeacted-band-invovled] is reported, DL interruption length is determined by the uplink Tx switching period, which is the next larger value from the set {35 us, 140 us, 210 us} w.r.t. the length of the switching period of the band pair, and the uplink Tx switching period is 210 us when the length of the switching period for the band pair is 210 us. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | RRM requirements for Tx switching across 3/4 bands are not complete. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 8.2.2.2.10D |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  | x |  | Test specifications |  |  |  | TS38.533 |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

<Start of Change 1>

8.2.2.2.10D	DL Interruptions at UE switching across three or four uplink bands

The DL interruption requirements at dynamic switching across three or four uplink bands specified in this clause are applicable for an NR inter-band CA configuration and SUL band configuration with inter-band CA when the capability [BandCombination-UplinkTxSwitch-r18] is present, and are only applicable for uplink switching mechanisms specified in clause [6.1.6] of TS 38.214 [26]. In the NR inter-band CA configuration and SUL band configuration with inter-band CA, the number of NR uplink bands with different carrier frequencies is three or four. NR UL carrier(s) in each of the three or four uplink bands are capable of one or two transmit antenna connector(s), according to the UE capability.

When dynamic switching across three or four uplink bands is conducted, UE is allowed to cause DL interruption of X OFDM symbols in NR downlink carrier(s) as indicated by uplinkTxSwitching-DL-Interruption [2]. The DL interruption starts from the first OFDM symbol which fully or partially overlaps with the UL Tx switching period located in uplink NR band(s) as indicated in RRC signalling [2]. The DL interruption lengths of X for single TAG and for 2-TAG are defined in Table 8.2.2.2.10D-1 and Table 8.2.2.2.10D-2.

Note: If one downlink carrier is indicated to be interrupted by two band pairs for dynamic switching simultaneously, the DL interruption length on the victim carrier is determined by the maximum of uplink switching periods of the two band pairs. If [uplinkTxSwitchingPeriod1T1Tto2T] is reported, DL interruption length is determined by the uplink Tx switching period, which is indicated by [uplinkTxSwitchingPeriod1T1Tto2T]. If [TBD on-unaffeacted-band-invovled] is reported, DL interruption length is determined by the uplink Tx switching period, which is the next larger value from the set {35 us, 140 us, 210 us} w.r.t. the length of the switching period of the band pair, and the uplink Tx switching period is 210 us when the length of the switching period for the band pair is 210 us.

No DL interruption is allowed in the NR downlink carrier(s) which is not indicated by uplinkTxSwitching-DL-Interruption. No DL interruption is allowed for some inter-band UL CA configurations and SUL band configurations with inter-band CA as specified in TS 38.101-1 [18].

Table 8.2.2.2.10D -1: DL interruption length on NR carrier(s) in the unit of OFDM symbols (X) for switching across three or four uplink bands for single TAG

|  | NR Slot length (ms) | Uplink Tx switching period Note1 |  |  |
| --- | --- | --- | --- | --- |
|  |  | 35us | 140us | 210us |
| 0 | 1 | 2 | 3 | 4 |
| 1 | 0.5 | 3 | 6 | 7 |
| 2 | 0.25 | 4 | 10 | 14 |
| Note 1: Uplink Tx switching period depends on UE capability [TBD]. Note 2: RTD=3us is assumed to derive the DL interruption length. |  |  |  |  |

Table 8.2.2.2.10D -2: DL interruption length on NR carrier(s) in the unit of OFDM symbols (X) for switching across three or four uplink bands for two TAGs

|  | NR Slot length (ms) | Uplink Tx switching period Note1 |  |  |
| --- | --- | --- | --- | --- |
|  |  | 35us | 140us | 210us |
| 0 | 1 | 2 | 3 | 4 |
| 1 | 0.5 | 3 | 6 | 8 |
| 2 | 0.25 | 4 | 10 | 14 |
| Note 1: Uplink Tx switching period depends on UE capability [TBD]. Note 2: RTD=9 us is assumed to derive the DL interruption length. |  |  |  |  |

<End of Change 1>