3GPP TSG RAN Meeting #100							                           RP-231391

# Taipei, June 12-14, 2023

Agenda Item:	9.2.5

Source:	ZTE, Sanechips

Title:	Discussion on scope of AI/ML for air interface

Document for:		Discussion and decision

- Introduction

Agreement in RAN1#111 meeting: Time domain CSI prediction using UE sided model is selected as a representative sub-use case for CSI enhancement. Note: Continue evaluation discussion in 9.2.2.1. Note: RAN1 defer potential specification impact discussion at 9.2.2.2 until the RAN1#112b-e, and RAN1 will revisit at RAN1#112b-e whether to defer further till the end of R18 AI/ML SI. Note: LCM related potential specification impact follow the high level principle of other one-sided model sub-cases. Working Assumption in RAN1#111 meeting: For the AI/ML based CSI prediction sub use case, the nearest historical CSI w/o prediction as well as non-AI/ML/collaboration level x AI/ML based CSI prediction approach are both taken as baselines for the benchmark of performance comparison, and the specific non-AI/ML/collaboration level x AI/ML based CSI prediction is reported by companies. · Note: the specific non-AI/ML based CSI prediction is compatible with R18 MIMO; collaboration level x AI/ML based CSI prediction could be implementation based AI/ML compatible with R18 MIMO as an example o It does not imply any restriction on future specification for CSI prediction ·FFS how to model the simulation cases for collaboration level x CSI prediction and LCM for collaboration level y/z CSI prediction Agreement in RAN1#112bis-e: In CSI prediction using UE-side model use case, whether to address the potential spec impact of CSI prediction depends on RAN#100 final conclusion, focusing on the following · data collection procedure, mainly including RS configuration, measurement and report configuration, reusing as much as possible what is defined for UE side use cases · monitoring procedure and metric for AI-based CSI prediction. · Model/functionality selection/switching and fine-tuning procedure. · Note: Discussion on potential specification impact is limited to aspects which would NOT duplicate the work in Rel-18 MIMO WI. · Note: Minimize LCM related potential specification impact discussion that follow the high-level principle of other one-sided model sub-cases.

In RAN1#111 meeting, time domain CSI prediction using UE sided model is selected as a representative sub-use case for CSI enhancement. However, the potential specification impact discussion was deferred due to the concern on the progress on Rel-18 MIMO enhancement. Furthermore, different benchmark schemes (i.e., the nearest historical CSI without prediction, non-AI/ML based CSI prediction and collaboration level x AI/ML based CSI prediction approach) were reached to evaluate the performance of time domain CSI prediction using UE sided model.

However, in RAN1#112bis-e meeting, the potential specification impact discussion was deferred again. Due to the limited time left for Rel-18 AI/ML in RAN1, it should be discussed in RAN#100 that whether to resume the potential specification impact discussion for CSI prediction and the corresponding scope.

In this contribution, we provide our views on AI/ML for time domain CSI prediction.

- Evaluation results

Observation in RAN1#112bis-e: For the AI/ML based CSI prediction, till the RAN1#112bis-e meeting, - 11 sources show that the AI/ML-based CSI prediction outperforms the benchmark of the nearest historical CSI, wherein - 5 sources show the gain of 14% ~ 26.47% using raw channel matrix as input. - 2 sources show the gain of 5.64% ~ 9.49% using precoding matrix as input, which is in general worse than using raw channel matrix as input - Note 1: spatial consistency is adopted in 1 source and not adopted in 5 sources. - Note 2: the above results are based on the following assumptions - The observation window considers to start as early as 15ms~50ms. - A future 4ms or 5ms instance from the prediction output is considered for calculating the metric. - UE speed is 30km/h. - The performance metric is SGCS in linear value for layer 1. Observation in RAN1#113: For the AI/ML based CSI prediction, till the RAN1#113 meeting, compared to the Benchmark#1 of the nearest historical CSI, in terms of SGCS, from UE speed perspective, in general the gain of AI/ML based solution is related with the UE speed: - For 10km/h UE speed, 4 sources [Fujitsu, Samsung, Xiaomi, InterDigital] observe 1.03%~6% gain, 1 source [CMCC] observes 21.93% gain. - For 30km/h UE speed, 2 sources [OPPO, ETRI] observes 6%~10.43% gain, 5 sources [ZTE, Fujitsu, Apple, Xiaomi, Spreadtrum] observe 18.72%~31.3% gain, and 3 sources [InterDigital, MediaTek, CMCC] observe 35%~ 41.75% gain, which are in general larger than 10km/h UE speed. - For 60km/h UE speed, 2 sources [Fujitsu, InterDigital] observe -3%~5% gain, 4 sources [Huawei, Samsung, vivo, CMCC] observe 11.2%~19.98% gain, which are in general smaller than 30km/h UE speed. - Note: the above results are based on the following assumptions - The observation window considers to start as early as 15ms~50ms. - A future 4ms or 5ms instance from the prediction output is considered for calculating the metric. - Raw channel matrix is considered as model input - The performance metric is SGCS in linear value for layer 1. - No post processing is considered. - No spatial consistency is considered by 11 sources [Fujitsu, Samsung, Xiaomi, InterDigital, CMCC, OPPO, ETRI, ZTE, Apple, Huawei, Spreadtrum]. 1 source [vivo] provides both results with spatial consistency and results w/o spatial consistency. - Note: Results refer to Table 5.1-1 of R1-2306059 Observation in RAN1#113: For the AI/ML based CSI prediction, till the RAN1#113 meeting, in terms of mean UPT, gains are observed compared to both Benchmark#1 of the nearest historical CSI and Benchmark#2 of a non-AI/ML based CSI prediction approach: - Compared to the benchmark of the nearest historical CSI: - For FTP traffic: - 1 source [Huawei] observes 1.2%~4.2% gain; - 1 source [Apple] observes 7.6%~8.5% gain; - 1 source [vivo] observes 9.7%~17.2% gain. - 1 source [MediaTek] observes 22.6%~ 48.6% gain. - For full buffer traffic: - 1 source [Nokia] observes 2%~3% gain; - 1 source [vivo] observes 8.7% gain. - 1 source [MediaTek] observes 1.01% gain. - Compared to the benchmark of an auto-regression based CSI prediction: - For FTP traffic: - 1 source [Huawei] observes 0.7%~3.1% gain; - 1 source [vivo] observes 3.4%~7.0% gain. - For full buffer traffic: - 1 source [vivo] observes 8.1% gain. - Note: the above results are based on the following assumptions - The UE speed is 30km/h or 60km/h. - The observation window considers to start as early as 15ms~50ms. - A future 4ms or 5ms instance from the prediction output is considered for calculating the metric. - Raw channel matrix is considered as model input - The performance metric is mean UPT for Max rank 1. - No post processing is considered. - No spatial consistency is considered - Note: Results refer to Table 5.1-8 of R1-2306059 Observation in RAN1#113: For the AI/ML based CSI prediction, till the RAN1#113 meeting, in terms of 5% UPT, gains are observed compared to both Benchmark#1 of the nearest historical CSI and Benchmark#2 of a non-AI/ML based CSI prediction approach: - Compared to the benchmark of the nearest historical CSI: - For FTP traffic: - 2 sources [Huawei, vivo] observes 4.5%~9.3% gain; - 3 sources [Huawei, Apple, vivo] observes 11.3%~20.1% gain; - For full buffer traffic: - 2 sources [Nokia, vivo] observe 6%~17.5% gain; - Compared to the benchmark of an auto-regression based CSI prediction: - For FTP traffic: - 2 sources [Huawei, vivo] observes 0.5%~16% gain; - For full buffer traffic: - 1 source [vivo] observes 11% gain. - Note: the above results are based on the following assumptions - The UE speed is 30km/h or 60km/h. - The observation window considers to start as early as 15ms~50ms. - A future 4ms or 5ms instance from the prediction output is considered for calculating the metric. - Raw channel matrix is considered as model input - The performance metric is mean UPT for Max rank 1. - No post processing is considered. - No spatial consistency is considered - Note: Results refer to Table 5.1-9 of R1-2306059

According to the evaluation results above, it was observed that time domain CSI prediction using UE sided model can achieve performance gains over different benchmark schemes (i.e., the nearest historical CSI without prediction, non-AI/ML based CSI prediction and collaboration level x AI/ML based CSI prediction approach). The performance gain is obvious especially when the raw channel is used as model input. Therefore, it’s better to resume the specification impact discussion after RAN#100 meeting for time domain CSI predication using UE-sided model. Concerning on the time limitation in RAN1, the discussion on time domain CSI prediction should reuse the high-level principle of other one-sided model sub-cases and Rel-18 MIMO enhancement for CSI prediction.

Observation 1: Time domain CSI prediction using UE sided model can achieve performance gains over different benchmark schemes (i.e., the nearest historical CSI without prediction, non-AI/ML based CSI prediction and collaboration level x AI/ML based CSI prediction approach).

Proposal 1: Resume the specification impact discussion after RAN#100 meeting for time domain CSI predication using UE-sided model, focusing on the following scope:

· Data collection procedure, mainly including RS configuration, measurement and report configuration, reusing as much as possible what is defined for UE side use cases

· Monitoring procedure and metric for AI-based CSI prediction.

· Note: Discussion on potential specification impact is limited to aspects which would NOT duplicate the work in Rel-18 MIMO WI.

· Note: Minimize LCM related potential specification impact discussion that follow the high-level principle of other one-sided model sub-cases.

- Conclusion

In this contribution, we provide our views on AI/ML for time domain CSI prediction. We have the following observation and proposal:

Observation 1: Time domain CSI prediction using UE sided model can achieve performance gains over different benchmark schemes (i.e., the nearest historical CSI without prediction, non-AI/ML based CSI prediction and collaboration level x AI/ML based CSI prediction approach).

Proposal 1: Resume the specification impact discussion after RAN#100 meeting for time domain CSI predication using UE-sided model, focusing on the following scope:

· Data collection procedure, mainly including RS configuration, measurement and report configuration, reusing as much as possible what is defined for UE side use cases

· Monitoring procedure and metric for AI-based CSI prediction.

· Note: Discussion on potential specification impact is limited to aspects which would NOT duplicate the work in Rel-18 MIMO WI.

· Note: Minimize LCM related potential specification impact discussion that follow the high-level principle of other one-sided model sub-cases.

4. Reference

[1] Chair's notes RAN1#111

[2] Chair's notes RAN1#112bis-e

[3] Chair's notes RAN1#113