3GPP TSG RAN Meeting #101						RP-231533

Bangalore, India, September 11-15, 2023

3GPP TSG-RAN WG4 Meeting # 108	R4-2314484

Toulouse, France, August 21 – August 25, 2023

# Title:	LS on RAN4 RRM work scope for NR NTN Ka band

# Response to:	-

# Release:	Release-18

Work Item:	NR_NTN_enh-Core

Source:	TSG RAN WG4

To:	TSG RAN

Cc:	-

Contact Person:

Name:	Dorin Panaitopol

E-mail Address:	dorin.panaitopol [at] thalesgroup [dot] com

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

# Attachments:	None

1. Overall Description:

RAN4 RF is currently investigating the scope of core specifications for above 10 GHz considering FDD mode for the following frequency bands:

| NTN operating band | UL (Earth-to-Space) | DL (Space-to-Earth) |
| --- | --- | --- |
| n5121 | 27.5 - 30.0 GHz | 17.3 - 20.2 GHz |
| n5112 | 28.35 - 30.0 GHz | 17.3 - 20.2 GHz |
| n5103 | 27.5 - 28.35 GHz | 17.3 - 20.2 GHz |
| Note 1: This band is applicable in the countries subject to CEPT ECC Decision(05)01 and ECC Decision (13)01. Note 2: This band is applicable in the USA subject to FCC 47 CFR part 25. Note 3: This band is applicable for Earth Station operations in the USA subject to FCC 47 CFR part 25. FCC rules currently do not include ESIM operations in this band (47 CFR 25.202). |  |  |

RAN4 discussions are still ongoing on how to define RRM requirements. However, RAN4 RRM is happy to announce that the scope of the work needed to define the RRM requirements (i.e. beam steering related) necessary for NR-NTN UEs operating in the NTN example band (i.e. Ka band) based on associated assumptions of the UE architecture has been identified as follows:

  - RRM requirements shall cover at least the following scenarios

    - Case-1: Stationary UE for GSO

    - Case-2: Stationary UE for LEO

    - Case-3: Mobile UE for GSO

  - The set of requirements shall include all RRM requirements relevant to NTN, based on Rel-17 NR NTN requirements unless critical issues are identified, including

    - UE uplink timing accuracy

    - RRC IDLE and INACTIVE mobility

    - (Conditional) Handover

    - RRC Re-establishment

    - RRC Connection Release with Redirection

    - Radio Link Monitoring

    - Link Recovery procedure (BFD/CBD)

    - Active TCI switching

    - Measurement Procedure

    - (L1/L3 measurement delay and scheduling/measurement restrictions)

    - Measurement Performance

    - FFS: UL spatial relation switching

  - The requirements need to cover the following UE architectures

    - Fully electronically-steered beam UEs (Type 1)

    - Fully mechanically-steered beam UEs (Type 2)

    - FFS if additional types of UEs shall be considered and subject to RAN4 RF session conclusions

  - Aim to reuse Rel-17 NR NTN requirements to the extent possible, except for beam sweeping aspects.

    - For Type 1 UEs the impact of beam sweeping on RRM requirements can be accounted for based on FR2-1 requirements in terms of scaling principles.

    - For Type 2 UEs additional studies are required to identify the methodology to define the requirements

2. Actions:

To RAN group: TSG RAN

ACTION: RAN4 respectfully asks TSG RAN to take it into consideration for work planning.

3. Date of Next RAN4 Meetings:

TSG RAN WG4#109			 9th of October – 13th of October 2023                                 Xiamen, China