3GPP TSG RAN Meeting #101	RP-231543

Bangalore, India, September 11-15, 2023

Agenda Item:	8A.2.3

Source:	Charter Communications Inc., CableLabs, Ericsson, Spark, Vodafone

Title:	WID Consideration on Sub Band Full Duplex operation

Document for:	Approval

# 1	Introduction

On RAN meeting #98e, a study on evolution of NR duplex operation was introduced in [1].

The detailed objectives were as follows:

- Identify applicable and relevant deployment scenarios (RAN1).

- Develop evaluation methodology for duplex enhancement (RAN1).

- Study the sub-band non-overlapping full duplex and potential enhancements on dynamic/flexible TDD (RAN1, RAN4).

    - Identify possible schemes and evaluate their feasibility and performances (RAN1).

    - Study inter-gNB and inter-UE CLI handling and identify solutions to manage them (RAN1).

    - Consider intra-sub band CLI and inter-sub band CLI in case of the sub-band non-overlapping full duplex.

    - Study the performance of the identified schemes as well as the impact on legacy operation assuming their co-existence in co-channel and adjacent channels (RAN1).

    - Study the feasibility of and impact on RF requirements considering adjacent-channel co-existence with the legacy operation (RAN4).

    - Study the feasibility of and impact on RF requirements considering the self-interference, the inter-sub band CLI, and the inter-operator CLI at gNB and the inter-sub band CLI and inter-operator CLI at UE (RAN4).

    - Note: RAN4 should be involved early to provide necessary information to RAN1 as needed and to study the feasibility aspects due to high impact in antenna/RF and algorithm design, which include antenna isolation, TX IM suppression in the RX part, filtering and digital interference suppression.

    - Summarize the regulatory aspects that must be considered for deploying the identified duplex enhancements in TDD unpaired spectrum (RAN4).

Note: For potential enhancements on dynamic/flexible TDD, utilize the outcome of discussion in Rel-15 and Rel-16 while avoiding the repetition of the same discussion.

In this document, we would like to provide guidance to be considered for the Rel-19 SBFD normative work because of the simulation results summarized in [2] and approved in RAN4#108 [3].  We would also like to provide guidance on potential enhancements on dynamic/flexible TDD, identified as part of the SID objectives.

# 2	SBFD operation in UL Slot

In the ad hoc minutes for SBFD co-existence study from RAN4 #108 [2], Charter commented that, “it’s RAN4 common observation: For the case with SBFD operation in UL slot, we observed a larger throughput degradation compared to SBFD operation on DL slot with TDD as the victim”.

4.1 Scenario 1 (Urban Macro to Urban Macro) case 2

4.4 Scenario 4 (Urban Macro to Urban Micro) case 2

4.5 Scenario 5 (Urban Micro to Urban Micro) case 2

In addition to this common observation, there was an agreement made in [3] and highlighted below:

- For Scenario 4 FR1 UMa-to-Umi, with case 2 (Case 2: aggressor SBFD DU victim NR TDD UL),100% grid shift and baseline assumption, higher than 5% throughput loss is observed which is worse than case 1 (Case 1: aggressor SBFD DU victim NR TDD DL)

  - It’s observed that the degradation is even worse with 50% grid shift based on the results in R4-2313631 submitted in RAN4#108

While RAN4 is finalizing the simulation results, some simulation results [6] presented in RAN4#108 captures the following observations showing large degradation with case 2 (aggressor SBFD DU victim NR TDD UL):

- The 5th percentile TDD UL throughput (case 2) in the FR1 UMa-to-UMa scenario degrades by 100% for all grid shift values due to ACI, which indicates legacy TDD loss of UL coverage at the cell edge because of the strong BS-to-BS interference introduced by SBFD. The average TDD UL throughput with 100% grid shift (289 m) degrades by 8%, and the degradation increases to 19%, 33%, 50%, and 63% when the grid shift is reduced to 50% (144 m), 25% (72 m), 10% (29 m), and 5% (14 m).

- The average TDD UL throughput (case 2) in the FR1 UMa-to-UMi scenario with 100% grid shift (167 m) degrades by 11% due to ACI, and the degradation changes to 28%, 25%, and 8% when the grid shift is reduced to 50% (83 m), 25% (42 m), and 10% (17 m).

- The average TDD UL throughput (case 2) in the FR1 UMi-to-UMi scenario with 100% grid shift (167 m) degrades by 10% due to ACI, and the degradation increases to 24%, 52%, and 81% when the grid shift is reduced to 50% (83 m), 25% (42 m), and 10% (17 m).

RAN1 also concluded that for SBFD deployment case 4 [two operators in adjacent frequency bands], the legacy TDD operator faces limited or large degradation for UL and DL performance when adjacent SBFD operator uses XXXXX slot format. TR 38.858 endorsed after RAN1#114 [5] captures the following conclusions—showing consistent UPT degradation for the legacy TDD operator for both UL and DL, more significant in the former:

For SBFD deployment case 4 (FR1) with 0% grid shift, and the total capability of spatial isolation and digital isolation for co-site inter-sector CLI is no less than 93 dB, spatial isolation for co-site adjacent-channel CLI is no less than 93dB, and SBFD with XXXXX slot format and large packet size are assumed,

< SBFD operator performance results and corresponding text omitted here >

- For the legacy TDD operator, regarding the performance impact of semi-static SBFD to legacy TDD of another operator, there may be limited or large degradation for UL and DL performance.

  - {-6.46%, -6.73%, -5.22%} / {-29.43%, -39.73%, -53.81%} for mean/5% DL UPT loss of legacy TDD operator.

  - {-16.16%, -24.42%, -27.10%} / {-16.18%, 0%, 0%} for mean/5% UL UPT loss of legacy TDD operator.

…

For SBFD deployment case 4 (FR1) with 0% grid shift, and the total capability of spatial isolation and digital isolation for co-site inter-sector CLI is equal to 93 dB, spatial isolation for co-site adjacent-channel CLI is equal to 93 dB, and SBFD with XXXXX slot format and large packet size are assumed,

< SBFD operator performance results and corresponding text omitted here >

- For the legacy TDD operator, regarding the performance impact of semi-static SBFD to legacy TDD of another operator, there may be limited or large degradation for UL and DL performance.

  - {-0.44, -3.39%, -4.45%} / {-1.25%, -6.93%, -7.97%} for mean/5% DL UPT loss of legacy TDD operator

  - {-7.43%, -30.66%, -39.94%} / {-16.18%, -46.23%, -49.99%} for mean/5% UL UPT loss of legacy TDD operator

…

For SBFD deployment case 4 (FR1) with 100% grid shift, and the total capability of spatial isolation and digital isolation for co-site inter-sector CLI is no less than 93 dB, and SBFD with XXXXX slot format and large packet size are assumed,

< SBFD operator performance results and corresponding text omitted here >

- For the legacy TDD operator, regarding the performance impact of semi-static SBFD to legacy TDD of another operator, there may be limited or large degradation for UL and DL performance.

  - {-0.35%, -3.31%, -5.38%} / {-2.64%, -9.41%, -7.68%} for mean/5% DL UPT loss of legacy TDD operator.

  - {-13.50%, -21.26, -16.74%} / {-24.39%, -0.53%, -0.90%} for mean/5% UL UPT loss of legacy TDD operator.

…

For SBFD deployment case 4 (FR1) with 100% grid shift, and the total capability of spatial isolation and digital isolation for co-site inter-sector CLI is equal to 93 dB, and SBFD with XXXXX slot format and large packet size are assumed,

< SBFD operator performance results and corresponding text omitted here >

- For the legacy TDD operator, regarding the performance impact of semi-static SBFD to legacy TDD of another operator, there may be limited or large degradation for UL and DL performance

  - {-0.35%, -2.94%, -4.37%} / {-1.40%, -7.02%, -6.72%} for mean/5% DL UPT loss of legacy TDD operator.

  - {-6.75%, -26.88%, -37.96%} / {-12.59%, -44.22%, -50.22%} for mean/5% UL UPT loss of legacy TDD operator.

In TR 38.858 endorsed after RAN1#114 [5], RAN1 concludes the following regarding the cause of the losses observed in the simulation results,

For SBFD deployment case 4, for the legacy TDD operator in the case of XXXXX slot format, adjacent channel gNB-gNB CLI causes loss.

The DL UPT loss at least comes from UE-to-UE CLI.

Observation 1:  For legacy TDD operations a larger throughput degradation is observed when SBFD operation is allowed in the legacy UL slot as compared to the case when SBFD operation is allowed only in the legacy DL slot.

Proposal 1:  For normative phase, SBFD operation should only be considered in the legacy DL slot, and some stipulations need to be added to prevent SBFD operation in the legacy UL slot.

# 3	Dynamic/Flexible TDD and Dynamic SBFD

In the Rel-18 SID [1], one of the objectives identified was to potentially find enhancements on dynamic/ flexible TDD specifically to study and identify solutions for inter-gNB and inter-UE CLI.

In RAN1, dynamic/flexible TDD was studied and a new scheme “dynamic SBFD” was also proposed by some companies at the later stage of the Rel-18 study item. For the latter, no conclusions/recommendations are drawn in TR 38.858. Moreover, for both schemes, all of the work focused on the single operator case, while the more practical case of two operators in adjacent frequency bands was not addressed.  All the observations/conclusions and recommended enhancements were aimed at mitigating “co-channel” interference—as opposed to “adjacent channel” interference.  Thereby, in our perspective, the study of dynamic/flexible TDD or dynamic SBFD is not complete from the standpoint of combating or limiting adjacent channel CLI.

Observation 2: In RAN1, dynamic/flexible TDD and the dynamic SBFD variant were not studied in the practical deployment scenario of two operators in adjacent bands, whereby the study does not consider performance degradation due to adjacent channel CLI.

RAN4, did not conduct any simulations with dynamic/flexible TDD in Rel-18 SI as similar discussions took place under Rel-16 and were captured in TR 38.828 [4]. Section 6.3 (Recommendations) of this TR, concerning Urban Macro to Urban Macro scenario in FR1, concludes:

-	“Performance degradation was observed from the BS-to-BS interference for macro-macro scenario, which suggests that dynamic TDD should not be operated in such scenarios.”

Observation 3: For Urban Macro to Urban Macro scenario in FR1 performance degradation was observed due to BS-to-BS interference, cf. TR 38.828.

Based on the above recommendation of Rel-16 TR, we believe that dynamic TDD and dynamic SBFD will cause comparable degradation in FR1 urban macro to urban macro scenario especially in case of non-coordinating adjacent networks (e.g., two operators in adjacent frequency bands).    We believe that the study of dynamic/flexible TDD and dynamic SBFD is not complete in Rel-18 SI, at least with respect to managing adjacent channel CLI.

Proposal 2: As reported in TR 38.828, there is severe degradation with dynamic/flexible TDD in certain scenarios, which will be comparable for dynamic SBFD. Further, RAN1 did not study the adjacent channel CLI mitigation/handling for either one of them. Thereby, dynamic/flexible TDD and dynamic SBFD should not be included in the normative work in Rel-19 duplexing without proper coexistence study on dual-operator scenarios.

# 4	Conclusions

In this document, we discussed SBFD operation in UL slot and dynamic/flexible TDD and we highlighted the following observations and proposals:

Observation 1 For legacy TDD operations a larger throughput degradation is observed when SBFD operation is allowed in UL slot as compared to the case when SBFD operation is allowed only in the DL slot.

Proposal 1:  For normative phase, SBFD operation should only be considered in DL slot, and some stipulations need to be added to prevent SBFD operation in the UL slot.

Observation 2: In RAN1, dynamic/flexible TDD and the newer dynamic SBFD variant were not studied in the practical deployment scenario of two operators in adjacent bands, whereby the study is not complete from the standpoint of combating or limiting adjacent channel CLI.

Observation 3: For Urban Macro to Urban Macro scenario in FR1 performance degradation was observed due to BS-to-BS interference, cf. TR 38.828.

Proposal 2: As reported in TR 38.828, there is severe degradation with dynamic/flexible TDD in certain scenarios, which will be comparable for dynamic SBFD. Further, RAN1 did not study the adjacent channel CLI mitigation/handling for either one of them. Thereby, dynamic/flexible TDD and dynamic SBFD should not be included in the normative work in Rel-19 duplexing without proper coexistence study on dual-operator scenarios.

# References

RP-231041, Revised SID: Study on evolution of NR duplex operation, December 2022.

R4-2313871, Ad hoc Minutes for SBFD co-existence Study, August 2023.

BS RF Demod Test session report for RAN4#108, August 2023.

TR 38.828-g10 v16.1.0, 09/2019 Cross Link Interference (CLI) handling and Remote Interference Management (RIM) for NR.

R1-2308675, TR 38.858 v0.5.0 for study on evolution of NR duplex operation

R4-2313817, NR duplex evolution adjacent-channel coexistence simulation results