3GPP TSG-RAN #101	 RP-231617

Bangalore, September 11-15, 2023

Title:		Ambient IoT deployment feasibilities

Agenda Item:    		9.2.2

Source:	     	Nokia, Nokia Shanghai Bell

Document for:	Discussion and decision

### 1 Introduction

Ambient IoT RAN led Study Item Rel-18 was approved in RAN#97e [1]. The motivation of Ambient IoT study is to support ultra-low cost and ultra-low power devices for the IoT applications. SA1 completed a study item on ambient power enabled IoT [2], which focuses on use cases and service requirements, and it includes the objective of defining performance requirements and KPIs as well.

Currently, there is on-going RAN study item on Ambient IoT (Internet of Things) in RAN [3], which focuses on 3GPP IoT technology, suitable for deployment in a 3GPP system, which relies on ultra-low complexity devices with ultra-low power consumption for the very-low end IoT applications. The study provides clear differentiation, i.e., addressing use cases and scenarios that cannot otherwise be fulfilled based on existing 3GPP LPWA IoT technology e.g., NB-IoT including reduced peak Tx power.

In terms of energy storage, the study will consider the following device characteristics:

- Pure battery less devices with no energy storage capability at all, and completely dependent on the availability of an external source of energy

- Devices with limited energy storage capability that do not need to be replaced or recharged manually.

Device categorization based on corresponding characteristics (e.g., energy source, energy storage capability, passive/active transmission, etc.) may be discussed during the study, in relation with the relevant use cases. The device’s peak power consumption shall be limited by its practical form factor for the intended use cases and shall consider its energy source.

The Ambient IoT devices support communication with reader via reflection or transmission which can be supported by very low complexity hardware. The Ambient IoT device can collect energy from environment, such as RF signals, solar energy, vibration, heat, etc., and can potentially also be equipped with a small capacity battery. Hence, Ambient IoT is a promising technique to achieve even lower cost/power requirements compared to existing cellular IoT technologies, e.g., NB-IoT, LTE-M, RedCap, etc.

The RAN level study will compare and assess the feasibility of meeting the design targets for relevant use cases based on the deployment scenario(s) appropriate to it and identify assumptions on required functionality to be supported.

An essential aspect for ambient IoT is the link budget limitations and the required UE and gNB grid to enable seamless discovery and communications with ambient IoT devices. The link budget considerations discussed here will be focused on backscattering devices.

### 2 Energy harvesting

An IoT device’s power requirements can be broadly grouped in sensing, processing, and communication. Sensor modules have a power consumption on the order of tens of microwatts (e.g. a humidity or IMU sensor require 3~4μW and a temperature sensor requires 60 μW). Current low-power MCUs (e.g. embedded ARM processors) can require as low as 1 mW for sensor data processing and can fall into ultra low power sleep modes when idle. On the other hand, active radios (WiFi, BLE) consume hundreds of milliwatts due to the presence of multiple active components and power-hungry amplifiers. It can be easily seen that during an active state of an IoT device, communications dominate the power consumption while at the other end of the spectrum, there are RFID tags with very limited functionalities, no sensor or processing modules, that consume less than ten microwatts in fully operational mode. The elimination of active transmission components (LOs, PAs) minimizes the power consumption and allows the full over-the-air energizing of the RFID tag from an RF source. In the middle of the two extremes, there is a new space for IoT devices that can support features such as sensing and energy-aware computation, while minimizing the communication power requirements.

Existing commercial RFID systems with UHF (915 MHz) readers have max EIRP of 36 dBm and passive RFID tags with RF harvesters have a total DC power consumption of 3 μW (-25 dBm DC). At such power levels, RF harvesters have very limited rectification efficiency, which means that a much higher RF power level has to be presented at the rectifier’s input to fully support the tag (output load). For low efficiency levels (e.g. 10%), as much as -15 dBm can be required at the harvester input terminal. An RFID tag dipole can have a gain as high as 2 dBi, which means that at least -17 dBm of illuminating RF power is required at the tag. For max EIRP = 36 dBm, this gives a link margin of 53 dB, or 12 meters max (in free-space), to support over-the-air operation of the tag. Bringing the advantage of much higher EIRP levels from a cellular BTS at 3.5 GHz (e.g. 64 dBm), for a tag of similar capabilities and DC consumption, the link margin increases. In addition to that, for the higher frequencies, a higher tag directivity can be achieved without sacrificing too much real estate for an antenna. For example, a tag antenna with 2-5 dBi gain can be achieved while maintaining a practical size. This then lowers the minimum illumination power level required and boosts the link margin. In section 3, example maximum power-up distances are calculated for passive tags in several cellular environments. All the above refer to the case where a tag is solely supported by an RF field, i.e. the tag is alive only when a strong carrier is present. It is clear that for higher capability devices (e.g. with processing and sensing) with a power consumption of 1 mW (0 dBm DC), a much higher power level has to illuminate the tag and/or additional harvesting methods have to be employed, to guarantee energy availability.

Additional options for energy availability at a backscatter tag include:

- Use of supercapacitors for energy buildup (slow charging) when RF power availability is low; discharge of capacitors when sufficient energy is available for a tag backscattering and/or sensing operation.

- Augmentation of RF harvesting with other sources, such as solar panels. Low-profile flexible solar panels can now even be used indoors in low-to-moderate lighting conditions.

- Thin-film batteries or coin-cell batteries as dedicated power sources. The additional cost can be tolerated in certain applications and the advantage of low or zero maintenance can far outweigh that additional cost.

- Combination of the above, depending on the application constraints.

Assumption 1: For higher capability devices (e.g., with processing and sensing capabilities as proposed in [2]) with a power consumption of 1 mW (0 dBm DC), a much higher power level is needed to illuminate the tag and/or additional energy sources need to be considered, to optimize energy availability.

#### 3 Link Budget for Ambient IoT Deployment Scenarios

With reference to the appendix A link budget calculations for an indoor UE activator - gNB reader scenario a summary of achievable coverage for monostatic and bi-static configurations for 3GPP Ambient IoT backscatter device type A & B is listed and compared to RFID backscatter type RAIN and active BLE as depicted in table 1.

Table 1. Coverage comparison for different standards, device types and link configuration

Some observations can be made for the 3 example cases in table x as follows:

- Case 1 (Mono-Static):

  - All backscatter devices are limited by the activation range which for mono-static is equal to the reader range.

  - Ambient IoT device B is outperforming RFID mainly due the lower activation level which compensates the lower activation TX power.

  - Ambient IoT device A has the lowest activation range limited to a few meters.

- Case 2 (Bi-static with maximum activation distance):

  - Both Ambient IoT device A & B outperforms RFID on reading range due to the favorable gNB reader performance compared to the RFID reader.

  - For the same reason Ambient IoT device A has longer reading range than BLE.

- Case 3 (Bi-static with 1m activation distance):

  - When the Ambient IoT activator is really close the reading range for the Ambient IoT devices is increased significantly supporting ISD of 500m for type A and higher for type B outperforming BLE.

These examples suggest that:

- Ambient IoT backscatter activation distance is limited to about 20 meters max for device type B and a couple of meters max for a device type A.

- For both Ambient IoT device type A and B a bi-static backscatter configuration is highly favorable for significant extension of the overall coverage over mono-static operation.

Given these constraints, a gNB only macro grid deployment seems not feasible for ambient IoT. To illuminate passive IoT devices an extremely dense gNB deployment would be needed which will not be viable from a cost point of view. Thus, the preferred deployment option is to enable UEs as ambient IoT illumination/activation devices with both gNB and UE’s as reader entities. The UE deployment in a cell is denser and dynamic in nature and the enablement of UEs as activators will make Ambient IoT deployment a realistic scenario. An enhanced illumination scenario is to use multiple UEs for simultaneously illumination for increased activation range.

Proposal 1: Study Ambient IoT with main emphasis on UE illumination/activation deployment scenarios.

#### 4 Further Deployment Enhancements

Use of reflection gain on A-IoT devices for range enhancement

Devices communicating with backscattering modulate information by alternating the antenna connection with different passive loads. These loads will alter amplitude and/or phase of the incident signal and the reflected signal will carry digital information. However, with passive loads, the reflection coefficient (ratio between reflected and incident signal voltage Γ = Vrefl/Vinc) magnitude at any given state will always be less than unity.

The use of passive loads is a necessity for A-IoT Type A devices that solely rely on energy harvesting, since they minimize the power required for communication. However, Type B devices that have a certain amount of energy storage available need not be limited to passive loads. Such devices can use negative resistance networks, which are single-port amplifiers with a reflection gain, i.e., |Γ| >> 1. These are also referred to as reflection amplifiers and can be used in the place of passive loads when a source of bias current is available. These reflection amplifiers can operate in the microwatt range and can, therefore, be supported by small batteries or other charge tanks.

A reflection amplifier with G = 20log10|Γ| = 15 dB power gain can increase the AIoT device modulation factor M= ¼|Γ1 – Γ0|2, which translates into increased backscatter signal SNR at the reader and longer operating range. A type B device that uses reflection gain in one modulation state and passive loading in the other (e.g. turning the amplifier on and off to perform OOK) will have for example Γ1= Gejφ1 and Γ0= 0 (or, in other cases, any value |Γ0| < 1). Then the modulation factor is M = ¼ G , which is greater than the best case M=1 for type B devices, as long as the power gain G > 2 (i.e., more than 3 dB reflection amplifier gain).

A type B device that utilizes the reflection amplifier in both states and performs antipodal modulation (BPSK) can have Γ1= Gejφ1 and Γ0 = -Γ1, making the modulation factor M = G, which is always greater than unity. This greatly enhances the backscatter reader SNR and thus the operating distance [5]. For the link budget formulas in Appendix A, in type B device calculations M = G can be used when BPSK modulation is considered.

In Table 1 it can be seen that a Type B device that can be activated 22 m away from the activator can be received by a reader 51 m away at the reference sensitivity level for 10 kbps BPSK. A reflection amplifier added with 15dB gain boosts the range to 205 m between the tag and the reader as also depicted in Table 1. For activation distance of 1 m (e.g., a UE activator close to the Type B device) the read range can reach several kilometers.

Utilization of reflection amplifiers will need careful considerations regarding activation power levels and frequency of operation. Since reflection amplifiers are negative resistance networks, they behave as sub-biased oscillators. Two practical issues to take into consideration are:

- High activation power levels can lead the reflection amplifier to unstable operation, i.e., self-oscillation due to injection of high input RF power.

- Reflection amplifiers are typically highly-tuned devices, therefore their activation is feasible over a narrow bandwidth.

Depending on implementation, the first point will determine the minimum distance between an Activator and a type B device, so that reflection gain operation is stable. The second point will determine the frequency allocation of the network and therefore the activator illumination frequency to guarantee benefit from the reflection gain.

Use of multiple A-IoT devices constellation for range enhancement

Mounting more than one A-IoT device on an asset in a defined physical constellation, e.g., a one- or two-dimensional array configuration, may increase the combined reflection gain in specific fixed directions. Assuming the phase of the A-IoT device reflection is adjustable and configurable for each A-IoT device in the constellation the direction of combined reflection gain enhancement can be adjusted to point towards specific readers.

The number of possible combined reflected beam configurations (phase settings) will be low for a low number of A-IoT devices implemented with low bit phase shifters and a full sweep of all possible combined beam configurations could be a simple and viable approach for each activation cycle. Knowing the order of the A-IoT devices used in an array configuration will be beneficial since the number of configured beam configurations can be reduced to a low number including only the best beam configurations.

Proposal 2: Include Ambient IoT reflection gain enhancement techniques into the Ambient IoT study.

Early coverage failure warning for low complexity Ambient IoT devices

Ambient IoT devices are limited in their communication range, especially device type A & B. Examples of activation and response ranges are listed in section 3 and these range limitations pose a challenge for the Ambient IoT device discovery and for mobility of the ambient IoT device and/or the activator/reader devices. The main limitation in the link budget is the activator to Ambient IoT device range, which may be down to a few meters for device type A and less than ISD for device type B. This is illustrated in Figure 1 in which the Ambient IoT Device is named a Tag.

Figure 1. Illustration of Ambient IoT activation and reading limitations for two different Ambient IoT device (Tag) positions.

In Figure 1 the activator is transmitting an activation signal at a given TX power level and two tag position scenarios are depicted.

- For the tag in position 1 the activation signal incident RF power is above the tag activation limit and the tag will wake up and start modulating the backscatter reflection. The backscatter level at the reader is above sensitivity limit and thus reading is successful. Even at reduced activator TX power level the tag activation and reading may remain successful.

- For the tag in position 2 the backscatter level at the reader is above sensitivity limit and thus reading is feasible, however, the activator incident RF power is below the tag activation limit and thus the backscatter will not be modulated by the tag and the reading will fail. At the reader side it is not straight forward to confirm that the failure root cause is the Activator-tag distance and there is no early warning.

Observation 1: An Ambient IoT device activation-reading session may fail due to Activator-tag distance (a) or due to tag-reader distance (b). At the reader side it is not straight forward to confirm root cause for a failed reading being (a) or (b)

Capability to predict and distinguish failures due to activator-tag distance form failures due to tag-reader distance will be highly beneficial enabling make-before-break activator/reader handover by early out of coverage warning securing a stable Ambient IoT communication in dynamic operating conditions.

Proposal 3: Include Ambient IoT communication link failure mechanisms and failure early warning identification schemes into the Ambient IoT study.

As an alternative to a handover upon an out of coverage warning for the Activator to Ambient IoT device link this early warning could trigger procedures to increase the power level available at the Ambient IoT device e.g., by increasing the number of involved activators.

#### 5 Conclusion

The contribution suggests the following proposals:

Assumption 1: For higher capability devices (e.g., with processing and sensing capabilities as proposed in [2]) with a power consumption of 1 mW (0 dBm DC), a much higher power level is needed to illuminate the tag and/or additional energy sources need to be considered, to optimize energy availability.

Assumption 2: The link budget for illuminating a passive IoT device will be the limiting factor for deployment. In realistic case, an illumination distance of less than 20m can be assumed for Device B or proximity (1-2 m) illumination for Device A.

Observation 1: An Ambient IoT device activation-reading session may fail due to Activator-tag distance (a) or due to tag-reader distance (b). At the reader side it is not straight forward to confirm root cause for a failed reading being (a) or (b)

Proposal 1: Study ambient IoT with main emphasis on UE illumination deployment scenarios.

Proposal 2: Include Ambient IoT reflection gain enhancement techniques into the Ambient IoT study.

Proposal 3: Include Ambient IoT communication link failure mechanisms and failure early warning identification schemes into the Ambient IoT study.

#### 6 References

- RP-222685, Study on Ambient IoT.

- S1-220192, New SID: Study on Ambient power-enabled Internet of Things.

- RP-223396 Study on Ambient IoT (Internet of Things) in RAN

- RP-223451 https://www.3gpp.org/ftp/TSG_RAN/TSG_RAN/TSGR_98e/Docs/RP-223451.zip

- J. Kimionis, A. Georgiadis, A. Collado and M. M. Tentzeris, "Enhancement of RF Tag Backscatter Efficiency With Low-Power Reflection Amplifiers," in IEEE Transactions on Microwave Theory and Techniques, vol. 62, no. 12, pp. 3562-3571, Dec. 2014, doi: 10.1109/TMTT.2014.2363835.

- J. D. Griffin and G. D. Durgin, "Complete Link Budgets for Backscatter-Radio and RFID Systems," in IEEE Antennas and Propagation Magazine, vol. 51, no. 2, pp. 11-25, April 2009, doi: 10.1109/MAP.2009.5162013.

### A Deployment Options and Link Budget Considerations

During the RAN#98 email discussion [4], it was highlighted the following Ambient IoT devices characteristics that can be used for defining Ambient IoT device types:

Proposal 5-4: Define the following topologies as starting points for the next meeting: - Topology (1):BS <-> Ambient IoT device - NOTE 1: Includes the possibility of BS Rx and BS Tx in different BSs - Topology (2): BS <-> intermediate node <-> Ambient IoT device - NOTE 1: Intermediate node can be relay, IAB, UE, repeater, etc. which is capable of ambient IoT - Topology (3): BS <-> assisting UE <-> Ambient IoT device <-> BS - FFS: If the two BS can be different - Topology (4): UE <-> Ambient IoT device - FFS: Topology (5) UE <-> Ambient IoT device <-> {BS or UE} NOTE: For potential topology (5), discuss its relation with other topologies, its necessity, etc. in RAN#99. NOTE for all topologies: The Ambient IoT device may be provided with carrier wave from another node(s) either inside or outside the topology NOTE for all topologies: The links in each topology may be bidirectional or unidirectional FFS: Whether to consider combination of different topologies in the study. FFS: BS, UE, or assisting UE could be multiple BSs, UEs or assisting UEs, respectively

For a transmitter (Tx) –  Tag – receiver (Rx) link, the following link parameters are relevant:

PT:		Output power from Tx

Ptag:		Received power at Tag

PR:		Received power at Rx

GT:		Tx antenna gain

GR:		Rx antenna gain

Gtag:		Tag antenna gain

λ:		Carrier wavelength

M:		Backscatter tag modulation factor (M=0.25 for passive tags, M=1 for semi-passive tags, Tag modulation factor depends on backscatter signal constellation.)

d1: 		Tx to Tag distance

d2: 		Tag to Rx distance

γ: 		Path loss exponent (γ=2 for free space, γ=3 for fading environment)

The received power at the tag from a given transmitter is:

The received power is entering the tag’s RF harvester (to be converted to DC power) or reflected for communication (backscattering operation).

At the receiver, the received backscatter signal power is:

Note that the above formula is generalized and applies to all backscatter deployment scenarios [6]. For a monostatic case it is imperative that d1 = d2 and GT may be equal to GR. For a bistatic case where the Tx and Rx use equal antenna gain (e.g. two access points) the formula can be simplified by setting GT = GR. For a bistatic case where different equipment is used for Tx and Rx (e.g. a UE and a BTS), all parameters can be set individually.

Considering a tag performing binary modulation with a bitrate of 10 kbps and a target (uncoded) BER = 1%, the required backscatter signal SNR at the Rx is SNRmin = 4.3 dB. Assuming a 15 kHz signal bandwidth, a receiver noise figure NF = 6 dB, a shadowing/fade margin of F=10 dB, the reader reference sensitivity is given by:

and has a value of approximately S = -112 dBm.

For battery-less tags that harvest RF energy (Device A), two conditions must be met for successful communication:

- Received illumination power at the tag Ptag must exceed a certain power up threshold Pthr in order for the tag to “wake up”. In the following calculations, a threshold of Pthr = -19 dBm is assumed (common value for passive RFID tags). This typically governs the “max operating range”.

- Received backscattered power at the receiver PR must exceed the read sensitivity limit S (S = -112 dBm in the following calculations)

For battery-assisted tags or tags that use other non-RF energy sources, such as solar cells (Device B), the Tx - Tag distance can be greatly increased due much lower Pthr and the second condition is dominant.

Three different bistatic use cases with UE illumination are depicted in Table 3 with some example communication ranges. The path loss exponent γ is assumed to be 3 for outdoor cases and 2.5 for indoor cases, while the Rx reference sensitivity, S, is -112 dBm for 10 kbps data rate. For passive tags (Device A), the modulation factor is M=0.25 and for semi-passive tag (Device B) M=1. Note that all tags are assumed to have an omni-directional antenna (Gtag = 2 dBi), which represents a baseline scenario. In Table 2 the first case from the left is illumination of tags in an area from a UE and reception from a fixed BTS. The middle case involves indoor small-cell deployment with illumination of tags from a UE and reception from small-cell access point. The case to the right involves indoor small-cell deployment with tag illumination and reading by two different UEs. The parameter settings used for the range calculations in table 2 are listed in table 1.

| UE Tx Power (PTX) | 19 dBm |
| --- | --- |
| UE Tx Gain (GTX) | 5 dBi |
| Rx Gain (GRX) | 5 dBi (UE), 27 dBi (gNB), 17 dBi (indoor gNB) |
| Rx Sensitivity (S) | -112 dBm |
| Tag Gain (GTag) | 2 dBi |
| Tag power-up threshold (Pthr) | -19 dBm |
| Tag Modulation Factor (M) | 0.25 (Device A), 1.00 (Device B) |
| Path Loss Exponent (γ) | 3.0 (outdoor urban), 2.5 (indoor) |
| Carrier Frequency (fc) | 3.5 GHz |

Table 2. Assumed parameter values for link budget calculations in Table 2.

|  | DTXàtag | DtagàRx | DTXàtag | DtagàRx | DTXàtag | DtagàRx |
| --- | --- | --- | --- | --- | --- | --- |
| Device A (in proximity to UE) | 1.1 m | 292.5 m | 1.1 m | 360.5m | 1.1 m | 116.5 m |
| Device B (in proximity to UE) | 1.5 m | 336.5 m | 1.5 m | 470.5 m | 1.5 m | 156.5 m |
| Device B (mid-distance) | 22.5 m | 22.5 m | 26.5 m | 26.5 m | 15 m | 15 m |

Table 3. UE illumination scenarios with different receive options for device type A & B.

Figure 2. Operating regions for UE-tag-gNB scenario.  Device A operation (left) is governed by power-up threshold. Device B operation (right) is receiver-sensitivity limited. Yellow indicates sufficient received SNR.

Figure 3 Operating regions for UE-tag-indoor gNB scenario.

Figure 4. Operating regions for UE-UE scenario.

It is interesting to note that for bistatic cases, the two links (illuminator (TX)-to-tag and tag-to-receiver(RX)) maximum distances depend on each other. For a short Tx-tag distance, a significantly higher tag-Rx distance can be achieved. The opposite is also true for semi-passive tags, but not for passive tags that need to be energized by the illuminator. Figure 2 shows this contrast for the UE illumination-gNB reception case, where a yellow shaded region corresponds to successful communication operation. The left sub-figure corresponds to an RF harvesting tag (Device A), for which the Tx-tag distance is very limited, since the tag power up threshold must be exceeded. Despite that fact, very long tag-Rx ranges can be achieved (several hundreds of meters) for a sufficiently sensitive receiver. The right sub-figure corresponds to an energy-assisted backscattering tag (Device B) that does not rely on RF harvesting. Since there is no RF power-up constraint, the total operating region is larger compared to Device A. Keeping a short Tx-tag range allows for exponentially larger tag-Rx range, as well as the opposite. Similar trends are observed for the case of UE illumination and indoor gNB reception (Figure 3), as well as UE illumination and UE reception (Figure 4).

Table 2 provides example numbers for the feasibility of each deployment, for RF harvesting tags (Device A) and energy-assisted tags (Device B). In general, Device B can afford longer Tx-tag ranges than passive tags, due to the relaxed power up requirements, and therefore can be used for larger area coverage.

For UE illumination cases, where the UE can be brought to the proximity of the tags, both Device A and Device B can be used, and the uplink range from the tag to a base station can reach 300 meters, which can accommodate similar inter-site distances (ISD).

Assumption 2: The link budget for illuminating a passive IoT device will be the limiting factor for deployment. In realistic case, an illumination distance of less than 20m can be assumed for Device B or proximity (1-2 m) illumination for Device A.