3GPP TSG-RAN #101	 RP-231618

Bangalore, India, September 11-15, 2023

Title:		Ambient IoT positioning

Agenda Item:    		9.2.2

Source:	     	Nokia, Nokia Shanghai Bell

Document for:	Discussion and decision

### 1 Introduction

Ambient IoT RAN led Study Item Rel-18 was approved in RAN#97e [1]. The motivation of Ambient IoT study is to support ultra-low cost and ultra-low power devices for the IoT applications. SA1 completed a study item on ambient power enabled IoT [2], which focuses on use cases and service requirements, and it includes the objective of defining performance requirements and KPIs as well.

Currently, there is on-going RAN study item on Ambient IoT (Internet of Things) in RAN [3], which focuses on 3GPP IoT technology, suitable for deployment in a 3GPP system, which relies on ultra-low complexity devices with ultra-low power consumption for the very-low end IoT applications. The study provides clear differentiation, i.e., addressing use cases and scenarios that cannot otherwise be fulfilled based on existing 3GPP LPWA IoT technology e.g. NB-IoT including reduced peak Tx power.

In terms of energy storage, the study will consider the following device characteristics:

- Pure battery less devices with no energy storage capability at all, and completely dependent on the availability of an external source of energy

- Devices with limited energy storage capability that do not need to be replaced or recharged manually.

Device categorization based on corresponding characteristics (e.g., energy source, energy storage capability, passive/active transmission, etc.) may be discussed during the study, in relation with the relevant use cases. The device’s peak power consumption shall be limited by its practical form factor for the intended use cases and shall consider its energy source.

The Ambient IoT devices support communication with reader via reflection or transmission which can be supported by very low complexity hardware. The Ambient IoT device can collect energy from environment, such as RF signals, solar energy, vibration, heat, etc., and can potentially also be equipped with a small capacity battery. Hence, Ambient IoT is a promising technique to achieve even lower cost/power requirements compared to existing cellular IoT technologies, e.g. NB-IoT, LTE-M, RedCap, etc.

The RAN level study will compare and assess the feasibility of meeting the design targets for relevant use cases on the basis of the deployment scenario(s) appropriate to it and identify assumptions on required functionality to be supported.

### 2 Positioning anchors using Ambient IoT backscatters

The scenario considered herein is an indoor asset tracking problem using cellular base stations for the asset positioning estimation. Similar to GPS, the system must be scalable to localize many target devices and achieve centimeter level accuracy. In an asset tracking system, following tracking requirements may exist: machinery and devices in shop floors, bin tracking, product tracking and person tracking. The required positioning accuracy for mobile objects in a factory floor may be smaller than 1m according to 3GPP TS 22.261 specification. A typical indoor positioning system requires at least three base stations to perform a trilateration at the receiver node using methods like time of arrival, time difference of arrival, angle of arrival or received signal strength-based methods. However, for the indoor scenario it is costly to deploy multiple base stations and challenging to maintain LOS conditions and low interference.  Map assisted localization requires a-priori mapping of the entire environment to get the help of multipaths which is labor intensive and any change in the environment will reduce the accuracy. Alternatively, a backscatter communication system can be deployed instead of active transmission. An example of such a solution is depicted in Figure 1 with the asset as RF source to maximize link budget for cases where the asset is close to the Ambient IoT devices (tags). gNB or PRU as RF source may also be considered.

Figure 1: Localization solution using Ambient IoT backscatters as positioning anchors (tags)

Proposal 1: Include Ambient IoT devices as positioning anchors for the use cases in the Ambient IoT study [1].

Furthermore, using Ambient IoT device type C as positioning anchors may allow for active transmission of positioning signals. However, A-IoT type C devices have limited capability and may not be NR clock synchronized and in this case provision for mitigating A-IoT device reference clock drift and obtain common perception of time of day may be studied.

### 3 Ambient IoT Positioning

Ambient IoT in 3GPP should enable ultra-low cost and ultra-low power ambient IoT devices. The density of such ambient IoT device deployment can be expected to be many orders of magnitudes higher compared to any known 3GPP deployment. Thus, a key element for such deployment is the localization of ambient IoT devices. It is not only required to know the identity of the ambient IoT device and to communicate with the device, but equally important is knowledge of the position of the ambient IoT device.

Figure 2: Ambient IoT positioning for 3GPP

Figure 2 shows an example of an Ambient IoT deployment based on multi-static backscattering with positioning capabilities. The positioning of ambient IoT devices can be based on the existing 3GPP positioning architecture, using a 3GPP node to trigger the event and one or more 3GPP nodes to listen to a given reference signal e.g., another gNB or Positioning Reference Unit (PRU). The positioning session can be controlled and monitored by the location management function (LMF). A positioning session can be enabled by a UE in the network or from the network side. Sidelink can be another mean to configure UEs in proximity of the ambient IoT device. Sidelink UEs with known locations could be used as anchors for Ambient IoT positioning as well.

Determining the relative or absolute position of A-IoT device using NR elements (like gNB or UEs) is a challenging task for the following reasons:

- The A-IoT device needs to accumulate sufficient charge to respond to an interrogation/activation signal send by an NR element. Since the NR NW does not know the current charge of an A-IoT device, it cannot effectively estimate how long time the device took to charge before responding, therefore the NW cannot tell, upon reception of the reply with a delay D, how much the propagation delays versus the charge delay (denoted as dc) contributed to the total delay D.

- The A-IoT device can be stimulated to respond on different carrier frequencies. However, some carriers may be busier/more interfered than others (i.e., occupied by either several concomitant opportunistic A-IoT discoveries or other NR traffic), therefore the A-IoT reply may be better received on some carriers than others. However, due to the opportunistic nature of the A-IoT discovery, the best carrier for localizing a given A-IoT device is not known in advance.

- In higher frequency ranges, the interrogator/activator NR element must additionally point the activation signal to the direction of the A-IoT device to ensure the device charges sufficiently to respond. However, the direction of the A-IoT relative to the activator is not known in advance, so the later element must choose a best direction blindly.

- To ensure positioning performance, the received signal quality (e.g., RSRP or SINR) at anchors shall meet certain positioning measurement requirement, e.g., the backscattered signal power received at anchors shall be not less than a given threshold. The received signal power at anchor reflected by the A-IoT device A/B is generally associated with the output power of PRS from the activator when the rectifier of A-IoT device works in the diode linear mode. However, it’s known from the link budget for A-IoT device A/B, if the received signal power at the A-IoT device is lower than a certain power up threshold, the activator would be not able to wake up the A-IoT device for positioning; Furthermore, an excessive output power of PRS from the activator would make the rectifier of A-IoT device  saturated, which would be not helpful to improve the received signal quality at anchors and further increase network interference and power consumption of the activator (e.g., UE). For this, it makes sense to control output power of PRS from the activator to ensure the positioning performance whilst minimizing network interference and power consumption of activator (e.g., UE).

- Depending on the device type, the AIoT devices synchronization to the NW may be problematic, thus methods that are robust to synchronization errors and other processing delays associated with the AIoT may be preferred.

Thus, to solve the localization problem, means also to solve the synchronization and interference mitigation associated problems.

Proposal 2: RAN to study AIoT localization problem in connection to aspects such as interference mitigation, power control and robustness against processing delays associated with the AIoT device.

#### 4 Ranging aspects

Determining the range (i.e., distance to a given NR element) of a multi-frequency Ambient IoT device using NR elements (like gNB or UEs) is a challenging task, with a main challenge being that the A-IoT device needs to accumulate sufficient charge to respond to an interrogation/activation signal sent by an NR element. Since the NR NW does not know the current charge of an A-IoT device, it cannot effectively estimate how long time the device took to charge before responding, therefore the NW cannot tell, upon reception of the reply with a delay D, how much the propagation delay versus the charge delay contributed to the total delay D.

Proposal 3: RAN to consider ranging procedures in which the NR reader is enabled to distinguish among different propagation delays and A-IoT device charge delays. Specifically:

### The propagation delays of the activation signal i.e., the time the activation signal takes to travel the distances activator – reader and activator – A-IoT device.

### The propagation delay of the A-IoT device signal i.e., the time the A-IoT device reply takes to travel the distance A-IoT device – Reader.

#### 5 Charging vs localization

The NR NW can discover/localize the A-IoT device only if the A-IoT device hears an activation signal loud enough so that it can start modulating the backscatter (Type A/B) or generate a response (Type C) which is loud enough to be heard by another nearby NR element (a gNB, RSU, UE, etc.).

That means that an A-IoT device may start replying to an activation signal as soon as the harvested energy, Eh, is above a minimal threshold, E1. However, for device type B/C, depending on how large Eh is, the modulated reflection/reply may be heard by a variable number of readers and may be measured with various degrees of accuracy.

In other words, for device type B/C, the more often the device needs to be localized (and thus the A-IoT device needs to respond), the less time the device has for energy harvesting, and thus the lower the transmit on-time and/or reflection gain/transmit power may be. As a result, the chances for successful localization are also lowered. To make matters worse, an A-IoT device whose localization has failed may be re-activated for backscatter/re-transmission immediately after failure, giving it very little time to charge and thus only marginally increase the chance of successful localization in the next round.

The NW must decide how often to activate an A-IoT device so that the device may be localized within the target latency and accuracy constraints, while giving the device enough time to charge sufficiently well to be detected accurately by enough readers.

For low resource overhead methods can be considered to combine IoT device positioning and charging procedures. The activator may transmit beam formed charging signal with the A-IoT device reporting charging efficiency for different beam directions.

### Proposal 4: RAN to consider procedures to enable the NW to detect, localize and learn the energy level and/or charging profile of an A-IoT device type A/B/C so that the NW can pause/resume/recover and optimize the next detection/localization session. Combination of charging and positioning procedures should also be considered.

#### 6 Conclusion

The contribution suggests the following proposals:

Proposal 1: Include Ambient IoT devices as positioning anchors for the use cases in the Ambient IoT study [1].

Proposal 2: RAN to study AIoT localization problem in connection to aspects such as interference mitigation, power control and robustness against processing delays associated with the AIoT device.

Proposal 3: RAN to consider ranging procedures in which the NR reader is enabled to distinguish among different propagation delays and A-IoT device charge delays. Specifically:

### The propagation delays of the activation signal i.e., the time the activation signal takes to travel the distances activator – reader and activator – A-IoT device.

### The propagation delay of the A-IoT device signal i.e., the time the A-IoT device reply takes to travel the distance A-IoT device – Reader.

### Proposal 4: RAN to consider procedures to enable the NW to detect, localize and learn the energy level and/or charging profile of an A-IoT device type A/B/C so that the NW can pause/resume/recover and optimize the next detection/localization session. Combination of charging and positioning procedures should also be considered.

#### 7 References

- RP-222685, Study on Ambient IoT.

- S1-220192, New SID: Study on Ambient power-enabled Internet of Things.

- RP-223396, Study on Ambient IoT (Internet of Things) in RAN.

- RP-230055, Ambient IoT link budget considerations.