3GPP TSG RAN Meeting #101	RP-231627

Bangalore, India, September 11-15, 2023

Agenda Item:	9.2.2

Source:	Ericsson

Title:	Input to Study on Ambient IoT in RAN

Document for:	Discussion, Decision

# 1	Introduction

This contribution presents our input to the ongoing RAN study item on Ambient IoT [1].

# 2	Device categorization

In this section, we discuss device categorization based on characteristics, such as energy storage capability and transmission/reception capabilities.

## 2.1	Categorization based on energy storage capability

For categorization based on energy storage, RAN#98e made the following working assumption:

Working assumption: This framework is used to categorize energy storage: - Storage 1: no storage at all - Storage 2: Up to E1 joules - Storage 3: Up to E2 joules FFS: In RAN#99 value(s) of E1, E2 and it is possible that E1=E2, in which case we have only two storage categories. Note in this case that storage 2 and 3 could be replaced by a single description such as “limited energy storage”, instead.

Further, RAN#99 discussed the following options for capturing the energy storage levels without reaching a conclusion [5]:

Proposal 5.1-1-v3: The TR captures energy storage levels as: - Option 1: Two levels - Storage 1: Without energy storage - Storage 2: With limited energy storage. - Option 2: Three levels - Storage 1: Without energy storage - Storage 2 and 3: Up to E1/E2 joules - For this case, 2 exemplary traffic models are chosen, and E1, E2 calculated accordingly. - Model 1: power = per-device design target, rate = 5 kbps, message size = {as agreed under design target}. - Model 2: power = per-device design target, traffic = continuously ON until end of an inventory process. - Option 3: Status quo, i.e. TR reports the three storage levels, and that E2 > E1. (If no agreement, this option results).

With regards to Option 3, we think the differentiated levels E1 and E2 are arbitrary. That is, E1 and E2 differentiation should be pursued only if there is a qualitative difference between the two (e.g., E1 corresponds to regular capacitor, and E2 to super-capacitor or re-chargeable battery, where the latter has higher cost or size).

With regards to Option 2, the need to agree on specific E1 and E2 values is unclear to us. What matters the most for the technical solution in RAN is the power consumption since it determines the coverage and the device complexity but combining this with a suitable energy storage size should be up to UE vendors (and could depend on the intended use case, type of energy harvesting etc.). As discussed in Section 4, RAN has already agreed (at least as working assumption) on the power consumption target. Leaving the energy storage levels open gives more leeway to WGs during protocol design.

With regards to Option 1, considering the limited time remaining to conclude the RAN SI, this could be easier to agree and capture in the TR. Therefore, our preference would be Option 1. Nevertheless, it is worth clarifying that the energy storage levels for Ambient IoT devices are expected to be several orders of magnitude lower than what an NB-IoT device would typically support (e.g., coin-cell or AA batteries).

There is no need to agree on specific E1 and E2 values during the RAN SI. It is enough to agree on the design targets for power consumption, data rate, and message size.

TR 38.848 captures energy storage levels as follows:

- Two levels

  - Storage 1: Without energy storage

  - Storage 2: With limited energy storage.

- Note: The energy storage levels are expected to be several orders of magnitude lower than what an NB-IoT device would typically support.

## 2.2	Categorization based on transmission/reception characteristics

For device categorization based on transmission characteristic, RAN#98e made the following agreement:

Agreement: The following set of Ambient IoT devices are considered in the SI: - Device A: No energy storage, no independent signal generation, i.e., backscattering transmission - Device B: Has energy storage, no independent signal generation, i.e., backscattering transmission. Use of stored energy can include amplification for reflected signals - Device C: Has energy storage, has independent signal generation, i.e., active RF component for transmission FFS: Whether to include device function FFS: Whether to include a target maximum power consumption for each device FFS: Whether/how to describe what stored energy is used for (in addition to the statement for Device B) FFS: if combination of these devices will be considered.

On the third and fourth FFSs above, RAN#99 made the following agreement:

The following FFS points from the RAN#98e agreement don’t need further action: - FFS: Whether/how to describe what stored energy is used for (in addition to the statement for Device B) - FFS: if combination of these devices will be considered

On the first FFS, we think that the reception characteristic of the devices, in addition to their transmission characteristic, should be clarified. That is, Device A, B, and C must be able to receive data/commands from the BS (or another network node). In other words, “UL-only” devices are not considered as part of the study. This helps to ensure coexistence of Ambient IoT devices in a cellular network, change configuration of the devices, reception of acknowledgements, provide firmware updates, receive security/integrity messages from the network, and so on. Therefore, we propose the following:

UL-only devices, which cannot receive data/control signalling from the BS or an intermediate node, are not considered as part of the RAN SI.

# 3	RAN design targets

One of the objectives in the SID is to formulate a set of RAN design targets for the Ambient IoT use cases.

- Formulate a set of RAN design targets based on the identified deployment scenarios and their characteristics for the relevant use cases, at least including - Power consumption - Complexity - Coverage - Data rate - Positioning accuracy

In RAN#99, the set of targets were updated as follows:

Agreement: Agree to set at least the design targets below in Ambient IoT in the RAN SI. - Device power consumption - Device complexity - Coverage - Data rate - Maximum message size (or maximum ‘TB’ size) - Latency - Positioning accuracy - Connection/device density - Device speed (FFS absolute or relative or both)

In this section, we present our views on most of the above RAN design targets.

## 3.1	Device power consumption

Currently, the following text is captured in TR 38.848 Clause 5.1 on target device power consumption [3]:

For Device A, the power consumption target during transmitting/receiving is [≤ 1 μW] or [≤ 10 μW], For Device B, the target during transmitting/receiving is such that: - Device A power consumption ≪ Device B power consumption < Device C power consumption; or - Device A power consumption ≤ Device B power consumption < Device C power consumption. The device power consumption for Device C is ≤ 1 mW to ≤ 10 mW.

The feasibility of the above power consumption values has been assessed in Section 4.1. Based on the assessment, we propose the following TP:

For Device A, the power consumption target during transmitting/receiving is [≤ 1 μW] or [≤ 10 μW], For Device B, the target during transmitting/receiving is such that: - Device A power consumption ≪ Device B power consumption < Device C power consumption.; or - Device A power consumption ≤ Device B power consumption < Device C power consumption. The device power consumption for Device C is ≤ 1 mW to ≤ 10 mW.

Adopt the TP above in TR 38.848.

The power consumption targets above are “orders of magnitude lower than the existing 3GPP LPWA technologies”, including NB-IoT with reduced peak Tx power [1]. For reference, the NB-IoT device power consumption is ~500 mW with 23 dBm transmit power and ~300 mW with 14 dBm transmit power [14].

## 3.2	Device complexity

On the target device complexity, TR 38.848 Clause 5.2 captures the following:

Editor’s note: In RAN#99, the following is a working assumption. For Device A, the complexity target is to be comparable to UHF RFID ISO18000-6C (EPC C1G2). For Device B, the target is such that: - Device A complexity < Device B complexity < Device C complexity. For Device C, the complexity target is to be orders-of-magnitude lower than NB-IoT.

According to the SID, the complexity of the Ambient IoT device shall be orders of magnitude lower than NB-IoT and eMTC [1]. Within the Ambient IoT device category, large differentiation in applicable use cases and cost/complexity are needed for passive and active devices to minimize market fragmentation. Although the market potential of RFID-like passive tags (Device A) in a 3GPP system compared to a non-3GPP system is unclear, the working assumption above provide sufficient differentiation between the device categories. Therefore, we propose to confirm the working assumption by removing “Editor’s note” from Clause 5.2.

Confirm the confirm the working assumption on device complexity by removing “Editor’s note: In RAN#99, the following is a working assumption” from Clause 5.2.

## 3.3	Coverage

On coverage, RAN#99 has discussed the following proposals without reaching consensus:

Proposal 6-3c: Coverage design target is: - Indoor device, and for Device A & B & C: 50 m assuming typical 3GPP inter-site distances - Outdoor device, and for at least Device C: 500 m assuming typical 3GPP inter-site distances - WGs would be left to refine relevant channel model(s), ISD(s), etc., as needed. Proposal 6-3c-v2: Coverage design target is: - Devices which are indoors device, and for Device A & B & C: 50 m assuming Typical indoor 3GPP inter-site distances - Devices which are outdoors device, and for at least Device C: 500 m assuming Typical outdoor 3GPP inter-site distances WGs would be left to refine relevant channel model(s), ISD(s), etc., as needed.

Before agreeing on the coverage target, however, the following aspects should be considered:

- First, it should be considered whether the coverage reference points are the same for the different topologies. In our view, for topologies 1, 2, and 3, the coverage reference points should be the BS and the tag, while for Topology 4, it should be UE and the tag.

- Second, it should be considered whether the coverage targets are the same for Device A, B, and C. In our view, RAN should have a solution-agnostic approach, i.e., for the use cases and service requirements, evaluate which solution is most feasible. Therefore, the coverage targets could be the same for all devices but could be different depending on if the device is an indoor or an outdoor scenario.

- Third, it should be considered whether the emitter-to-tag distance for Devices A and B be part of the coverage target. We think emitter-to-tag distance is an important aspect and should either be captured as part of the target or as a metric that can be reported as part of the assessment. We have a slight preference of the latter approach.

- Fourth, it should be considered whether the coverage target comes with certain band assumptions. Although the coverage would be different depending on the band assumptions (e.g., low bands or mid bands), it might be simpler to agree on a single target. However, the band assumptions could be reported as part of the assessment.

Based on the above considerations, we propose the following:

Coverage design target is:

- Indoor device (for Device A & B & C): 50 m

- Outdoor device (for Device C): 500 m

- Note 1: For Topology 1 & 2 & 3, the coverage reference points are the BS and the Tag. For Topology 4, the coverage reference points are the UE and the Tag.

- Note 2: For Device A & B, the emitter-to-tag distance should be reported as part of the assessment.

- WGs can refine the target based on band assumptions, relevant channel model(s), ISD(s), etc., as needed.

## 3.4	User experienced data rate

On the target user experienced data rate, TR 38.848 Clause 5.4 captures the following:

The user experienced data rate target is, at least for the uplink, maximum not less than 5 kbps, and minimum not less than 0.1 kbps.

The feasibility of the above target is discussed in Section 4.4.

## 3.5	Positioning accuracy

RAN#99 discussed the following positioning accuracy target but could not reach an agreement [5].

Proposal 6-3g: Positioning accuracy design target is: - Indoor: 3 m @ 90% - Outdoor: several 10 m @ 90%

The above target reflects the SA1 use case requirements, and therefore, we think that the target can be adopted by RAN. The discussion on the feasibility of the target is discussed in Section 4.5.

Positioning accuracy design target is:

- Indoor: 3 m @ 90%

- Outdoor: several 10 m @ 90%

## 3.6	Latency

For the use cases listed in TR 22.840, the end-to-end latency can be as low as 100 ms and high as 60 s. The end-to-end latency is not a well-defined metric in RAN. In RAN, the typical latency metrics are control plane latency, user plane latency, and latency for infrequent small packets [12]. Their definitions are copied below [12]:

## 7.4 Control plane latency Control plane latency refers to the time to move from a battery efficient state (e.g., IDLE) to start of continuous data transfer (e.g., ACTIVE). ## 7.5 User plane latency The time it takes to successfully deliver an application layer packet/message from the radio protocol layer 2/3 SDU ingress point to the radio protocol layer 2/3 SDU egress point via the radio interface in both uplink and downlink directions, where neither device nor Base Station reception is restricted by DRX. ## 7.6 Latency for infrequent small packets For infrequent application layer small packet/message transfer, the time it takes to successfully deliver an application layer packet/message from the radio protocol layer 2/3 SDU ingress point at the mobile device to the radio protocol layer 2/3 SDU egress point in the RAN, when the mobile device starts from its most "battery efficient" state.

There needs to be further investigation on how to translate end-to-end latency requirements as in TR 22.840 to “RAN appropriate” latency targets listed above. Additionally, for devices that rely on energy harvesting, it is unclear whether/how to consider the energy harvesting duration when defining the latency targets.

Consider the following example for illustrative purposes:

Assume Device C needs to transmit one 50-byte packet and the supported data rate of the device is 100 bps. Hence, the time to transmit one packet is 4 s. If the transmitter consumes 10 mW and the energy harvester output is 10 µW, the device needs to harvest energy for 4000 s to transmit one packet. Whether this energy harvesting duration should be considered as part of the latency needs clarification.

The above aspects merit further study, and therefore, we think that setting RAN latency target can be left to the WGs. In TR 38.848, it might be enough to capture the maximum and minimum end-to-end latency according to TR 22.840 and indicate that WGs can introduce corresponding RAN latency targets, e.g., considering the energy harvesting duration.

Further investigation is needed on how to translate end-to-end latency requirements as in TR 22.840 to RAN latency targets, such as control plane latency, user plane latency, or latency for infrequent small packets.

For devices that rely on energy harvesting, it is unclear whether/how to consider the energy harvesting duration when defining the latency targets.

Capture the maximum and minimum end-to-end latency according to TR 22.840 in TR 38.848 and indicate that WGs can introduce corresponding RAN latency targets, e.g., considering the energy harvesting duration.

## 3.7	Message size

With regards to message size, the maximum for the identified use cases according to TR 22.840 is 1000 bits. Therefore, this can be adopted as the target but leaving room to RAN1 to refine the target for TB design purposes (as also suggested in [5]).

Maximum message size target is:

- Hundreds of bits, and < 1000 bits

- RAN1 to refine as needed e.g., for TB size design.

# 4	Comparison and feasibility assessment

In this section, we compare and assess the feasibility of meeting the RAN design targets described in Section 4.

## 4.1	Device power consumption

To our understanding, the device power consumption target, as in Section 4.1, concerns the peak power consumption that a device may consume during transmission or reception. That is, the target concerns the power consumption of the transmitter (Tx) and receiver (Rx) chains, and not the overall power consumption of the device, which also includes the power consumed by the sensors, the micro-controller unit, the power management unit, and so on.

For Device A, the Tx chain typically consists of a set of impedances and an RF switch (to toggle between the absorptive state and the different impendences during the reflective state). The switch is an active component, whereas the impedances are passive components. The power consumption of the switch (and therefore that of the Tx chain) is less than 1 μW. The Rx chain for Device A typically consists of a comparator and an active envelope detector which collectively consume a few μWs of power. The Device A also consists of a rectifier to harvest RF energy. The rectifiers are typically CMOS diodes with transistors or Schottky diodes and they have very lower power consumption [15]. Therefore, the power consumption target of ≤ 10 μW is feasible for Device A. However, such simple Rx chains may require special modulation techniques (e.g., those used in UHF RFID ISO 18000-6C). Further studies on the compatibility of those modulation techniques with cellular networks as well as the assumptions on continuous illumination for device A are needed. Moreover, detection techniques that would require a mixer, a local oscillator (LO), or a multi-bit ADC are not feasible for Device A as then the power consumption is expected to range in the order of tens (or even hundreds) of micro-watts. The price to pay for such low Rx power consumption is poor interference rejection, poor sensitivity of the detector, and inability to receive and process larger messages (e.g., for firmware update).

Device B is similar to Device A, but it has an energy storage (e.g., a capacitor or supercapacitor) to help power the Rx circuitry during reception and reflection of illuminating signals. It might also be equipped with a reflection amplifier to improve the UL coverage. Depending on the reflection gain and the amplifier technology (e.g., tunnel diodes, bipolar transistor, etc.), the DC bias power consumption could be several tens of microwatts to few hundreds of microwatts [7]. Since Device B has an energy storage, it might be possible to equip Device B with a mixer, a ring oscillator, and a multi-bit ADC to demodulate signals from the base station. The power consumption is then expected to be in the order of hundreds of micro-watts.

For Device C, the Tx chain consists of DAC, filter, mixer, oscillator, and PA. The exact value of the Tx power consumption would depend on the output power from the PA, PA efficiency, type of LO (ring or PLL), and so on. For example, the Tx power consumption may exceed 3 mW if the output power is to be 0 dBm (same as Bluetooth Low Energy) with a PA efficiency of 30%. The Rx chain can be based on either homodyne or heterodyne architecture, where the main components include detector, ADC, filter, IF amplifier, mixer, LO (which is shared with the Tx chain), a detector, and in some cases, also LNA. The detector could be an active envelope detector or any other type of low-power non-coherent detector, as currently being studied in the WUR SI [6]. Depending on the carrier frequency, bandwidth, and the frequency accuracy, the power consumption of the Rx chain is expected to be in the range of 0.5 to 1 mW. Therefore, a power consumption of ≤ 10 mW is feasible for Device C.

Based on the initial analysis, the following device design target for power consumption during transmitting/receiving is feasible:

- Device A ≤ 10 μW

- Device A ≪ Device B < Device C

- Device C ≤ 10 mW

If Ambient IoT is to be a RAN1-led SI, detailed power consumption evaluation is conducted in RAN1. RAN1 can refine the RAN power consumption target, if needed.

## 4.2	Device cost/complexity

The RAN design target for device cost/complexity, as in Section 4.2, is a qualitative target rather than a quantitative one. To quantify the complexity, e.g., in terms of chip area, gate equivalent count, or modem bill-of-material [8],[9], requires extensive study and is therefore not well-suited for the RAN SI. Furthermore, the complexity depends on what functionalities (e.g., as listed in Section 7.2 of [5]) are expected to be supported by the device, which has not yet been discussed. Therefore, from RAN SI point-of-view, we think it is enough to capture the target in the TR (without assessing the feasibility) and leave it to the WGs to further study the cost/complexity and refine the target, if needed.

Further study in RAN1 is needed to assess the feasibility of the following device complexity design targets:

- Device A: Comparable to UHF RFID ISO18000-6C (EPC C1G2)

- Device A < Device B < Device C

- Device C: Orders-of-magnitude lower than NB-IoT

## 4.3	Coverage

RAN has not agreed on assumptions that are relevant to evaluating coverage of an Ambient IoT deployment. Nevertheless, we have performed the coverage (link budget) analysis based on assumptions that we believe are reasonable. The assumptions are provided in Appendix A.

In Table 1 to Table 4, we summarize the results of our coverage analyses for Device A, Device B, and Device C. For Device B, we consider cases with and without reflection amplifier. Furthermore, for Device B, we assume that the energy storage will help power the Rx circuitry and the reflection amplifier, and therefore, the device does not rely on RF harvesting to charge the energy storage (which could be done by other means).

Table 1: Achievable coverage for Device A for different frequencies and path-loss models.

| Frequency | Free space |  | 3GPP InF-DH-NLOS [10] |  |
| --- | --- | --- | --- | --- |
|  | Emitter→Tag distance1 | Tag→Reader distance | Emitter→Tag distance1 | Tag→Reader distance |
| 0.7 GHz | 5 m | 55 m | 3 m | 43 m |
| 2.6 GHz | 3 m | 73 m | 2 m | 54 m |
| Note 1: This is the maximum emitter to tag distance, i.e., beyond this distance the received power is lower than the harvester sensitivity (activation threshold), which is insufficient to activate the tag. |  |  |  |  |

Table 2: Achievable coverage for Device B (without reflection amplifier) for different frequencies and path-loss models

| Frequency | Free space |  | 3GPP InF-DH-NLOS [10] |  |
| --- | --- | --- | --- | --- |
|  | Emitter→Tag distance1, 2 | Tag→Reader distance | Emitter→Tag distance1, 2 | Tag→Reader distance |
| 0.7 GHz | 5 m | 55 m | 5 m | 26 m |
|  | 10 m | 27 m | 10 m | 13 m |
|  | 20 m | 14 m | 20 m | 6 m |
| 2.6 GHz | 5 m | 44 m | 5 m | 22 m |
|  | 10 m | 22 m | 10 m | 11 m |
|  | 20 m | 11 m | 20 m | 5 m |
| Note 1: For Device B, we assume that the energy storage will help power the Rx circuitry during reception, and therefore, the coverage is not limited by the -20 dBm harvester sensitivity. Note 2: The Device B (without reflection amplifier) that relies on RF harvesting is expected to have similar coverage as that of Device A for the same emitter→tag distance. |  |  |  |  |

Table 3: Achievable coverage for Device B (with reflection amplifier) for different frequencies and path-loss models

| Frequency | Free space |  | 3GPP InF-DH-NLOS [10] |  |
| --- | --- | --- | --- | --- |
|  | Emitter→Tag distance1 | Tag→Reader distance | Emitter→Tag distance1 | Tag→Reader distance |
| 0.7 GHz | 5 m | 546 m | 5 m | 213 m |
|  | 10 m | 273 m | 10 m | 108 m |
|  | 20 m | 138 m | 20 m | 54 m |
| 2.6 GHz | 5 m | 439 m | 5 m | 177 m |
|  | 10 m | 224 m | 10 m | 90 m |
|  | 20 m | 111 m | 20 m | 45 m |
| Note 1: For Device B, we assume that the energy storage will help power the Rx circuitry during reception and the reflection amplifier rand therefore, the coverage is not limited by the -20 dBm harvester sensitivity. |  |  |  |  |

Table 4: Achievable coverage for Device C for different frequencies and path-loss models

| Frequency | Free space |  |  | 3GPP InF-DH-NLOS [10] |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | BS→Tag distance | Tag→BS distance (Tx power = 0 dBm) | Tag→BS distance (Tx power = -10 dBm) | BS→Tag distance | Tag→BS distance (Tx power = 0 dBm) | Tag→BS distance (Tx power = -10 dBm) |
| 0.7 GHz | 18.7 km | 848 m | 273 m | 4.3 km | 360 m | 141 m |
| 2.6 GHz | 9.38 km | 668 m | 206 m | 2.28 km | 289 m | 114 m |

Based on the above tables, the following observations can be made:

For Devices A and Device B, there is strong dependency between emitter-to-tag and the tag-to-reader distance. The larger the emitter-to-tag distance for these devices, the smaller the tag-to-reader distance.

Devices A and B require continuous illumination using RF signals so as to forward the data to the reader.

In general, the emitter-to-tag distance limits the overall coverage for Devices A and B to very short-short range. Therefore, Devices A and B are not suitable for outdoor deployment (i.e., Deployment scenarios 2 and 4 with Topology 1).

For Devices A and Device B (without reflection amplifier), the indoor device coverage target of 50 m is feasible but only when the tag is within 5 m from the emitter. For Device B (with reflection amplifier), the target is feasible even when the tag is at few tens of meters from the emitter.

For Device C, the indoor as well as outdoor device coverage targets are easily feasible even with much lower transmit power values compared to NB-IoT.

An in-depth study is needed in the WG to assess the coverage of Ambient IoT devices under realistic operating conditions and by aligning the evaluation assumptions among companies.

## 4.4	User experienced data rate

For non-full buffer traffic, user experienced data rate is the 5%-percentile (5%) of the user throughput. User throughput (during active time) is defined as the size of a burst divided by the time between the arrival of the first packet of a burst and the reception of the last packet of the burst [12].

For full buffer traffic, user experienced data rate is calculated as [12]:

user experienced data rate = 5% user spectrum efficiency × bandwidth

Clearly, system-level simulations are needed to compute the user experienced data. This is well beyond the scope of the RAN SI. In general, the user experienced data rate of an Ambient IoT device would depend on a wide range of factors such as physical layer design (coding, modulation, etc), channel bandwidth, propagation conditions (indoor or outdoor, LOS or NLOS, etc.), link direction (UL or DL), overhead (sync, pilots, etc.), proximity to the emitter (for Device A/B), tag transmit power (for Device C), and so on.

The peak date rate is a more commonly used (and simpler) metric than user experienced data rate. Peak data rate is the highest theoretical data rate assuming error-free conditions, when all assignable radio resources for the corresponding link direction are utilised (i.e., excluding radio resources that are used for physical layer synchronisation, reference signals or pilots, guard bands and guard times) [12]. The peak data rate also depends on physical layer design and channel bandwidth but ignores other factors listed in the paragraph above. Since user experienced data rate depends on the user throughput, which is the number of correctly received bits, the peak rate needs to be higher to achieve a user experienced rate target of 5 kbps.

For Device A/B, the values reported in [13] for various state-of-the-art backscatter communication systems indicate that the peak rate of more than 5 kbps can be achieved. However, such high peak rate would require more complex Tx/Rx components and faster processing capability at the device which further increases the power consumption. This is also evident from [13] which shows that the power consumption is in general higher (> 10 µW) for higher peak rates. Therefore, whether it is feasible to achieve the data rate target of 5 kbps under the power consumption constraint for Device A/B is unclear and needs further study.

For Device C, assuming a channel bandwidth of one or a few PRBs and transmit power of 0 dBm, a peak rate of more than 5 kbps should be possible to achieve under the power consumption target of 10 mW.

Based on literature survey, a data rate of 5 kbps seems feasible for Device A/B. However, further study is needed to assess whether it is feasible to achieve this data rate target under the power consumption constraint for Device A/B.

For Device C, a peak rate of more than 5 kbps should be possible to achieve under the power consumption target of 10 mW.

## 4.5	Positioning accuracy

Localization and inventory (both of which requires positioning information) are key Ambient IoT use cases.

For Device A, there needs to be a charge carrier emitter in the vicinity to illuminate the device. It is shown in Table 1 that Device A needs to be within 5 m from the emitter. Therefore, the position of the device can be implicit from the coverage area of the emitter that has activated the device. The disadvantage is that it requires dedicated, dense, and costly deployment of emitters. More advanced positioning methods that requires heavy processing and a lot of power consumption may not be feasible for passive devices.

Device B also suffers from similar challenges as Device A as described above.

For Device C, on top of simple cell ID positioning methods, advanced positioning methods, including those already specified in 3GPP, should also in principle be possible. However, the low channel bandwidth and low transmit power (e.g., max 0 dBm) could impact the positioning accuracy. The different connectivity topologies (Topology 1, 2, 3, or 4) might also impact the accuracy of the different methods.

The above aspects need to be considered while assessing the feasibility of positioning accuracy targets in Section 4.5. Due to the limited time left to conclude the RAN SI, we think it is enough to capture in TR 38.848 the positioning accuracy target (clarifying that the target reflects SA1 requirement).

For Devices A and B, the position of the device can be implicit from the coverage area of the emitter, but this requires dedicated, dense, and costly deployment of emitters to locate the device.

For Device C, it may be possible to reuse existing 3GPP positioning methods.

For topologies not relying on a fixed deployment, for example, Topology 2, 3, or 4 with mobile UE as the intermediate node, assisting node or reader, positioning is also required for the intermediate node.

For topologies not relying on a fixed deployment, for example, Topology 2, 3, or 4 with mobile UE as the intermediate node, assisting node or reader, positioning may also be required for the mobile UE, in addition to the Ambient IoT device.

# 5	Conclusions and Recommendations

In this section, we provide our view on conclusions and recommendations to be captured in TR 38.848 Clause 7.

## 5.1	Active vs. passive devices

The most important aspect of the RAN SI is to identify if/how Ambient IoT is suitable for deployment in a 3GPP system. We elaborate on this aspect below by differentiating between passive devices (i.e., Devices A and B) and active devices (i.e., Device C).

- Coverage: The SID mandates that Ambient IoT “shall aim to provide better coverage compared to existing non-3GPP technologies for the relevant use cases” [1].  The link budget evaluation in Section 5.3 shows that a passive device must be within 5 m from an emitter to meet the potential indoor coverage target of, e.g., 50 m. This makes solutions based on passive devices effectively short-range solutions. That is, their coverage is not much better than existing non-3GPP technologies. The active devices, on the other hand, provides much better coverage than non-3GPP short-range technologies.

- Deployment cost: Dense deployments of emitters and/or readers are needed to ensure seamless coverage with passive devices. While the devices itself are cheap, the emitters and readers (e.g., BS) are often expensive. For reference, a typical UHF RFID reader costs more than US$500 [13].  A 3GPP RFID alternative can be expected to cost even more. This is prohibitively high CAPEX for many use cases. With active devices, however, there is possibility to reuse existing network deployments. That is, with a reuse of existing network hardware and existing sites, a relatively wide-area coverage can be supported at a low overall cost compared to passive devices.

- Non-3GPP alternatives: The short-range non-3GPP competitive solutions today typically operate in unlicensed bands. Furthermore, a very similar IEEE study entitled ‘AMP IoT’ was finalized in March and address the passive devices in unlicensed bands. Due to their operation in unlicensed bands, the non-3GPP short-range solutions have the benefit of lower OPEX for the customers. It is unclear why customers would adopt a 3GPP short-range solution providing little to no additional benefit. The competitive advantage of a 3GPP solution is the longer range and the possibility of more ubiquitous coverage stemming from licensed operation. This is possible only with active devices.

- Market prospects: As highlighted in Section 2.1, the passive devices are not suitable for deployment in outdoor scenarios. Passive devices are also not feasible for use cases with autonomously device originated (DO-DOA) traffic. This precludes the majority of the representative use cases listed in TR 38.848 Table 4.1-1. Considering also that there are non-3GPP backscattering solutions addressing similar use cases as that of a 3GPP passive solution, they are expected to have much smaller market compared to active solution.

The passive solution (corresponding to Devices A and B) is a short-range solution, and the benefit of a 3GPP solution for short-range is unclear compared to non-3GPP alternatives.

The active solution (corresponding to Device C) is expected to provide much better coverage than non-3GPP short-range solutions.

The IEEE ‘AMP IoT’ study, which is very similar to 3GPP ‘Ambient IoT’ and targets a solution in unlicensed bands including passive devices, was finalized in March.

3GPP’s competitive advantage is longer-range solutions with ubiquitous coverage within licensed frequency bands, providing indoor and/or outdoor coverage by reusing existing NW infrastructure.

Passive solution is expected to have higher overall deployment cost and smaller market compared to active solution.

Taking the above factors into consideration, we propose the following:

Make recommendation in TR 38.848 that potential RAN1-led study item in Rel-19 should focus on active solution (corresponding to Device C) and that the passive solutions (corresponding to Devices A and B) are not considered in the study item.

Make recommendation in TR 38.848 that potential RAN1-led study item in Rel-19 should prioritize operation in licensed band.

## 5.2	On connectivity topologies and deployment scenarios

As part of the RAN SI, the following connectivity topologies have been defined:

- Topology 1: BS ↔ Ambient IoT device

- Topology 2: BS ↔ intermediate node ↔ Ambient IoT device

- Topology 3: BS ↔ assisting node ↔ Ambient IoT device ↔ BS

- Topology 4: UE ↔ Ambient IoT device

Furthermore, the following deployment scenarios have also been defined:

- Deployment scenario 1: Device indoors, BS indoors

- Deployment scenario 2: Device indoors, BS outdoor

- Deployment scenario 3: Device indoors, UE-based reader

- Deployment scenario 4: Device outdoors, BS outdoors

- Deployment scenario 5: Device outdoors, UE-based reader

One of the limitations of RFID as captured in the SID is as follows [1]:

However, the limited reading range of a few meters usually requires handheld scanning which leads to labor intensive and time-consuming operations, or RFID portals/gates which leads to costly deployments.

The BS-free Ambient IoT deployment corresponding to Topology 4 and associated Deployment scenarios 3 and 5 is expected to have a similar limitation. That is, using mobile UE as an RFID-reader to read tags as in the BS-free deployment would require a mobile UE to be in the vicinity (i.e., within 5 m) of an Ambient IoT device. This is very restrictive and in practice nothing would be gained in terms of manual labour or time consumption if an employee must walk around with a UE.  Also, the BS-free deployment is not expected to guarantee service availability (e.g., for industrial applications) or provide any monetary value to communication service providers. Therefore, Topology 4 should not be considered in a potential RAN1-led study item in Rel-19.

Furthermore, to keep the workload to a manageable level in the WGs, we think the Topology 1 where there is direct connectivity with the BS should be prioritized over Topologies 2 and 3 where there is an intermediate/assisting node between the BS and the Ambient IoT device.

If Ambient IoT is to be a RAN1-led study item in Rel-19, Topologies 2, 3, and 4 (and associated Deployment scenarios 3 and 5) is not considered for further study in the RAN1-led study item in Rel-19.

## 5.3	CN connection

Several of the SA1 use cases mention functionality with CN impact: authentication, initial registration, security, etc. CN functions allow for registration, maintenance, charging/billing of devices, security, downlink reachability, roaming, etc. This is believed to be an advantage compared to non-3GPP solutions. The Ambient-IoT SID [1] contains the following statement regarding CN impact:

NOTE: The study shall not prioritize deployment aspects that should be coordinated with SA, e.g., public or private network, with or without CN connection.

In addition, a proposed SI on Ambient IoT has now been endorsed by SA2 and is pending approval in SA plenary [17]. According to the objectives, this SI suggests studying appropriate CN functionality for Ambient -IoT, potentially with simplifications and reductions, but still with some connection to CN. For RAN1 and RAN4 work is largely agnostic of the CN impact, but for RAN2 and RAN3 considering “with or without CN connection” would effectively double the work since both tracks must be investigated. Therefore, we propose the following:

If Ambient IoT is to be a RAN1-led study item in Rel-19, only solutions with connectivity to CN is studied.

## 5.4	Coexistence with existing network infrastructure

The coexistence with existing network infrastructure is key to successful deployment of any new 3GPP IoT technology. In fact, the SID has identified coexistence with network infrastructure as one of the aspects to consider for Ambient IoT.

- Identify the suitable deployment scenarios and their characteristics, at least for the use cases/services agreed in SA1’s “Study on Ambient power-enabled internet of Things”, comprising among at least the following aspects - […] - Coexistence with UEs and infrastructure in frequency bands for existing 3GPP technologies

Devices A and Device B which rely on backscattering transmission pose significantly more challenges to the existing infrastructure from coexistence perspective. Dedicated RF power transfer from CWTs, e.g., using a high-power continuous wave, or multiplexing of data and power transfer could cause severe inter-band/cell interference. If power boosting would be needed to boost the signal to an Ambient IoT device, it could lead to out-of-band emissions that may be difficult to mitigate. This is especially problematic if boosting is done near the band edge. Consequently, it might not be possible to deploy Ambient IoT with existing BS hardware, for e.g., new filters may be needed in the BS. That is, it might not be possible to deploy Ambient IoT with software upgrade of existing sites, hindering the ease of network roll-out of Ambient IoT.

Another aspect that might impact whether backscattering devices can be deployed in existing infrastructure is their capability. For example, depending on whether the device has frequency shifting capability or not (and to what extend), the BS (reader) may have to deal with different levels of interference from the carrier wave emitter.

With Device C, it may be possible to reuse existing network deployments, i.e., reuse existing network hardware and existing sites, and support Ambient IoT globally with a relatively wide-area coverage. Reuse of existing network hardware also makes it possible to deploy Ambient IoT with software upgrade of existing sites, and thus reducing time-to-market.

If Ambient IoT is to be a RAN1-led study item in Rel-19, ensure coexistence with existing network infrastructure as well as, where applicable, reuse of existing network deployments.

# 6	Conclusions

In the previous sections we made the following observations:

Observation 1	There is no need to agree on specific E1 and E2 values during the RAN SI. It is enough to agree on the design targets for power consumption, data rate, and message size.

Observation 2	Further investigation is needed on how to translate end-to-end latency requirements as in TR 22.840 to RAN latency targets, such as control plane latency, user plane latency, or latency for infrequent small packets.

Observation 3	For devices that rely on energy harvesting, it is unclear whether/how to consider the energy harvesting duration when defining the latency targets.

Observation 4	Based on the initial analysis, the following device design target for power consumption during transmitting/receiving is feasible:

Observation 5	Further study in RAN1 is needed to assess the feasibility of the following device complexity design targets:

Observation 6	For Devices A and Device B, there is strong dependency between emitter-to-tag and the tag-to-reader distance. The larger the emitter-to-tag distance for these devices, the smaller the tag-to-reader distance.

Observation 7	Devices A and B require continuous illumination using RF signals so as to forward the data to the reader.

Observation 8	In general, the emitter-to-tag distance limits the overall coverage for Devices A and B to very short-short range. Therefore, Devices A and B are not suitable for outdoor deployment (i.e., Deployment scenarios 2 and 4 with Topology 1).

Observation 9	For Devices A and Device B (without reflection amplifier), the indoor device coverage target of 50 m is feasible but only when the tag is within 5 m from the emitter. For Device B (with reflection amplifier), the target is feasible even when the tag is at few tens of meters from the emitter.

Observation 10	For Device C, the indoor as well as outdoor device coverage targets are easily feasible even with much lower transmit power values compared to NB-IoT.

Observation 11	Based on literature survey, a data rate of 5 kbps seems feasible for Device A/B. However, further study is needed to assess whether it is feasible to achieve this data rate target under the power consumption constraint for Device A/B.

Observation 12	For Device C, a peak rate of more than 5 kbps should be possible to achieve under the power consumption target of 10 mW.

Observation 13	For Devices A and B, the position of the device can be implicit from the coverage area of the emitter, but this requires dedicated, dense, and costly deployment of emitters to locate the device.

Observation 14	For Device C, it may be possible to reuse existing 3GPP positioning methods.

Observation 15	For topologies not relying on a fixed deployment, for example, Topology 2, 3, or 4 with mobile UE as the intermediate node, assisting node or reader, positioning may also be required for the mobile UE, in addition to the Ambient IoT device.

Observation 16	The passive solution (corresponding to Devices A and B) is a short-range solution, and the benefit of a 3GPP solution for short-range is unclear compared to non-3GPP alternatives.

Observation 17	The active solution (corresponding to Device C) is expected to provide much better coverage than non-3GPP short-range solutions.

Observation 18	The IEEE ‘AMP IoT’ study, which is very similar to 3GPP ‘Ambient IoT’ and targets a solution in unlicensed bands including passive devices, was finalized in March.

Observation 19	3GPP’s competitive advantage is longer-range solutions with ubiquitous coverage within licensed frequency bands, providing indoor and/or outdoor coverage by reusing existing NW infrastructure.

Observation 20	Passive solution is expected to have higher overall deployment cost and smaller market compared to active solution.

Based on the discussion in the previous sections we propose the following:

Proposal 1	TR 38.848 captures energy storage levels as follows:

Proposal 2	UL-only devices, which cannot receive data/control signalling from the BS or an intermediate node, are not considered as part of the RAN SI.

Proposal 3	Adopt the TP in Section 3.1 to TR 38.848.

Proposal 4	Confirm the confirm the working assumption on device complexity by removing “Editor’s note: In RAN#99, the following is a working assumption” from Clause 5.2.

- Coverage design target is:

- Indoor device (for Device A & B & C): 50 m

- Outdoor device (for Device C): 500 m

- Note 1: For Topology 1 & 2 & 3, the coverage reference points are the BS and the Tag. For Topology 4, the coverage reference points are the UE and the Tag.

- Note 2: For Device A & B, the emitter-to-tag distance should be reported as part of the assessment.

- WGs can refine the target based on band assumptions, relevant channel model(s), ISD(s), etc., as needed.

Positioning accuracy design target is:

- Indoor: 3 m @ 90%

- Outdoor: several 10 m @ 90%

Proposal 7	Capture the maximum and minimum end-to-end latency according to TR 22.840 in TR 38.848 and indicate that WGs can introduce corresponding RAN latency targets, e.g., considering the energy harvesting duration.

Maximum message size target is:

- Hundreds of bits, and < 1000 bits

- RAN1 to refine as needed e.g., for TB size design.

Proposal 9	If Ambient IoT is to be a RAN1-led SI, detailed power consumption evaluation is conducted in RAN1. RAN1 can refine the RAN power consumption target, if needed.

Proposal 10	Make recommendation in TR 38.848 that potential RAN1-led study item in Rel-19 should focus on active solution (corresponding to Device C) and that the passive solutions (corresponding to Devices A and B) are not considered in the study item.

Proposal 11	Make recommendation in TR 38.848 that potential RAN1-led study item in Rel-19 should prioritize operation in licensed band.

Proposal 12	If Ambient IoT is to be a RAN1-led study item in Rel-19, Topologies 2, 3, and 4 (and associated Deployment scenarios 3 and 5) is not considered for further study in the RAN1-led study item in Rel-19.

Proposal 13	If Ambient IoT is to be a RAN1-led study item in Rel-19, only solutions with connectivity to CN is studied.

Proposal 14	If Ambient IoT is to be a RAN1-led study item in Rel-19, ensure coexistence with existing network infrastructure as well as, where applicable, reuse of existing network deployments.

# References

RP-223396, “SID revised: Study on Ambient IoT”, Huawei, HiSilicon, December 2022.

Meeting report, RAN#99, March 2023.

TR 38.848 V0.2.0, “Study on Ambient IoT (Internet of Things) in RAN”, June 2023.

TR 22.840 V1.2.0, “Study on Ambient power-enabled Internet of Things”, June 2023.

RP-230801, “Results of offline discussions on ambient IoT”, Huawei (Rapporteur), March 2023.

RP-222644, “Revised SID: Study on low-power Wake-up Signal and Receiver for NR”, Vivo, Sept. 2022.

F. Amato et. al., “Tunneling RFID Tags for Long-Range and Low-Power Microwave Applications”, IEEE Journal of Radio Frequency Identification, vol. 2, no. 2, pp. 93-103, June 2018.

TR 36.888 V12.0.0, “Study on provision of low-cost Machine-Type Communications (MTC) User Equipments (UEs) based on LTE”, June 2013.

TR 38.875 V17.0.0, “Study on support of reduced capability NR devices”, March 2021.

TR 38.901 V17.0.0, “Study on channel model for frequencies from 0.5 to 100 GHz”, March 2022.

J. D. Griffin and G. D. Durgin, "Complete Link Budgets for Backscatter-Radio and RFID Systems" in IEEE Antennas and Propagation Magazine, vol. 51, no. 2, pp. 11-25, April 2009.

TR 38.913 V17.0.0, “Study on scenarios and requirements for next generation access technologies”, April 2022.

C. Xu et. al, "Practical Backscatter Communication Systems for Battery-Free Internet of Things: A Tutorial and Survey of Recent Research," IEEE Signal Processing Magazine, vol. 35, no. 5, pp. 16-27, Sept. 2018.

M. Lauridsen et. al., “An Empirical NB-IoT Power Consumption Model for Battery Lifetime Estimation”, IEEE Vehicular Technology Conference (VTC Spring), Porto, Portugal, 2018.

D. A. Loku Galappaththige, et. al., "Link Budget Analysis for Backscatter-Based Passive IoT”, IEEE Access, vol. 10, pp. 128890-128922, 2022.

S2-2310033, “New SID: Study on Architecture support enhancement of Ambient power-enabled Internet of Things”, OPPO, SA2#158, August 2023.

# Appendix

Assumptions used for generating results in Table 1 to Table 3 is shown in the table below. The methodology used for link budget analysis is based on [11].

| Parameters | For 0.7 GHz band | For 2.6 GHz band |
| --- | --- | --- |
| Transmit power from the emitter | 30 dBm | 26 dBm |
| Antenna gains at the emitter, tag, and reader | 13, -3, 13 dBi | 22, -3, 22 dBi |
| Total loss (feeder loss, cable loss, body loss, circuit loss, etc.) at the emitter, tag and reader | 3, 0, 3 dB | 3, 0, 3 dB |
| Fade margin for the forward (DL) and the backscattering (UL) links | 10, 5 (bistatic) for k=3 dB | 10, 8 (bistatic) for k = 3 dB |
| Polarization mismatch for the forward (DL) and the backscattering (UL) links | -3, -3 dB | -3, -3 dB |
| On object penalty, power transmission coefficient, modulation factor | 0.8, 0, -6 dB for cardboard | 0.8, 0, -6 dB for carboard |
| Interference from emitter to reader | 0 | 0 |
| Rx sensitivity at the reader | -92 dBm | -92 dBm |
| Harvester sensitivity (Device A) | -20 dBm [15] | -20 dBm [15] |
| Reflection amplifier gain (Device B) | 20 dB | 20 dB |

Assumptions used for generating results in Table 4 is shown below.

| Parameters | For 0.7 GHz band | For 2.6 GHz band |
| --- | --- | --- |
| Transmit power from the base station (DL) | 30 dBm | 26 dBm |
| Antenna gains at the base station and the device | 13, -3 dBi | 22, -3 dBi |
| Total loss (feeder loss, cable loss, body loss, circuit loss, etc.) at the base station and the device | 3, 0 dB | 3, 0 dB |
| Fade margin for both DL and UL | 10 for k = 3 dB | 10 for k = 3 dB |
| Polarization mismatch for both DL and UL | -3 dB | -3 dB |
| On object penalty | 0.8 for cardboard | 0.8 for carboard |
| Rx sensitivity at the base station | -92 dBm | -92 dBm |
| Transmit power from the device (UL) | 0, -10 dBm | 0, -10 dBm |
| Rx sensitivity at the device | -92 dBm | -92 dBm |

For Device C, it is assumed that the energy available at the device is greater than the energy consumed during UL transmission, and thus, DL and UL coverage calculations are decoupled. It is also assumed that the available energy at the device is sufficient to enable the DL receiver.