3GPP TSG RAN Meeting #101	RP-231630

Bangalore, India, September 11-15, 2023

Agenda item:	8A.2.12.5

Source:	Nokia, Nokia Shanghai Bell

Title:	Rel-19 Sidelink Evolution

Document for:	Discussion

# 1	Introduction

At the Rel-19 RAN Workshop in June, potential topics in Rel-19 as the second release of 5G-Advanced in 3GPP were discussed. NR SL Evolution is considered as one of the additional RAN1-led candidate topics [1].

According to chair’s summary of Rel-19 Workshop in [1], Rel-19 work will primarily focus on investing in 5G-Advanced commercial deployments to further improve performance and address critical needs, while it should also serve as a bridge to 6G. In this paper, we provide our view on objectives for Rel. 19 work in NR SL evolution serving the purpose of enabling commercial deployments.

# 2	Discussion

## 2.1	Motivation

The main motivation for 3GPP work on sidelink evolution in Rel. 18 was to expand the applicability of NR sidelink to commercial use cases.  For commercial sidelink applications, two key requirements were identified, namely increased data rate and support of new carrier frequencies for sidelink.

Increased data rate can be achieved with the support of sidelink carrier aggregation and sidelink over unlicensed spectrum. SL operation in FR1 unlicensed band is introduced in 3GPP Release 18. Sidelink carrier aggregation, on the other hand, was reduced to a very limited scope in order to make some Release 18 specifications possible within the reduced time where SL CA was treated. However, with the reduced scope as e.g. mode 2 resource allocation only, no handling of UE’s limited capability, same SCS, aligned PSFCH, and support of LTE features only, Release 18 sidelink CA will not be able to specify CA which effectively addresses the essential problem of enabling much higher data rates.

Release 19 sidelink evolution should focus on completion of Release 18 work to reach the targeted applicability of NR sidelink in commercial use cases.

## 2.2	Potential objectives

The proposed objective of the Work Item are as follows:

- Specify mechanism to enhance support of NR sidelink CA operation including

- Consider possible optimisation beyond LTE SL CA features for NR CA, e.g. support of different SCS among component carriers.

- The work is limited to FR1 licensed and unlicensed.

# 3	Conclusions

In this contribution, we discussed the motivation for necessary enhancement of NR sidelink CA in Rel-19 and provided potential objective.  The following is proposed:

Proposal:	Agree to include a WID on enhancement of NR SL carrier aggregation in Rel-19.

# References

- RWS-230488, Chair’s summary of Rel-19 Workshop, 3GPP RAN Release 18 WS