3GPP TSG-RAN5 Meeting #100	R5-234659

# Toulouse, France, 21st Aug 2023 – 25th Aug 2023

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.533 | CR | 2558 | rev | - | Current version: | 17.7.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Update to NR-U annexes E and H |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Qualcomm India Pvt Ltd |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_unlic-UEConTest |  |  |  |  |  | Date: |  |  | 2023-08-21 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | Several aspects are still marked as FFS for NR-U test cases. To complete some of them, annexes E and H need to be updated. |  |  |  |  |  |  |  |  |
| Summary of change: |  | - Added new section E.8 for EN-DC NR-U cell configuration. - Added new Table H.3.10 with ServingCellConfigCommon configuration for shared spectrum, both dynamic and semi-static channel access, as well as SMTC configuration. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Test case configuration will be incomplete. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | E.8, H.3.1, H.3.10 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  |  | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  |  | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  |  | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

<< Start of Changes >>

# E.8	Cell configuration mapping for EN-DC FR1 Shared Spectrum test cases in Chapter 10

Table E.8-1 defines the cell configuration mapping for EN-DC FR1 Shared Spectrum test cases in chapter 10 of this test specification.

Table E.8-1: Cell configuration mapping for EN-DC FR1 Shared Spectrum RRM testing

| TC | Description | 38.533 LTE Cell1 | 38.533 NR Cell2 | 38.533 NR Cell3 | 38.533 NR Cell4 | CA Type |
| --- | --- | --- | --- | --- | --- | --- |
| 10.3.1.2 |  | EN-DC FR1 Radio link monitoring out-of-sync test for PSCell configured with SSB-based RLM RS in non-DRX mode under CCA |  | LTE Cell 1 |  | NR Cell 1 |  |  |  |  |  |  |  |
| 10.3.1.3 |  | EN-DC FR1 Radio link monitoring in-sync test for PSCell configured with SSB-based RLM RS in non-DRX mode under CCA |  | LTE Cell 1 |  | NR Cell 1 |  |  |  |  |  |  |  |
| 10.3.2.1 |  | EN-DC FR1 E-UTRAN – NR interruptions during SCell operations with CCA |  | LTE Cell 1 |  | NR Cell 1 |  | NR Cell 6 |  |  |  | Intra-band |  |
|  |  |  |  |  |  |  |  | NR Cell 10 |  |  |  | Inter-band |  |
| 10.3.3.1 |  | EN-DC FR1 SCell Activation and Deactivation of known NR SCell with NR PSCell and NR SCell under CCA, 160 ms SCell measurement cycle |  | LTE Cell 1 |  | NR Cell 1 |  | NR Cell 6 |  |  |  | Intra-band |  |
|  |  |  |  |  |  |  |  | NR Cell 10 |  |  |  | Inter-band |  |
| 10.3.3.2 |  | EN-DC FR1 SCell Activation and Deactivation of known NR SCell with NR PSCell and NR SCell under CCA, 640 ms SCell measurement cycle |  | LTE Cell 1 |  | NR Cell 1 |  | NR Cell 6 |  |  |  | Intra-band |  |
|  |  |  |  |  |  |  |  | NR Cell 10 |  |  |  | Inter-band |  |
| 10.3.3.3 |  | EN-DC FR1 SCell Activation and Deactivation of unknown NR SCell with NR PSCell and NR SCell under CCA |  | LTE Cell 1 |  | NR Cell 1 |  | NR Cell 6 |  |  |  | Intra-band |  |
|  |  |  |  |  |  |  |  | NR Cell 10 |  |  |  | Inter-band |  |
| 10.3.4.1 |  | EN-DC FR1 Beam Failure Detection and Link Recovery Test for PSCell configured with SSB-based BFD and LR in non-DRX mode |  | LTE Cell 1 |  | NR Cell 1 |  |  |  |  |  |  |  |
| 10.3.4.2 |  | EN-DC FR1 Beam Failure Detection and Link Recovery Test for PSCell configured with SSB-based BFD and LR in DRX mode |  | LTE Cell 1 |  | NR Cell 1 |  |  |  |  |  |  |  |
| 10.4.1.1 |  | EN-DC FR1 Event-triggered reporting tests on PSCC without gaps under non-DRX and CCA |  | LTE Cell 1 |  | NR Cell 1 |  | NR Cell 2 |  |  |  |  |  |
| 10.4.1.2 |  | EN-DC FR1 Event-triggered reporting tests on PSCC without gaps under DRX and CCA |  | LTE Cell 1 |  | NR Cell 1 |  | NR Cell 2 |  |  |  |  |  |
| 10.4.1.3 |  | EN-DC FR1 Event-triggered reporting tests on PSCC with per-UE gaps under non-DRX and CCA |  | LTE Cell 1 |  | NR Cell 1 |  | NR Cell 2 |  |  |  |  |  |
| 10.4.1.4 |  | EN-DC FR1 Event-triggered reporting tests on PSCC with per-UE gaps under DRX and CCA |  | LTE Cell 1 |  | NR Cell 1 |  | NR Cell 2 |  |  |  |  |  |
| 10.5.1.1 | EN-DC FR1 intra-frequency SS-RSRP measurement accuracy on a CCA serving cell | LTE Cell 1 | NR Cell 1 | NR Cell 2 |  |  |
| 10.5.1.2 | EN-DC FR1-FR1 inter-frequency SS-RSRP measurement accuracy with CCA serving cell and CCA target cell | LTE Cell 1 | NR Cell 6 | NR Cell 3 |  |  |
| 10.5.2.1 | EN-DC FR1 intra-frequency SS-RSRQ measurement accuracy with serving cell and target cell under CCA | LTE Cell 1 | NR Cell 1 | NR Cell 2 |  |  |
| 10.5.2.2 | EN-DC FR1-FR1 inter-frequency SS-RSRQ measurement accuracy with serving cell and target cell under CCA | LTE Cell 1 | NR Cell 6 | NR Cell 3 |  |  |
| 10.5.3.1 | EN-DC FR1 intra-frequency SS-SINR measurement accuracy on PSCC under CCA | LTE Cell 1 | NR Cell 1 | NR Cell 2 |  |  |
| 10.5.3.2 | EN-DC FR1-FR1 inter-frequency SS-SINR measurement accuracy on PSCC under CCA | LTE Cell 1 | NR Cell 6 | NR Cell 3 |  |  |
| 10.5.3.3 | EN-DC FR1 intra-frequency SS-SINR measurement accuracy on SCC under CCA | LTE Cell 1 | NR Cell 1 | NR Cell 2 |  |  |

# E.9 to E.13

<< Skipped sections >>

## H.3.1	RRC messages and information elements contents exceptions for NR measurement configuration

<< Skipped tables >>

MeasObjectNR-DEFAULT

NR measurement object configuration.

Table H.3.1-3: MeasObjectNR-DEFAULT: NR intra-frequency measurement object configuration

| Derivation Path: TS 38.508-1 [14], Table 4.6.3-76 |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |  |
| MeasObjectNR::= SEQUENCE { |  |  |  |  |
| ssbFrequency | ARFCN-ValueNR of the SSB associated to serving cell | frequency of the serving cell | INTRA-FREQ MO |  |
|  | ARFCN-ValueNR of the SSB associated to SCell |  | Deactivated SCell |  |
|  | ARFCN-ValueNR of the SSB associated to inter-frequency neighbour cell |  | INTER-FREQ MO |  |
| referenceSignalConfig SEQUENCE { |  |  |  |  |
| ssb-ConfigMobility SEQUENCE { |  |  |  |  |
| ssb-ToMeasure | Not present |  |  |  |
|  | SSB-ToMeasure specified in 38.508-1 [14] Table 7.3.1-31 with condition SSB.7 FR2 OR SSB.8 FR2 |  | SSB.7 FR2 OR SSB.8 FR2 |  |
| deriveSSB-IndexFromCell | false |  |  |  |
|  | true |  | Synchronous cells OR RLM |  |
| ssb-PositionQCL-Common-r16 | n1 |  |  |  |  | SSB.1 CCA SSB.2 CCA |  |
|  |  | n2 |  |  |  | SSB.3 CCA SSB.4 CCA |  |
| } |  |  |  |  |
| } |  |  |  |  |
| absThreshSS-BlocksConsolidation SEQUENCE { |  |  |  |  |
| thresholdRSRP | 0 | SS-RSRP < -156dB | NOT SS-SINR |  |
| thresholdSINR | 0 | SS-SINR<-23 | SS-SINR |  |
| } |  |  |  |  |
| measCycleSCell-v1530 | sf640 |  | Deactivated SCell |  |
| rmtc-Config-r16 | FFS |  | RSSI OR Channel Occupancy |  |
| associatedMeasGapSSB-r17 | Not present |  |  |
|  | MeasGapId-r17 |  | MGe |
| } |  |  |  |  |

| Condition | Explanation |
| --- | --- |
| INTRA-FREQ MO | Configuration for NR MO associated to intra-frequency carrier |
| INTER-FREQ MO | Configuration for NR MO associated to inter-frequency carrier |
| Synchronous cells | SSB indices of neighbourcells can be derived from timing of serving cell |
| RLM | Configuration for RLM tests |
| Deactivated SCell | Configuration for SCell activation and deactivation tests |
| SS-SINR | Configuration for SS-SINR tests |
| MGe | Indicates measurement gap enhancements configuration |  |
| SSB.X CCA | SSB pattern SSB.X CCA configured as per section A.3A (X = {1,2,3,4}) |  |
| RSSI | Configuration associated to RSSI measurements |  |
| Channel Occupancy | Configuration associated to Channel Occupancy measurements |  |

<< Skipped tables >>

ReportConfigNR-DEFAULT

NR Report Configuration

Table H.3.1-4: ReportConfigNR-DEFAULT(a3-offset): NR report configuration for event A3

| Derivation Path: TS 38.508-1 [14], Table 4.6.3-142 with condition EVENT_A3 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| ReportConfigNR::= SEQUENCE { |  |  |  |
| reportType CHOICE { |  |  |  |
| eventTriggered SEQUENCE { |  |  |  |
| eventId CHOICE { |  |  |  |
| eventA3 SEQUENCE { |  |  |  |
| a3-Offset CHOICE { |  |  |  |
| rsrp | A3-offset*2 | The actual value is field value * 0.5 dB |  |
| rsrq | A3-offset*2 | The actual value is field value * 0.5 dB | RSRQ |
| sinr | A3-offset*2 | The actual value is field value * 0.5 dB | SINR |
| } |  |  |  |
| hysteresis | 0 | 0 dB |  |
| timeToTrigger | ms0 |  |  |
| } |  |  |  |
| } |  |  |  |
| reportQuantity-RsIndexes SEQUENCE { |  |  | SSB Index |
| rsrp | true |  |  |
| rsrq | false |  |  |
|  | true |  | RSRQ |
| sinr | false |  |  |
|  | true |  | SINR |
| } |  |  |  |
| maxReportCells | 2 |  |  |
| maxNrofRS-IndexesToReport | 2 |  | SSB Index |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

| Condition | Explanation |
| --- | --- |
| SSB Index | To include SSB Index |
| RSRQ | RSRQ trigger quantity |
| SINR | SINR trigger quantity |

<< Skipped tables >>

MeasResults-DEFAULT

measurement result for NR measurements.

Table H.3.1-7: MeasResults: measurement result for NR measurements

| Derivation Path: TS 38.508-1 [14], Table 4.6.3-79 with condition A3 |  |  |  |
| --- | --- | --- | --- |
| Information Element |  | Value/remark | Comment | Condition |
| measResults SEQUENCE { |  |  |  |  |
| measId |  | MeasId |  |  |
| measResultServingMOList SEQUENCE (SIZE (1..maxNrofServingCells)) OF SEQUENCE { |  | 2 entries |  |  |
| servCellId[1] |  | ServCellIndex of NR SpCell |  |  |
| measResultServingCell[1] SEQUENCE { |  |  |  |  |
| physCellId |  | PhysCellId of NR SpCell |  |  |
| measResult SEQUENCE { |  |  |  |  |
| cellResults SEQUENCE { |  |  |  |  |
| resultsSSB-Cell SEQUENCE { |  |  |  |  |
| rsrp |  | (0..127) |  |  |
| rsrq |  | (0..127) |  |  |
| sinr |  | (0..127) |  | SINR |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| servCellId[2] |  | ServCellIndex of NR SCell |  | Deactivated SCell |
| measResultServingCell[2] SEQUENCE { |  |  |  | Deactivated SCell |
| physCellId |  | PhysCellId of NR SCell |  |  |
| measResult SEQUENCE { |  |  |  |  |
| cellResults SEQUENCE { |  |  |  |  |
| resultsSSB-Cell SEQUENCE { |  |  |  |  |
| rsrp |  | (0..127) |  |  |
| rsrq |  | (0..127) |  |  |
| sinr |  | (0..127) |  | SINR |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| measResultNeighCells CHOICE { |  |  |  |  |
| measResultListNR SEQUENCE (SIZE(1..maxCellReport)) OF SEQUENCE { |  | 1 entry |  | INTRA-FREQ OR INTER-FREQ |
| physCellId[1] |  | PhysCellId of NR neighbour Cell |  |  |
| measResult[1] SEQUENCE { |  |  |  |  |
| cellResults SEQUENCE { |  |  |  |  |
| resultsSSB-Cell SEQUENCE { |  |  |  |  |
| rsrp |  | (0..127) |  |  |
| rsrq |  | (0..127) |  | RSRQ |
| sinr |  | (0..127) |  | SINR |
| } |  |  |  |  |
| } |  |  |  |  |
| rsIndexResults SEQUENCE { |  | n entires of ResultsPerSSB-Index | ResultsPerSSB-IndexList | SSB Index |
| resultsPerSSB-Index SEQUENCE { |  | entry [1] |  |  |
| ssb-Index |  | SSB-Index | an SS-Block within an SS-Burst |  |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| measResultListEUTRA SEQUENCE (SIZE (1..maxCellReport)) OF SEQUENCE { |  | 1 entry |  | INTER-RAT |
| eutra-PhysCellId [1] |  | PhysCellId of E-UTRA neighbour Cell |  |  |
| measResult[1] SEQUENCE { |  |  |  |  |
| rsrp |  | (0..97) |  |  |
| rsrq |  | (0..34) |  |  |
| } |  |  |  |  |
| cgi-Info |  | Not present |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |

| Condition | Explanation |
| --- | --- |
| SSB Index | To include SSB Index |
| INTRA-FREQ | Configuration for intra-frequency NR measurement tests |
| INTER-FREQ | Configuration for inter-frequency NR measurement tests |
| INTER-RAT | Configuration for inter-RAT EUTRA measurement tests |
| Deactivated SCell | Configuration for measurement on Deactivated SCell tests |
| RSRQ | RSRQ measurement reporting |
| SINR | SINR measurement reporting |

<< Skipped sections >>

## H.3.10	RRC messages and information elements contents exceptions for Shared Spectrum Access

Table H.3.10-1: ServingCellConfigCommon for Shared Spectrum Access

| Derivation Path: TS 38.508-1, Table 4.6.3-168 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| ServingCellConfigCommon ::= SEQUENCE { |  |  |  |
| ssb-PositionsInBurst CHOICE { |  |  |  |
| mediumBitmap | 10000000 |  | SSB.1 CCA |
|  | 10100000 |  | SSB.2 CCA |
|  | 11000000 |  | SSB.3 CCA |
|  | 11110000 |  | SSB.4 CCA |
| } |  |  |  |
| channelAccessMode-r16 CHOICE { |  |  |  |
| dynamic | NULL |  | Dynamic |
| semiStatic SEQUENCE { |  |  | SemiStatic |
| period | FFS |  |  |
| } |  |  |  |
| } |  |  |  |
| discoveryBurstWindowLength-r16 | ms1 | DBT.1 |  |
| ssb-PositionQCL-r16 | n1 |  | SSB.1 CCA SSB.2 CCA |
|  | n2 |  | SSB.3 CCA SSB.4 CCA |
| } |  |  |  |

| Condition | Explanation |
| --- | --- |
| SSB.X CCA | SSB pattern SSB.X CCA configured as per section A.3A (X = {1,2,3,4}) |
| Dynamic | Dynamic channel access |
| SemiStatic | Semi-static channel access |

Table H.3.10-2: SSB-MTC for Shared Spectrum Access

| Derivation Path: Table 38.508-1 Table 7.3.1-3 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| SSB-MTC ::= SEQUENCE { |  |  |  |
| periodicityAndOffset CHOICE { |  |  |  |
| [sf20] | [0] |  | [SMTC.1 CCA] [SMTC.2 CCA] |
| } |  |  |  |
| duration | [sf1] |  | [SMTC.1 CCA] |
|  | [sf5] |  | [SMTC.2 CCA] |
| } |  |  |  |

| Condition | Explanation |
| --- | --- |
| SMTC.X CCA | SMTC pattern SMTC.X CCA configured as per section [A.TBD (X = {1,2})] |

<< End of Changes >>