3GPP TSG-RAN5 Meeting #100	R5-233832

# Toulouse, France, 21st – 25th August 2023

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 36.579-7 | CR | 0038 | rev | - | Current version: | 16.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction of test case 5.7 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | MCC TF160 |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | MCProtoc16_enh2MCPTT_eMCData2-ConTest |  |  |  |  |  | Date: |  |  | 2023-07-19 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | R5-233293 has introduced MCX generic procedures 5.3.36 and 5.3.37 functional alias status determination, subscription and status change replacing the MCPTT specific ones. MCData test case 5.7 is basically the same as MCPTT test case 5.8 and therefore MCData test case 5.7 can refer to MCPTT test case 5.8 in the same way as this has been done for other MCData 5.X test cases. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Test description of MCData test case 5.7 replaced by refrences to MCPTT test case 5.8. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Inconsistent test specification |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.7 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  | X |  | Test specifications |  |  |  | TS 36.579-2 CR 0332 |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

## 5.7	Configuration / Functional Alias / Functional alias status determination / Activate functional alias / Deactivate functional alias

5.7.1	Test Purpose (TP)

(1)

with { UE (MCData Client) registered and authorised for MCData Service }

ensure that {

when { MCData User requests to determine the current status of a functional alias and later notification of status changes of a functional alias }

then { UE (MCData Client) sends a SIP SUBSCRIBE message to determine the current status of a functional alias and later notification of status changes of a functional alias and responds to the SIP NOTIFY message with a SIP 200 (OK) message }

}

(2)

with { UE (MCData Client) having already subscribed to determine the status of a functional alias }

ensure that {

when { MCData User requests to activate a functional alias }

then { UE (MCData Client) sends a SIP PUBLISH message to activate a functional alias and responds to the SIP NOTIFY message with a SIP 200 (OK) message }

}

(3)

with { UE (MCData Client) having already subscribed to determine the status of a functional alias }

ensure that {

when { MCData User requests to deactivate a functional alias }

then { UE (MCData Client) sends a SIP PUBLISH message to deactivate a functional alias and responds to the SIP NOTIFY message with a SIP 200 (OK) message }

}

(4)

with { UE (MCData Client) having already subscribed to determine the status of a functional alias }

ensure that {

when { MCData User requests to de-subscribe from determining the status of a functional alias }

then { UE (MCData Client) sends a SIP SUBSCRIBE message to de-subscribe from determining the status of a functional alias and responds to the SIP NOTIFY message with a SIP 200 (OK) message }

}

5.7.2	Conformance requirements

References: The conformance requirements covered in the present TC are specified in: TS 24.282 clause 22.2.1.2, 22.2.1.3. Unless otherwise stated these are Rel-16 requirements.

[TS 24.282, clause 22.2.1.2]

In order:

-	to indicate that an MCData user requests to activate one or more functional aliases;

-	to indicate that the MCData user requests to deactivate one or more functional aliases;

-	to refresh indication of an MCData user interest in one or more functional aliases due to near expiration of the expiration time of a functional alias with the status set to the "activated" state received in a SIP NOTIFY request in subclause 22.2.1.3;

-	to indicate that the MCData client entering into or exiting from a location area triggers one or more functional aliases to be activated;

-	to indicate that the MCData client entering into or exiting from a location area triggers one or more functional aliases to be deactivated; or

-	any combination of the above;

the MCData client shall generate a SIP PUBLISH request according to TS 24.229 [5], IETF RFC 3903 [34], and IETF RFC 3856 [39].

When the MCData user requests to deactivate a functional alias, the MCData client shall first check the <manual-deactivation-not-allowed-if-location-criteria-met> element within the <anyExt> element of the <entry> element corresponding to the functional alias within the <FunctionalAliasList> list element of the <anyExt> element of the <OnNetwork> element of the MCData user profile document (see the MCData user profile document in TS 24.484 [12]). If the functional alias has been activated due to a location area trigger and the <manual-deactivation-not-allowed-if-location-criteria-met> element is set to a value of "true", the MCData client shall suppress the MCData user’s request.

NOTE 1:	If the request is suppressed, a notification message can be displayed to the user.

In the SIP PUBLISH request, the MCData client:

1)	shall set the Request-URI to the public service identity identifying the originating participating MCData function serving the MCData user;

2)	shall include an application/vnd.3gpp.mcdata-info+xml MIME body. In the application/vnd.3gpp.mcdata-info+xml MIME body, the MCData client shall include the <mcdata-request-uri> element set to the MCData ID of the MCData user;

3)	shall include the ICSI value "urn:urn-7:3gpp-service.ims.icsi.mcdata" (coded as specified in TS 24.229 [5]), in a P-Preferred-Service header field according to IETF RFC 6050 [7];

4)	if the MCData client requests to activate one or more functional aliases, shall set the Expires header field according to IETF RFC 3903 [34], to 4294967295;

NOTE 2:	4294967295, which is equal to 232-1, is the highest value defined for Expires header field in IETF RFC 3261 [4].

5)	if the MCData client requests to deactivate one or more functional aliases, shall set the Expires header field according to IETF RFC 3903 [34], to zero; and

NOTE 3:	Activation and deactivation of functional alias cannot be performed with the same PUBLISH request.

6)	shall include an application/pidf+xml MIME body indicating per-user functional alias information according to subclause 22.3.1. In the MIME body, the MCData client:

a)	shall include all functional aliases where the MCData user requests activation for the MCData ID;

b)	shall include the MCData client ID of the targeted MCData client;

c)	shall not include the "status" attribute and the "expires" attribute in the <functionalalias> element;

d)	if the MCData client has received an indication that take over of a functional alias is possible and intends to take over a functional alias, shall include a <take-over> child element set to "true"; and

e)	shall set the <p-id-fa> child element of the <presence> root element to a globally unique value.

The MCData client shall send the SIP PUBLISH request according to TS 24.229 [5].

[TS 24.282, clause 22.2.1.3]

NOTE 1:	The MCData UE also uses this procedure to determine which functional alias have been successfully activated for the MCData ID.

In order to discover functional aliases:

1)	which are activated for the MCData user; or

2)	which another MCData user has activated;

the MCData client shall generate an initial SIP SUBSCRIBE request according to TS 24.229 [5], IETF RFC 3856 [39], and IETF RFC 6665 [36].

In the SIP SUBSCRIBE request, the MCData client:

1)	shall set the Request-URI to the public service identity identifying the originating participating MCData function serving the MCData user;

2)	shall include an application/vnd.3gpp.mcdata-info+xml MIME body. In the application/vnd.3gpp.mcdata-info+xml MIME body, the MCData client shall include:

a)	the <mcdata-request-uri> element set to the MCData ID of the targeted MCData user; and

b)	the <request-type> element in the <mcdata-Params> element of the <mcdatainfo> element set to the value "functional-alias-status-determination";

3)	shall include the ICSI value "urn:urn-7:3gpp-service.ims.icsi.mcdata" (coded as specified in TS 24.229 [5]), in a P-Preferred-Service header field according to IETF RFC 6050 [7];

4)	if the MCData client wants to receive the current status and later notification, shall set the Expires header field according to IETF RFC 6665 [36], to 4294967295;

NOTE 2:	4294967295, which is equal to 232-1, is the highest value defined for Expires header field in IETF RFC 3261 [4].

5)	if the MCData client wants to fetch the current state only, shall set the Expires header field according to IETF RFC 6665 [36], to zero;

6)	shall include an Events header field set to "presence"; and

7)	shall include an Accept header field containing the application/pidf+xml MIME type.

In order to re-subscribe or de-subscribe, the MCData client shall generate an in-dialog SIP SUBSCRIBE request according to TS 24.229 [5], IETF RFC 3856 [39], and IETF RFC 6665 [36]. In the SIP SUBSCRIBE request, the MCData client:

1)	if the MCData client wants to receive the current status and later notification, shall set the Expires header field according to IETF RFC 6665 [36], to 4294967295;

NOTE 3:	4294967295, which is equal to 232-1, is the highest value defined for Expires header field in IETF RFC 3261 [4].

2)	if the MCData client wants to de-subscribe, shall set the Expires header field according to IETF RFC 6665 [36], to zero;

3)	shall include an Events header field set to "presence"; and

4)	shall include an Accept header field containing the application/pidf+xml MIME type.

Upon receiving a SIP NOTIFY request according to TS 24.229 [5], IETF RFC 3856 [39], and IETF RFC 6665 [36], if SIP NOTIFY request contains an application/pidf+xml MIME body indicating per-user functional alias information  constructed according to subclause 22.3.1, then the MCData client shall determine the status of the MCData user for each functional alias in the MIME body. If the <p-id-fa> child element of the <presence> root element of the application/pidf+xml MIME body of the SIP NOTIFY request is included, the <p-id-fa> element value indicates the SIP PUBLISH request which triggered sending of the SIP NOTIFY request.

If the MCData client detected a functional alias activation or deactivation, it shall perform the procedure specified in subclause 8.2.6.

5.7.3	Test description

5.7.3.1	Pre-test conditions

Same pre-test conditions as for MCPTT test case 5.8 (TS 36.579-2 [24]) with the following exception(s):

-	The term "MCPTT" is replaced with "MCData"

5.7.3.2	Test procedure sequence

Same test procedure sequence as for MCPTT test case 5.8 (TS 36.579-2 [24]) with the following exception(s):

-	The term "MCPTT" is replaced with "MCData"

5.7.3.3	Specific message contents

Same specific message contents as for MCPTT test case 5.8 (TS 36.579-2 [24]) with the following exception(s):

-	The term "MCPTT" is replaced with "MCData"

-	Condition MCDATA is used for all messages.