3GPP TSG-RAN5 Meeting #100	R5-235411

# Toulouse, France, 21st Aug 2023 - 25th Aug 2023

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 36.579-7 | CR | 0045 | rev | 1 | Current version: | 16.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Addition of new test case 6.1.19 Session SDS group Pre-established Session CO |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | NIST |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | MCProtoc16_enh2MCPTT_eMCData2-ConTest |  |  |  |  |  | Date: |  |  | 2023-08-24 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | There was no test case for sending a group session SDS message while using a pre-established session |  |  |  |  |  |  |  |  |
| Summary of change: |  | Added test case 6.1.19 |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | A device could fail in the commercial market without testing this scenario |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.1.19 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | This is the final version of R5-234581 |  |  |  |  |  |  |  |  |

<START OF CHANGES>

### 6.1.19	On-network / Short Data Service (SDS) / SDS Session / Group SDS Session / Pre-established session / Client Originated (CO)

6.1.19.1	Test Purpose (TP)

(1)

with { UE (MCDATA Client) registered and authorised for MCDATA Service }

ensure that {

when { the MCDATA User requests to initiate a group SDS session using the media plane and using the pre-established session }

then { UE (MCDATA Client) sends a request to establish a group SDS session and a MSRP connection via a SIP REFER message and responds to the SIP re-INVITE message with a SIP 200 (OK) message and delivers the notification to the MCDATA User about successful communication establishement and sends a blank MSRP SEND message to bind the MSRP connection }

}

(2)

with { UE (MCDATA Client) having sent a blank MSRP SEND message to bind the MSRP connection }

ensure that {

when { UE (MCDATA Client) receives an MSRP 200 (OK) message in response to the blank MSRP SEND message }

then { UE (MCDATA Client) sends the group session SDS message via a MSRP SEND message with a disposition of "DELIVERY" }

}

(3)

with { UE (MCDATA Client) having sent a group session SDS message using the media plane with a disposition of "DELIVERY" }

ensure that {

when { UE (MCDATA Client) receives a disposition response via a MSRP SEND message }

then { UE (MCDATA Client) responds to the MSRP SEND message by sending a MSRP 200 (OK) message and delivers the notification to the MCDATA User }

}

(4)

with { UE (MCDATA Client) having established a group SDS session using the pre-established session }

ensure that {

when { the MCDATA User requests to release the group SDS session while keeping the pre-established session }

then { UE (MCDATA Client) sends a SIP REFER message to release the MCData session and keep the pre-established session and responds to the SIP re-INVITE message with a SIP 200 (OK) message and delivers the notification to the MCDATA User about successful termination }

}

6.1.19.2	Conformance requirements

References: The conformance requirements covered in the current TC are specified in: TS 24.282, clauses 9.2.5.3.1.1, 9.2.3.2.1, 9.2.5.4.1.1, TS 24.582 clauses 12.1, 6.1.2.2.1, 6.1.2.6. The following represents a copy/paste extraction of the requirements relevant to the test purpose; any references within the copy/paste text should be understood within the scope of the core spec they have been copied from. Unless otherwise stated, these are Rel-16 requirements.

[TS 24.282, clause 9.2.5.3.1.1]

Upon receiving a request from an MCData user to initiate group SDS session within the pre-established session, the MCData client shall generate a SIP REFER request outside a dialog as specified in IETF RFC 3515 [51] as updated by IETF RFC 6665 [36] and IETF RFC 7647 [52], and in accordance with the UE procedures specified in 3GPP TS 24.229 [5], with the clarifications given below.

The MCData client:

1)	shall set the Request URI of the SIP REFER request to the session identity of the pre-established session;

2)	shall set the Refer-To header field of the SIP REFER request as specified in IETF RFC 3515 [51] with a Content-ID ("cid") Uniform Resource Locator (URL) as specified in IETF RFC 2392 [33] that points to an application/resource-lists MIME body as specified in IETF RFC 5366 [18], and with the Content-ID header field set to this "cid" URL;

3)	shall include in the application/resource-lists MIME body a single <entry> element containing a "uri" attribute set to the MCData group identity, extended with the following parameters in the headers portion of the SIP URI:

NOTE:	Characters that are not formatted as ASCII characters are escaped in the following parameters in the headers portion of the SIP URI.

a)	an hname "body" parameter populated with:

i)	an application/sdp MIME body containing an SDP offer with media attributes specified in subclause 9.2.3.2.1, if if a group standalone SDS message is requested;

ii)	an application/vnd.3gpp.mcdata-info MIME body with:

A)	if a group standalone SDS message is requested, the <request-type> element set to a value of "group-sds". If a group SDS session is requested, the <request-type> element set to a value of "group-sds-session";

B)	the <mcdata-request-uri> element set to the MCData group identity;

C)	the <mcdata-client-id> element set to the MCData client ID of the originating MCData client; and

D)	if the MCData client is aware of active functional aliases and if an active functional alias is to be included in the SIP REFER request, the <functional-alias-URI> element set to the URI of the used functional alias;

4)	shall include a P-Preferred-Service header field set to the ICSI value "urn:urn-7:3gpp-service.ims.icsi.mcdata.sds" (coded as specified in 3GPP TS 24.229 [5]), according to IETF RFC 6050 [7];

5)	may include a P-Preferred-Identity header field in the SIP INVITE request containing a public user identity as specified in 3GPP TS 24.229 [5];

6)	shall include the following according to IETF RFC 4488 [53]:

a)	the option tag "norefersub" in the Supported header field; and

b)	the value "false" in the Refer-Sub header field;

7)	shall include a Target-Dialog header field as specified in IETF RFC 4538 [54] identifying the pre-established session;

8)	shall include the g.3gpp.mcdata.sds media feature tag in the Contact header field of the SIP REFER request according to IETF RFC 3840 [16]; and

9)	shall send the SIP REFER request according to 3GPP TS 24.229 [5].

On receiving a final SIP 2xx response to the SIP REFER request, the MCData client:

1)	shall interact with the media plane as specified in 3GPP TS 24.582 [15].

On receiving a SIP re-INVITE request within the pre-established session targeted by the sent SIP REFER request, the MCData client:

1)	if the <mcdata-communication-state> element in the application/vnd.3gpp.mcdata-info+xml MIME body of the SIP INVITE request is set to a value of "establish-success":

i)	shall notify MCData user about successful the MCData communication establishement;

2)	if the <mcdata-communication-state> element in the application/vnd.3gpp.mcdata-info+xml MIME body of the SIP INVITE request is set to a value of "establish-fail":

i)	shall notify MCData user about the MCData communication establishement failure; and

3)	shall interact with the media plane as specified in 3GPP TS 24.582 [15].

[TS 24.282, clause 9.2.3.2.1]

When composing an SDP offer according to 3GPP TS 24.229 [5], IETF RFC 4975 [17], IETF RFC 6135 [19] and IETF RFC 6714 [20] the MCData client:

1)	shall include an "m=message" media-level section for the MCData media stream consisting of:

a)	the port number;

b)	a protocol field value of "TCP/MSRP", or "TCP/TLS/MSRP" for TLS;

c)	a format list field set to ‘*’;

d)	an "a=sendonly" attribute;

e)	an "a=path" attribute containing its own MSRP URI;

f)	set the content type as "a=accept-types:application/vnd.3gpp.mcdata-signalling application/vnd.3gpp.mcdata-payload"; and

g)	set the a=setup attribute as "actpass"; and

2)	if end-to-end security is required for a one-to-one communication and the security context does not exist or if the existing security context has expired, shall include the MIKEY-SAKKE I_MESSAGE in an "a=key-mgmt" attribute as a "mikey" attribute value in the SDP offer as specified in IETF RFC 4567 [45].

[TS 24.282, clause 9.2.5.4.1.1]

Upon receiving a request from an MCData user to leave an MCData session within a pre-established session, the MCData client:

1)	shall interact with the media plane as specified in 3GPP TS 24.582 [15];

2)	shall generate an initial SIP REFER request outside a dialog in accordance with the procedures specified in 3GPP TS 24.229 [5], IETF RFC 4488 [53] and IETF RFC 3515 [51] as updated by IETF RFC 6665 [36] and IETF RFC 7647 [r7647];

3)	shall set the Request-URI of the SIP REFER request to the public service identity identifying the pre-established session on the MCData server serving the MCData user;

4)	shall include the Refer-Sub header field with value "false" according to rules and procedures of IETF RFC 4488 [53];

5)	shall include the Supported header field with value "norefersub" according to rules and procedures of IETF RFC 4488 [53];

6)	shall set the Refer-To header field of the SIP REFER request to the MCData session identity to leave;

7)	shall include the "method" SIP URI parameter with the value "BYE" in the URI in the Refer-To header field;

8)	shall include a Target-Dialog header field as specified in IETF RFC 4538 [54] identifying the pre-established session; and

9)	shall send the SIP REFER request according to 3GPP TS 24.229 [5].

Upon receiving a SIP 2xx response to the SIP REFER request, the MCData client shall interact with media plane as specified in 3GPP TS 24.582 [15].

On receiving a SIP re-INVITE request within the pre-established session targeted by the sent SIP REFER request, the MCData client:

1)	if the <mcdata-communication-state> element in the application/vnd.3gpp.mcdata-info+xml MIME body of the SIP INVITE request is set to a value of "terminated":

i)	shall notify MCData user about successful the MCData communication termination.

[TS 24.582, clause 12.1]

Upon establishing one-to-one or group SDS using media plane or one-to-one or group SDS session using pre-established call, the MCData client shall follow procedures as descried in subclause 6.1.

[TS 24.582, clause 6.1.2.2.1]

Upon receiving an indication to establish MSRP connection for SDS session as the originating MCData client, the MCData client:

1.	shall act as an MSRP client according to IETF RFC 6135 [12];

2.	shall act according to IETF RFC 6135 [12], as:

a.	an "active" endpoint, if a=setup attribute in the received SDP answer is set to "passive"; and

b.	an "passive" endpoint, if a=setup attribute in the received SDP answer is set to "active";

3.	shall establish the MSRP connection according to the MSRP connection parameters in the SDP answer received in the SIP 200 (OK) response according to IETF RFC 4975 [11];

4.	if acting as an "active" endpoint, shall send an empty MSRP SEND request to bind the MSRP connection to the MSRP session from the perspective of the passive endpoint according to the rules and procedures of IETF RFC 4975 [11] and IETF RFC 6135 [12];

Once the MSRP session is established, the MCData client:

1.	on receipt of an MSRP request in the MSRP session, shall follow the rules and procedures defined in IETF RFC 4975 [11] and in IETF RFC 6714 [13];

2.	If an MSRP SEND request indicates the use of chunking, shall wait until all further MSRP SEND requests for the remaining chunks have been received and shall reassemble the entire set of MSRP requests into the MCData SDS message before delivering the content to the application; and

3.	shall handle the received content as described in subclause 6.1.2.6.

On receiving MSRP 200 (OK) response to the first MSRP SEND request, the MCData client can generate and send an SDS message as specified in subclause 6.1.2.4, or can generate and send an SDS disposition notification for a received SDS message as specified in subclause 6.1.2.5, if requested.

Received content and disposition requests shall be handled as specified in subclause 6.1.2.6.

[TS 24.582, clause 6.1.2.6]

Upon receiving an SDS message, the MCData client:

1.	shall follow the procedure defined in subclause 6.1.1.3.2, with the following clarification:

a.	if SDS Disposition request type IE is present in the received SDS SIGNALLING PAYLOAD message then, shall send an SDS disposition notification as described in subclause 6.1.2.5.

Upon receiving an SDS disposition notification, the MCData client:

1.	shall decode the contents of the application/vnd.3gpp.mcdata-signalling MIME body; and

2.	shall deliver the notification to the user or application.

6.1.19.3	Test description

6.1.19.3.1	Pre-test conditions

System Simulator:

-	SS (MCData server)

-	For the underlying "transport bearer" over which the SS and the UE will communicate Parameters are set to the default parameters for the basic E-UTRA Single cell network scenarios, as defined in TS 36.508 [24] clause 4.4. The simulated Cell 1 shall belong to PLMN1 (the PLMN specified for MCDATA operation in the MCDATA configuration document).

IUT:

-	UE (MCData client)

-	The test USIM set as defined in TS 36.579-1 [2] clause 5.5.10 is inserted.

Preamble:

-	The UE has performed procedure 'MCData UE registration' as specified in TS 36.579-1 [2] clause 5.4.2B.

-	The UE has performed procedure 'MCX Authorization/Configuration and Key Generation' as specified in TS 36.579-1 [2] clause 5.3.2.

-	The UE has performed procedure 'MCX pre-established session establishment' as specified in TS 36.579-1 [2] clause 5.3.3.

-	UE States at the end of the preamble

-	The UE is in E-UTRA Registered, Idle Mode state.

-	The MCData Client Application has been activated and User has registered-in as the MCDATA User with the Server as active user at the Client.

6.1.19.3.2	Test procedure sequence

Table 6.1.19.3.2-1: Main Behaviour

| St | Procedure | Message Sequence |  | TP | Verdict |
| --- | --- | --- | --- | --- | --- |
|  |  | U - S | Message |  |  |
| 1 | Make the UE (MCData client) send a group session SDS message using the pre-established session with disposition request "DELIVERY". (NOTE 1) | - | - | - | - |
| 2 | Check: Does the UE (MCData client) correctly perform procedure 'CO MCData call establishment using a pre-established session' as described in TS 36.579-1 [2] Table 5.3C.12.3-1? | - | - | 1 | P |
| 3 | Check: Does the UE (MCData client) notify the user about successful MCData communication establishment? (NOTE 1) | - | - | 1 | P |
| 4 | Check: Does the UE (MCData client) correctly perform procedure 'CO MSRP message transfer' as described in TS 36.579-1 [2] Table 5.3C.4.3-1 to send an SDS message with disposition request "DELIVERY"? | - | - | 2 | P |
| 5 | Check: Does the UE (MCData client) correctly perform procedure 'CT MSRP message transfer' as described in TS 36.579-1 [2] Table 5.3C.5.3-1 to receive the disposition notification for the SDS message sent at step 3? | - | - | 3 | P |
| 6 | Check: Does the UE (MCData client) provide the disposition notification to the user? (NOTE 1) | - | - | 3 | P |
| 7 | Make the UE (MCData client) release the group session while keeping the pre-established session. (NOTE 1) | - | - | - | - |
| 8 | Check: Does the UE (MCData client) correctly perform procedure 'MCData CO call release keeping the pre-established session' as described in TS 36.579-1 [2] Table 5.3C.13.3-1? | - | - | 4 | P |
| 9 | Check: Does the UE (MCData client) provide a notification to the MCData user about successful MCData communication termination? (NOTE 1) | - | - | 4 | P |
| NOTE 1: This is expected to be done via a suitable implementation dependent MMI. |  |  |  |  |  |

6.1.19.3.3	Specific message contents

Table 6.1.19.3.3-1: SIP REFER from the UE (step 2, Table 6.1.19.3.2-1;
step 2, TS 36.579-1 [2] Table 5.3C.12.3-1)

| Derivation Path: TS 36.579-1 [2], Table 5.5.2.12-1 |  |  |  |  |
| --- | --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Reference | Condition |
| Message-body |  |  |  |  |
| MIME body part |  | Resource list |  |  |
| MIME-part-body | Resource-lists as described in Table 6.1.19.3.3-2 |  |  |  |

Table 6.1.19.3.3-2: Resource-lists in SIP REFER (Table 6.1.19.3.3-1)

Derivation Path: TS 36.579-1 [2], Table 5.5.3.3.1-3, condition PRE-ESTABLISH, MCD_grp with the uri attribute of the entry extended with the SIP URI header fields as specified in Table 6.1.19.3.3-3

Table 6.1.19.3.3-3: SIP header fields extending the uri attribute of the resource-lists' single entry
(Table 6.1.19.3.3-2)

| Derivation Path: TS 36.579-1 [2], Table 5.5.2.12-2 |  |  |  |  |
| --- | --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Reference | Condition |
| body |  |  |  |  |
| MIME body part |  | SDP Message |  |  |
| MIME-part-headers |  |  |  |  |
| Content-Type | “application/sdp” |  |  |  |
| MIME-part-body | SDP Message as described in Table 6.1.19.3.3-4 |  |  |  |
| MIME body part |  | MCData-Info |  |  |
| MIME-part-body | MCdata-Info as described in Table 6.1.19.3.3-5 |  |  |  |

Table 6.1.19.3.3-4: SDP in SIP header fields (Table 6.1.19.3.3-3)

Derivation Path: TS 36.579-1 [2], Table 5.5.3.1.1-3, condition PRE_ESTABLISHED_SESSION, SDP_OFFER, MCDATA_SDS, SDS_SESSION

Table 6.1.19.3.3-5: MCData-Info in SIP header fields (Table 6.1.19.3.3-3)

| Derivation Path: TS 36.579-1 [2], Table 5.5.3.2.1-3, condition MCD_grp |  |  |  |  |
| --- | --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Reference | Condition |
| mcdata-info |  |  |  |  |
| mcdata-Params |  |  |  |  |
| request-type | "group-sds-session" |  | TS 24.282 [31] clause 9.2.5.2.1.1 |  |

Table 6.1.19.3.3-6: MSRP SEND from the UE (step 4, Table 6.1.19.3.2-1;
step 1, TS 36.579-1 [2] Table 5.3C.4.3-1)

| Derivation Path: TS 36.579-1 [2], Table 5.5.12.1.1-1 |  |  |  |  |
| --- | --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Reference | Condition |
| Content-Type |  |  |  |  |
| media-type | "multipart/mixed" |  |  |  |
| data | Message or chunk of message as specified in table 6.1.19.3.3-7 |  |  |  |

Table 6.1.19.3.3-7: MIME Message (step 4, Table 6.1.19.3.2-1;
step 3, TS 36.579-1 [2] Table 5.3C.4.3-1)

| Derivation Path: RFC 2046 [38] |  |  |  |  |
| --- | --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Reference | Condition |
| MIME body part |  | MCData Data signalling message |  |  |
| MIME-part-headers |  |  |  |  |
| Content-Type | "application/vnd.3gpp.mcdata-signalling" |  |  |  |
| MIME-part-body | MCData Protected Payload Message containing SDS SIGNALLING PAYLOAD as described in table 6.1.19.3.3-8 |  |  |  |
| MIME body part |  | MCData Data message |  |  |
| MIME-part-headers |  |  |  |  |
| Content-Type | "application/vnd.3gpp.mcdata-payload" |  |  |  |
| MIME-part-body | DATA PAYLOAD as described in Table 6.1.19.3.3-9 |  |  |  |

Table 6.1.19.3.3-8: SDS SIGNALLING PAYLOAD (Table 6.1.19.3.3-7)

Derivation Path: TS 36.579-1 [2], Table 5.5.3.8.1-1, condition DELIVERED

Table 6.1.19.3.3-9: Data Payload (Table 6.1.19.3.3-7)

Derivation Path: TS 36.579-1 [2], Table 5.5.3.9.2-1

Table 6.1.19.3.3-10: MSRP SEND from the SS (step 5, Table 6.1.19.3.2-1;
step 1, TS 36.579-1 [2] Table 5.3C.5.3-1)

| Derivation Path: TS 36.579-1 [2], Table 5.5.12.1.2-1 |  |  |  |  |
| --- | --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Reference | Condition |
| Content-Type |  |  |  |  |
| media-type | "application/vnd.3gpp.mcdata-signalling" |  |  |  |
| data | MCData Protected Payload Message containing SDS NOTIFICATION as specified in table 6.1.19.3.3-11 |  |  |  |

Table 6.1.19.3.3-11: SDS NOTIFICATION (Table 6.1.19.3.3-10)

Derivation Path: TS 36.579-1 [2], Table 5.5.3.8.4-1, condition DELIVERED

<END OF CHANGES>