3GPP TSG-RAN5 Meeting #100	R5-234708

# Toulouse, France, 21st – 25th Aug. 2023

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.521-1 | CR | 2378 | rev | - | Current version: | 17.9.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Addition of MPR test requrements for inter-band CA Power Class 2 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, HiSilicon |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_PC2_CA_R17_2BDL_2BUL-UEConTest |  |  |  |  |  | Date: |  |  | 2023-08-01 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | MPR test requirements inter-band CA Power Class 2 was missing. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Adding MPR test requirements inter-band CA Power Class 2 based on the P-max configuration proposed in R5-234706. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | MPR testing for inter-band CA Power Class 2 can't be tested. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.2A.2.1 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

<Start of Changes>

6.2A.2.1	UE maximum output power reduction for CA (2UL CA)

Editor’s note:	Testing for Intra-band non-contiguous UL CA is not applicable for UE not supporting IE dualPA-Architecture due to the lack of core requirement.

6.2A.2.1.1	Test purpose

The number of RB identified in Table 6.2.2.3-1 is based on meeting the requirements for adjacent channel leakage ratio for CA and the maximum power reduction (MPR) for Inter-band CA due to Cubic Metric (CM).

6.2A.2.1.2	Test applicability

The requirements of this test apply to all types of NR Power class 3 UE release 15 and forward that support 2 UL CA.

NOTE:	Test execution is not necessary if TS 38.521-1 6.5A.2.4.1.1 is executed.

6.2A.2.1.3	Minimum conformance requirements

The minimum conformance requirements are defined in subclause 6.2A.2.0.

6.2A.2.1.4	Test description

6.2A.2.1.4.1	Initial conditions

Initial conditions are a set of test configurations the UE needs to be tested in and the steps for the SS to take with the UE to reach the correct measurement state.

The initial test configurations consist of environmental conditions, test frequencies, channel bandwidths and sub-carrier spacing based on NR CA configuration specified in table 5.5A.3.1-1 for inter-band UL CA, table 5.5A.1-1 for intra-band contiguous UL CA and table 5.5A.2-1 for intra-band non-contiguous UL CA. All of these configurations shall be tested with applicable test parameters for each CA configuration of test channel bandwidth and sub-carrier spacing, and are shown in table 6.2A.2.1.4.1-1 to table 6.2A.2.1.4.1-3b. The details of the uplink reference measurement channels (RMCs) are specified in Annexes A.2. Configurations of PDSCH and PDCCH before measurement are specified in Annex C.2.

Table 6.2A.2.1.4.1-1: Inter-band CA Test Configuration Table

| Initial Conditions |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Test Environment as specified in TS 38.508-1 [5] subclause 4.1 |  |  | Normal, TL/VL, TL/VH, TH/VL, TH/VH |  |  |
| Test Frequencies as specified in TS 38.508-1 [5] subclause4.3.1 |  |  | Low range for PCC and SCC High range for PCC and SCC |  |  |
| Test Channel Bandwidths as specified in TS 38.508-1 [5] subclause 4.3.1 |  |  | Lowest NRB_agg, Highest NRB_agg |  |  |
| Test SCS as specified in Table 5.3.5-1 |  |  | Lowest, Highest |  |  |
| Test Parameters for Channel Bandwidths |  |  |  |  |  |
| Test ID | Freq | Downlink Configuration | Uplink Configuration |  |  |
|  |  | for PCC & SCC | Modulation for all | RB allocation (NOTE 1) |  |
|  |  |  | CCs (NOTE 2) | PCC | SCC |
| 13 | Default | N/A for Maximum Power Reduction (MPR) test case | DFT-s-OFDM Pi/2 BPSK | Inner Full | Inner Full |
| 23 | Low |  | DFT-s-OFDM Pi/2 BPSK | Edge_1RB_Left | Edge_1RB_Left |
| 33 | High |  | DFT-s-OFDM Pi/2 BPSK | Edge_1RB_Right | Edge_1RB_Right |
| 43 | Default |  | DFT-s-OFDM Pi/2 BPSK | Outer Full | Outer Full |
| 54 | Default |  | DFT-s-OFDM Pi/2 BPSK | Inner Full | Inner Full |
| 64 | Low |  | DFT-s-OFDM Pi/2 BPSK | Edge_1RB_Left | Edge_1RB_Left |
| 74 | High |  | DFT-s-OFDM Pi/2 BPSK | Edge_1RB_Right | Edge_1RB_Right |
| 84 | Default |  | DFT-s-OFDM Pi/2 BPSK | Outer Full | Outer Full |
| 9 | Default |  | DFT-s-OFDM QPSK | Inner Full | Inner Full |
| 10 | Low |  | DFT-s-OFDM QPSK | Edge_1RB_Left | Edge_1RB_Left |
| 11 | High |  | DFT-s-OFDM QPSK | Edge_1RB_Right | Edge_1RB_Right |
| 12 | Default |  | DFT-s-OFDM QPSK | Outer Full | Outer Full |
| 13 | Default |  | DFT-s-OFDM 16 QAM | Inner Full | Inner Full |
| 14 | Low |  | DFT-s-OFDM 16 QAM | Edge_1RB_Left | Edge_1RB_Left |
| 15 | High |  | DFT-s-OFDM 16 QAM | Edge_1RB_Right | Edge_1RB_Right |
| 16 | Default |  | DFT-s-OFDM 16 QAM | Outer Full | Outer Full |
| 17 | Low |  | DFT-s-OFDM 64 QAM | Edge_1RB_Left | Edge_1RB_Left |
| 18 | High |  | DFT-s-OFDM 64 QAM | Edge_1RB_Right | Edge_1RB_Right |
| 19 | Default |  | DFT-s-OFDM 64 QAM | Outer Full | Outer Full |
| 20 | Low |  | DFT-s-OFDM 256 QAM | Edge_1RB_Left | Edge_1RB_Left |
| 21 | High |  | DFT-s-OFDM 256 QAM | Edge_1RB_Right | Edge_1RB_Right |
| 22 | Default |  | DFT-s-OFDM 256 QAM | Outer Full | Outer Full |
| 23 | Default |  | CP-OFDM QPSK | Inner Full | Inner Full |
| 24 | Low |  | CP-OFDM QPSK | Edge_1RB_Left | Edge_1RB_Left |
| 25 | High |  | CP-OFDM QPSK | Edge_1RB_Right | Edge_1RB_Right |
| 26 | Default |  | CP-OFDM QPSK | Outer Full | Outer Full |
| 27 | Default |  | CP-OFDM 16 QAM | Inner Full | Inner Full |
| 28 | Low |  | CP-OFDM 16 QAM | Edge_1RB_Left | Edge_1RB_Left |
| 29 | High |  | CP-OFDM 16 QAM | Edge_1RB_Right | Edge_1RB_Right |
| 30 | Default |  | CP-OFDM 16 QAM | Outer Full | Outer Full |
| 31 | Low |  | CP-OFDM 64 QAM | Edge_1RB_Left | Edge_1RB_Left |
| 32 | High |  | CP-OFDM 64 QAM | Edge_1RB_Right | Edge_1RB_Right |

| Test Parameters for Channel Bandwidths |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Test ID | Freq | Downlink Configuration | Uplink Configuration |  |  |
|  |  | for PCC & SCC | Modulation for all | RB allocation (NOTE 1) |  |
|  |  |  | CCs (NOTE 2) | PCC | SCC |
| 33 | Default |  | CP-OFDM 64 QAM | Outer Full | Outer Full |
| 34 | Low |  | CP-OFDM 256 QAM | Edge_1RB_Left | Edge_1RB_Left |
| 35 | High |  | CP-OFDM 256 QAM | Edge_1RB_Right | Edge_1RB_Right |
| 36 | Default |  | CP-OFDM 256 QAM | Outer Full | Outer Full |
| 37 | Default |  | NOTE 5 | Inner Full | Outer Full |
| NOTE 1: The specific configuration of each RB allocation is defined in Table 6.1-1. NOTE 2: DFT-s-OFDM Pi/2 BPSK test applies only for UEs which supports Pi/2 BPSK in FR1. NOTE 3: UE operating in TDD mode with Pi/2 BPSK modulation and UE indicates support for UE capability powerBoosting-pi2BPSK and the IE powerBoostPi2BPSK is set to 1 for bands n40, n41, n77, n78 and n79. NOTE 4: UE operating in FDD mode, or in TDD mode in bands other than n40, n41, n77, n78 and n79, or in TDD mode the IE powerBoostPi2BPSK is set to 0 for bands n40, n41, n77, n78 and n79. NOTE 5: The modulation is DFT-s-OFDM QPSK for PCC and CP-OFDM 256 QAM for SCC. |  |  |  |  |  |

Table 6.2A.2.1.4.1-2a: Intra-band contiguous CA Test Configuration Table for PC3 and PC2 (contiguous RB allocation)

| Initial Conditions |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Test Environment as specified in TS 38.508-1 [5] subclause 4.1 |  |  | Normal, TL/VL, TL/VH, TH/VL, TH/VH |  |  |
| Test Frequencies as specified in TS 38.508-1 [5] subclause4.3.1 |  |  | Low range High range |  |  |
| Test Channel Bandwidths as specified in TS 38.508-1 [5] subclause 4.3.1 |  |  | Lowest NRB_agg, Highest NRB_agg (NOTE 1) |  |  |
| Test SCS as specified in Table 5.3.5-1 |  |  | Lowest, Highest |  |  |
| Test Parameters for CA bandwidth class B and C |  |  |  |  |  |
| Test ID | DL configuration | UL configuration |  |  |  |
|  | for PCC & SCC | Modulations for all CCs (NOTE 2) |  |  | RB allocation (NOTE 3) |
| 1 | N/A |  |  | Pi/2 BPSK | Inner Full |
| 2 |  |  |  | Pi/2 BPSK | Outer Full |
| 3 |  |  |  | QPSK | Inner Full |
| 4 |  |  |  | QPSK | Outer Full |
| 5 |  | DFT-s-OFDM |  | 16QAM | Inner Full |
| 6 |  |  |  | 16QAM | Outer Full |
| 7 |  |  |  | 64QAM | Inner Full |
| 8 |  |  |  | 64QAM | Outer Full |
| 9 |  |  |  | 256QAM | Inner Full |
| 10 |  |  |  | 256QAM | Outer Full |
| 11 |  |  |  | QPSK | Inner Full |
| 12 |  |  |  | QPSK | Outer Full |
| 13 |  |  |  | 16QAM | Inner Full |
| 14 |  | CP-OFDM |  | 16QAM | Outer Full |
| 15 |  |  |  | 64QAM | Inner Full |
| 16 |  |  |  | 64QAM | Outer Full |
| 17 |  |  |  | 256QAM | Inner Full |
| 18 |  |  |  | 256QAM | Outer Full |
| NOTE 1: The Test CC Combination settings are checked separately for each CA Configuration, which applicable aggregated channel bandwidths are specified in Table 5.5A.1-1. NOTE 2: DFT-s-OFDM Pi/2 BPSK test applies only for UEs which supports Pi/2 BPSK in FR1. NOTE 3: The specific configuration of each RB allocation is defined in Table 6.1A-1a. NOTE 4: If the UE supports multiple CC Combinations in the CA Configuration with the same NRB_agg, only the combination with the highest NRB_PCC is tested. |  |  |  |  |  |

Table 6.2A.2.1.4.1-2b: Intra-band contiguous CA Test Configuration Table  for PC3 and PC2 (non-contiguous RB allocation)

| Initial Conditions |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Test Environment as specified in TS 38.508-1 [5] subclause 4.1 |  |  | Normal, TL/VL, TL/VH, TH/VL, TH/VH |  |  |
| Test Frequencies as specified in TS 38.508-1 [5] subclause4.3.1 |  |  | Low range High range |  |  |
| Test Channel Bandwidths as specified in TS 38.508-1 [5] subclause 4.3.1 |  |  | Lowest NRB_agg, Highest NRB_agg (NOTE 1) |  |  |
| Test SCS as specified in Table 5.3.5-1 |  |  | Lowest, Highest |  |  |
| Test Parameters for CA bandwidth class B and C |  |  |  |  |  |
| Test ID | DL configuration | UL configuration |  |  |  |
|  | for PCC & SCC | Modulations for all CCs (NOTE 2) |  |  | RB allocation (NOTE 3) |
| 1 | N/A |  |  | Pi/2 BPSK | Inner |
| 2 |  |  |  | Pi/2 BPSK | Outer 1 |
| 3 |  |  |  | Pi/2 BPSK | Outer 2 |
| 4 |  |  |  | QPSK | Inner |
| 5 |  |  |  | QPSK | Outer 1 |
| 6 |  |  |  | QPSK | Outer 2 |
| 7 |  | DFT-s-OFDM |  | 16QAM | Inner |
| 8 |  |  |  | 16QAM | Outer 1 |
| 9 |  |  |  | 16QAM | Outer 2 |
| 10 |  |  |  | 64QAM | Inner |
| 11 |  |  |  | 64QAM | Outer 1 |
| 12 |  |  |  | 64QAM | Outer 2 |
| 13 |  |  |  | 256QAM | Inner |
| 14 |  |  |  | 256QAM | Outer 1 |
| 15 |  |  |  | 256QAM | Outer 2 |
| 16 |  |  |  | QPSK | Inner |
| 17 |  |  |  | QPSK | Outer 1 |
| 18 |  |  |  | QPSK | Outer 2 |
| 19 |  |  |  | 16QAM | Inner |
| 20 |  |  |  | 16QAM | Outer 1 |
| 21 |  | CP-OFDM |  | 16QAM | Outer 2 |
| 22 |  |  |  | 64QAM | Inner |
| 23 |  |  |  | 64QAM | Outer 1 |
| 24 |  |  |  | 64QAM | Outer 2 |
| 25 |  |  |  | 256QAM | Inner |
| 26 |  |  |  | 256QAM | Outer 1 |
| 27 |  |  |  | 256QAM | Outer 2 |
| NOTE 1: The Test CC Combination settings are checked separately for each CA Configuration, which applicable aggregated channel bandwidths are specified in Table 5.5A.1-1. NOTE 2: DFT-s-OFDM Pi/2 BPSK test applies only for UEs which supports Pi/2 BPSK in FR1. NOTE 3: The specific configuration of each RB allocation is defined in Table 6.1A-1b. NOTE 4: If the UE supports multiple CC Combinations in the CA Configuration with the same NRB_agg, only the combination with the highest NRB_PCC is tested. |  |  |  |  |  |

For intra-band non-contiguous CA, the initial condition for MPR is specified by 2 tables. Table 6.2A.2.1.4.1-3a applies for MPR to meet -30dBm/MHz and Table 6.2A.2.1.4.1-3b applies for MPR to meet -13dBm/MHz. UE determines to configure test parameters based on specific channel bandwidth configurations and following rules:

If  AND{ [FCC1-FCC2 +X] > (NRB1*SCS1*12*0.001) + (0.5*NRB2*SCS2*12*0.001) - 0.5*CBW1 ,

[FCC1-FCC2+Y] > (0.5*NRB1*SCS1*12*0.001) + (NRB2*SCS2*12*0.001) - 0.5*CBW2 }

Test parameters in table 6.2A.2.1.4.1-3a apply

Else

Test parameters in table 6.2A.2.1.4.1-3b apply

Where

CC1 is the lower component carrier. And FCC1 is the centre frequency of CC1

CC2 is the lower component carrier. And FCC2 is the centre frequency of CC2

NRB1 and NRB2 are defined in table 5.3.2-1 for each carrier bandwidth for CC1 and CC2.

SCS1 and SCS2 are the subcarrier spacing for CC1 and CC2.

X is the distance between threshold frequency, where lower spectral emission mask below CC1 drops from -13 dBm / MHz to -25 dBm / MHz, and the lower edge of CC1 channel bandwidth. The values of X is listed in table 6.2A.2.1.4.1-3.

Y is the distance between threshold frequency, where upper spectral emission mask above CC2 drops from -13 dBm / MHz to -25 dBm / MHz, and the upper edge of CC2 channel bandwidth. The values of Y is listed in table 6.2A.2.1.4.1-3.

Table 6.2A.2.1.4.1-3: Values specified for X and Y for each channel bandwidth

| CBW | 5 MHz | 10 MHz | 15 MHz | 20 MHz | 25 MHz | 30 MHz | 40 MHz | 50 MHz | 60 MHz | 70 MHz | 80 MHz | 90 MHz | 100 MHz |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Value | 6 | 10 | 15 | 20 | 25 | 30 | 40 | 50 | 60 | 70 | 80 | 90 | 100 |

Table 6.2A.2.1.4.1-3a: Intra-band non-contiguous CA Test Configuration Table (MPRIM3 to meet -13dBm/MHz) when UE supporting IE dualPA-Architecture

| Initial Conditions |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Test Environment as specified in TS 38.508-1 [5] subclause 4.1 |  |  |  |  |  | Normal, TL/VL, TL/VH, TH/VL, TH/VH |  |  |  |  |
| Test Frequencies as specified in TS 38.508-1 [5] subclause4.3.1 |  |  |  |  |  | Low range and High range test frequencies as specified in tables for non-contiguous CA configuration with UL CA |  |  |  |  |
| Test Channel Bandwidths as specified in TS 38.508-1 [5] subclause 4.3.1 |  |  |  |  |  | Lowest NRB_agg, Highest NRB_agg (NOTE 1) |  |  |  |  |
| Test SCS as specified in Table 5.3.5-1 |  |  |  |  |  | Lowest, Highest |  |  |  |  |
| Test Parameters |  |  |  |  |  |  |  |  |  |  |
| Test ID | DL config for PCC & SCC | UL configuration |  |  |  |  |  |  |  |  |
|  |  | Modulations for all CCs (NOTE 2) |  | RB allocation LCRB@RBSTART (NOTE 2) |  |  |  |  |  |  |
|  |  |  |  | SCS 15 kHz |  |  | SCS 30 kHz |  | SCS 60 kHz |  |
|  |  |  |  | PCC | SCC |  | PCC | SCC | PCC | SCC |
| 1 | N/A |  | Pi/2 BPSK | Edge_1RB_Left | Edge_1RB_Right |  | Edge_1RB_Left | Edge_1RB_Right | Edge_1RB_Left | Edge_1RB_Right |
| 2 |  |  | Pi/2 BPSK | 8@0 | 8@0 |  | 4@0 | 4@0 | 2@0 | 2@0 |
| 3 |  |  | Pi/2 BPSK | 24@0 | 24@0 |  | 12@0 | 12@0 | 6@0 | 6@0 |
| 4 |  |  | Pi/2 BPSK | 45@0 | 45@0 |  | 24@0 | 24@0 | 12@0 | 12@0 |
| 53 |  |  | Pi/2 BPSK | 75@0 | 75@0 |  | 40@0 | 40@0 | 20@0 | 20@0 |
| 64 |  |  | Pi/2 BPSK | 120@0 | 120@0 |  | 60@0 | 60@0 | 30@0 | 30@0 |
| 75 |  |  | Pi/2 BPSK | 150@0 | 150@0 |  | 75@0 | 75@0 | 40@0 | 40@0 |
| 85 |  |  | Pi/2 BPSK | Outer_Full | Outer_Full |  | Outer_Full | Outer_Full | Outer_Full | Outer_Full |
| 9 |  |  | QPSK | Edge_1RB_Left | Edge_1RB_Right |  | Edge_1RB_Left | Edge_1RB_Right | Edge_1RB_Left | Edge_1RB_Right |
| 10 |  |  | QPSK | 8@0 | 8@0 |  | 4@0 | 4@0 | 2@0 | 2@0 |
| 11 |  |  | QPSK | 24@0 | 24@0 |  | 12@0 | 12@0 | 6@0 | 6@0 |
| 12 |  | D | QPSK | 45@0 | 45@0 |  | 24@0 | 24@0 | 12@0 | 12@0 |
| 133 |  | F | QPSK | 75@0 | 75@0 |  | 40@0 | 40@0 | 20@0 | 20@0 |
| 144 |  | T | QPSK | 120@0 | 120@0 |  | 60@0 | 60@0 | 30@0 | 30@0 |
| 155 |  | - | QPSK | 150@0 | 150@0 |  | 75@0 | 75@0 | 40@0 | 40@0 |
| 165 |  | s | QPSK | Outer_Full | Outer_Full |  | Outer_Full | Outer_Full | Outer_Full | Outer_Full |
| 17 |  | - | 16QAM | Edge_1RB_Left | Edge_1RB_Right |  | Edge_1RB_Left | Edge_1RB_Right | Edge_1RB_Left | Edge_1RB_Right |
| 18 |  | O | 16QAM | 8@0 | 8@0 |  | 4@0 | 4@0 | 2@0 | 2@0 |
| 19 |  | F | 16QAM | 24@0 | 24@0 |  | 12@0 | 12@0 | 6@0 | 6@0 |
| 20 |  | D | 16QAM | 45@0 | 45@0 |  | 24@0 | 24@0 | 12@0 | 12@0 |
| 213 |  | M | 16QAM | 75@0 | 75@0 |  | 40@0 | 40@0 | 20@0 | 20@0 |
| 224 |  |  | 16QAM | 120@0 | 120@0 |  | 60@0 | 60@0 | 30@0 | 30@0 |
| 235 |  |  | 16QAM | 150@0 | 150@0 |  | 75@0 | 75@0 | 40@0 | 40@0 |
| 245 |  |  | 16QAM | Outer_Full | Outer_Full |  | Outer_Full | Outer_Full | Outer_Full | Outer_Full |
| 25 |  |  | 64QAM | Edge_1RB_Left | Edge_1RB_Right |  | Edge_1RB_Left | Edge_1RB_Right | Edge_1RB_Left | Edge_1RB_Right |
| 26 |  |  | 64QAM | 8@0 | 8@0 |  | 4@0 | 4@0 | 2@0 | 2@0 |
| 27 |  |  | 64QAM | 24@0 | 24@0 |  | 12@0 | 12@0 | 6@0 | 6@0 |
| 28 |  |  | 64QAM | 45@0 | 45@0 |  | 24@0 | 24@0 | 12@0 | 12@0 |
| 293 |  |  | 64QAM | 75@0 | 75@0 |  | 40@0 | 40@0 | 20@0 | 20@0 |
| 304 |  |  | 64QAM | 120@0 | 120@0 |  | 60@0 | 60@0 | 30@0 | 30@0 |
| 315 |  |  | 64QAM | 150@0 | 150@0 |  | 75@0 | 75@0 | 40@0 | 40@0 |
| 325 |  |  | 64QAM | Outer_Full | Outer_Full |  | Outer_Full | Outer_Full | Outer_Full | Outer_Full |
| 33 |  |  | 256QAM | Edge_1RB_Left | Edge_1RB_Right |  | Edge_1RB_Left | Edge_1RB_Right | Edge_1RB_Left | Edge_1RB_Right |
| 34 |  |  | 256QAM | 8@0 | 8@0 |  | 4@0 | 4@0 | 2@0 | 2@0 |
| 35 |  |  | 256QAM | 24@0 | 24@0 |  | 12@0 | 12@0 | 6@0 | 6@0 |
| 36 |  |  | 256QAM | 45@0 | 45@0 |  | 24@0 | 24@0 | 12@0 | 12@0 |
| 373 |  |  | 256QAM | 75@0 | 75@0 |  | 40@0 | 40@0 | 20@0 | 20@0 |
| 384 |  |  | 256QAM | 120@0 | 120@0 |  | 60@0 | 60@0 | 30@0 | 30@0 |
| 395 |  |  | 256QAM | 150@0 | 150@0 |  | 75@0 | 75@0 | 40@0 | 40@0 |
| 405 |  |  | 256QAM | Outer_Full | Outer_Full |  | Outer_Full | Outer_Full | Outer_Full | Outer_Full |
| 41 |  |  | QPSK | Edge_1RB_Left | Edge_1RB_Right |  | Edge_1RB_Left | Edge_1RB_Right | Edge_1RB_Left | Edge_1RB_Right |
| 42 |  |  | QPSK | 8@0 | 8@0 |  | 4@0 | 4@0 | 2@0 | 2@0 |
| 43 |  |  | QPSK | 23@0 | 23@0 |  | 12@0 | 12@0 | 6@0 | 6@0 |
| 44 |  |  | QPSK | 45@0 | 45@0 |  | 23@0 | 23@0 | 12@0 | 12@0 |
| 453 |  |  | QPSK | 75@0 | 75@0 |  | 38@0 | 38@0 | 19@0 | 19@0 |
| 464 |  |  | QPSK | 120@0 | 120@0 |  | 60@0 | 60@0 | 30@0 | 30@0 |
| 475 |  |  | QPSK | 150@0 | 150@0 |  | 75@0 | 75@0 | 38@0 | 38@0 |
| 485 |  |  | QPSK | Outer_Full | Outer_Full |  | Outer_Full | Outer_Full | Outer_Full | Outer_Full |
| 49 |  |  | 16QAM | Edge_1RB_Left | Edge_1RB_Right |  | Edge_1RB_Left | Edge_1RB_Right | Edge_1RB_Left | Edge_1RB_Right |
| 50 |  | C | 16QAM | 8@0 | 8@0 |  | 4@0 | 4@0 | 2@0 | 2@0 |
| 51 |  | P | 16QAM | 23@0 | 23@0 |  | 12@0 | 12@0 | 6@0 | 6@0 |
| 52 |  | - | 16QAM | 45@0 | 45@0 |  | 23@0 | 23@0 | 12@0 | 12@0 |
| 533 |  | O | 16QAM | 75@0 | 75@0 |  | 38@0 | 38@0 | 19@0 | 19@0 |
| 544 |  | F | 16QAM | 120@0 | 120@0 |  | 60@0 | 60@0 | 30@0 | 30@0 |
| 555 |  | D | 16QAM | 150@0 | 150@0 |  | 75@0 | 75@0 | 38@0 | 38@0 |
| 565 |  | M | 16QAM | Outer_Full | Outer_Full |  | Outer_Full | Outer_Full | Outer_Full | Outer_Full |
| 57 |  |  | 64QAM | Edge_1RB_Left | Edge_1RB_Right |  | Edge_1RB_Left | Edge_1RB_Right | Edge_1RB_Left | Edge_1RB_Right |
| 58 |  |  | 64QAM | 8@0 | 8@0 |  | 4@0 | 4@0 | 2@0 | 2@0 |
| 59 |  |  | 64QAM | 23@0 | 23@0 |  | 12@0 | 12@0 | 6@0 | 6@0 |
| 60 |  |  | 64QAM | 45@0 | 45@0 |  | 23@0 | 23@0 | 12@0 | 12@0 |
| 613 |  |  | 64QAM | 75@0 | 75@0 |  | 38@0 | 38@0 | 19@0 | 19@0 |
| 624 |  |  | 64QAM | 120@0 | 120@0 |  | 60@0 | 60@0 | 30@0 | 30@0 |
| 635 |  |  | 64QAM | 150@0 | 150@0 |  | 75@0 | 75@0 | 38@0 | 38@0 |
| 645 |  |  | 64QAM | Outer_Full | Outer_Full |  | Outer_Full | Outer_Full | Outer_Full | Outer_Full |
| 65 |  |  | 256QAM | Edge_1RB_Left | Edge_1RB_Right |  | Edge_1RB_Left | Edge_1RB_Right | Edge_1RB_Left | Edge_1RB_Right |
| 66 |  |  | 256QAM | 8@0 | 8@0 |  | 4@0 | 4@0 | 2@0 | 2@0 |
| 67 |  |  | 256QAM | 23@0 | 23@0 |  | 12@0 | 12@0 | 6@0 | 6@0 |
| 68 |  |  | 256QAM | 45@0 | 45@0 |  | 23@0 | 23@0 | 12@0 | 12@0 |
| 693 |  |  | 256QAM | 75@0 | 75@0 |  | 38@0 | 38@0 | 19@0 | 19@0 |
| 704 |  |  | 256QAM | 120@0 | 120@0 |  | 60@0 | 60@0 | 30@0 | 30@0 |
| 715 |  |  | 256QAM | 150@0 | 150@0 |  | 75@0 | 75@0 | 38@0 | 38@0 |
| 725 |  |  | 256QAM | Outer_Full | Outer_Full |  | Outer_Full | Outer_Full | Outer_Full | Outer_Full |
| NOTE 1: The Test CC Combination settings are checked separately for each CA Configuration, which applicable aggregated channel bandwidths are specified in Table 5.5A.1. NOTE 2: Edge_1RB_Left, Edge_1RB_Right and Outer_Full RB allocation are specified in table 6.1-1 for each component carrier. NOTE 3: Only being tested when the channel bandwidth of both carriers are larger than 10 MHz. NOTE 4: Only being tested when the channel bandwidth of both carriers are larger than 20 MHz. NOTE 5: Only being tested when the channel bandwidth of both carriers are larger than 25 MHz. NOTE 6: DFT-s-OFDM Pi/2 BPSK test applies only for UEs which supports Pi/2 BPSK in FR1. |  |  |  |  |  |  |  |  |  |  |

Table 6.2A.2.1.4.1-3b: Intra-band non-contiguous CA Test Configuration Table (MPRIM3 to meet -30dBm/MHz) when UE supporting IE dualPA-Architecture

| Initial Conditions |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Test Environment as specified in TS 38.508-1 [5] subclause 4.1 |  |  |  |  |  | Normal, TL/VL, TL/VH, TH/VL, TH/VH |  |  |  |  |
| Test Frequencies as specified in TS 38.508-1 [5] subclause4.3.1 |  |  |  |  |  | Low range and High range test frequencies as specified in tables for non-contiguous CA configuration with UL CA |  |  |  |  |
| Test Channel Bandwidths as specified in TS 38.508-1 [5] subclause 4.3.1 |  |  |  |  |  | Lowest NRB_agg, Highest NRB_agg (NOTE 1) |  |  |  |  |
| Test SCS as specified in Table 5.3.5-1 |  |  |  |  |  | Lowest, Highest |  |  |  |  |
| Test Parameters |  |  |  |  |  |  |  |  |  |  |
| Test ID | DL config for PCC & SCC | UL configuration |  |  |  |  |  |  |  |  |
|  |  | Modulations for all CCs (NOTE 2) |  | RB allocation LCRB@RBSTART (NOTE 2) |  |  |  |  |  |  |
|  |  |  |  | SCS 15 kHz |  |  | SCS 30 kHz |  | SCS 60 kHz |  |
|  |  |  |  | PCC | SCC |  | PCC | SCC | PCC | SCC |
| 1 | N/A |  | Pi/2 BPSK | Edge_1RB_Left | Edge_1RB_Right |  | Edge_1RB_Left | Edge_1RB_Right | Edge_1RB_Left | Edge_1RB_Right |
| 2 |  |  | Pi/2 BPSK | 15@0 | 15@0 |  | 8@0 | 8@0 | 4@0 | 4@0 |
| 3 |  |  | Pi/2 BPSK | 45@0 | 45@0 |  | 24@0 | 24@0 | 12@0 | 12@0 |
| 43 |  |  | Pi/2 BPSK | 75@0 | 75@0 |  | 40@0 | 40@0 | 20@0 | 20@0 |
| 54 |  |  | Pi/2 BPSK | 120@0 | 120@0 |  | 60@0 | 60@0 | 30@0 | 30@0 |
| 65 |  |  | Pi/2 BPSK | 216@0 | 216@0 |  | 108@0 | 108@0 | 54@0 | 54@0 |
| 75 |  |  | Pi/2 BPSK | Outer_Full | Outer_Full |  | Outer_Full | Outer_Full | Outer_Full | Outer_Full |
| 8 |  |  | QPSK | Edge_1RB_Left | Edge_1RB_Right |  | Edge_1RB_Left | Edge_1RB_Right | Edge_1RB_Left | Edge_1RB_Right |
| 9 |  |  | QPSK | 15@0 | 15@0 |  | 8@0 | 8@0 | 4@0 | 4@0 |
| 10 |  |  | QPSK | 45@0 | 45@0 |  | 24@0 | 24@0 | 12@0 | 12@0 |
| 113 |  |  | QPSK | 75@0 | 75@0 |  | 40@0 | 40@0 | 20@0 | 20@0 |
| 124 |  | D | QPSK | 120@0 | 120@0 |  | 60@0 | 60@0 | 30@0 | 30@0 |
| 135 |  | F | QPSK | 216@0 | 216@0 |  | 108@0 | 108@0 | 54@0 | 54@0 |
| 145 |  | T | QPSK | Outer_Full | Outer_Full |  | Outer_Full | Outer_Full | Outer_Full | Outer_Full |
| 15 |  | - | 16QAM | Edge_1RB_Left | Edge_1RB_Right |  | Edge_1RB_Left | Edge_1RB_Right | Edge_1RB_Left | Edge_1RB_Right |
| 16 |  | s | 16QAM | 15@0 | 15@0 |  | 8@0 | 8@0 | 4@0 | 4@0 |
| 17 |  | - | 16QAM | 45@0 | 45@0 |  | 24@0 | 24@0 | 12@0 | 12@0 |
| 183 |  | O | 16QAM | 75@0 | 75@0 |  | 40@0 | 40@0 | 20@0 | 20@0 |
| 194 |  | F | 16QAM | 120@0 | 120@0 |  | 60@0 | 60@0 | 30@0 | 30@0 |
| 205 |  | D | 16QAM | 216@0 | 216@0 |  | 108@0 | 108@0 | 54@0 | 54@0 |
| 215 |  | M | 16QAM | Outer_Full | Outer_Full |  | Outer_Full | Outer_Full | Outer_Full | Outer_Full |
| 22 |  |  | 64QAM | Edge_1RB_Left | Edge_1RB_Right |  | Edge_1RB_Left | Edge_1RB_Right | Edge_1RB_Left | Edge_1RB_Right |
| 23 |  |  | 64QAM | 15@0 | 15@0 |  | 8@0 | 8@0 | 4@0 | 4@0 |
| 24 |  |  | 64QAM | 45@0 | 45@0 |  | 24@0 | 24@0 | 12@0 | 12@0 |
| 253 |  |  | 64QAM | 75@0 | 75@0 |  | 40@0 | 40@0 | 20@0 | 20@0 |
| 264 |  |  | 64QAM | 120@0 | 120@0 |  | 60@0 | 60@0 | 30@0 | 30@0 |
| 275 |  |  | 64QAM | 216@0 | 216@0 |  | 108@0 | 108@0 | 54@0 | 54@0 |
| 285 |  |  | 64QAM | Outer_Full | Outer_Full |  | Outer_Full | Outer_Full | Outer_Full | Outer_Full |
| 29 |  |  | 256QAM | Edge_1RB_Left | Edge_1RB_Right |  | Edge_1RB_Left | Edge_1RB_Right | Edge_1RB_Left | Edge_1RB_Right |
| 30 |  |  | 256QAM | 15@0 | 15@0 |  | 8@0 | 8@0 | 4@0 | 4@0 |
| 31 |  |  | 256QAM | 45@0 | 45@0 |  | 24@0 | 24@0 | 12@0 | 12@0 |
| 323 |  |  | 256QAM | 75@0 | 75@0 |  | 40@0 | 40@0 | 20@0 | 20@0 |
| 334 |  |  | 256QAM | 120@0 | 120@0 |  | 60@0 | 60@0 | 30@0 | 30@0 |
| 345 |  |  | 256QAM | 216@0 | 216@0 |  | 108@0 | 108@0 | 54@0 | 54@0 |
| 355 |  |  | 256QAM | Outer_Full | Outer_Full |  | Outer_Full | Outer_Full | Outer_Full | Outer_Full |
| 36 |  |  | QPSK | Edge_1RB_Left | Edge_1RB_Right |  | Edge_1RB_Left | Edge_1RB_Right | Edge_1RB_Left | Edge_1RB_Right |
| 37 |  |  | QPSK | 15@0 | 15@0 |  | 8@0 | 8@0 | 4@0 | 4@0 |
| 38 |  |  | QPSK | 45@0 | 45@0 |  | 23@0 | 23@0 | 12@0 | 12@0 |
| 393 |  |  | QPSK | 75@0 | 75@0 |  | 38@0 | 38@0 | 19@0 | 19@0 |
| 404 |  |  | QPSK | 115@0 | 115@0 |  | 58@0 | 58@0 | 29@0 | 29@0 |
| 415 |  | C | QPSK | 210@0 | 210@0 |  | 105@0 | 105@0 | 53@0 | 53@0 |
| 425 |  | P | QPSK | Outer_Full | Outer_Full |  | Outer_Full | Outer_Full | Outer_Full | Outer_Full |
| 43 |  | - | 16QAM | Edge_1RB_Left | Edge_1RB_Right |  | Edge_1RB_Left | Edge_1RB_Right | Edge_1RB_Left | Edge_1RB_Right |
| 44 |  | O | 16QAM | 15@0 | 15@0 |  | 8@0 | 8@0 | 4@0 | 4@0 |
| 45 |  | F | 16QAM | 45@0 | 45@0 |  | 23@0 | 23@0 | 12@0 | 12@0 |
| 463 |  | D | 16QAM | 75@0 | 75@0 |  | 38@0 | 38@0 | 19@0 | 19@0 |
| 474 |  | M | 16QAM | 115@0 | 115@0 |  | 58@0 | 58@0 | 29@0 | 29@0 |
| 485 |  |  | 16QAM | 210@0 | 210@0 |  | 105@0 | 105@0 | 53@0 | 53@0 |
| 495 |  |  | 16QAM | Outer_Full | Outer_Full |  | Outer_Full | Outer_Full | Outer_Full | Outer_Full |
| 50 |  |  | 64QAM | Edge_1RB_Left | Edge_1RB_Right |  | Edge_1RB_Left | Edge_1RB_Right | Edge_1RB_Left | Edge_1RB_Right |
| 51 |  |  | 64QAM | 15@0 | 15@0 |  | 8@0 | 8@0 | 4@0 | 4@0 |
| 52 |  |  | 64QAM | 45@0 | 45@0 |  | 23@0 | 23@0 | 12@0 | 12@0 |
| 533 |  |  | 64QAM | 75@0 | 75@0 |  | 38@0 | 38@0 | 19@0 | 19@0 |
| 544 |  |  | 64QAM | 115@0 | 115@0 |  | 58@0 | 58@0 | 29@0 | 29@0 |
| 555 |  |  | 64QAM | 210@0 | 210@0 |  | 105@0 | 105@0 | 53@0 | 53@0 |
| 565 |  |  | 64QAM | Outer_Full | Outer_Full |  | Outer_Full | Outer_Full | Outer_Full | Outer_Full |
| 57 |  |  | 256QAM | Edge_1RB_Left | Edge_1RB_Right |  | Edge_1RB_Left | Edge_1RB_Right | Edge_1RB_Left | Edge_1RB_Right |
| 58 |  |  | 256QAM | 15@0 | 15@0 |  | 8@0 | 8@0 | 4@0 | 4@0 |
| 59 |  |  | 256QAM | 45@0 | 45@0 |  | 23@0 | 23@0 | 12@0 | 12@0 |
| 603 |  |  | 256QAM | 75@0 | 75@0 |  | 38@0 | 38@0 | 19@0 | 19@0 |
| 614 |  |  | 256QAM | 115@0 | 115@0 |  | 58@0 | 58@0 | 29@0 | 29@0 |
| 625 |  |  | 256QAM | 210@0 | 210@0 |  | 105@0 | 105@0 | 53@0 | 53@0 |
| 635 |  |  | 256QAM | Outer_Full | Outer_Full |  | Outer_Full | Outer_Full | Outer_Full | Outer_Full |
| NOTE 1: The Test CC Combination settings are checked separately for each CA Configuration, which applicable aggregated channel bandwidths are specified in Table 5.5A.1. NOTE 2: Edge_1RB_Left, Edge_1RB_Right and Outer_Full RB allocation are specified in table 6.1-1 for each component carrier. NOTE 3: Only being tested when the channel bandwidth of both carriers are larger than 10 MHz. NOTE 4: Only being tested when the channel bandwidth of both carriers are larger than 20 MHz. NOTE 5: Only being tested when the channel bandwidth of both carriers are larger than 25 MHz. NOTE 6: DFT-s-OFDM Pi/2 BPSK test applies only for UEs which supports Pi/2 BPSK in FR1. |  |  |  |  |  |  |  |  |  |  |

1.	Connect the SS to the UE antenna connectors as shown in TS 38.508-1 [5] Annex A, Figure A.3.1.1.3 for TE diagram and section A.3.2 for UE diagram.

2.	The parameter settings for the cell are set up according to TS 38.508-1 [5] subclause 4.4.3.

3.	Downlink signals are initially set up according to Annex C.0, C.1, C.2, and uplink signals according to Annex G.0, G.1, G.2, G.3.0.

4.	The UL Reference Measurement Channel is set according to Tables 6.2A.2.1.4.1-1 to 6.2A.2.1.4.1-3b as appropriate.

5.	Propagation conditions are set according to Annex B.0.

6.	Ensure the UE is in State RRC_CONNECTED with generic procedure parameters Connectivity NR, Connected without release On, Test Mode On and Test Loop Function On according to TS 38.508-1 [5] clause 4.5. Message contents are defined in clause 6.2A.2.1.4.3.

6.2A.2.1.4.2	Test procedure

1.	Configure SCC according to Annex C.0, C.1, and C.2 for all downlink physical channels.

2.	The SS shall configure SCC as per TS 38.508-1 [5] clause 5.5.1. Message contents are defined in clause 6.2A.2.1.4.3.

3.	SS activates SCC by sending the activation MAC CE (Refer TS 38.321 [18], clauses 5.9, 6.1.3.10). Wait for at least 2 seconds (Refer TS 38.133[19], clause 9.3).

4.	SS sends uplink scheduling information for each UL HARQ process via PDCCH DCI format 0_1 for C_RNTI to schedule the UL RMC according to Tables 6.2A.2.1.4.1-1 to 6.2A.2.1.4.1-3b on both PCC and SCC as appropriate. Since the UE has no payload and no loopback data to send the UE sends uplink MAC padding bits on the UL RMC.

5.	Send continuously uplink power control "up" commands in every uplink scheduling information to the UE; allow at least 200ms starting from the first TPC command in this step for the UE to reach PUMAX level.

6.	Measure the sum of mean transmitted power over all component carriers in the CA configuration of the radio access mode. The period of measurement shall be at least the continuous duration of 1ms uplink. For TDD only slots consisting of only UL symbols are under test. For FDD band in inter-band CA with both TDD band and FDD band, only slots overlapping with only UL symbols in TDD are under test.

NOTE 1:	When switching to DFT-s-OFDM waveform, as specified in the test configuration table 6.2A.2.1.4.1-1, send an NR RRCReconfiguration message according to TS 38.508-1 [5] clause 4.6.3 Table 4.6.3-118 PUSCH-Config with TRANSFORM_PRECODER_ENABLED condition.

6.2A.2.1.4.3	Message contents

Message contents are according to TS 38.508-1 [5] subclause 4.6 and 5.4.

Table 6.2A.2.1.4.3-1: FrequencyInfoUL-SIB for inter-band CA

| Derivation Path: TS 38.508-1 [5] Table 4.6.3-62 FrequencyInfoUL-SIB |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| p-Max | 20 |  | Power class 3 and Inter-band CA Test IDs 1, 4-13, 16, 19, 23, 26, 27, 30, 37 |
|  | 17 |  | Power class 3 and Inter-band CA Test IDs 14, 15, 17, 18, 33 |
|  | 16 |  | Power class 3 and Inter-band CA Test IDs 2, 3, 22, 24, 25, 28, 29, 31, 32 |
|  | 14 |  | Power class 3 and Inter-band CA Test IDs 20, 21, 36 |
|  | 13 |  | Power class 3 and Inter-band CA Test IDs 34, 35 |

Table 6.2A.2.1.4.3-1a: FrequencyInfoUL-SIB for inter-band CA (Power Class 2 with both bands supporting PC3)

| Derivation Path: TS 38.508-1 [5] Table 4.6.3-62 FrequencyInfoUL-SIB |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| p-Max | 23 |  | Test IDs 5-13,37 |
|  | 22 |  | Test IDs 14, 15, 16, 23, 27 |
|  | 21 |  | Test IDs 17, 18, 19, 24, 25, 26, 28, 29, 30 |
|  | 20 |  | Test IDs 31, 32, 33 |
|  | 19 |  | Test IDs 20, 21, 22 |
|  | 17 |  | Test IDs 34, 35, 36 |

Table 6.2A.2.1.4.3-1b: FrequencyInfoUL-SIB for inter-band CA (PCC supporting PC3 and SCC supporting PC3)

| Derivation Path: TS 38.508-1 [5] Table 4.6.3-62 FrequencyInfoUL-SIB |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| p-Max | 23 |  | Test IDs 5-13, 16, 19, 23, 27, 37 |
|  | 22 |  | Test IDs 14, 15, 17, 18, 24, 25, 26, 28-33 |
|  | 21 |  | Test IDs 20, 21, 22 |
|  | 19 |  | Test IDs 34, 35, 36 |

Table 6.2A.2.1.4.3-2: FrequencyInfoUL-SIB for intra-band contiguous CA (contiguous RB allocation) for CA_n41C, CA_n78C

| Derivation Path: TS 38.508-1 [5] Table 4.6.3-62 FrequencyInfoUL-SIB |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| p-Max | 18 |  | Power class 3 and Test IDs 1, 3 ,5 |
|  | 16 |  | Power class 3 and Test IDs 11, 13 |
|  | 14 |  | Power class 3 and Test IDs 7, 15 |
|  | 11 |  | Power class 3 and Test IDs 2, 4, 6, 8, 9, 17 |
|  | 10 |  | Power class 3 and Test IDs 10, 12, 14, 16, 18 |
|  | 20 |  | Power class 2 and Test IDs 1, 3, 5 |
|  | 19 |  | Power class 2 and Test IDs 11, 13 |
|  | 18 |  | Power class 2 and Test IDs 7, 15 |
|  | 15 |  | Power class 2 and Test IDs 2, 4, 6, 8, 9, 17 |
|  | 14 |  | Power class 2 and Test IDs 10, 12, 14, 16, 18 |

Table 6.2A.2.1.4.3-2a: FrequencyInfoUL-SIB for intra-band contiguous CA (contiguous RB allocation) for CA_n48B

| Derivation Path: TS 38.508-1 [5] Table 4.6.3-62 FrequencyInfoUL-SIB |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| p-Max | 19 |  | Power class 3 and Test IDs 1, 3, 5 |
|  | 18 |  | Power class 3 and Test IDs 5, 11 |
|  | 17 |  | Power class 3 and Test IDs 7, 13 |
|  | 16 |  | Power class 3 and Test IDs 2, 4, 6, 15 |
|  | 15 |  | Power class 3 and Test IDs 8, 12, 14, 16 |
|  | 12 |  | Power class 3 and Test IDs 9, 10 |
|  | 11 |  | Power class 3 and Test ID 17, 18 |

Table 6.2A.2.1.4.3-2b: FrequencyInfoUL-SIB for intra-band contiguous CA (contiguous RB allocation) for CA_n77C

| Derivation Path: TS 38.508-1 [5] Table 4.6.3-62 FrequencyInfoUL-SIB |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| p-Max | 17 |  | Power class 3 and Test IDs 1, 3 ,5 |
|  | 16 |  | Power class 3 and Test IDs 11, 13 |
|  | 14 |  | Power class 3 and Test IDs 7, 15 |
|  | 11 |  | Power class 3 and Test IDs 2, 4, 6, 8, 9, 17 |
|  | 10 |  | Power class 3 and Test IDs 10, 12, 14, 16, 18 |

6.2A.2.1.5	Test requirement

The maximum output power, derived in step 6 shall be within the range prescribed by the nominal maximum output power and tolerance in Table 6.2A.2.1.5-1 to Table 6.2A.2.1.5-3.

Table 6.2A.2.1.5-1: UE Output Power for inter-band CA (2 UL CA) test requirements

| Test ID | PPowerClass (dBm) | MPRc (dB) |  | ΔTC,c (dB) |  | TL (dB) | PCMAX_L (dBm) | TLOW(PCMAX_L) (dB) | Upper limit (dBm) | Lower limit (dBm) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | PCC | SCC | PCC | SCC |  |  |  |  |  |
| 1 | 23 | 0.2 | 0.2 | 0 (1.52) | 0 (1.52) | 3 | 23 | 3 | 25+TT | 20-TT |
| 2 | 23 | 3.5 | 3.5 | 1.52 | 1.52 | 3 | 212 | 52 | 25+TT | 16-TT2 |
| 3 | 23 | 3.5 | 3.5 | 1.52 | 1.52 | 3 | 212 | 52 | 25+TT | 16-TT2 |
| 4 | 23 | 1.2 | 1.2 | 0 | 0 | 3 | 23 | 3 | 25+TT | 20-TT |
| 5 | 23 | 0 | 0 | 0 (1.52) | 0 (1.52) | 3 | 23 | 3 | 25+TT | 20-TT |
| 6 | 23 | 0.5 | 0.5 | 1.52 | 1.52 | 3 | 23 | 3 | 25+TT | 20-TT |
| 7 | 23 | 0.5 | 0.5 | 1.52 | 1.52 | 3 | 23 | 3 | 25+TT | 20-TT |
| 8 | 23 | 0.5 | 0.5 | 0 | 0 | 3 | 23 | 3 | 25+TT | 20-TT |
| 9 | 23 | 0 | 0 | 0 (1.52) | 0 (1.52) | 3 | 23 | 3 | 25+TT | 20-TT |
| 10 | 23 | 1 | 1 | 1.52 | 1.52 | 3 | 23 | 3 | 25+TT | 20-TT |
| 11 | 23 | 1 | 1 | 1.52 | 1.52 | 3 | 23 | 3 | 25+TT | 20-TT |
| 12 | 23 | 1 | 1 | 0 | 0 | 3 | 23 | 3 | 25+TT | 20-TT |
| 13 | 23 | 1 | 1 | 0 (1.52) | 0 (1.52) | 3 | 23 | 3 | 25+TT | 20-TT |
| 14 | 23 | 2 | 2 | 1.52 | 1.52 | 3 | 22.52 | 52 | 25+TT | 17.5-TT2 |
| 15 | 23 | 2 | 2 | 1.52 | 1.52 | 3 | 22.52 | 52 | 25+TT | 17.5-TT2 |
| 16 | 23 | 2 | 2 | 0 | 0 | 3 | 23 | 3 | 25+TT | 20-TT |
| 17 | 23 | 2.5 | 2.5 | 1.52 | 1.52 | 3 | 22.52 | 52 | 25+TT | 17.5-TT2 |
| 18 | 23 | 2.5 | 2.5 | 1.52 | 1.52 | 3 | 22.52 | 52 | 25+TT | 17.5-TT2 |
| 19 | 23 | 2.5 | 2.5 | 0 | 0 | 3 | 23 | 3 | 25+TT | 20-TT |
| 20 | 23 | 4.5 | 4.5 | 1.52 | 1.52 | 3 | 202 | 62 | 25+TT | 14-TT2 |
| 21 | 23 | 4.5 | 4.5 | 1.52 | 1.52 | 3 | 202 | 62 | 25+TT | 14-TT2 |
| 22 | 23 | 4.5 | 4.5 | 0 | 0 | 3 | 21.5 | 5 | 25+TT | 16.5-TT |
| 23 | 23 | 1.5 | 1.5 | 0 (1.52) | 0 (1.52) | 3 | 23 | 3 | 25+TT | 20-TT |
| 24 | 23 | 3 | 3 | 1.52 | 1.52 | 3 | 21.52 | 52 | 25+TT | 16.5-TT2 |
| 25 | 23 | 3 | 3 | 1.52 | 1.52 | 3 | 21.52 | 52 | 25+TT | 16.5-TT2 |
| 26 | 23 | 3 | 3 | 0 | 0 | 3 | 23 | 3 | 25+TT | 20-TT |
| 27 | 23 | 2 | 2 | 0 (1.52) | 0 (1.52) | 3 | 23 (22.52) | 3 (52) | 25+TT | 20-TT (17.5-TT2) |
| 28 | 23 | 3 | 3 | 1.52 | 1.52 | 3 | 21.52 | 52 | 25+TT | 16.5-TT2 |
| 29 | 23 | 3 | 3 | 1.52 | 1.52 | 3 | 21.52 | 52 | 25+TT | 16.5-TT2 |

| Test ID | PPowerClass (dBm) | MPRc (dB) |  | ΔTC,c (dB) |  | TL (dB) | PCMAX_L (dBm) | TLOW(PCMAX_L) (dB) | Upper limit (dBm) | Lower limit (dBm) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | PCC | SCC | PCC | SCC |  |  |  |  |  |
| 30 | 23 | 3 | 3 | 0 | 0 | 3 | 23 | 3 | 25+TT | 20-TT |
| 31 | 23 | 3.5 | 3.5 | 1.52 | 1.52 | 3 | 212 | 52 | 25+TT | 16-TT2 |
| 32 | 23 | 3.5 | 3.5 | 1.52 | 1.52 | 3 | 212 | 52 | 25+TT | 16-TT2 |
| 33 | 23 | 3.5 | 3.5 | 0 | 0 | 3 | 22.5 | 5 2 | 25+TT | 17.5-TT |
| 34 | 23 | 6.5 | 6.5 | 1.52 | 1.52 | 3 | 182 | 52 | 25+TT | 13-TT2 |
| 35 | 23 | 6.5 | 6.5 | 1.52 | 1.52 | 3 | 182 | 52 | 25+TT | 13-TT2 |
| 36 | 23 | 6.5 | 6.5 | 0 | 0 | 3 | 19.5 | 5 | 25+TT | 14.5-TT |
| 37 | 23 | 0 | 6.5 | 0 | 0 | 3 | 23 | 3 | 25+TT | 20-TT |
| NOTE 1: PPowerClass is the maximum UE power specified without taking into account the tolerance. NOTE 2: For transmission bandwidths confined within FUL_low and FUL_low + 4 MHz or FUL_high – 4 MHz and FUL_high. NOTE 3: TT for each frequency and channel bandwidth is specified in Table 6.2A.2.1.5-2. |  |  |  |  |  |  |  |  |  |  |

Table 6.2A.2.1.5-1a: UE Output Power for intra-band contiguous 2 UL CA of PC3 (contiguous RB allocation) test requirements (for CA_n41C)

| Configura | Power | MPR |  | TC | PCMAX_L |  | TL=2 | T(PCMAX_L) |  | Bandwidth class B |  | Bandwidth class C |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| tion ID | Class 3 (dBm) | BW class B | BW class C |  | BW class B | BW class C |  | BW class B | BW class C | Upper limit (dBm) | Lower limit (dBm) | Upper limit (dBm) | Lower limit (dBm) |
| 1 | 23 | 1 | 2.5 | 0 | 22 | 20.5 | 2 | 2 | 2.5 | 25+TT | 20-TT | 25+TT | 18-TT |
| 2 | 23 | 3.5 | 7 | 0 | 19.5 | 16 | 2 | 3.5 | 5 | 25+TT | 16-TT | 25+TT | 11-TT |
| 3 | 23 | 1 | 2.5 | 0 | 22 | 20.5 | 2 | 2 | 2.5 | 25+TT | 20-TT | 25+TT | 18-TT |
| 4 | 23 | 3.5 | 7 | 0 | 19.5 | 16 | 2 | 3.5 | 5 | 25+TT | 16-TT | 25+TT | 11-TT |
| 5 | 23 | 1.5 | 2.5 | 0 | 21.5 | 20.5 | 2 | 2 | 2.5 | 25+TT | 19.5-TT | 25+TT | 18-TT |
| 6 | 23 | 3.5 | 7 | 0 | 19.5 | 16 | 2 | 3.5 | 5 | 25+TT | 16-TT | 25+TT | 11-TT |
| 7 | 23 | 3 | 5 | 0 | 20 | 18 | 2 | 2.5 | 4 | 25+TT | 17.5-TT | 25+TT | 14-TT |
| 8 | 23 | 4 | 7 | 0 | 19 | 16 | 2 | 3.5 | 5 | 25+TT | 15.5-TT | 25+TT | 11-TT |
| 9 | 23 | 5.5 | 7 | 0 | 17.5 | 16 | 2 | 5 | 5 | 25+TT | 12.5-TT | 25+TT | 11-TT |
| 10 | 23 | 6 | 7.5 | 0 | 17 | 15.5 | 2 | 5 | 5 | 25+TT | 12-TT | 25+TT | 10.5-TT |
| 11 | 23 | 2 | 3.5 | 0 | 21 | 19.5 | 2 | 2 | 3.5 | 25+TT | 19-TT | 25+TT | 16-TT |
| 12 | 23 | 4 | 8 | 0 | 19 | 15 | 2 | 3.5 | 5 | 25+TT | 15.5-TT | 25+TT | 10-TT |
| 13 | 23 | 2.5 | 3.5 | 0 | 20.5 | 19.5 | 2 | 2.5 | 3.5 | 25+TT | 18-TT | 25+TT | 16-TT |
| 14 | 23 | 4 | 8 | 0 | 19 | 15 | 2 | 3.5 | 5 | 25+TT | 15.5-TT | 25+TT | 10-TT |
| 15 | 23 | 3.5 | 5 | 0 | 19.5 | 18 | 2 | 3.5 | 4 | 25+TT | 16-TT | 25+TT | 14-TT |
| 16 | 23 | 4 | 8 | 0 | 19 | 15 | 2 | 3.5 | 5 | 25+TT | 15.5-TT | 25+TT | 10-TT |
| 17 | 23 | 6.5 | 7 | 0 | 16.5 | 16 | 2 | 5 | 5 | 25+TT | 11.5-TT | 25+TT | 11-TT |
| 18 | 23 | 6.5 | 8 | 0 | 16.5 | 15 | 2 | 5 | 5 | 25+TT | 11.5-TT | 25+TT | 10-TT |
| NOTE 1: TT for each frequency and channel bandwidth is specified in Table 6.2A.2.1.5-3. |  |  |  |  |  |  |  |  |  |  |  |  |  |

Table 6.2A.2.1.5-1b: UE Output Power for intra-band contiguous 2 UL CA of PC3 (contiguous RB allocation) test requirements (for CA_n48B, CA_n77C)

| Configura | Power | MPR |  | TC | PCMAX_L |  | TL=2 | T(PCMAX_L) |  | Bandwidth class B |  | Bandwidth class C |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| tion ID | Class 3 (dBm) | BW class B | BW class C |  | BW class B | BW class C |  | BW class B | BW class C | Upper limit (dBm) | Lower limit (dBm) | Upper limit (dBm) | Lower limit (dBm) |
| 1 | 23 | 1 | 2.5 | 0 | 22 | 20.5 | 3 | 2 | 2.5 | 25+TT | 19-TT | 25+TT | 17.5-TT |
| 2 | 23 | 3.5 | 7 | 0 | 19.5 | 16 | 3 | 3.5 | 5 | 25+TT | 16-TT | 25+TT | 11-TT |
| 3 | 23 | 1 | 2.5 | 0 | 22 | 20.5 | 3 | 2 | 2.5 | 25+TT | 19-TT | 25+TT | 17.5-TT |
| 4 | 23 | 3.5 | 7 | 0 | 19.5 | 16 | 3 | 3.5 | 5 | 25+TT | 16-TT | 25+TT | 11-TT |
| 5 | 23 | 1.5 | 2.5 | 0 | 21.5 | 20.5 | 3 | 2 | 2.5 | 25+TT | 18.5-TT | 25+TT | 17.5-TT |
| 6 | 23 | 3.5 | 7 | 0 | 19.5 | 16 | 3 | 3.5 | 5 | 25+TT | 16-TT | 25+TT | 11-TT |
| 7 | 23 | 3 | 5 | 0 | 20 | 18 | 3 | 2.5 | 4 | 25+TT | 17-TT | 25+TT | 14-TT |
| 8 | 23 | 4 | 7 | 0 | 19 | 16 | 3 | 3.5 | 5 | 25+TT | 15.5-TT | 25+TT | 11-TT |
| 9 | 23 | 5.5 | 7 | 0 | 17.5 | 16 | 3 | 5 | 5 | 25+TT | 12.5-TT | 25+TT | 11-TT |
| 10 | 23 | 6 | 7.5 | 0 | 17 | 15.5 | 3 | 5 | 5 | 25+TT | 12-TT | 25+TT | 10.5-TT |
| 11 | 23 | 2 | 3.5 | 0 | 21 | 19.5 | 3 | 2 | 3.5 | 25+TT | 18-TT | 25+TT | 16-TT |
| 12 | 23 | 4 | 8 | 0 | 19 | 15 | 3 | 3.5 | 5 | 25+TT | 15.5-TT | 25+TT | 10-TT |
| 13 | 23 | 2.5 | 3.5 | 0 | 20.5 | 19.5 | 3 | 2.5 | 3.5 | 25+TT | 17.5-TT | 25+TT | 16-TT |
| 14 | 23 | 4 | 8 | 0 | 19 | 15 | 3 | 3.5 | 5 | 25+TT | 15.5-TT | 25+TT | 10-TT |
| 15 | 23 | 3.5 | 5 | 0 | 19.5 | 18 | 3 | 3.5 | 4 | 25+TT | 16-TT | 25+TT | 14-TT |
| 16 | 23 | 4 | 8 | 0 | 19 | 15 | 3 | 3.5 | 5 | 25+TT | 15.5-TT | 25+TT | 10-TT |
| 17 | 23 | 6.5 | 7 | 0 | 16.5 | 16 | 3 | 5 | 5 | 25+TT | 11.5-TT | 25+TT | 11-TT |
| 18 | 23 | 6.5 | 8 | 0 | 16.5 | 15 | 3 | 5 | 5 | 25+TT | 11.5-TT | 25+TT | 10-TT |
| NOTE 1: TT for each frequency and channel bandwidth is specified in Table 6.2A.2.1.5-3. |  |  |  |  |  |  |  |  |  |  |  |  |  |

Table 6.2A.2.1.5-1c: UE Output Power for intra-band contiguous 2 UL CA of PC3 (non-contiguous RB allocation) test requirements (for CA_n41C)

| Configura | Power | MPR |  | TC | PCMAX_L |  | TL | T(PCMAX_L) |  | Bandwidth class B |  | Bandwidth class C |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| tion ID | Class 3 (dBm) | BW class B | BW class C |  | BW class B | BW class C |  | BW class B | BW class C | Upper limit (dBm) | Lower limit (dBm) | Upper limit (dBm) | Lower limit (dBm) |
| 1 | 23 | 2 | 2.5 | 0 | 21 | 20.5 | 2 | 2 | 2.5 | 25+TT | 19-TT | 25+TT | 18-TT |
| 2 | 23 | 5.5 (3.5)1 | 6 (4)1 | 0 | 17.5 (19.5)1 | 17 (19)1 | 2 | 5 (3.5)1 | 5 (3.5)1 | 25+TT | 12.5-TT (16-TT)1 | 25+TT | 12-TT (15.5-TT)1 |
| 3 | 23 | 11.5 (7)2 | 13 (8.5)2 | 0 | 11.5 (16)2 | 10 (14.5)2 | 2 | 6 (5)2 | 6 (5)2 | 25+TT | 5.5-TT (11-TT)2 | 25+TT | 4-TT (9.5-TT)2 |
| 4 | 23 | 2 | 2.5 | 0 | 21 | 20.5 | 2 | 2 | 2.5 | 25+TT | 19-TT | 25+TT | 18-TT |
| 5 | 23 | 5.5 (3.5)1 | 6 (4)1 | 0 | 17.5 (19.5)1 | 17 (19)1 | 2 | 5 (3.5)1 | 5 (3.5)1 | 25+TT | 12.5-TT (16-TT)1 | 25+TT | 12-TT (15.5-TT)1 |
| 6 | 23 | 11.5 (7)2 | 13 (8.5)2 | 0 | 11.5 (16)2 | 10 (14.5)2 | 2 | 6 (5)2 | 6 (5)2 | 25+TT | 5.5-TT (11-TT)2 | 25+TT | 4-TT (9.5-TT)2 |
| 7 | 23 | 2.5 | 3 | 0 | 20.5 | 20 | 2 | 2.5 | 2.5 | 25+TT | 18-TT | 25+TT | 17.5-TT |
| 8 | 23 | 5.5 | 6 | 0 | 17.5 | 17 | 2 | 5 | 5 | 25+TT | 12.5-TT | 25+TT | 12-TT |
| 9 | 23 | 11.5 (7)2 | 13 (8.5)2 | 0 | 11.5 (16)2 | 10 (14.5)2 | 2 | 6 (5)2 | 6 (5)2 | 25+TT | 5.5-TT (11-TT)2 | 25+TT | 4-TT (9.5-TT)2 |
| 10 | 23 | 4.5 | 5 | 0 | 18.5 | 18 | 2 | 4 | 4 | 25+TT | 14.5-TT | 25+TT | 14-TT |
| 11 | 23 | 6 | 6 | 0 | 17 | 17 | 2 | 5 | 5 | 25+TT | 12-TT | 25+TT | 12-TT |
| 12 | 23 | 11.5 (7)2 | 13 (8.5)2 | 0 | 11.5 (16)2 | 10 (14.5)2 | 2 | 6 (5)2 | 6 (5)2 | 25+TT | 5.5-TT (11-TT)2 | 25+TT | 4-TT (9.5-TT)2 |
| 13 | 23 | 6 | 6.5 | 0 | 17 | 16.5 | 2 | 5 | 5 | 25+TT | 12-TT | 25+TT | 11.5-TT |
| 14 | 23 | 6.5 | 6.5 | 0 | 16.5 | 16.5 | 2 | 5 | 5 | 25+TT | 11.5-TT | 25+TT | 11.5-TT |
| 15 | 23 | 11.5 (7)2 | 13 (8.5)2 | 0 | 11.5 (16)2 | 10 (14.5)2 | 2 | 6 (5)2 | 6 (5)2 | 25+TT | 5.5-TT (11-TT)2 | 25+TT | 4-TT (9.5-TT)2 |
| 16 | 23 | 2.5 | 3.5 | 0 | 20.5 | 19.5 | 2 | 2.5 | 3.5 | 25+TT | 18-TT | 25+TT | 16-TT |
| 17 | 23 | 6.5 (4.5)1 | 7 (5)1 | 0 | 16.5 (18.5)1 | 16 (18)1 | 2 | 5 (4)1 | 5 (4)1 | 25+TT | 11.5-TT (14.5-TT)1 | 25+TT | 11-TT (14-TT)1 |
| 18 | 23 | 12 (7.5)2 | 14 (9.5)2 | 0 | 11 (15.5)2 | 9 (13.5)2 | 2 | 6 (5)2 | 6 (5)2 | 25+TT | 5-TT (10.5-TT)2 | 25+TT | 3-TT (8.5-TT)2 |
| 19 | 23 | 3 | 3.5 | 0 | 20 | 19.5 | 2 | 2.5 | 3.5 | 25+TT | 17.5-TT | 25+TT | 16-TT |
| 20 | 23 | 7 | 7 | 0 | 16 | 16 | 2 | 5 | 5 | 25+TT | 11-TT | 25+TT | 11-TT |
| 21 | 23 | 12 (7.5)2 | 14 (9.5)2 | 0 | 11 (15.5)2 | 9 (13.5)2 | 2 | 6 (5)2 | 6 (5)2 | 25+TT | 5-TT (10.5-TT)2 | 25+TT | 3-TT (8.5-TT)2 |
| 22 | 23 | 5 | 5 | 0 | 18 | 18 | 2 | 4 | 4 | 25+TT | 14-TT | 25+TT | 14-TT |
| 23 | 23 | 7 | 7 | 0 | 16 | 16 | 2 | 5 | 5 | 25+TT | 11-TT | 25+TT | 11-TT |
| 24 | 23 | 12 (7.5)2 | 14 (9.5)2 | 0 | 11 (15.5)2 | 9 (13.5)2 | 2 | 6 (5)2 | 6 (5)2 | 25+TT | 5-TT (10.5-TT)2 | 25+TT | 3-TT (8.5-TT)2 |
| 25 | 23 | 7.5 | 7.5 | 0 | 15.5 | 15.5 | 2 | 5 | 5 | 25+TT | 10.5-TT | 25+TT | 10.5-TT |
| 26 | 23 | 7.5 | 7.5 | 0 | 15.5 | 15.5 | 2 | 5 | 5 | 25+TT | 10.5-TT | 25+TT | 10.5-TT |
| 27 | 23 | 12 (7.5)2 | 14 (9.5)2 | 0 | 11 (15.5)2 | 9 (13.5)2 | 2 | 6 (5)2 | 6 (5)2 | 25+TT | 5-TT (10.5-TT)2 | 25+TT | 3-TT (8.5-TT)2 |
| NOTE 1: Outer 1 MPR for Pi/2 BPSK and QPSK is reduced by 2dB for aggregated allocation bandwidth > 10MHz NOTE 2: Outer 2 MPR is reduced by 4.5dB for aggregated allocation bandwidth > 10MHz NOTE 3: TT for each frequency and channel bandwidth is specified in Table 6.2A.2.1.5-3. |  |  |  |  |  |  |  |  |  |  |  |  |  |

Table 6.2A.2.1.5-1d: UE Output Power for intra-band contiguous 2 UL CA of PC3 (non-contiguous RB allocation) test requirements (for CA_n48B, CA_n77C)

| Configura | Power | MPR |  | TC | PCMAX_L |  | TL | T(PCMAX_L) |  | Bandwidth class B |  | Bandwidth class C |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| tion ID | Class 3 (dBm) | BW class B | BW class C |  | BW class B | BW class C |  | BW class B | BW class C | Upper limit (dBm) | Lower limit (dBm) | Upper limit (dBm) | Lower limit (dBm) |
| 1 | 23 | 2 | 2.5 | 0 | 21 | 20.5 | 3 | 2 | 2.5 | 25+TT | 18-TT | 25+TT | 17.5-TT |
| 2 | 23 | 5.5 (3.5)1 | 6 (4)1 | 0 | 17.5 (19.5)1 | 17 (19)1 | 3 | 5 (3.5)1 | 5 (3.5)1 | 25+TT | 12.5-TT (16-TT)1 | 25+TT | 12-TT (15.5-TT)1 |
| 3 | 23 | 11.5 (7)2 | 13 (8.5)2 | 0 | 11.5 (16)2 | 10 (14.5)2 | 3 | 6 (5)2 | 6 (5)2 | 25+TT | 5.5-TT (11-TT)2 | 25+TT | 4-TT (9.5-TT)2 |
| 4 | 23 | 2 | 2.5 | 0 | 21 | 20.5 | 3 | 2 | 2.5 | 25+TT | 18-TT | 25+TT | 17.5-TT |
| 5 | 23 | 5.5 (3.5)1 | 6 (4)1 | 0 | 17.5 (19.5)1 | 17 (19)1 | 3 | 5 (3.5)1 | 5 (3.5)1 | 25+TT | 12.5-TT (16-TT)1 | 25+TT | 12-TT (15.5-TT)1 |
| 6 | 23 | 11.5 (7)2 | 13 (8.5)2 | 0 | 11.5 (16)2 | 10 (14.5)2 | 3 | 6 (5)2 | 6 (5)2 | 25+TT | 5.5-TT (11-TT)2 | 25+TT | 4-TT (9.5-TT)2 |
| 7 | 23 | 2.5 | 3 | 0 | 20.5 | 20 | 3 | 2.5 | 2.5 | 25+TT | 17.5-TT | 25+TT | 17-TT |
| 8 | 23 | 5.5 | 6 | 0 | 17.5 | 17 | 3 | 5 | 5 | 25+TT | 12.5-TT | 25+TT | 12-TT |
| 9 | 23 | 11.5 (7)2 | 13 (8.5)2 | 0 | 11.5 (16)2 | 10 (14.5)2 | 3 | 6 (5)2 | 6 (5)2 | 25+TT | 5.5-TT (11-TT)2 | 25+TT | 4-TT (9.5-TT)2 |
| 10 | 23 | 4.5 | 5 | 0 | 18.5 | 18 | 3 | 4 | 4 | 25+TT | 14.5-TT | 25+TT | 14-TT |
| 11 | 23 | 6 | 6 | 0 | 17 | 17 | 3 | 5 | 5 | 25+TT | 12-TT | 25+TT | 12-TT |
| 12 | 23 | 11.5 (7)2 | 13 (8.5)2 | 0 | 11.5 (16)2 | 10 (14.5)2 | 3 | 6 (5)2 | 6 (5)2 | 25+TT | 5.5-TT (11-TT)2 | 25+TT | 4-TT (9.5-TT)2 |
| 13 | 23 | 6 | 6.5 | 0 | 17 | 16.5 | 3 | 5 | 5 | 25+TT | 12-TT | 25+TT | 11.5-TT |
| 14 | 23 | 6.5 | 6.5 | 0 | 16.5 | 16.5 | 3 | 5 | 5 | 25+TT | 11.5-TT | 25+TT | 11.5-TT |
| 15 | 23 | 11.5 (7)2 | 13 (8.5)2 | 0 | 11.5 (16)2 | 10 (14.5)2 | 3 | 6 (5)2 | 6 (5)2 | 25+TT | 5.5-TT (11-TT)2 | 25+TT | 4-TT (9.5-TT)2 |
| 16 | 23 | 2.5 | 3.5 | 0 | 20.5 | 19.5 | 3 | 2.5 | 3.5 | 25+TT | 17.5-TT | 25+TT | 16-TT |
| 17 | 23 | 6.5 (4.5)1 | 7 (5)1 | 0 | 16.5 (18.5)1 | 16 (18)1 | 3 | 5 (4)1 | 5 (4)1 | 25+TT | 11.5-TT (14.5-TT)1 | 25+TT | 11-TT (14-TT)1 |
| 18 | 23 | 12 (7.5)2 | 14 (9.5)2 | 0 | 11 (15.5)2 | 9 (13.5)2 | 3 | 6 (5)2 | 6 (5)2 | 25+TT | 5-TT (10.5-TT)2 | 25+TT | 3-TT (8.5-TT)2 |
| 19 | 23 | 3 | 3.5 | 0 | 20 | 19.5 | 3 | 2.5 | 3.5 | 25+TT | 17-TT | 25+TT | 16-TT |
| 20 | 23 | 7 | 7 | 0 | 16 | 16 | 3 | 5 | 5 | 25+TT | 11-TT | 25+TT | 11-TT |
| 21 | 23 | 12 (7.5)2 | 14 (9.5)2 | 0 | 11 (15.5)2 | 9 (13.5)2 | 3 | 6 (5)2 | 6 (5)2 | 25+TT | 5-TT (10.5-TT)2 | 25+TT | 3-TT (8.5-TT)2 |
| 22 | 23 | 5 | 5 | 0 | 18 | 18 | 3 | 4 | 4 | 25+TT | 14-TT | 25+TT | 14-TT |
| 23 | 23 | 7 | 7 | 0 | 16 | 16 | 3 | 5 | 5 | 25+TT | 11-TT | 25+TT | 11-TT |
| 24 | 23 | 12 (7.5)2 | 14 (9.5)2 | 0 | 11 (15.5)2 | 9 (13.5)2 | 3 | 6 (5)2 | 6 (5)2 | 25+TT | 5-TT (10.5-TT)2 | 25+TT | 3-TT (8.5-TT)2 |
| 25 | 23 | 7.5 | 7.5 | 0 | 15.5 | 15.5 | 3 | 5 | 5 | 25+TT | 10.5-TT | 25+TT | 10.5-TT |
| 26 | 23 | 7.5 | 7.5 | 0 | 15.5 | 15.5 | 3 | 5 | 5 | 25+TT | 10.5-TT | 25+TT | 10.5-TT |
| 27 | 23 | 12 (7.5)2 | 14 (9.5)2 | 0 | 11 (15.5)2 | 9 (13.5)2 | 3 | 6 (5)2 | 6 (5)2 | 25+TT | 5-TT (10.5-TT)2 | 25+TT | 3-TT (8.5-TT)2 |
| NOTE 1: Outer 1 MPR for Pi/2 BPSK and QPSK is reduced by 2dB for aggregated allocation bandwidth > 10MHz NOTE 2: Outer 2 MPR is reduced by 4.5dB for aggregated allocation bandwidth > 10MHz NOTE 3: TT for each frequency and channel bandwidth is specified in Table 6.2A.2.1.5-3. |  |  |  |  |  |  |  |  |  |  |  |  |  |

Table 6.2A.2.1.5-1e: UE Output Power for intra-band non-contiguous 2 UL CA of PC3 test requirements (MPRIM3 to meet -13dBm/MHz)

| Test ID | PPowerClass，CA (dBm) | MPR (dB) | ΔTC (dB) | PCMAX_L (dBm) | T(PCMAX_L) (dB) | TL (dB) | Upper limit (dBm) | Lower limit (dBm) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1,2 | 23 | 9 | 0 | 14 | 5 | 3 | 25+TT | 9-TT |
| 3 | 23 | 8 | 0 | 15 | 5 | 3 | 25+TT | 10-TT |
| 4 | 23 | 7 | 0 | 16 | 5 | 3 | 25+TT | 11-TT |
| 5 | 23 | 6.5 | 0 | 16.5 | 5 | 3 | 25+TT | 11.5-TT |
| 6 | 23 | 5.5 | 0 | 17.5 | 5 | 3 | 25+TT | 12.5-TT |
| 7,8 | 23 | 4 | 0 | 19 | 3.5 | 3 | 25+TT | 16.5-TT |
| 9,10 | 23 | 9 | 0 | 14 | 5 | 3 | 25+TT | 9-TT |
| 11 | 23 | 8 | 0 | 15 | 5 | 3 | 25+TT | 10-TT |
| 12 | 23 | 7 | 0 | 16 | 5 | 3 | 25+TT | 11-TT |
| 13 | 23 | 6.5 | 0 | 16.5 | 5 | 3 | 25+TT | 11.5-TT |
| 14 | 23 | 5.5 | 0 | 17.5 | 5 | 3 | 25+TT | 12.5-TT |
| 15,16 | 23 | 4 | 0 | 19 | 3.5 | 3 | 25+TT | 16.5-TT |
| 17,18 | 23 | 9 | 0 | 14 | 5 | 3 | 25+TT | 9-TT |
| 19 | 23 | 8 | 0 | 15 | 5 | 3 | 25+TT | 10-TT |
| 20 | 23 | 7 | 0 | 16 | 5 | 3 | 25+TT | 11-TT |
| 21 | 23 | 6.5 | 0 | 16.5 | 5 | 3 | 25+TT | 11.5-TT |
| 22 | 23 | 5.5 | 0 | 17.5 | 5 | 3 | 25+TT | 12.5-TT |
| 23,24 | 23 | 4 | 0 | 19 | 3.5 | 3 | 25+TT | 16.5-TT |
| 25,26 | 23 | 9 | 0 | 14 | 5 | 3 | 25+TT | 9-TT |
| 27 | 23 | 8 | 0 | 15 | 5 | 3 | 25+TT | 10-TT |
| 28 | 23 | 7 | 0 | 16 | 5 | 3 | 25+TT | 11-TT |
| 29 | 23 | 6.5 | 0 | 16.5 | 5 | 3 | 25+TT | 11.5-TT |
| 30 | 23 | 5.5 | 0 | 17.5 | 5 | 3 | 25+TT | 12.5-TT |
| 31,32 | 23 | 4 | 0 | 19 | 3.5 | 3 | 25+TT | 16.5-TT |
| 33,34 | 23 | 9 | 0 | 14 | 5 | 3 | 25+TT | 9-TT |
| 35 | 23 | 8 | 0 | 15 | 5 | 3 | 25+TT | 10-TT |
| 36 | 23 | 7 | 0 | 16 | 5 | 3 | 25+TT | 11-TT |
| 37 | 23 | 6.5 | 0 | 16.5 | 5 | 3 | 25+TT | 11.5-TT |
| 38 | 23 | 5.5 | 0 | 17.5 | 5 | 3 | 25+TT | 12.5-TT |
| 39,40 | 23 | 4 | 0 | 19 | 3.5 | 3 | 25+TT | 16.5-TT |
| 41,42 | 23 | 9 | 0 | 14 | 5 | 3 | 25+TT | 9-TT |
| 43 | 23 | 8 | 0 | 15 | 5 | 3 | 25+TT | 10-TT |
| 44 | 23 | 7 | 0 | 16 | 5 | 3 | 25+TT | 11-TT |
| 45 | 23 | 6.5 | 0 | 16.5 | 5 | 3 | 25+TT | 11.5-TT |
| 46 | 23 | 5.5 | 0 | 17.5 | 5 | 3 | 25+TT | 12.5-TT |
| 47,48 | 23 | 4 | 0 | 19 | 3.5 | 3 | 25+TT | 16.5-TT |
| 49,50 | 23 | 9 | 0 | 14 | 5 | 3 | 25+TT | 9-TT |
| 51 | 23 | 8 | 0 | 15 | 5 | 3 | 25+TT | 10-TT |
| 52 | 23 | 7 | 0 | 16 | 5 | 3 | 25+TT | 11-TT |
| 53 | 23 | 6.5 | 0 | 16.5 | 5 | 3 | 25+TT | 11.5-TT |
| 54 | 23 | 5.5 | 0 | 17.5 | 5 | 3 | 25+TT | 12.5-TT |
| 55,56 | 23 | 4 | 0 | 19 | 3.5 | 3 | 25+TT | 16.5-TT |
| 57,58 | 23 | 9 | 0 | 14 | 5 | 3 | 25+TT | 9-TT |
| 59 | 23 | 8 | 0 | 15 | 5 | 3 | 25+TT | 10-TT |
| 60 | 23 | 7 | 0 | 16 | 5 | 3 | 25+TT | 11-TT |
| 61 | 23 | 6.5 | 0 | 16.5 | 5 | 3 | 25+TT | 11.5-TT |
| 62 | 23 | 5.5 | 0 | 17.5 | 5 | 3 | 25+TT | 12.5-TT |
| 63,64 | 23 | 4 | 0 | 19 | 3.5 | 3 | 25+TT | 16.5-TT |
| 65,66 | 23 | 9 | 0 | 14 | 5 | 3 | 25+TT | 9-TT |
| 67 | 23 | 8 | 0 | 15 | 5 | 3 | 25+TT | 10-TT |
| 68 | 23 | 7 | 0 | 16 | 5 | 3 | 25+TT | 11-TT |
| 69 | 23 | 6.5 | 0 | 16.5 | 5 | 3 | 25+TT | 11.5-TT |
| 70 | 23 | 5.5 | 0 | 17.5 | 5 | 3 | 25+TT | 12.5-TT |
| 71,72 | 23 | 4 | 0 | 19 | 3.5 | 3 | 25+TT | 16.5-TT |

Table 6.2A.2.1.5-1f: UE Output Power for intra-band non-contiguous 2 UL CA of PC3 test requirements (MPRIM3 to meet - 30dBm/MHz)

| Test ID | PPowerClass，CA (dBm) | MPR (dB) | ΔTC (dB) | PCMAX_L (dBm) | T(PCMAX_L) (dB) | TL (dB) | Upper limit (dBm) | Lower limit (dBm) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1,2 | 23 | 15 | 0 | 8 | 6 | 3 | 25+TT | 2-TT |
| 3 | 23 | 14.5 | 0 | 8.5 | 6 | 3 | 25+TT | 2.5-TT |
| 4 | 23 | 13.5 | 0 | 9.5 | 6 | 3 | 25+TT | 3.5-TT |
| 5 | 23 | 12.5 | 0 | 10.5 | 6 | 3 | 25+TT | 4.5-TT |
| 6,7 | 23 | 11.5 | 0 | 11.5 | 6 | 3 | 25+TT | 5.5-TT |
| 8,9 | 23 | 15 | 0 | 8 | 6 | 3 | 25+TT | 2-TT |
| 10 | 23 | 14.5 | 0 | 8.5 | 6 | 3 | 25+TT | 2.5-TT |
| 11 | 23 | 13.5 | 0 | 9.5 | 6 | 3 | 25+TT | 3.5-TT |
| 12 | 23 | 12.5 | 0 | 10.5 | 6 | 3 | 25+TT | 4.5-TT |
| 13,14 | 23 | 11.5 | 0 | 11.5 | 6 | 3 | 25+TT | 5.5-TT |
| 15,16 | 23 | 15 | 0 | 8 | 6 | 3 | 25+TT | 2-TT |
| 17 | 23 | 14.5 | 0 | 8.5 | 6 | 3 | 25+TT | 2.5-TT |
| 18 | 23 | 13.5 | 0 | 9.5 | 6 | 3 | 25+TT | 3.5-TT |
| 19 | 23 | 12.5 | 0 | 10.5 | 6 | 3 | 25+TT | 4.5-TT |
| 20,21 | 23 | 11.5 | 0 | 11.5 | 6 | 3 | 25+TT | 5.5-TT |
| 22,23 | 23 | 15 | 0 | 8 | 6 | 3 | 25+TT | 2-TT |
| 24 | 23 | 14.5 | 0 | 8.5 | 6 | 3 | 25+TT | 2.5-TT |
| 25 | 23 | 13.5 | 0 | 9.5 | 6 | 3 | 25+TT | 3.5-TT |
| 26 | 23 | 12.5 | 0 | 10.5 | 6 | 3 | 25+TT | 4.5-TT |
| 27,28 | 23 | 11.5 | 0 | 11.5 | 6 | 3 | 25+TT | 5.5-TT |
| 29,30 | 23 | 15 | 0 | 8 | 6 | 3 | 25+TT | 2-TT |
| 31 | 23 | 14.5 | 0 | 8.5 | 6 | 3 | 25+TT | 2.5-TT |
| 32 | 23 | 13.5 | 0 | 9.5 | 6 | 3 | 25+TT | 3.5-TT |
| 33 | 23 | 12.5 | 0 | 10.5 | 6 | 3 | 25+TT | 4.5-TT |
| 34,35 | 23 | 11.5 | 0 | 11.5 | 6 | 3 | 25+TT | 5.5-TT |
| 36,37 | 23 | 15 | 0 | 8 | 6 | 3 | 25+TT | 2-TT |
| 38 | 23 | 14.5 | 0 | 8.5 | 6 | 3 | 25+TT | 2.5-TT |
| 39 | 23 | 13.5 | 0 | 9.5 | 6 | 3 | 25+TT | 3.5-TT |
| 40 | 23 | 12.5 | 0 | 10.5 | 6 | 3 | 25+TT | 4.5-TT |
| 41,42 | 23 | 11.5 | 0 | 11.5 | 6 | 3 | 25+TT | 5.5-TT |
| 43,44 | 23 | 15 | 0 | 8 | 6 | 3 | 25+TT | 2-TT |
| 45 | 23 | 14.5 | 0 | 8.5 | 6 | 3 | 25+TT | 2.5-TT |
| 46 | 23 | 13.5 | 0 | 9.5 | 6 | 3 | 25+TT | 3.5-TT |
| 47 | 23 | 12.5 | 0 | 10.5 | 6 | 3 | 25+TT | 4.5-TT |
| 48,49 | 23 | 11.5 | 0 | 11.5 | 6 | 3 | 25+TT | 5.5-TT |
| 50,51 | 23 | 15 | 0 | 8 | 6 | 3 | 25+TT | 2-TT |
| 52 | 23 | 14.5 | 0 | 8.5 | 6 | 3 | 25+TT | 2.5-TT |
| 53 | 23 | 13.5 | 0 | 9.5 | 6 | 3 | 25+TT | 3.5-TT |
| 54 | 23 | 12.5 | 0 | 10.5 | 6 | 3 | 25+TT | 4.5-TT |
| 55,56 | 23 | 11.5 | 0 | 11.5 | 6 | 3 | 25+TT | 5.5-TT |
| 57,58 | 23 | 15 | 0 | 8 | 6 | 3 | 25+TT | 2-TT |
| 59 | 23 | 14.5 | 0 | 8.5 | 6 | 3 | 25+TT | 2.5-TT |
| 60 | 23 | 13.5 | 0 | 9.5 | 6 | 3 | 25+TT | 3.5-TT |
| 61 | 23 | 12.5 | 0 | 10.5 | 6 | 3 | 25+TT | 4.5-TT |
| 62,63 | 23 | 11.5 | 0 | 11.5 | 6 | 3 | 25+TT | 5.5-TT |

Table 6.2A.2.1.5-2: Test Tolerance for inter-band CA (Maximum Power Reduction (MPR))

| TT for overall output power (dB) |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | PCell |  |  |  |  |  |
|  |  |  | BW ≤ 40MHz |  |  | 40MHz < BW ≤ 100MHz |  |  |
|  |  |  | f ≤ 3.0GHz | 3.0GHz < f ≤ 4.2GHz | 4.2GHz < f ≤ 6.0GHz | f ≤ 3.0GHz | 3.0GHz < f ≤ 4.2GHz | 4.2GHz < f ≤ 6.0GHz |
| SCell |  | f ≤ 3.0GHz | 0.7 | 1.0 | 1.0 | 1.0 | 1.0 | 1.0 |
|  | BW ≤ 40MHz | 3.0GHz < f ≤ 4.2GHz | 1.0 | 1.0 | 1.0 | 1.0 | 1.0 | 1.0 |
|  |  | 4.2GHz < f ≤ 6.0GHz | 1.0 | 1.0 | 1.0 | 1.0 | 1.0 | 1.0 |
|  |  | f ≤ 3.0GHz | 1.0 | 1.0 | 1.0 | 1.0 | 1.0 | 1.0 |
|  | 40MHz < BW ≤ 100MHz | 3.0GHz < f ≤ 4.2GHz | 1.0 | 1.0 | 1.0 | 1.0 | 1.0 | 1.0 |
|  |  | 4.2GHz < f ≤ 6.0GHz | 1.0 | 1.0 | 1.0 | 1.0 | 1.0 | 1.0 |

Table 6.2A.2.1.5-3: Test Tolerance for intra-band CA (Maximum Power Reduction (MPR))

| TT for overall output power (dB) |  |  |
| --- | --- | --- |
| Aggregation BW | f ≤ 3.0GHz | 3.0GHz < f ≤ 6.0GHz |
| BW ≤ 40MHz | 0.7 | 1.0 |
| 40MHz < BW ≤ 100MHz | 1.0 | 1.0 |
| 100MHz < BW ≤ 200MHz | FFS | FFS |

Table 6.2A.2.1.5-4: UE Output Power for intra-band contiguous 2 UL CA of PC2 (contiguous RB allocation) test requirements (for CA_n41C, CA_n78C)

| Configura | Power | MPR |  | TC | PCMAX_L |  | TL=2 | T(PCMAX_L) |  | Bandwidth class B |  | Bandwidth class C |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| tion ID | Class 2 (dBm) | BW class B | BW class C |  | BW class B | BW class C |  | BW class B | BW class C | Upper limit (dBm) | Lower limit (dBm) | Upper limit (dBm) | Lower limit (dBm) |
| 1 | 26 | 2.0 | 2.5 | 0 | 24 | 23.5 | 3 | 3 | 3 | 28+TT | 21.-TT | 28+TT | 20.5-TT |
| 2 | 26 | 4.0 | 7.0 | 0 | 22 | 19.0 | 3 | 2 | 3.5 | 28+TT | 19.-TT | 28+TT | 15.5-TT |
| 3 | 26 | 2.0 | 2.5 | 0 | 24 | 23.5 | 3 | 3 | 3 | 28+TT | 21.-TT | 28+TT | 20.5-TT |
| 4 | 26 | 4.0 | 7.0 | 0 | 22 | 19.0 | 3 | 2 | 3.5 | 28+TT | 19.-TT | 28+TT | 15.5-TT |
| 5 | 26 | 2.5 | 2.5 | 0 | 23.5 | 23.5 | 3 | 3 | 3 | 28+TT | 20.5-TT | 28+TT | 20.5-TT |
| 6 | 26 | 4.0 | 7.0 | 0 | 22 | 19.0 | 3 | 2 | 3.5 | 28+TT | 19.-TT | 28+TT | 15.5-TT |
| 7 | 26 | 3.0 | 5.0 | 0 | 23 | 21.0 | 3 | 2 | 2 | 25+TT | 20.-TT | 25+TT | 18.-TT |
| 8 | 26 | 4.5 | 7.0 | 0 | 21.5 | 19.0 | 3 | 2 | 3.5 | 25+TT | 18.5-TT | 25+TT | 15.5-TT |
| 9 | 26 | 5.5 | 7.0 | 0 | 20.5 | 19.0 | 3 | 2.5 | 3.5 | 25+TT | 17.5-TT | 25+TT | 15.5-TT |
| 10 | 26 | 6.0 | 7.5 | 0 | 20 | 18.5 | 3 | 2.5 | 4 | 25+TT | 17.-TT | 25+TT | 14.5-TT |
| 11 | 26 | 2.5 | 3.5 | 0 | 23.5 | 22.5 | 3 | 3 | 2 | 25+TT | 20.5-TT | 25+TT | 19.5-TT |
| 12 | 26 | 5.0 | 8.0 | 0 | 21 | 18.0 | 3 | 2 | 4 | 25+TT | 18.-TT | 25+TT | 14.-TT |
| 13 | 26 | 3.0 | 3.5 | 0 | 23 | 22.5 | 3 | 2 | 2 | 25+TT | 20.-TT | 25+TT | 19.5-TT |
| 14 | 26 | 5.0 | 8.0 | 0 | 21 | 18.0 | 3 | 2 | 4 | 25+TT | 18.-TT | 25+TT | 14.-TT |
| 15 | 26 | 3.5 | 5.0 | 0 | 22.5 | 21.0 | 3 | 2 | 2 | 25+TT | 19.5-TT | 25+TT | 18.-TT |
| 16 | 26 | 5.0 | 8.0 | 0 | 21 | 18.0 | 3 | 2 | 4 | 25+TT | 18.-TT | 25+TT | 14.-TT |
| 17 | 26 | 6.5 | 7.0 | 0 | 19.5 | 19.0 | 3 | 3.5 | 3.5 | 25+TT | 16.-TT | 25+TT | 15.5-TT |
| 18 | 26 | 6.5 | 8.0 | 0 | 19.5 | 18.0 | 3 | 3.5 | 4 | 25+TT | 16.-TT | 25+TT | 14.-TT |
| NOTE 1: TT for each frequency and channel bandwidth is specified in Table 6.2A.2.1.5-3. |  |  |  |  |  |  |  |  |  |  |  |  |  |

Table 6.2A.2.1.5-5: UE Output Power for intra-band contiguous 2 UL CA of PC2 with TxD (contiguous RB allocation) test requirements (for CA_n41C, CA_n78C)

FFS

Table 6.2A.2.1.5-6: UE Output Power for intra-band contiguous 2 UL CA of PC2 (non-contiguous RB allocation) test requirements (for CA_n41C)

| Configura | Power | MPR |  | TC | PCMAX_L |  | TL | T(PCMAX_L) |  | Bandwidth class B |  | Bandwidth class C |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| tion ID | Class 2 (dBm) | BW class B | BW class C |  | BW class B | BW class C |  | BW class B | BW class C | Upper limit (dBm) | Lower limit (dBm) | Upper limit (dBm) | Lower limit (dBm) |
| 1 | 26 | 3.0 | 3 | 0 | 23.0 | 23.0 | 3 | 2 | 2 | 28+TT | 20.-TT | 28+TT | 20.-TT |
| 2 | 26 | 6.5 (4.5)1 | 7.5 (5.5)1 | 0 | 19.5 (21.5)1 | 18.5 (20.5)1 | 3 | 3.5 (2)1 | 4 (2.5)1 | 28+TT | 16.-TT (18.5-TT)1 | 28+TT | 14.5-TT (17.5-TT)1 |
| 3 | 26 | 13.0 (8.5)2 | 13.5 (9.0)2 | 0 | 13 (17.5)2 | 12.5 (17.0)2 | 3 | 5 (5)2 | 6 (5)2 | 28+TT | 8.-TT (12.5-TT)2 | 28+TT | 6.5-TT (12.0-TT)2 |
| 4 | 26 | 3.0 | 3.0 | 0 | 23 | 23.0 | 3 | 2 | 2 | 28+TT | 20.-TT | 28+TT | 20.-TT |
| 5 | 26 | 6.5 (4.5)1 | 7.5 (5.5)1 | 0 | 19.5 (21.5)1 | 18.5 (20.5)1 | 3 | 3.5 (2)1 | 4 (2.5)1 | 28+TT | 16.-TT (18.5-TT)1 | 28+TT | 14.5-TT (17.5-TT)1 |
| 6 | 26 | 13.0 (8.5)2 | 13.5 (9.0)2 | 0 | 13 (17.5)2 | 12.5 (17.0)2 | 3 | 5 (5)2 | 6 (5)2 | 28+TT | 8.-TT (12.5-TT)2 | 28+TT | 6.5-TT (12.0-TT)2 |
| 7 | 26 | 3.0 | 3.0 | 0 | 23 | 23.0 | 3 | 2 | 2 | 28+TT | 20.-TT | 28+TT | 20.-TT |
| 8 | 26 | 6.5 | 7.5 | 0 | 19.5 | 18.5 | 3 | 3.5 | 4 | 28+TT | 16.-TT | 28+TT | 14.5-TT |
| 9 | 26 | 13.0 (8.5)2 | 13.5 (9.0)2 | 0 | 13 (17.5)2 | 12.5 (17.0)2 | 3 | 5 (5)2 | 6 (5)2 | 28+TT | 8.-TT (12.5-TT)2 | 28+TT | 6.5-TT (12.0-TT)2 |
| 10 | 26 | 5.0 | 5.0 | 0 | 21 | 21.0 | 3 | 2 | 2 | 28+TT | 18.-TT | 28+TT | 18.-TT |
| 11 | 26 | 6.5 | 7.5 | 0 | 19.5 | 18.5 | 3 | 3.5 | 4 | 28+TT | 16.-TT | 28+TT | 14.5-TT |
| 12 | 26 | 13.0 (8.5)2 | 13.5 (9.0)2 | 0 | 13 (17.5)2 | 12.5 (17.0)2 | 3 | 5 (5)2 | 6 (5)2 | 28+TT | 8.-TT (12.5-TT)2 | 28+TT | 6.5-TT (12.0-TT)2 |
| 13 | 26 | 6.5 | 6.5 | 0 | 19.5 | 19.5 | 3 | 3.5 | 3.5 | 28+TT | 16.-TT | 28+TT | 16.-TT |
| 14 | 26 | 7.0 | 7.5 | 0 | 19 | 18.5 | 3 | 3.5 | 4 | 28+TT | 15.5-TT | 28+TT | 14.5-TT |
| 15 | 26 | 13.0 (8.5)2 | 13.5 (9.0)2 | 0 | 13 (17.5)2 | 12.5 (17.0)2 | 3 | 5 (5)2 | 6 (5)2 | 28+TT | 8.-TT (12.5-TT)2 | 28+TT | 6.5-TT (12.0-TT)2 |
| 16 | 26 | 3.5 | 3.5 | 0 | 22.5 | 22.5 | 3 | 2 | 2 | 28+TT | 19.5-TT | 28+TT | 19.5-TT |
| 17 | 26 | 7.0 (5.0)1 | 8.0 (6.0)1 | 0 | 19 (21.0)1 | 18.0 (20.0)1 | 3 | 3.5 (2)1 | 4 (2.5)1 | 28+TT | 15.5-TT (18.0-TT)1 | 28+TT | 14.-TT (17.0-TT)1 |
| 18 | 26 | 14.0 (9.5)2 | 14.5 (10.0)2 | 0 | 12 (16.5)2 | 11.5 (16.0)2 | 3 | 6 (5)2 | 6 (5)2 | 28+TT | 6.-TT (11.5-TT)2 | 28+TT | 5.5-TT (11.0-TT)2 |
| 19 | 26 | 3.5 | 3.5 | 0 | 22.5 | 22.5 | 3 | 2 | 2 | 28+TT | 19.5-TT | 28+TT | 19.5-TT |
| 20 | 26 | 7.0 | 8.0 | 0 | 19 | 18.0 | 3 | 3.5 | 4 | 28+TT | 15.5-TT | 28+TT | 14.-TT |
| 21 | 26 | 14.0 (9.5)2 | 14.5 (10.0)2 | 0 | 12 (16.5)2 | 11.5 (16.0)2 | 3 | 6 (5)2 | 6 (5)2 | 28+TT | 6.-TT (11.5-TT)2 | 28+TT | 5.5-TT (11.0-TT)2 |
| 22 | 26 | 5.0 | 5.0 | 0 | 21 | 21.0 | 3 | 2 | 2 | 28+TT | 18.-TT | 28+TT | 18.-TT |
| 23 | 26 | 7.0 | 8.0 | 0 | 19 | 18.0 | 3 | 3.5 | 4 | 28+TT | 15.5-TT | 28+TT | 14.-TT |
| 24 | 26 | 14.0 (9.5)2 | 14.5 (10.0)2 | 0 | 12 (16.5)2 | 11.5 (16.0)2 | 3 | 6 (5)2 | 6 (5)2 | 28+TT | 6.-TT (11.5-TT)2 | 28+TT | 5.5-TT (11.0-TT)2 |
| 25 | 26 | 7.5 | 7.5 | 0 | 18.5 | 18.5 | 3 | 4 | 4 | 28+TT | 14.5-TT | 28+TT | 14.5-TT |
| 26 | 26 | 7.5 | 8.0 | 0 | 18.5 | 18.0 | 3 | 4 | 4 | 28+TT | 14.5-TT | 28+TT | 14.-TT |
| 27 | 26 | 14.0 (9.5)2 | 14.5 (10.0)2 | 0 | 12 (16.5)2 | 11.5 (16.0)2 | 3 | 6 (5)2 | 6 (5)2 | 28+TT | 6.-TT (11.5-TT)2 | 28+TT | 5.5-TT (11.0-TT)2 |
| NOTE 1: Outer 1 MPR for Pi/2 BPSK and QPSK is reduced by 2dB for aggregated allocation bandwidth > 10MHz NOTE 2: Outer 2 MPR is reduced by 4.5dB for aggregated allocation bandwidth > 10MHz NOTE 3: TT for each frequency and channel bandwidth is specified in Table 6.2A.2.1.5-3. |  |  |  |  |  |  |  |  |  |  |  |  |  |

Table 6.2A.2.1.5-7: UE Output Power for intra-band contiguous 2 UL CA of PC2 with TxD (non-contiguous RB allocation) test requirements (for CA_n41C, CA_n78C)

FFS

Table 6.2A.2.1.5-8: UE Output Power for inter-band CA (2 UL CA) of PC2 test requirements (both bands supporting PC3)

| Test ID | PPowerClass (dBm) | MPRc (dB) |  | ΔTC,c (dB) |  | TL (dB) | PCMAX_L (dBm) | TLOW(PCMAX_L) (dB) | Upper limit (dBm) | Lower limit (dBm) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | PCC | SCC | PCC | SCC |  |  |  |  |  |
| 5 | 26 | 0 | 0 | 0 (1.52) | 0 (1.52) | 3 | 26.0 (24.52) | 3 | 28+TT | 23-TT (21.5-TT2) |
| 6 | 26 | 0.5 | 0.5 | 1.52 | 1.52 | 3 | 24.0 | 3 | 28+TT | 21-TT |
| 7 | 26 | 0.5 | 0.5 | 1.52 | 1.52 | 3 | 24.0 | 3 | 28+TT | 21-TT |
| 8 | 26 | 0.5 | 0.5 | 0 | 0 | 3 | 25.5 | 3 | 28+TT | 22.5-TT |
| 9 | 26 | 0 | 0 | 0 (1.52) | 0 (1.52) | 3 | 26.0 (24.52) | 3 | 28+TT | 23-TT (21.5-TT2) |
| 10 | 26 | 1 | 1 | 1.52 | 1.52 | 3 | 23.5 | 3 | 28+TT | 20.5-TT |
| 11 | 26 | 1 | 1 | 1.52 | 1.52 | 3 | 23.5 | 3 | 28+TT | 20.5-TT |
| 12 | 26 | 1 | 1 | 0 | 0 | 3 | 25.0 | 3 | 28+TT | 22-TT |
| 13 | 26 | 1 | 1 | 0 (1.52) | 0 (1.52) | 3 | 25.0 (23.52) | 3 | 28+TT | 22-TT (20.5-TT2) |
| 14 | 26 | 2 | 2 | 1.52 | 1.52 | 3 | 22.5 | 5 | 28+TT | 17.5-TT |
| 15 | 26 | 2 | 2 | 1.52 | 1.52 | 3 | 22.5 | 5 | 28+TT | 17.5-TT |
| 16 | 26 | 2 | 2 | 0 | 0 | 3 | 24.0 | 3 | 28+TT | 21-TT |
| 17 | 26 | 2.5 | 2.5 | 1.52 | 1.52 | 3 | 22.0 | 5 | 28+TT | 17-TT2 |
| 18 | 26 | 2.5 | 2.5 | 1.52 | 1.52 | 3 | 22.0 | 5 | 28+TT | 17-T |
| 19 | 26 | 2.5 | 2.5 | 0 | 0 | 3 | 23.5 | 3 | 28+TT | 20.5-TT |
| 20 | 26 | 4.5 | 4.5 | 1.52 | 1.52 | 3 | 20.0 | 6 | 28+TT | 15-TT |
| 21 | 26 | 4.5 | 4.5 | 1.52 | 1.52 | 3 | 20.0 | 6 | 28+TT | 15-TT |
| 22 | 26 | 4.5 | 4.5 | 0 | 0 | 3 | 21.5 | 5 | 28+TT | 18.5-TT |
| 23 | 26 | 1.5 | 1.5 | 0 (1.52) | 0 (1.52) | 3 | 24.5 (232) | 3 | 28+TT | 21.5-TT (20-TT2) |
| 24 | 26 | 3 | 3 | 1.52 | 1.52 | 3 | 21.5 | 5 | 28+TT | 16.5-TT |
| 25 | 26 | 3 | 3 | 1.52 | 1.52 | 3 | 21.5 | 5 | 28+TT | 16.5-TT |
| 26 | 26 | 3 | 3 | 0 | 0 | 3 | 23.0 | 3 | 28+TT | 20-TT |
| 27 | 26 | 2 | 2 | 0 (1.52) | 0 (1.52) | 3 | 24.0 (22.52) | 3 (52) | 28+TT | 21-TT (17.5-TT2) |
| 28 | 26 | 3 | 3 | 1.52 | 1.52 | 3 | 21.5 | 5 | 28+TT | 16.5-TT |
| 29 | 26 | 3 | 3 | 1.52 | 1.52 | 3 | 21.5 | 5 | 28+TT | 16.5-TT |
| 30 | 26 | 3 | 3 | 0 | 0 | 3 | 23.0 | 3 | 28+TT | 20-TT |
| 31 | 26 | 3.5 | 3.5 | 1.52 | 1.52 | 3 | 21.0 | 5 | 28+TT | 16-TT |
| 32 | 26 | 3.5 | 3.5 | 1.52 | 1.52 | 3 | 21.0 | 5 | 28+TT | 16-TT |
| 33 | 26 | 3.5 | 3.5 | 0 | 0 | 3 | 22.5 | 5 | 28+TT | 19.5-TT |
| 34 | 26 | 6.5 | 6.5 | 1.52 | 1.52 | 3 | 18.0 | 5 | 28+TT | 13-TT |
| 35 | 26 | 6.5 | 6.5 | 1.52 | 1.52 | 3 | 18.0 | 5 | 28+TT | 13-TT |
| 36 | 26 | 6.5 | 6.5 | 0 | 0 | 3 | 19.5 | 5 | 28+TT | 14.5-TT |
| 37 | 26 | 0 | 6.5 | 0 | 0 | 3 | 23.9 | 3 | 28+TT | 20.9-TT |
| NOTE 1: PPowerClass is the maximum UE power specified without taking into account the tolerance. NOTE 2: For transmission bandwidths confined within FUL_low and FUL_low + 4 MHz or FUL_high – 4 MHz and FUL_high. NOTE 3: TT for each frequency and channel bandwidth is specified in Table 6.2A.2.1.5-2. |  |  |  |  |  |  |  |  |  |  |

Table 6.2A.2.1.5-8a: UE Output Power for inter-band CA (2 UL CA) of PC2 test requirements (PCC supporting PC3 and SCC supporting PC2)

| Test ID | PPowerClass (dBm) | MPRc (dB) |  | ΔTC,c (dB) |  | TL (dB) | PCMAX_L (dBm) | TLOW(PCMAX_L) (dB) | Upper limit (dBm) | Lower limit (dBm) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | PCC | SCC | PCC | SCC |  |  |  |  |  |
| 5 | 26 | 0 | 0 | 0 (1.52) | 0 (1.52) | 3 | 26.0 | 3 | 28+TT | 23-TT |
| 6 | 26 | 0.5 | 3.5 | 1.52 | 1.52 | 3 | 24.0 | 3 | 28+TT | 21-TT |
| 7 | 26 | 0.5 | 3.5 | 1.52 | 1.52 | 3 | 24.0 | 3 | 28+TT | 21-TT |
| 8 | 26 | 0.5 | 0.5 | 0 | 0 | 3 | 26.0 | 3 | 28+TT | 23-TT |
| 9 | 26 | 0 | 0 | 0 (1.52) | 0 (1.52) | 3 | 26.0 | 3 | 28+TT | 23-TT |
| 10 | 26 | 1 | 3.5 | 1.52 | 1.52 | 3 | 23.8 | 3 | 28+TT | 20.8-TT |
| 11 | 26 | 1 | 3.5 | 1.52 | 1.52 | 3 | 23.8 | 3 | 28+TT | 20.8-TT |
| 12 | 26 | 1 | 1 | 0 | 0 | 3 | 26.0 | 3 | 28+TT | 23-TT |
| 13 | 26 | 1 | 1 | 0 (1.52) | 0 (1.52) | 3 | 26.0 (25.32) | 3 | 28+TT | 23-TT (22.3-TT2) |
| 14 | 26 | 2 | 3.5 | 1.52 | 1.52 | 3 | 23.3 | 3 | 28+TT | 20.3-TT |
| 15 | 26 | 2 | 3.5 | 1.52 | 1.52 | 3 | 23.3 | 3 | 28+TT | 20.3-TT |
| 16 | 26 | 2 | 2 | 0 | 0 | 3 | 25.8 | 3 | 28+TT | 22.8-TT |
| 17 | 26 | 2.5 | 3.5 | 1.52 | 1.52 | 3 | 23.1 | 3 | 28+TT | 20.1-TT |
| 18 | 26 | 2.5 | 3.5 | 1.52 | 1.52 | 3 | 23.1 | 3 | 28+TT | 20.1-TT |
| 19 | 26 | 2.5 | 2.5 | 0 | 0 | 3 | 25.3 | 3 | 28+TT | 22.3-TT |
| 20 | 26 | 4.5 | 4.5 | 1.52 | 1.52 | 3 | 21.8 | 5 | 28+TT | 16.8-TT |
| 21 | 26 | 4.5 | 4.5 | 1.52 | 1.52 | 3 | 21.8 | 5 | 28+TT | 16.8-TT |
| 22 | 26 | 4.5 | 4.5 | 0 | 0 | 3 | 23.3 | 3 | 28+TT | 20.3-TT |
| 23 | 26 | 1.5 | 1.5 | 0 (1.52) | 0 (1.52) | 3 | 26.0 (24.82) | 3 | 28+TT | 23-TT (21.8-TT2) |
| 24 | 26 | 3 | 3.5 | 1.52 | 1.52 | 3 | 22.9 | 5 | 28+TT | 17.9-TT |
| 25 | 26 | 3 | 3.5 | 1.52 | 1.52 | 3 | 22.9 | 5 | 28+TT | 17.9-TT |
| 26 | 26 | 3 | 3 | 0 | 0 | 3 | 24.8 | 3 | 28+TT | 21.8-TT |
| 27 | 26 | 2 | 2 | 0 (1.52) | 0 (1.52) | 3 | 25.8 (24.32) | 3 | 28+TT | 22.8-TT (21.3-TT2) |
| 28 | 26 | 3 | 3.5 | 1.52 | 1.52 | 3 | 22.9 | 5 | 28+TT | 17.9-TT |
| 29 | 26 | 3 | 3.5 | 1.52 | 1.52 | 3 | 22.9 | 5 | 28+TT | 17.9-TT |
| 30 | 26 | 3 | 3 | 0 | 0 | 3 | 24.8 | 3 | 28+TT | 21.8-TT |
| 31 | 26 | 3.5 | 3.5 | 1.52 | 1.52 | 3 | 22.8 | 5 | 28+TT | 17.8-TT |
| 32 | 26 | 3.5 | 3.5 | 1.52 | 1.52 | 3 | 22.8 | 5 | 28+TT | 17.8-TT |
| 33 | 26 | 3.5 | 3.5 | 0 | 0 | 3 | 24.3 | 3 | 28+TT | 21.3-TT |
| 34 | 26 | 6.5 | 6.5 | 1.52 | 1.52 | 3 | 19.8 | 5 | 28+TT | 14.8-TT |
| 35 | 26 | 6.5 | 6.5 | 1.52 | 1.52 | 3 | 19.8 | 5 | 28+TT | 14.8-TT |
| 36 | 26 | 6.5 | 6.5 | 0 | 0 | 3 | 21.3 | 5 | 28+TT | 16.3-TT |
| 37 | 26 | 0 | 6.5 | 0 | 0 | 3 | 24.6 | 3 | 28+TT | 21.6-TT |
| NOTE 1: PPowerClass is the maximum UE power specified without taking into account the tolerance. NOTE 2: For transmission bandwidths confined within FUL_low and FUL_low + 4 MHz or FUL_high – 4 MHz and FUL_high. NOTE 3: TT for each frequency and channel bandwidth is specified in Table 6.2A.2.1.5-2. |  |  |  |  |  |  |  |  |  |  |

For the UE which supports inter-band NR CA configuration, inter-band NR-DC configuration, SUL configuration or inter-band EN-DC configuration, ΔTIB,c as specified in 6.2A.4.0.2 for NR CA, 6.2B.4.0.2 for NR-DC, clause 6.2C.2 for SUL, or TS 38.521-3 [14] clause 6.2B.4.2 for EN-DC applies. Unless otherwise stated, ΔTIB,c is set to zero. In case the UE supports more than one of band combinations for CA, NR-DC, SUL or EN-DC, and an operating band belongs to more than one band combinations then

a)	When the operating band frequency range is ≤ 1 GHz, the applicable additional ΔTIB,c shall be the average value for all band combinations defined in clause 6.2A.4.0.2, 6.2B.4.0.2, 6.2C.2 in this specification and 6.2B.4.2 in TS 38.521-3 [14], truncated to one decimal place that apply for that operating band among the supported band combinations. In case there is a harmonic relation between low band UL and high band DL, then the maximum ΔTIB,c among the different supported band combinations involving such band shall be applied

b)	When the operating band frequency range is > 1 GHz, the applicable additional ΔTIB,c shall be the maximum value for all band combinations defined in clause 6.2A.4.0.2, 6.2B.4.0.2, 6.2C.2 in this specification and 6.2B.4.2 in TS 38.521-3 [14] for the applicable operating bands.

<End of Changes>