3GPP TSG-RAN5 Meeting #100	R5-234863

# Toulouse, France, August 21st – 25th, 2023

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.521-1 | CR | 2412 | rev | - | Current version: | 17.9.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Addition of additional test frequencies for AMPR NS_50 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, HiSilicon, CMCC |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_UE_PC2_n39-UEConTest |  |  |  |  |  | Date: |  |  | 2023-08-01 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | The test configuration of AMPR NS_50 is specified with test frequencies other than default low/mid/high ranges. The test frequency configuration needs to be specified for these additional test frequencies. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Adding test frequencie configurations for AMPR NS_50. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The test case might not be executed correctly. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.2.3 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  |  | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  |  | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  |  | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

<Start of Changes>

6.2.3	UE additional maximum output power reduction

Editor’s note: The following aspects are either missing or not yet determined:

- Tests for network signalling values NS_40, NS_09 not complete.

- The requirements of this test apply in test case 6.5.3.3 Additional Spurious Emissions for network signalling values NS_07, NS_44, NS_46, NS_47, NS_48, and NS_49 to all types of NR Power Class 2 and 3 UE release 16 forward, and UE release 15 if the corresponding channel bandwidths are supported.

6.2.3.1	Test purpose

Additional emission requirements can be signalled by the network. Each additional emission requirement is associated with a unique network signalling (NS) value indicated in RRC signalling by an NR frequency band number of the applicable operating band and an associated value in the field additionalSpectrumEmission. Throughout this specification, the notion of indication or signalling of an NS value refers to the corresponding indication of an NR frequency band number of the applicable operating band, the IE field freqBandIndicatorNR and an associated value of additionalSpectrumEmission in the relevant RRC information elements [6].

<Unchanged Sections Skipped>

6.2.3.3.19	A-MPR for NS_50

Table 6.2.3.3.19-1: A-MPR regions for NS_50 (Power Class 3)

| Channel Bandwidth (MHz) | RBstart*12*SCS (MHz) | LCRB*12*SCS (MHz) | A-MPR |
| --- | --- | --- | --- |
| 25 MHz | ≤ LCRB*12*SCS - 5 | > 5 | A7 |
|  | ≤ 6.48 | ≤ 1.44 | A8 |
|  |  | ≤ 3.6 | A9 |
| 30 MHz | ≤ LCRB*12*SCS - 5 | > 5 | A7 |
|  | ≤ 8.64 | ≤ 1.44 | A8 |
|  |  | ≤ 3.6 | A9 |
| 40 MHz | ≤ 4.32 | > 0 | A1 |
|  | > 4.32, ≤ 10.44 | ≤ 10.8 | A3 |
|  | > 4.32, ≤ 18 | > 10.8 | A2 |
|  | > 18, ≤ 31.68 | > max (31.68 – RBstart*12*SCS, 0) | A6 |
|  | > 31.68 | > 0 | A5 |
| NOTE 1: The A-MPR values are specified in Table 6.2.3.3.19-2. |  |  |  |

Table 6.2.3.3.19-2: A-MPR for NS_50 (Power Class 3)

| Modulation/Waveform |  | A1 (dB) | A2 (dB) | A3 (dB) | A5 (dB) | A6 (dB) | A7 (dB) | A8 (dB) | A9 (dB) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | Outer/Inner | Outer/Inner | Outer/Inner | Outer/Inner | Outer/Inner | Outer/Inner | Outer/Inner | Inner |
| DFT-s-OFDM | Pi/2 BPSK | ≤ 11 | ≤ 7 | ≤ 3 | ≤ 5 | ≤ 2 | ≤ 4 | ≤ 2 |  |
|  | QPSK | ≤ 11 | ≤ 7 | ≤ 3 | ≤ 5 | ≤ 2 | ≤ 5 | ≤ 2 |  |
|  | 16 QAM | ≤ 11 | ≤ 7 | ≤ 3 | ≤ 5 | ≤ 2 | ≤ 5 | ≤ 2.5 |  |
|  | 64 QAM | ≤ 11 | ≤ 7 | ≤ 3 | ≤ 5 |  | ≤ 5 |  |  |
|  | 256 QAM | ≤ 11 | ≤ 7 |  | ≤ 5 |  | ≤ 5 |  |  |
| CP-OFDM | QPSK | ≤ 12 | ≤ 8 | ≤ 4.5 | ≤ 5 | ≤ 3.5 | ≤ 6.5 |  | ≤ 3.0 |
|  | 16 QAM | ≤ 12 | ≤ 8 | ≤ 4.5 | ≤ 5 | ≤ 3.5 | ≤ 6.5 |  | ≤ 3.0 |
|  | 64 QAM | ≤ 12 | ≤ 8 | ≤ 4.5 | ≤ 5 |  | ≤ 6.5 |  |  |
|  | 256 QAM | ≤ 12 | ≤ 8 |  |  |  | ≤ 6.5 |  |  |

Table 6.2.3.3.19-3: A-MPR regions for NS_50 (Power Class 2)

| Channel Bandwidth (MHz) | RBstart*12*SCS (MHz) | LCRB*12*SCS (MHz) | A-MPR |
| --- | --- | --- | --- |
| 10 MHz | ≤ 1.44 | < 1.44 | A5 |
|  | ≤ 1.8 | ≥ 2.7+2* RBstart*12*SCS | A4 |
|  | >1.8 | ≥ 8.1- RBstart*12*SCS | A4 |
| 15 MHz | ≤ 2.88 | < 2.7 | A5 |
|  | ≤ 3.24 | ≥ 2.7+2* RBstart*12*SCS | A3 |
|  | >3.24 | ≥ 12.42- RBstart*12*SCS | A4 |
| 20 MHz | ≤ 4.32 | < 3.6 | A5 |
|  | ≤ 4.5 | ≥ 3.6+2* RBstart*12*SCS | A3 |
|  | >4.5 | ≥ 17.1- RBstart*12*SCS | A4 |
| 25 MHz | ≤ LCRB*12*SCS – 5 | > 5 | A2 |
|  | ≤ 6.48 | ≤ 1.44 | A5 |
|  | > 8.28 | > max (21.6 – RBstart*12*SCS, 0), <RBstart*12*SCS+5 | A4 |
|  | >1.8, ≤6.48 | > 1.44, ≤ 3.6 | A6 |
|  | > LCRB *12*SCS – 5, ≤ 1.8 | > 1.44 | A4 |
| 30 MHz | ≤ LCRB*12*SCS – 5 | >5 | A2 |
|  | ≤ 7.56 | ≤ 1.44 | A5 |
|  | >1.8, ≤7.56 | > 1.44, ≤ 3.6 | A6 |
|  | ≤ 1.8 | >1.44, <RBstart*12*SCS+5 | A4 |
|  | > 10.8 | > max (26.64 – RBstart*12*SCS, 0), <RBstart*12*SCS+5 | A4 |
| 40 MHz | ≤ 4.32 | > 0 | A1 |
|  | > 4.32 | > RBstart*12*SCS + 11.88 | A8 |
|  | > 4.32, ≤ 12.96 | ≤ 10.8 | A3 |
|  | > 4.32, ≤ 18 | > 10.8, <= RBstart*12*SCS + 11.88 | A7 |
|  | > 18, ≤ 31.68 | > max (31.68 – RBstart*12*SCS, 0) | A4 |
|  | > 31.68 | > 0 | A9 |
| NOTE 1: The A-MPR values are specified in Table 6.2.3.3.19-4. |  |  |  |

Table 6.2.3.3.19-4: A-MPR for NS_50 (Power Class 2)

| Modulation/Waveform |  | A1 (dB) | A2 (dB) | A3 (dB) | A4 (dB) | A5 (dB) | A6 (dB) | A7 (dB) | A8 (dB) | A9 (dB) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | Outer/ Inner | Outer/ Inner | Outer/ Inner | Outer/ Inner | Outer/ Inner | Outer/ Inner | Outer /Inner | Outer /Inner | Outer /Inner |
| DFT-s-OFDM | Pi/2 BPSK | ≤ 11.5 | ≤4.5 | ≤ 4 | ≤ 2.5 | ≤ 4 | ≤ 1 | ≤ 3.5 | ≤ 10 | ≤ 8 |
|  | QPSK | ≤ 11.5 | ≤ 5.5 | ≤ 4 | ≤ 2.5 | ≤ 4 | ≤ 1 | ≤ 3.5 | ≤ 10 | ≤ 8 |
|  | 16 QAM | ≤ 11.5 | ≤ 5.5 | ≤ 4 | ≤ 2.5 | ≤ 4 | ≤ 1.5 | ≤ 3.5 | ≤ 10 | ≤ 8 |
|  | 64 QAM | ≤ 11.5 | ≤ 5.5 | ≤ 4 |  | ≤ 4 |  |  | ≤ 10 | ≤ 8 |
|  | 256 QAM | ≤ 11.5 | ≤ 5.5 |  |  |  |  |  | ≤ 10 | ≤ 8 |
| CP-OFDM | QPSK | ≤ 12.5 | ≤ 7 | ≤ 5.5 | ≤ 4 | ≤ 4 | ≤ 2 | ≤ 5 | ≤ 11 | ≤ 8 |
|  | 16 QAM | ≤ 12.5 | ≤ 7 | ≤ 5.5 | ≤ 4 | ≤ 4 |  | ≤ 5 | ≤ 11 | ≤ 8 |
|  | 64 QAM | ≤ 12.5 | ≤ 7 | ≤ 5.5 | ≤ 4 | ≤ 4 |  | ≤ 5 | ≤ 11 | ≤ 8 |
|  | 256 QAM | ≤ 12.5 | ≤ 7 |  |  |  |  |  | ≤ 11 | ≤ 8 |

<Unchanged Sections Skipped >

Table 6.2.3.4.1-32: Test Configuration table for NS_50 (Power Class 2)

| Initial Conditions |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Test Environment as specified in TS 38.508-1 [5] subclause 4.1 |  |  |  |  |  |  | Normal |  |  |  |
| Test Frequencies as specified in TS 38.508-1 [5] subclause 4.3.1 |  |  |  |  |  |  | (See Freq column for test frequency, and configuration detail specified in Table 6.2.3.4.1-32a, Table 6.2.3.4.1-32a and Table 6.2.3.4.1-32c) |  |  |  |
| Test Channel Bandwidths as specified in TS 38.508-1 [5] subclause 4.3.1 |  |  |  |  |  |  | 10MHz, 15MHz, 20MHz, 25 MHz, 30MHz, 40MHz |  |  |  |
| Test SCS as specified in Table 5.3.5-1 |  |  |  |  |  |  | Lowest, Highest |  |  |  |
| A-MPR test parameters for NS_50 (Power Class 2) |  |  |  |  |  |  |  |  |  |  |
| Test ID | Fc (MHz) | Ch BW (MHz) | SCS (kHz) | Downlink Configuration | Uplink Configuration |  |  |  |  |  |
|  |  |  |  |  | Modulation (Note 2) |  | RB allocation (Note 1) |  |  |  |
|  |  |  |  |  |  |  | SCS 15 kHz | SCS 30 kHz |  | SCS 60 kHz |
| 1 | 1890 | 10 | Default | N/A | DFT-s-OFDM | QPSK | Edge_1RB_left (A5) |  |  |  |
| 2 | 1890 | 10 | Default |  |  | QPSK | Outer_Full (A4) |  |  |  |
| 3 | 1890 | 10 | Default |  |  | QPSK | 40@11 (A4) | 18@6 (A4) |  | / |
| 4 | 1892.5 | 15 | Default |  |  | QPSK | Edge_1RB_left (A5) |  |  |  |
| 5 | 1892.5 | 15 | Default |  |  | QPSK | Outer_Full (A3) |  |  |  |
| 6 | 1892.5 | 15 | Default |  |  | QPSK | 60@19 (A4) | 27@10 (A4) |  | 15@5 (A4) |
| 7 | 1895 | 20 | Default |  |  | QPSK | Edge_1RB_left (A5) |  |  |  |
| 8 | 1895 | 20 | Default |  |  | QPSK | Outer_Full (A3) |  |  |  |
| 9 | 1895 | 20 | Default |  |  | QPSK | 80@26 (A4) | 36@13 (A4) |  | 18@7 (A4) |
| 10 | 1897.5 | 25 | Default |  |  | QPSK | Outer_Full (A2) |  |  |  |
| 11 | 1897.5 | 25 | Default |  |  | QPSK | Edge_1RB_left (A5) |  |  |  |
| 12 | 1897.5 | 25 | Default |  |  | QPSK | 75@48 (A4) | 36@25 (A4) |  | 18@13 (A4) |
| 13 | 1897.5 | 25 | Default |  |  | QPSK | 20@11 (A6) | 10@6 (A6) |  | 5@3 (A6) |
| 14 | 1897.5 | 25 | Default |  |  | QPSK | 27@0 (A4) | 12@0 (A4) |  | 6@0 (A4) |
| 15 | 1900 | 30 | Default |  |  | QPSK | Outer_Full (A2) |  |  |  |
| 16 | 1900 | 30 | Default |  |  | QPSK | Edge_1RB_left (A5) |  |  |  |
| 17 | 1900 | 30 | Default |  |  | QPSK | 20@11 (A6) | 10@6 (A6) |  | 5@3 (A6) |
| 18 | 1900 | 30 | Default |  |  | QPSK | 27@0 (A4) | 12@0 (A4) |  | 6@0 (A4) |
| 19 | 1900 | 30 | Default |  |  | QPSK | 90@63 (A4) | 45@32 (A4) |  | 24@18 (A4) |
| 20 | 1900 | 40 | Default |  |  | QPSK | Outer_Full (A1) |  |  |  |
| 21 | 1900 | 40 | Default |  |  | QPSK | 180@25 (A8) | 90@13 (A8) |  | 40@7 (A8) |
| 22 | 1900 | 40 | Default |  |  | QPSK | 60@25 (A3) | 30@13 (A3) |  | 15@7 (A3) |
| 23 | 1900 | 40 | Default |  |  | QPSK | 90@25 (A7) | 45@13 (A7) |  | 20@7 (A7) |
| 24 | 1900 | 40 | Default |  |  | QPSK | 108@101 (A4) | 54@51 (A4) |  | 25@26 (A4) |
| 25 | 1900 | 40 | Default |  |  | QPSK | 36@177 (A9) | 16@89 (A9) |  | 6@45 (A9) |
| 26 | 1890 | 10 | Default |  |  | 256 QAM | Edge_1RB_left (A5) |  |  |  |
| 27 | 1890 | 10 | Default |  |  | 256 QAM | Outer_Full (A4) |  |  |  |
| 28 | 1890 | 10 | Default |  |  | 256 QAM | 40@11 (A4) | 18@6 (A4) |  | / |
| 29 | 1892.5 | 15 | Default |  |  | 256 QAM | Edge_1RB_left (A5) |  |  |  |
| 30 | 1892.5 | 15 | Default |  |  | 256 QAM | Outer_Full (A3) |  |  |  |
| 31 | 1892.5 | 15 | Default |  |  | 256 QAM | 60@19 (A4) | 27@10 (A4) |  | 15@5 (A4) |
| 32 | 1895 | 20 | Default |  |  | 256 QAM | Edge_1RB_left (A5) |  |  |  |
| 33 | 1895 | 20 | Default |  |  | 256 QAM | Outer_Full (A3) |  |  |  |
| 34 | 1895 | 20 | Default |  |  | 256 QAM | 80@26 (A4) | 36@13 (A4) |  | 18@7 (A4) |
| 35 | 1897.5 | 25 | Default |  |  | 256 QAM | Outer_Full (A2) |  |  |  |
| 36 | 1897.5 | 25 | Default |  |  | 256 QAM | Edge_1RB_left (A5) |  |  |  |
| 37 | 1897.5 | 25 | Default |  |  | 256 QAM | 75@48 (A4) | 36@25 (A4) |  | 18@13 (A4) |
| 38 | 1897.5 | 25 | Default |  |  | 256 QAM | 20@11 (A6) | 10@6 (A6) |  | 5@3 (A6) |
| 39 | 1897.5 | 25 | Default |  |  | 256 QAM | 27@0 (A4) | 12@0 (A4) |  | 6@0 (A4) |
| 40 | 1900 | 30 | Default |  |  | 256 QAM | Outer_Full (A2) |  |  |  |
| 41 | 1900 | 30 | Default |  |  | 256 QAM | Edge_1RB_left (A5) |  |  |  |
| 42 | 1900 | 30 | Default |  |  | 256 QAM | 20@11 (A6) | 10@6 (A6) |  | 5@3 (A6) |
| 43 | 1900 | 30 | Default |  |  | 256 QAM | 27@0 (A4) | 12@0 (A4) |  | 6@0 (A4) |
| 44 | 1900 | 30 | Default |  |  | 256 QAM | 90@63 (A4) | 45@32 (A4) |  | 24@18 (A4) |
| 45 | Low | 40 | Default |  |  | 256 QAM | Outer_Full (A1) |  |  |  |
| 46 | Low | 40 | Default |  |  | 256 QAM | 180@25 (A8) | 90@13 (A8) |  | 40@7 (A8) |
| 47 | Low | 40 | Default |  |  | 256 QAM | 60@25 (A3) | 30@13 (A3) |  | 15@7 (A3) |
| 48 | Low | 40 | Default |  |  | 256 QAM | 90@25 (A7) | 45@13 (A7) |  | 20@7 (A7) |
| 49 | Low | 40 | Default |  |  | 256 QAM | 108@101 (A4) | 54@51 (A4) |  | 25@26 (A4) |
| 50 | Low | 40 | Default |  |  | 256 QAM | 36@177 (A9) | 16@89 (A9) |  | 6@45 (A9) |
| 51 | 1890 | 10 | Default | N/A | CP-OFDM | QPSK | Edge_1RB_left (A5) |  |  |  |
| 52 | 1890 | 10 | Default |  |  | QPSK | Outer_Full (A4) |  |  |  |
| 53 | 1890 | 10 | Default |  |  | QPSK | 41@11 (A4) | 18@6 (A4) | / |  |
| 54 | 1892.5 | 15 | Default |  |  | QPSK | Edge_1RB_left (A5) |  |  |  |
| 55 | 1892.5 | 15 | Default |  |  | QPSK | Outer_Full (A3) |  |  |  |
| 56 | 1892.5 | 15 | Default |  |  | QPSK | 60@19 (A4) | 28@10 (A4) | 13@5 (A4) |  |
| 57 | 1895 | 20 | Default |  |  | QPSK | Edge_1RB_left (A5) |  |  |  |
| 58 | 1895 | 20 | Default |  |  | QPSK | Outer_Full (A3) |  |  |  |
| 59 | 1895 | 20 | Default |  |  | QPSK | 80@26 (A4) | 38@13 (A4) | 17@7 (A4) |  |
| 60 | 1897.5 | 25 | Default |  |  | QPSK | Outer_Full (A2) |  |  |  |
| 61 | 1897.5 | 25 | Default |  |  | QPSK | Edge_1RB_left (A5) |  |  |  |
| 62 | 1897.5 | 25 | Default |  |  | QPSK | 74@47 (A4) | 37@24 (A4) | 19@13 (A4) |  |
| 63 | 1897.5 | 25 | Default |  |  | QPSK | 20@11 (A6) | 10@6 (A6) | 5@3 (A6) |  |
| 64 | 1897.5 | 25 | Default |  |  | QPSK | 27@0 (A4) | 13@0 (A4) | 6@0 (A4) |  |
| 65 | 1900 | 30 | Default |  |  | QPSK | Outer_Full (A2) |  |  |  |
| 66 | 1900 | 30 | Default |  |  | QPSK | Edge_1RB_left (A5) |  |  |  |
| 67 | 1900 | 30 | Default |  |  | QPSK | 20@11 (A6) | 10@6 (A6) | 5@3 (A6) |  |
| 68 | 1900 | 30 | Default |  |  | QPSK | 27@0 (A4) | 13@0 (A4) | 6@0 (A4) |  |
| 69 | 1900 | 30 | Default |  |  | QPSK | 88@61 (A4) | 44@31 (A4) | 22@16 (A4) |  |
| 70 | 1900 | 40 | Default |  |  | QPSK | Outer_Full (A1) |  |  |  |
| 71 | 1900 | 40 | Default |  |  | QPSK | 191@25 (A8) | 93@13 (A8) | 44@7 (A8) |  |
| 72 | 1900 | 40 | Default |  |  | QPSK | 60@25 (A3) | 30@13 (A3) | 15@7 (A3) |  |
| 73 | 1900 | 40 | Default |  |  | QPSK | 91@25 (A7) | 46@13 (A7) | 23@7 (A7) |  |
| 74 | 1900 | 40 | Default |  |  | QPSK | 115@101 (A4) | 55@51 (A4) | 25@26 (A4) |  |
| 75 | 1900 | 40 | Default |  |  | QPSK | 39@177 (A9) | 17@89 (A9) | 6@45 (A9) |  |
| 76 | 1890 | 10 | Default |  |  | 256 QAM | Edge_1RB_left (A5) |  |  |  |
| 77 | 1890 | 10 | Default |  |  | 256 QAM | Outer_Full (A4) |  |  |  |
| 78 | 1890 | 10 | Default |  |  | 256 QAM | 41@11 (A4) | 18@6 (A4) | / |  |
| 79 | 1892.5 | 15 | Default |  |  | 256 QAM | Edge_1RB_left (A5) |  |  |  |
| 80 | 1892.5 | 15 | Default |  |  | 256 QAM | Outer_Full (A3) |  |  |  |
| 81 | 1892.5 | 15 | Default |  |  | 256 QAM | 60@19 (A4) | 28@10 (A4) | 13@5 (A4) |  |
| 82 | 1895 | 20 | Default |  |  | 256 QAM | Edge_1RB_left (A5) |  |  |  |
| 83 | 1895 | 20 | Default |  |  | 256 QAM | Outer_Full (A3) |  |  |  |
| 84 | 1895 | 20 | Default |  |  | 256 QAM | 80@26 (A4) | 38@13 (A4) | 17@7 (A4) |  |
| 85 | 1897.5 | 25 | Default |  |  | 256 QAM | Outer_Full (A2) |  |  |  |
| 86 | 1897.5 | 25 | Default |  |  | 256 QAM | Edge_1RB_left (A5) |  |  |  |
| 87 | 1897.5 | 25 | Default |  |  | 256 QAM | 74@47 (A4) | 37@24 (A4) | 19@13 (A4) |  |
| 88 | 1897.5 | 25 | Default |  |  | 256 QAM | 20@11 (A6) | 10@6 (A6) | 5@3 (A6) |  |
| 89 | 1897.5 | 25 | Default |  |  | 256 QAM | 27@0 (A4) | 13@0 (A4) | 6@0 (A4) |  |
| 90 | 1900 | 30 | Default |  |  | 256 QAM | Outer_Full (A2) |  |  |  |
| 91 | 1900 | 30 | Default |  |  | 256 QAM | Edge_1RB_left (A5) |  |  |  |
| 92 | 1900 | 30 | Default |  |  | 256 QAM | 20@11 (A6) | 10@6 (A6) | 5@3 (A6) |  |
| 93 | 1900 | 30 | Default |  |  | 256 QAM | 27@0 (A4) | 13@0 (A4) | 6@0 (A4) |  |
| 94 | 1900 | 30 | Default |  |  | 256 QAM | 88@61 (A4) | 44@31 (A4) | 22@16 (A4) |  |
| 95 | 1900 | 40 | Default |  |  | 256 QAM | Outer_Full (A1) |  |  |  |
| 96 | 1900 | 40 | Default |  |  | 256 QAM | 191@25 (A8) | 93@13 (A8) | 44@7 (A8) |  |
| 97 | 1900 | 40 | Default |  |  | 256 QAM | 60@25 (A3) | 30@13 (A3) | 15@7 (A3) |  |
| 98 | 1900 | 40 | Default |  |  | 256 QAM | 91@25 (A7) | 46@13 (A7) | 23@7 (A7) |  |
| 99 | 1900 | 40 | Default |  |  | 256 QAM | 115@101 (A4) | 55@51 (A4) | 25@26 (A4) |  |
| 100 | 1900 | 40 | Default |  |  | 256 QAM | 39@177 (A9) | 17@89 (A9) | 6@45 (A9) |  |
| NOTE 1: The specific configuration of each RB allocation is defined in Table 6.1-1 unless otherwise stated in this table. |  |  |  |  |  |  |  |  |  |  |

Table 6.2.3.4.1-32a: Additional test frequencies for NS_50 (SCS=15 kHz, ΔFRaster = 15 kHz)

| CBW [MHz] | carrierBandwidth [PRBs] | Range |  | Carrier centre [MHz] | Carrier centre [ARFCN] | point A [MHz] | absoluteFrequencyPointA [ARFCN] | offsetToCarrier [Carrier PRBs] | SS block SCS [kHz] | GSCN | absoluteFrequencySSB [ARFCN] |  | Offset Carrier CORESET#0 [RBs] Note 2 | CORESET#0 Index (Offset [RBs]) Note 1 | offsetToPointA (SIB1) [PRBs] Note 1 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 10 | 52 | DL & UL | - | 1890 | 378000 | 1885.32 | 377064 | 0 | 30 | 4725 | 378030 | 10 | 0 | 1(6) | 6 |
| 15 | 79 | DL & UL | - | 1892.5 | 378500 | 1885.39 | 377078 | 0 | 15 | 4729 | 378290 | 8 | 7 | 7(16) | 23 |
| 20 | 106 | DL & UL | - | 1895 | 379000 | 1885.46 | 377092 | 0 | 15 | 4718 | 377530 | 2 | 0 | 1(2) | 2 |
| 25 | 133 | DL & UL | - | 1897.5 | 379500 | 1885.53 | 377106 | 0 | 15 | 4743 | 379470 | 8 | 17 | 12(38) | 55 |
| 30 | 160 | DL & UL | - | 1900 | 380000 | 1885.6 | 377120 | 0 | 15 | 4729 | 378290 | 6 | 6 | 7(16) | 22 |
| 40 | 216 | DL & UL | - | 1900 | 380000 | 1880.56 | 376112 | 0 | 15 | 4729 | 378290 | 6 | 12 | 12(38) | 50 |

Table 6.2.3.4.1-32b: Additional test frequencies for NS_50 (SCS=30 kHz, ΔFRaster = 30 kHz)

| CBW [MHz] | carrierBandwidth [PRBs] | Range |  | Carrier centre [MHz] | Carrier centre [ARFCN] | point A [MHz] | absoluteFrequencyPointA [ARFCN] | offsetToCarrier [Carrier PRBs] | SS block SCS [kHz] | GSCN | absoluteFrequencySSB [ARFCN] |  | Offset Carrier CORESET#0 [RBs] Note 2 | CORESET#0 Index (Offset [RBs]) Note 1 | offsetToPointA (SIB1) [PRBs] Note 1 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 10 | 52 | DL & UL | - | 1890 | 378000 | 1885.68 | 377136 | 0 | 30 | 4725 | 378030 | 10 | 0 | 2(2) | 4 |
| 15 | 79 | DL & UL | - | 1892.5 | 378500 | 1885.66 | 377132 | 0 | 15 | 4729 | 378290 | 2 | 3 | 3(8) | 22 |
| 20 | 106 | DL & UL | - | 1895 | 379000 | 1885.82 | 377164 | 0 | 15 | 4754 | 380410 | 2 | 20 | 9(20) | 80 |
| 25 | 133 | DL & UL | - | 1897.5 | 379500 | 1885.8 | 377160 | 0 | 15 | 4743 | 379470 | 2 | 7 | 9(20) | 54 |
| 30 | 160 | DL & UL | - | 1900 | 380000 | 1885.96 | 377192 | 0 | 15 | 4729 | 378290 | 6 | 2 | 3(8) | 20 |
| 40 | 216 | DL & UL | - | 1900 | 380000 | 1880.92 | 376184 | 0 | 15 | 4729 | 378290 | 6 | 4 | 9(20) | 48 |

Table 6.2.3.4.1-32c: Additional test frequencies for NS_50 (SCS=60 kHz, ΔFRaster = 15 kHz, without CORESET#0)

| CBW [MHz] | carrierBandwidth [PRBs] | Range |  | Carrier centre [MHz] | Carrier centre [ARFCN] | point A [MHz] | absoluteFrequencyPointA [ARFCN] | offsetToCarrier [Carrier PRBs] | SS block SCS [kHz] | GSCN | absoluteFrequencySSB [ARFCN] |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 10 | 52 | DL & UL | - | 1890 | 378000 | 1886.04 | 377208 | 0 | 15 | - | 377568 |
| 15 | 79 | DL & UL | - | 1892.5 | 378500 | 1886.02 | 377204 | 0 | 15 | - | 377564 |
| 20 | 106 | DL & UL | - | 1895 | 379000 | 1886.36 | 377272 | 0 | 15 | - | 377632 |
| 25 | 133 | DL & UL | - | 1897.5 | 379500 | 1886.34 | 377268 | 0 | 15 | - | 377628 |
| 30 | 160 | DL & UL | - | 1900 | 380000 | 1886.32 | 377264 | 0 | 15 | - | 377624 |
| 40 | 216 | DL & UL | - | 1900 | 380000 | 1881.64 | 376328 | 0 | 15 | - | 376688 |

Editor’s note: The following lines belong at the end of subclause 6.2.3.4.1. As new tables are added to this section, these lines should always follow the tables

1.	Connect the SS to the UE antenna connectors as shown in TS 38.508-1 [5] Annex A, Figure A.3.1.1.1 for TE diagram and section A.3.2 for UE diagram.

2.	The parameter settings for the cell are set up according to TS 38.508-1 [5] subclause 4.4.3.

3.	Downlink signals are initially set up according to Annex C.0, C.1, C.2 and uplink signals according Annex G.0, G.1, G.2 and G.3.0.

4.	The UL Reference Measurement channels are set according to the applicable table from Table 6.2.3.4.1-1 to Table 6.2.3.4.1-30.

5.	Propagation conditions are set according to Annex B.0.

6.	Ensure the UE is in state RRC_CONNECTED with generic procedure parameters Connectivity NR, Connected without release On, Test Mode On and Test Loop Function On according to TS 38.508-1 [5] clause 4.5. Message contents are defined in clause 6.2.3.4.3.

< End of Changes >